Multimedia

Standards Update

New Multimedia IDs

July 28, 2026

Revision: 1.1.27

Information in this document is subject to change without notice and does not represent a commitment on the part of Microsoft Corporation.  The software described in this document is furnished under license agreement or nondisclosure agreement.  The software may be used or copied only in the accordance with the terms of the agreement.  It is against the law to copy the software on any medium except as specifically allowed in the license or nondisclosure agreement.

No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, for any purpose without the express written permission of Microsoft Corporation.

This standards update is for informational purposes only.  Microsoft makes no warranties, expressed or implied in this documnet.

Microsoft, MS, MS-DOS, XENIX and the Microsoft logo are registered trademarks and Windows is a trademark of Microsoft Corporation.  Other trade names mentioned herein are trademarks of their respective manufacturers.

Copyright 1992, Microsoft Corporation.  All Rights Reserved.

Table of Contents

µOverview	3

Where to Look for Information	3

Versions of this Document	3

Part Numbers	3

Manufacturer IDs	4

Products IDs	7

MM_MICROSOFT Product IDs	7

MM_CREATIVE Product IDs	7

MM_MEDIAVISION Product IDs	8

MM_ARTISOFT Product IDs	8

MM_TURTLE_BEACH Product IDs	8

MM_IBM Product IDs	8

MM_VOCALTEC Product IDs	8

MM_ROLAND Product IDs	8

MM_DIGISPEECH Product IDs	11

MM_NEC Product IDs	11

MM_ATI Product IDs	11

MM_WANGLABS Product IDs	11

MM_TANDY Product IDs	11

MM_VOYETRA Product IDs	12

MM_ANTEX Product IDs	12

MM_ICL_PS Product IDs	15

MM_INTEL Product IDs	15

MM_GRAVIS Product IDs	15

MM_VAL Product IDs	16

MM_INTERACTIVE Product IDs	16

MM_YAMAHA Product IDs	16

MM_EVEREX Product IDs	17

MM_ECHO Product IDs	17

MM_SIERRA Product IDs	18

MM_CAT Product IDs	18

MM_APPS Product IDs	18

# Overview

This standards update presents new and updated information for multimedia manufacture and product IDs.  This document is also available as part of the Multimedia Developer Registration Kit.  The MDRK is used to register multimedia data and ids as well as new MCI command sets..  This document is the result of companies requesting and registering new data types.  This document builds on the standard manufacturer and product IDs and WAVE format tags that are contained in:

1.	The Multimedia Development Kit (MDK) 1.0 Programmer's Reference

2.	The Windows 3.1 Software Development Kit (SDK)'s Multimedia Programmer's Reference

3.	The Multimedia Programmer's Reference book from Microsoft Press

The new IDs defined here have been registered with Microsoft.  If you want to register your own IDs, fill out and return the Multimedia Developer Registration Kit included in this kit.

## Where to Look for Information

All constants and structures defined in this document are contained in MMREG.H, which is included in this kit.

Current versions of this document as well as other technical update and technical notes and sample code are available from the sources listed in the Multimedia Document Overview, included in this kit.

## Versions of this Document

This document is continually being updated and expanded.  Eventually the information presented in this document will be placed in the standard reference for the multimedia IDs standards from Microsoft, such as the Multimedia Programmer's Reference from MS-Press.

When refering to standards defined in this document, please refer to the data and version number printed on the cover page.

Please refer to the document , which is included in this kit, for lists of other documents and sample code.

## Part Numbers

This document is contained in the following kit #s:

098-39211	Kit Win 3.1 MCI & RIFF Documnation

098-39212	Kit Win 3.1 MCI & RIFF Update

# Manufacturer IDs

| Reg Date | Manufacturer IDs | ### | Description |
| --- | --- | --- | --- |
| 10/01/91 | MM_MICROSOFT | 1 | Microsoft Corporation |
| 10/01/91 | MM_CREATIVE | 2 | Creative Labs Inc. |
| 10/01/91 | MM_MEDIAVISION | 3 | Media Vision Inc. |
| 01/01/92 | MM_FUJITSU | 4 |  |
| 10/01/91 | MM_ARTISOFT | 20 | Artisoft Inc. |
| 01/01/92 | MM_TURTLE_BEACH | 21 |  |
| 10/01/91 | MM_IBM | 22 | International Bussiness Machines Corp. |
| 03/27/92 | MM_VOCALTEC | 23 | Vocaltec LTD. 20 McDonald St. Ramat Gan 52514 ISRAEL |
| 03/27/92 | MM_ROLAND | 24 |  |
| 04/21/92 | MM_DIGISPEECH | 25 | Digispeech, Inc. 2464 Embarcadero Way Palo Alto, CA 94303 Tel: 415/494-8086 |
| 04/23/92 | MM_NEC | 26 | NEC |
| 04/23/92 | MM_ATI | 27 | ATI |
| 05/20/93 | MM_WANGLABS | 28 | Wang Laboratories, Inc. One Industrial Avenue Lowell, MA 01851 |
| 06/08/93 | MM_TANDY | 29 | Tandy Corporation 1800 One Tandy Center Fort Worth, Texas 76102 Tel: 817/390-3011 |
| 07/01/92 | MM_VOYETRA | 30 | Voyetra 333 5th Avenue Pelham, NY 10803 Tel: 914 /38-4500 FAX: 914/738-6946 |
| 08/03/92 | MM_ANTEX | 31 | Antex Electronics 16100 South Figueroa Street Gardena, California 90248 Tel: 310/532-3092 FAX: 310/532-8509 Compuserve: 70254,3651 |
| 08/03/92 | MM_ICL_PS | 32 | ICL PERSONAL SYSTEMS Station House Market Street Bracknell United Kingdom RG11 1LH Tel: +44 344 480932 FAX: +44 344 480042 |

All numbers in all tables are in decimal unless specifically hex (ie, 0x0030).

Manufacturer IDs, Continued

| Reg Date | Manufacturer IDs | ### | Description |
| --- | --- | --- | --- |
| 08/13/92 | MM_INTEL | 33 | Intel Corporation 5200 NE Elam Young Pkwy Hillsboro, OR 97124 Tel: 503/696-2448 |
| 08/14/92 | MM_GRAVIS | 34 | Advanced Gravis 1602 Carolina Street Unit #D12 Bellingham, WA 98226 Tel: 604/431 5020 |
| 08/25/92 | MM_VAL | 35 | Video Associates Labs, Inc. 4926 Spicewood Springs Road Austin, TX 78759 Tel: 512/346-5781 FAX: 512/346-9407 |
| 09/14/92 | MM_INTERACTIVE | 36 | InterActive, Inc 204 North Main Humboldt, South Dakota 57035 Tel: 605/363-5117 Tel: 800/292-2112 FAX: 605/363-5102 |
| 09/25/92 | MM_YAMAHA | 37 | Yamaha Corporation of America Systems Technology Division 981 Ridder Park Drive San Jose, CA 95131 Tel: 408 437-3133 Fax: 408 437-8791 |
| 09/25/92 | MM_EVEREX | 38 | Everex Systems, Inc. 48431 Milmont Drive Fremont, CA 94538 Tel: 510 498-1111 Fax: 510 651-0728 |
| 09/25/92 | MM_ECHO | 39 | Echo Speech Corporation 6460 Via Real Carpinteria, CA 93013 Tel: 805 684-4593 Fax: 805 684-6628 |
| 11/05/92 | MM_SIERRA | 40 | Sierra Semiconductor Corp. 2075 North Capitol Ave. San Jose, CA 95132 Tel: 408 263-9300 Fax: 408 263-3337 |
| 11/05/92 | MM_CAT | 41 | Computer Aided Technologies 10132 Monroe Drive Dallas, TX 75229 Tel: 214 350-0888 Fax: 214 904-0888 |
| 12/16/92 | MM_APPS | 42 | APPS Software International 4417 North Saddlebag Trail Scottsdale, AZ 85251 Tel: 602 947-2227 Fax: 602 947-2280 |
| 01/04/93 | MM_DSP_GROUP | 43 | DSP Group, Inc 4050 Moorpark Avenue San Jose, CA 95117 Tel: 408 985-0722 Fax: 408 985-2108 |

All numbers in all tables are in decimal unless specifically hex (ie, 0x0030).

# Products IDs

## MM_MICROSOFT Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 10/01/91 | MM_MIDI_MAPPER | 1 | MIDI Mapper |
| 10/01/91 | MM_WAVE_MAPPER | 2 | Wave Mapper |
| 10/01/91 | MM_SNDBLST_MIDIOUT | 3 | Sound Blaster MIDI output port |
| 10/01/91 | MM_SNDBLST_MIDIIN | 4 | Sound Blaster MIDI input port |
| 10/01/91 | MM_SNDBLST_SYNTH | 5 | Sound Blaster internal synthesizer |
| 10/01/91 | MM_SNDBLST_WAVEOUT | 6 | Sound Blaster waveform output |
| 10/01/91 | MM_SNDBLST_WAVEIN | 7 | Sound Blaster waveform input |
| 10/01/91 | MM_ADLIB | 9 | Ad Lib-compatible synthesizer |
| 10/01/91 | MM_MPU401_MIDIOUT | 10 | MPU401-compatible MIDI output port |
| 10/01/91 | MM_MPU401_MIDIIN | 11 | MPU401-compatible MIDI input port |
| 10/01/91 | MM_PC_JOYSTICK | 12 | Joystick adapter |
| 09/28/92 | MM_PCSPEAKER_WAVEOUT | 13 | PC Speaker waveform output |
| 09/28/92 | MM_MSFT_WSS_WAVEIN | 14 | MS Audio Board waveform input |
| 09/28/92 | MM_MSFT_WSS_WAVEOUT | 15 | MS Audio Board waveform output |
| 09/28/92 | MM_MSFT_WSS_FMSYNTH_STEREO | 16 | MS Audio Board Stereo FM Synthesizer |
| 09/28/92 | MM_MSFT_WSS_OEM_WAVEIN | 18 | MS OEM Audio Board waveform input |
| 09/28/92 | MM_MSFT_WSS_OEM_WAVEOUT | 19 | MS OEM Audio Board waveform Output |
| 09/28/92 | MM_MSFT_WSS_OEM_FMSYNTH_STEREO | 20 | MS OEM Audio Board Stereo FM Synthesizer |
| 09/28/92 | MM_MSFT_WSS_AUX | 21 | MS Audio Board Auxiliary Port |
| 09/28/92 | MM_MSFT_WSS_OEM_AUX | 22 | MS OEM Audio Auxiliary Port |

## MM_CREATIVE Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 10/01/91 | MM_CREATIVE_SB15_WAVEIN | 1 | SB (r) 1.5 Waveform Input |
| 10/01/91 | MM_CREATIVE_SB20_WAVEIN | 2 | SB (r) 2.0 Waveform Input |
| 10/01/91 | MM_CREATIVE_SBPRO_WAVEIN | 3 | SB Pro (r) Waveform Input |
| 10/01/91 | MM_CREATIVE_SBP16_WAVEIN | 4 | SBP16 (r) Waveform Input |
| 10/01/91 | MM_CREATIVE_SB15_WAVEOUT | 101 | SB (r) 1.5 Waveform Output |
| 10/01/91 | MM_CREATIVE_SB20_WAVEOUT | 102 | SB (r) 2.0 Waveform Output |
| 10/01/91 | MM_CREATIVE_SBPRO_WAVEOUT | 103 | SB Pro (r) Waveform Output |
| 10/01/91 | MM_CREATIVE_SBP16_WAVEOUT | 104 | SBP16 (r) Waveform Output |
| 10/01/91 | MM_CREATIVE_MIDIOUT | 201 | SB (r) MIDI output port |
| 10/01/91 | MM_CREATIVE_MIDIIN | 202 | SB (r) MIDI input port |
| 10/01/91 | MM_CREATIVE_FMSYNTH_MONO | 301 | SB (r) FM Synthesizer |
| 10/01/91 | MM_CREATIVE_FMSYNTH_STEREO | 302 | SB Pro (r) Stereo FM Synthesizer |
| 10/01/91 | MM_CREATIVE_AUX_CD | 401 | SB Pro (r) aux (CD) |
| 10/01/91 | MM_CREATIVE_AUX_LINE | 402 | SB Pro (r) aux (Line in) |
| 10/01/91 | MM_CREATIVE_AUX_MIC | 403 | SB Pro (r) aux (mic) |

## MM_MEDIAVISION Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 10/01/91 | MM_PROAUD_MIDIOUT | 21 | MediaVision MIDI output port |
| 10/01/91 | MM_PROAUD_MIDIIN | 22 | MediaVision MIDI input port |
| 10/01/91 | MM_PROAUD_SYNTH | 23 | MediaVision Synthesizer |
| 10/01/91 | MM_PROAUD_WAVEOUT | 24 | MediaVision Waveform output |
| 10/01/91 | MM_PROAUD_WAVEIN | 25 | MediaVision Waveform input |
| 10/01/91 | MM_PROAUD_MIXER | 26 | MediaVision Mixer |
| 10/01/91 | MM_PROAUD_AUX | 27 | MediaVision aux |
| 03/31/92 | MM_MEDIAVISION_THUNDER | 32 | Thunderboard Sound Card |

## MM_ARTISOFT Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 10/01/91 | MM_ARTISOFT_SBWAVEIN | 1 | Artisoft Sound Board Waveform Input |
| 10/01/91 | MM_ARTISOFT_SBWAVEOUT | 2 | Artisoft Sound Board Waveform Output |

## MM_TURTLE_BEACH Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |

## MM_IBM Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 03/31/92 | MM_MMOTION_WAVEAUX | 1 | IBM M-Motion Auxiliary Device |
| 03/31/92 | MM_MMOTION_WAVEOUT | 2 | IBM M-Motion Waveform Output |
| 03/31/92 | MM_MMOTION_WAVEIN | 3 | IBM M-Motion Waveform Input |

## MM_VOCALTEC Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 04/21/92 | MM_VOCALTEC_WAVEOUT | 1 | Vocaltec Waveform output port |
| 04/21/92 | MM_VOCALTEC_WAVEIN | 2 | Vocaltec Waveform input port |

## MM_ROLAND Product IDs

Product Name: SC-7  GENERAL MIDI SOUND MODULE

Product Information:

The Roland SC-7 sound module contains a large collection of high-quality sounds

and MIDI interface, and is designed to support the General MIDI System.

·	General MIDI System Level 1 Specifications

The SC-7 accurately reproduces any GM Score as it complies with General MIDI

System Level 1 specifications.

·	16-Part Multi-Timbral and 28-Voice Polyphony

The SC-7 is a 16-Part(including the Drum Part) multi-timbral sound module and

its maximum polyphony is 28-Voice.

·	Reverb and Chorus

The digital reverb and chorus effects are available to enhance your music.

·	Simple Connections

Serial cables allow you to connect your SC-7 directly to your computer without

the need for an external MIDI interface.

·	Two sets of Stereo Inputs

Two stereo inputs allow you to mix signals from external sources(audio output

from your computer, CD-ROM drive or cassette player for example) with the

SC-7's output.

·	Compact Design

Its compact design makes the SC-7 ideal for integration into any desktop music

system.

Product Name: S-MPU/AT  INTELLIGENT MIDI PROCESSING UNIT

Product Information:

The Super MPU is an intelligent type MIDI processing unit that can process MIDI

data without increasing the work of the host computer even on a multi-media

platform where many different softwares are being involved and should work at

the same time.  The Super MPU is equipped with a 16 bit CPU for stable and

delay-free MIDI data processing, and also with an 8 bit CPU for processing

SMPTE information ( two CPUs are included ).

In the future, we may need to record or play back MIDI data, animation on a CD-

ROM player, using more than one application qwing to the growth of multi-media.

This will, however, increase the work load on a host computer.  Even in such

conditions, the Super MPU enables stable MIDI data processing without applying

too much work load to the applications.

The Super MPU has 401 emulation mode, so you can use any software compatible

with the MPU-401.

·	The speed of the message processing is higher and the I/O ports are easier to

use

I/O Ports

Equipped with two different data buses

It is provided with two types of data bus, an 8 bit data bus for

compatibility with the MPU-401 and a 16 bit data bus for higher speed

data communication with the host computer.

Equipped with Interface Buffer

Each of the I/O Ports is provided with a buffer memory that makes it

possible to transmit or receive more than one function or message at the

same time.

Equipped with the Event Buffer

Individually for playback and recording, event buffers with as many as

2000 words are provided, reducing the work of the host for MIDI data

processing.  That is, even when animation or computer graphics are

involved, stable and delay-free playback or recording can be performed

easily.

·	Improvement in MIDI Processing

Two independent MIDI OUTs are equipped

Two independent MIDI INs are equipped

Rechannelizing of MIDI messages

MIDI Message Filter

MIDI Message Modify

Monitoring the message in the same timing as it is sent from the MIDI OUT

·	Processing the SMPTE Time Code

Equipped with the SMPTE Time Code Reader/Generator

Compatible with all types of Frame Rate

SMPTE Time Code Error Correction Function

·	Resolution

24, 96, 120, 480 or 960 TPQN ( ticks per quarter note ) can be selected.

·	Other functions

Tempo Recording Function

Reserve Play Function that allows to set the starting time for playing

Sending the Cue Event to the Host at the specified time

·	Specifications

Connectors

MIDI Connector ( IN x 2, OUT x 2 )

SMPTE Jacks ( IN, OUT )

Connector Box Sockect

Interrupt Line Selector Switch : IRQ5, 7, 9, 10, 11

I/O Port Address Selector Switch : 0330h - 13F0h

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 12/01/92 | MM_ROLAND_MPU401_MIDIOUT | 15 | Roland MPU-401 |
| 12/01/92 | MM_ROLAND_MPU401_MIDIIN | 16 | Roland MPU-401 |
| 12/01/92 | MM_ROLAND_SMPU_MIDIOUTA | 17 | S-MPU/AT Intelligent MIDI Processing Unit |
| 12/01/92 | MM_ROLAND_SMPU_MIDIOUTB | 18 | S-MPU/AT Intelligent MIDI Processing Unit |
| 12/01/92 | MM_ROLAND_SMPU_MIDIINA | 19 | S-MPU/AT Intelligent MIDI Processing Unit |
| 12/01/92 | MM_ROLAND_SMPU_MIDIINB | 20 | S-MPU/AT Intelligent MIDI Processing Unit |
| 12/01/92 | MM_ROLAND_SC7_MIDIOUT | 21 | Roland SC-7 General MIDI Sound Module |
| 12/01/92 | MM_ROLAND_SC7_MIDIIN | 22 | Roland SC-7 General MIDI Sound Module |

## MM_DIGISPEECH Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 04/21/92 | MM_DIGISP_WAVEOUT | 1 | Digispeech Waveform output port |
| 04/21/92 | MM_DIGISP_WAVEIN | 2 | Digispeech Waveform input port |

## MM_NEC Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |

## MM_ATI Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |

## MM_WANGLABS Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 05/20/92 | MM_WANGLABS_WAVEIN1 | 1 | Input audio wave device present on the CPU board of the following Wang models: Exec 4010, 4030 and 3450; PC 251/25C, PC 461/25S and PC 461/33C |
| 05/20/92 | MM_WANGLABS_WAVEOUT1 | 2 | Output audio wave device present on the CPU board of the Wange models listed above. |

## MM_TANDY Product IDs

Tandy Corporation is a U.S., multinational retailer and manufacturer

of high-quality consumer electronics and personal computers.

Tandy is America's largest consumer electronics and personal computer

retailer operating over 7,400 stores and dealer/franchise outlets

in its distribution system comprising Radio Shack, McDuff Electronics,

VideoConcepts, The Edge in Electronics, Computer City, and Incredible Universe.

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 12/29/92 | MM_TANDY_VISWAVEIN | 1 |  |
| 12/29/92 | MM_TANDY_VISWAVEOUT | 2 |  |
| 12/29/92 | MM_TANDY_VISBISSSYNTH | 3 | Tandy Internal MIDI Synthesizer |
| 12/29/92 | MM_TANDY_SENS_MMAWAVEIN | 4 | Tandy Sensation! MMA |
| 12/29/92 | MM_TANDY_SENS_MMAWAVEOUT | 5 | Tandy Sensation! MMA |
| 12/29/92 | MM_TANDY_SENS_MMAMIDIIN | 6 | Tandy Sensation! MMA |
| 12/29/92 | MM_TANDY_SENS_MMAMIDIOUT | 7 | Tandy Sensation! MMA |
| 12/29/92 | MM_TANDY_SENS_VISWAVEOUT | 8 | Tandy Sensation! VIS-Compatible device |
| 12/29/92 | MM_TANDY_PSSJWAVEIN | 9 | Tandy PSSJ (DAC) Waveform Input |
| 12/29/92 | MM_TANDY_PSSJWAVEOUT | 10 | Tandy PSSJ (DAC) Waveform Output |

## MM_VOYETRA Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |

## MM_ANTEX Product IDs

Founded in Southern California in December 1983, Antex Electronics designs and manufactures high performance, PC based digital audio products for OEMS, VARS and system integrators worldwide.

Product Name:   SX-12 Digital Audio Processor Waveform input

Product ID:     MM_ANTEX_SX12_WAVEIN

The Antex Series 2/Model SX-12 digital audio processor add-in board features stereo (two channel), high fidelity, direct-to-disk, digital sound sampling and reproduction at rates from 6.25 to 50kHz.  The SX-12 incorporates Sigma Delta (64 times oversampling) Analog-to-Digital and Digital-to-Analog conversion in addition to a TMS320C25 Digital Signal Processor for data formating and compression.

Product Name:   SX-12 Digital Audio Processor Waveform input

Product ID:     MM_ANTEX_SX12_WAVEOUT

The Antex Series 2/Model SX-12 digital audio processor add-in board features stereo (two channel), high fidelity, direct-to-disk, digital sound sampling and reproduction at rates from 6.25 to50kHz.  The SX-12 incorporates Sigma Delta (64 times oversampling) Analog-to-Digital and Digital-to-Analog conversion in addition to a TMS320C25 Digital Signal Processor for data formating and compression.

Product Name:   SX-15 Digital Audio Processor Waveform input

Product ID:     MM_ANTEX_SX15_WAVEIN

The Antex Series 2/Model SX-15 digital audio processor add-in board features stereo (two channel), high fidelity, direct-to-disk, digital sound sampling and reproduction at rates from 6.25 to 50kHz.  The SX-15 incorporates Sigma Delta (64 times oversampling) Analog-to-Digital and Digital-to-Analog conversion in addition toa TMS320C51 Digital Signal Processor for data formating and compression.

Product Name:   SX-15 Digital Audio Processor Waveform output

Product ID:     MM_ANTEX_SX15_WAVEOUT

The Antex Series 2/Model SX-15 digital audio processor add-in board features stereo (two channel), high fidelity, direct-to-disk, digital sound sampling and reproduction at rates from 6.25 to 50kHz.  The SX-15 incorporates Sigma Delta (64 times oversampling)  Analog-to-Digital and Digital-to-Analog conversion in addition to a TMS320C51 Digital Signal Processor for data formating and compression.

Product Name:   VP625 Digital Audio Processor Waveform input

Product ID:     MM_ANTEX_VP625_WAVEIN

The Antex VP625 digital audio processor add-in board features direct-to-disk digital sound sampling and reproduction at rates from 8-16kHz. The VP625 incorporates 10 bit Analog-to-Digital and Digital-to-Analog conversion in addition to 3:1 ADPCM compression.

Product Name:   VP625 Digital Audio Processor Waveform output

Product ID:     MM_ANTEX_VP625_WAVEOUT

The Antex VP625 digital audio processor add-in board features direct-to-disk digital sound sampling and reproduction at rates from 8-16kHz. The VP625 incorporates 10 bit Analog-to-Digital and Digital-to-Analog conversion in addition to 3:1 ADPCM compression.

Product Name:   AudioPort Stereo Digital Audio Recorder Waveform input

Product ID:     MM_ANTEX_AUDIOPORT22_WAVEIN

The Antex Audioport external digital audio processor features stereo (two channel), high fidelity, direct-to-disk, digital sound sampling and reproduction at rates from 9.8 to 25.2kHz.  The Audioport incorporates 12 bit Analog-to-Digital and Digital-to-Analog conversion, in addition to software selectable 4:1 ADPCM compression.  The Audioport also offers stereo feed through and mixing capability and conveniently connects to a standard parallel

printer port.

Product Name:   AudioPort Stereo Digital Audio Recorder Waveform output

Product ID:     MM_ANTEX_AUDIOPORT22_WAVEOUT

The Antex Audioport external digital audio processor features stereo (two channel), high fidelity, direct-to-disk, digital sound sampling and reproduction at rates from 9.8 to 25.2kHz.  The Audioport incorporates 12 bit Analog-to-Digital and Digital-to-Analog conversion, in addition to software selectable 4:1 ADPCM compression.  The Audioport also offers stereo feed through and mixing capability and conveniently connects to a standard parallel

printer port.

Product Name:   AudioPort Stereo Digital Audio Recorder auxillary feedthrough

Product ID:     MM_ANTEX_AUDIOPORT22_FEEDTHRU

The Antex Audioport external digital audio processor features stereo (two channel), high fidelity, direct-to-disk, digital sound sampling and reproduction at rates from 9.8 to 25.2kHz.  The Audioport incorporates 12 bit Analog-to-Digital and Digital-to-Analog conversion, in addition to software selectable 4:1 ADPCM compression.  The Audioport also offers stereo feed through and mixing capability and conveniently connects to a standard parallel printer port.

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 08/03/92 | MM_ANTEX_SX12_WAVEIN | ??? | SX-12 Digital Audio Processor Waveform input |
| 08/03/92 | MM_ANTEX_SX12_WAVEOUT | ??? | SX-12 Digital Audio Processor Waveform input |
| 08/03/92 | MM_ANTEX_SX15_WAVEIN | ??? | SX-15 Digital Audio Processor Waveform input |
| 08/03/92 | MM_ANTEX_SX15_WAVEOUT | ??? | SX-15 Digital Audio Processor Waveform output |
| 08/03/92 | MM_ANTEX_VP625_WAVEIN | ??? | VP625 Digital Audio Processor Waveform input |
| 08/03/92 | MM_ANTEX_VP625_WAVEOUT | ??? | VP625 Digital Audio Processor Waveform output |
| 08/03/92 | MM_ANTEX_AUDIOPORT22_WAVEIN | ??? | AudioPort Stereo Digital Audio Recorder Waveform input |
| 08/03/92 | MM_ANTEX_AUDIOPORT22_WAVEOUT | ??? | AudioPort Stereo Digital Audio Recorder Waveform output |
| 08/03/92 | MM_ANTEX_AUDIOPORT22_FEEDTHRU | ??? | AudioPort Stereo Digital Audio Recorder auxillary feedthrough |

## MM_ICL_PS Product IDs

ICL is a major supplier of total business computing solutions worldwide. ICL has a comprehensive range of PCs, from Notebooks to Desktop PCs and Deskside Servers, along with the integration tools and expertise to build successful user solutions.

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |

## MM_INTEL Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 08/13/92 | MM_INTELOPD_WAVEIN | 1 | HID2 WaveAudio Input driver |
| 08/13/92 | MM_INTELOPD_WAVEOUT | 101 | HID2 WaveAudio Output driver |
| 08/13/92 | MM_INTELOPD_AUX | 401 | HID2 Auxiliary driver (required for mixing functions) |

## MM_GRAVIS Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |

## MM_VAL Product IDs

Video Associates Labs, which was founded in 1975, manufactures and sells computer-based audio and video products.

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 08/25/92 | MM_VAL_MICROKEY_AP_WAVEIN | ??? | MicroKey/AudioPort Waveform Input |
| 08/25/92 | MM_VAL_MICROKEY_AP_WAVEOUT | ??? | MicroKey/AudioPort Waveform Output |

## MM_INTERACTIVE Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 09/14/92 | MM_INTERACTIVE_WAVEOUT | 0x45 |  |
| 09/14/92 | MM_INTERACTIVE_WAVEIN | 0x45 |  |

## MM_YAMAHA Product IDs

The Yamaha Corporation of America, Systems Technology Division, was founded in April 1987 to manufacture and market integrated circuits, board-level products, and complete systems to OEMs and VARs.  The division's product line includes custom and semi-custom ASICs, standard circuits for graphics, communications and audio applications.

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 09/25/92 | MM_YAMAHA_GSS_SYNTH | 0x01 | Yamaha Gold Sound Standard FM sythesis driver |
| 09/25/92 | MM_YAMAHA_GSS_WAVEOUT | 0x02 | Yamaha Gold Sound Standard wave output driver |
| 09/25/92 | MM_YAMAHA_GSS_WAVEIN | 0x03 | Yamaha Gold Sound Standard wave input driver |
| 09/25/92 | MM_YAMAHA_GSS_MIDIOUT | 0x04 | Yamaha Gold Sound Standard midi output driver |
| 09/25/92 | MM_YAMAHA_GSS_MIDIIN | 0x05 | Yamaha Gold Sound Standard midi input driver |
| 09/25/92 | MM_YAMAHA_GSS_AUX | 0x06 | Yamaha Gold Sound Standard auxillary driver for mixer functions |

## MM_EVEREX Product IDs

Everex Systems, Inc. is the manufacturer of the STEP and TEMPO brand PC-Compatible Desktop computer systems, the Everex line of notebook PC-portable systems, and a new line of multiprocessor and fault-tolerant desktop computer systems directed at the server/traditional mini-computer marketplace. In addition, Everex manufactures and resells peripheral products, such as the high capacity 1G tape backup system.  Everex was founded in 1982, and became a publicly held corporation in 1987 (NASDDAQ = EVRX).

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 09/25/92 | MM_EVEREX_CARRIER | 0x01 | Everex Carrier SL/25 Notebook |

Everex Carrier SL/25 Notebook Product Information:

1.	Advanced power management through use of the Intel 386SL CPU and APM spec support for Windows 3.1 and DOS 5.0.

2.	Removable hard drive provides easy upgrade ability, ultimate data security, better serviceability, and flexibility.

3.	Continuous battery system allows swapping of the NiCad battery without interrupting system operation.

4.	SimulScan(TM), the ability to display on the LCD only, the external CRT only, or on both displays at the same time to enhance presentation abilities while on the road.

5.	Internal built in trackball provides convenient and easy-to-use portable pointing device.

6.	Built-in AM-quality microphone and speaker provides capability for voice annotation within applications supporting OLE, for text-to-speech voice synthesis, etc.

7.	Virus detection with limited virus protection. System has the ability to indicate possible virus infection.

## MM_ECHO Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |

## MM_SIERRA Product IDs

Sierra Semiconductor Corporation is a leading supplier of mixed analog digital chip sets that provide solutions for the PC.  Sierra participates in the modem, graphics and sound chip set business by providing high volume board manufacturers with complete solutions.

The AriaÔ family of chip sets provides the user with the combination of music synthesis based on sampled instruments (the Aria Synthesizer), and speech recognition (the Aria Listener).  Aria Listener is a speaker independent recognition algorithm that allows users to command and control their applications.

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 11/05/92 | MM_SIERRA_ARIA_MIDIOUT | 20 | Sierra Aria MIDI Output |
| 11/05/92 | MM_SIERRA_ARIA_MIDIIN | 21 | Sierra Aria MIDI Input |
| 11/05/92 | MM_SIERRA_ARIA_SYNTH | 22 | Sierra Aria MIDI Synthesizer |
| 11/05/92 | MM_SIERRA_ARIA_WAVEOUT | 23 | Sierra Aria Waveform output |
| 11/05/92 | MM_SIERRA_ARIA_WAVEIN | 24 | Sierra Aria Waveform input |
| 11/05/92 | MM_SIERRA_ARIA_AUX | 25 | Sierra Aria Auxiliary device (CD-Audio) |
| 11/05/92 | MM_SIERRA_ARIA_AUX2 | 26 | Sierra Aria Auxiliary 2 device |

## MM_CAT Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 11/05/92 | MM_CAT_WAVEOUT | 1 | CAT Waveform Output |

## MM_APPS Product IDs

APPS is a developer of electronic publishing and multimedia applications and tools for Microsoft Windows.

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |

## MM_DSP_GROUP Product IDs

| Reg Date | Products IDs | ### | Description |
| --- | --- | --- | --- |
| 01/04/93 | MM_DSP_GROUP_TRUESPEECH | 1 | High Quality 9.54:1 Speech Compression Vocoder |