***********************************************************************

*                                                                     *

*          MAC Driver Installation - Microsoft LAN Manager            *

*                    Intel TokenExpress Adapters                      *

*                                                                     *

*                          Release 5.0                                *

*                                                                     *

***********************************************************************

1.     RELEASE DESCRIPTION

1.1    Changes

The MAC Drivers have been upgraded to conform with the latest

version of the "3Com/Microsoft LAN Manager Network Driver Interface

Specification", namely version 2.0.1. The changes made are minor and

will normally be of no consequence for the user. The status and

statistics collection performed have however been changed and the

Intel Runtime Statistics and Status utility (OMACSTAT) has been

changed accordingly, refer the file OMACSTAT.DOC in the directory:

a:\mslanman\doc.

A new version of the adapter resident MAC code has been released by

Texas Instrument for the 16/4 Mbit adapters. This version (1.03) is

now used by the MAC drivers.

1.2 Qualification

The MAC driver qualification tests have been extended to include

qualification with DOS version 5.00

2. DOCUMENTATION ERRATA

2.1    PROTOCOL.INI - correction to "Guide to Operation"

The valid range has been changed for the following configuration

parameters in PROTOCOL.INI:

Min    Max    Default

-----------------------------------------------

MaxTransmits          2    100          6    (OS/2)

MaxTransmits          2      6          6    (DOS)

RecBufSize        256    5000        512

RecBufCount          4     10         10

Under DOS the total receive buffer capacity RecbufSize * RecBufCount

is limited to:

- About 7K bytes in an RPL workstation.

- About 20K bytes in a disk based workstation.

Small letters are allowed in the driver name parameter:

DriverName = "OLITOK$"

or

DriverName = "olitok$"

2.2 Programmed I/O - correction to "Guide to Operation"

Programmed I/O (Pseudo-DMA) with the 16/4 ISA adapter is

supported by the OS/2 driver. Note, that the max throughput of a

server using programmed i/o is reduced to about 25 % at 16 MB and

50 % at 4 MB compared to the standard master mode DMA.

3. DOCUMENTATION

The directory a:\mslanman\doc contains documentation, which has not

yet been added to the 'Guide to Operations'.

There are 5 files in this doc directory:

- RPL.DOC

Describes installation of the DOS driver files on an RIPL server.

- OS2RPL.DOC

Describes installation of OS2 driver files on an RIPL server.

- ERRCODE.DOC

Describes the adapter error codes, which may be generated by

the driver during driver initialization or adapter open.

- OPTIMIZE.DOC

Describes changes to configuration parameters in PROTOCOL.INI,

which may save memory or increase performance.

- OMACSTAT.DOC

Describes how to use the Runtime Statistics and Status Utility.