Quick reference  ISA 16/4 Token-Ring Adapter  switches.

The slide switch is used to select the speed on the Token Ring,

4 or 16 Mbps as labeled.

Switch #  1         2 .. 6       7,8        9,10       11,12

RPL       RPL Address  Interrupt  I/O Address  DMA

ENABLE    A17 .. A13   select     select       select

\_______/

|	      |		  |	     |	       D1  D0

|	      |		  |	     |	       on  on  DMA level 5 *

|	      |		  |	     |	       on  off DMA level 6

|	      |		  |	     |	       off on  DMA level 7

|	      |		  |	     |	       off off Programmed I/O

|	      |		  |	     |		       transfer (1)

|	      |		  |

|	      |		  |	  A1   A0

|	      |		  |       off  off  A20-A23, A30-A3F *

|	      |		  |	  on   off  A24-A27, A40-A4F

|	      |		  |	  off  on   A50-A53, A60-A6F

|	      |		  |	  on   on   A54-A57, A70-A7F

|	      |

|	      |	       I1   I0

|	      |	       on   on	Interrupt level 9 (2 in a PC/XT) *

|	      |	       on   off Interrupt level 3

|	      |	       off  on  Interrupt level 10 (not in a PC/XT)

|	      |	       off  off Interrupt level 11 (not in a PC/XT)

|	   ___|___

|	  /       \

|	  Location of 8 K address space for RPL PROM. The PROM can

|	  be placed on any 8 K boundary from C0000 to FE000.

|	  The switches define bits 13 through 17 of this address,

|	  with 'on' being logic 0, 'off' being logic 1.

|	  Bits 18,19 of the address are implicitly logic 1 always.

|

|	  Default is:

|

|	  fixed          switches         fixed

|        A19 A18   A17 A16 A15 A14 A13   A12 A11 ..

|	            on  on  off off on			*

|	   1   1     0   0   1   1   0     0   0  ..

|

|	  which causes the PROM to appear at CC000 - CDFFF.

EN

on   PROM is disabled (should be used when no PROM is installed) *

off  PROM is enabled

Notes:

*	denotes default switch postion

(1)	When installed in a PC/AT, this position forces the

driver / adapter to transfer data using programmed I/O.

It is not normally used. It may be necessary if the machine

does not support the AT Bus Master Mode properly.

When installed in a PC/XT or in an 8 bit slot of a PC/AT,

programmed I/O is used regardless of the switchsettings.

------------O-----------

The ISA 16/4 adapter may be installed in a PC/XT machine provided

that there is physically room for the AT part of the edge connector.

NOVELL NetWare drivers:

The NetWare 386 server driver does not currently support data

transfer using programmed I/O. With this driver the ISA adapter MUST

transfer data using Bus Master Mode.

The switchsetting D0,D1 = off,off is consequently illegal with this

driver.

The MC 16/4 and EISA 16/4 adapters always transfer data by becoming

bus masters.

The NetWare drivers always enable Early Token Release when running

at 16 Mbps.

------------O-----------