PUREDATA  C O N N E C T I O N

PRODUCT BULLETIN

Revised  Date:   /  /                   Bulletin No. 6503

Original Date: 07/04/91                 Document No. 0650000300

ECO No.      NA

Product: PUREDATA 10BASE-T products

_________________________________________________________________

Ethernet 10BASE-T

The 10BASE-T protocol describes an Ethernet Local Area

Network (LAN) implementation of CSMA/CD at 10 megabits per

second, baseband, Unshielded Twisted Pair (UTP) wire media.  The

establishment of an IEEE standard (IEEE 802.3i) for support of

the Ethernet protocol utilizing this wiring scheme in a

point-to-point (star/tree) topology for systems interconnect is

an important industry move.

Ethernet was originally designed as a method of providing a

high speed communications channel between mid range (mini

computers) and mainframe systems.  The common bus topology did

not typically require a large amount of connection points.  As

the Ethernet protocol gained acceptance in the desktop computer

environment, the requirement to interconnect large numbers of

micro computers in a local area network grew.  As more and more

micro computers were added to the LAN, the cabling constraints of

a common bus topology became more evident.

Interconnecting a large number of devices into a local

area network using a common bus topology can be a logistically

difficult process.  The implementation of 10BASE-T provides a

much more flexible (distributes star) type topology for Ethernet.

This type of topology historically has been much easier to

install and manage from a cabling viewpoint.  The technical

benefit of a distributed star topology and the economical

benefits derived from the use of UTP type wire make 10BASE-T an

attractive network solution.  This document will describe the

general characteristics of the CSMA/CD Ethernet communications

protocol and the implementation of 10BASE-T technology.

AN EXPLANATION TO ETHERNET (CSMA/CD)

Carrier Sense, Multiple Access (CSMA) is a "common

bus" media sharing scheme in which stations listen in to

what's happening on the network media (cable).  If the cable is

not in use, the station is permitted to transmit its' message.

CSMA is combined in IEEE 802.3 with a means of performing

Collision Detection;  hence CSMA/CD.

The multiple access feature of the protocol allows the

connection of many devices to the same common bus transmission

media.  The carrier sense feature allows each station to detect

activity on the channel;  this may be termed "listen before

talking".  A station will refrain from transmitting if it

detects that the channel is active.  At times, it is possible

that due to the time it takes for the signal from one station to

transit across the network (propagation delay) that two stations

can almost simultaneously detect that the channel is available

and initiate a transmission.  In this case a collision occurs.

Identification of the fact that a collision has occurred

takes place at the interface card level.  Circuitry on the

interface card senses the additive voltage on the media and

informs the higher levels that a collision has taken place.  In

the event of a collision, the stations involved emit a JAM pulse

to ensure that all other stations are aware that a collision has

occurred.  Subsequently, the involved stations "back off"

for a semi-random timeout period.  The length of the timeout

period is controlled by the individual stations network address.

Since no two network addresses are identical, no timeout period

will be of the same length.  If a station, upon re-entering

communications experiences further sequential collisions, this

back off time is extended for each successive collision to a

point that the station assumes that the channel is no longer

operative.

Collisions do occur in Ethernet and their random occurrence

cannot be predicted.  However, statistically, the incidence and

frequency of collisions on an Ethernet LAN is quite low.  The

Ethernet protocol experiences little loading effect until

utilization of the media approaches 80 percent.  This implies

that data transmissions are occurring on the media for 80% of the

time or more.  Typical implementations of Ethernet (or for that

matter, any access protocol) generally do not exceed 30 to 40

percent of media utilization at any time.  In this case, the wire

is quiet (available) for 40 to 70 percent of the time.

Additionally, there is only a short time interval following

the initiation of a transmission during which there is the

possibility of a collision.  This time interval is termed "slot

time".  The slot time interval is represented by the time

required to transmit 512 bits (64 octets) of data.  This number

represents the number of bits that must be transmitted to

completely fill the wire (end-to-end).  Following transmission of

these first 512 bits, all stations on the segment are aware that

a transmission is in progress and will refrain from transmitting.

For this reason, data frames must be of a certain (512 bit)

minimum length.  This length is controlled by the length of the

LLC data field in the frame.

The slot time interval is the ONLY time during

transmission of data that Ethernet is susceptible to a collision.

Once the slot time interval has passed, the station is said to

have acquired the channel, and the rest of the frame is

transmitted contention free.  Contention access systems (CSMA/CD)

therefore function better when the average frame size exceeds the

slot time interval, since most of the frame will then be

transmitted without interference by collision.  Note that

substituting cable of different impedances and lengths may

significantly alter the propagation delay and in consequence the

slot time interval.  Collisions that occur following expiry of

the slot time interval are considered as serious "system

failures", and their cause should be investigated and

corrected immediately.

Only the stations with immediate transmission

requirements are in contention for the media.  Additionally,

since the incidence of collisions on most Ethernet LAN's is low,

this fair distribution of access coupled with the high data rate

makes Ethernet one of the most efficient communications protocols

available.

��������������������������������������������������������������������ͻ

�                Direction of Propagation                            �

�         ����������������������������������������>>                 �

�     ��������������������������������������������������ͻ           �

�     �    Frame size is LESS than minimum frame size    �           �

�     ��������������������������������������������������ͼ           �

��������������������������������������������������������������������Ķ

�      �                                                   �         �

�      WS                                                  WS        �

�     (A)                                                 (B)        �

�        Note that station "A" has completed transmission            �

�        Prior to Station "B" sensing that the media is in use       �

�                                                                    �

� If Station "B" initiates a transmission in response to finding     �

� the media to be inactive, a collision will result.  Additionally,  �

� this type of collision generally results in a FATAL ERROR          �

��������������������������������������������������������������������ͼ

COMPONENTS OF AN ETHERNET LAN

While standard implementations of Ethernet require interface

cards and wire, 10BASE-T requires another component known as a

concentrator.  Wiring concentrators are used in 10BASE-T to

provide for a common point for wire distribution.  The advantage

of using a concentrator and a star type topology is that a bad

cable can be easily isolated, and its effect on the LAN can be

minimized.

10BASE-T CONCENTRATOR

In 10BASE-T each port on the concentrator connects to only

one workstation.  Concentrators are connected via the 10BASE2

(thin wire) BNC attachment incorporated in the Pure Data device.

Optionally, the concentrators may be connected via the active

RJ-45 ports (assuming that the patch cable provides for inversion

of transmit and receive pairs).

Pure Data manufactures a concentrator for 10BASE-T

(unshielded twisted pair) Ethernet PDC8023A-T12.  The

Ethernet concentrator is an active 10BASE-T repeater that

provides a common wiring point for 12 10BASE-T connections.  The

concentrator also functions as a media converter (via the 10BASE2

port).

Major Features;

= 10 megabit per second IEEE 802.3 10BASE-T standard

= Ethernet UTP wiring. (Standard allows 100 meters of UTP

from concentrator to workstation)

= 12 RJ-45 ports

= Built-in BNC thin wire Ethernet port (10BASE2)

= Link integrity test on each port continuously checks

the line from workstation to concentrator

= Automatically disconnects ports if either the link

integrity test fails or the card transmits continuously

(jabbers), to prevent network failure

= Automatically re-connects ports when failures are

corrected

= Test port detects incorrect twisted pair cable polarity

to simplify installation (receive pairs only)

= All data connections on front panel

= Rack mount package for 19" racks.  Low profile 1.75"

high

Indicators

= Link integrity LED per UTP port (Green)

= Jabber LED per port (Red)

= Power, traffic and collision LED's

= Test port polarity indication LED

How a Concentrator Works

Signalling on a 10BASE-T network is predicated upon two

factors;

- CSMA/CD

- Link integrity

Link integrity is a function of Ethernet products that

support the IEEE 802.3i 10BASE-T specifications.  Every 1/60th of

a second, a pulse is asserted onto the wire between a

concentrator and the interface card (or another concentrator).

This pulse is interpreted as a link integrity check.  If the

check passes, then the node is permitted to remain as a part of

the LAN.  In the event of a failure of link integrity, the

concentrator will automatically disable the associated port and

will turn the green link integrity LED OFF.

As well, the card in question will also discontinue the

green LED indication of proper link integrity.

The establishment of proper link integrity is of critical

importance in 10BASE-T.  It is vital that the receivers (which

are tested by link integrity) on each side of the link be

verified as operational prior to allowing the adapter to

participate in communications.  Otherwise, an adapter with a

non-functional receiver may continuously detect a quiet channel.

This will result in communications disruption when this adapter

initiates a transmission without knowledge of the true state of

the transmission media.

A concentrator counts as a device on the 10BASE2 segment

to which it is connected.  Additionally, since signals are

regenerated and re-timed at the concentrator level, this device

is also considered to be a repeater.  The following table

illustrates the use of the concentrator LED's.

Interface Cards

In order for the microcomputer to communicate

successfully on the network, an adapter, sometimes referred to an

a Network INterface Card (NIC) is required.

This adapter fits into a bus slot on the computer and is

used to connect the computer bus to the network cabling

structure.  Pure Data manufactures 8 and 16 bit multiseries

interface cards as well as a 16 bit micro channel adapter and an

Ethernet card for the Toshiba laptop.

�������                                                       ��Ϳ

�������������������������������������������������������������Ĵ�Ĵ

�����            E1 E2                                      ���  Diagnostic

����� BOOT PROM   � �                                         ��� LEDs

����� ���������   � �      �������         ����       E4    �ܳ�Ŀ

����� ���������            �������         ����      �����  �۳� � DB-15

�����                      �������         ����             �۳� � Connector

�����                                      ����             �߳���

�����                                                 E3 �    ��

�����                                                    �  �Ĵ���Thin-coax

����������������������������������Ŀ� � ڿ� � � � � � ���Ŀ �Ĵ����Network

��������������������   �����   Connector

�

E3 - Link Integrity (enable/disable)

��Ϳ E4 - Reverse Polarity (enable/disable)

��������������������������������Ĵ�Ĵ

E3 E4 ���

�  �    ��� (1) Card Activity LED (red)

�������         ����       �  �    ��� (2) Network Activity (grn)

�������         ����               ��� (3) Reverse Polarity LED (red)

�������         ����               ��� (4) Link Integrity LED (grn)

����               ��

��

�Ĵÿ 10 Base T

�������Ŀ� � ڿ� � � � � � ���Ŀ �Ĵ�� Network

��������������������   �����  Connector

�

10BASE-T ETHERNET CABLING RULES

The cable media specified for 10BASE-T is unshielded

twisted pair wire that corresponds to the specifications outlined

for IBM Type 3 wire;

- 24 AWG copper tinned, solid conductors

- 100 - 105 ohm characteristic impedance

- Minimum of two twists per foot

From a wiring installation and maintenance viewpoint, the

10BASE-T distributed star topology provides greater flexibility

than the more common bus topology that is generally associated

with Ethernet.  The concept of one concentrator port to one

workstation makes isolation of individual node problems much

simpler.

Unshielded twisted pair for local area network

communications is rapidly becoming the wiring system of choice

for most LAN installations.  This technology is economical and

easy to install and maintain.  For additional information on the

technical issues surrounding the application of UTP type wire for

local area networks, the reader is directed to the Pure Data mini

series selection entitled "Local Area Network Cabling

issues", available through Pure Data Technical Services,

Education.

General Cabling Rules

= Maximum segment length (concentrator to interface) is

100 meters

= Maximum number of repeaters/concentrators between any

two points on the LAN is four (4)

= Repeaters/concentrators count as devices on each segment to

which they are connected

= Uses two (2) pairs of UTP wire

= Connector type is RJ-45 modular, eight position

= BNC connectors on the concentrator MUST be terminated

with a 50 ohm "T"-connector impedance

Workstations are connected to concentrator ports by no

more than 100 meters of unshielded twisted pair wire.  No daisy

chaining of workstations is permitted.   Concentrators are

typically located in a centralized wiring closet and are

interconnected via the 10BASE2 connectors.  This will provide for

the maximum number of supported workstations on the LAN.  For

example;  there are three segments of 10BASE2 available to

connect concentrators (or other communicating devices).  Each of

the three segments is capable of supporting 30 devices.  Each

concentrator supports up to twelve (12) workstations.  Therefore,

a single 10BASE2 backbone system may support up to 90

concentrators, thus rendering connection points for 1080

workstations.

RJ-45 connections for unshielded twisted pair wiring are

polarity sensitive, and considerable care must be taken during

installation of the wiring system to maintain correct polarity.

The adapters will automatically compensate for reversed polarity

on the receive pairs, as will the concentrator.  If the

concentrator displays an indication that the receive pairs are

indeed reversed, attempts should be made immediately to correct

the situation, regardless of whether the hardware will compensate

or not.

The following table illustrates the proper pin connections

for the RJ-45 connector;

�������������������������������������ͻ

�RJ-45  MDI (Media Dependent Interface�

�������������������������������������͹

�  1  �  TD +                         �

�������������������������������������Ķ

�  2  �  TD -                         �

�������������������������������������Ķ

�  3  �  RD +                         �

�������������������������������������Ķ

�  4  �  Not used by 10BASE-T         �

�������������������������������������Ķ

�  5  �  Not used by 10BASE-T         �

�������������������������������������Ķ

�  6  �  RD-                          �

�������������������������������������Ķ

�  7  �  Not used by 10BASE-T         �

�������������������������������������Ķ

�  8  �  Not used by 10BASE-T         �

�������������������������������������ͼ

The Pure Data Ltd. Technical Support Hotline is available every weekday from:

8am to 8pm (EST) in Canada (Monday-Thursday, Friday 8am-5pm)  (416) 731-9884

8am to 8pm (EST) in USA (Monday-Thursday, Friday 8am-5pm)     (800) 661-8210

9am to 5pm (GMT) in the UK                                  (44) 71 924-3505

A staff of knowledgeable personnel is ready to assist users with any situations

involving Pure Data Ltd. products.  For late breaking news and product updates

refer to the README and .DOC files contained on the Support Disk.

PUREDATA LTD.             PUREDATA INC.              PUREDATA LTD.

180 West Beaver Creek Rd, 1740 South I-35, Suite 140 Unit 10, Port House

Richmond Hill, Ontario,   Carrollton, Texas          Plantation Wharf, York Rd.

L4B 1B4 CANADA            75006 USA                  London SW11 3TY, U.K

(416) 731-6444           (214) 242-2040             44-71-924-3505

FAX  (416) 731-7017       FAX (214) 242-9487         FAX 44-71-585-3103

BBS  (416) 492-5980       BBS (214) 242-3225         BBS 44-71-738-2910