****************************************************************************

M U L T I  -  F U N C T I O N  -  P E R I P H E R A L   68901

****************************************************************************

$FFFFFA00 [R/W] ******

$FFFFFA01 [R/W] 76543210 ......................................... GPIP-Data

$FFFFFA02 [R/W] ******

$FFFFFA03 [R/W] 76543210 ....................................... Active-Edge

||||||||

||||||++-- negative edge

|||||+---- Midi-IRQ has a positive edge

+++++----- negative edge

$FFFFFA04 [R/W] ******

$FFFFFA05 [R/W] 76543210 .................................... Data-Direction

||||||||

++++++++-- I/O-Ports are used as input.

$FFFFFA06 [R/W] ******

$FFFFFA07 [R/W] 76543210 ................................ Interrupt-Enable A

$FFFFFA08 [R/W] ******

$FFFFFA09 [R/W] 76543210 ................................ Interrupt-Enable B

$FFFFFA0A [R/W] ******

$FFFFFA0B [R/W] 76543210 ............................... Interrupt-Pending A

$FFFFFA0C [R/W] ******

$FFFFFA0D [R/W] 76543210 ............................... Interrupt-Pending B

$FFFFFA0E [R/W] ******

$FFFFFA0F [R/W] 76543210 ............................ Interrupt-In-Service A

$FFFFFA10 [R/W] ******

$FFFFFA11 [R/W] 76543210 ............................ Interrupt-In-Service B

$FFFFFA12 [R/W] ******

$FFFFFA13 [R/W] 76543210 .................................. Interrupt-Mask A

$FFFFFA14 [R/W] ******

$FFFFFA15 [R/W] 76543210 .................................. Interrupt-Mask B

$FFFFFA16 [R/W] ******

$FFFFFA17 [R/W] 76543___ .................................. Interrupt-Vektor

$FFFFFA18 [R/W] ******

$FFFFFA19 [R/W] ___43210 ................................... Timer-A-Control

$FFFFFA1A [R/W] ******

$FFFFFA1B [R/W] ___43210 ................................... Timer-B-Control

$FFFFFA1C [R/W] ******

$FFFFFA1D [R/W] _654_210 ................................. Timer-C+D-Control

$FFFFFA1E [R/W] ******

$FFFFFA1F [R/W] 76543210 ...................................... Timer-A-Data

$FFFFFA20 [R/W] ******

$FFFFFA21 [R/W] 76543210 ...................................... Timer-B-Data

$FFFFFA22 [R/W] ******

$FFFFFA23 [R/W] 76543210 ...................................... Timer-C-Data

$FFFFFA24 [R/W] ******

$FFFFFA25 [R/W] 76543210 ...................................... Timer-D-Data

$FFFFFA26 [R/W] ******

$FFFFFA27 [R/W] 76543210 ................ Synchronous-Character (??unused??)

$FFFFFA28 [R/W] ******

$FFFFFA29 [R/W] 76543210 ........................ USART-Control (??unused??)

$FFFFFA2A [R/W] ******

$FFFFFA2B [R/W] 76543210 ...................... Receiver-Status (??unused??)

$FFFFFA2C [R/W] ******

$FFFFFA2D [R/W] 76543210 ................... Transmitter-Status (??unused??)

$FFFFFA2E [R/W] ******

$FFFFFA2F [R/W] 76543210 ........................... USART-Data (??unused??)

----------------------------------------------------------------------------

General information

----------------------------------------------------------------------------

According to the 'ATARI FALCON 030 Service Guide' and the 'FALCON 030

Schematic Sheets' the serial portion of the MFP is not used, although the

XMIT- and RCV-Interrupts are enabled. Maybe they are used by the SCC.

The MFP is used in a way that is compatible with the ST. There are no

ignificant changes, except the new interrupthandles. You can program the

MFP like on ST.

The new interruptchannels on FALCON 030:

IRQ-adr. prio. status   source             system use it as

----------------------------------------------------------------------------

$0100    00    locked   I/O-Port 0         Printer Busy

$0104    01    locked   I/O-Port 1         Printer Acknowledge

$0108    02    enabled  I/O-Port 2         Midi-IRQ

$010C    03    locked   I/O-Port 3         DSP-IRQ

$0110    04    locked   Timer D            Timer or ??baud generator??

$0114    05    enabled  Timer C            Timer or 200 Hz System Timer

$0118    06    enabled  I/O-Port 4         IKBR/Midi-IRQ

$011C    07    locked   I/O-Port 5         IDE/FDD-IRQ

$0120    08    locked   Timer B            Timer or Display Enable-Signal

$0124    09    enabled  XMIT Error         ??unused??

$0128    10    enabled  XMIT Buffer empty  ??unused??

$012C    11    enabled  RCV Error          ??unused??

$0130    12    enabled  RCV Buffer full    ??unused??

$0134    13    locked   Timer A            Timer or DMA-Sound Interrupt

$0138    14    locked   I/O-Port 6         RS 232 Ring Indicator

$013C    15    locked   I/O-Port 7         DMA-Sound Interrupt

############################################################################

#################### research and documentation by AURA ####################

############################################################################