Did you know that ... there is a lot of information that may be accessed

from the ROM BIOS area in your IBM PC, regarding the operating characteristics

and options found on your own IBM PC? After careful analysis of data found in

the IBM Technical Reference manual a summary of the most useful information and

where/how it may be referenced has been prepared.

By specifying a DEF SEG=&H40 in any BASIC program, it is possible to

reference the following vectors (fields) in the ROM BIOS area by using a PEEK

function and the following offsets from the current segment as defined by the

DEF SEG statement.

&H0       -  RS232 Addresses on your IBM PC.

This will allow you to tell how many (up to

four) async cards are attached, if any.

&H8       -  Printer Addresses on your IBM PC.

This will tell you what printer addresses,

and how many (up to four) exist. Each is

addressed by a two byte Hex value.

&H10      -  Equipment Flag.

This field describes the setting of the

options switches. It describes what optional

devices are attached to the system. The

following lists the bit-significance of this

field:

Bit 0 - indicates that there are diskette

drives on the system.

Bit 1 - not used.

Bit 2,3 - Planar Ram Size (00=16K 10=32K 01=48K

11=64K)

Bit 4,5 - Initial Video Mode (00=Unused

10=40x25 Color

01=80x25 Color

11=80x25 Mono)

Bit 6,7 - Number of Diskette Drives (00=1 10=2

01=3 11=4) only if bit 0 = 1.

Bit 8   - Unused

Bit 9,10,11 - Number of RS232 Cards attached

Bit 12  - Game I/O Attached

Bit 13  - Not used

Bit 14,15   - Number of printers attached

&H13      -  Memory Size in K bytes.

&H15      -  I/O RAM Size in K bytes.

&H17      -  Keyboard Flag -- the following lists the masks

set to describe current keyboard status:

Byte 1;

&H80 - Insert state active

&H40 - Caps Lock State Has been toggled

&H20 - Num Lock State has been toggled

&H10 - Scroll Lock State has been toggled

&H08 - Alternate Shift key depressed

&H04 - Control Shift key depressed

&H02 - Left Shift key depressed

&H01 - Right Shift key depressed

Byte 2;

&H80 - Insert Key is depressed

&H40 - Caps Lock Key is depressed

&H20 - Num Lock Key is depressed

&H10 - Scroll Lock key is depressed

&H08 - Suspend key has been toggled

&H49      -  Current CRT mode

&H00 - 40x25 BW

&H01 - 40x25 Color

&H02 - 80x25 BW

&H03 - 80x25 Color

&H04 - 320x200 Color

&H05 - 320x200 BW

&H06 - 640x200 BW

&H07 - 80x25 B&W Card -- specialized use, used

internal to the video routines.

&H4A      -  Number of CRT columns

&H50      -  Cursor Position (one of eight)

&H60      -  Current cursor mode

&H6C      -  Low word of Timer count

&H6E      -  High word of Timer count

&H71      -  &H07 - Break key depressed

&HFA6E    -  Beginning of character regen memory

&HFF53    -  PRTSC routine address

#