Towards reducing the hardware complexity of feature detection-

based models

Bassem Medawar        and         Andrew Noetzel1

Polytechnic University

333 Jay St. Brooklyn, NY 11201

____________________

1. This paper will be presented at the International Joint

Conference on Neural Networks, Jan. 15-19, 1990, Washington, DC.

Abstract

A model for feature detection-based pattern recognition is

presented.  It attempts to improve on the hardware complexity of

existing models.  Traditionally, feature detection has been

implemented with brute force duplication of template-based

feature detectors, offering little scalability.  This model

eliminates the need to duplicate complex feature detectors using

instead operators to transform patterns.

1. Introduction

It has been shown [1] that the brain uses feature detection, in

its visual pattern recognition task.  Many researches have

attempted to capture the brain's pattern recognizing capability

in abstract models.  In their attempts, some have tried to remain

faithful to the biological principles underlying the functioning

and organization of the brain [2]. Others borrowed from the brain

the most important principles and tried to cast them in any model

that could be demonstrated to work [3,4].  The model in this

paper follows the latter approach.

The work done by Fukushima will first be examined, representing

earlier models of its class.  A new model, which attempts to

overcome their limitations, will then be described.  Finally the

paper will conclude with a description of future improvements to

the model.

2. Earlier work

Models which loosely follow the brain's architecture, have done

so based on the following elementary principles:

a) The neuron (as a threshold element) is the building block of

those models.

b) The neuron's output can be viewed as a boolean corresponding

to on and off activation states, or as a positive (bounded)

real number representing the activation rate of the neuron.

c) The pattern recognizing network is layered.

d) Each layer contains feature detecting cells with increasing

level of conceptual complexity, the higher the layer level.

Many models in the neural network literature applied those

principles.  Focus will be centered on Fukushima's model, because

it is the most elaborate and has been shown to work with a

(relatively) large retina of 128x128 pixels [5].

Fukushima's Neocognitron model [3,6] has layers with two levels

each.  The lower level is made up of groups of template matchers.

The higher level neurons take their inputs from groups at the

lower level that recognize the same object at different

positions.  The net effect is feature detectors tolerant of

displacement and slight distortion.

Fukushima's neocognitron suffers the following problems.  First,

hardware is not amiable to large scale implementations: in one

case [6], a simple 19x19 retina, 4 layer network implementation

required over 40 thousand cells, excluding non-responding ones.

Second, each learned template is duplicated in many positions

after being trained in only one position: this is problematic for

hardware implementations as well as being biologically

implausible.

3. The model

This model [fig. 1] is based on the premise that instead of

taking the feature detector to the pattern (multiplying the

number of feature detectors), we bring the pattern to the feature

detector (multiplying the pathways.)  As the example of the

neocognitron shows, duplicating complex feature detectors is

costly, in terms of number of cells.  What we hope to achieve is

a reduction in the overall number of cells.

The retina is thus divided into several marginally overlapping

receptive fields (RF's) of uniform size.  Simple hard-wired

operators provide a many-to-one mapping from the RF area to the

feature detector.  Those operators are divided into classes.  For

instance, on the lowest layer, the classes are displacement,

scaling, and rotation.  On higher layers, the classes include

positional and set operators.  Each class has its variations

within each RF, depending on where the operator maps from, and

the degree of the mapping.  For example, the displacement class

has variations which corresponds to the direction and the amount

of the displacement.

On the next level, within a layer, feature detectors take their

input from the output of the operators weighed by the optimal

feature pattern.  The output of those feature detectors

represents the degree of success with which a particular operator

maps into the optimal feature.  From this large pool of feature

detector outputs within a receptive field, the best variation and

degree for each class is selected.  Then, the optimum values

across the layer from each RF are combined to choose the best

class of operators.  This choice represents the consensus as to

which class of operators best maps into some feature.

The consensus is then fed back to lower levels, allowing each RF

to reset its own image of the retina according to the new

transformation.  Notice that while the application of the

operator class is enforced upon the layer, each RF implements the

transformation in a way that generates optimal mapping.

After one class is selected within a layer, the class is then

inhibited allowing another class to win in a second round.  The

process is then repeated until a threshold of desirable outcome

is exceeded.  The feature with best degree of success, can thus

be said to have been recognized.

[Fig. 1 is inserted about here]

Having recognized a feature for each RF on the first layer, the

output of the first layer is fed into the second layer.  A

similar process of transformation and recognition is carried out

in the second layer.  Finally, on the topmost layer, a feature

detector which conceptually represents an object is selected and

the whole process terminates.

4. The model's weakness

Simulating this model necessitated additional hardware that was

not originally envisioned.  While its design premise is simple,

the number of cells required to implement it is proportional to

the number of RF's, the number of classes, the number of

variations within a class, and the number of features within each

layer.

The model does not lend itself to a nice and simple mathematical

model to support it, and mathematical properties of the model are

not practical to implement.  For example, while it is

mathematically sound to say that two features are different if

one can not be generated from the other by applying any sequence

of operators in any order.  This property taken to the extreme is

not practical to use in order to incorporate self-organization

into the model.  While that the model can be augmented with

learning rules that change the weights on the feature detectors,

it fails to address how a whole new class of operators can be

learned.

The model has been shown in practice to fail under certain

circumstances: the wrong sequence of transformations is applied

leading to either faulty recognition or no recognition at all.

This is a result of the lack of communication between neighboring

RF's.

5. Conclusion and future work

A feature detection-based model was presented that was designed

to address the limitations of previous models.  The model was

developed from an innovative idea.  Although the model achieved

its objective of lesser overall hardware complexity, it had few

limitations of its own.

Currently, work is in progress on a new model.  This model

incorporates communication between neighboring RF, coupled with

hill climbing techniques to pick the best transformation to apply

within an RF image.  In effect, this will result in a reduction

in the number of pathways as only few transformations will be

implemented at a time.

References

[1] Kuffler S. W., Nicholls J. G., and Martin A. R., From Neuron

to Brain, 2nd Ed.  Sinauer Associates Inc. Publishers,

Sunderland, MA, 1984.

[2] Linsker R., Self-Organization in a Perceptual Network.  IEEE

Computer, March 1988, pp. 105-117.

[3] Fukushima K., and Miyake S., Neocognitron: A new algorithm

for pattern recognition tolerant of deformations and shifts

in position.  Pattern Recognition, Vol. 15, No. 6, pp. 455-

469, 1982.

[4] Widrow B., Adaline and Madaline - 1963.  IEEE 1st

International Conference on Neural Networks.  Vol. 1, pp.

143-157.

[5] Menon M. M., and Heinemann K. G., Classification of patterns

using a self-organizing neural network.  Neural Networks,

Vol. 1, pp. 201-215, 1988.

[6] Fukushima K.,  A neural network for visual pattern

recognition.  IEEE Computer, March 1988, pp. 65-74.