NAME

pr_nl -- print a carriage return/line feed to printer

pr_puts -- print a string to the printer

pr_putline -- print a string, and a cr/lf to the printer

pr_eject -- send a formfeed to the printer

pr_carret -- send a carriage return to the printer

pr_set -- set the number of the current printer

pr_putc -- send a character to the printer

SYNOPSIS

all routines except pr_putc are type void

unless noted, no parameters are needed:

pr_putline(str);  char *str;

pr_puts(str);     char *str;

pr_set(num);      int num 0, 1, or 2

r = pr_putc(ch);  char ch;  int r; (returned status)

DESCRIPTION

This series of routines use the low level functions

blpr() and blprstat() to access the LPT? ROM-BIOS drivers

directly.  pr_putc() returns the same status that is returned

by blprstat().  Be sure to use pr_set() to set which LPT is

to be accessed.  Note that the library defaults to LPT1:,

and pr_set() may be ignored if that printer is to be used.

EXAMPLE

pr_set(LP1);        /* LP1, LP2, LP3 defined in smdefs.h */

pr_putline("Hello World");

pr_eject();

This function is found in SMDLx.LIB for the Datalight Compiler