Speaker to Sound Blaster Amplifier Converter

There has always been times that I would like to control my speakers volume

level ( louder or softer ) than what the pc does. Since the sound blaster uses

external speakers and incorporates an amplifier, I will show you a 1 wire

connection to make this simple adaptor..

PARTS REQUIRED

1 micro hook radioshack 270-355 (if you don't want this permanent.

1 1,000 ohm resister (1Kohm) 1/4 watt will due..

1 shorting jumper radioshack 276-1512 (used to connect to motherboard)

manual from your mother board

sound blaster card

screwdriver, solder, soldering iron, tape or heat shrink tube, 12 inch or so

thin wire.

For temporary connection:

micro hook          1k resistor           1 shorting jumper

J------------------/\/\/\/\-----------------n

(Tape or shrink any connections after solder or bare wire)

Unplug your computer, open the case, remove the soundblaster card, with the

screwdriver. Identify R4 R3 by the mike jack, volume control.

Block diagram of sound blaster

------------------------------------------------------------------------|

|

R4    (A)   ----  |

--/\/\/\-|   ! MIC  |

-/\/\/\-|       ----  |

R3    (B)            |

() c38     |

() c37       |

|

(( |

( volume

( control

(( |

-----------------------------------------------------------------------|

Connection points (A), (B) are the 2 separate channels (left speaker / right

speaker) which ever is best in your situation. Connect the micro hook clip to

either point (A) or point (B) but not both. Now you can put your sound blaster

card back into the computer slot.

Next open your mother board manual. Read and identify the speaker jack.

on mine it is a 4 pin jack..   1)speaker data 2)no connect 3)ground 4)+5vdc.

Identify the SPEAKER DATA pin. Now connect the remaining wire from the sound

blaster, to the mother boards speaker data pin.

You are all ready to close the computer, and plug the power cord back in.

This Hardware Hack is made available form:

DIGITAL IMAGES (414) 338-9999