Hello.

I  have been working on a NetWare space-war game (for up to six players)  for

the  past few months, and one of the programming requirements was to be  able

to  send and receive IPX packets. Since the project is in QuickBASIC  4.0,  I

had to write my own IPX routines (I have routines in Pascal and "C").

I  am  releasing my IPX routines into the public domain so that you need  not

have  to write your own. I am taking a lot of time here doing this to meet  a

need. If I put forth effort now, it may save dozens of folks time by them not

having to figure it out themselves.

This  document is intended to highlight some of the information you will need

to know about IPX. It is NOT intended to replace better information.

To begin. . . . .

There  should have been seven files in the ZIP/PAK/LZH/ZOO/ARJ/ARC  file  you

downloaded. The file files should be:

IPX.BAS             All routines in one file

IPXS.BAS            Sample IPX Send Code

IPXS.EXE            Sample Executable IPX Send

IPXR.BAS            Sample IPX Receive Code

IPXR.EXE            Sample Executable IPX Receive

CHAT.BAS            A Chat program using these routines

IPX.DOC             This Document

If  you  do  not have all of these files, and would like a copy of them  all,

call  me, David Rice, at 714-458-6000. You may also leave a message  at  Fido

Node 102/902 (The Skeptic Tank).

These  two  routines  are NOT included with the QuickBASIC  NetWare  Function

Library I wrote previously. That library may be found on various BBSs by  the

name  QBNW-20.ZIP,  which  includes  Print  Queues,  connection  information,

network address retrieval, etc. If you downloaded this file thinking you were

getting QBNW-20, well, this ain't it.

Note  that  the routines here are IPX and =not= SPX. I chose to use  IPX  for

several  reasons. The first is, it's faster than SPX. Another is, IPX packets

have  a  slightly larger datagram area, and can thus carry more  information.

The  third is, IPX is a "connectionless" packet protocol whereas SPX requires

a  connection  before a packet may be sent and received. With IPX,  the  user

need not be logged onto the network: she or he need only load IPX.COM.

IPX does NOT guarantee delivery, as SPX does. When an IPX packet is received,

its  header will contain the address of the computer that sent it. This means

that  even though IPX does not guarantee delivery, the program you  write  to

send and receive the packets can do the checking itself. For example:

Computers A and B are running your program.

Computer  A  sends a packet to Computer B and then  checks  the

packet's  INUSE  flag.  When  that  flag  turns  to  zero  (0),

Computer A goes into listen mode. If the INUSE flag stays  non-

zero  for  five or ten seconds (you decide), there  may  be  an

error,  and  your  program may handle that  error  as  you  the

programmer sees fit.

So  now  Computer  B finishes whatever it was doing,  and  goes

into  listen  mode. It says, "Hey! I have a  packet  here."  It

reads  the packet, setting the INUSE flag to zero (see  how  it

works?).  It  should  then  go into  Send  mode,  and  send  an

acknowledgment to Computer A, saying "Got it, thanks a  bunch!"

It  will know which computer to send it to, because the  packet

it received has the address in the header.

Though I suspect you may send multiple IPX packets without checking to see if

the  first arrived before sending the second, there is a good reason to  wait

for  the  confirmation. IPX packets are not sequenced, as SPX ones  are.  The

computer doing the "Listening" may very well receive the second packet before

it  receives the first! We wouldn't want the space ship to blow up before the

torpedo hits it, now would we?! Of course not!

I  suggest you use the first four bytes of the Datagram (the area that  holds

the packets' data) as a sequence field. Every time a computer sends a packet,

add 1 to this counter. This way, the receiving computer will know that it has

missed a packet when it reads the Datagram's sequence value and notices  that

it incremented by 2 (or more) instead of 1.

If  you  do use a Sequence field, the size of your Datagram will go from  546

bytes to 542 bytes.

IPX  packets may be fragmented. This means that if you have data that  is  in

logical  chunks, you may specify those chunks within the ECB structure.  Your

size limit is still 546 bytes. Why do multible fragments? Suppose you wish to

send  in one packet several different variables, such as A$, B$, C$, and  D$.

You could define five fragments: one for the 30-byte header, and four for the

four variables. Behold:

TYPE ECBStructure

LinkAddressOff   AS Integer

.

.

.

FragAddOfs1   AS INTEGER

FragAddSeg1   As INTEGER

FragSize1   AS INTEGER

FragAddOfs2   AS INTEGER

FragAddSeg2   AS INTEGER

FragSize2   AS INTEGER

.

.

FragAddOfs5   AS INTEGER

FragAddSeg5   AS INTEGER

FragSize5   AS INTEGER

END TYPE

'

DIM ECBS AS ECBStructure

'         '

ECBS.FragAddOfs1 = VARPTR(IPXS)

ECBS.FragAddSeg1 = VARGEG(IPXS)

ECBS.FragSize1 = 30

ECBS.FragAddOfs2 = VARPTR(A$)

ECBS.FragAddSeg2 = VARSEG(A$)

ECBS.FragSize2 = LEN(A$)

.

.

ECBS.FragAddOfs5 = VARPTR(D$)

ECBS.FragAddSeg5 = VARSEG(D$)

ECBS.FragSize5 = LEN(D$)

In  the send sample IPXS, the Destination Network was set to four zeros.  See

the  code,  IPXS.DestNode. This means that the IPX packet will be distributed

on the default network: the one that the computer is hard cabled to--- if you

are  sending packets across bridges or otherwise to another network, YOU MUST

KNOW THAT NETWORK'S ADDRESS AND PUT IT IN THIS FIELD.

In  the send sample, IPXS, the Destination  Node Address was set to six  FFs.

This  means the packet will be "broadcast" to all computers. If you  wish  to

send  a packet to a specific computer, you must place that computer's address

in IPXR.DestNode.

Enhanced Program Flow:

Computer A sends a broadcast packet to DestNode FFFFFFFFFFFF,  saying

"Anyone out there?"

Computer  B receives the broadcast packet and sends an acknowledgment

to Computer A. "Yep, I'm here. What of it?"

Computer  A receives the acknowledgment packet, and MAKES A  NOTE  OF

ITS  ADDRESS.  From  then  on, it sends only  specifically  addressed

packets.

Along   comes   Computer   C.  It  sends  a  broadcast   packet   (on

FFFFFFFFFFFF) Saying "Anyone out there?"

Both  Computer  A  and  Computer B send  acknowledgment  packets  to

Computer C.

Both Computer A and Computer B make a note of Computer C's address.

Computer C makes a note of Computer A's and Computer B's address.

Now all computers know each others' address, and none need broadcast

on  FFFFFFFFFFFF. If a fourth computer, Computer D, comes along,  it

makes a broadcast on FFFFFFFFFFFF, and all computers make a note  of

the new computer's address, and the new computer makes a note of the

other computers' address.

It's not difficult to keep track of all this. Just REDIM an array to hold all

of  the Network Addresses (IPXS.DestNet) and another to hold the Network Node

Addresses  (IPXS.DestNode), and keep track of how many computers are  running

your computer. This is called a Distribution List.

If  you're  a bright lass or lad, you will have noticed something wrong  with

sending  packets  out as broadcasts (FFFFFFFFFFFF). If  a  computer  sends  a

packet on a socket, it may very well be the only one to receive it! In  other

words, it can, and will, be talking to itself. The solution, of course, is to

use  two sockets: one to send, one to listen. However, that makes yet another

problem! Consider:

Your program sends on Socket 1, and listens on Socket 2. THAT'S the

problem! Any other computer running your program will be doing  the

same  damn thing; listening on a socket that none send on  but  all

listen to, and sending on a socket all send on but none listen  to.

That's no way to strike up a meaningful conversation.

If  you  try and be smart, and say "I'll listen and send on  both,"

you'll  just end up talking to yourself again, and that's where  we

came into this house of mirrors!

The solution? If you must broadcast, check ALL received packets for

the source node, and if it came from one's own computer, ignore it!

That is, even though you're talking to yourself, you don't have  to

listen, let alone talk back.

Suppose  a  computer runs your program, and it is the only one at the  moment

running  it. This computer must wait for other computers to run your program.

You  could  have  the computer send and receive on the same  socket:  send  a

packet then listen for a packet. When the computer receives a packet that  is

not  self-addressed  (Node  number), that means a new  computer  has  started

running your program.

For  the same scenario above, but using two sockets (which is a better idea),

your  computer  could  send on one socket and listen on  another.  After  the

computer  listens for 3 or 5 seconds, have your program SWITCH  SOCKETS,  but

AFTER  your program issues an IXPCancelS call on the original SEND.  See  the

CHAT.BAS program.

If  one  computer wants to quit the program (for instance, his or  her  space

ship  gets blasted to hell-an-gone), it is deemed proper etiquette  for  that

computer  to  send a packet to all of the other computers, so that  they  may

remove that computer from their distribution list.

However,  you may wish to have one of the computers running your  program  to

act  as  a  "server." That is, the first computer to run your  program  would

respond to new computer's broadcasts by sending the new computer ALL  OF  THE

OTHER COMPUTERS' ADDRESSES. All other computers would ignore broadcasts. This

is  how  my  space wars game tells a new computer which other  computers  are

playing  the  game. When that player exits the game, the  job  of  server  is

passed on to another computer still playing the game.

(The   above  description  does  not  describe  a  SAP  (Service  Advertising

Protocol). SAPs are different animals completely. A SAP is much like  an  IPX

packet,  but  it  is  sent every 60 seconds, and it adds its  name  to  every

Bindery on every file server on the network, even over bridges and routers.)

Your  program  should also keep track of unsent packets. If a packet  is  not

sent  within  30  seconds  (give or take), you may assume  that  there  is  a

problem: the destination computer may have exited the program. When a  packet

has  not been delivered, your program should automatically remove it from the

distribution list--- assume the user quit the program.

You  may also want to have a "user time-out." If the program detects that the

user  has done nothing for ten minutes, have her or his computer send a  "I'm

Quitting  This Program" message to all of the other users, so that  they  may

remove  that  computer from their distribution list. Then  have  her  or  his

program quit.

In  the  sample  programs, the Datagram size was set to 546  bytes.  You  may

actually  send from 0 to 546 bytes. I just set it to 546 bytes in the  sample

as the default.

Why  would  someone send a zero-byte Datagram? Suppose in  the  program  flow

above,  Computer  A sends a broadcast on FFFFFFFFFFFF. All this  computer  is

interested  in  is telling the other computers currently using  your  program

that  it  is running your program: the IPX header holds Computer A's address,

and  Computer  A has no need to send any data in the Datagram. Likewise,  the

acknowledgment packets need not send any data in the Datagram,  because  they

are  only telling Computer A their addresses, which is contained in their IPX

packet headers.

If  the packet does not send any data, set the variable ECBS.FragSize  to  30

(which is the size of the header). Likewise, if the size of the data you wish

to send is, for instance, 30 bytes, set ECBS.FragSize to 60 (30 bytes for the

header, and 30 bytes for the Datagram). There is no reason to send a 546 byte

packet when you do not need to.

Packet type is important. There are eight type ranges defined:

0         Unknown Type

1         Routing Information Packet

2         Echo Packet

3         Error Packet

4         Packet Exchange Packet (PXP or PEP)

5         Sequenced Packet Protocol Packet (SPX)

16-31     Experimental

17        NetWare Core Protocol

For IPX, you must use type 0 or 4.

The  sockets you use is also very important. You cannot use just any sockets,

because  there very well may be programs your networks are using that already

use  that socket for their processes. There are several known sockets you  do

NOT want to use. These are (in HEX):

1         Routing information

2         Echo

3         Error

20-3F     Experimental

1-0BB8    Xerox's usage

0BB9--    Dynamically assignable

0451      Novell File Server

0452      Novell Service Advertising Packet

0453      Novell Routing

0455      Novell NetBIOS

0456      Novell Diagnostic

If  you call Novell, they will assign you socket numbers, above &H8000.  Tell

them  how  many  you  need,  and your program's name.  You  do  not  want  to

inadvertently  step on someone else's sockets. You may wish to  ask  for  one

more than you think you'll need, to be on the safe side.

However, there's a much better way to choose a socket. Let IPX do it for you!

If  you  set your socket to zero, IPX will assign a random socket  number  to

your program.

You  may also arbitrarily assign a socket number from within the "dynamically

assignable" range, such as I did in IPXS and IPXR (&H5555). This is  a  range

from &H4000 to &H7FFF.

It  is  a good idea to set up SEVERAL ECBs, one for each process your program

will  perform.  That  is,  one  for  Sending,  one  for  Listening,  one  for

Broadcasts, one for each specific process your program will do.

You  may  build  a packet and send it at a predetermined time  by  using  the

IPXSchedule  call.  You  pass  to this routine  the  number  of  clock  ticks

(approximately  1/18  of a second each tick) to delay. When  the  delay  time

counts  down to zero, the packet is sent, no matter what your program happens

to  be  doing at the time. This can be handy for performing repetitive packet

sends.

You  may  cancel  an event, whether it be a Scheduled, Send,  or  Listen,  by

calling  IPXCancel. If the event has already occurred, you  can't  cancel  it

(obviously).

Want to know how long an IPX event took? Call IPXMarker before and then after

performing a send or listen, then subtract the first result from the  second.

The time will be in 1/18 of seconds, from 0 to 65,535.

You  may use the call GETMYADDRESS to get the computer's network address.  It

will return the Network and Node address, in Binary and Hex.

It is considered proper to send IPX Disconnects to computers you are finished

talking to. This will tell the other computer's network driver to abandon all

services  it  was performing FOR YOU; it will continue to perform  all  other

services.

When  you set an address variable, the routines expect it to be a String,  in

Binary.  For  instance, if the address is A0000F18, use the  FUNCTION  called

HexToBinary$("A0000F18") to convert from Hex to String Binary. To go  in  the

opposite direction, i.e. to convert a String of Binary, use TurnToHex.

The  first  step  in sending or receiving an IPX packet is  to  determine  if

IPX.COM  has  been loaded. If IPX.COM is not loaded, and you try  to  send  a

packet,  the computer will probably lock up. EMM386.EXE will give you  a  GPE

(General Protection Error). If this happens, you must reboot the machine.

Use  the FUNCTION IPXInstalled% to see if IPX.COM is installed. It returns  a

zero if it is =NOT= installed.

Your computer may open many sockets at once. You could have your program send

packets  on  one socket, and receive packets on another socket. If  you  call

OPENSOCKET and receive a Status other than 0 or FF, do not attempt to send  a

packet on that socket--- see the Novell documentation on SHELL.CFG on how  to

increase the default socket count.

After the socket is opened, you may send packets.

After    each   SENDPACKET   call,   your   program   "should"   also    call

RELENQUISHCONTROL. This allows your computer to give up some CPU time to  the

IPX  handler. This is NOT required, however (unless your computer program  is

acting as a server, or multi-tasking). It is just good packet etiquette.

When  you  are finished with a socket, please close it with CLOSESOCKET.  The

routines will close a socket for you when the program terminates, but  it  is

still good practice to close it specifically in the program.

Please note that in the IPXS (send) sample, the packet is sent only once, and

then  a  loop to check the InUseFlag is entered. Do =NOT= place the  call  to

SENDPACKET within the loop.

Also note in the IPXR (receive) sample, the call to SOCKETLISTEN is =NOT=  in

a loop.

Once  your computer program issues a Send or Listen, IPX.COM will handle  the

packet.  IPX.COM  will  modify the state of the  ECB  (Event  Control  Block)

depending  on the status of the packet. All you need do is monitor  the  ECB,

while IPX.COM does its best to deliver the packet, or receive the packet.

If  you  plan on writing a multi-user game, you may wish to look  into  SAPs,

where  one computer playing the game acts as the game's server, and  all  the

other  computers  send  information to this server.  The  server  would  then

distribute  this information to all computers playing the game: this  way,  a

host  game-playing computer need only send and receive IPX packets  from  one

computer (the server), while the server sends to all computers in the game.

Take  the  SAP approach for your game one step further. Suppose your  program

periodically checks each player's computer to determine the fastest  computer

playing. Your program could assign that computer as the server. Then  when  a

faster  computer starts to play your game, the server could  switch.  If  the

server quits, the next fastest computer could act as the server.

Use  the IPXMarker routine to determine which computer is the fastest in  IPX

event handling: the 80486 50Mhz machine might be the fastest machine, but  if

it  is on another network, over a bridge, gateway, or router, it may well  be

the slowest in delivering packets!

There is much more to IPX and SPX use. However, I only went through the  very

basics in this document.

Finally,  any  information in this document could be completely wrong;  also,

the  programs I wrote could also be wrong. I take no responsibility,  and  no

blame,  if  you find that these routines fail to work, wreak havoc with  your

network  or love life, or get you fired from your job because your boss  says

"That idiot programmer crashed the system again!" one too many times.

Good Luck!

David Rice

July 24, 1992