Digital Audio Filter Design and Implementation Utility

by William Mills

Runs under Windows 95 and Windows NT.

INSTALLATION

The program is distributed as a zip file. To install, copy the file

filter.zip to the directory in which you want the program installed.

Then type:

pkunzip -d filter

If you choose the directory c:\filter for installation, everything will

work properly. If you install it in any other directory, you must edit

the file filter.cfg, replacing all ocurrences of "c:\filter" with the

name of the directory where it was installed.

To uninstall the program, delete the directory where it was installed.

Installation creates the following files and directories:

filter.exe - the program

filter.cfg - the configuration file for the program (see OTHER FILES below)

filter.doc - this program description

The gen, cas, and pz subdirectories contain sample highpass, lowpass,

bandpass, and bandreject filters of the three types described below

under FILTER FILES. The spc subdirectory contains sample filter

specification files for the sample filters. The wavin directory contains

a wave file called test.wav. See the file test.doc in that directory for

a description of test.wav. The other three subdirectories contain no

files/ They are named cfg, scr, and wavout. See OTHER FILES below for a

description of the file types which go in these subdirectories.

PROGRAM DESCRIPTION

Supports causal digital audio filters, both general and cascade

2nd-order. Magnitude and phase responses and the specification of the

desired filter are graphically displayed together to facilitate filter

design. For cascade filters, the pole/zero pattern on the Z-plane unit

circle is also displayed. Applies both general and cascade filters to

16-bit mono PCM wave files of any sample rate, one at a time or in

batches. Many of the methods used in the program are adapted from Roman

Kuc, "Introduction to Digital Signal Processing".

Cascade filter design is accomplished by graphic placement of poles and

zeros on the unit circle using a mouse, or by editing an ASCII text file

which specifies the pole/zero pattern. The latter method is useful if

greater precision is desired than can be accomplished using the mouse.

This method allows the design of filters with any magnitude response,

within the limits of the precision of the coefficients (15 significant

digits). A general filter can be designed by first designing a cascade

filter and then saving it as a general filter coefficient file.

Once a filter has been designed, its coefficients can be saved as an

ASCII text file. For cascade filters, the pole/zero pattern can also be

saved to an ASCII text file. General or cascade filters can be imported

by entering the coefficients or pole/zero pattern into an ASCII text

file and loading it into the program. Imported cascade filters can the

be saved as general or cascade pole/zero files. However, the program

can't save an imported general filter as a cascade filter. Another

limitation is that program can't save an imported cascade filter as a

pole/zero or general filter unless the first numerator coefficient is

equal to 1 for all sections. However, such filters can be applied to

wave files.

FILTER FILES

The program uses four types of ASCII filter files. Suggested file

extensions are indicated in parentheses below. All filter files and

other ASCII text files used by the program use whitespace to separate

fields, where whitespace consists of one or more tabs, spaces, and/or

carriage returns.

1. General (.gen) files:

Contains filter coefficients preceded by "numerator" or "denominator".

At least one numerator coefficient must be specified, and the first

numerator coefficient must be 1. If the last field is "denominator" or

there is no "denominator" string in the file, then no denominator

coefficients are specified. The file can contain at most 50 numerator

or denominator coefficients.

2. Cascade pole/zero (.pz) files:

Contains pole or zero radius and angle specifications on the unit

circle. Radii > 1 are not allowed. Angles are specified in degrees. Each

pole or zero specification begins with "pole" or "zero", and is followed

by two numbers which are the radius and angle, in that order. For

imaginary singularities (those not on the horizontal axis), only one

member of the complex conjugate pair should be specified; it is

understood that the other is also to be included in the filter

coefficients. The file can't contain poles and zeros which result in

more than 100 filter sections. Pole/Zero files can be converted to

cascade coefficient files.

3. Cascade coefficient (.cas) files:

Contains filter coefficients for each section. The numerator

coefficients are preceded by "numerator", and the denominator

coefficients by "denominator". Each section must have at least one

numerator coefficient, and the first numerator coefficient must be 1.

No denominator coefficient is indicated by no coefficient after

"denominator". No more than 3 numerator or 2 denominator coefficients

are allowed in a section.

4. Desired filter specification (.spc) files

To aid in filter design, the desired pass and stopbands can be displayed

graphically, with the magnitude and phase responses superimposed.

Passbands are green and stopbands are red. This allows the user to

compare the effect of changes to the pole/zero pattern on the magnitude

and phase response of the filter in comparison to the desired filter

specification.

The filter specification file contains pass and stopband specifications.

The first field must be "lowpass", "highpass", "bandpass", or

"bandreject". The second field is "samplerate", and the third is the

sample rate (no commas in the number). The rest of the specifications

begin with either "stopband" or "passband", followed by two or three

numbers, depending on the filter type. The last number is the

attenuation specification in dB, a negative number, for all filter

types. The first one or two numbers are pass or stopband frequency

specifications in Hz. The stop and passband specifications can be in any

order.

Lowpass and highpass specifications:

For passbands, the first number is the passband frequency. For lowpass

filters, this is upper limit of the frequency range over which there is

to be little attenuation; for highpass filters it is the lower limit.

The second number is the maximum allowable attenuation over the

passband.

For stopbands, the first number is the stopband frequency. For lowpass

filters, this is the lower limit of the frequency range over which there

is to be high attenuation; for highpass filters it is the upper limit.

The second number is the minimum allowable attenuation over the

stopband.

Examples:

lowpass                         highpass

samplerate 11025                samplerate 11025

passband 1378.125 -1            stopband 2756.25 -50

stopband 2756.25 -50            passband 4134.375 -1

Bandpass and bandreject specifications:

Bandpass filters specifications need two stopbands, one lower frequency

and the other higher; bandreject specifications need two passbands.

For bandpass specifications, the first two numbers in the passband

specification are the lower and upper limits, in that order, of the

passband frequency range; for bandreject, the first two numbers in the

stopband specification are the limits of the stopband range.

Examples:

bandpass                        bandreject

samplerate 11025                samplerate 11025

stopband 882 -50                passband 918 -1

passband 2315 3363 -1           stopband 2450 3062 -50

stopband 4795 -50               passband 4288 -1

OTHER FILES

Three other types of files are used by the program.

1. Wave (.w*) files:

Must be 16-bit mono PCM audio files in the wave format. The suggested

file extension is w followed by 0 to 2 characters.

2. Script (.scr) files:

Script files specify which files are to be batch filtered (see below

under MENU CHOICES). These ASCII text files contain file names separated

by whitespace. Only the filename and extension should be included; the

drive and directory are specified elsewhere.

3. Configuration (.cfg) files:

These are ASCII text files containing information about directories and

files to be used by the program. It is suggested that the different

kinds of files used by the program be stored in separate directories.

These directories, as well as specific files, can be specified in a

configuration file. Each directory or file specification consists of two

fields, separated by whitespace. The first is the directory or file

type, and the second is the directory or file name. The following

example shows all of the directory and file types that can be specified.

They can be specified in any order.

Example:

cfg_dir c:\filter\cfg

scr_dir c:\filter\scr

gen_dir c:\filter\gen

cas_dir c:\filter\cas

pz_dir c:\filter\pz

spc_dir c:\filter\spc

wav_in_dir c:\filter\wavin

wav_out_dir c:\filter\wavout

scr_file filelist.scr

gen_file lowpass.gen

cas_file lowpass.cas

pz_file lowpass.pz

spc_file lowpass.spc

wav_in_file test.wav

wav_out_file testout.wav

Separate directories can be specified for input and output wave files.

This is useful when batch filtering. Sometimes it is more convenient to

specify the same directory for input and output wave file. If no

directory is specified for a type of file, the program will read and

write to/from the current directory. If files needed by the program are

not specified, the program will prompt the user to choose them.

When the program starts up, it attempts to read the file filter.cfg in

the directory which contains the program file filter.exe. If the file

exists, it sets the initial configuration. Other configurations can be

stored (in a configuration file directory if desired). The current

configuration can be saved to a file (see MENU CHOICES below).

MAGNITUDE AND PHASE UNIT SAMPLE RESPONSE DISPLAY

Magnitude and phase responses are displayed in the four windows on the

right side of the display. The horizontal (frequency) axis ranges from 0

to the Nyquist frequency in all four windows.

The top three windows display magnitude response. The vertical

(magnitude) axis of the Magnitude window display is on a linear scale,

ranging from 0 to the maximum magnitude response. It gives a good

overall view of the magnitude response. The Passband and Stopband

windows display magnitude response on different dB scales, and are

useful for examining the pass and stopbands more closely. The Passband

window displays the attenuation of the filter from 0 at the top to -4

dB. The Stopband window ranges from 0 to -80 dB. The Phase window

displays phase response, from pi at the top to -pi at the bottom.

Each if the four windows can be made larger by clicking on the middle

icon in its upper right corner. Clicking on the middle icon of an

enlarged window restores it to its original size.

Z-PLANE UNIT CIRCLE DISPLAY

The large square on the left side of the display containing the unit

circle is used to display and edit the poles and zeros of cascade

filters. Poles are represented as blue squares, zeros as yellow circles.

The filter coefficients which correspond to imaginary poles or zeros

(those not on the horizontal axis) reflect the singularities in the

display and also their complex conjugate counterparts in the other half

of the unit circle, although these are not displayed. Editing of a

complex pole or zero also affects its complex conjugate counterpart.

The two concentric circles inside the unit circle have radii of .96 and

.6. Poles and zeros should usually be placed at radii of .6 or greater;

the effect of singularities with smaller radii is likely to be too small

to be useful. Poles should usually be placed at radii of .96 or less so

that roundoff errors will not have a large effect on magnitude response.

When a cascade filter is loaded from a pole/zero or cascade coefficient

file, the poles and zeros appear on the unit circle. Singularities can

be moved, added and deleted. Moving can be accomplished by positioning

the mouse cursor within one pixel of the dot in the center of the

singularity, depressing the left mouse button, dragging the singularity

to the desired position, and then letting up on the left mouse button.

Singularities can be deleted by dragging outside the square which

contains the unit circle. A new singularity can be created by

double-clicking anywhere in the square except within one pixel of the

center of a singularity. A dialog box appears which allows the user to

choose whether a pole or zero is to be created. The mouse cursor becomes

a crosshair. The new singularity is created by double-clicking, and the

cursor returns to its previous form.

Another way to delete or move a singularity is to double-click within on

e pixel of its center. A dialog box appears allowing a choice between

deleting or moving. If moving is chosen, a crosshair cursor appear, and

another double-click will place the singularity in its new position.

MENU CHOICES

File

Allows the user to specify files used by the program.  This can also be

done from a configuration file (see OTHER FILES above). Other choices

are to load or save a configuration, clear the current filter from

memory (Start Over), or exit the program.

Directory:

Allows the user to specify directories used by the program. This cas also

be done from a configuration file (see OTHER FILE above).

Load:

Allows the user to load a filter file into memory. After a filter has

been loaded into memory, its magnitude and phase responses are plotted.

If the current filter is a cascade filter, its pole/zero pattern will

also be displayed.

Save As:

Allows the user to save the current filter as a general, cascade, or

pole/zero filter file. If the current filter is a general filter, the

only possible choice is to save as a general filter.

Apply:

Allows the user to apply a filter (general, cascade, or pole/zero) to

one or more wave files. Either the current filter (the one displayed) or

a filter file can be used. If Batch Apply is chosen, the user is

prompted to choose a script file which specifies the files to be

filtered (see OTHER FILES above). Filters in the input wave file

directory are filtered, and the results are stored as wave files in the

output wave file directory. The Set Gain option allows samples to be

multiplied by a gain factor before conversion to 16-bit integers (see

GAIN below).

About:

Displays this message:

You may freely distribute this software, but only if you don't charge

money for it. If you like it, please send a contribution ($25 suggested).

I will make custom modifications or additions, for a fee. I am available

for consulting work in DSP, speech recognition, and digital audio, and

Pascal and C/C++ programming for Windows, DOS, and Unix.

William Mills

2588 N. Tulip Tree Rd.

Nashville, IN 47448

E-mail: bmills@dcwi.com

FLOATING POINT INACCURACIES

Calculations are done in double floating point precision, and numbers

are saved with 15 significant digits. However, the computer's handling

of real numbers is not perfect. Although these small errors have no

appreciable effect on the filters, a few caveats are in order.

Sometimes the fifteenth digit is not accurate, particularly when many

computations have been done, e.g. converting a cascade filter to a

general filter and back again. Sometimes, values which should be 0.0 are

represented as very small numbers (close to 10 ^ -16).

LOW PIXEL RESOLUTION INACCURACIES

Because the precision with which the user can place singularities on the

unit circle is low, it is possible to accidentally place a singularity

outside the unit circle, which causes an error message.

A more serious error can occur when the user intends to place a

singularity on the real (horizontal) axis, but its actual position is

slightly above or below it. This difference can have a large effect on

the filter; also, an imaginary pole or zero (along with its complex

conjugate counterpart) may produce a filter with more singularities than

are necessary. It is advisable to examine pole/zero files designed by

placing singularities with mouse to see if this has occurred.

If the resolution available using the mouse is insufficient to place a

singularity accurately enough, the pole/zero file can be saved and then

modified using a text editor to fine-tune the position of the

singularity. This is not necessary for most filter designs.

GAIN

Before a filter is applied to wave files, the maximum value of the

magnitude response to the unit sample is computed. A gain factor is then

calculated by dividing this value into 1.0. All samples are multiplied

by this gain factor before the sample values are converted from double

floating point to 16-bit integers.

A second gain factor can be specified under Set Gain in the Apply menu.

Its default value is 1.0. It is applied at the same time as the first

gain factor. Once it has been changed from its default, it will be

applied to all subsequent filtering, so it is important to remember to

reset it if necessary.