SocketWrench/VBÔ Custom Control

User’s Guide and Technical Reference

Version 2.0

Copyright Ó 1995-1996, Catalyst Development Corporation. All rights reserved.

SocketWrench/VB is a trademark of Catalyst Development Corporation.

Microsoft is a registered trademark, and Windows and Visual Basic are trademarks of Microsoft Corporation.

Introduction

With the acceptance of TCP/IP as a standard platform-independant network protocol, and the explosive growth of the Internet, the Windows Sockets API (application program interface) has emerged as the standard for network programming in the Windows environment. This document will introduce  the basic concepts behind Windows Sockets programming and get you started with your first application. If you’re already familiar with sockets programming, feel free to skip this section. It is assumed that the reader is familiar with Visual Basic and has installed the SocketWrench/VB custom control.

The Windows Sockets specification was created by a group of companies, including Microsoft, in an effort to standardize the TCP/IP suite of protocols under Windows. Prior to Windows Sockets, each vendor developed their own proprietary libraries, and although they all had similar functionality, the differences were significant enough to cause problems for the software developers that used them. The biggest limitation was that, upon choosing to develop against a specific vendor’s library, the developer was “locked” into that particular implementation. A program written against one vendor’s product would not work with another’s. Windows Sockets was offered as a solution, leaving developers and their end-users free to choose any vendor’s implementation with the assurance that the product will continue to work.

There are two general approaches that you can take when creating a Visual Basic program that uses Windows Sockets: code directly against the API, or use a custom control (VBX or OCX) which provides an interface to the library by setting properties and responding to events. The first approach, while possible, is not recommended. The Windows Sockets API was designed for use by C programmers, and some of the functions are fairly unpleasant to use in the Visual Basic environment. You’ll end up writing a great deal of code geared towards working around the incompatibilities between the API and Visual Basic, rather than writing the code for your application.

A custom control provides a more “natural” programming interface, and allows you to avoid much of the error-prone drudgery commonly associated with sockets programming. By dropping the control on a form, setting some properties and responding to events, you can quickly and easily write an Internet-enabled application. And because of the nature of custom controls in general, the learning curve is low and experimentation is easy. SocketWrench/VB provides a comprehensive interface to the Windows Sockets library and will be used to build a simple client-server application in the next section of this document. Before we get started with the control, however, we’ll cover the basic terminology and concepts behind sockets programming in general.

Transmission Control Protocol

When two computers wish to exchange information over a network, there are several components that must be in place before the data can actually be sent and received. Of course, the physical hardware must exist, which typically includes network interface cards (NICs) and wiring of some type to connect them. Beyond this physical connection, however, computers also need to use a protocol which defines the parameters of the communication between them. In short, a protocol defines the “rules of the road” that each computer must follow so that all of the systems in the network can exchange data. One of the most popular protocols in use today is TCP/IP, which stands for Transmission Control Protocol/Internet Protocol.

By convention, TCP/IP is used to refer to a suite of protocols, all based on the Internet Protocol (IP). Unlike a single local network, where every system is directly connected to each other, an internet is a collection of networks, combined into a single, virtual network. The Internet Protocol provides the means by which any system on any network can communicate with another as easily as if they were on the same physical network. Each system, commonly referred to as a host, is assigned a unique 32-bit number which can be used to identify it over the internetwork. Typically, this address is broken into four 8-bit numbers separated by periods. This is called dot-notation, and looks something like “192.43.19.64”. Some parts of the address are used to identify the network that the system is connected to, and the remainder identifies the system itself. Without going into the minutia of the Internet addressing scheme, just be aware that there are three “classes” of addresses, referred to as “A”, “B” and “C”. The rule of thumb is that class “A” addresses are assigned to very large networks, class “B” addresses are assigned to medium sized networks, and class “C” addresses are assigned to smaller networks (networks with less than approximately 250 hosts).

When a system sends data over the network using the Internet Protocol, it is sent in discrete units called datagrams, also commonly referred to as packets. A datagram consists of a header followed by application-defined data. The header contains the addressing information which is used to deliver the datagram to it’s destination, much like an envelope is used to address and contain postal mail. And like postal mail, there is no guarantee that a datagram will actually arrive at it’s destination. In fact, datagrams may be lost, duplicated or delivered out of order during their travels over the network. Needless to say, this kind of unreliability can cause a lot of problems for software developers. What’s really needed is a reliable, straight-forward way to exchange data without having to worry about lost packets or jumbled data.

To fill this need, the Transmission Control Protocol (TCP) was developed. Built on top of IP, TCP offers a reliable, full-duplex byte stream which may be read and written to in a fashion similar to reading and writing a file. The advantages to this are obvious: the application programmer doesn’t need to write code to handle dropped or out-of-order datagrams, and instead can focus on the application itself. And because the data is presented as a stream of bytes, existing code can be easily adopted and modified to use TCP.

TCP is known as a connection-oriented protocol. In other words, before two programs can begin to exchange data they must establish a “connection” with each other. This is done with a three-way handshake in which both sides exchange packets and establish the initial packet sequence numbers (the sequence number is important because, as mentioned above, datagrams can arrive out of order; this number is used to ensure that data is received in the order that it was sent). When establishing a connection, one program must assume the role of the client, and the other the server. The client is responsible for initiating the connection, while the server’s responsibility is to wait, listen and respond to incoming connections. Once the connection has been established, both sides may send and receive data until the connection is closed.

User Datagram Protocol

Unlike TCP, the User Datagram Protocol (UDP) does not present data as a stream of bytes, nor does it require that you establish a connection with another program in order to exchange information. Data is exchanged in discrete units called datagrams, which are similar to IP datagrams. In fact, the only features that UDP offers over raw IP datagrams are port numbers and an optional checksum.

UDP is sometimes referred to as an unreliable protocol because when a program sends a UDP datagram over the network, there is no way for it to know that it actually arrived at it’s destination. This means that the sender and receiver must typically implement their own application protocol on top of UDP. Much of the work that TCP does transparently (such as generating checksums, acknowledging the receipt of packets, retransmitting lost packets and so on) must be performed by the application itself.

With the limitations of UDP, you might wonder why it’s used at all. UDP has the advantage over TCP in two critical areas: speed and packet overhead. Because TCP is a reliable protocol, it goes through great lengths to insure that data arrives at it’s destination intact, and as a result it exchanges a fairly high number of packets over the network. UDP doesn’t have this overhead, and is considerably faster than TCP. In those situations where speed is paramount, or the number of packets sent over the network must be kept to a minimum, UDP is the solution.

Hostnames

In order for an application to send and receive data with a remote process, it must have several pieces of information. The first is the IP address of the system that the remote program is running on. Although this address is internally represented by a 32-bit number, it is typically expressed in either dot-notation or by a logical name called a hostname. Like an address in dot-notation, hostnames are divided into several pieces separated by periods, called domains. Domains are hierarchical, with the top-level domains defining the type of organization that network belongs to, with sub-domains further identifying the specific network.

In this figure, the top-level domains are “gov” (government agencies), “com” (commercial organizations), “edu”  (educational institutions) and “net” (Internet service providers). The fully qualified domain name is the specified by naming the host and each parent sub-domain above it, separating them with periods. For example, the fully qualified domain name for the “mars” host would be “mars.olympus.com”. In other words, the system “mars” is part of the “olympus” domain (a company’s local network) which in turn is part of the “com” domain (a domain used by all commercial enterprises).

In order to use a hostname instead of a dot-address to identify a specific system or network, there must be some correlation between the two. This is accomplished by one of two means: a local host table or a name server. A host table is a text file that lists the IP address of a host, followed by the names that it’s known by. Typically this file is named hosts and is found in the same directory in which the TCP/IP software has been installed. A name server, on the other hand, is a system (actually, a program running on a system) which can be presented with a hostname and will return that host’s IP address. This approach is advantageous because the host information for the entire network is maintained in one centralized location, rather than being scattered about on every host on the network.

Service Ports

In addition to the IP address of the remote system, an application also needs to know how to address the specific program that it wishes to communicate with. This is accomplished by specifying a service port, a 16-bit number that uniquely identifies an application running on the system. Instead of numbers, however, service names are usually used instead. Like hostnames, service names are usually matched to port numbers through a local file, commonly called services. This file lists the logical service name, followed by the port number and protocol used by the server.

A number of standard service names are used by Internet-based applications and these are referred to as well-known services. Some common services are:

| Service Name | Function |
| --- | --- |
| echo | Used to echo data back to the program that sent it. This is commonly used to test an application to make sure that a network connection can be established successfully. |
| ftp | Used to transfer files between computer systems using the File Transfer Protocol. |
| telnet | Used to provide terminal emulation services for the remote host. |
| pop3 | Used to retrieve electronic mail messages residing in a “mailbox” on a remote host. |
| smtp | Used to send electronic mail to a remote host using the Simple Mail Transfer Protocol. |
| http | Used to retrieve HTML documents, images, sounds and other resources found on the World Wide Web. |

Remember that a service name or port number is a way to address an application running on a remote host. Because a particular service name is used, it doesn’t guarantee that the service is available, just as dialing a telephone number doesn’t guarantee that there is someone at home to answer the call.

Sockets

The previous sections described what information a program needs to communicate over a TCP/IP network. The next step is for the program to create what is called a socket, a communications end-point that can be likened to a telephone. However, creating a socket by itself doesn’t let you exchange information, just like having a telephone in your house doesn’t mean that you can talk to someone by simply taking it off the hook. You need to establish a connection with the other program, just as you need to dial a telephone number, and to do this you need the socket address of application that you want to connect to. This address consists of three key parts: the protocol family, Internet Protocol (IP) address and the service port number.

We’ve already talked about the IP address and service port, but what’s the protocol family? It’s a number which is used to logically designate the group that a given protocol belongs to. Since the socket interface is general enough to be used with several different protocols, the protocol family tells the underlying network software which protocol is being used by the socket. In our case, the Internet Protocol family will always be used when creating sockets. With the protocol family, IP address of the system and the service port number for the program that you want to exchange data with, you’re ready to establish a connection.

Client-Server Applications

Programs written to use TCP are developed using the client-server model. As mentioned previously, when two programs wish to use TCP to exchange data, one of the programs must assume the role of the client, while the other must assume the role of the server. The client application initiates what is called an active open. It creates a socket and actively attempts to connect to a server program. On the other hand, the server application creates a socket and passively listens for incoming connections from clients, performing what is called a passive open. When the client initiates a connection, the server is notified that some process is attempting to connect with it. By accepting the connection, the server completes what is called a virtual circuit, a logical communications pathway between the two programs. It’s important to note that the act of accepting a connection creates a new socket; the original socket remains unchanged so that it can continue to be used to listen for additional connections. When the server no longer wishes to listen for connections, it closes the original passive socket.

To review, there are five significant steps that a program which uses TCP must take to establish and complete a connection.  The server side would follow these steps:

- Create a socket.

- Listen for incoming connections from clients.

- Accept the client connection.

- Send and receive information.

- Close the socket when finished, terminating the conversation.

In the case of the client, these steps are followed:

- Create a socket.

- Specify the address and service port of the server program.

- Establish the connection with the server.

- Send and receive information.

- Close the socket when finished, terminating the conversation.

Only steps two and three are different, depending on if it’s a client or server application.

Blocking vs. Non-Blocking Sockets

One of the first issues that you’ll encounter when developing your Windows Sockets applications is the difference between blocking and non-blocking sockets. Whenever you perform some operation on a socket, it may not be able to complete immediately and return control back to your program. For example, a read on a socket cannot complete until some data has been sent by the remote host. If there is no data waiting to be read, one of two things can happen: the function can wait until some data has been written on the socket, or it can return immediately with an error that indicates that there is no data to be read.

The first case is called a blocking socket. In other words, the program is “blocked” until the request for data has been satisfied. When the remote system does write some data on the socket, the read operation will complete and execution of the program will resume. The second case is called a non-blocking socket, and requires that the application handle the situation appropriately. Programs that use non-blocking sockets typically use one of two methods when sending and receiving data. The first method, called polling, is when the program periodically attempts to read or write data from the socket using a timer. The second, and preferred method, is to use what is called asynchronous notification. This means that the program is notified whenever a socket event takes place, and in turn can respond to that event. For example, if the remote program writes some data to the socket, a “read event” is generated so that program knows it can read the data from the socket at that point.

For historical reasons, the default behavior is for socket functions to “block” and not return until the operation has completed. Under Windows this can introduce some special problems because when the program blocks, it enters what is called a “message loop”, where it continues to process messages sent to it by Windows and other applications. Since messages are being processed, this means that the program can be re-entered at a different point, with the blocked operation “parked” on the program’s stack. For example, consider a program that attempts to read some data from the socket when a button is pressed. Because no data has been written yet, it blocks and the program goes into a message loop. The user then presses a different button, which causes code to be executed, which in turn attempts to read data from the socket, and so on. Obviously, this can pose a big problem. To resolve it, the Windows Sockets standard states that there may only be one outstanding blocked call per thread of execution. This means that, in the example given above, the second read would fail with an error indicating that a blocking operation is already in progress. While blocking sockets may be convenient in some situations, their use is generally discouraged because of the complications that they can introduce into the program.

The SocketWrench control facilitates the use of non-blocking sockets by firing events when appropriate. For example, a Read event is generated whenever the remote host writes on the socket, which tells your application that there is data waiting to be read. The use of non-blocking sockets will be demonstrated in the next section, and is one of the key areas in which a control has a distinct advantage over coding directly against the Windows Sockets API.

In summary, there are three general approaches that can be taken when building an application with the control  in regard to blocking or non-blocking sockets:

- Use a blocking (synchronous) socket. In this mode, the program will not resume execution until the socket operation has completed. This method is only recommended for relatively small applications. Because blocking sockets must allow messages to be processed in Windows, it is possible that your blocked application can be re-entered at a different point. This approach can lead to complex interactions (and difficult debugging) if there are multiple active controls in use by the application.

- Use a non-blocking (asynchronous) socket, which allows your application to respond to events. For example, when the remote system writes data to the socket, a Read event is generated for the control. Your application can respond by reading the data from the socket, and perhaps send some data back, depending on the context of the data received.

- Use a combination of blocking and non-blocking socket operations. The ability to switch between blocking and non-blocking modes “on the fly” provides a powerful and convenient way to perform socket operations. Note that the warning regarding blocking sockets also applies here.

If you decide to implement your application using a blocking socket connection, it may call a blocking hook function that will allow your application to be re-entered. The Windows Sockets API will fail most socket operations while a blocking socket call is in progress, so your application must be prepared to handle this. The State property can be used to determine the state of a blocking socket.

Programming With SocketWrench/VB

Because SocketWrench has a large number of properties, you might feel overwhelmed when you start reading through the technical reference material. Don’t worry -- you only need to understand how to use a handful of properties and events to get started. Once you’ve become more comfortable and knowledgeable about sockets programming, you’ll appreciate the power and flexibility that SocketWrench gives you.

Each control that you use corresponds to one socket, which may or may not be connected to a remote host. If you need access to multiple sockets, you must use multiple controls, typically as a control array. This is most commonly needed when your application acts a server and must be able to handle several connections at one time.

A Sample Client Program

The sample program that will be used throughout this document is a simple tool that can be used to connect with an echo server, a program which echoes back any data that’s sent to it. Later on, we’ll also cover how to implement your own echo server.

The first step, after starting Visual Basic, is to include the SocketWrench control in your new project. In Visual Basic 3.0, select the File|Add File option from the menu and a file selection dialog will be displayed. Enter the complete pathname of the control, such as c:\windows\system\cswskctl.vbx. In Visual Basic 4.0, select Tools|Custom Controls. To add the control to the list of available controls, click on the Browse button and enter the complete pathname of the control. Note that when using the ActiveX controls, they will already be listed, so you will only need to check the control (or controls) that you’re interested in.

After the control has been added to the tool palette, you will also need to include the CSTOOLS.BAS file that was copied to the directory that you specified during installation. Select the File|Add File menu option and enter the full pathname of the file (this procedure is the same for both versions 3.0 and 4.0 of Visual Basic).

To begin, you’ll need to start Visual Basic and create a form that has three labels, three text controls, a button and the SocketWrench control. The form might look something like this:

When executed, the user will enter the name or IP address of the system in the Text1 control, the text that is to be echoed in the Text2 control, and the server’s reply will be displayed in the Text3 control. The Command1 button will be used to establish a connection with the remote server. The Text2 and Text3 controls should be created with their Enabled properties initially set to False.

Some essential properties of the SocketWrench control, called Socket1, needs to be initialized.. The best place to do this is in the form’s Load subroutine. The code should look like this:

Sub Form_Load ()

Socket1.AddressFamily = AF_INET

Socket1.Protocol = IPPROTO_IP

Socket1.SocketType = SOCK_STREAM

Socket1.Binary = False

Socket1.BufferSize = 1024

Socket1.Blocking = False

End Sub

These six properties should be set for every instance of the SocketWrench control:

| AddressFamily | This property is part of the socket address, and should always be set to a value of AF_INET, which is global constant with the integer value of 2. At this time, the Windows Sockets interface only supports the IP address family. |
| --- | --- |
| Protocol | This property determines which protocol is going to be used to communicate with the remote application. Most commonly, the value IPPROTO_IP is used, which means that the protocol appropriate for the socket type will be used. |
| SocketType | This property specifies the type of socket that is to be created. It may be either of type SOCK_STREAM or SOCK_DGRAM. The stream-based socket uses the TCP protocol, and data is read and written on the socket as a stream of bytes, similar to how data in a file is accessed. The datagram-based socket uses the UDP protocol, and data is read and written in discrete units called datagrams. Most sockets that you will create will be of the stream variety. |
| Binary | This property determines how data should be read from the socket. If set to a value of True, then the data is received unmodified. If set to False, the data is interpreted as text, with the carriage return and linefeed characters stripped from the data stream. Each receive returns exactly one line of text. |
| BufferSize | This property is used only for stream-based (TCP) sockets. It specifies the amount of memory, in bytes, that should be allocated for the socket’s send and receive buffers. |
| Blocking | This property specifies if the application should wait for a socket operation to complete before continuing. By setting this property to False, that indicates that the application will not wait for the operation to complete, and instead will respond to events generated by the control. This is the recommended approach to take when designing your application. |

The next step is to establish a connection with the echo server. This is done by including code in the Command1 button’s Click event. The code should look like this:

Sub Command1_Click ()

Socket1.HostName = Trim$(Text1.Text)

Socket1.RemotePort = IPPORT_ECHO

Socket1.Action = SOCKET_CONNECT

End Sub

The Text1 edit control should contain the name or IP address of a system that has an echo server running (most UNIX and Windows NT based systems do have such a server). The properties which have been used to establish the connection are:

| HostName | This property should be set to either the host name of the system that you want to connect to, or it’s IP address in dot-notation. |
| --- | --- |
| RemotePort | This property should be set to the number of the port which the remote application is listening on. Port numbers below 1024 are considered reserved by the system. In this example, the echo server port number is 7, which is specified by using the global constant IPPORT_ECHO. |
| Action | This property initiates some action on the socket. The SOCKET_CONNECT action tells the control to establish a connection using the appropriate properties that have been set. A related action, SOCKET_CLOSE, instructs the control to close the connection, terminating the conversation with the remote server. |

Both HostName and RemotePort are known as reciprocal properties. This means that by changing the property, the another related property will also change to match it. For example, when you assign a value to the HostName property, the control will determine it’s IP address and automatically set the HostAddress property to the correct value. The reciprocal property for RemotePort is the RemoteService property. For more information about these properties, refer to the Technical Reference section.

Because the socket is non-blocking (i.e.: the Blocking property has been set to a value of False), the program will not wait for the connection to be established. Instead, it will return immediately and respond to the Connect event in the SocketWrench control. The code for that event should look like this:

Sub Socket1_Connect ()

Text2.Enabled = True

Text3.Enabled = True

End Sub

This tells the application that when a connection has been established, enable the edit controls so that the user can send and receive information from the server.

Now that the code to establish the connection has been included, the next step is to actually send and receive data to and from the server. To do this, the Text2 control should have the following code added to it’s KeyPress event:

Sub Text2_KeyPress (KeyAscii As Integer)

If KeyAscii = 13 Then

Socket1.SendLen = Len(Text2.Text)

Socket1.SendData = Text2.Text

KeyAscii = 0: Text2.Text = ""

End If

End Sub

If the user presses the Enter key in the Text2 control, then that text is sent down to the echo server. The properties used to send data are as follows:

| SendLen | This property specifies the length of the data being sent to the server. It should always be set before the data is written to the socket. After the data has been sent, the value of the property is adjusted to indicate the actual number of bytes that have been written. |
| --- | --- |
| SendData | Setting this property causes the data assigned to it to be written to the socket. The number of bytes actually written may be less than the amount specified in the SendLen property if the socket buffers become full. |

Once the data has been sent to the server, it immediately sends the data back to the client. This generates a Read event in SocketWrench, which should have the following code:

Sub Socket1_Read (DataLength As Integer, IsUrgent As Integer)

Socket1.RecvLen = DataLength

Text3.Text = Socket1.RecvData

End Sub

The properties used to receive the data are as follows:

| RecvLen | This property specifies the maximum number of bytes that should be read from the socket. After the data has been received, the value is changed to reflect the number of bytes actually read. |
| --- | --- |
| RecvData | Reading this property causes data to be read from the socket, up to the maximum number of bytes specified by the RecvLen property. If the socket is non-blocking and there is no data to be read, an error is generated. |

The Read event is passed two parameters, the number of bytes that are available to be read, and a flag that specifies if the data is urgent (also known as “out-of-band” data, the use of urgent data is an advanced topic outside of the scope of this document). For more information about the Read event, please refer to the Technical Reference section.

The last piece of code to add to the sample is to handle closing the socket when the program is terminated by selecting Close on the system menu. The best place to put socket cleanup code in the form’s Unload event, such as:

Sub Form_Unload (Cancel As Integer)

If Socket1.Connected Then Socket1.Action = SOCKET_CLOSE

End

End Sub

This should be rather self-explanatory. The only new property that has been introduced is the Connected property, which is a Boolean flag. If it is True, then a connection has been established. With all of the properties and event code needed for the sample client application completed, all that’s left to do is run the program!

The first thing that you’ll probably notice is that if you specify an incorrect host name or address, an error will be generated and the program will stop. Of course, in a real application you’d need to provide extensive error checking. SocketWrench errors start at 24,000 and correspond to the error codes used by the Windows Sockets library. Most errors will occur when setting the host name, address, service port or Action property.

Building An Echo Server

The next step is to implement your own echo server. To accomplish this, we’ll modify the client application to function as a server as well. The side benefit is that this will allow you to test both the client and server application on your local system.

Remember that the first thing that a server application must do is listen on a local port for incoming connections. You know that an application is attempting to connect with you when the Accept event is generated for the SocketWrench control. There are two methods which you can use to accept an incoming connection: set the Action property to the value SOCKET_ACCEPT, or set the Accept property.

Setting the Action property is the simplest of the two methods. As you’ll recall, the act of accepting a connection causes a second socket to be created. The original listening socket continues to listen for more connections, while the second socket can be used to communicate with the client that connected to you. When you set the Action property to SOCKET_ACCEPT, what you’re telling the control to do is to close the original listening socket, and from that point on, the control can be used to communicate with the client. While this is convenient, it is also limiting -- since the listening socket has been closed, no more clients can connect with your program, effectively limiting it to a single client connection.

The more flexible approach is to set the Accept property to the value passed as an argument to the Accept event. However, this cannot be done by the control that is listening for connections because it is in use. You have to use another, unused control to accept the connection. The problem is, how many clients are going to attempt to connect to you? Of course, you could drop a fixed number of SocketWrench controls on your form, thereby limiting the number of connections, but that’s not a very good design. The better approach is to create a control array which can be dynamically loaded when a connection is attempted by a client, and unloaded when the connection is closed. This is the approach that we’ll take in our server code sample.

The first thing to do is to add a second SocketWrench control to your form, and make it a control array. Initially there will only be one control in the array, identified as Socket2(0). This control will be responsible for listening for client connections. Just as with the client socket control, several of the control’s properties should be initialized in the form’s Load subroutine. The new subroutine should look like this:

Sub Form_Load ()

Socket1.AddressFamily = AF_INET

Socket1.Protocol = IPPROTO_IP

Socket1.SocketType = SOCK_STREAM

Socket1.Binary = False

Socket1.BufferSize = 1024

Socket1.Blocking = False

Socket2(0).AddressFamily = AF_INET

Socket2(0).Protocol = IPPROTO_IP

Socket2(0).SocketType = SOCK_STREAM

Socket2(0).Blocking = False

Socket2(0).LocalPort = IPPORT_ECHO

Socket2(0).Action = SOCKET_LISTEN

LastSocket = 0

End Sub

The only thing that is new here is the LocalPort property and the LastSocket variable. The LocalPort property is used by server applications to specify the local port that it’s listening on for connections. By specifying the standard port used by echo servers, any other system can connect to yours and expect the program to echo back whatever is sent to it.

The LastSocket variable is defined in the “general” section of the Visual Basic application as an integer. It is used to keep track of the next index value that can be used in the control array.

By setting the Action property to SOCKET_LISTEN in the form’s Load event, the program will start listening for connections as soon as the program starts executing. When a client tries to connect with your server, Socket2’s Accept event will fire. The code for this event should look like this:

Sub Socket2_Accept (Index As Integer, SocketId As Integer)

Dim I As Integer

For I = 1 To LastSocket

If Not Socket2(I).Connected Then Exit For

Next I

If I > LastSocket Then

LastSocket = LastSocket + 1: I = LastSocket

Load Socket2(I)

End If

Socket2(I).AddressFamily = AF_INET

Socket2(I).Protocol = IPPROTO_IP

Socket2(I).SocketType = SOCK_STREAM

Socket2(I).Binary = True

Socket2(I).BufferSize = 1024

Socket2(I).Blocking = False

Socket2(I).Accept = SocketId

End Sub

The first statement loads a new instance of the SocketWrench control as part of the Socket2 control array. The next six lines initialize the control’s properties, and then the Accept property is set to the value of the SocketId parameter that is passed to the control. After executing this statement, the control is now ready to start communicating with the client program.

Since it’s the job of an echo server to echo back whatever is sent to it, we have to add code to the control’s Read event, which tells it that the client has sent some data to us. It looks like this:

Sub Socket2_Read (Index As Integer, DataLength As Integer,

IsUrgent As Integer)

Socket2(Index).RecvLen = DataLength

Socket2(Index).SendLen = DataLength

Socket2(Index).SendData = Socket2(Index).RecvData

End Sub

Finally, when the client closes the connection, the socket control must also close it’s end of the connection. This is accomplished by adding a line of code in the socket’s Disconnect event:

Sub Socket2_Disconnect (Index As Integer)

Socket2(Index).Action = SOCKET_CLOSE

End Sub

To make sure that all of the socket connections are closed when the application is terminated, the following code should be included in the form’s Unload event:

Sub Form_Unload (Cancel As Integer)

Dim I As Integer

If Socket1.Connected Then Socket1.Action = SOCKET_CLOSE

If Socket2(0).Listening Then Socket2(0).Action = SOCKET_CLOSE

For I = 1 To LastSocket

If Socket2(I).Connected Then Socket2(I).Action = SOCKET_CLOSE

Next I

End

End Sub

The only new property shown here is the Listening property, which like the Connected property, is a Boolean flag. If the control is listening for incoming connections, this property will return True, otherwise it returns False. This is added only as an extra “sanity check”, and the property should always return True for this instance of the control.

SocketWrench Technical Reference

The SocketWrench/VB custom control provides network communication services for your Visual Basic application using a third-party TCP/IP protocol stack and compatible Windows Sockets library. The control offers the following features:

- Client and server functionality in a single control

- Support for blocking and non-blocking socket operations

- Convenient “text” mode for receiving data a line at a time

- Support for both stream and datagram protocols

- Send and receive urgent (out-of-band) data

Instead of using API calls, virtually all socket functions can be performed by setting control properties and responding to events. For those developers who are not familiar with the details of socket programming, SocketWrench can also insulate them from many of the common pitfalls, without sacrificing functionality or flexibility.

Each control that you use corresponds to one socket (which may or may not be connected to a remote host). If you need access to multiple sockets, you must use multiple controls, typically as a control array. This is most commonly needed when your application acts a server and must be able to handle several connections at one time.

Buffering Socket Data

SocketWrench may be configured to buffer data transfers between your application and the remote host to minimize the number of calls made to the Windows Sockets library. Buffering can only be enabled for stream sockets by setting the BufferSize property before the socket is created. Both blocking and non-blocking sockets can be buffered.

Binary vs. Text Mode

Buffered sockets can receive data in either binary or text mode, depending on the value of the Binary property. By default, all data read from the socket is returned to your application exactly as it was sent by the remote host. In some cases, however, it would be more convenient to receive data as lines of text, similar to reading a text file. By setting the Binary property to False, each read on the socket is terminated when a linefeed is encountered in the data stream. Keep in mind that this only affects how data is received by your application and has no effect on data being written to the socket.

Server Implementation

In addition to using SocketWrench to connect to remote hosts as a client, you can use the same control to create a server application. To do this, you create a listening socket which waits for connections from clients. When a client connects to your socket, you are notified via the Accept event (this event is only generated for non-blocking sockets). At this point, you may either accept the connection or close the listening socket. When accepting a connection, you have two options:

- Use the SOCKET_ACCEPT action on the listening socket to accept the connection. Since this action closes the listening socket, any other clients attempting to connect to your server will be rejected.

- Set the Accept property of an unused socket to the handle of the listening socket. This will allow the unused socket to accept the connection on behalf of the listening socket, which will continue to listen for additional connections.

Of the two approaches, the first is simpler and suitable for small, special purpose servers that do not need to handle multiple client connections. The second approach is more powerful, but also more complex. To handle a variable number of client connections, the recommended approach is to create a control array of sockets that can be loaded and unloaded as needed.

Commonly Used Properties

Although SocketWrench has a large number of properties, only a subset of them will be used with any frequency in your applications. Here are the properties that you should become familiar with first:

| Property | Description |
| --- | --- |
| Action | Determines the action to be taken on the socket. The various actions that can be taken on the socket are defined in the CSTOOLS.BAS file as global constants. |
| Binary | Determines how data is read from the socket. If set to False, then data is returned as text terminated by newlines. The socket must be buffered and connected using a streams-based protocol (i.e.: TCP) |
| Blocking | Set or return the blocking mode for the socket. By default, sockets are blocking, and no socket events will be generated by the control. |
| HostAddress | Specifies the IP address of the remote host that the socket will be sending data to or receiving data from. Setting this property will automatically update the HostName property. |
| HostName | Specifies the name of the remote host. Setting this property will automatically update the HostAddress property. |
| Protocol | Specifies the protocol to use for this socket. A value of 0 indicates that the appropriate protocol for the socket type selected is used. |
| RecvData | A read-only property that returns data to be read from the socket. This property uses the RecvLen property to determine the maximum number of bytes to read. |
| RecvLen | Specifies the number of bytes to read from the socket. After reading the RecvData property, this property is changed to reflect the number of bytes actually read. |
| RemotePort | Specifies the port number that a remote server is listening. Setting this value causes the RemoteServer property to be updated. |
| RemoteService | Specifies the service name of the remote port a server is listening to. Setting this value causes the RemotePort property to be updated. |
| SendData | A write-only property that is used to write data to the socket. The SendLen property specifies the maximum number of bytes that may be written. |
| SendLen | Specifies the maximum number of bytes that may be written to the socket. After sending the data, this property is changed to reflect the number of bytes actually written. |
| SocketType | Specifies the type of socket to be created. The type will commonly be either SOCK_STREAM or SOCK_DGRAM. These values are defined as global constants in the CSTOOLS.BAS file. |

System Requirements

The sockets control requires Microsoft Windows 3.1 or later, Visual Basic 2.0 or later and a TCP/IP product that supports the Windows Sockets 1.1 specification. The WINSOCK.DLL library must be located in the Windows directory or in a directory that is in the search path. Microsoft distributes a free TCP/IP stack for Windows for Workgroups 3.11, and one is included with the base operating system for Windows 95 and Windows NT.

When you distribute your application that uses SocketWrench, you should install the CSWSKCTL.VBX file in the Windows system directory. Note that it is not required that the end-user of your application use the same TCP/IP product that you’ve used for development.

Accept Property

Description	A write-only property, that when set to the value of a listening socket’s handle, accepts the connection. The listening socket is not closed and may continue to listen for new client connections.

Visual Basic	[form.]Socket.Accept = handle%

Remarks	Setting this property to a socket that is not listening for connections will generate an error. If the Accept property is set to the value of the current socket’s handle, the effect is the same as if the SOCKET_ACCEPT action has been taken.

Data Type	Integer

See Also	Action Property, Backlog Property, Listening Property, Accept Event

Action Property

Description	Setting this property causes the socket to take some action, such as creating a socket, connecting to a remote system or closing a connection.

Visual Basic	[form.]Socket.Action = action%

Remarks	The following table lists the actions that a socket control may take:

| Value | Constant | Description |
| --- | --- | --- |
| 1 | SOCKET_OPEN | Create a socket using the AddressFamily, Protocol and SocketType properties. This action is commonly used to create datagram sockets, or if supported by the TCP vendor, raw sockets. |
| 2 | SOCKET_CONNECT | Connect to a remote system specified by the HostAddress or HostName properties. If a socket has not already been created, this action will create it. |
| 3 | SOCKET_LISTEN | Listen on a socket for incoming connections on the port specified by the LocalPort or LocalService properties. If a socket has not already been created, this action will create it. |
| 4 | SOCKET_ACCEPT | Accept an incomming connection on the socket. The listening socket connection is closed and the client is connected. To resume listening for new connections, the Action must be set to SOCKET_LISTEN again. This approach allows only one incomming connection per listening socket. |
| 5 | SOCKET_CANCEL | Cancel the current blocking operation. This action only has meaning for blocking sockets. |
| 6 | SOCKET_FLUSH | Flush the contents of the send and receive socket buffers. |
| 7 | SOCKET_CLOSE | Close the socket, and if connected, break the connection with the remote system. This action should be taken before the control is unloaded from the form. |

Action Property

| 8 | SOCKET_ABORT | Immediately close the socket without waiting for remaining data to be written out. This action should only be taken under extreme circumstances. |
| --- | --- | --- |
| 9 | SOCKET_STARTUP | Explicitly load and initialize the Windows Sockets library. Typically this action is not required because this is done automatically when any control function is taken which requires network access (such as attempting a connection to a remote host). |
| 10 | SOCKET_CLEANUP | Explicitly unload the Windows Sockets library. Typically this action is not required because this is done when the control is unloaded. |

Data Type	Integer

See Also	Connected Property, Listening Property, State Property

AddressFamily Property

Description	Sets or returns the address family for the socket.

Visual Basic	[form.]Socket.AddressFamily [= aftype%]

Remarks	The address family value should be set to the constant AF_INET. This property is included only for completeness and future expansion.

Data Type	Integer

See Also	Protocol Property, SocketType Property

AutoResolve Property

Description	Determines if host names and addresses are automatically resolved..

Visual Basic	[form.]Socket.AutoResolve [= { True | False }]

Remarks	Setting the AutoResolve property determines if the control automatically resolves host names and addresses specified by the HostName and HostAddress properties. If set to True, setting the HostName property will cause the control to automatically determine the corresponding IP address and set the HostAddress property accordingly. Likewise, setting the HostAddress property will cause the control to determine the host name and set the HostName property. Setting the property to False prevents the control from resolving host names until a connection attempt is made.

Note When using the domain name service (DNS), setting the HostName or HostAddress property may cause the application to block, sometimes for several seconds, until the name or address is resolved. To prevent this behavior, set AutoResolve to False.

AutoResolve Property

Data Type	Integer (Boolean)

See Also	HostAddress Property, HostName Property

AtMark Property

Description	A read-only property that returns True if the next receive will return urgent data.

Visual Basic	[form.]Socket.AtMark

Remarks	This property can only be used if the socket type is SOCK_STREAM and the InLine property has been set to True. Reading this property is the same as using the SIOCATMARK option with the ioctlsocket function.

Data Type	Integer (Boolean)

See Also	RecvData Property, Urgent Property, Read Event

Backlog Property

Description	Sets or returns the number of connections that may be accepted by a listening socket.

Visual Basic	[form.]Socket.Backlog [= backlog%]

Remarks	This property must be set to the desired value before the SOCKET_LISTEN action is taken. If the value is less than one or greater than five, it’s value is silently changed to nearest legal value.

Data Type	Integer

See Also	Accept Property, Listening Property, Accept Event

Binary Property

Description	Sets or returns the binary mode flag for the current socket. If set to a value of False, then subsequent reads on the socket return lines of text terminated by newlines.

Visual Basic	[form.]Socket.Binary [= { True | False }]

Remarks	This property may only be set for buffered sockets of type SOCK_STREAM, and may not be changed after a connection has been established with a remote system.

Data Type	Integer (Boolean)

See Also	BufferSize Property, RecvData Property

Blocking Property

Description	Sets or returns the blocking state of the socket.

Visual Basic	[form.]Socket.Blocking [= { True | False }]

Remarks	Setting this property determines if socket operations complete synchronously or asynchronously. If set to True, then each socket operation (such as sending or receiving data) will return when the operation has completed or timed-out. If set to False, socket operations return immediately. If the operation would result in the socket blocking (such as attempting to receive data when none has been written), the error WSAEWOULDBLOCK is generated. Socket events such as Accept, Disconnect, Read and Write are only fired if the socket is non-blocking.

If the socket is made blocking, the Blocking event is fired before the blocking operation starts, and it is possible to cancel the operation at that point.

Blocking Property

Data Type	Integer (Boolean)

See Also	IsBlocked Property, Blocking Event

Broadcast Property

Description	Determines if datagrams should be broadcast over the network

Visual Basic	[form.]Socket.Broadcast [= { True | False }]

Remarks	If set to a value of True, the datagram written to the socket will be broadcast to all systems on the network. Use of this property is restricted to SOCK_DGRAM socket types.

Data Type	Integer (Boolean)

See Also	InLine Property, KeepAlive Property, ReuseAddress Property, Route Property

BufferSize Property

Description	Set the send and receive buffer sizes for the socket

Visual Basic	[form.]Socket.BufferSize [= bufsize%]

Remarks	This property sets the size of the send and receive buffers for SOCK_STREAM socket types. A buffer size of 0 indicates that no buffering should be done. Buffering reads and writes on the socket is recommended if the socket is non-blocking, and required if the Binary property has been set to a value of False.

Data Type	Integer

See Also	Binary Property, RecvData Property, Read Event

Connected Property

Description	Return if the socket is connected to a remote host

Visual Basic	[form.]Socket.Connected

Remarks	This read-only property is set to a value of True if the socket was connected with the SOCKET_CONNECT action, or if a connection was accepted on a listening socket.

Data Type	Integer (Boolean)

See Also	Listening Property

GetFirstHost Property

Description	Return the name of the first host in the host table

Visual Basic	[form.]Socket.GetFirstHost

Remarks	Reading this property returns the name of the first host in the specified host file. If there is no host file, or the file is empty, this property will return an empty string.

Data Type	String

See Also	GetNextHost Property, HostFile Property

GetNextHost Property

Description	Return the name of the next host in the host table

Visual Basic	[form.]Socket.GetNextHost

Remarks	Reading this property returns the name of the next host in the specified host file. If the last host entry has been read, this property will return an empty string.

Data Type	String

See Also	GetFirstHost Property, HostFile Property

Handle Property

Description	Returns the handle (descriptor) for the current socket

Visual Basic	[form.]Socket.Handle

Remarks	This read-only property returns the handle to the current socket. If the socket has not been opened, a value of -1 is returned. This property can be used in conjunction with direct calls to the Windows Sockets API.

Data Type	Integer

HostAddress Property

Description	Set or return the IP address of the remote host

Visual Basic	[form.]Socket.HostAddress [= addr$]

Remarks	This property can be used to set the IP address for a remote system that you wish to communicate with. If the address is valid and matches an entry in the host table, the HostName property will be changed to match the address.

Data Type	String

See Also	HostName Property, LocalAddress Property, PeerAddress Property

HostFile Property

Description	Sets or returns the name of the host file

Visual Basic	[form.]Socket.HostFile [= hostfile$]

Remarks	This property should be set to the name of the host file used by the Windows Sockets library. If no directory is provided as part of the file name, the directories listed in the PATH environment variable are searched. If the host file is located, each host entry is read into memory and returned by the GetFirstHost and GetNextHost functions.

Note that the host file must be a standard UNIX-style text file. Blank lines, or any characters that follow a hash-mark (#) are ignored.

Data Type	String

See Also	GetFirstHost Property, GetNextHost Property

HostName Property

Description	Set or return the name of the remote host

Visual Basic	[form.]Socket.HostName [= hostname$]

Remarks	This property should be set to the name of the remote system that you wish to communicate with. If the name is found in the host table, the HostAddress property is updated to reflect the IP address of the host.

Note that it is legal to assign an IP address to this property, but it is not legal to assign a host name to the HostAddress property.

Data Type	String

See Also	HostAddress Property, LocalName Property, PeerName Property

InLine Property

Description	Sets or returns if urgent data is received in-line with non-urgent data

Visual Basic	[form.]Socket.InLine [= { True | False }]

Remarks	This property controls how urgent (out-of-band) data is handled when reading data from the socket. If set to a value of True, urgent data is placed in the data stream along with non-urgent data. To determine if the data that is being read is urgent, the AtMark property can be read.

If you expect that your application must handle urgent data that is not sent in-line, it is not recommended that you use text mode or buffer the socket. Because urgent data is sent outside of the normal data stream, all receives are unbuffered.

Data Type	Integer (Boolean)

See Also	Urgent Property, Read Event

Interval Property

Description	Set or return the number of milliseconds between calls to the control's Timer event

Visual Basic	[form.]Socket.Interval [= milliseconds]

Remarks	This property specifies the number of milliseconds between calls to the Timer event. A value of zero indicates that the timer is disabled and no events will be generated. The maximum interval value is 65536 milliseconds, which is slightly more than one minute.

There are a limited number of timers available (16 in the Windows environment). Setting the interval when no timers are available will generate a runtime error.

Data Type	Long

See Also	Timer Event

IsBlocked Property

Description	Return if the current socket is blocked performing an operation

Visual Basic	[form.]Socket.IsBlocked

Remarks	This property returns True if the current socket is blocked performing an operation. However, since the Windows Sockets API does not allow functions to be re-entered during a blocking operation for a given task, this property could return False and a socket operation may still fail with the WSAEWOULDBLOCK error if multiple sockets have been created by the application.

Data Type	Integer (Boolean)

See Also	Blocking Property, Blocking Event

IsClosed Property

Description	Return if the socket has been closed by the remote host

Visual Basic	[form.]Socket.IsClosed

Remarks	This property returns True if the socket connection has been closed by the remote host. Note that it is possible to continue to receive data on a closed socket if the socket was buffered.

Data Type	Integer (Boolean)

See Also	IsReadable Property, IsWritable Property

IsReadable Property

Description	Return if data can be read from the socket without blocking

Visual Basic	[form.]Socket.IsReadable

Remarks	This property returns True if data can be read from the socket without blocking. For non-blocking sockets, this property can be checked before the application attempts to read the socket, preventing a WSAEWOULDBLOCK error.

Data Type	Integer (Boolean)

See Also	IsClosed Property, IsWritable Property, Read Event

IsWritable Property

Description	Return if data can be written to the socket without blocking

Visual Basic	[form.]Socket.IsWritable

Remarks	This property returns True if data can be written to the socket without blocking. For non-blocking sockets, this property can be checked before the application attempts to write to the socket, preventing a WSAEWOULDBLOCK error.

Data Type	Integer (Boolean)

See Also	IsClosed Property, IsReadable Property, Write Event

Library Property

Description	Sets or returns the file name of the Windows Sockets library

Visual Basic	[form.]Socket.Library [= libfile$]

Remarks	The Library property returns the full pathname of the Windows Sockets library loaded by the control. Setting the property to the file name of an appropriate DLL causes that library to be loaded. By default, a file named WINSOCK.DLL is loaded for 16-bit versions of the control, and WSOCK32.DLL for 32-bit versions.

Library Property

Note This should be the first property set by the control when it is loaded. Attempting to set this property after a library has already been loaded will generate an error.

Data Type	String

KeepAlive Property

Description	Set or return if “keep alives” are sent on a connected socket

Visual Basic	[form.]Socket.KeepAlive [= { True | False }]

Remarks	Setting this property to a value of True indicates that packets are to be sent to the remote system when no data is being exchanged to keep the connection active. This property can only be set for SOCK_STREAM socket types.

Data Type	Integer (Boolean)

See Also	Broadcast Property, InLine Property, ReuseAddress Property, Route Property

LastError Property

Description	Set or return the last error that occurred on the socket

Visual Basic	[form.]Socket.LastError [= errval%]

Remarks	This property can be read to determine the last error that occurred for this socket. If a value is assigned to this property, it must either be zero (to clear the error) or a valid socket error code.

Data Type	Integer

See Also	LastError Event

Linger Property

Description	Set or return the number of seconds to linger on a close

Visual Basic	[form.]Socket.Linger [= nsecs%]

Remarks	Setting this property to a value greater than zero indicates that the SOCKET_CLOSE action should wait up to the specified number of seconds for any data on the socket to be written before it is closed. A value of zero indicates that the socket should be closed immediately (but gracefully, without data loss).

Data Type	Integer

See Also	Disconnect Event

Listening Property

Description	Returns if the socket is listening for connections

Visual Basic	[form.]Socket.Listening

Remarks	This read-only property returns True if the socket is listening for connections after the SOCKET_LISTEN action is taken.

Data Type	Integer (Boolean)

See Also	Accept Property, Backlog Property, Accept Event

LocalAddress Property

Description	Return the IP address of the local host

Visual Basic	[form.]Socket.LocalAddress

Remarks	This read-only property returns the local host’s IP address in dot notation (four numbers seperated by periods).

Data Type	String

See Also	HostAddress Property, LocalName Property, PeerAddress Property

LocalName Property

Description	Return the name of the local host

Visual Basic	[form.]Socket.LocalName

Remarks	This read-only property returns the name of the local host. The name that is returned depends on the configuration of the TCP/IP software.

Data Type	String

See Also	HostName Property, LocalAddress Property, PeerName Property

LocalPort Property

Description	Set or return the port number for a local listening socket

Visual Basic	[form.]Socket.LocalPort [= portno%]

Remarks	This property is used to set the port number that a local server will listen on for connections. If the port number specifies a well-known port, the LocalService property will be updated with that name.

Data Type	Integer

LocalPort Property

See Also	LocalService Property, RemotePort Property

LocalService Property

Description	Set or return the name of a well-known local port

Visual Basic	[form.]Socket.LocalService [= servicename$]

Remarks	This property is used to set the port that a local server will listen on for connections. If the service name does not exist, an error is generated. The LocalPort property is updated to reflect the service’s port number.

Data Type	String

See Also	LocalPort Property, RemoteService Property

MaxDatagram Property

Description	Return the maximum size of a UDP datagram

Visual Basic	[form.]Socket.MaxDatagram

Remarks	The MaxDatagram property returns the maximum number of bytes that may be transmitted in a single write operation on a SOCK_DGRAM type socket. Attempting to send more than this number of bytes will result in an error, with no data being written.

Data Type	Long

See Also	MaxSockets Property, SendData Property, SocketType Property

MaxSockets Property

Description	Return the maximum number of sockets that can be created

Visual Basic	[form.]Socket.MaxSockets

Remarks	The MaxSockets property returns the maximum number of sockets that may be created, system wide. This value is determined by the underlying TCP/IP stack.

Data Type	Integer

See Also	MaxDatagram Property

Peek Property

Description	Set or return if data is to be removed from the socket when read

Visual Basic	[form.]Socket.Peek [= { True | False }]

Remarks	If this property is set to a value of  True, the data that is read from the socket is not removed, and may be read again. This property is automatically reset to a value of  False after data has been read from the socket.

Data Type	Integer (Boolean)

See Also	RecvData Property, Read Event

PeerAddress Property

Description	Return the IP address of the remote peer

Visual Basic	[form.]Socket.PeerAddress

Remarks	This read-only property returns the IP address of a the peer in dot notation (four numbers seperated by periods). The “peer” is the remote system that is either connected to, or was last sent data (if a connectionless socket is being used).

Data Type	String

See Also	HostAddress Property, LocalAddress Property, PeerName Property

PeerName Property

Description	Return the name of the remote peer

Visual Basic	[form.]Socket.PeerName

Remarks	This read-only property returns the name of the peer. ). The “peer” is the remote system that is either connected to, or was last sent data (if a connectionless socket is being used).

Data Type	String

See Also	HostName Property, LocalName Property, PeerAddress Property

Protocol Property

Description	Set or return the protocol that should be used to create the socket

Visual Basic	[form.]Socket.Protocol [= proto%]

Remarks	This property may only be set before a socket has been created, or after it has been closed. Supported socket protocols are:

Protocol Property

| Value | Constant | Description |
| --- | --- | --- |
| 0 | IPPROTO_IP | Default IP protocol. This value indicates that the protocol appropriate for the socket type should be used, and is the default. |
| 6 | IPPROTO_TCP | Transmission Control Protocol. This protocol should be used with stream sockets. |
| 17 | IPPROTO_UDP | User Datagram Protocol. This protocol should be used with datagram sockets. |

There may be other protocols supported by your vendor, or in future versions of the Windows Sockets specification. Consult your TCP/IP documentation to determine what protocols are valid.

Data Type	Integer

See Also	AddressFamily Property, SocketType Property

RecvData Property

Description	Return data read from the socket

Visual Basic	[form.]Socket.RecvData

Remarks	This property returns data that has been read from the socket, up to the number of bytes specified by the RecvLen property. If no data is available to be read, an error will be generated if the socket is non-blocking. If the socket is blocking, the program will stop until data is written on the socket, or the socket is closed.

Note that there may be fewer bytes returned than specified by the RecvLen property. If the socket is in text mode, only the characters up to a newline are returned by this property.

Data Type	String

See Also	Peek Property, RecvLen Property, RecvNext Property, SendData Property, Urgent Property, Read Event

RecvLen Property

Description	Set the maximum number of bytes to read, or return the number of bytes read

Visual Basic	[form.]Socket.RecvLen [= maxlen%]

Remarks	If set to a value, this specifies the maximum number of bytes that may be returned by the RecvData property. After the data has been read, RecvLen contains the actual number of bytes that have been read.

Note that it is common for the number of bytes to be read from the socket to be fewer than that which was specified. The application should never make any assumptions about the number of bytes received. If an error occurs, or the socket is closed, this property will have a value of zero.

RecvLen Property

Data Type	Integer

See Also	RecvData Property, RecvNext Property, Read Event

RecvNext Property

Description	Return the number of bytes available to be read from the socket

Visual Basic	[form.]Socket.RecvNext

Remarks	This read-only property returns the number of bytes that are remaining in the socket and available to be read. If the socket is buffered, it is possible that this property will return a non-zero value after the socket has been closed.

Data Type	Integer

See Also	RecvData Property, RecvLen Property, Read Event

RemotePort Property

Description	Set or return the port number for a remote connection

Visual Basic	[form.]Socket.RemotePort [= portno%]

Remarks	This property is used to set the port number that a local client will use to establish a connection with a remote system. If the port number specifies a well-known port, the RemoteService property will be updated with that name.

Data Type	Integer

See Also	LocalPort Property, RemoteService Property

RemoteService Property

Description	Set or return the name of a well-known remote port

Visual Basic	[form.]Socket.RemoteService [= servicename$]

Remarks	This property is used to set the port that a local client will use to establish a connection with a remote system. If the service name does not exist, an error is generated. The RemotePort property is updated to reflect the service’s port number.

Data Type	String

See Also	LocalService Property, RemotePort Property

ReuseAddress Property

Description	Set or return if an address can be reused

Visual Basic	[form.]Socket.ReuseAddress [= { True | False }]

Remarks	Setting this property to a value of True allows the address that the socket is listening on to be reused. If the LocalPort property is set to 0, then this property is ignored.

Data Type	Integer (Boolean)

See Also	Broadcast Property, InLine Property, KeepAlive Property, Route Property

Route Property

Description	Set or return if packets should be routed

Visual Basic	[form.]Socket.Route [= { True | False }]

Remarks	Setting this property to False tells the socket library that packets are not to be routed, but rather sent directly to the network interface.

Data Type	Integer (Boolean)

See Also	Broadcast Property, InLine Property, KeepAlive Property, ReuseAddress Property

SendData Property

Description	Write data to the socket

Visual Basic	[form.]Socket.SendData = data$

Remarks	By assigning a value to this property, the data in the string is written to the socket. If the socket is buffered, the data is copied to the send buffer and control immediately returns to the program. If the socket is non-blocking, and the socket is out of buffer space, the error WSAEWOULDBLOCK will be generated. If the socket is blocking, the program will wait until the data can be sent.

The maximum number of bytes written on the socket is determined by the SendLen property. It is possible that fewer than the number of bytes specified will be written to the socket. After the data has been written, the SendLen property will be changed to reflect the number of bytes actually written to the socket. If the socket is non-blocking and the

send fails with WSAEWOULDBLOCK, the Write event will be fired when the socket can be written to again.

Data Type	String

See Also	SendLen Property, Timeout Property, Urgent Property, Timeout Event, Write Event

SendLen Property

Description	Set or return the maximum number of bytes to write to the socket

Visual Basic	[form.]Socket.SendLen [= datalen%]

Remarks	If set to a value, this property specifies the maximum number of bytes that may be written to the socket. If this value is greater than the length of the string being sent, the value is ignored and all of the characters are sent.

After data has been written to the socket, this property is updated to reflect the actual number of bytes written.

Data Type	Integer

See Also	SendData Property, Write Event

Shutdown Property

Description	Stop reading and/or writing on the socket

Visual Basic	[form.]Socket.Shutdown [= what%]

Remarks	This write-only property shuts down reading and/or writing on the socket. Any further attempt to send or receive data will return an error on the socket. The possible values that may be assigned to this property are:

| Value | Constant | Description |
| --- | --- | --- |
| 0 | SOCKET_READ | All subsequent attempts to read data from the socket are disallowed. If the socket is buffered, it is possible that data may still be read until the buffer is exhausted. |
| 1 | SOCKET_WRITE | All subsequent attempts to write data to the socket are disallowed. If the socket is buffered, it is possible that data may be written until the buffer is full. |
| 2 | SOCKET_READWRITE | Both reads and writes to the socket are disallowed. |

Note that shutting down a socket is not the same as closing it. The socket will remain connected, and no resources will be freed until the SOCKET_CLOSE action is taken.

Data Type	Integer

See Also	IsReadable Property, IsWritable Property

SocketType Property

Description	Sets or returns the type of socket that should be created

Visual Basic	[form.]Socket.SocketType [= socktype%]

SocketType Property

Remarks	This property may only be set before a socket has been created, or after it has been closed. Supported socket types are:

| Value | Constant | Description |
| --- | --- | --- |
| 1 | SOCK_STREAM | Stream socket used with the TCP protocol. This type of socket provides a reliable byte stream that may be read similar to the way a file is read. |
| 2 | SOCK_DGRAM | Datagram socket used with the UDP protocol. This type of socket is used to transfer datagrams using a fast (but unreliable) protocol. Datagrams may arrive out of sequence, or not at all. Retransmission of lost datagrams is the responsibility of the application. |
| 3 | SOCK_RAW | A raw socket that is commonly used to send ICMP messages over the network. This socket type may or may not be supported by your TCP/IP vendor. |

There may be other types of sockets supported by your vendor, or in future versions of the Windows Sockets specification. Consult your TCP/IP documentation to determine what socket types are valid.

Data Type	Integer

See Also	AddressFamily Property, Protocol Property

State Property

Description	Return the current state of the socket

Visual Basic	[form.]Socket.State

Remarks	This read-only property returns the state of the socket. This property should be checked on blocking sockets to determine if the socket is in use before taking some action. The possible values returned by this property are:

| Value | Constant | Description |
| --- | --- | --- |
| 0 | SOCKET_UNUSED | The socket has not been created. Attempts to use the socket will generate an error. |
| 1 | SOCKET_IDLE | The socket exists, but is not currently in use. A blocking socket operation can be executed at this point. |
| 2 | SOCKET_LISTENING | The socket is listening for connections from remote hosts |
| 3 | SOCKET_CONNECTING | The socket is in the process of connecting to a remote host |
| 4 | SOCKET_ACCEPTING | The socket is in the process of accepting a connection from a remote client |
| 5 | SOCKET_RECEIVING | The socket is in the process of receiving data |

State Property

| 6 | SOCKET_SENDING | The socket is in the process of sending data |  |
| --- | --- | --- | --- |
| 7 | SOCKET_CLOSING | The socket is being closed. Subsequent attempts to access the socket will result in an error. |  |

Note that for non-blocking sockets, the only possible states that may be returned are SOCKET_UNUSED, SOCKET_IDLE or SOCKET_CLOSING.

Data Type	Integer

See Also	IsBlocked Property, Blocking Event, Cancel Event, Timeout Event

Timeout Property

Description	Set or return the amount of time until a blocking operation fails

Visual Basic	[form.]Socket.Timeout [= msecs&]

Remarks	Setting this property specifies the number of milliseconds until a blocking operation fails with the error WSAETIMEDOUT. A value of zero indicates that the blocking operation should wait indefinitely.

Data Type	Long

See Also	Accept Property, Action Property, RecvData Property, SendData Property, Timeout Event

Urgent Property

Description	Send or receive urgent data

Visual Basic	[form.]Socket.Urgent [= { True | False }]

Remarks	This Boolean property affects how the RecvData and SendData properties read or write data to the socket. If set to a value of True, urgent (out-of-band) data will be read or written. All reads or writes of urgent data are unbuffered. The property value will automatically be reset to a value of False after the socket has been read or written. Not all implementations may support more than one byte of urgent data if the data is not being received in-line. Refer to the InLine property for additional information.

Data Type	Integer (Boolean)

See Also	RecvData Property, SendData Property, Read Event

Vendor Property

Description	Returns the name of the TCP/IP vendor

Visual Basic	[form.]Socket.Vendor

Vendor Property

Remarks	This read-only property returns a string that contains the name of the vendor that has supplied the Windows Sockets library that is being used.

Data Type	String

See Also	Version Property

Version Property

Description	Returns the version of the Windows Sockets library

Visual Basic	[form.]Socket.Version

Remarks	This read-only property returns a string that specifies the version of the Windows Sockets library that is being used. Note that at least version 1.1 is required for use with this control.

Data Type	String

See Also	Vendor Property

Accept Event

Description	The Accept event is generated when a remote host connects to a listening socket.

Visual Basic	Sub Socket_Accept ([Index As Integer,] SocketID As Integer)

Remarks	This event is generated for sockets that are listening for connections from a remote host. A connection with the remote system is not actually established until it has been accepted by the listening server.

The SocketID argument specifies the socket descriptor of the listening socket. To accept the connection, one of the following actions must be taken:

- The program sets the Action property to a value of SOCKET_ACCEPT. This allows the socket that was listening to connect with the remote host. However, this method closes the listening socket, so only one host can establish a connection with the application.

- A second socket has it’s Accept property set to the value of SocketID. The second socket accepts the connection on behalf of the server, and the original socket may continue to listen for additional connections.

Once the connection has been established, the PeerAddress or PeerName properties may be used to determine the name of the remote host that has connected with the application.

See Also	Accept Property, Action Property, PeerAddress Property, PeerName Property

Blocking Event

Description	The Blocking event is generated whenever a blocking operation occurs

Visual Basic	Sub Socket_Blocking ([Index As Integer,] Status As Integer, Cancel As Integer)

Remarks	This event is generated immediately before a blocking operation takes place, and provides a kind of blocking hook function for the application.

The Status argument specifies which blocking action is about to be taken. For a list of values, refer to the State property.

The Cancel argument allows the blocking operation to be canceled if the value is set to True. The blocking function will fail with the error WSAEINTR.

It is not recommended that code be placed in this event that would take any significant amount of time to complete or calls DoEvents to yield time to other tasks.

See Also	State Property, Cancel Event

Cancel Event

Description	The Cancel event is generated when a blocking operation is canceled

Visual Basic	Sub Socket_Cancel ([Index As Integer,] Status As Integer, Response As Integer)

Remarks	This event is generated when a blocking operation on the socket, such as sending or receiving data, is canceled with the SOCKET_CANCEL action.

The Status argument specifies which blocking operation was canceled. For a list of values, refer to the State property.

The Response argument determines if a WSAEINTR error is generated in response to the canceled operation. The possible values are:

| Value | Constant | Description |
| --- | --- | --- |
| 0 | SOCKET_ERRIGNORE | Ignore the canceled event and return as though the operation completed successfully. |
| 1 | SOCKET_ERRDISPLAY | Return the WSAEINTR error to the canceled socket operation. This is the default response to the event. |

Setting Response to ignore the canceled event can result in unexpected errors. For example, if a blocking read on the socket is cancelled, the WSAEWOULDBLOCK error may be returned.

See Also	State Property, Blocking Event, LastError Event

Connect Event

Description	The Connect event is generated when a connection is established

Visual Basic	Sub Socket_Connect ( [Index As Integer] )

Remarks	This event is generated when a connection is made with a remote host as a result of a SOCKET_CONNECT action, or when a connection is accepted on a listening socket.

See Also	Action Property, Disconnect Event

Disconnect Event

Description	The Disconnect event is generated when the socket is closed

Visual Basic	Sub Socket_Disconnect ( [Index As Integer] )

Remarks	This event occurs when a remote host closes the socket connection. It is not generated when the SOCKET_CLOSE action is taken. When this event is generated, the local socket should be closed to free the resources allocated to it.

See Also	Action Property, Connect Event

LastError Event

Description	The LastError event is generated when a socket error occurs

Visual Basic	Sub Socket_LastError ([Index As Integer,] ErrCode As Integer, ErrMsg As String, Response As Integer)

Remarks	This event is generated when an error occurs during a socket operation. Visual Basic errors (e.g.: Invalid Property) will not generate this event.

The ErrCode argument specifies the error that has occurred on the socket. For a list of errors, refer to Appendix A.

The ErrMsg argument is a string that describes the error that occurred.

The Response argument determines if the socket error generates a Visual Basic error. The possible values are:

| Value | Constant | Description |
| --- | --- | --- |
| 0 | SOCKET_ERRIGNORE | Ignore the error and return as though the operation completed successfully. |
| 1 | SOCKET_ERRDISPLAY | Return the error to Visual Basic, which may be trapped by the application. This is the default response to the event. |

See Also	Cancel Event

Read Event

Description	The Read event is generated when data is available to be read

Visual Basic	Sub Socket_Read ([Index As Integer,] DataLength As Integer, IsUrgent As Integer)

Remarks	This event is generated for non-blocking sockets when data is available to be read from the socket.

The DataLength argument specifies the number of bytes that can be read from the socket.

The IsUrgent argument specifies if the data to be read is marked as urgent. A non-zero value indicates that the Urgent property should be set to True to receive the out-of-band data. This flag is only set if the InLine property is False.

It is possible for the RecvNext property to return a value greater than DataLength if additional data has been written to the socket while executing code inside the event.

See Also	IsReadable Property, Write Event

Timeout Event

Description	The Timeout event is fired when a blocking operation times out

Visual Basic	Sub Socket_Timeout ([Index As Integer,] Status As Integer, Response As Integer)

Remarks	This event is generated when a blocking socket operation, such as sending or receiving data, times out.

The Status argument specifies which blocking operation timed out. For a list of values, refer to the State property.

The Response argument determines if a WSAETIMEDOUT error is generated. The possible values are:

| Value | Constant | Description |
| --- | --- | --- |
| 0 | SOCKET_ERRIGNORE | Ignore the timeout and return as though the operation completed successfully. |
| 1 | SOCKET_ERRDISPLAY | Return the WSAETIMEDOUT error to the timed-out socket operation. This is the default response to the event. |

See Also	State Property, Timeout Property, Cancel Event

Timer Event

Description	The Timer event is fired when the control’s preset timer interval expires

Visual Basic	Sub Socket_Timer ( [Index As Integer] )

Remarks	This event is generated when the control's timer interval has elapsed. The frequency is specified in milliseconds by setting the Interval property.

See Also	Interval Property

Write Event

Description	The Write event is generated when data can be written to the socket

Visual Basic	Sub Socket_Write ( [Index As Integer] )

Remarks	This event is generated for non-blocking sockets when data can be written to the socket after a previous attempt failed with the WSAEWOULDBLOCK error.

See Also	IsWritable Property, Read Event

Appendix A - Error Codes

The error codes listed here are based on Windows Sockets errors (which, in turn, are based on the error codes that are returned by the Berkeley sockets implementation). The base error value has been increased by 14000 to be compatible with Visual Basic.

| 24004 | WSAEINTR | Blocking function was canceled |
| --- | --- | --- |
| 24009 | WSAEBADF | Invalid socket descriptor passed to function |
| 24013 | WSAEACCES | Access denied |
| 24014 | WSAEFAULT | Invalid address passed to function |
| 24022 | WSAEINVAL | Invalid socket function call |
| 24024 | WSAEMFILE | No socket descriptors are available |
| 24035 | WSAEWOULDBLOCK | Socket would block on this operation |
| 24036 | WSAEINPROGRESS | Blocking function in progress |
| 24037 | WSAEALREADY | Function being canceled has already completed |
| 24038 | WSAENOTSOCK | Invalid socket descriptor passed to function |
| 24039 | WSAEDESTADDRREQ | Destination address is required |
| 24040 | WSAEMSGSIZE | Datagram was too large to fit in specified buffer |
| 24041 | WSAEPROTOTYPE | Specified protocol is the wrong type for this socket |
| 24042 | WSAENOPROTOOPT | Socket option is unknown or unsupported |
| 24043 | WSAEPROTONOSUPPORT | Specified protocol is not supported |
| 24044 | WSAESOCKTNOSUPPORT | Specified socket type is not supported in this address family |
| 24045 | WSAEOPNOTSUPP | Socket operation is not supported |
| 24046 | WSAEPFNOSUPPORT | Specified protocol family is not supported |
| 24047 | WSAEAFNOSUPPORT | Specified address family is not supported by this protocol |
| 24048 | WSAEADDRINUSE | Specified address is already in use |
| 24049 | WSAEADDRNOTAVAIL | Specified address is not available |
| 24050 | WSAENETDOWN | Network subsystem has failed |
| 24051 | WSAENETUNREACH | Network cannot be reached from this host |
| 24052 | WSAENETRESET | Network dropped connection on reset |
| 24053 | WSAECONNABORTED | Connection was aborted due to timeout or other failure |
| 24054 | WSAECONNRESET | Connection was reset by remote network |
| 24055 | WSAENOBUFS | No buffer space is available |
| 24056 | WSAEISCONN | Socket is already connected |
| 24057 | WSAENOTCONN | Socket is not connected |
| 24058 | WSAESHUTDOWN | Socket connection has been shut down |
| 24060 | WSAETIMEDOUT | Operation timed out before completion |
| 24061 | WSAECONNREFUSED | Connection refused by remote network |
| 24064 | WSAEHOSTDOWN | Remote host is down |
| 24065 | WSAEHOSTUNREACH | Remote host is unreachable |
| 24091 | WSASYSNOTREADY | Network subsystem is not ready for communication |
| 24092 | WSAVERNOTSUPPORTED | Requested version is not available |
| 24093 | WSANOTINITIALIZED | Windows sockets library not initialized |
| 25001 | WSAHOST_NOT_FOUND | Authoritative Answer Host not found |
| 25002 | WSATRY_AGAIN | Non-authoritative Answer Host not found |
| 25003 | WSANO_RECOVERY | Non-recoverable error |
| 25004 | WSANO_DATA | No data record of requested type |