Synchronized Voice Server Function Set

RunPager Software Engineering

For Visual Basic Client-Server Voice Telephony Operations On Dialogic Voice Boards

All of the information in this document is subject to change without notice. No part of this document may be reproduced

or transmitted in any form or by any means without the express written permission of RunPager Software Engineering

© 1998 - RunPager Software Engineering, All rights reserved.

Printed in the United States

Use of this software is restricted by the terms of the license agreement. ( -see svsorder.wri )

Software Copyright © 1998 - RunPager Software Engineering

Windows™ and Visual Basic™ are trademarks of Microsoft Corporation.

Dialogic™ is a trademark of Dialogic Corporation.		HTTP://WWW.DIALOGIC.COM

Contact Information:	Bob Johnson

RunPager Software Engineering

1824 Point View Avenue, Youngstown, Ohio 44502

(330) 788-3073, RunPager@NEO.RR.COM

Ordering Information:	See File SVSORDER.WRI

This document may be easier to use with your display set to 800 x 600.   Use WordPad.exe and set the page margins to .5" left margin and .5" right margin.

Operating Systems Requirements

SVSERVER and SNSERVER run on Windows 95/98 and single processor NT4.0 operating systems.

Visual Basic Voice Telephony Software Tool For Dialogic Boards

SVSERVER is a synchronized voice server which allows Visual Basic and\or any database or application

language which supports the Microsoft DDEML to drive Dialogic professional voice boards.  This download supplies a sample Visual Basic 6.0 client program that access the voice server software.  In addition, this documentation is provided to enhance your understanding of the voice client server interaction and operation in

developing voice telephony applications for the SVSERVER.  VB4.0 samples are available upon request.

Client-Server Operation

The client application sends function commands as an ascii string via the Dynamic Data Exchange medium of

Microsoft Windows DDEML to the SVSERVER application which receives the commands and executes them through an

interface to the Dialogic Multithread Function Library.  When the function completes, the SVSERVER returns the Dialogic

function status and any data to the client application.  The design of SVSERVER permits multiple client applications

having different channel assignments to interact with the voice server simultaneously.  For instance, one application may

be running voice mail on channels 1 to 4 while some other  automated telephony application runs using channels 5 to 8.

This methodology may be implemented in Visual Basic or any database or application language that supports the

Microsoft Windows DDEML.  The sample in this documentation is limited to VB Text Controls.  The user should first

review the initiate-response interaction of the client-server operation and then study the sample synchronous applications

SVSAMPLE and SNSAMPLE.  Due to the very voluminous nature of asynchronous coding, no example has been offered

but is available on request.

Simplicity of Design

The SVSERVER  application handles approximately one hundred and seventy functions consisting of nine ez_

ease of use synchronous functions, twenty two vs_ utility functions and about one hundred and forty dx_ or sr_ Dialogic

functions.  This range makes it both easy to use and a powerful implementation of the full Dialogic function set.  It may

be operated in one or two threads for asynchronous modes of operation or one thread per channel for synchronous

modes.  Versions are available in from 1 to 128 lines.  Future updates will add functions for handling the remainder of the Dialogic Function Libraries.

Getting Started

Unzip in an empty folder.  Rename the folder SVSERVER and copy to the root directory, C:\SVSERVER.  If you

do not place it in the root directory you will have problems running the sample Visual Basic programs provided.

Evaluation Version of the Software

The evaluation version of the software is provided in both source and executable for the sample applications and

is limited to 1 line of operation in the SVSERVER and\or SNSERVER.

The Dialogic Configuration Manager and SVSERVER Synchronized Voice Server

The Dialogic Configuration Manager must start the boards before running SVSERVER or SNSERVER

applications and the SVSERVER or SNSERVER application must be running prior to a client application sending Dialogic

voice function commands to it.  Please Note: The SVSERVER and SNSERVER applications should be launched with

command line arguments.  There are a number of methodologies available to develop telephony software with

SVSERVER and SNSERVER as follows:

I. SVSERVER Applications

1)	A single client application driving a single SVSERVER multithreaded process.

The SVSAMPLE application demonstrates a VB client application designed to run eight lines via a single process

multithreaded SVSERVER.

DIALOGIC Channel 1

DIALOGIC Channel 2

CLIENT APPLICATION		<-DDE->	SVSERVER	<--->	DIALOGIC Channel 3

DIALOGIC Channel 4

---

DIALOGIC Channel n

To run as a one thread per channel synchronous operation use the term "MultiThread" in the command string

used to start the SVSERVER voice server.  (-See the VB6.0 application SVSample procedure: ShellSVServer)

Result = Shell(Trim$(SVServerFilePath) & " MultiThread", 1)

To limit the number of lines use "MaxLines nn".  For instance if you have a 32 line version of SVSERVER but

you want to limit the number of active lines to 8 use the term "MaxLines 8".  This prevents the software from creating

unused threads and wasting cpu time.

Result = Shell(Trim$(SVServerFilePath) & " MultiThread, MaxLines " & Trim$(Str(MAXLines)), 1)

2)	Several client applications with differing line assignments driving a single SVSERVER multithreaded process.

DIALOGIC Channel 1

CLIENT APPLICATION 1	<-DDE->				DIALOGIC Channel 2

CLIENT APPLICATION 2	<-DDE->	SVSERVER	<--->	DIALOGIC Channel 3

CLIENT APPLICATION 3	<-DDE->				DIALOGIC Channel 4

---

DIALOGIC Channel n

Each client application has different channel assignments.  Lines are assigned when the DDE conversation is

established between the client application and the SVSERVER.

For Asynchrous Operations:

To run as a single thread process where the single thread is blocked with wait event functions in asynchronous

mode use the command:

Result = Shell(Trim$(SVServerFilePath), 1)

For a dual thread asynchronous operation include the term "EventThread" in the command line.  The wait events

will then run under the separate event thread.  This way only the event thread will be blocked by wait events.

Result = Shell(Trim$(SVServerFilePath) & " EventThread", 1)

II. SNSERVER Applications

1)	A single client application driving multiple SNSERVER single thread processes.

The SNSAMPLE application demonstrates a VB client application designed to run eight lines via eight single

process SNSERVER applications.  (-See the VB6.0 application SNSample procedure: ShellSNServer)

SNSERVER 01	<--->	DIALOGIC Channel 1

SNSERVER 02	<--->	DIALOGIC Channel 2

CLIENT APPLICATION		<-DDE->	SNSERVER 03	<--->	DIALOGIC Channel 3

SNSERVER 04	<--->	DIALOGIC Channel 4

---		---	---

SNSERVER nn	<--->	DIALOGIC Channel n

2)	Multiple client applications each driving a single SNSERVER single thread processes.

CLIENT APPLICATION 01	<-DDE->	SNSERVER 01	<--->	DIALOGIC Channel 1

CLIENT APPLICATION 02	<-DDE->	SNSERVER 02	<--->	DIALOGIC Channel 2

CLIENT APPLICATION 03	<-DDE->	SNSERVER 03	<--->	DIALOGIC Channel 3

CLIENT APPLICATION 04	<-DDE->	SNSERVER 04	<--->	DIALOGIC Channel 4

---				---		---		---	---

CLIENT APPLICATION nn	<-DDE->	SNSERVER nn	<--->	DIALOGIC Channel n

Client-Server Interaction

Please Note: The term line and channel may be used interchageably in this document.

Client Application Initiates				SVSERVER Voice Server Responds

1)	Establishes a conversation with the SVSERVER

Text1.LinkMode = 0	'	break connection

Text1.LinkTopic = "SVServer|SVSFunction"

Text1.LinkItem = "Line1"

Text1.LinkMode = 3	'	establish connection

do this once for each line

2)	SVSERVER responds to DDEML accordingly

3)	Sends a function command to the SVSERVER

Text1.Text = "dx_open"	'	open the line

Text1.LinkPoke		'	send to SVServer

4)	SVSERVER  executes the function then advises the 								client application when it has completed the function 								that it has status data to return.

5)	Notification arrives via the text controls Link Notify Event

The text control's link notify procedure then requests

that SVSERVER send the data

Private Sub Text1_LinkNotify

On Error Resume Next

Text1.Text = ""

Text1.LinkRequest	'	request data

End Sub

6)	SVSERVER sends the function status to the client 								application's text control

7)	Status data arrives in the application's text control

LNE=3, FNC=100, STA=4, ERR=0, RTN=2, RSL:

The client application's main loop watches for the

status information to arrive and processes it when it comes.

It then continues sending other commands and receiving status.

The status.  From the client application's point of view this is a simple send-receive, send-receive scenario:

1)	Send the function command to the server.

2)	Do whatever ... until notification is received from the server.

3)	Send the request that the server return the status.

4)	Receive the function status and data from the sever.

If the text control's link mode is set to automatic then this process is reduced even further to just send-receive.

The link notify event is handled automatically.

1)	Send the function command to the server.

2)	Receive the function status and data from the sever.

Returned Function Status

Example	LNE=3, FNO=100, STA=4, ERR=0, RTN=2, RSL: Resultant Extended Data Returned Here

Line Number			LNE=	1 - 32

Status				STA=	SVSERVER Function Progress Status

STATUS_FUNCTION_NONE		0	SVServer Channel Idle

STATUS_FUNCTION_READY		1	Function Ready To Run

STATUS_FUNCTION_RUNNING		2	Function Started

STATUS_FUNCTION_NOTIFIED		3	Client Application Notified

STATUS_FUNCTION_COMPLETE	4	Function Completion

Return				RTN=	Value Returned From Dialogic Function, -1 or < 0  = Failure

Result				RSL:	Resultant Extended Data From Dialogic Function -see function specifics

Error				ERR=	Errors Pertain To Operation Of SVSERVER Only, Not Dialogic Errors

Function Not Found			1	Syntax Error In Function Name

Memory Allocation Error			2	Unable To Allocate Memory Space

Index Value Out Of Range		3	Index Value Out Of Range

Parameter Count Error			4	Too Many Or Too Few Parameters Sent

Function Not Implemented		5	Function Not Implemented

Null Pointer				6	Null Pointer Encountered, Bad Pointer

Empty Parameter Value			7	Empty Parameter Value (,,)

Negative Value Of Function No		< 0	Negative Value Specifies Function Number In Error

Functions			FNO=		Function Identification Number

ag_getctinfo				0

ag_getxmitslot				1

ag_listen				2

ag_unlisten				3

ATDV_ERRMSGP			4

ATDV_IOPORT				5

ATDV_IRQNUM				6

ATDV_LASTERR			7

ATDV_NAMEP				8

ATDV_SUBDEVS			9

ATDX_ANSRSIZ				10

ATDX_BDNAMEP			11

ATDX_BDTYPE				12

ATDX_BUFDIGS			13

ATDX_CHNAMES			14

ATDX_CHNUM				15

ATDX_CONNTYPE			16

ATDX_CPERROR			17

ATDX_CPTERM				18

ATDX_CRTNID				19

ATDX_DBMASK				20

ATDX_DEVTYPE			21

ATDX_DIGBUFMODE			22

ATDX_DTNFAIL				23

ATDX_EVTCNT				24

ATDX_FRQDUR				25

ATDX_FRQDUR2			26

ATDX_FRQDUR3			27

ATDX_FRQHZ				28

ATDX_FRQHZ2				29

ATDX_FRQHZ3				30

ATDX_FRQOUT				31

ATDX_FWVER				32

ATDX_HOOKST				33

ATDX_LINEST				34

ATDX_LONGLOW			35

ATDX_NUMCHAN			36

ATDX_PHYADDR			37

ATDX_PRODUCT			38

ATDX_RINGCNT				39

ATDX_SHORTLOW			40

ATDX_SIZEHI				41

ATDX_STATE				42

ATDX_TERMMSK			43

ATDX_TONEID				44

ATDX_TRCOUNT			45

dx_addspddig				46

dx_addtone				47

dx_addvoldig				48

dx_adjsv				49

dx_blddt				50

dx_blddtcad				51

dx_bldst				52

dx_bldstcad				53

dx_bldtngen				54

dx_chgdur				55

dx_chgfreq				56

dx_chgrepcnt				57

dx_close				58

dx_clrcap				59

dx_clrdigbuf				60

dx_clrsvcond				61

dx_clrtpt				62

dx_deltones				63

dx_dial					64

dx_distone				65

dx_enbtone				66

dx_filechsize				67

dx_fileclose				68

dx_filecommit				69

dx_filedup				70

dx_filedup2				71

dx_fileerrno				72

dx_filefilelength				73

dx_filefstat				74

dx_fileisatty				75

dx_filelocking				76

dx_fileopen				77

dx_fileopen_osfhandle			78

dx_fileread				79

dx_fileseek				80

dx_filesetmode				81

dx_filesopen				82

dx_filestat				83

dx_filetell				84

dx_fileumask				85

dx_filewrite				86

dx_getctinfo				87

dx_getcursv				88

dx_getdig				89

dx_getevt				90

dx_getparm				91

dx_getr2mf				92

dx_getsvmt				93

dx_getxmitslot				94

dx_gtcallid				95

dx_gtextcallid				96

dx_gtsernum				97

dx_initcallp				98

dx_listen				99

dx_open				100

dx_play					101

dx_playf				102

dx_playiottdata				103

dx_playtone				104

dx_playvox				105

dx_playwav				106

dx_rec					107

dx_recf					108

dx_reciottdata				109

dx_recvox				110

dx_recwav				111

dx_route				112

dx_routerxtx				113

dx_sendevt				114

dx_set_lasterr				115

dx_setdigtyp				116

dx_setevtmsk				117

dx_setgtdamp				118

dx_sethook				119

dx_setparm				120

dx_setrings				121

dx_setsvcond				122

dx_setsvmt				123

dx_setuio				124

dx_stopch				125

dx_TSFStatus				126

dx_unlisten				127

dx_wink					128

dx_wtcallid				129

dx_wtring				130

ez_dial					131

ez_getdigits				132

ez_hangup				133

ez_hookflash				134

ez_pickup				135

ez_playfile				136

ez_playfiles				137

ez_playmultp				138

ez_recfile				139

sr_dishdlr				140

sr_enbhdlr				141

sr_getboardcnt				142

sr_getevtdatap				143

sr_getevtdev				144

sr_getevtlen				145

sr_getevttype				146

sr_getparm				147

sr_hold					148

sr_NotifyEvent				149

sr_release				150

sr_setlasterr				151

sr_setparm				152

sr_waitevt				153

sr_waitevtEx				154

vs_allocate				155

vs_deallocate				156

vs_free_setup				157

vs_get_ca_param			158

vs_get_devices				159

vs_get_dv_tpt				160

vs_get_dx_iott				161

vs_get_dx_svcb				162

vs_get_dx_svmt				163

vs_get_dx_xpb				164

vs_load_setup				165

vs_save_setup				166

vs_set_ca_param			167

vs_set_devices				168

vs_set_dv_tpt				169

vs_set_dv_tpt_size			170

vs_set_dx_iott				171

vs_set_dx_iott_size			172

vs_set_dx_svcb				173

vs_set_dx_svmt				174

vs_set_dx_xpb				175

vs_set_timer				176

SVServer Ease of Use Function Definitions

Each of these functions implement a set of Dialogic functions and setup all necessary SVSERVER structures as a means of simplifying the processing of commonly used commands.  Since these functions run a number of Dialogic functions an error in one of them may be indicated by a negative value of that function number being returned in ERR=.

ez_dial:		if on-hook go off-hook and dial a number

Dialogic	none	uses ATDX_HOOKST, dx_sethook, dx_clrcap, dx_dial, ATDV_LASTERR, 			ATDV_ERRMSGP

SVServer	ez_dial, dialstrp, wait, NULL, mode

where	dialstrp = ascii dial string

wait = wait time delay for dialtone in milliseconds

NULL sets capp pointer to NULL, otherwise set to 0

mode = synchronous/asynchronous + call analysis parameter

e.g.	"ez_dial, 3305551234, 1000, NULL, 1"

-see	Dialogic function dx_dial Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

ez_getdigits:		initiates the collection of digits

Dialogic	none	uses dx_clrtpt, dx_getdig, ATDX_TERMMSK, ATDV_LASTERR, ATDV_ERRMSGP

SVServer	ez_getdigits, digits, digitmask, initial, silence, maxtime, mode

where	digits = the number of digits to accumulate from 1 to 32

digitmask = specific digit(s)

intitial = initial silence period allowed before digit input to generate termination

silence = silence period allowed after digit input to generate termination

maxtime = time period allowed

mode = must be synchronous

e.g.	"ez_getdigits , 4, 0, 40, 20, 300, 0"

-see	Dialogic function dx_getdig Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

ez_hangup:		tests hook state, if off-hook then it hangs up the line

Dialogic	none	uses ATDX_HOOKST, dx_sethook

SVServer	ez_hangup

ez_hookflash:		if on-hook go on-hook for time delay in milliseconds then back off-hook

Dialogic	none	uses ATDX_HOOKST, dx_sethook

SVServer	ez_hookflash, timeonhook

timeonhook = time between hangup and pickup, if no timeonhook then default = 500 ms

e.g.	"ez_hookflash" or "ez_hookflash, 500"

ez_pickup:		tests hook state, if on-hook then it picks up the line

Dialogic	none	uses ATDX_HOOKST, dx_sethook

SVServer	ez_pickup

ez_playfile:		plays a .vox or .wav file

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_playf or dx_playwav, ATDX_TERMMSK, ATDV_LASTERR, 			ATDV_ERRMSGP

SVServer	ez_playfile, fNamepath, digits, digitmask, mode

where	fNamepath = file path

digits = number of digits to allow input before generating termination

digitmask = specific digit(s) to allow input before generating termination

mode = must be synchronous

e.g.	"ez_playfile, " & Trim$(CFGTempData.CFGApplicationText(2)) & ", 1, 0, 0"

-see	Dialogic function dx_playf Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

ez_playfiles:		plays a series of files, one after the other in the order entered

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_playf or dx_playwav, ATDX_TERMMSK, ATDV_LASTERR, 			ATDV_ERRMSGP

SVServer	ez_playfile, digits, digitmask, mode, fNamepath1, fNamepath2, ... fNamepathn

where	digits = number of digits to allow input before generating termination

digitmask = specific digit(s) to allow input before generating termination

mode = must be synchronous

fNamepath1, fNamepath2, ... fNamepathn = file paths

e.g.	"ez_playfiles, 1, 0, 0, " & Trim$(Filepath1) & ", " & Trim$(Filepath2)& ", " & Trim$(Filepath3)

-see	Dialogic function dx_playf Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

ez_playmultp:		plays a series of up to 20 files, one after the other in the order entered using dx_playiottdata

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_playiottdata, ATDX_TERMMSK, ATDV_LASTERR,

ATDV_ERRMSGP

SVServer	ez_playmultp, digits, digitmask, mode, fNamepath1, fNamepath2, ... fNamepathn

where	digits = number of digits to allow input before generating termination

digitmask = specific digit(s) to allow input before generating termination

mode = must be synchronous

fNamepath1, fNamepath2, ... fNamepathn = file paths

e.g.	"ez_playmultp, 1, 0, 0, " & Trim$(Filepath1) & ", " & Trim$(Filepath2)& ", " & Trim$(Filepath3)

Note: This command is useful for stringing numbers, dates, time, etc. together for play.

Note: This command is the same as ez_playfiles except it opens all of the files first then uses the

dx_playiottdata command to play the files.  The user must setup the DX_XPB structure.

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

ez_recfile:		records a .vox or .wav file

Dialogic none	uses dx_clrtpt, dx_clrdigbuf, dx_recf or dx_recwav, ATDX_TERMMSK, ATDV_LASTERR, 			ATDV_ERRMSGP

SVServer	ez_recfile, fNamepath, digits, digitmask, initial, silence, mode

where	digits = the number of digits to accumulate before generating termination

digitmask = to allow input before generating termination

intitial = initial silence period allowed before voice to generate termination

silence = silence period allowed after voice to generate termination

mode = must be synchronous

e.g.	"ez_recfile, " & Trim$(CFGTempData.CFGApplicationText(4)) & ", 1, 0, 40, 20, 1"

-see	Dialogic function dx_recf Help file for parameter values

Note: If return = -1 then error = ATDV_LASTERR, result = ATDV_ERRMSGP.

SVServer Special Status Function Definitions

status:			returns the channels current function status

Dialogic	none

SVServer	status

Note: This command be sent via a Line item or SVSCallback item.

stop:			stops all channel io, closes all channels and terminates the SVSERVER

Dialogic	none	uses dx_stopch and dx_close

SVServer	stop

Note: This command be sent via a Line item or SVSCallback item.

stopch			stops input/output on the specified channel

Dialogic	none	uses dx_stopch in asynch mode on a seperate topic/item conversation

SVServer	stopch, channel

where	channel is the channel number returned when the line was opened

requires separate text control and separate item SVSCallback for events

Note: This command must be sent via the SVSCallback item to stop the io synchronous

operation blocking the channel.  The SVSCallback item is normally used for asynchronous wait

events. This command returns nothing.  The io function stopped returns it's status.

separate additional text control and topic used for "stopch"

initialize	Text2.LinkMode = 0	'	break connection

Text2.LinkTopic = "SVServer|SVSFunction"

Text2.LinkItem = "SVSCallback"

Text2.LinkMode = 3	'	establish connection

then send	Text2.Text = "stopch, " & Trim$(Str$(Channel))

Text2.LinkPoke

This link responds only to the "stop", "status" and "stopch" commands.

SVServer Utility Function Definitions

This set of functions are provided as a means of allocating and deallocating space in SVSERVER for the structures required by Dialogic functions and supplying the parameters to fill them.  They also allow the user to examine the current setting of structure values.  The DV_TPT and DX_IOTT structures must be allocated by the user for each line where they are used.  They may be allocated via the vs_set_dv_tpt_size and vs_set_dx_iott_size functions respectfully.  All other structures in SVServer are automatically allocated, one set for each line as shown in the channel table below.

typedef struct {

.

.

CT_DEVINFO		ct_devinfo;

DV_DIGIT		dv_digit;

DV_TPT		*dv_tpt_ptr;	// user must allocate this structure using vs_set_dv_tpt_size

DX_CAP		dx_cap, *dx_cap_ptr;

DX_CST		*dx_cst_ptr;

DX_EBLK		dx_eblk;

DX_IOTT		*dx_iott_ptr;	// user must allocate this structure using vs_set_dx_iott_size

DX_SVMT		dx_svmt;

DX_SVCB		dx_svcb[20];

DX_XPB		dx_xpb;

SC_TSINFO		sc_tsinfo;

TN_GEN		tn_gen;

.

.

} ChannelTable;

ChannelTable			chtable[MAXCHAN];

vs_allocate:		dynamically allocates a character memory buffer

Dialogic none	utility function normally implemented in C code

SVServer	vs_allocate, size

where	size = the number of bytes to allocate

Note: These buffer pointer indexes are not the same as those used by ez_save_setup.

returns	char buffer ptr index, refers to 1 of an array of 256 indexes starting with 0

vs_deallocate:		deallocates a character memory buffer setup by vs_allocate

Dialogic	none	utility function normally implemented in C code

SVServer	vs_deallocate, index

where	index = char buffer ptr index returned from vs_allocate

Note: These buffer pointer indexes are not the same as those used by ez_save_setup.

vs_get_ca_param:	returns the setting for the parameter of the channels DX_CAP call analysis structure

Dialogic	 none	utility function normally implemented in C code

SVServer	vs_get_ca_param, param

where	-see vs_set_ca_param for param identification number 0 - 56

returns	parameter value

vs_get_devices:	returns the array of devices which are used with vs_set_devices & sr_waitevtEx

Dialogic	none	utility function normally implemented in C code

SVServer	vs_get_devices

returns	dev, dev, ... dev

vs_get_dv_tpt:		returns the current setting for a channel's DV_TPT structure array element

Dialogic	none	utility function normally implemented in C code

SVServer	vs_get_dv_tpt, index

where	index = DV_TPT structure array index, 1, 2, 3, ... n

returns	in RSL: type, termno, length, flags, data, rfu

-see Dialogic Help file for DV_TPT structure values

vs_get_dx_iott:		returns the current setting for a channel's DX_IOTT structure array element

Dialogic	none	utility function normally implemented in C code

SVServer	vs_get_dx_iott, index

where	index = DX_IOTT structure array index, 1, 2, 3, ... n

returns	in RSL: type, rfu, fhandle, bufno, offset, length

-see Dialogic Help file for DX_IOTT structure values

vs_get_dx_svcb:	returns one of twenty dx_svcb structure entries for the channel

Dialogic	none	utility function normally implemented in C code

SVServer	vs_get_dx_svcb, index

where	index = array index value 0 - 19

returns	type, adjsize, digit, digtype

-see Dialogic Help file for DX_SVCB structure values

vs_get_dx_svmt:	returns the DX_SVMT structure

Dialogic	none	utility function normally implemented in C code

SVServer	vs_get_dx_svmt

returns	dec, dec, dec, dec, dec,	dec, dec, dec, dec, dec,	org, inc, inc, inc, inc, inc, inc, inc, inc, inc, inc

vs_get_dx_xpb:	returns the DX_XPB structure elements

Dialogic	none	utility function normally implemented in C code

SVServer	vs_get_dx_xpb

returns 	FileFormat, DataFormat, SamplesPerSec, BitsPerSample

-see Dialogic Help file for DX_XPB structure values

vs_free_setup:		releases a memory buffer which were setup by vs_save_setup

Dialogic	none	utility function normally implemented in C code

SVServer	vs_free_setup, index

where	index = buffer ptr index, refers to 1 of an array of 256 indexes starting with 0

Note: These buffer pointer indexes are not the same as those used by vs_allocate

e.g.	"vs_free_setup, 0"

vs_load_setup:		reloads the DV_TPT, DX_IOTT, DX_CAP, DX_SVCB, or TN_GEN structure from a 			memory buffer previously saved using vs_save_setup

Dialogic	none	utility function normally implemented in C code

SVServer	vs_load_setup, index

where	index = memory buffer pointer index returned from vs_save_setup

Note: These buffer pointer indexes are not the same as those used by vs_allocate.

e.g.	"vs_load_setup, 0"

returns	the number of array elements loaded

vs_save_setup:		allocates a memory buffer and saves the setup for the channels current setting of

DV_TPT, DX_IOTT, DX_CAP, DX_SVCB, or TN_GEN structure for reuse via vs_load_setup

Dialogic none	utility function normally implemented in C code

SVServer	vs_save_setup, type		( allocation utility )	( creation utility )

where	type = 	1 - DV_TPT_TYPE	vs_set_dv_tpt_size	vs_set_dv_tpt

2 - DX_IOTT_TYPE	vs_set_dx_iott_size	vs_set_dx_iott

3 - DX_CAP_TYPE

4 - DX_SVCB_TYPE	saves an array of twenty = dx_svcb[20]

5 - TN_GEN_TYPE

e.g.	"vs_save_setup, 1"

returns	the memory buffer ptr index, 1 of an array of 256 indexes starting with 0

Note: You must save this index in order to reload the structure from the buffer.

Note: These structures are initially allocated and created using ( vs_ ) utility functions. The 			saving and reloading of these structures eliminates having to create the structures with each use.

Note: These buffer pointer indexes are not the same as those used by vs_allocate

vs_set_ca_param:	sets the parameter value for the channels DX_CAP call analysis structure

Dialogic	none	utility function normally implemented in C code

SVServer	vs_set_ca_param, param, value

where	param = 			value =	-see Dialogic Help for call analysis values

0	vs_ca_nbrdna		# of rings before no answer.

1	vs_ca_stdely		Delay after dialing before analysis.

2	vs_ca_cnosig		Duration of no signal time out delay.

3	vs_ca_lcdly		Delay after dial before lc drop connect

4	vs_ca_lcdly1		Delay after lc drop con. before msg.

5	vs_ca_hedge		Edge of answer to send connect message.

6	vs_ca_cnosil		Initial continuous noise timeout delay.

7	vs_ca_lo1tola		acceptable pos. dev of short low sig.

8	vs_ca_lo1tolb		acceptable neg. dev of short low sig.

9	vs_ca_lo2tola		% acceptable pos. dev of long low sig.

10	vs_ca_lo2tolb		% acceptable neg. dev of long low sig.

11	vs_ca_hi1tola		% acceptable pos. dev of high signal.

12	vs_ca_hi1tolb		% acceptable neg. dev of high signal.

13	vs_ca_lo1bmax		Maximum interval for shrt low for busy.

14	vs_ca_lo2bmax		Maximum interval for long low for busy.

15	vs_ca_hi1bmax		Maximum interval for 1st high for busy

16	vs_ca_nsbusy		Num. of highs after nbrdna busy check.

17	vs_ca_logltch		Silence deglitch duration.

18	vs_ca_higltch		Non-silence deglitch duration.

19	vs_ca_lo1rmax		Max. short low  dur. of double ring.

20	vs_ca_lo2rmin		Min. long low  dur. of double ring.

21	vs_ca_intflg		Operator intercept mode.

22	vs_ca_intfltr		Minimum signal to qualify freq. detect.

23	vs_rfu1			reserved for future use

24	vs_rfu2			reserved for future use

25	vs_rfu3			reserved for future use

26	vs_rfu4			reserved for future use

27	vs_ca_hisiz		Used to determine which lowmax to use.

28	vs_ca_alowmax		Max. low before con. if high >hisize.

29	vs_ca_blowmax		Max. low before con. if high <hisize.

30	vs_ca_nbrbeg		Number of rings before analysis begins.

31	vs_ca_hi1ceil		Maximum 2nd high dur. for a retrain.

32	vs_ca_lo1ceil		Maximum 1st low dur. for a retrain.

33	vs_ca_lowerfrq		Lower allowable frequency in hz.

34	vs_ca_upperfrq		Upper allowable frequency in hz.

35	vs_ca_timefrq		Total duration of good signal required.

36	vs_ca_rejctfrq		Allowable % of bad signal.

37	vs_ca_maxan		Maximum duration of answer.

38	vs_ca_ansrdgl		Silence deglitching value for answer.

39	vs_ca_mxtimefrq	max time for 1st freq to remain in bounds

40	vs_ca_lower2frq		lower bound for second frequency

41	vs_ca_upper2frq		upper bound for second frequency

42	vs_ca_time2frq		min time for 2nd freq to remains in bounds

43	vs_ca_mxtime2frq	max time for 2nd freq to remain in bounds

44	vs_ca_lower3frq		lower bound for third frequency

45	vs_ca_upper3frq		upper bound for third frequency

46	vs_ca_time3frq		min time for 3rd freq to remains in bounds

47	vs_ca_mxtime3frq	max time for 3rd freq to remain in bounds

48	vs_ca_dtn_pres		Length of a valid dial tone (def=1sec)

49	vs_ca_dtn_npres	Max time to wait for dial tone (def=3sec)

50	vs_ca_dtn_deboff	The dialtone off debouncer (def=100ms)

51	vs_ca_pamd_failtime	Wait for AMD/PVD after cadence break(default=4sec)

52	vs_ca_pamd_minring	min allowable ring duration (def=1.9sec)

53	vs_ca_pamd_spdval	char Set to 2 selects quick decision (def=1)

54	vs_ca_pamd_qtemp	char The Qualification template to use for PAMD

55	vs_ca_noanswer		time before no answer after 1st ring default=30

56	vs_ca_maxintering	Max inter ring delay before connect (8 sec)

vs_set_devices:	sets the array of devices used with sr_waitevtEx

Dialogic	none	utility function normally implemented in C code

SVServer	vs_set_devices, dev, dev, ... dev

where	dev = channel device number returned when the line device is opened

Note: Sending vs_set_devices with no devices clears the devices array.

vs_set_dv_tpt:		sets up the dv_tpt structure array element

Dialogic	none	utility function normally implemented in C code

SVServer	vs_set_dv_tpt, index, type, termno, length, flags, data, rfu

where	where	index = tpt stucture array index value 0, 1, 2, ...

-see Dialogic Help file for DV_TPT structure values

Note: Type must not be a linked list IO_LINK.

vs_set_dv_tpt_size:	allocates space for the dv_tpt structure

Dialogic	none	utility function normally implemented in C code

SVServer	vs_set_dv_tpt_size, size

where	size = the number of dv-tpt structure entries

Note: dx_clrtpt allocates the dv_tpt structure as well

vs_set_dx_iott:		sets up the dx_iott structure array element

Dialogic	none	utility function normally implemented in C code

SVServer	vs_set_dx_iott, index, type, rfu, fhandle, bufno, offset, length

where	where	index = iott stucture array index value 0, 1, 2, ...

bufno = character memory buffer pointer index allocated by vs_allocate if IO_MEM is set in type

fhandle = file handle returned by dx_fileopen if IO_DEV is set in type, otherwise 0

-see Dialogic Help file for DX_IOTT structure values

Note: Type must not be a linked list IO_LINK.

vs_set_dx_iott_size:	allocates space for the dx_iott structure

Dialogic	none	utility function normally implemented in C code

SVServer	vs_set_dx_iott_size, size

where	size = the number of dx_iott structure entries

vs_set_dx_svcb:	sets up one of twenty dx_svcb structure entries for the channel or clears the array

Dialogic	none	utility function normally implemented in C code

SVServer	vs_set_dx_svcb, index, type, adjsize, digit, digtype

or	vs_set_dx_svcb, clearall	clears the entire 20 entry array if clearall is = 1 or nonzero

where	index = array index value 0 - 19

-see Dialogic Help file for DX_SVCB structure values

vs_set_dx_svmt:	sets up the dx_svmt structure

Dialogic	none	utility function normally implemented in C code

SVServer	vs_set_dx_svmt, value1, value2, value3, ... value21

where	-see Dialogic Help file for DX_SVMT structure values

vs_set_dx_xpb:	sets up the dx_xpb structure

Dialogic	none	utility function normally implemented in C code

SVServer	vs_set_dx_xpb, wFileFormat, wDataFormat, nSamplesPerSec, nBitsPerSample

where	-see Dialogic Help file for DX_XPB structure values

vs_set_timer:		sets interal timing functions in the SVServer

Dialogic none	utility function normally implemented in C code

SVServer	vs_set_timer, interval, repeat, show

where	interval = the timer interval in milliseconds

repeat = the timer count to wait before sending repeat status notification

show = the timer count SVServer window stays open

default	vs_set_timer, 250, 2, 8

Note: The user should not change these values unless advised by the developer.

Dialogic Voice API Function Definitions

Please consult the Dialogic Help files for information on the input parameters and return values for these functions.  Each line, or channel, has it's own set of structures with pointers set automatically but the user must supply parameters to fill the structures via the utility functions provided above or Dialogic functions below.  The user must also, allocate space for each channel's DV_TPT and DX_IOTT structures when called for.  They may be allocated via the vs_set_dv_tpt_size and vs_set_dx_iott_size functions respectfully. Channel device handles need not be included unless indicated by the SVServer command shown in bold typeface.

ag_getctinfo:

Dialogic		int ag_getctinfo( int chdev, CT_DEVINFO *ct_devinfop );

SVServer	ag_getctinfo

ag_getxmitslot:

Dialogic		int ag_getxmitslot( int chdev, SC_TSINFO *sc_tsinfop );

SVServer	ag_getxmitslot

ag_listen:

Dialogic		int ag_listen( int chdev, SC_TSINFO *sc_tsinfop );

SVServer	ag_listen

ag_unlisten:

Dialogic		int ag_unlisten( int chdev );

SVServer	ag_unlisten

ATDV_ERRMSGP:	returns an ASCIIZ error message string

Dialogic	char *ATDV_ERRMSGP( long ddd );

SVServer	ATDV_ERRMSGP

ATDV_IOPORT:	returns the port number

Dialogic	long ATDV_IOPORT( long ddd );

SVServer	ATDV_IOPORT

ATDV_IRQNUM:	returns the interrupt (irq) number

Dialogic	long ATDV_IRQNUM( long ddd );

SVServer	ATDV_IRQNUM

ATDV_LASTERR:	returns a long that indicates the error that occurred

Dialogic	long ATDV_LASTERR( long ddd );

SVServer	ATDV_LASTERR

ATDV_NAMEP:		returns an ASCIIZ string which contains the device name

Dialogic	char *ATDV_NAMEP( long ddd );

SVServer	ATDV_NAMEP

ATDV_SUBDEVS:	returns the number of subdevices for the device

Dialogic	long ATDV_SUBDEVS( long ddd );

SVServer	ATDV_SUBDEVS

ATDX_ANSRSIZ:	returns the duration of the answer

Dialogic		long ATDX_ANSRSIZ( int ddd );

SVServer	ATDX_ANSRSIZ

ATDX_BDNAMEP:	returns the board name

Dialogic		char * ATDX_BDNAMEP( int ddd );

SVServer	ATDX_BDNAMEP

ATDX_BDTYPE:	returns the board type

Dialogic		long ATDX_BDTYPE( int ddd );

SVServer	ATDX_BDTYPE

ATDX_BUFDIGS:	returns the number of uncollected digits

Dialogic		long ATDX_BUFDIGS( int ddd );

SVServer	ATDX_BUFDIGS

ATDX_CHNAMES:	returns an array of channel names

Dialogic		char ** ATDX_CHNAMES( int ddd );

SVServer	ATDX_CHNAMES, index

where	index = the channel number of the channel on the board, e.g. = 0, 1, 2, 3

ATDX_CHNUM:		returns the channel number

Dialogic		long ATDX_CHNUM( int ddd );

SVServer	ATDX_CHNUM

ATDX_CONNTYPE:	returns the connection type

Dialogic		long ATDX_CONNTYPE( int ddd );

SVServer	ATDX_CONNTYPE

ATDX_CPERROR:	returns the error

Dialogic		long ATDX_CPERROR(  int ddd );

SVServer	ATDX_CPERROR

ATDX_CPTERM:	returns the last Call Analysis termination

Dialogic		long ATDX_CPTERM(   int ddd );

SVServer	ATDX_CPTERM

ATDX_CRTNID:		returns the tone identifier

Dialogic		long ATDX_CRTNID( int ddd );

SVServer	ATDX_CRTNID

ATDX_DTNFAIL:	returns the code for the dial tone that failed to appear

Dialogic		long ATDX_DTNFAIL( int ddd );

SVServer	ATDX_DTNFAIL

ATDX_DBMASK:

Dialogic		long ATDX_DBMASK( int ddd );

SVServer	ATDX_DBMASK

ATDX_DEVTYPE:	returns the device type

Dialogic		long ATDX_DEVTYPE( int ddd );

SVServer	ATDX_DEVTYPE

ATDX_DIGBUFMODE:

Dialogic		long ATDX_DIGBUFMODE( int ddd );

SVServer	ATDX_DIGBUFMODE

ATDX_EVTCNT:

Dialogic		long ATDX_EVTCNT( int ddd );

SVServer	ATDX_EVTCNT

ATDX_FRQDUR:	returns the first frequency duration in 10ms units

Dialogic		long ATDX_FRQDUR( int ddd );

SVServer	ATDX_FRQDUR

ATDX_FRQDUR2:	returns the second frequency duration in 10ms units

Dialogic		long ATDX_FRQDUR2(  int ddd );

SVServer	ATDX_FRQDUR2

ATDX_FRQDUR3:	returns the third frequency duration in 10ms units

Dialogic		long ATDX_FRQDUR3( int ddd );

SVServer	ATDX_FRQDUR3

ATDX_FRQHZ:		returns the frequency of the answered signal in hz

Dialogic		long ATDX_FRQHZ( int ddd );

SVServer	ATDX_FRQHZ

ATDX_FRQHZ2:	returns the frequency of the second detected in hz

Dialogic		long ATDX_FRQHZ2( int ddd );

SVServer	ATDX_FRQHZ2

ATDX_FRQHZ3:	returns the frequency of the third detected in hz

Dialogic		long ATDX_FRQHZ3( int ddd );

SVServer	ATDX_FRQHZ3

ATDX_FRQOUT:	returns the percentage of a single tone frequency

Dialogic		long ATDX_FRQOUT( int ddd );

SVServer	ATDX_FRQOUT

ATDX_FWVER:		returns the version number of the D/4x firmware

Dialogic		long ATDX_FWVER( int ddd );

SVServer	ATDX_FWVER

ATDX_HOOKST:	returns the current hook state

Dialogic		long ATDX_HOOKST( int ddd );

SVServer	ATDX_HOOKST

ATDX_LINEST: 	returns a bitmapped representation of the activity

Dialogic		long ATDX_LINEST( int ddd );

SVServer	ATDX_LINEST

ATDX_LONGLOW:	returns the duration of the longer silence

Dialogic		long ATDX_LONGLOW( int ddd );

SVServer	ATDX_LONGLOW

ATDX_NUMCHAN:

Dialogic		long ATDX_NUMCHAN( int ddd );

SVServer	ATDX_NUMCHAN

ATDX_PHYADDR:	returns the physical address of the board

Dialogic		long ATDX_PHYADDR( int ddd );

SVServer	ATDX_PHYADDR

ATDX_PRODUCT:

Dialogic		long ATDX_PRODUCT( int ddd );

SVServer	ATDX_PRODUCT

ATDX_RINGCNT:	returns the ring count

Dialogic		long ATDX_RINGCNT( int ddd );

SVServer	ATDX_RINGCNT

ATDX_SHORTLOW:	returns the duration of the shorter silence

Dialogic		long ATDX_SHORTLOW( int ddd );

SVServer	ATDX_SHORTLOW

ATDX_SIZEHI:		returns the duration of the initial non-silence

Dialogic		long ATDX_SIZEHI( int ddd );

SVServer	ATDX_SIZEHI

ATDX_STATE:		returns the current state of the channel

Dialogic		long ATDX_STATE( int ddd );

SVServer	ATDX_STATE

ATDX_TERMMSK:	returns the channel's last termination bitmap

Dialogic		long ATDX_TERMMSK( int ddd );

SVServer	ATDX_TERMMSK

ATDX_TONEID:		returns the user-defined tone id

Dialogic		long ATDX_TONEID( int ddd );

SVServer	ATDX_TONEID

ATDX_TRCOUNT:	returns the number of the last play/record transfer count

Dialogic		long ATDX_TRCOUNT( int ddd );

SVServer	ATDX_TRCOUNT

dx_addspddig:		sets a DTMF digit to adjust speed

Dialogic		int dx_addspddig( int chdev, char digit, short adjval );

SVServer	dx_addspddig, digit, adjval

dx_addtone:		adds the tone

Dialogic		int dx_addtone( int chdev, unsigned char digit, unsigned char digtype );

SVServer	dx_addtone, digit, digtype

dx_addvoldig:		sets a DTMF digit to immediately adjust volume

Dialogic		int dx_addvoldig( int chdev, char digit, short adjval );

SVServer	dx_addvoldig, digit, adjval

dx_adjsv:		adjusts the speed or the volume

Dialogic		int dx_adjsv( int chdev, unsigned short tabletype, unsigned short action, unsigned short 			adjsize );

SVServer	dx_adjsv, tabletype, action, adjsize

dx_blddt:		defines a simple dual frequency tone

Dialogic		int dx_blddt( unsigned int id,   unsigned freq1, unsigned int fq1dev, unsigned int freq2, 			unsigned int fq2dev, unsigned int mode );

SVServer	dx_blddt, id, freq1, fq1dev, freq2, fq2dev, mode

dx_blddtcad:		defines a simple dual frequency cadence tone

Dialogic		int dx_blddtcad( unsigned int ToneID, unsigned int freq1,   unsigned int fq1dev, unsigned 			int freq2, unsigned int fq2dev, unsigned int OnTime,  unsigned int OnTimeDev, unsigned int 			OffTime, unsigned int OffTimeDev, unsigned int repcnt );

SVServer	dx_blddtcad, ToneID, freq1, fq1dev, feq2, fq2dev, OnTime, OnTimeDev, OffTime, 			OffTimeDev, repcnt

dx_bldst:		defines a simple single frequency tone

Dialogic		int dx_bldst( unsigned int ToneID, unsigned int freq, unsigned int fqdev, unsigned int 			mode );

SVServer	dx_bldst, ToneID, freq, fqdev, mode

dx_bldstcad:		defines a simple single frequency cadence tone

Dialogic		int dx_bldstcad( unsigned int ToneID, unsigned int freq, unsigned int fqdev, unsigned int 			OnTime,  unsigned int OnTimeDev, unsigned int OffTime, unsigned int OffTimeDev, unsigned 			int repcnt );

SVServer	dx_bldstcad, ToneId, freq, fqdev, OnTime, OnTimeDev, OffTime, OffTimeDev, 			repcnt

dx_bldtngen:		sets up the tone generation template

Dialogic		void dx_bldtngen( TN_GEN *tngenp, unsigned short freq1, unsigned short freq2,short 			amplitude1, short amplitude2, short duration );

SVServer	dx_bldtngen, freq1, freq2, amplitude1, amplitude2, duration

dx_chgdur:		alters the standard definition of the duration component

Dialogic		int dx_chgdur( int typetype, int on, int ondv, int off, int offdv );

SVServer	dx_chgdur, typetype, on, ondv, off, offdv

dx_chgfreq:		changes the standard definition for one of the PerfectCall Call Analysis tones

Dialogic		int dx_chgfreq( int tonetype, int fq1, int dv1, int fq2, int dv2 );

SVServer	dx_chgfreq, tonetype, fq1, dv1, fq2, dv2

dx_chgrepcnt:		changes the standard definition for one of the PerfectCall Call Analysis tones

Dialogic		int dx_chgrepcnt( int tonetype, int repcount );

SVServer	dx_chgrepcnt, tonetype, repcount

dx_close:		closes Dialogic devices

Dialogic		int dx_close( int dev );

SVServer	dx_close

dx_clrcap:		clears all the fields in a DX_CAP structure

Dialogic		void dx_clrcap( DX_CAP *capp );

SVServer	dx_clrcap

or	dx_clrcap, NULL

where	NULL sets the capp pointer to NULL

dx_clrdigbuf:		clears the digits present in the firmware digit buffer

Dialogic		int dx_clrdigbuf( int chdev );

SVServer	dx_clrdigbuf

dx_clrsvcond:		clears any speed or volume adjustment conditions

Dialogic		int dx_clrsvcond( int chdev );

SVServer	dx_clrsvcond

dx_clrtpt:		allocates and clears the DV_TPT structure

Dialogic		int dx_clrtpt( DV_TPT *tptp, int size );

SVServer	dx_clrtpt, size

dx_deltones:		removes all user-defined tones

Dialogic		int dx_deltones( int chdev );

SVServer	dx_deltones

dx_dial:		dials an ASCIIZ string

Dialogic		int dx_dial( int chdev, const char *dialstrp, const DX_CAP *capp, unsigned short mode );

SVServer	dx_dial, dialstrp, mode

dx_distone:		disables the detection of TONE ON

Dialogic		int dx_distone( int chdev, int toneID, int evtMask );

SVServer	dx_distone, toneID, evtMask

dx_enbtone:		enables the detection of TONE ON

Dialogic		int dx_enbtone( int chdev, int ToneID, int evtMask );

SVServer	dx_enbtone, ToneID, evtMask

dx_filechsize:

Dialogic		int dx_filechsize ( int handle, long size );

SVServer	dx_filechsize, handle, size

dx_filecommit:

Dialogic		int dx_filecommit ( int handle );

SVServer	dx_filecommit, handle

dx_fileclose:		closes the file associated with the handle

Dialogic		int dx_fileclose ( int handle );

SVServer	dx_fileclose, handle

dx_filedup:

Dialogic		int dx_filedup ( int handle );

SVServer	dx_filedup, handle

dx_filedup2:

Dialogic		int dx_filedup2 ( int handle1, int handle2 );

SVServer	dx_filedup2, handle1, handle2

dx_fileerrno:

Dialogic		int dx_fileerrno( void );

SVServer	dx_fileerrno

dx_filefilelength:

Dialogic		long dx_filefilelength( int handle );

SVServer	dx_filefilelength, handle

dx_filefstat:

Dialogic		int dx_filefstat ( int handle, struct _stat * buffer );

SVServer	function not implemented

dx_fileisatty:

Dialogic		int dx_fileisatty( int handle );

SVServer	dx_fileisatty, handle

dx_filelocking:

Dialogic		int dx_filelocking ( int handle, int mode, long bytes );

SVServer	dx_filelocking, handle, mode, bytes

dx_fileopen:		opens the file specified by filep

Dialogic		int dx_fileopen ( const char * filep, int flags, int pmode );

SVServer	dx_fileopen, filepath, flags, pmode

dx_fileopen_osfhandle:

Dialogic		int dx_fileopen_osfhandle( long osfhandle, int flags );

SVServer	dx_fileopen_osfhandle, osfhandle, flags

dx_fileread:		returns the number of bytes read by the application

Dialogic		int dx_fileread ( int handle, void * buffer, unsigned int count );

SVServer	dx_fileread, handle, bufptr_index, count

where	handle is the file handle returned by dx_fileopen

bufptr_index is the index of a buffer allocated by vs_allocate

count is the number of bytes to read

dx_fileseek:		moves the file pointer associated with handle

Dialogic		long dx_fileseek ( int handle, long offset, int origin );

SVServer	dx_fileseek, handle, offset, origin

dx_filesetmode:

Dialogic		int dx_filesetmode( int handle, int mode );

SVServer	dx_filesetmode, handle, mode

dx_filesopen:

Dialogic		int dx_filesopen ( const char * filep, int oflag, int shflag, int pmode );

SVServer	dx_filesopen, filepath, oflag, shflag, pmode

dx_filestat:

Dialogic		int dx_filestat ( const char * path, struct _stat * buffer );

SVServer	function not implemented

dx_filetell:

Dialogic		long dx_filetell ( int handle );

SVServer	dx_filetell, handle

dx_fileumask:

Dialogic		int dx_fileumask ( int pmode );

SVServer	dx_fileumask, pmode

dx_filewrite:		writes count byes from buffer into the file associated with handle

Dialogic		int dx_filewrite ( int handle, void * buffer, unsigned int count );

SVServer	dx_filewrite, handle, bufptr_index, count

where	handle is the file handle returned by dx_fileopen

bufptr_index is the index of a buffer allocated by vs_allocate

count is the number of bytes to write

dx_getctinfo:

Dialogic		int dx_getctinfo( int chDev, CT_DEVINFO *ct_devinfop );

SVServer	dx_getctinfo

dx_getcursv:		returns the specified channel's current speed

Dialogic		int dx_getcursv( int chdev, int *curvolp, int *curspeedp );

SVServer	dx_getcursv

dx_getdig:		initiates the collection of digits

Dialogic		int dx_getdig( int chdev, const DV_TPT *tptp, DV_DIGIT *digitp, unsigned short mode );

SVServer	dx_getdig, mode

dx_getevt:		used to synchronously monitor channels

Dialogic		int dx_getevt( int chdev, DX_EBLK *eblkp, int timeout );

SVServer	dx_getevt, timeout

dx_getparm:		obtains the current parameter settings

Dialogic		int dx_getparm( int dev, unsigned long parm, void * valuep );

SVServer	dx_getparm, parm

or	dx_getparm, dev, parm

dx_getr2mf:

Dialogic		int dx_getr2mf( int Chdev, const char *src1_ptr, unsigned short src1_len, const char 			*src2_ptr, unsigned short src2_len,char *dest_ptr, unsigned short dest_len, unsigned short mode 			);

SVServer	dx_getr2mf, src1index, src1len, src2index, src2len, dstindex, dstlen, mode

dx_getsvmt:		returns the contents of the Speed or the Volume Modification Table

Dialogic		int dx_getsvmt( int ChDev, unsigned short tableType, DX_SVMT *svmtp );

SVServer	dx_getsvmt, tableType

dx_getxmitslot:

Dialogic		int dx_getxmitslot( int chDev, SC_TSINFO *sc_tsinfop);

SVServer	dx_getxmitslot

dx_gtcallid:

Dialogic		int dx_gtcallid( int Chdev, unsigned char *bufp );

SVServer	dx_gtcallid

dx_gtextcallid:

Dialogic		int dx_gtextcallid( int Chdev, int infotype, unsigned char *bufp );

SVServer	dx_gtextcallid, infotype

dx_gtsernum:

Dialogic		int dx_gtsernum ( int devd, int subfcn, void *pBuff );

SVServer	dx_gtsernum, subfcn

dx_initcallp:		initializes and activates PerfectCall Call Analysis

Dialogic		int dx_initcallp( int chdev );

SVServer	dx_initcallp

dx_listen:

Dialogic		int dx_listen( int chDev, SC_TSINFO *sc_tsinfop );

SVServer	dx_listen

dx_open:		opens a voice device

Dialogic		int dx_open( const char *namep, int oflags );

SVServer	dx_open 			Note: No device name or flags is necessary

or	dx_open, namep			but optionally include name.

where	namep = device name e.g. "dxxxB1C1"

returns 	the device handle in RTN=

if ( namep, oflags ) is not used automatic name assignment is as follows:

Line1 ="dxxxB1C1",	Line2 ="dxxxB1C2",	Line3 ="dxxxB1C3",	Line4 ="dxxxB1C4",

Line5 ="dxxxB2C1",	Line6 ="dxxxB2C2",	Line7 ="dxxxB2C3",	Line8 ="dxxxB2C4",

Line9 ="dxxxB3C1",	Line10="dxxxB3C2",	Line11="dxxxB3C3",	Line12="dxxxB3C4",

Line13="dxxxB4C1",	Line14="dxxxB4C2",	Line15="dxxxB4C3",	Line16="dxxxB4C4",

Line17="dxxxB5C1",	Line18="dxxxB5C2",	Line19="dxxxB5C3",	Line20="dxxxB5C4",

Line21="dxxxB6C1",	Line22="dxxxB6C2",	Line23="dxxxB6C3",	Line24="dxxxB6C4",

Line25="dxxxB7C1",	Line26="dxxxB7C2",	Line27="dxxxB7C3",	Line28="dxxxB7C4",

Line29="dxxxB8C1",	Line30="dxxxB8C2",	Line32="dxxxB8C3",	Line32="dxxxB8C4",

dx_play:		plays recorded voice data

Dialogic		int dx_play( int ChDev, DX_IOTT *iottp, const DV_TPT *tptp, unsigned short mode );

SVServer	dx_play, mode

where	DX_IOTT and DV_TPT must be setup by vs_set_dx_iott and vs_set_dv_tpt prior to running

mode = synchronous or asynchronous

dx_playf:		permits you to synchronously play voice data

Dialogic		int dx_playf(int ChDev, const char *fNamep, const DV_TPT *tptp, unsigned short mode 			);

SVServer	dx_playf, fNamepath, mode

where	DV_TPT must be setup by vs_set_dv_tpt prior to running

fNamepath =  file path

mode = synchronous

dx_playiottdata:

Dialogic		int dx_playiottdata( int ChDev, DX_IOTT * iottp, const DV_TPT * tptp, const DX_XPB * 			xpbp, unsigned short mode );

SVServer	dx_playiottdata, mode

where	DX_IOTT, DV_TPT & DX_XPB must be setup by vs_set_dx_iott, vs_set_dv_tpt & 			vs_set_dx_xpb prior to running

mode = synchronous or asynchronous

dx_playtone:		plays the tone defined by the TN_GEN template

Dialogic		int dx_playtone( int ChDev, const TN_GEN *tngenp, const DV_TPT *tptp, int mode );

SVServer	dx_playtone, mode

dx_playvox:		plays voice data stored in a single VOX file

Dialogic		int dx_playvox( int ChDev, const char * fNamep, const DV_TPT *tptp, const DX_XPB * 			xpbp,unsigned short mode );

SVServer	dx_playvox, fNamepath, mode

or	dx_playvox, fNamepath, NULL, mode

where	DV_TPT and DX_XPB must be setup by vs_set_dv_tpt and vs_set_dx_xpb

fNamepath =  file path

mode = synchronous

dx_playwav:		plays voice data stored in a single WAVE file

Dialogic		int dx_playwav( int ChDev, const char * fNamep, const DV_TPT *tptp, unsigned short 			mode );

SVServer	dx_playwav, fNamepath, mode

where	DV_TPT must be setup by vs_set_dv_tpt prior to running

fNamepath =  file path

mode = synchronous

dx_rec:			records voice data from a single channel

Dialogic		int dx_rec(int ChDev, DX_IOTT *iottp, const DV_TPT *tptp, unsigned short mode );

SVServer	dx_rec, mode

where	DX_IOTT and DV_TPT must be setup by vs_set_dx_iott and vs_set_dv_tpt prior to running

mode = synchronous or asynchronous

dx_recf:		permits voice data to be recorded

Dialogic		int dx_recf( int ChDev, const char *fNamep, const DV_TPT *tptp, unsigned short mode );

SVServer	dx_recf, fNamepath, mode

where	DV_TPT must be setup by vs_set_dv_tpt prior to running

fNamepath =  file path

mode = synchronous

dx_reciottdata:		records voice data to multiple destinations

Dialogic		int dx_reciottdata( int ChDev,DX_IOTT * iottp, const DV_TPT *tptp, DX_XPB * 			xpbp,unsigned short mode );

SVServer	dx_reciottdata, mode

where	DX_IOTT, DV_TPT & DX_XPB must be setup by vs_set_dx_iott, vs_set_dv_tpt & 			vs_set_dx_xpb prior to running

mode = synchronous or asynchronous

dx_recvox:		records voice data to a single VOX file

Dialogic		int dx_recvox( int ChDev, const char * fNamep, const DV_TPT *tptp, DX_XPB * 			xpbp,unsigned short mode );

SVServer	dx_recvox, fNamepath, mode

or	dx_recvox, fNamepath, NULL, mode

where	DV_TPT and DX_XPB must be setup by vs_set_dv_tpt and vs_set_dx_xpb 			fNamepath =  file path

mode = synchronous

dx_recwav:		records voice data to  a single WAVE file

Dialogic		int dx_recwav( int ChDev, const char * fNamep, const DV_TPT *tptp, DX_XPB * 			xpbp,unsigned short mode );

SVServer	dx_recwav, fNamepath, mode

or	dx_recwav, fNamepath, NULL, mode

where	DV_TPT and DX_XPB must be setup by vs_set_dv_tpt and vs_set_dx_xpb

fNamepath =  file path

mode = synchronous

dx_route:

Dialogic		int dx_route( int BdDev, int Chan, int TimeSlot );

SVServer	dx_route, BdDev, Chan, TimeSlot

dx_routerxtx:

Dialogic		int dx_routerxtx( int BdDev, int Chan, int RxTimeSlot, int TxTimeSlot );

SVServer	dx_routerxtx, BdDev, Chan, RxTimeSlot, TxTimeSlot

dx_sendevt:

Dialogic		int dx_sendevt( int devd, unsigned long evttype, void *evtdatap, short evtlen, unsigned 			short flags );

SVServer	function not implemented

dx_set_lasterr:

Dialogic		int dx_set_lasterr( int ddd, long error );

SVServer	dx_set_lasterr, error

dx_setdigbuf:		sets the digit buffering mode

Dialogic		int dx_setdigbuf(chdev,mode)

SVServer	function not implemented, not found in source library

dx_setdigtyp:		controls the types of digits

Dialogic		int dx_setdigtyp( int ChDev, unsigned short DigitMask );

SVServer	dx_setdigtyp, DigitMask

dx_setevtmsk:		enables detection of the Call Status Transition (CST) event

Dialogic		int dx_setevtmsk( int ChDev, unsigned int mask );

SVServer	dx_setevtmsk, mask

dx_setgtdamp:		sets up the amplitudes

Dialogic		void dx_setgtdamp( short Min1, short Max1, short Min2, short Max2 );

SVServer	dx_setgtdamp, Min1, Max1, Min2, Max2

dx_sethook:		provides control of the hookswitch state

Dialogic		int dx_sethook( int ChDev, int HookState, unsigned short mode );

SVServer	dx_sethook, HookState, mode

dx_setparm:		allows you to set the physical parameters

Dialogic		int dx_setparm( int Dev, unsigned long parm, const void * valuep );

SVServer	dx_setparm, parm, value

or	dx_setparm, dev, parm, value

dx_setrings:

Dialogic		int dx_setrings( int devd, unsigned int nrings );

SVServer	dx_setrings, nrings

dx_setsvcond:		sets adjustments and adjustment conditions

Dialogic		int dx_setsvcond( int ChDev, unsigned short numBlks, const DX_SVCB *svcbp );

SVServer	dx_setsvcond, numBlks

dx_setsvmt:		updates the speed or volume

Dialogic		int dx_setsvmt( int ChDev, unsigned short TableType, const DX_SVMT *svmtp, 			unsigned short flag );

SVServer	dx_setsvcond, TableType, flag

dx_setuio:		allows an application to install user-defined read(), write(), and lseek() functions

Dialogic		int dx_setuio( DX_UIO uio );

SVServer	function not implemented

dx_stopch:		forces termination of currently active I/O functions

Dialogic		int dx_stopch( int ChDev, unsigned short mode );

SVServer	dx_stopch, mode

dx_TSFStatus:

Dialogic		int dx_TSFStatus ( void );

SVServer	dx_TSFStatus

dx_unlisten:

Dialogic		int dx_unlisten( int chDev );

SVServer	dx_unlisten

dx_wink:

Dialogic		int dx_wink( int ChDev, unsigned short mode );

SVServer	dx_wink, mode

dx_wtcallid:

Dialogic		int dx_wtcallid( int Chdev, int nrings, int timeout,unsigned char *bufp );

SVServer	dx_wtcallid, nrings, timeout

dx_wtring:		waits for a specified number of rings

Dialogic		int dx_wtring( int ChDev, int numRings, int HookState, int timeout );

SVServer	dx_wtring, numRings, HookState, timeout

Dialogic SRL API Function Definitions

Please consult the Dialogic Help files for information on the input parameters and return values for these functions.

sr_dishdlr:		disables the handler

Dialogic		long sr_dishdlr( long ddd, unsigned long event_type, long (*hdlr)( void *parm ) );

SVServer	function not implemented

sr_enbhdlr:		enables the handler function

Dialogic		long sr_enbhdlr( long ddd, unsigned long event_type, long (*hdlr)( void *parm ) );

SVServer	function not implemented

sr_getboardcnt:	retrieve the count of boards of a particular type

Dialogic		long sr_getboardcnt( char * brdname, int * brdcnt );

SVServer	sr_getboardcnt, brdname, brdcnt

sr_getevtdatap:	returns the address of the variable data block

Dialogic		void *sr_getevtdatap( unsigned long eventhandle );

SVServer	sr_getevtdatap, eventhandle

where	eventhandle is returned when sr_waitevtEx completes

returns	in RSL: "EVT=%d, DTA=%d"

sr_getevtdev:		returns the Dialogic device handle

Dialogic		long sr_getevtdev( unsigned long event );

SVServer	sr_getevtdev, event

sr_getevtlen:		returns the length of the variable data

Dialogic		long sr_getevtlen( unsigned long eventhandle );

SVServer	sr_getevtlen, eventhandle

where	eventhandle is returned when sr_waitevtEx completes

sr_getevttype:		returns the event type for the current event

Dialogic		long sr_getevttype( unsigned long eventhandle );

SVServer	sr_getevttype, eventhandle

where	eventhandle is returned when sr_waitevtEx completes

sr_getparm:		returns the value of an SRL parameter

Dialogic		long sr_getparm( long ddd, long parmid, void* valuep );

SVServer	sr_getparm, ddd, parmid

sr_hold:

Dialogic		long sr_hold();

SVServer	sr_hold

sr_NotifyEvent:		sends event notification to a window

Dialogic		void sr_NotifyEvent( HWND handle, unsigned int message, unsigned int flag );

SVServer	sr_NotifyEvent, message, flag

returns	normal function status line e.g. LNE=2, FNO=148, STA=4, ERR=0, RTN=0, RSL:

on completion runs the functions sr_getevtdev(); sr_getevttype();

requires	separate text control and separate item SVSCallback for events

returns 	LNE=%d, HDL=%d, DEV=%d, TYP=%d as eventline, eventhandle, eventdev, eventtype

in the separate text control

separate additional text control and topic receives the event status returned

initialize		Text2.LinkMode = 0	'	break connection

Text2.LinkTopic = "SVServer|SVSFunction"

Text2.LinkItem = "SVSCallback"

Text2.LinkMode = 3	'	establish connection

This link responds only to the "stop", "status" and "stopch" commands.

sr_release:

Dialogic		long sr_release();

SVServer	sr_release

sr_setlasterr:

Dialogic		void sr_setlasterr( long ddd, long error );

SVServer	function not implemented

sr_setparm:		allows the application to change the value

Dialogic		long sr_setparm( long ddd, long parmid, void* valuep );

SVServer	sr_setparm, ddd, parmid, value

sr_waitevt:		waits for events

Dialogic		long sr_waitevt( long tmout );

SVServer	sr_waitevt, timeout

returns	normal function status line e.g. LNE=2, FNO=152, STA=4, ERR=0, RTN=0, RSL:

on completion runs the functions sr_getevtdev(); sr_getevttype();

requires	separate text control and separate item SVSCallback for events

returns 	LNE=%d, HDL=%d, DEV=%d, TYP=%d as eventline, eventhandle, eventdev, eventtype

in the separate text control

separate additional text control and topic receives the event status returned

initialize		Text2.LinkMode = 0	'	break connection

Text2.LinkTopic = "SVServer|SVSFunction"

Text2.LinkItem = "SVSCallback"

Text2.LinkMode = 3	'	establish connection

This link responds only to the "stop", "status" and "stopch" commands.

sr_waitevtEx:		wait for events on certain devices

Dialogic		long sr_waitevtEx( long *handlep, int count, long tmout, long *evthp );

SVServer	sr_waitevtEx, count, tmout

returns	normal function status line e.g. LNE=2, FNO=153, STA=4, ERR=0, RTN=0, RSL:

on completion runs the functions sr_getevtdev(eventhandle); sr_getevttype(eventhandle);

requires	separate text control and separate item SVSCallback for events

returns 	LNE=%d, HDL=%d, DEV=%d, TYP=%d as eventline, eventhandle, eventdev, eventtype

in the separate text control

separate additional text control and topic receives the event status returned

initialize		Text2.LinkMode = 0	'	break connection

Text2.LinkTopic = "SVServer|SVSFunction"

Text2.LinkItem = "SVSCallback"

Text2.LinkMode = 3	'	establish connection

This link responds only to the "stop", "status" and "stopch" commands.