RAS Setup for NT 3.5 and Telesys (beta)

Written by Glenn K. Smith - 11/8/94

Microcomputer Technologies

University of Texas at Austin

(512) 471-3241

micro-gripe@uts.cc.utexas.edu

These are quick and dirty instructions for setting up

Remote Access Service (RAS) to work with Telesys.

Setup consists of three parts.  The first part involves adding

RAS services to your network control panel. The second part deals

with setting up the script and making a Phone Book entry for Telesys.  The

third is the script used for connecting to Telesys.

------

Network Control Panel

You'll need to have your NT 3.5 setup disks or CD (only way to go) for this

part if you haven't already installed RAS.

To install RAS, just click on the ADD SOFTWARE button and select Remote

Access Service.

To configure it, click on the CONFIGURE SOFTWARE button.  You'll need to click

on the ADD button to add a modem port.  If you've already added one, click

on CONFIGURE.

At the CONFIGURE dialog, you'll need to pick your modem type, Port Usage, and

settings.  If you are only using SLIP, you may want to select Dial Out Only for

the port usage.  The settings options are self explainitory.  You probably want

to select HARDWARE flow control if you have a high speed modem.

After configuring your modem, you'll need to configure your network settings

by clicking on the NETWORK button.  Most of these deal with DIAL IN calls.

At the network dialog, only select TCP/IP for both the Dial Out and Server

settings.  Telesys doesn't support the other modes, however, since the caller

is connection directly to you, your system can supply these protocols.

You'll need to configure your TCPIP settings too.  At this dialog, select

either Entire Network or This Computer only (your choice) for Allow Remote

TCP/IP clients to access.  I usually use This computer only.  Next, select

the DHCP to assign remote TCP/IP client address (or make up your own).  Since

they will be calling you, it's your decision.  The DHCP implies you have a

Domain Controller someplace.

Finally, select Allow Any authentification including clear text for the

Encryption settings.  Unless you're paranoid about incoming stuff, it probably

won't make much difference.  You have to use this option will calling Telesys

anyway.

That's about it for this part.  When you get back to the Network Settings

dialog, click ok and watch NT do it's thing.  You'll probably have to restart

afterwards.  I've noticed you have to restart each time you pick a different

modem or COM port.

------

Remote Access Service

Configuring RAS is the easist part of the whole process.  When you first start

up RAS, it will tell you to add a Phone Book entry.  If you already have one,

select it and click on the edit button.

At the Edit Phone Book Entry dialog, fill in the blanks. The Telesys number

is 475-9996.  Don't forget to add a *70, before the phone number to cancel

call waiting.

Click on the checkbox for Authenticate using current user name and password.

This will prevent you from having to type in the same information when you

dial out.  I believe this is only for connection to another NT system.

For Port, select your COM port where your modem is located.

You should have a button that reads Advanced >>, click on it to bring up

five buttons on the bottom of the dialog.  We only need the modem, network,

and security buttons.

Clickng on the modem button will allow you to select a baud rate, and select

what hardware features you want.  I recommend all three (if your modem supports

all the features).

Clicking the network button allows you to specifiy what PPP and SLIP settings

you want.  For our purposes, select PPP and uncheck NetBEUI and IPX.  Telesys

doesn't support the other modes.  Make sure Request LCP is selected (it will

work without it).  Next, click on TCP/IP settings and select Server assigned IP,

Server assigned name server,  VJ compression, and default gateway.  If you have

connection problems, try turning off VJ compression.

Finally, click the security button.  Make sure Accept any authentication

including clear text is selected.  Telesys only does validations this way.

Next, at the After Dialing, click on the down arrow and select Telesys.

That's all there is to it (whew!).  When you get back to the main RAS screen,

quit the program and follow the instructions for installing the script.  After

which, you can restart RAS and click on DIAL to call Telesys.  If any problems

occur, you'll get some sort of dialog on the screen.

------

SWITCH.INF

Copy the file SWITCH.INF to your /SYSTEM32/RAS directory. This will allow RAS to

recognize the new Telesys script.  The script is included incase you didn't

download the file.  Just edit your existing SWITCH.INF put in this script.

Whether you downloaded the file, or type it in manually, you'll need to

replacethe lines that read COMMAND=myusername<cr> and COMMAND=mypassword<cr>

with your Telesys information.  Be sure to have the <cr> at the end of the

lines or the script won't work properly.

What the script does is log onto Telesys and instead of going into SLIP mode,

it goes into PPP mode with SLIP only support.  Telesys only supports TCP/IP

with PPP.  The main reason I chose to do this is that I hate having to

enter my IP address in each time I called in with RAS.  SLIP support for NT

does not support BOOTP, while PPP does (go figure)!

If you make changes to the script, I believe you have to quit RAS and

restart it.  I think it reads all the .INF files into memory when you use

them and doesn't recognize the changes.

;-------------------------------------------------------------------------

; Sample switch entry

; -------------------

; Here is a sample entry for a switch that we need to talk to after the

; modems have connected:

;

; The sequence of operations is:

; 1) Wait for the "Login" prompt

; 2) Send in the login name and wait for "Password" prompt

; 3) Send Password and proceed.

;

;

;   [ExampleSwitch]

;

;   ; Wait until we get the "login:" prompt

;   COMMAND=

;   OK=<match>"Login"

;

;   ; Give the account and wait for "password:" prompt

;   COMMAND=MyLoginName<cr>

;   OK=<match>"Password"

;

;   ; Give the password and ignore response (we're done)

;   COMMAND=MyPassword<cr>

;   OK=<ignore>

;

;-------------------------------------------------------------------------

[Telesys]

; Written by Glenn K. Smith - 11/8/94

; Microcomputer Technologies

; University of Texas at Austin

; (512) 471-3241

; micro-gripe@uts.cc.utexas.edu

;

; If any errors occur, the script will abort.

;

; This is a simple script.  I hope to improve it once

; Microsoft comes out with documentation.

;

; If you have any questions, please call 471-3241.

;

; Wait until a username prompt, skip Telesys banner stuff.

COMMAND=<cr>

OK=<match>"Username:"

LOOP=<ignore>

; Send username

; Modify this line for your information.

COMMAND=mysuername<cr>

OK=<match>"Password:"

LOOP=<ignore>

; Send password

; Modify this line for your information.

COMMAND=mypassword<cr>

OK=<match>">"

; Establish terminal mode

COMMAND=terminal download<cr>

OK=<match>">"

; Establish ppp mode

; For Telesys this bogus, PPP supports SLIP only

; but under NT 3.5, this beats having to enter in

; the IP address each time. :-}

COMMAND=ppp default<cr>

CONNECT=<match>"IP"

LOOP=<ignore>