Tips and Tricks for Using the SupraFAXModem with PCB/FaxMail

by Rod L. Renner, SysOp

The Lightning Rod BBS

Silver Spring MD 20904

BBS: 301-622-0708 (v.32bis)

Introduction

I run a PCBoard system using a SupraFAXmodem v.32bis modem.  The modem

has worked well with this system, but I have been interested in using its

FAX capability as well.  Mark Herring's PCB/FaxMail gateway provided an

excellent means to implement this feature, but his program was written

primarily for use with Bit Corporation's BitFax software, which does not

currently support the Supra modem.  The only DOS based facsimile software

program that currently supports the Supra modem is FaxTalk Plus by

Thought Communications, Inc.  Fortunately, it has enough of the critical

features needed to implement a working FaxMail gateway on the PCBoard

system.

Since BitFax and FaxTalk Plus operate differently, setting up a FaxTalk

Plus based system requires several changes in procedure, some of which

were determined by trial and error, or by calling Thought Communications'

Tech Support line.  It was also necessary to write a customized program,

called FPCHECK, to read the FaxTalk Plus log file and report to FaxMail

the success or failure of the fax transfer.  A copy of FPCHECK is

included in this package.

In order to use FaxTalk Plus with FaxMail, the user should have read or

reviewed the documentation that accompanies both products.  These notes

are intended to point out key settings or techniques which can be used to

make the FaxMail/FaxTalk combination work.

FaxTalk Plus Installation and Configuration

Install the FaxTalk Plus programs following directions in the FaxTalk

documentation and FaxMail instructions.  These examples assume that the

FaxTalk Plus software is installed in C:\PCB\FAX and that the location of

the WORK directory for outgoing messages, including those converted to a

FaxTalk format, is C:\FAXOUT.  If you use a different directory, please

adjust these instructions accordingly.  There are several important

settings that must be used if it to be successful.

CONFIGURE option in the FINSTALL program.

o   NOTE that FaxTalk Plus requires that the modem return numeric

result codes ('V0' setting in the 'Modem Init String') rather

than word result codes which are required by PCBoard.  Before

returning to PCBoard, the modem must be reset to its original

settings for PCBoard by sending an ATZ command; otherwise,

PCBoard will not be able to communicate with the modem.  This

can be done with a simple ECHO command or by running a shareware

program such as ATSEND.

CONFIG options in the FAXPLUS program.

o   Under the 'SYSTEM' menu, be sure to set the following critical

options:

Background Operation [Y/N]                         - N

Automatic Answering [Y/N]                          - N

Number of Time(s) to Try Dialing [1-8]             - 1

Error Correction Mode Enabled [Y/N]                - N

Send File Directory                - C:\FAXOUT

Background Operation must be set to [N]o in order to insure that

the program waits while the fax is sent and control is not

allowed to proceed until completion or failure.  Since this is a

send-only system, Automatic Answering should be set to [N]o.

The Number of Time(s) to Try Dialing MUST be set to [1] time!!

If this number is set higher than 1 and the transmission fails,

FaxTalk would automatically schedule it for retransmission, but

allow control to continue resulting in unpredictable, but

undesirable consequences!  Due to a firmware error with the

Supra modem (according to Thought Communications) and by

frustrating trial-and-error on my part, Error Correction Mode

must be Disabled ([N]o).  Enabling ECM causes the fax program to

hang up unexpectedly after transmitting one page or less and to

report a transfer failure.  Finally, the Send File Directory may

be set to point to the location of the WORK directory for FaxMail.

o   Under the 'CONVERT' menu, set the following option:

Delete Converted Images After Sending [Y/N]          - Y

This setting will cause the FaxTalk converted images to be

deleted after they are sent, as there is no need to keep them

around.  FaxMail will take care of deleting the original ASCII

test file.

Other settings are at the users discretion or at the suggestion of the

FaxTalk software.

FaxMail Main Menu Selections

Paths & Filenames

Name/Loc of the WORK file................. C:\FAXOUT\WORK

The path chosen should match the Send File Directory setting as

used when configuring FaxTalk Plus SYSTEM menu.  I recommend

that the filename (WORK) NOT contain a file extension.  This

allows FaxTalk to create the pages of converted images with

filenames WORK.001, WORK.002, etc.  This will also be the

directory in which FaxTalk writes its log file (SEND.LOG).

Fax Software DOS command.................. RUNFAX %FILE %PHONE

In the C:\PCB\FAX directory, create a BATch file called

RUNFAX.BAT which contains the following commands:

CONVERT %1 /e /f

SENDAFAX %1 %2

These two FaxTalk programs convert the %FILE file passed by

FaxMail (C:\FAXOUT\WORK) into faxable images (WORK.001 etc.),

then sends the converted image to the recipient fax at the

phone number.  '/e' tells CONVERT that WORK file is ASCII text

while '/f' selects fine resolution.

Fax Software VERIFY command........  FPCHECK C:\FAXOUT\SEND.LOG

FPCHECK.EXE is a customized program that will read the last

entry in the log file created by SENDAFAX, then create the

file named "OK" in the current directory to report the status

(SUCCESS, BUSY, or NOGOOD) back to the FaxMail gateway.  Be

sure to copy FPCHECK.EXE included in this package into the

C:\PCB\FAX directory.

Cycle count for temporary work files................. 0

Set this value to 0 as each fax will be processed before the

next one is sent.

Use other values as suggested in the FaxMail documentation for

Paths & Filenames.

Fax Type Definitions

The *TYPE: and *INCLUDE: commands are probably not available with

the FaxTalk program unless the user can think of a clever way to

convert an uploaded file into a FaxTalk file and imbed into the

message the appropriate '&&filename' statement as outlined in the

FaxTalk manual.

FAX.BAT or $$LOGOFF.BAT files

The sequence of commands for running the FaxMail gateway using a sample

FAX.BAT file would look something like this:

CD \PCB\FAX          <- Make this the default directory

FAXTSR /l            <- Loads FAXTSR into memory

FAXMAIL /SCAN        <- Run FaxMail gateway

FAXTSR /u            <- Unload (remove) FAXTSR from memory

ECHO ATZ >COM1       <- Reset the modem!! (Use the right COM port)

CD \PCB              <- Change back to \PCB directory

BOARD                <- Rerun PCBoard

NOTE the 'ECHO ATZ >COMx' command.  It is VERY IMPORTANT to include a

command to reset the modem on its COM port back to its PCBoard settings,

particularly the use of word result codes, otherwise PCBoard will be

unable to communicate with the modem and will fail to recycle properly.

It is also OK to use a shareware program such as ATSEND to accomplish

this.

Tricks on Rescheduling a Failed Fax Connection

Occasionally, the SupraFAX fails to make a connection with a remote fax

machine on the first try.  Because FaxTalk must be told not to redial the

number, the reported NOGOOD prevents it from being retried.  However,

there is a way to have the attempt tried several times through clever use

of options.  FPCHECK has the '/B' option which will report a 'Failed'

attempt as a BUSY rather than a NOGOOD.  FaxMail will mark these messages

as BUSY, which can then be retried, rather than as Failed which stops

further attempts.  By setting up two configuration files for FaxMail

using the /CFG= option (one calling FPCHECK with the /B option and the

second without the /B), FaxMail can be called several times in a row to

retry Failed connections.  Here is a sample EVENT.SYS file that

illustrates the technique:

CD \PCB\FAX                    <- Make this the default directory

FAXTSR /l                      <- Loads FAXTSR into memory

FAXMAIL /SCAN /CFG=WITHB       <- Run FaxMail marking Failed as Busy

FAXMAIL /BUSY /CFG=WITHB       <- Rerun FaxMail for Busy or Failed

FAXMAIL /BUSY /CFG=WITHOUTB    <- Try a third time but allow Failures

FAXTSR /u                      <- Unload (remove) FAXTSR from memory

ECHO ATZ >COM1                 <- Reset the modem!!

CD \PCB                        <- Change back to \PCB directory

BOARD                          <- Rerun PCBoard

Note that the last call to FaxMail makes certain that messages which fail

3 times (or were BUSY then fail twice) do not continue to be marked BUSY.

Final Notes

This information and the FPCHECK program were tested with FaxMail 1.01

and FaxTalk Plus v1.65C using a SupraFAXmodem V.32bis modem with the

"J" (Caller ID) chip.

Good luck and success in using the FaxMail door with FaxTalk Plus and

the SupraFAXmodem!  I imagine there may be better ways to implement

this combination, but I find this does work quite adequately.  Feel free

to experiment.

I offer this information and programs for free and on an AS IS basis!

There is ABSOLUTELY NO WARRANTIES OR GUARANTEES OF ANY KIND associated

with the use of this information or programs.  USE ENTIRELY AT YOUR OWN

RISK AND EXPENSE.

FaxMail 1.01 is Copyright 1993 by Mark Herring.

SupraFAXmodem is a tradeMark of the Supra Corporation.

FaxTalk is a registered trademark of Thought Communications, Inc.