MultiDoor

Shareware version 1.0 - 01 Jan 93

Copyright 1993 Affordable Computer Services

All Rights Reserved

Written by George Steinmetz

DESCRIPTION

MultiDoor is a front end utility for BBSes (bulletin board systems).  It's

features include:

o Caller ID detection when used with a modem that transmits Caller ID.  (As of

this release, MultiDoor has only been tested on SupraFAXModem V.32bis with

Caller ID ROMs installed.)

o A caller's number can be compared against a list of phone numbers, and

specific actions can be taken when a call is placed from a number on the

list.

o Distinctive ringing (two phone numbers on one phone line, each phone number

rings differently) is supported, allowing one number for the BBS and another

number for voice calls.

o Saves extended connect information (protocol and compression used) when used

with a modem that reports this information.

o For multi-node systems, MultiDoor can be used to notify callers that the BBS

is down while you perform maintenance on another node.

INSTALLATION

In the examples that use a front end mailer, FrontDoor is used as a sample.  If

you are using another front end mailer, replace CD\FD with the path of the

front end mailer, and replace FD with the command used to execute the front end

mailer. (If the command to run the mailer is a batch file, then replace FD with

the word CALL followed by the batch file name.)

Wildcat!

Create directory \WC30\MDOOR and copy MDOOR.EXE to it.  Check CAT.BAT to

make sure that the line that runs WILDCAT is written as "WILDCAT %1 %2".

Create MDOORBAT.BAT:

@ECHO OFF

:START

CD\WC30\MDOOR

MDOOR /A /N:%WCNODEID%

IF ERRORLEVEL 200 GOTO END

:BBS

REM --- Delete the next line if FrontDoor is used ---

CALL BBS%WCNODEID%

REM --- Delete the next two lines if FrontDoor is not used ---      CD\FD

FD

GOTO START

:END

To run the BBS, enter MDOORBAT.

RemoteAccess

Create directory \RA\MDOOR and copy MDOOR.EXE to it.  If not using a front

end mailer, create MDOORBAT.BAT:

@ECHO OFF

:START

REM --- Replace the word "node" below with the node number ---

SET MDOOR=/A /N:node /P:\RA\MDOOR

CD\RA

RA -M\RA\MDOOR\MDOOR.EXE -E20

IF ERRORLEVEL 20 GOTO END

GOTO START

:END

SET MDOOR=

If using a front end mailer, create MDOORBAT.BAT:

@ECHO OFF

:START

CD\RA

REM --- Replace the word "node" below with the node number ---

MDOOR\MDOOR /A /N:node /P:\RA\MDOOR

IF ERRORLEVEL 200 GOTO END

RA -M\FD\FD.EXE -E20

IF ERRORLEVEL 20 GOTO END

GOTO START

:END

Edit MultiDoor's connect rate errorlevels to match RA's configuration.  To

run the BBS, enter MDOORBAT.

PCBoard

Create directory \MDOOR and copy MDOOR.EXE to it.  Check BOARD.BAT to make

sure that the line that runs PCB### is written as "PCB### %1 %2".  Create

MDOORBAT.BAT:

@ECHO OFF

:START

REM --- Replace the word "node" below with the node number ---

SET NODE=node

CD\MDOOR

MDOOR /A /N:%NODE%

IF ERRORLEVEL 200 GOTO END

:BBS

REM --- Delete the next two lines if FrontDoor is used ---    CD\PCB%NODE%

CALL \MDOOR\BBS%NODE%

REM --- Delete the next two lines if FrontDoor is not used ---

CD\PCB%NODE%FD

FD

GOTO START

:END

To run the BBS, enter MDOORBAT.

RUN DOOR

This selection is where the program accepts calls.  Running MultiDoor with the

/A command line option will cause MultiDoor to automatically start in this menu

selection.

If a FOSSIL driver was selected in configuration, then the modem port will be

opened through the FOSSIL driver.  If the FOSSIL driver is not found, then an

error will display along with five beeps, and the modem port will be opened in

the standard fashion.  If the modem does not respond to the initialization

string with OK after three attempts, it will also display an error message and

beep five times.  If the modem does not initialize, you can press Escape to

abort the modem initialization and return to the menu.

Once the modem is initialized, MultiDoor will wait for a call to be received or

until it is time to process an event (see Edit Event List below).

RUN BBS DOWN

To perform BBS maintenance on a multi-node BBS, run this option on that last

node in the telephone hunt sequence, and take the other nodes' modems off hook.

When this selection is entered, you will be prompted for the date and time you

anticipate the BBS to be back online.  The entries are freeform, so you may

enter the date and time anyway you wish.  The entered date and time will be

saved to disk for the next time MultiDoor is run in bbs down mode.

EDIT CONFIGURATION

Any options with an up and down arrow at the end of the prompt signifies to use

the Page Up and Page Down keys to make your selection.  No arrows mean to type

in your selection.  When you are finished, pres Escape to save and exit.  (Do

not press Escape to save while making a text entry, as the entry will not be

saved.  Press Enter first to save the selection, then press Escape.)

Modem port

Valid options: 1 through 4

This is the COM port the modem is attached to.

DTE baud rate

Valid options: 300 through 115200

This is the baud rate between the computer and the modem.

Lock DTE?

If using a modem that does not support data compression, enter N to change

the baud rate to match the connection speed.  If using a modem with data

compression, enter Y.

Handshaking

Valid options: None, XON/XOFF, RTS/CTS, Both

Default twit action

When entering a phone number in the twit list, the selected default option

will appear first.  The other options can be selected by entering Page Up or

Page Down.  (See Edit Twit List below for a description of each feature.)

Modem setup string

Optional modem command to be sent after each call.  The following options are

all entered together as the setup string, starting with the command AT.

Add E0 if the modem is not already configured for no echo.

Add H0 if the BBS takes the modem off-hook after the caller disconnects.

Add #CID=1 to enable Caller ID on SupraFAXModem V.32bis modems.

It is better to program all modem settings to the modem's memory, then this

command can be set to ATZ.

Off-hook command

Sent to pick up the phone, normally ATH1.  If distinctive ringing is being

used, then delete this line.

Write to MDOORLOG.nnn?

If Y, writes all status information, including the phone number read by

Caller ID, to MDOORLOG.nnn, where ### is the node number.

Path\filename of activity log

Enter the path and filename of the BBS's activity log for the node in use.

Caller ID and connect information will be appended to the beginning of each

call.  If you want to keep MultiDoor and your BBS's activity logs as two

separate files, then enter a filename different from the BBS's activity log.

To disable this feature, leave the selection blank.

Path\filename of Caller ID phone no

To create a file containing the phone number detected by Caller ID, enter a

path and filename here.  If no phone number is received or the number is out

of area, this file will not be created.

Path\filename of banner file

If a front end mailer is being used, the caller never sees MultiDoor in

operation, since the mailer is the program that answers the phone.  If this

selection contains a path and filename, then the file specified will be

created.  It will be a text file containing the same information that a

caller would get if MultiDoor was answering the call.  This consists of the

version number, registration notice, copyright information and phone number

information if selected (see "Show phone numbers to caller" below).

For example, FrontDoor, has the option of displaying a text file (usually

named \FD\BANNER.TXT) when it loads the BBS.  If you are not currently using

BANNER.TXT, then enter \FD\BANNER.TXT here and enable it in FrontDoor.  If

you already use BANNER.TXT, then rename it to BANNER.OLD, configure MultiDoor

to save to \FD\BANNER.NEW, and change this:

CD\FD

FD

to this:

CD\FD

COPY /B BANNER.OLD+BANNER.NEW BANNER.TXT

FD

Hint:  To ease confusion to the callers, you could make the last line of

BANNER.OLD:

FrontDoor is being loaded by

This will make it appear to the caller as:

FrontDoor is being loaded by

MultiDoor version ...

Use FOSSIL driver?

A FOSSIL driver is not required to run MultiDoor; just enter N.  However, if

you already have a FOSSIL driver installed, then you can enter Y or N.  If

MultiDoor is running a BBS that uses a FOSSIL driver (such as RemoteAccess)

and a front end mailer is not being used, enter Y.  If a front end is being

run, you can enter Y or N, as the mailer will connect to the modem and

configure the FOSSIL driver itself.

Exit to front end mailer?

If Y, after Caller ID and distinctive ringing is checked, MultiDoor exits.

FrontDoor or another front end mailer should then be run to answer the call

and connect.  MultiDoor's banner file should be displayed to the caller, even

if you are not displaying phone number information.

Caller ID prompt

If your modem transmits Caller ID information and you have Caller ID service

on your phone line, enter the text the modem transmits just before the

telephone number.  For example, if the SupraFAXModem received a call from

908-449-9357, it would transmit NMBR = 9084499357.  Therefore, the Caller ID

prompt should be "NMBR = " (do not enter quotes).  Note the space after the

equals sign.  This makes the very next character the first digit of the phone

number.  If the space wasn't entered, then the phone number would be read as

" 9084499357", but MultiDoor will strip leading spaces in case it is entered

as "NMBR =".

Show phone numbers to caller?

If Y, displays caller's phone number, node number, and node's phone number at

logon to the caller.

Support distinctive ringing?

MultiDoor supports distinctive ringing (two phone numbers ring the same

phone, but with different ringing cadences).  If the BBS line has distinctive

ringing, enter Y here.

BBS line

Valid options: 1 through 2

This selection only appears if distinctive ringing is answered Y.  Enter the

line that BBS calls will be received on.  If a call is received on the line

configured as the BBS line, then the modem answers.  If the call is not on

the BBS line, then it does not answer and simulates a phone ringing through

the PC's speaker.  (Phones' ringers should be turned off to avoid being

disturbed when BBS calls are received.)  When the call is answered or the

caller hangs up, then MultiDoor resets for the next call.

Time zone

Enter the time zone the BBS's clock is set to.  The time zone is not used

internally; it is displayed to the caller when BBS Down is run.  The

recommended setting is, for example, Eastern time.  Note that you should not

say, for example, Eastern Standard Time or EST when you are in daylight

savings time (use Eastern Daylight Time or EDT instead, and remember to

change the name when changing back to standard time).  If Z or z is entered,

no space will be printed between the time and the time zone (1200Z instead of

1200 Z).

Node phone number

Enter the phone number that the caller dials to call the node.

Foreground color

Background color

Sets color combinations to override the default settings and also overrides

the FCOLOR and BCOLOR environment variables.  Illegal color combinations,

such as blue on blue, or bright blue on blue, are not allowed.  (The inverse

of bright blue on blue would be blue on blue.)

EVENTS

MultiDoor can process up to 20 daily events.  Enter the time of each event in

24 hour format.  When the event time is reached, MultiDoor will exit with the

errorlevel specified in the errorlevel configuration screen.  If an event was

missed due to another event taking longer than it should, or the BBS being

down, each event will run once regardless of how many times it was missed. To

avoid running a missed event, delete the event from the event configuration

screen and reenter it.

EDIT TWIT LIST

If Caller ID is enabled, the caller's phone number can be compared against a

list of up to 20 phone numbers.  If the number matches, then a special command

can be performed as opposed to running the BBS.  The commands are:

Don't answer   (lets phone keep ringing)

Hang up        (picks up, waits one second and hangs up)

Send noise     (connect, send random characters & disconnect)

Exit w/ errlvl (drop to DOS with errorlevel of twit number + 20)

Send msg, disc (send MSGn.TXT and disconnect, n = twit number)

Send msg, BBS  (send MSGn.TXT, then run BBS)

False BBS Down (send message saying the BBS is down for maintenance)

Never BBS Down (run the BBS even if BBS DOWN is being run)

For the send message commands, example text filenames are MSG1.TXT, MSG2.TXT,

MSG12.TXT. etc.

EDIT ERRORLEVELS

Various errorlevels are used to relay information from MultiDoor to its batch

file.  If you want to change the errorlevels from their defaults (perhaps so

they are the same errorlevels as your front end mailer, if you use one), enter

this selection to modify the errorlevels.  To get a list of errorlevels to

configure your batch files, enter this selection and press the PrintScrn key to

print out the errorlevels.  The only errorlevel you can not change is the

errorlevel for program error, errorlevel 255.

EDIT BBS1.BAT

This command's name will vary according to the node listed in configuration.

If node 100 is entered, then this command will be EDIT BBS100.BAT.  This

command changes how BBSn.BAT (n = node number) will be created.  When

configured correctly, BBSn.BAT will start the BBS software and tell the BBS

that a caller has connected at a specified baud rate.  Once the call is

complete, the BBS should exit, which will allow the main batch file to continue

and rerun MultiDoor.

The following is the default BBSn.BAT configuration (for Wildcat! BBSes):

BBS command........ @CAT /B

Error correction... MNP

Local login........ LOCAL

Connect    300..... 300

Connect   1200..... 1200

Connect   2400..... 2400

Connect   4800..... 4800

Connect   7200..... 7200

Connect   9600..... 9600

Connect  12000..... 12000

Connect  14400..... 14400

Connect  19200..... 19200

Connect  38400..... 38400

Connect  57600..... 57600

Connect 115200..... 115200

Sample batch files: @CAT /B 2400MNP

@CAT /B LOCAL

To test the configuration, look at the two samples.  They show how the batch

file will look if a connection is made at 2400 bps with error correction, and

if a local logon is made by pressing F1 while waiting for calls.

To create the space between the @CAT /B and the baud rate, there is a space

after @CAT /B in the BBS command line.  (The default is not "@CAT /B", it is

"@CAT /B ".)  Likewise, if your BBS software requires a space between the baud

rate and the command line option signifying error correction is used, then

enter a space as the first character of the error correction command line.

The @ is to prevent the command line from displaying when using DOS version 3.3

or above.  It may be removed if you wish to see the command line flash on the

screen while the BBS software is loading or if you are running DOS version 3.21

or below.

COMMAND LINE OPTIONS

MDOOR [/A] [/N:node] [/P:path] [/D:number]

/A - Automatic mode

When in automatic mode, MultiDoor starts in the receive calls mode instead of

the main menu.  MultiDoor will be in the same mode (BBS up or down) as it was

when it was last exited.

/N - Node ID (1-999)

On a multi-node BBS, enter the BBS's node number here.  If /N is not used (as

on single-node BBSes), the node will be 1.

/P - Path of MultiDoor data files

If MultiDoor's data files are not in the current directory, enter the data

directory's path here.

/D - Demo caller ID phone number (for testing if you don't have Caller ID)

This is for debugging/playing purposes only.  To test the Caller ID features

if you don't have Caller ID, enter (for example) MDOOR /D:908-449-9357, set

the Caller ID prompt in the Edit Configuration screen to anything, enter

receive calls and call the modem.  All features pertaining to Caller ID,

including the twit list, will act as if 908-449-9357 actually called.

Options can be entered either on the command line or in the environment

variable MDOOR by entering SET MDOOR=[options].  (The command line is searched

first, then the environment variable.)

NOTE TO BBS USERS AND SHAREWARE VENDORS

If you received this file from a BBS, the following message should appear when

you use PKUNZIP to extract this program:

Authentic files Verified!   # VBI707   Affordable Computer Services

If this message did not display, then the .ZIP file you received is not the

original.  Call The Other BBS at the number displayed below to download the

original .ZIP file (you can download on your first call).

CUSTOM PROGRAMMING

If custom programming is requested, contact Affordable Computer Services by

mail at the address shown below.

REGISTRATION

If you like this program, please mail a check or money order for $20 (New

Jersey residents add state sales tax) to:

Affordable Computer Services

PO Box 969

Belmar, NJ 07719-0969

Along with the registration fee, please also enclose the following:

o The name and version of the program you are registering.

o Your name or company name exactly as you wish it spelled on your registered

copy (upper and lower case).

o Your mailing address.

o A telephone number you can be reached at so I can confirm your registration.

You may also register by using the American Express Card.  You may register

with the card by one of the following methods:

o By mail at the above address.

o By voice at 908-449-3836 or 800-929-0110*.

o By   fax at 908-449-9357 or 800-929-0631*.

o Online (The Other BBS) at 908-988-0636.

* If possible, please do not use the 800 numbers except for registering, as I

can not afford to give technical support toll free.

To process your registration, you must supply the above information as well as

the following information:

o Your name exactly as it appears on the card.

o Your card's account number.

o Your card's expiration date.

o Your card's billing address.

When registering with the American Express card on The Other BBS, you will

receive your registration ID in a private message within 24 hours of your

registration.

Any questions about this program may be addressed to George Steinmetz on The

Other BBS, 908-988-0636 (RIME Echomail ID: OTHERBBS), or write, call or fax

Affordable Computer Services at the address and phone numbers listed above.

REVISION HISTORY

Version 1.0 - 01 Jan 93

o Original release.