CLOCK Version 3.52                              November 19, 1993

Version 3.52

CLOCK is a shareware program.

See Appendix A for registration information.

$10 (or $25 for full source code)

Ronald Q. Smith

11 Black Oak Road

St. Paul, MN 55127-6204

(c) Copyright 1991-1993 Ronald Q. Smith

CLOCK Version 3.52                              November 19, 1993

CONTENTS

I.  INTRODUCTION  . . . . . . . . . . . . . . . . . . . . . .   1

APPENDIX A:  DEFINITION OF SHAREWARE  . . . . . . . . . . . .  62

APPENDIX B:  CLOCK TYPES  . . . . . . . . . . . . . . . . . .  65

B.1  Type 0 - PC/AT  . . . . . . . . . . . . . . . . . .  65

B.2  Type 1 - Zenith Z-18x portables, Supersport 20  . .  66

B.3  Type 2 - Various Zenith Data System PCs.  . . . . .  66

B.4  Type 3 - Memory-Mapped Clocks . . . . . . . . . . .  67

B.5  Type 4 - Direct Register I/O Bus Clock  . . . . . .  68

B.6  Type 5 - Mitsubishi 8088 PCs  . . . . . . . . . . .  70

B.7  Type 6 - Indirect Register I/O Bus Clock  . . . . .  71

B.8  Type 7 - Complex I/O Bus Clock  . . . . . . . . . .  72

B.9  Type 8 - Generic I/O Bus Clock  . . . . . . . . . .  73

B.10 Type 9 - Quadram I/O Bus Clock  . . . . . . . . . .  74

B.11 Type A - AT&T 6300, 6300 PLUS, 6300 WGS . . . . . .  75

B.12 Type B - Hyundai 8088 Clock . . . . . . . . . . . .  76

B.13 Type C - Multi I/O Card Clock . . . . . . . . . . .  76

APPENDIX C:  REVISION HISTORY . . . . . . . . . . . . . . . .  78

CLOCK Version 3.52                              November 19, 1993

I.  INTRODUCTION

A.DefinitionsYour PC  contains  two  clocks.   One  of  these  is

referred  to in  this document  and in messages  from CLK  as the

"calendar clock."  The other is referred to as the "DOS clock."

CALENDAR CLOCK

The  calendar clock is the  clock that retains  the date and

time when your system is turned  off.  It is the clock  that

requires  a battery and  for that reason  is also called the

battery-protected  clock.    Various  hardware vendors  also

refer to it as the real-time clock, the  CMOS clock, and the

day clock.   In most  systems it is  a bit of  logic on  the

motherboard or other  add-in board and  consists of a  clock

chip, a  battery, and some control logic.  Some older clocks

are on  little  mini-boards that  plug into  a cable  (e.g.,

floppy cable) or socket under another chip.

The calendar clock typically has  a set of internal counters

that maintain the  seconds, minutes, hours, day of week, day

of month, month, and year.  Some don't allow  you to set the

seconds and can  only be set on the minute  even though they

may allow  you  to read  the seconds.   Many  of them  don't

maintain the year.  They expect the software to  simply keep

the year up to date by writing it to the chip every time you

boot.  Some of the  calendar clocks don't handle leap years.

CLOCK.SYS  will  handle the  leap  years  for you  on  those

clocks.

The calendar clock is normally read once  when the system is

booted and  the time is then written to  the DOS clock.  The

calendar clock is  then normally never read again, but it is

written if you change the time.  Since the calendar clock is

usually substantially more accurate  than the DOS clock, one

of the  functions that CLOCK.SYS can  perform for you  is to

periodically read  the  calendar  clock and  reset  the  DOS

clock.

DOS CLOCK

The DOS clock is really not a clock although it is used like

one.  It is really just a counter that increments at a known

rate (18.2 times per  second).  The DOS clock is also called

CLOCK Version 3.52                              November 19, 1993

the  BIOS clock,  the interval  timer, and  the incrementing

counter.

When  you  boot  the  system,  your  clock  driver  software

(CLOCK.SYS, the  driver built  into DOS,  or another  add-in

software driver)  reads the calendar clock  and converts the

date  and time to the number  of days since January 1, 1980,

and the number of seconds since midnight on the current day.

The number  of days is  just saved inside the  clock driver.

The  number of seconds  is converted to  a number of counter

ticks  using a  little more  exact  number than  18.2.   The

number of ticks is then stored in the counter by calling the

BIOS.

When you  ask DOS  to read  the clock,  it  calls the  clock

driver which reads the counter by calling the BIOS, converts

the number of ticks back to seconds, and  returns the number

of days that it has saved  and the number of seconds to DOS.

The BIOS  checks to see  if the counter goes  past midnight.

If it does, it resets the counter to zero and tells the next

caller that the day  has advanced.  In  that case the  clock

driver increments the day that is has saved before returning

it to DOS.

Unfortunately, the  BIOS only  tells the first  caller after

midnight  that the  day has  advanced.   That is  the common

reason for the  day not advancing on your PC  when you leave

an application running over midnight.  Many applications use

the  BIOS counter  internally  just to  get  interval timing

information.  When they do, DOS doesn't see that the day has

advanced.   This is  another problem that  CLOCK.SYS solves.

It  checks to see if the time  ever appears to run backwards

and increments the day if it does.

B.OverviewCLOCK consists  of two programs that  give you complete

control over the  operation of your  calendar (battery-protected)

clock and your DOS/BIOS  internal timer.  The normal  DOS mode of

operation is to  read the calendar clock when you boot the system

and set the internal timer.  Thereafter all read operations refer

only to the internal timer while time setting operations write to

both clocks.  In  levels of DOS prior  to 3.3, even time  setting

operations only wrote to the internal timer.  In order to set the

calendar clock you had to run the SETUP program.

CLOCK Version 3.52                              November 19, 1993

Some of the drawbacks  to the standard  DOS clock operation  that

the CLOCK programs address are:

�    There is  no  provision to  switch  time zones  without

resetting the system time.

�    DOS  does not handle automatic changes between daylight

savings time and standard time.

�    The DOS/BIOS internal timer often drifts quite  rapidly

away  from  the time  maintained  by the  more accurate

calendar clock.

�    Even  the calendar  clock drifts  slowly away  from the

accurate  time.  DOS  provides no way  to automatically

adjust for that drift.

�    If your  battery is failing  or some other  hardware or

software problem messes  up the  time in your  calendar

clock, there is no way to detect that except by manual-

ly displaying the time and checking it.

�    If you have to run a program that occasionally  changes

the  time incorrectly, there is no  way to protect your

clock from  those  changes.   You  can also  have  this

problem  if  you are  a parent  whose children  set the

clock when you don't want  them to, an instructor whose

students reset the time, or  an expert young user whose

parents mess up your clock.

�    If  you need to experiment with special times and dates

but don't want to have to reset your clock  afterwards,

DOS has no mechanism to help.  In order to test end-of-

month processing for your  application, you have to set

the  date and time,  run your  tests, and then  set the

time and date back.

�    The standard DOS and BIOS software on some PCs can miss

the change  to a new  day when the  PC is left  on over

midnight.

�    Your clock might not handle automatic changes to  a new

year or may not understand leap years.

CLOCK Version 3.52                              November 19, 1993

�    You may have an  older PC from  before the PC/AT  clock

standardization and your vendor no  longer releases new

versions of DOS  for that PC.  You are stuck on DOS 3.2

or earlier or you have to give up your calendar clock.

If  any of those circumstances cause  problems for you, CLOCK can

help.  The two  primary programs are  CLOCK.SYS and CLK.EXE.   In

addition, if you are a software developer, CLKDEMO is provided in

source code to show  you how to program the extended clock opera-

tions into your own applications.

CLOCK.SYS is a clock device driver (CLOCK$ device).  It

replaces the DOS internal clock driver.  CLOCK.SYS pro-

vides  for  automatic time  zone conversion.    It also

allows you to periodically or continuously get the time

from the  calendar clock.   This can  totally eliminate

problems with  missing day changes and  realtime clocks

that  are inaccurate.   CLOCK.SYS provides  support for

many types  of calendar  clocks including  many of  the

original add-in clocks  of the 8088 era.  If your clock

is not currently supported, I want to hear from you.  I

will be happy to add it to the software.

CLK.EXE  is a  program  that allows  you to  separately

control your  DOS/BIOS internal clock  and battery-pro-

tected  calendar clock.  Most importantly, CLK.EXE will

handle time zones and will automatically switch between

standard and  daylight savings time  for you.   It also

supports automatic adjustment for calendar clock drift,

checks  for incorrect  or unlikely  times, and  can re-

strict time changes.

CLKDEMO.EXE is a small part of CLK.EXE that is provided

in source  code (C language)  form so that you  can see

how    to    add   clock    control    to    your   own

applications.II.CLOCK.SYS    -    Replacement    Device

DriverCLOCK.SYS is  a device  driver that replaces  the

built-in  CLOCK$ device  driver in  DOS levels  2.1 and

above.   CLOCK.SYS adds many functions  not provided in

the  standard DOS CLOCK$ device driver.   You won't see

mention  of the DOS  CLOCK$ device  driver in  your DOS

CLOCK Version 3.52                              November 19, 1993

user's guides.  You will see a very small mention of it

in the system's  programmers reference  manuals if  you

are an  assembly language developer.   It is  there and

DOS  was specifically  designed to  let you  replace it

with one  of your own even  if the manual  doesn't tell

you that.

A.Unique Clock Hardware  SupportCLOCK.SYS was originally  written

especially for those people whose clocks are not supported by new

levels of DOS.  This  usually occurs because your vendor  stopped

providing  new levels  of DOS  adapted for  your  computer model.

Before  the days of the PC/AT  and standard clock interfaces, DOS

was usually sold  directly to hardware vendors who  customized it

for  their system and delivered it with the system.  In addition,

many of  the early clocks  were provided  by third-party,  after-

market vendors who had no  idea what system it would be  used in.

Since those systems and clocks have been  out of production for a

long time, most  of the vendors have  stopped providing upgrades.

Yet most  of those  PCs will  happily run  DOS 3.3  or above  and

especially DOS  5.0 except that you have to give up your calendar

clock.

CLOCK.SYS supports a wide variety of different clock hardware and

BIOS interfaces.  As far as possible, CLOCK.SYS will automatical-

ly determine  what  clock interface  to use.    If the  automatic

determination  does  not  work  for  your system,  you  may  tell

CLOCK.SYS specifically what interface type  to use.  CLOCK.SYS is

also  required  by anyone  who wants  to use  CLK as  it provides

functions not provided  by the normal  DOS CLOCK$ device  driver.

CLOCK.SYS is fully compatible with  the DOS device driver on  all

modern PCs and many older ones.

If your PC clock is not currently supported but you would like to

remove  the restrictions  on what  versions of  DOS you  can use,

please contact  me by mail or on CompuServe.   I will be happy to

add  support for your clock.  I will need some technical informa-

tion and help from you in getting it to work so I will give you a

$0 usage license for your help.  Please see section  II.C for the

current list of supported clocks and Appendix B for a description

of those clocks.

CLOCK Version 3.52                              November 19, 1993

B.Installing CLOCK.SYSTo  install the  device driver  include the

following line in your CONFIG.SYS file.

Syntax    device=[d:][path\]clock.sys [type][,io_addr] [/N]

or

device=[d:][path\]clock.sys 3[,segment[,write0,

write1,read]] [/N]

or

device=[d:][path\]clock.sys A[,base_year] [/N]

Parameters     d:

Identifies the drive containing CLOCK.SYS.  If  d:

is not provided, the boot drive is assumed.

path\

Specifies the directory containing CLOCK.SYS.   If

path\ is not given, the root directory is assumed.

CLOCK.SYS

Is the name of the CLOCK.SYS file.  You may change

the name  if you  wish, but you  must not  use the

.COM or .EXE extensions.

type

Is the kind of calendar clock on  your system.  If

you do not provide type, CLOCK.SYS will probe your

system   to  determine   which  clock   you  have.

CLOCK.SYS is  usually able  to detect the  type of

calendar  clock without trouble.   It is possible,

however, that you have some other hardware in your

system that could fool  CLOCK.SYS.  For that  rea-

son, you might need to  provide the type of clock.

See section II.C and Appendix B for more  informa-

tion about the clock types.

io_addr

Is  the I/O  bus address  of your  clock hardware.

Some of  the types  of clocks  that are  supported

CLOCK Version 3.52                              November 19, 1993

were set  up at a  wide variety of  I/O addresses.

CLOCK.SYS  probes  all the  commonly used  I/O ad-

dresses for these  clocks.   However, you or  your

vendor may have installed your clock at an address

that CLOCK.SYS  doesn't know about.   By providing

the  address, you  make it  much more  likely that

CLOCK.SYS will correctly find your clock.  Even if

you don't know what  type of clock you  have, pro-

viding the I/O address may be enough for CLOCK.SYS

to determine which one it is.  io_addr is current-

ly used with clock types 4, 6,  7, 8, 9, B, and C.

Normally this is  240, 2C0, or 340 for types 4, 6,

and 7.  It is 210 or 310 for type 9.  It is E0 for

type  B.  And, it is 338  for type C.  All numbers

are in  hex.  For  example, an AST  Research clock

might be:

device=[d:][path\]clock.sys 4,2C0

If you  do  not specify  the "io_addr",  CLOCK.SYS

will try to determine the correct address by prob-

ing.

If you are sure that you have an I/O clock but you

are not sure which one  it is, specify type 8  and

supply the io_addr if  possible.  Type 8  does not

represent a specific clock.  Rather it attempts to

determine whether your clock is a type 4, 6, 7, 9,

or B using  a sequence of  tests similar to  those

used by  some of the  I/O clock vendors  to figure

out what kind of clock you have.

If you  have a  memory mapped  clock, you  may use  the

second format to specify the addresses to be used.   If

you don't know the addresses, CLOCK.SYS will attempt to

determine them by probing.

segment

Is  the base  address used  to  access the  clock.

Usually  your documentation will  give the segment

address as  four hex digits such as  FE00 or F000.

Some addresses may be less than four digits (e.g.,

70).   Other documentation  may show  addresses in

CLOCK Version 3.52                              November 19, 1993

the  form aaaa:bbbb.    In this  case the  segment

address is  the aaaa  value.  Sometimes  these ad-

dresses are  written as 0xaaaa,  aaaah, aaaaH,  or

0Xaaaa.  In all of these forms there is a 2- to 4-

digit hex number that is the segment address.

write0

Is the offset used to write 0 bits.  It must  also

be a hex  address and may be 0,  F002, or anything

else the vendor chose.  If the addresses are given

in  the aaaa:bbbb form, use  the bbbb.  With these

clocks,  you read and  write the  clock by reading

the  memory addresses.   For example,  reading ad-

dress segment:write0 results in writing a zero bit

to the clock.   Writing an appropriate  length se-

quence of zero and one bits sets the clock.

write1

Is the offset used to write 1 bits.

read

Is the offset used to read bits.

If  your documentation  gives the  segment address

but isn't  clear about  the offsets, just  specify

the  segment address  and CLOCK.SYS  will try  all

offset combinations that it knows about.

If you specify one of the offsets, you must speci-

fy them all.

The third format is used with the AT&T 6300 clocks.

base_year

Is used for the AT&T 6300  clocks (type=A).  Dates

prior to that year or more than 7 years after that

year will  not be handled  correctly.  If  no base

year  is specified, 1992 will  be used.  Dates be-

tween the  start of the  base year and the  end of

the seventh year later are handled (e.g., 1992-1-1

until 1999-12-31).

/N

CLOCK Version 3.52                              November 19, 1993

Indicates that  you do  not want CLOCK.SYS  to at-

tempt  to  detect missed  date changes.   Normally

CLOCK.SYS  will  try  to detect  when  a  date has

changed that was  not reported by the BIOS.   This

usually  happens when  a program  is running  past

midnight and calls the BIOS to read the time rath-

er than calling DOS.  That program  sees the date-

change   indication  from   the   BIOS   and   DOS

(CLOCK.SYS) never gets a chance to change the day.

However,  the logic  that  CLOCK.SYS uses  can  be

fooled by programs  that change the time  with di-

rect calls  on the BIOS  and may advance  the date

incorrectly.   This is very rare, but if your date

advances unexpectedly or by more than one day, you

may want to use this option.

Notes     DOS interfaces                    ______________

All DOS DATE  and TIME  commands and all  DOS date  and

time system calls will operate  with no apparent change

to  you or  any program.   Unless you have  used CLK to

change the time handling, your clocks are  handled just

as DOS would handle them.

Dates and leap years                    ____________________

CLOCK.SYS supports all valid DOS dates from 1980-1-1 to

2079-12-31.   It supports dates to  2099-12-31 for many

of the clocks.  CLOCK.SYS also handles year changes and

leap years automatically for clock types  that don't do

it themselves.

Time zones and daylight savings time                    ____________________________________

CLOCK.SYS  in conjunction with CLK.EXE provides support

for multiple  time zones  and automatic  conversion be-

tween daylight  savings time  and standard time  at the

appropriate  times  of  the  year.   For  this  reason,

CLOCK.SYS  may be  of great  use  to you  even if  your

version of DOS already supports your clock correctly.

Clock synchronization                    _____________________

Normally, CLOCK.SYS  operates  identically to  the  DOS

CLOCK$ device driver.   At boot it  sets the DOS  clock

from the calendar  clock.  Any time you change the date

or time it writes to both clocks.

CLOCK Version 3.52                              November 19, 1993

With CLK.EXE  you have several  other options.   If you

have  specified a  time zone  and offset  with CLK.EXE,

CLOCK.SYS will adjust  all times read from  and written

to the calendar clock by the time zone offset.  You can

also set  it to read the calendar clock rather than the

DOS  clock for  some  or all  time  requests to  ensure

continued synchronization of  the two clocks.   You can

also totally separate the  two clocks so that  DOS time

and  date functions  only read  and write  the DOS/BIOS

internal clock.

Time setting limits                    ___________________

CLK.EXE  can tell CLOCK.SYS to apply limits to the time

changes that  it will  accept  or stop  accepting  time

changes at all.  For example, you can tell CLOCK.SYS to

accept no  changes that would  set the clock  backward.

You can also inhibit all changes.

Password Protection                    ___________________

CLK.EXE can  establish  a password  to  control  future

changes to the time zone, connection, and  time setting

limits.   Once  the password  is  set, all  attempts to

change those values  will simply be ignored  unless the

correct password  is  supplied.   You  may  change  the

daylight versus standard time  flag and the  continuous

time display without knowing the password.

Continuous Time Display                    _______________________

CLK.EXE can tell CLOCK.SYS to display the time continu-

ously at any position on your screen.   You specify the

cursor coordinates, display  attributes, and 12-hour or

24-hour  mode  and  CLOCK.SYS will  update  the display

every second.

Saving Settings Across Boots                    ____________________________

With  CLK.EXE  you  can  save  the   current  state  of

CLOCK.SYS at  any time  in a form  that can be  used on

your next boot.  You can set up your time zone, setting

limits, connection mode, continuous display, and  pass-

word  and then save  a new version  of CLOCK.SYS.  This

new  version  will  automatically  start  running  with

without having to scan for your clock type and with all

your standard state automatically in effect.

CLOCK Version 3.52                              November 19, 1993

C.Clock TypeThere are almost as many different implementations of

PC  clocks as there are different types  of PCs.  Until the PC/AT

came along, there  was no standard for  battery-protected clocks,

and  each  PC and  add-in  vendor was  likely  to  implement them

differently.  With the PC/AT a standard and documented  interface

for such clocks  was provided.   Since that  time, most PCs  have

followed the PC/AT standard.  It is earlier  PCs, especially 8088

PCs, that don't follow the standard.  Since these are early  PCs,

there  is also  a high probability  that the vendor  is no longer

providing  modified versions  of  DOS with  built-in support  for

their clocks.   Most  add-in clock  vendors never  did provide  a

modified  DOS  and  you had  to  manage your  clock  with utility

programs.  This leaves you faced  with the choice between staying

with an early version of DOS or giving up your clock.

CLOCK.SYS is  intended to  free you from  those limitations.   It

will work with  all versions of  DOS from  2.10 through 5.00  and

undoubtedly beyond.  It  also attempts to automatically determine

what kind of  clock you have and install  the correct support for

it.  However, there are often enough differences between PCs that

the  automatic determination  may not  find anything or  may even

make the  wrong choice.  To  give you more control  of the situa-

tion, you may specify the clock  interface by giving an interface

number.

It will  not always  be easy to  decide what interface  number to

use.   You will at least  need some technical information  on how

your clock works or  on the BIOS calls  used to control the  bat-

tery-protected clock.  If you find a description below that seems

to match, give it a  try.  The most that should go  wrong is that

your time  and date will be set incorrectly, but then they proba-

bly weren't working anyhow.

If you aren't sure what interface to use or if you are sure  that

your clock is not supported, LET ME KNOW.  I WOULD LIKE  TO HELP.

You can help me make CLOCK.SYS better for everyone.  I would even

like to  hear from  you if  CLOCK.SYS did work  for you  and your

clock isn't currently  listed.  I  would like to  be able to  add

your PC  manufacturer and model  number to the supported  list so

others know what to use.  If CLOCK.SYS did not work for you, I am

interested  in making the changes  so that it does  work.  If you

can  provide me with the necessary  technical information, I will

make the changes.

CLOCK Version 3.52                              November 19, 1993

Usually,  a technical  reference  manual with  hardware and  BIOS

interfaces is the best source.  Even if you don't  have one, your

original  hardware  vendor  may be  willing  to  send  you enough

information to  allow you  to program  the clock.    If all  else

fails, we may even  be able to use an old copy  of DOS or a clock

setting utility to  figure out how it works.   Before we do that,

check your license to be sure that it is not forbidden  to disas-

semble the software to see how the clock works.

The following table  shows the systems that  have been determined

to work  with existing clock  types.   If your system  is in  the

list, try the clock type indicated.   If not, look through Appen-

dix B which tells you  about the characteristics of each type  of

clock and see if anything seems to match your system type.

CLOCK Version 3.52                              November 19, 1993

������������������������������������������������������ͻ

�     SYSTEM                �  CLOCK TYPE              �

������������������������������������������������������Ķ

�     Amstrad PC1521DD      �  0                       �

������������������������������������������������������Ķ

�     AST Research          �  8 (4, 6, or 7) See dis- �

�       Six-Pack            �  cussion above and in    �

�       I/O Plus            �  Appendix B              �

�       etc.                �                          �

������������������������������������������������������Ķ

�     AT&T 6300             �  A                       �

������������������������������������������������������Ķ

�     Hyundai 8088          �  B                       �

������������������������������������������������������Ķ

�     IBM PC/AT and most    �  0                       �

�     PCs since then.       �                          �

������������������������������������������������������Ķ

�     Leading Edge Model D  �  5                       �

������������������������������������������������������Ķ

�     Mitsubishi 8088       �  5                       �

������������������������������������������������������Ķ

�     Multi I/O Card        �  C                       �

������������������������������������������������������Ķ

�     Quadram Corp.         �  9                       �

�       QuadCard            �                          �

�       QuadCard II         �                          �

�       QuadCard 512+       �                          �

������������������������������������������������������Ķ

�     SMT No-Slot Clock     �  3                       �

������������������������������������������������������Ķ

�     Sperry PC-1           �  5                       �

������������������������������������������������������Ķ

�     Sperry PC/HT          �  5                       �

������������������������������������������������������Ķ

�     Tandy 1200HD          �  4                       �

������������������������������������������������������Ķ

�     Vendex Turbo-888-XT   �  4                       �

������������������������������������������������������Ķ

�     Zenith Supersport 20  �  1                       �

������������������������������������������������������Ķ

�     Zenith Z-18x          �  1                       �

������������������������������������������������������ͼ

CLOCK Version 3.52                              November 19, 1993

D.ExamplesFor almost everyone,  the only format you  ever need to

use is:

device=[d:][path\]clock.sys

If CLOCK.SYS is  in the root directory of your  boot device, this

is:

device=clock.sys

CLOCK.SYS will  quite happily load  into the upper  memory blocks

(UMBs) provided  by DOS 5.0 and by other software.  With DOS 5.0,

you might use:

devicehigh=clock.sys

With some memory managers you can even load CLOCK.SYS  into upper

memory blocks on  earlier levels of DOS.  See your manual for the

commands to use.  Some of  these memory managers will create UMBs

on 8088 and  80286 systems.   CLOCK.SYS  loads and  runs high  on

those systems as well.

With a memory mapped clock that uses  an address combination that

CLOCK.SYS  doesn't search automatically and with CLOCK.SYS stored

in the C:\UTILITY directory, you might use:

device=c:\utility\clock.sys 3,ffff,f000,f004,f008

Please note that the memory addresses chosen for this example are

not likely to match those of your clock and you should substitute

the addresses appropriate to your clock.

If you have an I/O  clock with the same interface as  the Quadram

Corp. clock but a different I/O address, you might use:

device=clock.sys 9,3fc

if your I/O base address were 3fc.

If you have one of the AT&T 6300 system types and you occasional-

ly need to set your clock prior to 1992-1-1, you might use:

device=clock.sys a,1988

E.Future  EnhancementsOf course,  I hope  to add  more clocks  as

people ask for them.   Those old PCs are still good for  a lot of

what we  do even if they  are sometimes relegated to  a secondary

status.

There are no other outstanding enhancements currently planned for

CLOCK.SYS.III.CLK.EXE  -  Clock   Control  ProgramCLK  gives  you

complete  control over the  operation of  the calendar  clock and

CLOCK Version 3.52                              November 19, 1993

DOS/BIOS clock.   You  can operate  them separately  or together.

You  can  establish  time  zones, daylight  savings  time  rules,

automatic adjustment, synchronization rules, and more.

A.CLK CommandSyntax clk [function] [function ...] [/I[file]] [/?]

or in full form

clk [A=�sss.cc] [C=A|D|R|W] [D=[N][x,y,a[,12|,24]]

[L=...] [M=...] [R=A|B-hh:mm:ss|F+hh:mm:ss|N|R]

[S=C|D]    [TZ=...]    [W=-back,+forward]     [/B]

[/I[file][,C]]  [/M[file][,C]] [/P[N]]  [/S[file]]

[/?]

Parameters     function

There  are many  functions that  may be  used with

CLK.EXE and  they are discussed  in the  following

paragraphs.   You may use any  number of functions

on a  single command  line and  they will  be pro-

cessed in  the  order  in which  you  enter  them.

However, if  any error is discovered in any of the

functions, none of them are processed.  Thus,

clk c=r tz=:est

is the same as the two command lines:

clk c=r

clk tz=:est

except that an error in either of the functions in

the  first example cause  the whole  command to be

ignored.

The case (upper  or lower) of any of  the function

names, options,  or values  is ignored except  for

the TZ  function's time  zone names.   Even there,

the case is not used.  It is simply  preserved for

later display.

You can display a help message about  any function

or  option by  immediately following  the function

letter or  option letter with ?.  Thus M? displays

help about the M=  function and /I? displays  help

about the I  option.  /?  displays a help  message

about the CLK command line.

CLOCK Version 3.52                              November 19, 1993

none

If no function or option is provided, CLK displays

the current  state of operation  of CLOCK.SYS  and

the current time from both the  calendar clock and

the DOS/BIOS clock.

If you have used CLK to set a time  zone (TZ func-

tion), the current time zone name and offset value

are  displayed.     These  values  are  saved   in

CLOCK.SYS so that they may  be used for time func-

tions.

The connection state of the clocks (C function) is

displayed  as is the current restrictions (R func-

tion) on clock changes.

All  uses  of CLK  (except errors  and /?)  end by

displaying the  same information.  You always know

the state of your two clocks after execution.

A=�sss.cc

The Adjust function is used to set a daily adjust-

ment factor in seconds and 100ths of seconds.

This function assumes that you have some knowledge

of the  rate at  which your calendar  clock drifts

with respect to "real" time.   Presumably you have

periodically set your  clock using a  program that

calls the Naval Observatory or NIST and  noted the

difference  over  time.   Some such  programs will

help you to calculate the drift rate.  You can get

a very good adjustment from a decent quartz watch.

The value may be a positive or  negative number in

the range -326.99  to +326.99 seconds per  day but

reasonable values are likely to be a fraction of a

second.

The first time  you use A=, your clock  is not ad-

justed.  The value of  A and the current time  are

saved in  a file  for later  use.   Later you  can

choose to run CLK with the /I option to invoke the

file.  The elapsed time  since the last adjustment

CLOCK Version 3.52                              November 19, 1993

(or the time you last changed A) in days and frac-

tions is multiplied by  A to calculate any adjust-

ment that needs to be applied.  Only whole seconds

of adjustment are applied.

The  A= function only  sets the adjustment factor.

The actual  adjustment is  carried out  by the  L=

function.  You  don't have to  enter the L=  func-

tion.  CLK will do that for you the first time you

use A= and  will keep  it up to  date as it  later

makes adjustments.

NOTE:  Do  not use the  A= function with the  AT&T

6300 clocks  (Type A) or the  Quadram clocks (Type

9).  They  only permit the time  to be set  to the

nearest minute.

C=A

Connects the two  clocks for All operations.   All

read operations go directly to the calendar clock.

The calendard clock is read, the time  is adjusted

for any time zone offset, and the adjusted time is

written to the DOS clock and returned to the call-

er.  With this  function, your calendar clock  es-

sentially becomes  the only  clock.  However,  the

DOS clock is set each time so that any application

reading the clock directly (without going  through

DOS) sees the same  time that DOS reports.   It is

unwise to use  this function if your clock is slow

(some I/O clocks and some memory-mapped clocks) or

if  it only increments  in units of  seconds (most

clocks).  Any time zone offset is applied to reads

and writes.

C=D

Disconnects  the two  clocks.   Any change  of the

date  and time  will  affect the  DOS  clock only.

Reads also use  the DOS clock only.  This function

allows  you to perform  any operation on  the time

without  affecting your  calendar  clock.   It  is

useful for dealing with  ill-behaved programs that

need to  change the time and  for testing programs

at different dates and times.   You can later  use

CLOCK Version 3.52                              November 19, 1993

the C=R,  C=A, or other functions  to re-establish

the  value in  your  DOS clock  from the  calendar

clock.

C=R

Connects  the two clocks  for writing and periodic

Reading.   This  function may  be  used when  your

clocks drift  slowly with  respect to  each other.

If the  time has not  been read for about  10 sec-

onds, the calendar clock will be read, adjusted by

the offset if  any, and the adjusted  time written

to the DOS clock and returned to the caller.  This

periodic  Read assures you  that the  clocks never

get  very  far out  of  synchronization with  each

other.   Some  calendar clocks  take a  relatively

long  time to read  and most don't  have 100ths of

seconds.  By only  reading the calendar clock when

no other time request has  occurred in the last 10

seconds,  we minimize  both the  overhead and  any

visibility of time jumps.

C=W

Connects the two clocks for Writing.  This is  the

default  mode of  operation and  is the  same mode

that the DOS CLOCK$  device driver supports as its

only mode.  Reading the  time is served by the DOS

clock.   Writing the time sets  both clocks.  If a

time  zone offset  has previously  been specified,

all  writes to the clock  are adjusted so that the

calendar clock  remains on  the  base time.    For

example, if the  calendar clock is set to  UTC and

you specify that  you want to use CST which has an

offset of 6 hours from UTC, all writes of the time

will have 6 hours added to them  before writing to

the calendar clock.   Thus the  DOS clock and  the

calendar clock remain at a constant time offset.

D=N

The D=  function controls  a continuous time  dis-

play.  D=N turns off the continuous display.

CLOCK Version 3.52                              November 19, 1993

D=x,y,a[,12|,24]

Starts a continuous  time display at cursor  posi-

tion x,y with  display attribute a.   Cursor posi-

tion 0,0 is  the upper left corner of  the screen.

The typical screen format runs from 0,0  to 79,24.

The  display attribute is  a two-digit  hex number

that gives  the background and  foreground colors.

The high-order digit is the background color (0 to

7 usually)  and the  low-order digit is  the fore-

ground or character  color (0 to F).   The default

display is a  24-hour clock in the  form HH:MM:SS.

If  you add the ",12", the  display will use a 12-

hour clock.  No AM or PM indication is given.

D=72,24,79,12 will place the  display at the right

edge of the  last line of most screens.   The text

will be in bright  blue on a white  background and

the hours will run from 12 through 11.

D= by itself will  resume a previous display  that

was turned off  by D=N.   You do  not have to  re-

enter all the values.

The  D= functions  should  not be  used if  CLK is

executed  from within Windows.   If you  try to do

so,  you will  received  an error  message.   This

avoids  a  hung system  which  would occur  if CLK

didn't check.

L=YYYY-MM-DD,hh:mm:ss,�sss.cc

The L= function  is not intended to be used on the

command line, but will  be accepted if you provide

it.   The L=  function is normally  created by CLK

and saved in  the CLK.INI  file for  later use  in

processing adjustments.  The date and  time of the

last adjustment (or the most recent time  you gave

an  A= function on the  command line) are given by

the YYYY-MM-DD,hh:mm:ss.   Any non-digit  except a

space may separate the  sub-fields.  The date  and

time must  be followed by a comma  and any adjust-

ment  remainder.   The adjustment remainder  is in

the same form as the value for the A= function but

CLOCK Version 3.52                              November 19, 1993

will usually be a fraction of a second.  It is the

adjustment  that was  left over  the last  time an

adjustment was  actually  made  since  only  whole

seconds of changes are made to the clock.

The L= function triggers an adjustment  if needed.

Generally, it  will follow  the  A= function  that

sets the adjustment factor.   Both of these  func-

tions normally appear in the CLK.INI file.

The L= function  may also be used by  itself to do

simple arithmetic  on the clocks.   Use it  on the

command line with any earlier date and time and an

adjustment of the  number of seconds  (positive or

negative)  that you want added to  the clock.  The

change will be made  and the last adjustment  time

set to the current time.  For example,

clk l=1990-1-1,0:0:0,-4

will subtract 4 seconds from the clocks.

M=�sss.cc,YYYY-MM-DD,hh:mm:ss

The  M= function is used to help you calculate the

automatic adjustment  A= value  for your  calendar

clock.  The M= function is used to enter the manu-

al adjustments that you make to the clock in addi-

tion to the automatic adjustments.  The �sss.cc is

the amount  of  manual adjustment  that  you  have

made.  This may be adjustments made by setting the

clock  with the DOS DATE  and TIME commands or ad-

justments  made by  a time  setting program.   The

YYYY-MM-DD,hh:mm:ss is the date and time at  which

the  adjustment was  made.   It is  very important

that the  date and time  be given with  respect to

the calendar clock and not the DOS  clock, if dif-

ferent.  If you do  not supply the date and  time,

the  current date  and time  will be  used.   This

should be sufficiently  accurate if you just  com-

pleted making the adjustement.

The first M= value that you enter  is used only to

set a base date and time.  Then as you enter addi-

tional values in the  future, CLK will calculate a

new value for the A= function by  adding the aver-

CLOCK Version 3.52                              November 19, 1993

age manual  adjustment per  day to the  current A=

value.  Up  to 8 values are  saved in the  CLK.INI

file (or other file  that you specify).  The  more

values you provide and the longer the time period,

the more  accurate the  adjustment will  be.   For

example, if you can only determine  the adjustment

to  the nearest second  (likely), you may  want to

have a  week or  more between  manual adjustments.

The only  restriction is  that the first  and last

manual adjustement must be within one year of each

other.  If  you enter more than 8,  the oldest two

adjustement  values are  collapsed  into a  single

value.

Supplying  a new A= value, presumably based on the

calculations from CLK,  deletes all the M=  values

from the file.  You may either specify a /I option

to specify the file and  read all the functions in

the file, a /M option to specify the file and only

read  the M= functions,  or CLK will automatically

specify the  /M option for  you (but it  will only

know to look for CLK.INI in the directory contain-

ing CLK.EXE).  You may also edit the M=  functions

in the  file and use the /M  option to force a re-

calculation.

When CLK  calculates  a new  adjustment value,  it

tries to determine if any of the M= values are out

of  line with  the rest  of the  values.   It will

ignore, but provide  an error message,  if any  of

the values are  too far away from all  of the oth-

ers.  CLK calculates an average and standard devi-

ation and  ignores all  values (of  adjustment per

day) that are more than 2 standard deviations from

the average.

R=A

The R= function sets restrictions on time changes.

R=A inhibits  All time changes.  When this mode is

set, all attempts to  change the time are  ignored

by CLOCK.SYS.  This includes time  changes attemp-

ted by CLK including time adjustments.  See R=N.

CLOCK Version 3.52                              November 19, 1993

R=B-hh:mm:ss

This function limits the amount of Backward change

of the time.   Any change of more than  the speci-

fied time is ignored.  The "-" is optional  as are

the offset fields.

clk r=b

inhibits all backward changes.

CLK R=B::2

only  allows  backward changes  of two  seconds or

less This might permit CLK to perform  adjustments

but effectively stop other backward changes.

R=F+hh:mm:ss

This  function limits the amount of Forward change

of the time.  Any  change of more than the  speci-

fied time is ignored.   The "+" is optional as are

the offset fields.

CLK R=F+1:00:02

limits forward changes  to 1  hour and 2  seconds.

The R=B and R=F limits  do not apply to the  auto-

matic  conversions between  standard and  daylight

savings time  as these  don't actually  change the

time.  The R=B and R=F functions are also indepen-

dent of the R=A function.

R=N

This function reverses the R=A function and inhib-

its No clock  changes.  The R=A  is independent of

the R=B and R=F.  You can set backward and forward

limits with  R=B and R=F.   Later you  can inhibit

all changes with R=A.  When you do R=N, the previ-

ously set R=B and R=F limits again have an effect.

R=R

This function Resets the limits established by R=B

and R=F.  No limits  are in effect after executing

this function.  It has no effect on the R=A mode.

CLOCK Version 3.52                              November 19, 1993

S=C

Sets  both clocks  to  the current  time from  the

calendar clock.   You  can use  this  even if  the

clocks  are  disconnected  to make  the  DOS clock

equal to the calendar clock.  The time zone offset

is set to  00:00:00 and the time zone  name is set

to UTC to indicate that both clocks are set to the

base time.

S=D

Sets both clocks  to the current time from the DOS

clock.  The value from the DOS clock is written to

the calendar clock.   The time zone offset  is set

to 00:00:00 and  the zone name  is not changed  to

indicate  that both clocks  are set to  your local

time.

TZ=...

Sets the DOS clock to the correct local time based

on the value of the expression  following the TZ=.

This expression  conforms  exactly  to  the  POSIX

1003.1 standard  for time zone handling.   CLK as-

sumes that the  calendar clock currently  contains

UTC  (also known  as GMT,  Greenwich, ZULU,  and Z

time).    UTC  ("Universal Time,  Coordinated"  is

usually spoken as "coordinated universal time") is

the  name of the  international standard time that

is provided by  WWV, WWVH, the  Naval Observatory,

and NIST as well as many other services around the

world.    Most  Unix(r)  systems also  support  TZ

although  many only  partially  support the  POSIX

standard.    See the  notes later  for  a complete

description of the TZ= function.

You  can  put  the  CLK  TZ=...  command  in  your

AUTOEXEC.BAT file.  You should put it prior to any

other statements that  may create or update  files

or  you run the risk  of having confusing times in

your  file directory  (MAKE  could get  especially

confused.).  If you  provide a TZ= function,  your

____________________

Unix is a registered trademark of Unix Systems Laboratories.

CLOCK Version 3.52                              November 19, 1993

system  will automatically  adjust to  the correct

time  zone every time you boot.   Note that no at-

tempt is made to change between daylight and stan-

dard  time except  when  you  execute  CLK  TZ=...

Adjusting on the fly is very dangerous.  Many pro-

grams  can produce incorrect  results or even cor-

rupt  your data if  time takes a  big step forward

and are even  more likely  to do so  if time  runs

backward.   See section III.C for  a discussion of

the TZ= format.

W=-back,+forward

The W= function is used to establish Warning  lim-

its  on the time.   Each  time the W=  function is

processed (usually  from  the CLK.INI  file),  the

current time  and the previous  time are compared.

If the  current time is outside the -back,+forward

range around the previous  time, a warning message

is displayed requiring  you to press  a key.   The

program will  also exit with  an ERRORLEVEL  of 1.

The full form of the W= function is:

W=N-days/hh:mm:ss,+days/hh:mm:ss,

YYYY-MM-DD,hh:mm:ss

The days sub-field is a  value from 0 to 250 days.

The days  sub-field  combined with  the time  sub-

fields  gives the maximum interval around the pre-

vious  time within  which  the  current time  must

fall.   The previous date and time follow the sec-

ond comma.  For example,

clk w=-0/00:00:02,+91

will  establish a  range of  minus two  seconds to

plus 91  days.  The first time  you use this func-

tion nothing will happen except  that CLK will add

the current date and time to the command and store

it in the  CLK.INI file.  The next time you use if

from  the CLK.INI  file  with CLK /I,  the current

date and  time will  be compared  to the  previous

date and time and the range.   If the current time

is outside  the range, a message  is displayed and

CLK pauses.

CLOCK Version 3.52                              November 19, 1993

The sign characters and  all of the sub-fields are

optional.  Missing sub-fields are treated as zero.

The particular separators are not important except

for  the commas.   Any non-digit  character can be

used for the other separators.  The time and  date

fields are appended  to the function  by CLK  when

the CLK.INI file  is written and are  the previous

time to which the next comparison will be made.  A

typical  function  might be  W=,31.   This command

will  display a  warning  whenever time  has moved

backwards at  all or  has moved  forwards by  more

than 31 days  since the last time you used CLK /I.

If you use CLK /I (see below) in your AUTOEXEC.BAT

file,  this is  very likely  to detect  a miss-set

clock or bad clock battery.  Yet it won't generate

a message  on  every boot.  You will  only get  an

"unnecessary" message  if it has been  more than a

month since you last used the PC.

If the "N" character (or "n")  appears immediately

after the "=", CLK does not pause and wait for you

to press a key after printing the warning message.

The "N" allows you to  use CLK in a .BAT file  and

check for  the warning with the IF ERRORLEVEL com-

mand.

/B

The B option  tells CLK to remain  in command mode

even if no other functions or options are present.

The  /B in  itself performs  no operation,  but it

avoids  CLK going into full-screen mode if all you

want to do is display the current status (e.g., in

a .BAT file).

/I[,C]

or

/Ifile[,C]

The I option  tells CLK to  read the CLK.INI  file

(or  "file" if that is given) for additional func-

tions.  These functions are usually A=, L=, and W=

functions that were previously written to the file

by  CLK.  You may add  any other functions (but no

options) to the file and they will be processed as

CLOCK Version 3.52                              November 19, 1993

if you had entered them at the end of the  command

line.  If  you don't specify  "file" or you  don't

give a path  as part of  "file", CLK will use  the

directory which  contains CLK.EXE.   Thus the  de-

fault  is to  use  CLK.INI in  the  directory that

contains CLK.EXE.

If you enter an A=, L=, M=, or W= function or  CLK

finds them in  the file, the file  will be updated

with  the new  values  for the  current time  when

appropriate.   If an  adjustment is  made, the  L=

function  will reflect  the date  and time  of the

latest adjustment.   CLK will  update or  create a

CLK.INI file  even if you don't use  the /I option

if you use  the A=, L=, M=, or W= functions on the

command line.

It is very useful to  store your standard TZ=, C=,

and R= functions  in the CLK.INI file.   Then your

AUTOEXEC.BAT file  can  simply contain  a  CLK  /I

command to  set your  clocks  into their  standard

modes, adjust the  time, and check  for bad  clock

values.  You  can also  reestablish your  standard

modes after changes with the same command  entered

from the DOS prompt or a .BAT file.

The file may be hidden  and still be updated with-

out error.  However,  to update a read-only  file,

you must  add ,C to  the end of the  option.  This

indicates that CLK  may Change  the attributes  of

the file in order to update it.  CLK will  restore

the original  attributes.  System files may not be

updated.

/M[,C]

or

/Mfile[,C]

The  M option is identical  to the I option except

that only  the M= function lines are read from the

file.   Its primary  use is internal  to CLK  when

processing  an M= function  from the command line.

You may also use /Mfile to specify the location of

the  file when entering  M= values.   You may also

CLOCK Version 3.52                              November 19, 1993

use  the M  option to  cause recalculation  of the

adjustment factor at any time (e.g., after editing

the file).

/P

or

/PN

Controls the  use of  password protection  of mode

changes.   When you use /P, CLK will ask you for a

new password.  It will then ask  you to repeat the

password to ensure that you entered what you want.

It does  not display the password as you enter it.

The password is then passed to CLOCK.SYS and pass-

word protection is  enabled.  On  future calls  to

CLOCK.SYS, you will  be asked for the  password if

protection is enabled.  You may use /PN to disable

password protection, but you will have to  provide

the current password to  do so.  If used  with the

/S feature, the password protection will remain in

effect even across reboots.  /P should be the last

field on a CLK command line.

/S

or

/Sfile

Saves the current copy  of CLOCK.SYS as it resides

in memory.   If you  do not specify  the file, the

memory version is written to file CLOCK.NEW in the

same directory  that contains CLK.EXE.   While you

may  directly overwrite  the  file CLOCK.SYS,  you

should be very  careful doing so as  whatever your

current  state  is will  take effect  on  the next

boot.

PLEASE   MAKE  A  BACKUP  COPY  of  your  original

CLOCK.SYS before  replacing it with a  saved file.

The saved  file does  not do  any probing  at boot

time as  it already knows exactly  what your clock

is.   It does not  need any parameters on  the de-

vice= line in CONFIG.SYS.

The /S  option may be extremely  useful for anyone

with multiple  similar systems  to maintain.   You

CLOCK Version 3.52                              November 19, 1993

can set  up all  of your options  once on  one ma-

chine, save those with the /S option, and then use

the  saved  file to  directly  install on  all the

other machines.  If  you use password  protection,

no  one can  change  the other  systems  except by

modifying  CONFIG.SYS  to  remove the  loading  of

CLOCK.SYS.

You will  still need to  run CLK as part  of every

boot operation  (e.g., in AUTOEXEC.BAT)  to do any

automatic adjustment (A= and  L= functions), check

for clock failures (W= function), and check to see

whether the clock should use the standard  or day-

light offsets (TZ= function).  These functions are

not  processed automatically by  CLOCK.SYS because

they would  increase the size of the device driver

to an unacceptable limit.

/?

"/?" or any  error will cause  CLK to display  the

usage format.  If you have an error in any  field,

CLK first displays that field.

TZ=:?

"TZ=:?" or any error in a TZ= function that refers

to a  pre-defined time zone causes  CLK to display

all  the current pre-defined  time zone  names and

their associated values.

B.Using CLK with WindowsIf you use the TZ= function  and also use

WINDOWS, you  will  need to  modify your  SYSTEM.INI  file.   The

SYSTEM.INI  file is  found in  your  WINDOWS directory.   WINDOWS

performs a function very similar to the C=R function, but WINDOWS

uses the  BIOS to  go directly  to the  hardware clock  and hence

bypasses  CLOCK.SYS.    The  result  will  usually be  your  time

changing everytime you enter WINDOWS.

Fortunately you  can avoid that  by editing the  SYSTEM.INI file.

In the SYSTEM.INI file, find the line that looks like:

[386Enh]

Immediately after that line or anywhere in the  [386Enh] section,

insert a line that looks like:

CLOCK Version 3.52                              November 19, 1993

SyncTime=false

C.Format  of TZ= FunctionThe format of  TZ=... can get quite com-

plicated  as the standard is  trying to deal  with all time zones

and daylight  savings time  rules  around the  world.   The  full

format is:

TZ=:zone

or

TZ=STD[offset][DAY[offset][,start[/time],end[/time]]]

Parameters     zone

The form TZ=:zone is  the one you are most  likely

to  need.  CLK knows the offsets and daylight sav-

ings time  rules for most of the  U.S. and all you

have to supply is the name of your time zone.  The

time zones currently known to CLK are:

EST  Eastern U.S. with automatic daylight

CST  Central U.S. with automatic daylight

MST  Mountain U.S. with automatic daylight

PST  Pacific U.S. with automatic daylight

HST  Hawaiian with no daylight savings time

MET  Mediterranean with automatic daylight

You can use CLK TZ=:? to display the list  of pre-

defined zone names and their values.

If you would like your favorite time added, please

send me the  names, offset from UTC,  and any day-

light savings  time rules that are  different from

the U.S.  NOTE:  The names can be composed of  any

characters except digits,  plus, minus, and comma.

They can  be up to  32 characters  long.  Case  is

preserved in  the names only for  later display by

CLK.  You can call your zone MyZone.

Case is not important in matching  the zone names.

tz=:cst  is the equivalent of  TZ=:CST.  Both will

find the definition of CST.

You don't have to wait for me to modify CLK in order to

use your local rules or just try something out.  That's

what the long form is for.

CLOCK Version 3.52                              November 19, 1993

STD

is  any time zone name  for standard time that you

want to use.  See the note above for the rules.

offset

is  the difference between your local time and UTC

and is expressed as  hh[:mm[:ss]] with an optional

leading + or -.   Offsets for times West of Green-

wich are positive and those  East are negative.  A

24-hour  clock  is  used with  times  running from

00:00:00  to  23:59:59.   Mountain  Standard  time

would be expressed as:

TZ=MST+07:00:00

or

TZ=MST7

and that's what you would use if you  live in Ari-

zona  and  don't  want any  daylight  savings time

adjustments.

DAY

is the  time zone name for  daylight savings time.

If you  don't use  daylight savings  time in  your

location, you should not supply this or any of the

following fields.

The offset for  daylight time is in  the same form

as  for standard  time.   If you  don't give  this

offset, it is  assumed to be  one hour later  than

standard time.  Thus a more complete specification

of the Eastern time zone would be:

TZ=EST+05:00:00EDT+04:00:00

or

TZ=EST5EDT4

or

TZ=EST5EDT     (since daylight  is  one  hour

later by default)

start

is the day  of the year on which  daylight savings

time starts.  There are three formats that you can

use to give this day.  They are given below.

CLOCK Version 3.52                              November 19, 1993

end

is  the day  of the  year on  which  standard time

starts again.   Note  that in  the Southern  hemi-

sphere it is quite possible for "end" to be earli-

er in  the year than  "start" as  daylight savings

time will include December and January.  end is in

the  same format as  start.  Note  from the syntax

above  that you  must provide  end if  you provide

start.

time

is the  time of day  that the change  takes place.

It  is in exactly the same format as offset except

that leading +  and - signs  are not allowed.   If

you  do not specify the time,  02:00:00 is used as

the default.

The day of the year form for start and end may be any of:

Jn

where n is a Julian day of the year from 1 to 365.

This  form does not allow you to refer to February

29.   Day 59  is always February 28  and Day 60 is

always March 1.  This is most useful  in locations

where  a particular  calendar date  is  used every

year.

n

where n is a Julian day of the year from 0 to 365.

This form  does allow you to refer to February 29.

January 1  is day  0, February 28  is day  58, and

either February 29 or March 1 is day 59  depending

on whether it is a leap year or not.  Not the most

useful of  forms but  it does deal  with locations

that might specify  February 29  as the date  that

time changes.

Mm.w.d

is used in places like the U.S. that do not speci-

fy a  date, but rather  specify a rule.   m is the

month  (1  to  12).   d  is the  day  of  the week

(0=Sunday,  1=Monday, ...  6=Saturday).  w  is the

week of the month  (1 to 5).  Week 1 indicates the

CLOCK Version 3.52                              November 19, 1993

first time that day d occurs in that month.   Week

5 indicates  the last  time that  day d occurs  in

that month  whether it  is the  4th  or 5th  time.

Thus the U.S. rule is

M4.1.0/02:00:00,M10.5.0/02:00:00

or the first Sunday in April at 2AM until the last

Sunday in October at 2AM.

The long form for Central U.S. time is:

TZ=CST+06:00:00CDT+05:00:00,M4.1.0/02:00:00,M10.5.0/02:00:00

or

TZ=CST6CDT5,M4.1.0/2,M10.5.0/2     (just leaving out zeros)

or

TZ=CST6CDT,M4.1.0,M10.5.0          (since 2AM is default)

or

TZ=CST6CDT                         (since  the U.S.  rule is

the default)

D.Using CLKThe most common use of CLK will be to handle your time

zone and daylight  savings time  changes.  You  need only do  the

following:

1.   Set your clocks to UTC.

2.   Include the following line in your AUTOEXEC.BAT file.

CLK TZ=:zone   (where you  use one  of the  pre-defined

zones)

or

CLK TZ=...     (where you define your own rules)

That's it.  Everything else will take care of itself.

The only other time you will need to do anything is if you decide

to correct your calendar clock to a more accurate time.  You will

probably use TIMESET or  one of the other programs  that call the

Naval Observatory or  NIST or maybe just your watch that you have

set accurately.

If you don't want to have to change the configuration of the time

setting program, you  may want to  have it always  set UTC or  at

least your  local standard time.  If you  want to have it set UTC

time, do the following:

CLOCK Version 3.52                              November 19, 1993

CLK S=C C=W

run your time set program

CLK TZ=...     (your normal TZ value)

The  C=W in the first  line is only required  if you normally run

with the clocks disconnected.

Note that if you tell the time set program what your current time

zone is, you don't have to do anything unless you have the clocks

disconnected.

Notes     Leaving your clock on local time                    ________________________________

If you don't  want to  be bothered with  UTC, CLK  will

quite happily work with your calendar clock set to  its

current  value.   For example,  let's  assume that  you

currently have both clocks set to Pacific Standard Time

and you don't  really want to  be bothered changing  to

UTC, but you would like future changes between standard

and daylight time  to be  handled automatically.   Just

use the following statement in AUTOEXEC.BAT:

CLK TZ=PST0PDT

CLK believes whatever you tell it and will quite happi-

ly  operate assuming  that PST  is equal  to UTC.   You

could even  use  CLK with  your calendar  clock set  to

Pacific Daylight Time by using:

CLK TZ=PST1PDT

Avoiding missed days when your PC stays on all night,

and missed years and leap days on type 4 and 6 clocks                    _____________________________________________________

If your  PC is often left on over midnight and seems to

miss the change to a new day, use:

CLK C=R TZ=...

in your AUTOEXEC.BAT file to have CLOCK.SYS periodical-

ly read the time and date from the calendar clock.   If

your clock is a  type 4 or 6 clock  that doesn't update

years automatically and doesn't handle  leap days, then

this setting also  ensures that leaving your PC on over

the end of  a year or across February  29 doesn't cause

you to get the wrong date.

Using CLK.INI to simplify your commands                    _______________________________________

If you always use  the same functions, you may  want to

CLOCK Version 3.52                              November 19, 1993

edit the CLK.INI file that is included in this release.

You  can include your TZ=, C=,  and R= functions in the

file and just use:

CLK /I

in your AUTOEXEC.BAT file.

Adding and subtracting time                    ___________________________

You can pretty much do anything including simple arith-

metic  on your  clocks by  using the  functions in  the

correct  sequence.  Let's  say that you  want to add 12

hours to the  current time in  both clocks because  you

inadvertently set  AM time  instead of  PM  time.   The

following function sequence will do that:

CLK TZ=Igoofed-12 S=D TZ=:pst

The TZ=Igoofed-12 (or use any other name  for the zone)

sets your DOS clock to 12 hours later than the calendar

clock.   (Yes, I know that time zone offsets seem back-

wards since you  subtract them from the  base time, but

that's the way the official standards have them.)

The S=D sets both clocks to  the value in the DOS clock

with a zero offset.

The TZ=:pst should be  replaced by whatever your normal

TZ  function is or even by a  /I to pull in your normal

settings.

If you normally have your clocks disconnected, insert a

C=W function as the first function.

Correcting inadvertent problems                    _______________________________

Just remember that except for the S=D function, CLK and

CLOCK.SYS always  start  with the  assumption that  the

calendar clock is  the base and  do all their  calcula-

tions from that point.   Thus you can play  with TZ and

other functions as much as you want and always get back

to  your normal local time just by using your normal TZ

command.  The calendar clock will not have been affect-

ed  unless you did  something to set  the time or date.

You can  avoid even that problem  by disconnecting your

clocks  (C=D) or  restricting time  changes (R=A).   In

fact, if you have trouble with some  program messing up

CLOCK Version 3.52                              November 19, 1993

your clocks, try the following:

CLK C=D

execute program

CLK S=C C=W (or C=R) TZ=...  (your normal)

If you  have your standard  C= and  TZ= functions in  a

file, you can simplify this as:

CLK C=D

execute program

CLK S=C /I

You  may even  want to make  up batch  files containing

this  sequence to run those troublesome programs.  Some

games are major causes of problems because they set the

DOS  clock  running faster  or  slower  than normal  to

control the pace of the game.  As long  as they restore

the normal rate when they exit, the above sequence will

eliminate any clock drift problems.

Fast DOS clock drift                    ____________________

Some high-speed communication programs  will also cause

the DOS  clock to run slow by  causing the BIOS to miss

clock interrupts.  Fortunately,  a C=R function  should

virtually eliminate problems of this type.

E.Examples     clk A=0.34

sets an adjustment factor of 0.34 seconds per day.  The

A=0.34  function will  be written  to the  CLK.INI file

along with an  L= function with the current  time and a

zero remaining adjustment.  Every time thereafter, that

CLK is run with the /I option, it will  check to see if

it  is time  to adjust the  clocks.   About every third

day, it will add 1 second to both clocks.

clk s=c /i

sets  the DOS clock to  the current calendar clock time

without any  time zone adjustment.   The /i  causes all

functions  in the CLK.INI file to  be added.  If one of

them is a tz function, we have corrected the DOS  clock

for any changes made to it and re-established  our time

zone.  This is a good command line to use following a

CLOCK Version 3.52                              November 19, 1993

clk c=d

and other operations  that may have led to an incorrect

time in the DOS clock.

clk c=r tz=:est r=b-00:00:02 r=f+00:00:02

sets CLOCK.SYS  to read the calendar  clock about every

10  seconds and re-synchronize  the DOS clock  with it;

applies the appropriate Eastern  time zone (standard or

daylight depending on  date); and prohibits changes  to

the clock of more than 2 seconds in either direction.

F.ErrorsWhen errors are  detected, CLK prints a message.  It also

sets the ERRORLEVEL on each program termination as follows:

0    Normal termination or only warning messages.

1    Time outside warning range.  Warning issued.

2    Error in command line or  CLK.INI file functions.  The func-

tion with the error is displayed followed by the appropriate

usage rules.

3    Internal error or system error (e.g., unable to open  handle

to CLOCK.SYS).

G.Future  EnhancementsSeveral enhancements are planned for future

releases of CLK.

-    Full screen interface that  displays the current status

of all the things that CLK and  CLOCK.SYS know and lets

you change them just by editing the screen.  The rather

clunky command  line interface needs  to remain as  its

compact  function definitions  are what  you want  in a

.BAT file.

-    Direct dialing of the Naval  Observatory or NIST to get

the correct  time with automatic  creation of  M= func-

tions from the results.

IV.CLKDEMO.EXEA  simple   version  of  the  CLK  program,  called

CLKDEMO, is  provided in  both executable and  source code  form.

CLKDEMO only provides support for the C= functions and displaying

the  clock status.  It  is included to show  you how to add clock

handling  to your own applications.  CLKDEMO  is written in C and

CLOCK Version 3.52                              November 19, 1993

is compiled with Microsoft C 5.1.

A.CLKDEMO Command LineSyntax  CLKDEMO [C=A|D|R|W] [/?]

The functions provided  work exactly as they do in  CLK.  CLKDEMO

also displays the status and the current times.

Since  CLKDEMO.EXE is merely a subset of CLK.EXE, it isn't terri-

bly useful as a utility in its own  right.  Its value is that the

source code  is  provided.   There are  three  source code  files

provided.   CLKDEMO.C is  the main  program and performs  command

interpretation and execution.  IOCTL.C is a set of functions that

read  and  write the  CLOCK$ device  driver  using the  DOS IOCTL

functions.   IOCTL.H is a  set of definitions  which are used  by

CLKDEMO and  IOCTL.C.   CLKTYP.H is  a list  of  the clock  types

currently supported by these programs.  It is used to display the

type.

V.IOCTL - API TO CLOCK.SYSIOCTL is provided to allow you to build

your  own  applications  to  use  the additional  functions  that

CLOCK.SYS   provides.    There   are  several  APIs  (Application

Programming Interfaces) provided  as part of IOCTL.   These  APIs

are sufficient to perform all of the extended functions.

All  of the  APIs are defined  to be  directly callable  from all

Microsoft compatible  languages including MASM,  FORTRAN, Pascal,

and  BASIC.   Languages other than  C refer  to the  API names as

upper case characters.  Object code versions are provided for the

small,  medium, and  large memory  models.   These may  be linked

directly with your  applications.  The  object code versions  are

SIOCTL.OBJ, MIOCTL.OBJ, and LIOCTL.OBJ for the small, medium, and

large models respectively.

A.Data StructuresData structures are  defined in ioctl.h that are

used in the calls to the IOCTL functions.

CLOCK Version 3.52                              November 19, 1993

/* "modes" holds flags that control the operation of CLOCK.SYS */

struct modes {

unsigned : 9;

unsigned day_light: 1;

unsigned disp_24 : 1;

unsigned disp_tim : 1;

unsigned pw_ena : 1;

unsigned disabl : 1;

unsigned chk_forw : 1;

unsigned chk_back : 1;

};

/* "off" holds a time offset in hours, minutes, and seconds. */

struct off {

int hour;

int minute;

int second;

};

/* "date_time" is a structure that holds a date and time.    */

struct date_time {

int year;

int month;

int day;

struct off time;

int hsecond;

};

/* "time_zone" holds a time zone name and offset.   */

struct time_zone {

struct off offs;

unsigned char zone[32];

};

CLOCK Version 3.52                              November 19, 1993

/* "limit"  is used to  hold the data  that sets time  limits for

changes.    */

struct limit {

struct off back;

struct off forward;

};

/* "disp" is used  to hold the  data that specifies the  location

and attributes of the continuous time display.   */

struct disp {

int x;

int y;

int attribute;

};

/* CLOCK_DATA  is the CLOCK.SYS  data that is  returned by clksta

(AKA CLKSTA for  FORTRAN, Pascal, and BASIC  languages).  Because

we need  to be  language independent, all  of the  data is  word-

aligned.   Only the data types int  and character are used.  Some

of the int data is encoded bits or addresses.  */

struct CLOCK_DATA {

int connected;               /* Connection state          */

struct modes mode;           /* Mode bits                 */

struct date_time cal_time;   /* Current date and time     */

struct time_zone standard;   /* Offset of standard UTC    */

struct time_zone daylight;   /* Daylight offset           */

struct limit rules;          /* Maximum clock movement    */

struct disp display;         /* Time display loc and attr */

unsigned char vers[6];       /* CLOCK.SYS version nnn.nn  */

int clock_type;              /* Type of clock installed   */

int ct_1;                    /* Data for clock type       */

int ct_2;

int ct_3;

int ct_4;

unsigned int clock_seg;      /* Segment CLOCK.SYS loaded at*/

unsigned int clock_long;     /* Size of CLOCK.SYS          */

};

This data structure is treated as the following integer array  in

the FORTRAN and BASIC languages.

CLOCK Version 3.52                              November 19, 1993

BASIC          _____

OPTION BASE 1

DIM CDATA(66)

FORTRAN          _______

INTEGER*2 CDATA(66)

ARRAY INDEX         CONTENTS                 STRUCTURE ELEMENT          ___________         ________                 _________________

CDATA(1)       =    Connection state         connected

CDATA(2)       =    Mode bits                mode

CDATA(3)       =    Year from calendar clock cal_time.year

CDATA(4)       =    Month                    cal_time.month

CDATA(5)       =    Day                      cal_time.day

CDATA(6)       =    Hour                     cal_time.offs.hour

CDATA(7)       =    Minute                   cal_time.offs.minute

CDATA(8)       =    Second                   cal_time.offs.second

CDATA(9)       =    100th of second          cal_time.hsecond

CDATA(10)      =    Standard hours from UTC  standard.offs.hour

CDATA(11)      =    Standard minutes         standard.offs.minute

CDATA(12)      =    Standard seconds         standard.offs.second

CDATA(13) -                                  standard.zone

CDATA(28)      =    Standard time zone LJSF

CDATA(29)      =    Daylight hours from UTC  daylight.offs.hour

CDATA(30)      =    Daylight minutes         daylight.offs.minute

CDATA(31)      =    Daylight seconds         daylight.offs.second

CDATA(32) -                                  daylight.zone

CDATA(47)      =    Daylight time zone LJSF

CDATA(48)      =    Maximum hours backward   rules.back.hour

CDATA(49)      =    Maximum minutes backward rules.back.minute

CDATA(50)      =    Maximum seconds backward rules.back.second

CDATA(51)      =    Maximum hours forward    rules.forward.hour

CDATA(52)      =    Maximum minutes forward  rules.forward.minute

CDATA(53)      =    Maximum seconds forward  rules.forward.second

CDATA(54)      =    x cursor position        display.x

CDATA(55)      =    y cursor position        display.y

CDATA(56)      =    attribute                display.attribute

CDATA(57) -                                  vers

CDATA(59)      =    CLOCK.SYS version LJSF

CDATA(60)      =    Clock type               clock_type

CDATA(61)      =    I/O address, base memory ct_1

segment, or base year

CDATA(62)      =    Read memory offset       ct_2

CLOCK Version 3.52                              November 19, 1993

CDATA(63)      =    Write-0 memory offset    ct_3

CDATA(64)      =    Write-1 memory offset    ct_4

CDATA(65)      =    Segment address          clock_seg

where CLOCK.SYS is loaded

CDATA(66)      =    Length of CLOCK.SYS      clock_long

B.clksta�  Summary

C          _

#include <ioctl.h>

extern int pascal far clksta(struct CLOCK_DATA *cdata);

BASIC          _____

OPTION BASE 1

DIM CDATA(66)

DECLARE FUNCTION CLKSTA%(BYVAL Addr AS INTEGER)

STATUS% = CLKSTA(VARPTR(CDATA(1)))

FORTRAN          _______

INTEGER*2 CDATA(66)

INTEGER*2 STATUS

INTEGER*2 CLKSTA

STATUS = CLKSTA(CDATA)

�  Description

clksta  returns  all  of  the information  about  CLOCK.SYS  in a

structure.   In FORTRAN and BASIC the  structure appears to be an

integer array.  The above  structure declarations can be approxi-

mated in  Pascal.  For a  description of the values  of each ele-

ment,  see the individual APIs below.  The elements not described

as part of the input to other functions are:

unsigned char vers[6];      /* CLOCK.SYS version nnn.nn     */

int clock_type;             /* Type of clock installed      */

int ct_1;                   /* Data for clock type          */

CLOCK Version 3.52                              November 19, 1993

int ct_2;

int ct_3;

int ct_4;

unsigned int clock_seg;     /* Segment CLOCK.SYS loaded at  */

unsigned int clock_long;    /* Size of CLOCK.SYS            */

CDATA(57) -

CDATA(59)      =    CLOCK.SYS version LJSF characters

CDATA(60)      =    Clock type (see type numbers in appendix B)

CDATA(61)      =    I/O address,  base  memory segment,  or  base

year

CDATA(62)      =    Read memory offset

CDATA(63)      =    Write-0 memory offset

CDATA(64)      =    Write-1 memory offset

CDATA(65)      =    Segment address at which CLOCK.SYS is loaded

CDATA(66)      =    Length in bytes of CLOCK.SYS

vers or CDATA(57) - CDATA(59)

Is  the version  number of  the CLOCK.SYS  that is  in-

stalled.   This  is returned  as  a 6-character,  left-

justified, space-filled  (LJSF) character string.   For

example, "3.49  ".

clock_type or CDATA(60)

Is the type number of the clock being used.  See Appen-

dix B for the type numbers.

ct_1 or CDATA(61)

Is the value of the first clock parameter.  This is the

base I/O  address for I/O  bus clocks, the  base memory

segment address  for memory-mapped clocks,  or the base

year for the AT&T clocks.

ct_2 or CDATA(62)

Is the read offset for memory-mapped clocks.

ct_3 or CDATA(63)

Is the write 0-bits offset for memory-mapped clocks.

ct_4 or CDATA(64)

Is the write 1-bits offset for memory-mapped clocks.

CLOCK Version 3.52                              November 19, 1993

clock_seg or CDATA(65)

Is the  segment address  at which CLOCK.SYS  is loaded.

CLOCK.SYS always starts at offset zero in this segment.

clock_long or CDATA(66)

Is the length of CLOCK.SYS in bytes.

� Arguments         cdata

is a pointer to  a location to place the

current    state     information    from

CLOCK.SYS.

�  Return Value

If clksta is successful, the  function returns 0.  Otherwise,  it

returns a non-zero value and  _doserrno is set to the correspond-

ing error code.  Possible values are:

0001h     Invalid  function.    This  should  indicate  that

CLOCK.SYS is not  installed.  It is the error that

the DOS clock device driver would return  since it

doesn't handle these functions.

0005h     Access denied.   Indicates that DOS would not open

a handle for read access.  You may not have enough

handles available.

0006h     Invalid handle.  Should not happen.

000Dh     Invalid data.  CLOCK.SYS reported a bad byte count

or  otherwise did not  process the request.   This

should not happen.

C.setpw� Summary

C          _

#include <ioctl.h>

extern void pascal far setpw(unsigned char *pw);

CLOCK Version 3.52                              November 19, 1993

BASIC          _____

DECLARE SUB SETPW(BYVAL S AS INTEGER)

A$ = "MyPasswd"

CALL SETPW(SADD(A$))

Note that A$ must be at least 8 characters long.  Only the  first

8 characters will be used.

FORTRAN          _______

CHARACTER*8 PW

PW = "MyPasswd"

CALL SETPW(PW)

�  Description

The  setpw function sets  the password  to be used  on subsequent

calls to any  of the following functions.   These functions auto-

matically  supply  the  most  recent  password  when  they   call

CLOCK.SYS.  You  need not ever  call setpw if  you are not  using

password  protection.  CLOCK.SYS will only  check the password if

password protection is enabled through the stmode function.

� Arguments         pw

is a  pointer  to the  password  string.

This string should be  exactly 8 charac-

ters long and must be at least 8 charac-

ters long.  The password should also  be

LJSF  as every character is significant.

Case is significant in passwords.

CLOCK Version 3.52                              November 19, 1993

�  Example

#include <ioctl.h>

main()

{

unsigned char pw[9] = "MyPasswd";

/* Set password to use on subsequent function calls.  */

setpw(pw);

}

D.connec�  Summary

C          _

#include <ioctl.h>

extern int pascal far connec(int *conn);

BASIC          _____

DECLARE FUNCTION CONNEC%(CONN AS INTEGER)

STATUS% = CONNEC(CONN)

FORTRAN          _______

INTEGER*2 CONN, STATUS, CONNEC

STATUS = CONNEC(CONN)

�  Description

The connec function  provides a means  of setting the  connection

state of CLOCK.SYS.

CLOCK Version 3.52                              November 19, 1993

�  Arguments   conn

specifies the new connection mode.   The val-

ues are:

0 = disconnected (C=D)

1 = connected for writes only (C=W)

2  = connected for  writes and periodic reads

(C=R)

3 = connected for all operations (C=A)

� Return Value

If  the return  value is zero,  the function  succeeded.   If the

return value is non-zero, an error has occurred and  _doserrno is

set to the corresponding error code.  Possible values are:

0001h     Invalid  function.    This  should  indicate  that

CLOCK.SYS is not installed.   It is the error that

the DOS clock device driver  would return since it

doesn't handle these functions.

0005h     Access denied.  Indicates that DOS would not  open

a handle for read access.  You may not have enough

handles available.

0006h     Invalid handle.  Should not happen.

000Dh     Invalid data.  CLOCK.SYS reported a bad byte count

or otherwise  did not process  the request.   This

should not happen.

CLOCK Version 3.52                              November 19, 1993

�  Example

#include <stdio.h>

#include <ioctl.h>

main()

{

int conn_write = 1;

int status;

/* Set clock to connected for writes */

status = connec(&conn_write);

if (status)

printf("Error %X in connec\n", status);

}

E.newpw�  Summary

C          _

#include <ioctl.h>

extern int pascal far newpw(unsigned char *pw);

BASIC          _____

DECLARE FUNCTION NEWPW%(BYVAL S AS INTEGER)

A$ = "MyPasswd"

STATUS% = NEWPW(SADD(A$))

Note  that A$ must be at least 8 characters long.  Only the first

8 characters will be used.

FORTRAN          _______

CHARACTER*8 PW

INTEGER*2 STATUS, NEWPW

PW = "MyPasswd"

STATUS = NEWPW(PW)

CLOCK Version 3.52                              November 19, 1993

�  Description

The  newpw  function sets  a new  password  for CLOCK.SYS.   This

function  is not sufficient  to enable password  protection.  You

must  also use stmode to  turn on password checking.   BE SURE to

use newpw before you use stmode to turn on password checking.  If

you turn on password checking without first  setting the password

to something that  you know, you  will never be  able to use  the

CLOCK.SYS functions again until you reboot.

� Arguments         pw

is  a pointer  to  the password  string.

This string should  be exactly 8 charac-

ters long and must be at least 8 charac-

ters long.  The  password should also be

LJSF as every  character is significant.

Case is significant in passwords.

� Return Value

If the  return value  is zero,  the function  succeeded.  If  the

return value is  non-zero, an error has occurred and _doserrno is

set to the corresponding error code.  Possible values are:

0001h     Invalid  function.    This  should  indicate  that

CLOCK.SYS is not installed.  It  is the error that

the DOS clock device driver  would return since it

doesn't handle these functions.

0005h     Access denied.  Indicates that DOS would not  open

a handle for read access.  You may not have enough

handles available.

0006h     Invalid handle.  Should not happen.

000Dh     Invalid data.  CLOCK.SYS reported a bad byte count

or otherwise  did not process  the request.   This

should not happen.

CLOCK Version 3.52                              November 19, 1993

�  Example

#include <stdio.h>

#include <ioctl.h>

main()

{

unsigned char pw[9] = "MyPasswd";

int status;

/* Set new password for CLOCK.SYS  */

status = setpw(pw);

if (status)

printf("Error %X in setpw\n", status);

}

F.rstrct�  Summary

C          _

#include <ioctl.h>

extern int pascal far rstrct(int *bhour, int *bmin, int *bsec,

int *fhour, int *fmin, int *fsec);

BASIC          _____

DECLARE  FUNCTION RSTRCT%(BHOUR%,  BMIN%,  BSEC%, FHOUR%,  FMIN%,

FSEC%)

FORTRAN          _______

INTEGER*2 BHOUR, BMIN, BSEC, FHOUR, FMIN, FSEC

INTEGER*2 RSTRCT

STATUS = RSTRCT(BHOUR, BMIN, BSEC, FHOUR, FMIN, FSEC)

CLOCK Version 3.52                              November 19, 1993

�  Description

The rstrct  function sets backwards  and forwards limits  on time

changes.   Any time  changes  that exceed  these limits  will  be

ignored.  For example,  if a backward limit of 2  seconds is set,

no change that would move the clock backwards more than 2 seconds

will be processed.  Setting the limits protects you agains keying

in a bad time  and agains programs  that may incorrectly set  the

time.   Yet you can still  change the time  within the prescribed

limits.  For example, you  may set a backward limit of  2 seconds

to permit CLK to adjust the time by up to  2 seconds while inhib-

iting  any other backwards changes.   These limits have no effect

on changing between  standard and daylight  times as that is  not

really a change in the "time".

� Arguments         bhour:bmin:bsec

is the backward limit  in hours, minutes

and seconds.

fhour:fmin:fsec

is the forward limit in hours,  minutes,

and seconds.

� Return Value

If  the return  value is  zero, the  function succeeded.   If the

return value is non-zero, an error has occurred  and _doserrno is

set to the corresponding error code.  Possible values are:

0001h     Invalid  function.    This  should  indicate  that

CLOCK.SYS  is not installed.  It is the error that

the DOS  clock device driver would return since it

doesn't handle these functions.

0005h     Access denied.  Indicates that  DOS would not open

a handle for read access.  You may not have enough

handles available.

0006h     Invalid handle.  Should not happen.

000Dh     Invalid data.  CLOCK.SYS reported a bad byte count

or otherwise  did not  process the request.   This

should not happen.

CLOCK Version 3.52                              November 19, 1993

�  Example

#include <stdio.h>

#include <ioctl.h>

main()

{

int bhour = 0;

int bmin = 0;

int bsec = 2;

int fhour = 1;

int fmin = 0;

int fsec = 0;

int status;

/*  Set time  change  limits to  2 seconds  backwards and  1 hour

forwards  */

status = rstrct(&bhour, &bmin, &bsec, &fhour, &fmin, &fsec);

if (status)

printf("Error %X in rstrct\n", status);

else

printf("Time limits set to "

"-%.2d:%.2d:%.2d,+%.2d:%.2d:%.2d\n",

bhour, bmin, bsec, fhour, fmin, fsec);

}

G.stmode�  Summary

C          _

#include <ioctl.h>

extern int pascal far stmode(struct modes *mode);

CLOCK Version 3.52                              November 19, 1993

BASIC          _____

DECLARE FUNCTION STMODE%(MODES%)

FORTRAN          _______

INTEGER*2 MODES

INTEGER*2 STMODE

STATUS = STMODE(MODES)

�  Description

The stmode  function sets  a  variety of  operating modes.    The

argument  is actually a  structure of  bits.  Each  bit specifies

whether an operating mode is on (1) or off (0).

� Arguments         modes

is  set of  bits that  specify operating

modes.  The bit values are:

unsigned : 9;            Unused

unsigned day_light: 1;   0x0200 (512)

unsigned disp_24 : 1;    0x0400 (1024)

unsigned disp_tim : 1;   0x0800 (2048)

unsigned pw_ena : 1;     0x1000 (4096)

unsigned disabl : 1;     0x2000 (8192)

unsigned chk_forw : 1;   0x4000 (16384)

unsigned chk_back : 1;   0x8000 (32768)

� Return Value

If the  return value  is zero,  the function  succeeded.   If the

return value is non-zero, an error has occurred and  _doserrno is

set to the corresponding error code.  Possible values are:

0001h     Invalid  function.    This  should  indicate  that

CLOCK.SYS is not  installed.  It is the error that

the  DOS clock device driver would return since it

doesn't handle these functions.

0005h     Access denied.  Indicates  that DOS would not open

a handle for read access.  You may not have enough

CLOCK Version 3.52                              November 19, 1993

handles available.

0006h     Invalid handle.  Should not happen.

000Dh     Invalid data.  CLOCK.SYS reported a bad byte count

or otherwise did  not process the  request.   This

should not happen.

�  Example

#include <ioctl.h>

main()

{

struct CLOCK_DATA cdata;

/* Set daylight savings time.  */

clksta(&cdata);            /* Get current mode settings. */

cdata.mode.day_light = 1;  /* Turn on daylight */

stmode(&cdata.modes);

}

H.stzone�  Summary

C          _

#include <ioctl.h>

extern int pascal far stzone(int *shour, int *smin, int *ssec,

unsigned char sname[32], int *dhour, int *dmin,

int *dsec, unsigned char dname[32]);

CLOCK Version 3.52                              November 19, 1993

BASIC          _____

DECLARE  FUNCTION  STZONE%(SHOUR%,  SMIN%,  SSEC%,  BYVAL  SN  AS

INTEGER, DHOUR%, DMIN%, DSEC%, BYVAL DN AS INTEGER)

SNAME$ = "32-CHARACTER STANDARD ZONE NAME "

DNAME$ = "Daylight savings time zone name "

STATUS% = STZONE(6,0,0,SADD(SNAME$),5,0,0,SADD(DNAME$))

FORTRAN          _______

CHARACTER*32 SNAME, DNAME

INTEGER*2 STZONE, STATUS

SNAME = "CST"

DNAME = "CDT"

STATUS = STZONE(6, 0, 0, SNAME, 5, 0, 0, DNAME)

�  Description

The stzone function  sets the standard time and  daylight savings

time offsets from  the calendar clock  and the standard and  day-

light time zone  names or mnemonics.   Note that the  offsets are

subtracted from the calendar  clock to get the local  time.  Thus

the standard time offset from UTC to EST is 5:0:0.

� Arguments         SHOUR:SMIN:SSEC

is the time difference  between the cal-

endar clock and local standard time.  It

may be a positive of negative value.  If

negative, all three values must be nega-

tive.

SNAME

is the  standard time zone name  or mne-

monic.  It  must be a 32-character  LJSF

character  string.   As  with  all other

character  strings used  by these  func-

tions,  there  is  no  terminating  NULL

character as  is usual in C.   The other

languages do  not  readily  handle  NULL

terminated strings.   In C  applications

CLOCK Version 3.52                              November 19, 1993

you  may define  the character  array to

contain  33  characters,  but  you  must

ensure that the NULL character does  not

occur in the first 32 characters.

DHOUR:DMIN:DSEC

is the time  difference between the cal-

endar clock and  local daylight  savings

time.   Whether the  standard offset  or

the daylight offset is currently used by

CLOCK.SYS is  controlled by  the setting

of the day_light mode bit (see stmode).

DNAME

is the  daylight savings time  zone name

or mnemonic.  It must  be a 32-character

LJSF character string.

� Return Value

If the  return value  is zero,  the function  succeeded.  If  the

return value is  non-zero, an error has occurred and _doserrno is

set to the corresponding error code.  Possible values are:

0001h     Invalid  function.    This  should  indicate  that

CLOCK.SYS is not installed.  It  is the error that

the DOS clock device driver  would return since it

doesn't handle these functions.

0005h     Access denied.  Indicates that DOS would not  open

a handle for read access.  You may not have enough

handles available.

0006h     Invalid handle.  Should not happen.

000Dh     Invalid data.  CLOCK.SYS reported a bad byte count

or otherwise  did not process  the request.   This

should not happen.

CLOCK Version 3.52                              November 19, 1993

�  Example

#include <stdio.h>

#include <ioctl.h>

main()

{

unsigned char sname[33] = "PST                    ";

unsigned char dname[33] = "PDT                    ";

int shour = 8;

int smin = 0;

int ssec = 0;

int dhour = 7;

int dmin = 0;

int dsec = 0;

struct CLOCK_DATA cdata;

/* Get current status.   */

clksta(&cdata);

/* Set time zone to U.S. Pacific time.  */

stzone(&shour, &smin, &ssec, sname, &dhour, &dmin, &dsec,

dname);

/* Select daylight savings time.  */

cdata.mode.day_light = 1;

stmode(&cdata.mode);

}

I.tdisp�  Summary

C          _

#include <ioctl.h>

extern int pascal far tdisp(int *dispx, int *dispy, int *attr);

CLOCK Version 3.52                              November 19, 1993

BASIC          _____

DECLARE FUNCTION TDISP%(DISPX%, DISPY%, ATTR%)

STATUS% = TDISP(72, 0, &H79)

FORTRAN          _______

INTEGER*2 TDISP, STATUS

STATUS = TDISP(0, 72, 0x79)

�  Description

The tdisp  function sets the  coordinates and  attribute for  the

continuous  time display.    tdisp  does not  start  or stop  the

continuous  display nor does it set  the 12-hour or 24-hour mode.

Those are all provided by the stmode function.

� Arguments         DISPX

is the x  (horizontal) cursor coordinate

for  the continuous  time display.   The

rightmost character on a line is at x=0.

Be sure to leave enough space at the end

of the line to accommodate the entire 8-

chararacter time display.   For example,

x=72 is the last position to  specify on

an 80-character line.

DISPY

is  the y  (vertical) cursor  coordinate

for  the continuous  time display.   The

top line on the display is y=0.

ATTR

is the display attribute  to use for the

continuous time display.  ATTR is usual-

ly  expressed as  two  hex digits.   The

first  (high-order) digit  is the  back-

ground  color (0  to 7)  and the  second

digit is the text or foreground color (0

to  F).   For example, 0A  would display

bright  green letters  on a  black back-

CLOCK Version 3.52                              November 19, 1993

ground.   Background  colors usually  do

not  include the  bright  colors.    The

color numbers are:

0    Black

1    Blue

2    Green

3    Cyan

4    Red

5    Magenta

6    Brown

7    White

8    Grey

9    Bright blue

A    Bright green

B    Brigth cyan

C    Bright red

D    Bright magenta

E    Yellow

F    Bright White

� Return Value

If  the return  value is zero,  the function  succeeded.   If the

return value  is non-zero, an error has occurred and _doserrno is

set to the corresponding error code.  Possible values are:

0001h     Invalid  function.    This  should  indicate  that

CLOCK.SYS is not  installed.  It is the error that

the DOS clock device driver would return since  it

doesn't handle these functions.

0005h     Access denied.  Indicates  that DOS would not open

a handle for read access.  You may not have enough

handles available.

0006h     Invalid handle.  Should not happen.

000Dh     Invalid data.  CLOCK.SYS reported a bad byte count

or otherwise  did not process  the request.   This

should not happen.

CLOCK Version 3.52                              November 19, 1993

�  Example

#include <stdio.h>

#include <ioctl.h>

main()

{

struct CLOCK_DATA cdata;

int dispx = 72;

int dispy = 0;

int attr = 0x79;

/* Get current CLOCK.SYS modes.  */

clksta(&cdata);

/* Turn on 12-hour display at end of top line of screen.

Use bright blue text on a white backgroun.          */

tdisp(&dispx, &dispy, &attr);

cdata.mode.disp_tim = 1;

cdata.mode.disp_24 = 0;

stmode(&cdata.mode);

}

VI.CLOCK.SYS -  ASSEMBLY LANGUAGE  APICLOCK.SYS has  a couple  of

additional APIs (Application Programming Interfaces) that are not

present in the normal DOS CLOCK$ device driver.  These interfaces

are provided  by the  DOS IOCTL  (Input/Output  of ConTroL  data)

functions.   The Output Control Data to Character Device function

is used to set new operating values for CLOCK.SYS for the C=, D=,

R=, TZ=, and /P functions.  The Input Control Data from Character

Device  function is used  to get the current  status and the date

and time from the calendar clock.

A.Opening the  CLOCK$ DeviceIn order  to use the  additional APIs

provided by CLOCK.SYS you must open a handle to the CLOCK$ device

driver.  You do this using the DOS  INT 21H Open File with Handle

(3DH)  function specifying read-write  access.  You  then use the

handle to  invoke the other functions.   Sample assembly language

is:

CLOCK Version 3.52                              November 19, 1993

MOV  AX,3D00H            ; Open with read and write access

MOV  DX,OFFSET filename  ; Pointer to "CLOCK$" ASCIIZ string

; DS:DX must point to the string

INT  21H                 ; Call DOS function handler

;    AX contains the handle on return if carry flag is not set.

B.Sending  New  Values   to  CLOCK.SYSTo  change  the   state  of

CLOCK.SYS,  you  use the  Send Control  Data to  Character Device

(4403H) DOS  function.  You  send data to specify  the connection

type,  modes,   time  zone  offsets   and  names,   restrictions,

continuous  display information, and password.   You may send any

combination  of these in  any order.   You  must always  send the

current password  as the first 8 bytes.   If password checking is

not  enabled, it doesn't matter what the 8 bytes contain, but you

must still send the 8 bytes.

The  data stream  that  you send  to  CLOCK.SYS  consists of  the

current password followed by a selection index, the data for that

selection, another selection index, its data, and so on.

The selection  indices and the  data stream associated  with each

one are:

INDEX     DATA ITEMS     COMMENTS          _____     __________     ________

1         connected      two  byte   integer  (0 : C=D,  1 : C=W,

2 : C=R, 3 : C=A)

2         mode           two byte  integer (see stmode for  a de-

scription of the bits)

3         standard       time_zone structure for standard time

daylight       time_zone structure for daylight time

4         rules          limits structure

5         display        disp structure

6         password       8 bytes LJSF of new password

CLOCK Version 3.52                              November 19, 1993

Sample assembly language to send new information to CLOCK.SYS:

curpw     DB   8 dup(?)       ; Current password

DW   1              ; Change connection

DW   2              ;  to C=R

DW   4              ; Change limits

DW   0,0,2,1,0,0    ;  to R=B-::2,F+1

DW   6              ; Set new password

DB   "MyPasswd"     ;  to MyPasswd (case is important)

DW   2              ; Change modes

DW   0D000H         ;  to enable limits and password

;  also sets standard time and

;  no display

count     equ $-curpw         ; Length to send

...

MOV  AX,4403H       ; Send control data function

MOV  BX,handle      ; Value returned from open

MOV  CX,count       ; Value = number of bytes to send

MOV  DX,OFFSET curpw ; Location of bytes to be sent

; DS:DX must point to the buffer

INT  21H            ;

; OK if carry not set.  CX will equal bytes actually taken.

C.Getting Current  Status of CLOCK.SYSTo  get the current  status

and time,  you use the Receive Control Data from Character Device

(4402H) function.   You can read all of  the CLOCK_DATA structure

or any smaller amount of it that  you need, but you can only read

starting at the beginning

Sample assembly language for reading the clock status is:

MOV  AX,440CH            ; Read control data function

MOV  BX,handle           ; Value returned from open

MOV  CX,count            ; Number of bytes to read

MOV  DX,OFFSET buffer    ; Location to store the data

; DS:DX must point to the buffer

INT  21H                 ; Call DOS

; Data returned if carry not set.

; CX will contain the actual number of bytes transferred.

CLOCK Version 3.52                              November 19, 1993

APPENDIX A:  DEFINITION OF SHAREWARE

Shareware  distribution  gives  users a  chance  to  try software

before  buying it.  If you try  a Shareware  program and continue

using  it, you  are  expected  to register.  Individual  programs

differ  on  details --  some  request  registration while  others

require it, some specify a maximum trial period.   With registra-

tion,  you get anything from  the simple right  to continue using

the software to an updated program with printed manual.

Copyright laws apply  to both Shareware and  commercial software,

and the copyright  holder retains all rights, with a few specific

exceptions  as stated below.  Shareware authors  are accomplished

programmers, just  like commercial authors, and  the programs are

of comparable  quality. (In both  cases, there are  good programs

and bad ones!) The main difference  is in the method of distribu-

tion.  The  author  specifically grants  the  right  to copy  and

distribute the software, either to all and sundry or to a specif-

ic group. For  example, some  authors require written  permission

before a commercial disk vendor may copy their Shareware.

Shareware  is a distribution method, not  a type of software. You

should  find software  that  suits  your  needs  and  pocketbook,

whether it's  commercial or Shareware. The Shareware system makes

fitting your  needs easier, because you  can try before  you buy.

And because the  overhead is low, prices are  low also. Shareware

has the  ultimate money-back guarantee  -- if  you don't use  the

product, you don't pay for it.

DISCLAIMER - AGREEMENT

Users  of CLOCK must accept this  disclaimer of warranty:  "CLOCK

is supplied  as is.   The  author disclaims  all warranties,  ex-

pressed or implied, including, without limitation, the warranties

of merchantability  and of fitness for  any purpose.   The author

assumes no liability for damages, direct  or consequential, which

may result from the use of CLOCK."

CLOCK is a  "shareware program" and is  provided at no charge  to

the  user for  evaluation.   Feel  free  to  share it  with  your

friends, but please  do not give  it away altered  or as part  of

another  system.  The essence of  "user-supported" software is to

provide  personal computer  users with  quality software  without

CLOCK Version 3.52                              November 19, 1993

high  prices, and  yet to  provide incentive  for programmers  to

continue  to develop  new  products.   If you  find  this program

useful  and find  that you  are using CLOCK  and continue  to use

CLOCK after a reasonable trial period, you must make  a registra-

tion payment of $10 to Ronald Q. Smith.  The $10 registration fee

will license  one copy  for use  on any one  computer at  any one

time.  For a registration fee of $25, I will immediately send you

the  latest version of CLOCK  including the source  code and this

document as a WordPerfect 5.1 file.  You may  use the source code

for your own maintenance of  CLOCK or as a learning tool  for any

software  that you develop.  You  may not use all  or part of the

source code in any software that you develop or release to others

without the permission of Ronald Q. Smith.

You must  treat this software  just like a  book.  An  example is

that this software may be used by any number of people and may be

freely moved  from one computer  location to another,  so long as

there is  no possibility of  it being used at  one location while

it's being used at another.  Just as a book cannot be read by two

different persons at the same time.

Commercial users of  CLOCK must register and pay for their copies

of CLOCK within  30 days of first  use or their license  is with-

drawn.   Site-License  arrangements  may be  made  by  contacting

Ronald Q. Smith.

Anyone distributing CLOCK for any kind of remuneration must first

contact Ronald Q. Smith at the address below for authorization.

You are encouraged to pass a  copy of CLOCK along to your friends

for evaluation.  Please encourage them to register their copy  if

they  find  that they  can use  it.   All  registered  users will

receive a copy of the latest version of the CLOCK system.

Send the fees and any inquiries to:

Ronald Q. Smith

11 Black Oak Road

North Oaks, MN 55127-6204

You may also contact me via CompuServe mail at userid  71620,514.

I will  be happy to respond  to any problems  and suggestions for

future capabilities.

CLOCK Version 3.52                              November 19, 1993

You may register CLOCK through the CompuServe Shareware Registra-

tion program.  CLOCK.SYS is found under registration ID 104.  The

fee  for registering  through CompuServe  is $12 (to  cover their

charge) and will be added directly to your CompuServe bill.

CLOCK Version 3.52                              November 19, 1993

APPENDIX B:  CLOCK TYPES

B.1  Type 0 - PC/AT

IBM PC/AT and  all fully compatible  PCs.   Most versions of  DOS

fully support  this clock  interface without  a separate  driver.

Use CLOCK.SYS to provide support for CLK.EXE and CLKDEMO.EXE.  If

you don't  need the  extra functions of  CLK, you  probably don't

need CLOCK.SYS.  However,  there were a couple  of late 8088  PCs

that adopted  the PC/AT  clock standard but  did not  provide the

PC/AT recognition  sentinel.  DOS  will not  find the clock,  but

CLOCK.SYS will.

Amstrad PC1521DD systems use this type of clock.

Automatic  determination checks  location  F000:FFFE for  the hex

value FC.

BIOS calls are used  to read and write the clock values which are

usually  stored in  the CMOS  RAM along with  other configuration

information.  The primary interfaces are:

Get Date: MOV  AH,4

INT  1AH

Returns the year in  CX and month and  day in

DX in BCD  format (e.g., a hex  display would

show:  CX=1991, DX=1009 for October 9, 1991).

Set Date: MOV  AH,5

Load CX and DX as above for Get Date

INT  1AH

Get Time: MOV  AH,2

INT  1AH

Returns hours  and minutes in CX  and seconds

in DH in BCD format.   May also return 100ths

of seconds in DL (e.g., CX=2359, DX=5900  for

1 second before midnight).

Set Time: MOV  AH,3

Load CX and DX as for Get Time

INT  1AH

CLOCK Version 3.52                              November 19, 1993

B.2  Type 1 - Zenith Z-18x portables, Supersport 20

Automatic determination  checks location  F000:FFFA for  the two-

byte hex value 4D32 ("M2").

BIOS calls are used to read and write the clock.

Get Date: MOV  AH,2

INT  1AH

Returns CX=year  (1980 - 2079) in  binary and

DH=month, DL=day.  All values are binary.   A

hex display of the above date would look like

CX=7C7, DX=A09.

Set Date: MOV  AH,3

Load CX and DX as for Get Date.

INT  1AH

Get Time: MOV  AH,4

INT  1AH

Returns CH=hours, CL=minutes, DH=seconds.   A

hex display would show: CX=173B, DX=3B00.

Set Time: MOV  AH,5

Load CX and DX as for Get Time.

INT  1AH

B.3  Type 2 - Various Zenith Data System PCs.

Automatic  determination  looks  at location  F000:800A  for  the

pattern 00,F0,"ZDS".

Get Date: MOV  AH,2AH

INT  1AH

Returns CX and DX as for type 1.

Set Date: MOV  AH,2BH

Load CX and DX as for type 1.

INT  1AH

CLOCK Version 3.52                              November 19, 1993

Get Time: MOV  AH,2CH

INT  1AH

Returns CX and DX as for type 1.

Set Time: MOV  AH,2DH

Load CX and DX as for type 1.

INT  1AH

B.4  Type 3 - Memory-Mapped Clocks

Memory-mapped clock  used on  some Zenith  Data  Systems PCs  and

other similar clocks including many slot-less or  no-slot clocks.

The SMT  (Systems Manufacturing  Technology)  No-Slot clocks  are

supported by this type.

Automatic determination  is to  attempt to  read the  clock.   If

values other than all 0FFH or  0 are obtained, this clock type is

chosen.  All known segments and offsets are tried.

Several different  memory mapping  approaches are  used and  many

different address  possibilities are  supported.   In  general  a

segment address in the ROM region  (C800H through FFFFH) is used.

Offsets from that  segment address are  used to read  bits, write

zero bits, and write one  bits.  Documentation may specify  those

segments and addresses  in several forms.   For example,  segment

F000 with offset F002 is the same as segment FF00 with offset 2.

Each memory reference reads or writes a single bit.   Read opera-

tions  are used  for  all references  with  separate offsets  for

reading a bit, writing a zero bit, and writing a one bit.

Clock segments and addresses currently probed include:

�����������������������������������������������������ͻ

�SEGMENT�WRITE 0  �WRITE 1 �READ �DECR �LAST �VENDOR  �

�����������������������������������������������������Ķ

�F000   � F002    �F003    �F004 �0000 �F000 �(Zenith)�

�����������������������������������������������������Ķ

�FE00   �    2    �   3    �   5 �0080 �C800 �SMT     �

�����������������������������������������������������Ķ

�FE00   �    0    �   2    �   8 �0080 �C800 �SMT     �

�����������������������������������������������������ͼ

CLOCK Version 3.52                              November 19, 1993

Multiple address  combinations may be  tried for each  clock pat-

tern.  The  DECR value is  subtracted from the SEGMENT  value for

another attempt at  probing until  the value equals  LAST.   Note

that  like the  SEGMENT address,  DECR and  LAST are in  units of

paragraphs (16-bytes).  Thus DECR of  80 is equivalent to 800H or

2048 bytes.

When the addresses used  by a memory-mapped clock  are determined

by probing,  the segment and read  address is displayed.   As the

read  address varies  the  most from  one  clock manufacturer  to

another it is a good indication of the type of clock found.

Data is stored in BCD in  bytes that you construct by shifting in

the bits.  The most important data bytes are:

0 = Hundredths of seconds

1 = Seconds

2 = Minutes

3 = hours

5 = days

6 = months

7 = years mod 1980

B.5  Type 4 - Direct Register I/O Bus Clock

Many add-in  clocks on expansion  boards use this type  of clock.

Some built-in  clocks in PCs  are also accessed  similarly.  This

clock was used on many AST Research I/O  boards for 8088 PCs such

as  the Six-Pack Plus,  IO Plus,  etc.  Also  used on  some Tandy

systems (e.g., 1200HD) and the Vendex Turbo-888-XT.

Automatic determination is to attempt to read, complement, write,

re-read, and re-complement  one of the  registers.  If the  first

value  read and the  re-complemented second value  are equal, the

register exists and is presumed to be the clock.

See also the description for types 6  and 7.  If you aren't  sure

which variant you have, specify type 8 and automatic probing will

determine if it  is really a type 4,  6, or 7.   For the greatest

chance of detecting the correct  clock type, supply the base  I/O

address if at all possible.

NOTE:  If the  clock is not correctly initialized (e.g., you have

CLOCK Version 3.52                              November 19, 1993

just replaced the battery), CLOCK.SYS  must initialize it so that

the  counters will start working.   CLOCK.SYS determines that the

clock has  been previously initialized by looking at register 14H

(see B below).  If register B contains  the value 0DEH, the clock

is presumed to be initialized.   If not, CLOCK.SYS will clear all

the registers to zero and start the counters running.  While this

form  of  checking  for initialization  was  commonly  used, your

previous software may  have used a different method.   If so, the

first time  you run CLOCK.SYS  your date and  time may be  set to

zero (1980-1-1 00:00:00).  If  that happens, just enter the  date

and time.  It should work OK after that without resetting to zero

on further boots.

Each clock register is implemented as a separate I/O register ad-

dress.  Clock  base I/O addresses of  240H, 2C0H, 300H,  and 340H

are supported by automatic probing.  Similar clocks at other base

I/O addresses  are supported by  specifying type  4 and the  base

address. The  address range used  is the base plus  0 through 1FH

(e.g., 2C0H through 2DFH).

Important registers are (using 2C0 as the example):

2C1  =  100ths of seconds

2C2  =  seconds

2C3  =  minutes

2C4  =  hours

2C6  =  days

2C7  =  months

2C9  =  month last time clock was read/set

2CA  =  years

2CB  =  initialized flag (0DEH if initialized)

2D4  =  Status

All values are in BCD.  That means that each register consists of

two 4-bit nybbles (a nybble is to a nibble as a byte is to a bite

- I haven't seen this used recently, but as a bit  of trivia, the

first IBM 360 principles of operation manuals used  that term and

that spelling - at least  in a draft copy  that we got in  1965).

Each nybble contains one  decimal digit of the value.   Years are

mod 1980 (i.e., 0 means 1980, 99 means 2079).

Since these  clocks do not keep  the year and do  not handle leap

years for you, CLOCK.SYS does the necessary  calculations to keep

CLOCK Version 3.52                              November 19, 1993

the year  correctly maintained  and to adjust  the date  for leap

days.   The  clocks do provide  a couple  of memory  locations in

which CLOCK.SYS maintains the  year and previous month.   As long

as you boot  your system before a full year has passed, CLOCK.SYS

will correctly maintain the date.

B.6  Type 5 - Mitsubishi 8088 PCs

These were  sold by Sperry as the PC-1 and PC/HT, by Leading Edge

as the Model D, by Mitsubishi, and others.

Automatic determination checks location F000:FFB3 for  the string

"MITSUBISHI".  Later Mitsubishi PCs which used a different  clock

had the same string at a  different location.  We hope all  early

versions had it at this location.

BIOS functions are used to access the clock.

Get Date: MOV  AH,4

INT  1AH

Returns   CL=year  mod  1980,  DH=month,  and

DL=day all in  binary.  Also sets  AL=0FFH if

the clock is busy  and the request should  be

retried.

Set Date: MOV  AH,5

Load CL, DX as for Get Date.

INT  1AH

Returns AL=0FFH if busy.

Get Time: MOV  AH,2

INT  1AH

Returns  CH=hour,  CL=minute, DH=second,  and

AL=status.  AL=0FFH  means busy.   AL=0 means

24-hour mode.  AL=1  means AM and AL=3  means

PM.

Set Time: MOV  AH,3

Load AL, CX, DH as for Get Time.

INT  1AH

Returns AL=0FFH if busy.

CLOCK Version 3.52                              November 19, 1993

NOTE:  Some early Mitsubishi PCs came with fixed disk controllers

that were not DOS compatible.  In addition to the clock modifica-

tions, the DOS for those systems also handled the disk controller

variant.  If you have  such a PC, CLOCK.SYS is not  sufficient to

enable you to  go above DOS  level 3.2.   You will  also need  to

replace your fixed disk controller.   I strongly suggest that you

create a boot floppy for your current DOS level before you try to

install your new DOS level.  Then boot your new DOS from a floppy

disk, do SYS C: and copy COMMAND.COM to C:, then try a disk boot.

If the disk boot fails, boot the floppy you just created and do a

SYS C: and  copy COMMAND.COM to C:  to restore your old  DOS ver-

sion.  At that point, your best bet is to simply give up and stay

with DOS 3.2. If you want to proceed, you must replace your fixed

disk controller.

If SYS  C: gives you  an error  status that indicates  that there

isn't room  for the new  DOS version  (very possible when  moving

from 3.2 to 3.3 or above), you must backup your entire fixed disk

and use  FORMAT C:/S to place the new version of DOS on the disk.

While this is a  hassle, at least if your PC won't  boot from the

disk, the fall-back approach is identical.  There will be room to

put DOS 3.2 or earlier back on the fixed disk.

B.7  Type 6 - Indirect Register I/O Bus Clock

Used on AST Research boards and  others.  Typically uses the same

I/O base address  as type  4.   However the  clock registers  are

selected by sending the register  address to the base I/O address

and then  reading or writing  data at the  base I/O address  + 1.

The internal register numbers and their contents are the same  as

for type 4.

Automatic determination by probing the address register.   If the

low-order bit is one, it may be this type of clock or type 7.  We

then look  further to determine if it  is type 7.   If you aren't

sure which type of clock you have but  believe it is type 4, 6 or

7, specify type 8 and automatic probing will detect which variant

it is.  If at  all possible specify the base I/O  address as that

will increase  the likelihood that  the correct determination  is

made.

CLOCK Version 3.52                              November 19, 1993

I/O  addresses used  are (again  using  2C0 as  the example  base

address):

2C0    =  Address register.  Writing a value to this address

selects the internal register to read or write.

2C1    =  Data  register.  Reading  or writing this address,

reads or  writes the internal  register previously

selected by writing 2C0.

The internal register numbers are (the values to write to 2C0):

1  =  100ths of seconds

2  =  seconds

3  =  minutes

4  =  hours

6  =  days

7  =  months

9  =  month last time date was read or written

A  =  year

B  =  initialization flag

14  =  status

Other processing is the same as type 4.

B.8  Type 7 - Complex I/O Bus Clock

Used on AST Research boards and others.   Typically uses the same

base I/O address  as types 4 and  6.  Type 7 uses  a more complex

register access sequence than type 6.   Type 7 can be detected by

looking to see if the value in register D has bit 1 (value=2) set

which type  6 will not.   If you aren't sure which  type of clock

you have  but believe it is  type 4, 6  or 7, specify type  8 and

automatic  probing will detect  which variant it  is.   If at all

possible specify the base I/O  address as that will increase  the

likelihood that the correct determination is made.

The base I/O address is used as an address register  and the base

+ 1  as a data register.   This is the same as  for clock type 6.

However, the internal registers and  the form of the commands  is

quite  different.   The primary difference  is that  the internal

registers only hold four bits  of data each.  Thus  each internal

register represents a single decimal digit.

CLOCK Version 3.52                              November 19, 1993

The I/O addresses are used as follows (using 2C0 as an example):

2C0     = Address register.  The number  of an internal reg-

ister plus 80H  is written to this I/O  address to

select an internal register.

2C1     = Data register.    Four bits  of data  are read  or

written  using  this I/O  address from  or  to the

internal register  previously selected  by writing

to 2C0.

The internal registers are:

0  =  Units digit of seconds

1  =  Tens digit of seconds

2  =  Units digit of minutes

3  =  Tens digit of minutes

4  =  Units digit of hours

5  =  Tens digit of hours

6  =  Day of week

7  =  Units digit of day of month

8  =  Tens digit of day of month

9  =  Units digit of month

A  =  Tens digit of month

B  =  Units digit of year

C  =  Tens digit of year

D  =  Function register

E  =  Function register

F  =  Function register

B.9  Type 8 - Generic I/O Bus Clock

Type unknown.  Probe only for types 4,  6, 7, 9, and B.  Use type

8 if you are sure that you have an I/O bus clock (i.e., the clock

is on an add-in card) but aren't sure whether it is type 4, 6, 7,

9, or B.  If at all possible,  supply the base I/O address of the

clock as this will greatly increase the likelihood of the correct

handling method being chosen.

CLOCK Version 3.52                              November 19, 1993

B.10 Type 9 - Quadram I/O Bus Clock

This  type  was  used by  Quadram  Corporation  on  the QuadCard,

QuadCard II, and QuadCard 512+.  It may  have been used on others

as well.

Automatic determination checks to see that I/O registers exist at

310H or 210H.

This clock  used four I/O  control registers and  twelve internal

registers.  The I/O registers (using 310H as the example) are:

310  =  Data register (used to move data values)

311  =  Address register (used to select internal register)

312  =  Function register (Read, write, hold counter)

313  =  Control register (Enables read or write function)

The internal registers are:

0 = Units digit of seconds

1 = Tens digit of seconds

2 = Units digit of minutes

3 = Tens digit of minutes

4 = Units digit of hours

5 = Tens  digit of hours - Also uses bit 2 (=4) to indicate

PM if in  12-hour mode and bit 3 (=8) to select 24-hour

mode

6 = Day of week (we don't use this)

7 = Units digit of days

8 = Tens digit of  days - Also uses bit  3 (=8) to indicate

that this  is a  leap year.   This bit  must be  set by

software whenever there is less than 366 days until the

next leap day.  If you  do not boot within that period,

leap day will not be taken.

9 = Units digit of months

10 = Tens digit of months

11 = Units digit of year mod 1900 or 2000

12 = Tens digit of year mod 1900 or 2000

NOTE:  If you have this type  of clock, you can only set the time

to  the minute.  Whenever the time is set, the seconds are always

set to zero.  You should set the time as the minute changes.  For

that reason, you  will not want  to use the automatic  adjustment

CLOCK Version 3.52                              November 19, 1993

capability of CLK with  this type of clock as it  will try to set

the time to the nearest second.

B.11 Type A - AT&T 6300, 6300 PLUS, 6300 WGS

Automatic  probing is accomplished  by trying to  read the clock.

If the read attempt using the BIOS is successful (carry flag  not

set) and the values returned look like a valid time, the clock is

assumed to be present.

NOTE:  AT&T clocks  do not advance the year on January  1.  It is

necessary  to  change the  date at  the  beginning of  each year.

Also,  AT&T clocks always clear the seconds to zero when the time

is changed.  For this reason  you should always set the date  and

time at  the beginning of a minute.   You should also not use the

automatic adjustment feature of CLK with these clocks.

A base year may be specified on the DEVICE= line.  If it is  not,

1992 will be  used.  This  base will be  good for all  dates from

1992-1-1 till 1999-12-31.

BIOS functions are used with this clock.

Get Date: MOV  AH,0FEH

INT  1AH

Returns BX = Days since  base, CH = Hours, CL

= Minutes, DH = Seconds,  DL = 100ths of sec-

onds

Set Date: MOV  AH,0FFH

MOV  BX,Days since base

MOV  CH,Hours

MOV  CL,Minutes

MOV  DH,0

MOV  DL,0

INT  1AH

CLOCK Version 3.52                              November 19, 1993

B.12 Type B - Hyundai 8088 Clock

Automatic determination probes  the control registers.   If they,

appear to be present, CLOCK tries to read  the clock.  Legal time

and date values will select this clock.

This clock uses I/O addresses E0H through EFH as follows:

E0  =  Units digit of seconds

E1  =  Tens digit of seconds

E2  =  Units digit of minutes

E3  =  Tens digit of minutes

E4  =  Units digit of hours

E5  =  Tens digit of hours

E6  =  Day of week

E7  =  Units digit of days

E8  =  Tens digit of days

E9  =  Units digit of month

EA  =  Tens digit of month

EB  =  Units digit of year

EC  =  Tens digit of year

ED  =  Control

EE  =  Control

EF  =  Control

B.13 Type C - Multi I/O Card Clock

This clock is  an I/O clock that  appears at address 338H  in the

one case we have  found it.  It is similar  to, but not identical

to, the Quadram clock in its operation pattern.  The manufacturer

is not known, but  the clock appears  on a board manufactured  in

Taiwan and with a label of Multi I/O Card.   This card has a game

port, floppy controller, printer, clock, etc.

Probing tries to read the clock and compare  the values for legal

ranges.

It uses three  I/O registers.  These are 338H,  339H, and 33AH in

the one  example found.   Register 338H  is a selection  and mode

register used to set read and write modes and select the internal

register.   It  is also  read to  read the  value in  an internal

register.  Register 339H is written to set a value in an internal

register.  Register 33AH is apparently a control register that is

used to start and stop the clock updates.

CLOCK Version 3.52                              November 19, 1993

The internal registers are:

0 = Units digit of seconds

1 = Tens digit of seconds

2 = Units digit of minutes

3 = Tens digit of minutes

4 = Units digit of hours

5 = Tens digit of hours - Also uses bit 2 (=4) to  indicate

PM if  in 12-hour mode and bit 3 (=8) to select 24-hour

mode

6 = Day of week

7 = Units digit of days

8 = Tens digit  of days - Also uses  bit 3 (=8) to indicate

that  this is  a leap year.   This  bit must  be set by

software whenever there is less than 366 days until the

next leap day.  If you  do not boot within that period,

leap day will not be taken.

9 = Units digit of months

10 = Tens digit of months

11 = Units digit of year mod 1900 or 2000

12 = Tens digit of year mod 1900 or 2000

CLOCK Version 3.52                              November 19, 1993

APPENDIX C:  REVISION HISTORY

931119 -  Version 3.52: Fix bug  when 5th day occurs on  last day

of month  so we don't change the  time season too soon.

Fix bug  in displaying correct clock type  when type is

specified on DEVICE= line in CONFIG.SYS.  Change  prob-

ing  for type 4  clocks to increase  likelihood that we

will find them  even if they  aren't initialized.   Fix

bug in continuous display that  can cause it to display

digits in reverse order.

930909 -  Version 3.51:  Allow Quadram clocks  to be set  even if

seconds are non-zero.  Ignore seconds.

930720 -  Version 3.50:  Improve documentation by  explaining the

two types of clocks in  the introduction.  Also put the

Windows note  in a  separate section  so it  is in  the

table of contents and easier to find.

930707 -  Version 3.49: Fix writing to clock type C.

930701 -  Version 3.48: Add  clock type C.  Fix searching for AST

clock I/O addresses.   A bug crept in at  about version

3.40 that could cause addresses to be missed.

930410 -  Version 3.47: Add MET time zone  to list of pre-defined

time zones (GMT-1).  Document use of D=  to resume last

display.  Fix  exit on TZ=:?.   Add documentation about

modifying SYSTEM.INI when using TZ= in conjunction with

WINDOWS.

930403 -  Version 3.46:  Fix time of switch to  daylight or stan-

dard.  It  was based on GMT and should  have been based

on local time.

930311 -  Version 3.45: Correct M= adjustment for  the pending A=

adjustment.

930309 -  Version 3.44: Modified M= to automatically set the date

and time on the L= to the same as the last M= if the L=

is earlier.   It also sets any  remaining adjustment to

zero.   This is because the  clock was adjusted exactly

at the point represented by the M=.

CLOCK Version 3.52                              November 19, 1993

930306 -  Version 3.43: Fixed bug with  M= function not forcing a

read  of the A= functions in  CLK.INI and thus not cor-

rectly displaying the previous and new adjustment.

930304 -  Version 3.42:  Changed time  moving backwards  logic to

only look  for earlier  hours.  At  least one  case has

been seen of a program that set the time backward a few

ticks  by  direct calls  on  the BIOS  and  resulted in

CLOCK.SYS advanceing the day.   Now we only advance the

day if the time  moves backwards an hour or more.  Also

added  the /N option for  CLOCK.SYS that shuts off this

logic  completely if  the  date advances  unexpectedly.

Also fixed a bug in locating  the CLK.INI file when the

A= function is used by itself.

930220 -  Version 3.41: Changed timing window prevention logic in

CLOCK.SYS to  control another possible case  of the day

advancing twice over midnight.

930212 -  Version 3.40: Added  the M= function  and /M option  to

provide  calculation of  the adjmustment value.   Added

help for all functions and options. Added the /B option

in anticipation of  later support of full-screen  mode.

Added the ,C  flag on the  /I and /M  options to  allow

updating a  read-only file.   Added error  messages for

I/O errors when updating the file.  Fixed timing window

in  CLOCK.SYS  that  could  occur  when the  continuous

display  is operational  that could  cause the  date to

advance unpredictably.

930204 -  Version  3.31: Fixed bug  in CLOCK.SYS with  some clock

types causing  system lockups  or  bad data  to be  re-

turned.

930130 -  Version  3.30:  Added support  for  a password  and for

saving the  current CLOCK.SYS  state as  a new  copy of

CLOCK.SYS for future  booting.   Added continuous  time

display.   Changed API  to  support direct  calls  from

BASIC,  FORTRAN, and  Pascal  as well  as  C and  MASM.

Changed status display to be easier to read.

930115 -  Version 3.24: Converted document to WordPerfect 5.1 and

rewrote  much  of  it.   Provide  document  in standard

CLOCK Version 3.52                              November 19, 1993

release that can either be viewed on-line or printed.

930110 -  Version 3.23:  Added support  for R=  and W=  commands.

Fixed handling of CLK.INI file so that we don't  try to

update  it if  it is  read only  or is  in a  read-only

sub-directory.   We also don't  update it  if we  can't

find the  [clk]  section as  it may  be someone  else's

file.

930106 -  Version 3.22:  Fixed bug in  CLK.EXE that caused  it to

move the CLK.INI file to the root directory.

930104 -  Version 3.21: Fixed bug in CLOCK.SYS expecting zero for

100ths of seconds from CMOS clock but some BIOSs  don't

zero the register.  Fixed bug in CLK  with synchroniza-

tion algorithm not correctly waiting for a new second.

930102 -  Version 3.20:  Add support for a command file, automat-

ic adjustment, and improved synchronization.  My thanks

to Eric Smith for the ideas that lead to these enhance-

ments.

921230 -  Version 3.11:  Simplified logic in CLOCK.SYS for check-

ing if calendar clock should be read.

921229 -  Version  3.10: Add support for constant synchronization

with calendar clock.  Fix bug in calculating dates near

the end of leap years.

921226 -  Version 3.02: Recompile CLK.EXE and CLKDEMO.EXE to only

use 8088 instructions.  Version 3.01 inadvertently com-

piled with  80286 instructions which  can lock up  8088

systems.

921221 -  Version  3.01:  Fixed a  couple  of bugs  in  beta test

version 3.00 with non-PC/AT clocks.  Added logic to CLK

to  display the  pre-defined  time zones  on  errors or

request.

921219 -  Version 3.00: Added support for automatic time zone and

daylight  savings time adjustment.   CLOCK.SYS  now has

the  ability to manage the DOS  real time clock and the

battery-protected clock separately.  CLK.EXE is used to

CLOCK Version 3.52                              November 19, 1993

convert a base value  in the battery-protected clock to

a correct local time in the DOS clock.

920614 -  Version 2.24: Fixed bug in probing of AT&T 6300 clocks.

Some of the clocks do  not set AL on successful  opera-

tion.

920604 -  Version  2.23:  Fixed  bug causing  fall  through  into

memory probing when only I/O probing was wanted.

920411 -  Version 2.22: Changed order of probing to probe memory-

mapped clocks last as  they are being spuriously  found

on some Tandy systems.

920401 -  Version 2.21:   Changed pattern of I/O  address probing

to probe all possible I/O addresses for one type (e.g.,

4) before  trying any for  another type.   Try to avoid

finding the wrong type.  Added the type B  clock to the

type 8 probe.

920328 -  Version 2.20:  Changed order of I/O address probing for

clock types  4,  6, and  7  to avoid,  where  possible,

accessing other boards.   Most commonly used  I/O clock

addresses, I hope, are now probed first.

920322 -  Version 2.19:  Improved probing for I/O clocks to check

that legal values are found  in all the data registers.

Several  of the I/O  clocks use  operation sequences so

much like each other that  it is possible to choose the

wrong one if we don't  actually read the time and  date

and see  if the  values are  legal.   This approach  to

probing may possibly result in not finding a clock that

is present but  that has  been totally reset.   Such  a

clock  should be  found on  a second  try to  boot with

CLOCK.SYS as the counters will have advanced by then.

920320 -  Version 2.18:   Added  probing for  generic PC/AT  type

clock on systems that don't have the PC/AT signature in

the BIOS.

920307 -  Version 2.17:  Added Hyundai 8088 clock.

920112 -  Version 2.16:  Added AT&T 6300 clock.

CLOCK Version 3.52                              November 19, 1993

920111 -  Version 2.15:  Fixed memory address used by one type of

SMT No-Slot clock.

920105 -  Version 2.14:   Added  support for a  wider variety  of

memory mapped  clocks including the SMT  (Systems Manu-

facturing Technology) No-Slot clocks.

911208 -  Version 2.13:   Add probing for Vendex Headstart Turbo-

888-XT system and any other  I/O clock at 300H.  Change

I/O clock probing logic to make it  less likely to find

the wrong type by accident.

911129 -  Version 2.12:  Fix handling of Quadram clocks (type 9).

They can only set seconds to zero so generate  an error

if anyone tries to set the seconds to any other value.

911128 -  Version  2.11:    Fix bug  in  handling  clock type  9,

Quadram clocks that left  clock turned off after opera-

tions.   Also fix bug  in busy determination  for clock

types 4  and 6 that could result in a decision that the

clock was not operational.

911121 -  Version 2.10:  Add clock types 6 ,7 and 9.   Add type 8

to do  specific probing for  the various I/O  bus clock

types.

911109 -  Version 2.05:   Fix bug  in leap  day handling for  I/O

clocks.  Fix bug that  caused hung systems when setting

the date or time on Z-18x portables.

911026 -  Version  2.04:  Add support for I/O clocks at addresses

240H  and  340H.   Reduce resident  memory  required by

clock driver  to only that required  for specific clock

type.

911021 -  Version 2.03:   Change  handling of  new day  flag from

BIOS to accommodate  BIOSs that indicate multiple  days

have passed.

911020 -  Version 2.02:  Fix bug in returning  date on clock read

functions.  Added message in automatic determination to

indicate type found.

CLOCK Version 3.52                              November 19, 1993

911010  - Version  2.01:     Added  automatic   determination  of

Mitsubishi  8088 PC clocks.   Fixed handler to keep the

BIOS counter  set properly  for programs  that read  it

directly.   Also  returns  a  finer  granularity  value

(100ths of seconds) on reads.

911009  - Version 2.00:   Modified clock driver to  support addi-

tional clock types.  Thanks to Eric Smith (no relation)

for help in finding out how the Zenith PC clock worked.

911005  - Version 1.20:  Changed prologue and epilogue to conform

to  DOS 5.00 documentation.  Seems  to work OK with DOS

5.0 without the changes,  but we might as well  be con-

sistent to save problems later.

910903  - Version 1.10:  Added display  of current date and  time

when initializing.

910518  - CLOCK version 1.00.   Initial version developed  at the

request of a friend with  a Sperry PC/HT who wanted  to

move to DOS 4.01.

1984    - Write clock  device driver for AST clock  for IBM PC-1.

This serves  as the introduction to what device drivers

and especially the CLOCK$ driver are all about.