DESCRIPTION OF MODEM/FAX COMMUNICATION STANDARDS (v2.3)

by

Steve Meirowsky

---------------------------------------------------------------------------

STANDARD NAME    DESCRIPTION

---------------------------------------------------------------------------

CCITT v.17       Standard for FAX at 14400 bps.

CCITT v.21       Standard for 300 bps.  (FSK).

Bell  103J

Bell  202        Old standard for 1200 bps, half duplex. Leased lines.

CCITT v.22       Standard for 1200 bps. (DPSK).

Bell  212A

CCITT v.22bis    Standard for 2400 bps. (DPSK).

CCITT v.26ter    Standard for 2400 bps with echo cancellation.

Bell  208        Standard for 4800 bps. Used on leased lines.

CCITT v.27ter    Standard for G3 FAX at 4800 bps with fall back to 2400 bps.

CCITT v.29       Standard for G3 FAX at 9600 bps with fall back to 7200 and

4800 bps.  Also v.27ter/v.29 is used by CompuCom modems to

transfer data.  Half duplex.  Reverse direction bps (slow?).

CCITT v.32       Standard for 9600 bps with echo cancellation.  Fall back

to 4800 bps.  Both direction simultaneously.  (QAM).

Backward compatible with v.22bis.

CCITT v.32bis    Standard for 14400 bps with echo cancellation.  Fall back

to 12000, 9600, and 7200 bps.  (TCM).  Both directions

simultaneously.  Backward compatible with v.32.

???   v.32terbo  Standard for 19200 bps with echo cancellation.  Fall back

to 16800 bps and v.32bis.  (TCM).  Both directions

simultaneously.  Announced in January 93.  This standard was

originally known as v.32ter.  This standard was created by

modem vendors including AT&T, Penril, and Data Race.

CCITT v.33       Standard for 14400 with fall back to 12000 bps.  European.

???   v.FC       Interim v.FAST standard for 24000 bps.  Basically this is

similar to v.FAST without the 28800 bps rate.  Rockwell has

has announced this interim standard with other modem makers!

CCITT v.Fast     Standard for 28800 bps with echo cancellation and is not

FINALIZED!  Fall back to 24000, 19200, 16800, and v.32bis.

The first modems are already available but will have to be

upgraded after the CCITT approves this standard in mid-1994.

This will have a much greater capability to adapt modem

speeds to meet existing conditions (more than any existing

modem). This will probably be the last analog modem standard

and has been nicknamed "v.last" by some insiders.  This

standard was originally known as v.32bisbis and rumors say

that it will be called v.34 when finally approved by the

CCITT.  Most modem vendors feel that proprietary non-CCITT

protocols will fade away and v.Fast to take over!

USR HST          Proprietary US Robotics HST modulation.  14400 bps in one

direction and 450 bps in reverse direction.  Fall back to

12000, 9600, and 4800 bps.  A higher speed of 16800 bps is

also available on newer USR modems.

Zyxel            Proprietary Zyxel modulation at 16800 bps.  A higher speed

of 19200 bps is also available on newer Zyxel modems.

PEP              Proprietary modulation at 18000 bps.  This popular high

speed modulation predates the v.32bis standard.  It is

mainly found in UNIX environments.  Telebit has an inovation

in the area of built-in UUCP spoofing.  This spoofing only

support UUCP sliding window of 3.  These modems will not

take advantage of the newer UUCP protocol that use a sliding

window of 7!  This is an asymmetric modulation scheme meaning

that in only communicates in one direction at a time.

Turbo PEP        Proprietary modulation at 23000 bps.  This is based on

multiple carrier signals giving it the ability to adjust

to line conditions in increment much smaller than those

of other modulation schemes.  This also uses an asymmetric

modulation scheme and suffers similar to PEP.

Hays V-Series    Proprietary full-duplex 9600 baud.  Hays its own CCITT LAP-B

error correction and its own scheme for data compression.

EIA Class2 FAX   Fax standard called G2.  ???

ISDN             Integrated Services Digital Network.  This uses fiber optic

cable.  It is currently available in larger cities within

3 miles (has been lengthen) of the central office. Available

in two flavors: Primary Rate Interface service and Basic

Rate Interface service.  Basic, also known as 2B+D provides

users with two 64Kbit/sec channels for voice and data and

one 16Kbit/sec channel used by the carrier for call setup

and network management.  The primary, also known as 23B+D

provides 23 64Kbit/sec channels and one 16Kbit/sec channel.

Some recent west coast subscribers will be paying a one

time $75 fee for a network connection device and $97 per

month for service, which includes two voice lines.

SMDS             Switched Multi-Megabit Data Service.  Uses fiber optics.

In R&D Phase.  ???

HDSL             High-rate Digital Subscriber Link.  Bell Labs and Bellcore

have recently developed this technology.  It is capable of

carryint 1.5megabits/sec over copper phone lines!  Limited

to about 3 miles from the central office.  Benificial since

provides high speed data rate without running new fiber

optic cable to the customer.  This is still in the labs.

Caller ID        Callers phone number is transmitted in v.21 format between

the 1st and 2nd ring.

---------------------------------------------------------------------------

MODE          BPS       BAUD      MODULATION      NOTES

---------------------------------------------------------------------------

Bell103        300       300           FSK        Data. Also Bell108 & Bell113

Bell202       1200      1200           FSK        Data

Bell212A      1200       600         4-DPSK       Data

Bell201       2400      1200         4-DPSK       Data

Bell208       4800      1600         8-QAM        Used by IRS

v.17          7200      2400        16-TCM        FAX

v.17          9600      2400        32-TCM

v.17         12000      2400        64-TCM

v.17         14400      2400       128-TCM

v.21           300       300           FSK        Same as BELL 103J.

v.21 ch2       300       300           FSK        Fax

v.22          1200       600         4-DPSK       Same as BELL 212A

v.22bis       2400       600        16-QAM        Also supports v.22

v.23          1200fwd    300           FSK        Data

75rev     75           FSK

v.26          2400      2400         4-DPSK       Data

v.27ter       2400      1200         8-DPSK       G3 (Group III) FAX

v.27ter       4800      1600         8-DPSK

v.29          4800      2400         4-QAM        G3 (Group III) FAX

v.29          9600      2400         8-QAM

v.29          9600      2400        16-QAM

v.32          4800      2400         4-QAM        Also supports v.22bis.

v.32          9600      2400        16-QAM        Uncoded

v.32          9600      2400        32-TCM

v.32bis       7200      2400        16-TCM        Also supports v.32.

v.32bis       9600      2400        32-TCM

v.32bis      12000      2400        64-TCM

v.32bis      14400      2400       128-TCM

v.33         12000      2400        64-TCM        European Data.

v.33         14400      2400       128-TCM

v.32terbo    16800      2400       256-TCM        Also supports v.32bis.

v.32terbo    19200      2400       512-TCM

v.FC         24000      ????       ???-TCM        Also supports v.32bis.

v.Fast       28800      ????       ???-TCM        Also supports v.32bis.

ISDN         65536     65536       FiberOptic     Digital

The baud rate over the phone line means that more than two tones are used at

the baud frequency to achieve the bps rate.  4-DPSK means four tones...

Most higher rate methods use a DSP to shape the waveform of an 1800Hz carrier.

A single cycle of this carrier can carry up to eight bits of data by varing

both amplitude and phase.

---------------------------------------------------------------------------

NAME      DESCRIPTION OF ERROR CORRECTION AND DATA COMPRESSION

---------------------------------------------------------------------------

MNP1      Hardware error correction.

MNP2      Hardware error correction.

MNP3      Hardware error correction.  Strips off start and stop bits during

transmission over phone line.

MNP4      Hardware error correction.  Fastest way to send compressed files.

Strips off start and stop bits during transmission.  Also adapts

to line conditions using Adaptive Packet Assembly, a method to

match the quality of the connection.  The better the connection

the larger the packet sent.

MNP5      Hardware error correction and data compression (up to 2-to-1

compression on TEXT).  MNP5 and above are proprietary and thus

must be licensed by other modem vendors.  Do NOT use to transfer

compressed files since it may take longer to send!

Remember, the 2-to-1 compression is theoritical and not typical!!!

MNP6      Additional Universal Link Negotiation and Statistical Duplexing

to MNP5 service

MNP7      Hardware data compression (up to 3-to-1 compression on TEXT).

MNP8      Nothing available ???

MNP9      Additional Enhanced Data Compression with v.32 modems  Also adds

Enhanced Universal Link Negotiation, which allows connection to

both MNP and non-MNP modems at the highest performance level.

MNP10     Hardware error correction and data compression.  Useful for data

transmission over cellular telephones or adverse conditions.

v.42      CCITT error correction.  LAPM.  Fastest way to send compressed

files.  Strips off start and stop bits during transmission.

This incorporates a fallback to MNP 1,2,3 if LAPM, the primary

protocol, is not available.

v.42bis   CCITT data compression (up to 4-to-1 compression on TEXT).

Uses a version of the Lempel-Ziv data compression algorithm.

Remember, the 4-to-1 compression is theoritical and not typical!!!

---------------------------------------------------------------------------

GLOSSARY

baud  means Actual rate of symbols trasmitted per second.

bps   means Bits Per Second.

kbps  means Thousand Bits Per Second.

MNP   means Microcom Networking Protocol.  Defined by Microcom.

CCITT means International Telephone and Telegraph Consultative Committee.

FSK   means Frequency Shift Keying.

DPSK  means Differential Phase Shift Keying.

QAM   means Quadrature Amplitude Modulation.  Combination of PSK and AM.

TCM   means Trellis Coded Modulation.

LAPM  means Link Access Procedure for Modems.

TIES  means Time Independent Escape Sequence.  AT commands patented by Hayes.

DTMF  means Dual-Tone MultiFrequency.  The touch tones you hear when you dial.

ASL   means Adaptive Speed Leveling.  Created by U.S. Robotics?

---------------------------------------------------------------------------

HISTORY

v2.3  09/08/93  Added v.FC, PEP, Turbo PEP.  Updated v.Fast.

v2.2  05/11/93  Updated v.Fast, v.32terbo, MNP4-10, v.17, v.32terbo.

v2.1  01/18/92  Added information about v.Fast and digital protocol

from an article in INFOWORLD, Jan.13,1992.

v2.0  01/01/92  Added modulation chart from a modem data sheet.

v1.2  ??/??/91  Added more 9600 and 14400 bps modem standards.

v1.1  ??/??/91  Added fax standards.

v1.0  ??/??/91  First draft.

---------------------------------------------------------------------------

COPYRIGHT

You may ONLY use this on your BBS if you keep the entire file intact!

(You cannot remove the authors name or contact information)

All rights are reserved by Steve Meirowsky and New World Designs.

---------------------------------------------------------------------------

CONTACT

If you have any corrections or additions please contact:

New World Information Service BBS

Steve Meirowsky, SYSOP

PO Box 12217

Wichita, KS 67277-2217

BBS: (316)262-1829, 300-14400 bps (v.32bis since February 1992)

---------------------------------------------------------------------------