SLIPPER and CSLIPPER Release 1.5

--------------------------------

Slipper is a replacement for SLIP8250.  It provides a basic functionality

packet driver using the SLIP protocol.  It is written completely from scratch

using Turbo Pascal and Assembler.  Slipper has (hopefully) been designed for

high baud rate operation with standard non-fifo 8250 chips.

Usage:   slipper [com<digit 1-4>] [vec=<hex vector>] [baud=<speed>]

[ether] [nohwhs] [?]

for Van Jacobson TCP Header Compression, use the alternative form

cslipper ....

options (in any order):

com<digit 1-4>   The serial comm ports 1 to 4.

com1      base=3f8, irq=4  (default)

com2      base=2f8, irq=3

com3      base=3e8, irq=4

com4      base=2e8, irq=3

port=<hex port>  override the com port setting  (0000-FFFF)

irq=<hex digit>  override the com irq setting  (0-F)

vec=<hex vector> the packet driver interrupt vector

default is 60

baud=<speed>     baud rate of the connection

default is the current speed setting.

max = 115200

ether            simulate an ethernet board. (IP and ARP only)

default is non-ethernet

nohwhs           disable hardware handshakes.

keepalive        enable a once per minute keep alive byte to

be transmitted.

?                display the commands.

examples

slipper

- com1, vector=60H, baud=whatever defined, non-ethernet

slipper com3 vec=63 baud=38400 ether

- com3, vector=63H, baud=38400, ethernet simulation

also, you can type

slipper ?

for the commands.

Send any comments or bug reports to peter@psychnet.psychol.utas.edu.au

Version 1.1 release adds support for 16550 chips.  (FIFO trigger level 4 on

receive)

Version 1.2 fixes some 16550 bugs and includes support for CSLIP.  Due to the

bulkiness of supporting cslip, there are two version of slipper provided.  The

standard version is called SLIPPER, and the CSLIP version is called CSLIPPER.

The command line parameters are the same for both versions.

Version 1.3 fixes some bugs in the CSLIP part, and does a slight

tidyup/speedup of some of the code and reduces stack requirements.  Also

added a handle for RARP packet types in the ether simulation.

Version 1.4 fixes a bug dissallowing using a vector pointing to an

IRET instruction, and also adds an option to disable hardware handshaking.

Version 1.5 fixes some more bugs, noticeably, a bug in CSLIP processing of

urgent data.