Instrument Scenery Sets (Copyright 1994 John M. Trindle)

For Microsoft Flight Simulator 5.

Version 1.12

How to use:

----------

Unzip this file into your \FLTSIM5\SCENERY directory.

Run INST-ISS.  Use the cursor keys to move your highlight bar

around, and spacebar to check/uncheck items.  When complete, press

PgDn.

INST-ISS converts *.ISS files to *.BGL (FS5 scenery format) files.

It excludes airports, VORs/ILSs, or Navaids based on the REMOVE.LST

file.

ISS files provide the approach navigation aids, runways, weather

reporting and aircraft placement data for Microsoft Flight

Simulator version 5.0.

These files add ILS, LOC, LDA, and SDF data, as well as marker

beacons.  Very few NDBs or VORs need to be added.

All navaid and runway placement is based on FAA data.  Where

the runway heading contradicts the published ILS approach course (and

there is no documented offset in the approach plate), the runway is

rotated to line up with the ILS.  These differences are the results of

different vintages of navaid versus runway survey data.

You may place yourself near the threshold of any runway in the

ISS data set.  Airports are grouped by state, and named by the nearest

city.

Weather information is available on a valid advisory frequency for the

airport.  The frequency used with usually be the designated CTAF

(Common Traffic Advisory Frequency) for the airport.  When in doubt,

try the ATIS/AWOS frequency, Tower Frequency or Unicom/Multicom

(122.8, 122.7, 122.95, 122.9).  Weather can be received within

15 nautical miles of an airport.

Troubleshooting:

---------------

Symptom: Invalid response when tuned to ILS frequency.

Approach: Use the Nav menu to check the identification of the navaid

you have tuned.  It may not be within the flying area, or you may have

an inappropriate frequency tuned.  In addition, some airports have

multiple approaches on the same frequency.  Using the Nav menu will

allow you to choose which approach should be used.

Symptom:  Unable to tune exact COM frequency.

Approach: Make sure you can tune to the nearest 0.025 MHz.  This

option is handled in Preferences.  The frequency may be listed

as 119.925, but is processed and displayed by the program as 119.92.

Symptom:  Conflicts with existing scenery.

If you find an airport which is duplicated in ISS and default or

add-on scenery, you may eliminate the ISS version by editing

REMOVE.LST and re-installing the region.

REMOVE.LST has lines of the following formats:

[WORLD?/Default Scenery]

This describes a PACKAGE.  The FS5 default scenery has BGL files

which fit the format WORLD?.BGL (i.e., world1.bgl, world2.bgl, etc.).

The / mark separates the filespec from the description of the package.

All items listed after a line starting with square brackets will be

applied to installation if that package is checked.

; Text

Lines starting with a semicolon are comment lines and are ignored

by the installation programs.

aIAD

Lines starting with the lower case letter 'a' indicate an airport

(and associated scenery) to be excluded.

vHCM

vI-PHF

Lines starting with the lower case letter 'v' indicate a VOR or ILS

to be excluded.

nNLC

Lines starting with the lower case letter 'n' indicate a NDB

to be excluded.

Licensing Policy:

----------------

This data is licensed as SHAREWARE.  That is, you may try before you

buy.  You may copy and distribute this file in its entirity.  You may

reach me at 70303,3506 at Compuserve or 70303.3506@compuserve.com

If you use ISS scenery, please send a check to:

John Trindle

T-Square Company

4056 Midlands Road

Williamsburg, VA 23188

Include your name, address, telephone number, and the product you are

registering.  E-mail address if applicable.

Single Region (NE3)             - $10

Up to 4 Single Regions          - $20

Continental US                  - $40

Registering gets you on our mailing list... and free updates

for your registered area for 2 years (updates approx every 6 months).

Region list:

NE-1:     ME, NH, VT, RI, MA, CT

NE-2:     NY, PA

NE-3:     NJ, DE, MD, WV, DC, VA

SE-1:     KT, TN, MS, AL

SE-2:     NC, SC, GA

SE-3:     FL, US Virgin Islands, Puerto Rico

EC-1:     MI

EC-2:     IN, OH

EC-3:     WI, IL

NC-1:     MN, ND, SD, IA

NC-2:     MO, NE, KS

SC-1:     OK, AR, LA

SC-2:     TX

SW-1:     NE, UT, CO, AZ, NM

SW-2:     CA

NW-1:     WA, OR, ID, MT, WY

>>>Updates to ISS:<<<

v1.01 addressed ILS reception problems in the original files.

v1.02 reduced "floating aircraft" problems and reduced Type 2

file size.

v1.04 adds displaced thresholds and the proper variety of approach

lights.  Boundary problems in some western areas have been solved,

and much of the "floating runway" or "floating airplane" problems.

v1.05 restores Runway numbers, which have been missing since the

beginning.  Oddly, no one reported this error.

v1.06 changes terrain again, plus fixes turf runways with NE/SW

type runway IDs and eliminates the cutoff of long airport names in

the placement menu.  Also adds back in "C"enter "L"eft and "R"ight

runway identifiers.

v1.1 provides installation with selective exclusion.

v1.11 sped up the Scenery install, fixed ATIS frequencies for

a few hundred airports, and fixed ILS frequencies for 7 of the

16 regions (all East of the Mississippi except for NC2).  VASI

and Glide Slope touchdown spots for runways with displaced

thresholds have been fixed.

v1.12 incorporates the changes made in ISSFIX (that is, glide

slopes are restored, and Phantom Runway 18 is eliminated from

ATIS messages).

v1.13 restored marker functionality which disappeared with v1.1

(#@%#^$!!!) and allows the autopilot to follow the localizer.