HANDS ON Quick Start and Quick Fixes      Page 1

This Product is Shareware, if you continue to use it at all

Please Register and legalise your usage.

_______

____|__     |               (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    Member

February, 1995 This is yet another EASY UP AND RUNNING release

for Hands On.  It is easier to "get going" than previous

releases.  Try it.

Quick Start

Getting the System Up and Running in three parts:

1   Setup       - configuring to your hardware.

2   Operation   - initial test driving.

3   Quick Fixes - when the d**n thing doesn't work.  Yikes!

1   Setup

This is a 'fallback' from the full setup.  Try the full setup

first, either from the HOMENU.BAT menus or by typing in

"1STHAND SETUP".

Version 5.1 of Hands On includes the optional (well maybe not

if someone has removed the choice) HOMINS.EXE setup utility

that dramatically reduces the choices.  You can run it to

simplify the setup, then edit the appropriate .HO files

(HOHOST.HO or HO.HO) for your specific needs.  You can also

save HOHOSTIN.EXE somewhere, then rename HOMINS.EXE to

HOHOSTIN.EXE.  The automated setup will use the 'new'

HOHOSTIN.EXE instead of the original.

HANDS ON Quick Start and Quick Fixes      Page 2

The installation and setup automatically guides you through

the process described here.  But if you have problems then you

will want to know more about whats going on.  Hey, I like to

know "whats going on" in any case, so my guess is that other

people will like the idea too.  Therefore both the easy

menuing description and the lower level description are

provided in most cases.  Please, there are often some

alternate ways to get from 'A' to 'B', and only one or two are

suggested here.  Some people will immediately see better

routes than those suggested here.  Please use whatever does

the job best for you in your situation.  The product is

designed to be very customizable to meet your specific needs.

Run the 1STHAND.EXE program first.  It is designed to help you

get going.  For a directed setup use "1STHAND SETUP" and it

will guide you through the setup.  The SETUP option in the

HOMENU does this for you automatically.  You may need to

peruse the HO.DOC manual and tailor the final part of the

installation to meet your needs.  When you are in the HOHOSTIN

portion of the setup, please be sure to create a consistent

.HO file:  (basically HO.HO for HO.EXE and HOHOST.HO for

HOHOST.EXE).  Some of you will have noticed that the basic

setup using HOHOSTIN does NOT provide access to everything.

This is to keep things simple.  If you need to do 'extra'

things like setup for DIRECT access instead of MODEM access,

then you need to edit the matching .HO file (HO.HO for HO and

HOHOST.HO for HOHOST).

You may have already used HOHOSTIN to configure the host or

remote, and may prefer to bypass this paragraph.  The first

thing is to ensure that the modem works with the software that

came with the modem.  They are basically guaranteed to be

compatible.  If/Once you know that they work together, steal

the settings from the original software (write them down or

print screen them) and transfer the values to HANDS ON.  Start

up hohostin.exe and set it to match your recorded values.  The

automatic setup includes about 50 modems in its database, so

you might look there for something close enough.  (If you call

your own number, hang up after the dialing is completed so the

phone company can call the phone.) If it fails consider

changing from port 1 to 2 or vice versa.  If it still fails

after that check out the quick fixes below.  If you have

arrived here during the HANDS ON installation process, you can

simply try dialing, and change the port setting with hohostin

if the call fails.

Peruse and configure HOMODSET.DAT with your text editor, then

HANDS ON Quick Start and Quick Fixes      Page 3

run HOMODSET.  This not only sets the modem into a reasonable

state for HANDS ON, but in doing so it will confirm (if

everything is OK) or deny (if it reports an ERROR) that you

have indeed selected the right address and interrupt for your

modem.  There is no point in proceeding further until HOMODSET

will run and report everything OK.  You can tinker with the

settings in HOMODSET.DAT to customise your arrangements.  For

example "ATS0=0" will tell your modem to NEVER answer the

phone.  "ATS0=3" will tell it to answer on the third ring.

Reinstall, or reconfigure, to another COM port with hohostin

if it continues to report ERRORs.  This ensures that you have

the correct COM port and the modem correctly setup.

The modem MUST be set to echo the results back to the screen:

"CONNECT 19200" etc.  Change the initialization string until

it does.  The modem database only provides simple strings, to

improve reliability.  You will need to add bits for tuned

performance with other known modems.  We have supplemental

information for this purpose, if you ask.

2 Operation - initial test driving.

You will quickly discover that Hands On's automatic menuing

dialing and the like are designed to be very open.  There are

two main reasons for this openness:

A) many users only ever call one number and can include the

number in the particular .HO file or with HODIAL (for the

Host) and completely automate the entire connection.

B) consultants have their own databases by client and phone

number that are likely to be used with a batch file to

automate dialing.  Many people use variously named .HO setup

files.  You can also use the TeleShare TSPHONE menu/database

for this purpose.

Hands On requires more initial setup, and then provides a more

productive, and easier, low memory, usage than conventional

retail competitors.

7.1 Host Computer - and HOHOST

Either use HOMENU or directly run the HOHOST program.  If the

Remote computer is to originate the 'phone call, then this may

be all that is necessary.  Although, its often useful to

include an ATtention command "ATA\r" in the connect string to

HANDS ON Quick Start and Quick Fixes      Page 4

get the modem's attention.  If the Host will originate the

call then you might start HOHOST with a .HO file (HOHOST.HO is

the default name) that contains a 'phone number in the connect

string so that number will be dialed automatically.  Or you

can use the HODIAL to dial numbers.  Third, you can select

1STHAND'S PHONE menu entry and you will be given some helpful

options with F1 context sensitive help.  This last choice is

best if you will want to call more than one other number.  The

1sthand scheme is designed more for the remote to call the

host, but both directions work with the differences outlined

in the on-line help.  The 1sthand dialing scheme actually

creates a MAKECALL.BAT batch file that the homenu

automatically runs.  So if you directly tell it to dial and it

just quits, look for the makecall.bat that has been created

and run that.  This scheme allows you to integrate this

capability into other software.

Once HOHOST is resident and the call is established, the host

computer can be used normally.  The only difference is that

everything that appears on the screen is also being repeated

on the Remote computer's screen.

The best way to remove HOHOST from memory is to run HOREMOVE.

The HOMENU set includes this option.  It is also possible to

use utilities such as marknet/relnet.  Note that some common

mark/release utilities can NOT be used, because they do not

restore the interrupt mask register.  You will find the

RS232OFF.EXE program in the distribution, which can be used to

reset your machine's COM ports, if you drive the machine's

settings wonky while playing around.

If you are using passwords, then ensure they are EXACTLY the

same on both ends.  Try to avoid such 'extras' until you get

up and running.

7.2 Remote Computer

You can use an HOMENU option to start the REMOTE or:

Use the command:  HO [will use HO.HO configuration file]

or  HO filename      [use filename.HO configuration file]

If the Remote computer was to originate the call, then the

modem initialization string in the configuration file will

usually include a dial command to call the host computer.  If

you want to call "Mike's" machine you can create a MIKE.HO

HANDS ON Quick Start and Quick Fixes      Page 5

setup file that contains configuration information including

the phone number and password for that specific connection.

You would then use "HO MIKE" to call that machine.  Use the

PHONE menu selection for this kind of easy .HO file setup, and

when you want to be able to call more than one other number.

If the host computer was to originate the call, then the

initialization string may contain an answer 'phone command.

Note that in this case it is important that the HANDS ON

program not be run until after the 'phone starts ringing!

(You might try just adding the "ATA\r" to the initialization

string just to see what happens (the modem whines a lot).) A

MUCH better way is to have a blank initialization string, and

include a trigger string to respond to RING from the modem - A

sample trigger string for the bottom of a .HO file follows:

"RING" "ATA\r"  "" 40000

This can be translated as follows:  When the modem finds

somebody calling it transmits a "RING" that the Hands On

software detects and answers with the "ATA\r" string.  The

next "" is a blank follow-up response.  The 40000 is a delay

factor.  For more information see scenario A7 in HO.DOC.

If the configuration file contains no modem initialization

string, then the user at the remote computer can enter modem

commands manually.  (You would type in 'ATA' then tap

Enter/Return) This is an alternate scheme for a consultant

wishing to dial many different clients.  (Before carrier is

detected, HANDS ON behaves much like a normal communications

program.)

Once both HANDS ON and HOHOST are running and connected to

each other, they will automatically adjust the host computer's

"baud" rate (if this feature is enabled) and the host computer

will check the password provided by the Remote computer.

Provided the password is correct, the Remote computer will

effectively be able to take over the host computer, or

alternatively to observe whatever the host computer is doing.

Any key typed on the Remote computer will be acted upon by the

host computer, and anything that appears on the host

computer's screen will also appear on the Remote computer's

screen.

It is sometimes necessary to execute a command on the Remote

computer, rather than transmitting the keystroke to the host

computer.  This is achieved by holding down some combination

HANDS ON Quick Start and Quick Fixes      Page 6

of left shift, right shift, ctrl and alt all at the same time

(the exact combination is specified in the .HO file currently

in use).  When you do this a list of all the possible commands

will be displayed, and you can then type the required command

key:

3 Quick Fixes - when the d**n thing doesn't work.

The most basic and common problems that you are likely to bump

into are related to the modem.  The following advice should be

considered like a sandwich, with general advice like this

above and below and various likely materials in between.

Dvorak's Guide to PC Telecommunications has a list of the most

common problems, which has been interpreted and amplified

below.  Remember that modems are NOT completely standardized

and do NOT provide useful clues to less experienced users.

When you ran HOHOSTIN in the directed installation did you

P. Create consistent .HO file (eg. HO.HO or HOHOST.HO.  Do it!) ?

If not please rerun HOHOSTIN itself, or via 1sthand, and

create both HO.HO and HOHOST.HO.

Once again, run HOMODSET, as described above, then consider

if:

1) Call waiting has not been cancelled.  You can often dial

70# or *70 or try setting both modems to ATS10=255 for a long

wait to bypass call waiting.  Try such long delays if you are

using a cellular phone.

2) An external modem cable is bad or loose.  Symptoms vary but

include behaviour that may lead you to check that ANSI.SYS is

installed.

3) The phone line is disconnected at the wall or the modem.

Test push the connections together.  We know someone who took

days to realise that putting new insulation into the

building's walls wrecked the phone line extension.

4) The communications software is incorrectly setup.  Try

N81.  It has to be trying to work with the correct port,

preferably COM1 or COM2.  Try changing ports.  A fair number

HANDS ON Quick Start and Quick Fixes      Page 7

of P.C.s cannot use COM3 or COM4 for communications (or

anything else that matters).  See 10) below.

5) Modem switches or jumpers are incorrectly setup.  Is the

modem switched on?  (Our most common problem is leaving the

external modem turned off for the first call of the day).

6) The modem is competing with another device for a COM port.

No you cannot physically attach a mouse and a modem to the

same port, but you can do so with software.  Test by

deactivating the mouse, or scanner, or whatever software it

might be.  Look at the back of the machine to see what is

attached to which port.  Try another COM port.

7) Its the wrong type of cable.  Sorry you cannot tell

visually.  Direct connection and modem cables look alike, but

are wired differently.  We're experts because we also use the

Little Big Lan network with serial connections (highly

recommended for basic networking).  Just because XYZ software

DOES work with the cable does NOT mean that the cable is

'ok'.

8) Switches inside the computer are set incorrectly.

Conflicting computer interrupts can cause significant hair

loss in people trying to solve the problem.  We supply the

RS232OFF.EXE to reset interrupts that have gone nuts (You just

type in "RS232OFF").  Yes, the original settings can all be

fine, but some other software zonks the interrupts.  A common

example is Procomm which apparently resets more ports than

just the one it is using.  Microsoft's MSD.exe program is

commonly available for interrupt checking.

9) There is a BIOS problem.  AMI bios's have problems with

16550 chips.  P. C. Softsmith has found common problems with

16550 chips user inserted into cards.  Many cards are really

designed to work with older chips and do not let a 16550 chip

activate properly.  Normal diagnostics (and technicians) will

pass everything, but a separate test will find the 16550

acting like an older chip.  If you buy the card with a factory

inserted 16550, they should match and work fine.  We can

supply, or you can download, software from a BBS that 'fixes'

the AMI bios to 16550 problem.

10) The software is incompatible with the modem.  Their two

features sets fail to overlap in some crucial area.  Try some

other software, even a simple dialer, to make sure you have

correctly isolated the problem.  We find this problem when

people upgrade from something like a 2400 baud modem to a

HANDS ON Quick Start and Quick Fixes      Page 8

14400, or return to using their spare 2400 baud modem.  Change

the settings to match the modem.

11) The modem is broken or defective.  We have found some

modems that overheat then gradually fail are very good tools

for detecting problems with telephone lines.  Voice checking a

phone line does not indicate its condition for a modem, but

your 'phone company can likely check it remotely from their

office.

12) There are missing portions on the screen when running the

remote access software over 19200 baud.  Use a tool like MSD

to check if the COM chips are 8250's, and upgrade to 16550's

if they are.  Or slow down.  Other software can also

interfere, so check your crop or fleet of resident or TSR

software and prune it judiciously.  We generally find that we

can use Hands On at 38400 over any direct link including XT's

to 386's, but your situation will be different.  We use 115200

baud daily between two development machines.

13) A modern high speed modem will work fine, but may not be

able to 'talk' to an older 2400 modem if the initialization

strings are not compatible.  Use a SIMPLE initialization

string on the high speed modem.

14) We have noticed problems with a 14.4K modem on a 386-40

using an older 8250 chip.  Its ALWAYS a good idea to upgrade

to 16550 chips.

15) There are some manuals of various kinds included in the

default HANDS ON distribution.  Some judicious grubbing about

will drag out a range of other tests and tools.

Try to get anything working, then steal its

settings/information for the other things that you want to get

running.  If you have the system basically working, then keep

changing small portions out from the working core until they

work.  As an example, we often just erase the setup file and

restart when a problem continues, then rebuild with

HOHOSTIN.EXE (which is available via 1sthand.exe if you

wish).  Many problems disappear once everything is reset to

original defaults.

Above all smile and keep trying.  Its a challenge not a

problem.

vic

HANDS ON Quick Start and Quick Fixes      Page 9

Vic Williams

P. C. Softsmith                               ASP member/author

606 - 6455 Willingdon Ave                     (604) 433 - 5189

Burnaby, B. C., Canada                        CIS 75020,2664

V5H 4E4                    INTERNET:vic_williams@mindlink.bc.ca