ERRORS.DOC                                           19 August 1992

----------

This is the unofficial compendium of error messages that may appear

during AutoCAD sessions.  They have been assembled from earlier files

and subsequent messages.  Included are short descriptions of what may

cause such errors and what, if anything, can be done to correct the

offending situation.  Any error messages not appearing in this document

and their solutions can be sent to Brian Boatright at 76216,3252. This

file will be updated periodically.

Errors Encountered during Drawing Initialization:

------------------------------------------------

Command list interruption (a, b)

Indicates the use of a lisp command before the drawing environment has

been fully initialized.  Although I'm not sure, I think the first number

"a" refers to the nesting level and the second number "b" refers to the

command used.  The error can occur in ACAD.LSP, or a lisp routine loaded

from ACAD.LSP, or from a menu's *.MNL ... in all cases, the error

indicates that a lisp expression like (command) or (setvar) is being

used outside of a (defun).  The best policy is to contain all lisp

expressions in (defun), or else in the special S::STARTUP (defun).

EREAD     Entity Read

One of the most common errors, this generally occurs when AutoCAD is

reading the drawing file for editing purposes. As AutoCAD unpacks the

drawing database, it may encounter a corrupted entity, or a sequence of

bytes that constitutes gibberish in terms of AutoCAD's structured,

indexed database, or an unexpected entity. This may also occur when the

drawing database is being written back out.  Drawing files can be

corrupted due to a hardware error, power surge or abnormal AutoCAD

session termination. Common culprits are bad data sectors on floppies or

hard drives, corrupted archives, and drive controller errors. The

following indicate the specific causes:

EREAD 1 =  Entity size exceeded the size indicated in the header.

EREAD 2 =  Pointer found where the entity record was expected.

EREAD 3 =  Disk error while reading an entity.

EREAD 4 =  Disk error while writing an entity.

EREAD 5 =  Tcode out of range (see Tcode).

EREAD 6 =  CRC error after initial successful drawing load.

Usually the error occurs after the drawing is nearly loaded and AutoCAD

is regenerating the drawing.  At this point one can usually Ctrl-C out

of the regen and get to a command prompt.  Sometimes a save or a

wblock-save will filter out the error.	Occasionally a DXFOUT and

subsequent DXFIN into a new drawing have worked.  If these fail, usually

the only recourse is to use the backup file. In a later section recovery

methods are discussed and a number of programs mentioned that may, in

some cases, recover the original drawing file.

EREGEN    Entity Regen

Indicates that AutoCAD has encountered an error while generating a

specific entity. Once an entity is read, it has to be generated on

screen. This means reducing its component fields and geometry into

a sequence of graphics requests.  If the parameters associated with

the entity being generated result in an error, AutoCAD will abort.

An example of this would be an out-of-context entity such as a

vertex encountered while no "polyline" entity was being generated.

GETSM

Similar to SMIO

HMATH     High Precision Math

Indicates that one of AutoCAD's mathematical library functions has

generated an error.  A negative circle radius (caused by a corrupted

database), or invalid rotation angles, for example, may result in these

errors.  Editing a DXF file and scanning for abnormal numbers generally

identifies the problem.

LTINIT    Linetype Table Initialization

Indicates that AutoCAD has encountered an error while reading and

initializing the linetype section of the drawing file.  This may be

remedied by entering a <Ctrl-C> as soon as the Drawing Editor

begins regenerating the drawing file, and then using the DXFOUT

command to create a DXF file.  One can then examine the DXF file

for invalid linetype descriptions/entries.

MENU (1,3)

Release 12 reorganized the ACAD.MNU into ACAD.MNL (for AutoLISP

routines), and ACAD.DCL (for dialog boxes).  You need to run the

ACADR12.LSP in your ACAD.LSP to be able to load old menues.

: OPEN ERROR -- File not found

If you see this error message while loading ACAD.EXE, it probably indicates

that an ADS file listed in your ACAD.ADS is not in the specified directory.

OUT OF RAM  or  OUT OF ENVIRONMENT SPACE

Try placing the following line into your Config.sys file:

SHELL=COMMAND.COM /P /E:1024

Be sure to include the spaces.  Sometimes ACADFREERAM is not set right,

but if it always happens with the same drawing it may mean that AutoCAD

is being instructed to set up a table which is incredibly big -- like an

array with 1,000,000 copies of a block.

SMIO      Symbol Table I/O

Indicates that AutoCAD has encountered an error while reading or writing

to the symbol table section of the drawing file, which contains the

Layer, Linetype, Style, View, UCS, and VPORT entries.  Depending on when

this happened (while editing, during a PLOT, etc.), it could point to

corruption of the drawing file or of one of the temporary files

maintained in I/O page space by the drawing editor.

TCODE

Indicates a bad header record.  Possible work around is to examine the

header fields in the DXF file if the file is salvageable in that

fashion.

Errors Encountered during Drawing Editor Operation

------------------------------------------------

386|VMM: Stack pointer in unmapped pages

This error indicates a memory conflict involving protected mode drivers

and/or protected mode AutoCAD.  The message is being sent by the Virtual

Memory Manager working in protected mode.

AFPAGER

This message indicates the disk drive where the paging file is stored

has been filled.  Correct by either selecting another paging drive, or

freeing disk space.

BAGENT1

BAGENT1 error indicates that an invalid request has been received

from AutoLISP.  That's a pretty strange error to be getting, assuming

that the version of ACADL you're using matches the version of ACAD. Try

reloading either or both executables.

CS:EIP   (Code Segment: Extra (External?) Instruction Pointer)

This message indicates that a protected mode error has occurred. It is

usually caused by a TSR or ADI driver trying to write data into an area

of protected memory.

CVFIO     Compressed Vector File I/O

Indicates that the routine generating vectors for a plot or printer

plot sequence has encountered an error while reading or writing to

its temporary files.

DISK FULL

The "disk full" error message and the problems with saving blocks are

most likely related to a problem that AutoCAD had in addressing hard

disk partitions larger than 32 Meg (Actually this size may vary. The

number of cluster used on a harddisk seems to be the real culprit. This

may vary with different harddisk formatting schemes.)  This problem was

corrected in AutoCAD Release 9.04.

MWRITE failed m,n

An MWRITE error indicates that AutoCAD was trying to write the header

portion of the drawing file to disk (as for SAVE, END, or WBLOCK), but

the operation failed for some reason. The numbers indicate the expected

file position following the write (the length of the drawing file

header) and the actual file position.

NULL POINTER ERROR

Usually caused by a conflict in protected memory that is caused when a

TSR or ADI driver tries to write data into the protected area. May be

also caused by bad memory chips or lack of available disk space.

RFSUBS    Refresh File Subroutines

Indicates that AutoCAD's attempts to read/write to the (temporary)

refresh file have failed.  AutoCAD maintains a "refresh file" that

describes the present state of the graphics screen and refers to this

file during redraws, zooms and pans.  The error indicates that a

read/write to this temporary file has failed. Misbehaving memory boards

and hard disk controllers could cause this message to crop up.

RFSUBS NXT    (Same as above)

The message indicates that an I/O error of some sort (possibly a

premature end-of-file condition) occurred while AutoCAD was attempting

to read one of its temporary files (in this case, the "refresh" file,

which is AutoCAD's version of a display list). AutoCAD read a record

length from the file and then attempted  to read the associated record,

but encountered an error in doing so. This could be due to a software

error (bug), to any number of memory or disk hardware problems, to

improper assignment of extended memory environment variables, or to an

unrecognized RAM disk.

SCANDR    Scan Database Record

This indicates that AutoCAD has encountered an error while attempting to

update its temporary files at run-time.  While the AutoCAD Drawing

Editor is running, AutoCAD frequently has to mark changes and new

additions to the temporary files containing the present "state" of the

drawing database.  If any of these attempts to update the drawing

database or to update the files referenced by the Drawing Editor fail,

AutoCAD may abort with the above error.

SCANDR 10:  Unterminated complex entity recovery required.

The message means that during the scanning of the drawing file a entity

was found that wasn't correctly terminated.  Most likely a polyline

wasn't terminated properly in the file.  If this doesn't happen often it

was probably a small glitch like a bad sector on the hard disk, a power

fluctuation, or sunspots.  If you keep getting errors, however, you have

a problem somewhere.

XPAGEIO   EXtended I/O Paging

Indicates that AutoCAD's paging routines have encountered an error

during a read or write to its Extended I/O pages.  AutoCAD's Extended

I/O page space contains any detected LIM Expanded Memory and

conventional Extended Memory divided into "pages". As the drawing editor

generates data that does not fit in 640K of RAM, AutoCAD's paging

routines store data in any available Extended I/O page space.

Subsequent read requests are also routed to this data cache.  Failure to

read or write successfully to this chained set of data pages results in

this error.

Errors Specific to the MS-DOS / PC Environment

------------------------------------------------

CHECKFD     Check File Descriptor

Indicates that AutoCAD's file descriptor table has encountered an

error during bookkeeping.  It may be trying to close a file that

has already been closed.

FMTOUT	    Format Output

This is an error message in Release 10.  It indicates that AutoCAD's

output formatting code has encountered an error during initialization or

at run-time.  This error tends to indicate a problem with the math

co-processor.  Ensure that the motherboard switches for the math

co-processor are set correctly, and that the co-processor is seated

properly in the board sockets.

FREE      Free memory block

Indicates that AutoCAD has encountered an error while attempting to

release a block of memory.  This could be caused by an improper

operating environment or by the presence of TSR programs.  This could

point to a software bug (losing track of the buffer address, or

overwriting the housekeeping info in the buffer).  It could also be

caused by faulty hardware (dropped bit in memory or on disk).

Miscellaneous Errors

------------------------------------------------

SHELL: Error swapping temp file

Indicates a hardware error encountered while paging to extended memory

or hard disk during execution of the Shell command.

SHELL: Insufficient memory to continue

Indicates that the AutoCAD shell handler has detected insufficient free

memory prior to invoking a DOS program.  While this can be caused by an

incorrect entry in the ACAD.PGP file for the program in question, it can

also be caused by the presence of a TSR program or a device driver that

has been loaded, and is fielding an interrupt that points back to the

heart of AutoCAD's code space. AutoCAD expects to "own" all such

interrupts (ones that point back to itself).  Whenever it encounters one

that it does not own, AutoCAD does not swap to DOS, to avoid running the

risk of de-stabilizing the machine (by swapping out a critical device

driver).  Network drivers and interrupt-driven mouse drivers like the

Mouse Systems Mouse driver are common culprits here.

VS: Not a string record  [Virtual String Record]

Indicates that AutoCAD has encountered an error while reading a

string from the ACADVS.OVL overlay file.  This file contains most

of the prompts and text information associated with AutoCAD.  A VS

error indicates that the ACADVS.OVL file has been corrupted. Reload

the file from your original set of diskettes and ensure that your

machine is set up properly.

POSSIBLE CAUSES AND GENERAL ERROR RECOVERY

------------------------------------------

Almost all errors are caused by corruption of one of the following:

1. The drawing file that AutoCAD is reading

2. The temporary files that AutoCAD has generated

3. AutoCAD's working environment under MS-DOS

Drawing files can be corrupted due to several reasons:	floppy

diskettes may be bent or dirty, disk drives may need cleaning and

alignment, a voltage spike during a read or write to disk can

corrupt data, the disk drive may be highly fragmented, over-heated,

etc.  Remember that you cannot recover the drawing file in question

using a binary/hex editor to patch the data, or one of the popular

unerase utilities under MS-DOS, like Norton Utilities.	Norton's Disk

Doctor also will not recover corrupted drawing files since it only

relocates all the readable bytes to another area of the disk, ignoring

the unreadable ones.  AutoCAD drawing files are extremely sensitive and

as much as one misplaced bytes can render the file unreadable.	The hex

dump that follows the Error message is merely a traceback of internal

addresses, telling the AutoCAD programmer the sequence of most recent

function calls that preceded the error.  It holds little significance

for the general customer.

Temporary files can be corrupted at run-time due to the reasons

above, and also due to improper configuration.  For example, if you

are running AutoCAD on a machine with EXTended memory, ensure that

AutoCAD has detected the presence of any RAM disks or partitioned

EXPanded memory.  If it has failed to do so, then any data that

AutoCAD has paged into its Extended I/O page space runs the risk of

being over-written by other information that is being written to

the RAM disk or the "other memory partition".

Under MS-DOS, AutoCAD establishes and maintains a tight working

environment consisting of memory blocks, data resources and

software interrupts.  Should an ill-behaved TSR program or

a device driver alter this carefully maintained state without

AutoCAD's knowledge, any data that AutoCAD is operating on runs the

risk of being corrupted during execution.

Whenever these error messages occur, AutoCAD is almost certainly

reporting a genuine case of data or environment corruption.  On

rare occasion, this may point back to a software bug within

AutoCAD.  These cases are almost always repeatable and occur due to

a specific sequence of events; e.g., if drawing a circle and

trimming it to 1/4" of its radius always generates an error in any

drawing, then you may have discovered a software bug.

What do I do if I encounter one of these errors ?

First of all, don't panic.  If the drawing file itself is corrupted, you

will encounter the error while calling up the drawing on any machine or

copy of AutoCAD.  In this situation, you can either:  return to the

backup of the drawing file in question (backing up your data on a

regular basis is critical), or use the procedure outlined in Appendix F

of the Reference Manual under "Disaster Handling". If neither of these

options work, you may choose to call on the following vendors for their

drawing file salvaging utilities. Please note that this does not

constitute an endorsement from Autodesk on the performance of these

utilities.

Name of Utility          Company and Contact Information

AutoSave                 Cyco Automation     800 336-6644

Retriever                Pan Dimensional     707 869-0682

D2D                      Great SoftWest      800 231-6880