�������������������������������������������������������������������������ͻ

�                                                                         �

�                      What is Data Compression?                          �

�                                                                         �

�������������������������������������������������������������������������ͼ

This document briefly describes the effects of compression on modem

data transmissions.

WHAT IS DATA COMPRESSION?

Data compression allows modems to increase through-put without

increasing the baud rate (transmission speed).  With data compression,

the sending modem compresses the data into a compact form.  The

receiving modem decompresses the data back to its original form.  You

save time and money because your information is transmitted much more

quickly.

V.42bis is a data compression standard for 9600 bps modems.  It

compresses data by as much as 4:1, depending on the type of file you

send.  Thus, a 9600 baud modem can transmit your data at up to

38,400bps (bits per second) using V.42bis.

Another method of data compression is a de facto standard known as

Microcom Networking Protocol (MNP) Level 5.  MNP 5 provides up to 2:1

data compression.  This gives a 2400 baud modem and effective

throughput of as much as 4800 bps, and a 9600 baud as much as 19,200

bps.

Data compression can occur only when the two communicating modems

support the same compression techniques.  If modems supports more than

one type of compression, the communicating modems will use the most

powerful technique common to both.

���������������������������������������������������������������������������

November 8, 1991          Intel FaxBACK # 4304                       Page 1