�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

�������������������������������������������������������������������������ͼ

ANSWER MODE

Used by the modem when answering a call from another computer. The

modem uses "originate mode" when calling another computer. Although

your modem can operate in either mode, some modems operate only in

originate mode.  To communicate with these modems, your modem can

switch to answer mode after placing (originating) a call.

ASCII (American Standard Code for Information Interchange)

A standard method of encoding characters that enables text generated by

one program or computer to be understood by another program or

computer.

ASYNCHRONOUS COMMUNICATION

Communication between computers at irregular intervals.  To handle this

kind of communication, the modem adds start and stop bits to each

character it sends.  The start bit lets the receiving computer know

when a new character has started.  The stop bit indicates the end of

the character.

AT COMMAND SET

The set of industry-standard commands used to control the modem. Each

command line must start with the two-character attention code AT (or

at).

ATTENTION CODE

The two-character sequence AT or at. Signals the modem that one or more

modem commands are to follow.

AUTOMATIC ANSWERING (OR AUTO ANSWER)

The modem automatically answers the phone after a certain number of

rings.  This is in contrast to manual answering, where a person sends

an AT command to the modem to cause the modem to answer a ringing

telephone.

AUTOMATIC DIALING

The modem automatically dials the telephone.  This is in contrast to

manual dialing, where a person dials the number.

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                       Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

AUTO-RELIABLE MODE

The modem automatically negotiates with the remote modem for a

connection, determines whether or not it can use error control and data

compression during a transmission, and determines which error control

and data compression protocol it can use.

BAUD RATE

The number of symbols transmitted per second.  This is not always the

same as the bps rate (see "Bits per second"), because a given symbol,

or baud, may have more than one bit.

BELL 103 AND 212A (AMERICAN BELL STANDARD)

A North American standard that specifies the way modems transmit

information.

*   Version 103 applies to asynchronous modems transmitting at speeds

up to 300 bps.

*   Version 212A applies to full duplex asynchronous or synchronous

modems that transmit at speeds up to 1200 bps.

BITS PER SECOND (BPS)

The speed at which a modem sends or receives information. For example,

a modem that operates at 2400 bits per second can transfer 2400 binary

digits each second.  A character normally consists of seven or eight of

these binary digits, plus the start and stop bits that separate the

character from other transmitted characters.

BUFFER

A storage location in its internal RAM where the modem can temporarily

store data it receives until it can process the data.

CARRIER

The base signal used to transmit data across a telephone line. The

modem modulates this signal (alters its frequency or phase) to encode

the data to be transmitted.

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                       Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

CCITT

(Consultative Committee for International Telephone and Telegraph)

An international committee that specifies the way modems and fax

machines transmit information to ensure compatibility among modems.

(Note:  bis means the second version of a particular standard.)

*   CCITT V.21 -- 300 bps modems

*   CCITT V.22 -- 600 and 1200 bps modems

*   CCITT V.22bis -- 2400 bps modems

*   CCITT V.32 -- 4800 and 9600 bps modems

*   CCITT V.32bis -- 4800, 7200, 9600, 12000, and 14400 bps modems

*   CCITT V.42 -- error control

*   CCITT V.42bis -- data compression

*   CCITT V.54 -- testing

CLEAR TO SEND (CTS)

An RS-232C signal that tells the computer it can start sending

information.  See also Request to send (RTS).

COMMAND MODE

One of the two operating modes of the modem, sometimes called local

mode or terminal mode.  (The other is on-line mode.) In command mode,

the modem interprets any information it receives from the local

computer (or terminal) as modem commands.  It tries to perform the

commands sent to it, and it returns result codes indicating the results

of the commands.  The modem enters command mode when you turn on or

reboot your computer (or terminal), or when you type the escape

sequence (normally +++) while the modem is in on-line mode.

COMMUNICATION (COM) PORTS

The connectors and accompanying circuitry on your computer that let you

connect serial devices (usually serial printers, modems, or mice) to

your computer and communicate with these devices. Communication ports

are also called serial ports.  To keep track of the devices, DOS

assigns names that begin with the letters COM to communication ports

(such as COM1 and COM2).

COMMUNICATIONS SOFTWARE

A program that sets up your modem and gives you a human interface to

the various functions of your modem.  Examples of DOS-based

communications programs are Smartcom, Procomm, and Crosstalk. Examples

of communications programs for the Macintosh are Quick Link II,

MicroPhone, and Red Ryder.

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                       Page 3

�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

CONFIGURATION PROFILE

The current operating characteristics of your modem, stored in the

modem's S-registers.  Whenever you issue a modem command to change one

of the operational characteristics (such as setting the volume control

or turning the speaker on or off), the modem changes the values in the

S-registers to reflect your changes.

DATA CARRIER DETECT (DCD)

An RS-232C signal that indicates the modem is receiving a carrier

signal from a remote modem.

DATA COMPRESSION

The encoding of text or data so that it takes up less space (fewer

bits).  CCITT V.42bis and MNP class 5 are two different data

compression protocols.  Data compression allows your modem to transmit

more information in a shorter period of time and thus increases your

modem's data throughput.

DATA SET READY (DSR)

An RS-232C signal that tells the computer (or terminal) the modem is

connected to the telephone line.

DATA TERMINAL READY (DTR)

An RS-232C signal that tells the modem the local computer (or terminal)

is ready for data transmission.

DIAL MODIFIERS

AT commands that you enter along with the AT command D (dial) to

further instruct the modem when dialing the telephone.  Dial modifiers

specify things like whether to use pulse or tone dialing, when and how

long to pause between numbers, and whether to dial a stored number.

DUPLEX

The way modems exchange data:  half duplex or full duplex.  With half

duplex transmissions, only one modem can send data at a time.  Full

duplex transmissions allow both modems to send data simultaneously.

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                       Page 4

�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

ERROR CONTROL

The encoding of text or data so that a receiving modem can detect and

sometimes correct errors in data transmissions.  LAPM and MNP classes 1

through 4 are two different error control protocols.

ESCAPE SEQUENCE

A sequence of three characters (normally +++) that switches the modem

from the on-line mode to the command mode without breaking the

telephone connection.

FILE TRANSFER PROTOCOL

An error-checking protocol for file transfers (such as KERMIT, XMODEM,

or YMODEM).  It monitors information sent with each block of data.  If

the received data doesn't match the information used to check the

quality of data, the system notifies the sender that an error has

occurred and asks for a retransmission.

FLOW CONTROL

The mechanism that regulates the flow of data between two devices.

Modems typically have two methods of flow control software flow control

(XON/XOFF) and hardware flow control (CTS/RTS).

GUARD TIME

A period of time during which the modem must not receive characters.

The escape sequence (normally +++) has a guard time to ensure that data

sent from a remote modem isn't interpreted as an escape sequence.  The

factory setting for the guard time is one second.  For the modem to

interpret an escape code as valid, there must be one second before and

one second after the escape sequence in which the modem doesn't receive

any characters.

HAYES COMPATIBILITY

Software (and people) communicate with modems using the AT command set

originally developed by Hayes Corporation.  Modems that use the AT

command set are said to be Hayes-compatible.

HOOK FLASH

The act of hanging up the telephone for a short period of time (one-

half second).  This operation is useful in PBX systems that interpret a

hook flash as a signal to perform other services, such as transferring

a call.

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                       Page 5

�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

LAPM (LINK ACCESS PROCEDURE FOR MODEMS)

LAPM is an error control protocol specified by CCITT V.42.  LAPM

provides error control when your modem is communicating with another

modem that supports LAPM.

LINE NOISE

Random signal disturbances that sometimes occur over telephone lines.

Noise can disrupt communications and corrupt the transmitted data.

LOOPBACK TEST

Diagnostic test where characters that are sent to the modem are

immediately sent back from the modem so the computer can compare the

characters sent with the characters received.

MANUAL DIALING

Dialing a remote modem from a telephone connected to the modem. This is

in contrast to automatic dialing, where the modem dials the number.

MNP (MICROCOM NETWORKING PROTOCOL)

Provides error control and data compression when your modem is

communicating with another modem that supports MNP.  (MNP classes 1

through 4 are specified by CCITT V.42 as a backup error control scheme

for LAPM.)  Your modem supports five MNP classes. Each MNP class has

all the features of the previous class plus its own.

*   MNP class 1 (also called block mode):  Sends data in one direction

at a time.  About 70 percent as fast as data transmissions using no

error correction.

*   MNP class 2 (also called stream mode):  Sends data in both

directions at the same time.  About 84 percent as fast as data

transmissions using no error correction.

*   MNP class 3:  Sending modem strips start and stop bits from data

block before sending it.  Receiving modem adds start and stop bits

before passing the data to the receiving computer. About 8 percent

faster than data transmissions using no error correction.

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                       Page 6

�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

*   MNP class 4:  Monitors the quality of the connection and

streamlines the information in the headers of data blocks. If the

telephone line is relatively noise-free, the modem sends larger

blocks of data to increase throughput. If the telephone line is

noisy, the modem sends smaller blocks of data so that less data

will have to be resent. This means more successful transmissions on

the first try. About 20 percent faster than data transmissions

using no error correction.

*   MNP class 5:  Provides data compression. Sending modem detects

redundant data and recodes it to fewer bits to increase the

effective throughput. Receiving modem decompresses the data before

passing it to the receiving computer. Up to 100 percent faster than

data transmissions using no data compression or error correction,

depending on the kind of data transmitted. (Files that are already

compressed will seem to contain less redundant data and may

therefore take longer to transmit using MNP class 5 than they would

if you were using no data compression.)

MODEM (DERIVED FROM MODULATOR/DEMODULATOR)

A modem transforms (modulates) digital information into an analog

signal that can be sent across a telephone line. It also demodulates an

analog signal it receives from the telephone line, converting the

information contained in the signal back into digital information.

NONVOLATILE MEMORY

Memory that retains the information stored in it even if you turn off

the computer.  There are two kinds: ROM and NVRAM.  ROM is programmed

with information at the factory, and you can't change it.  NVRAM is

used to store configuration information, and you can changed it with

modem commands.

NVRAM (NONVOLATILE RAM)

See Nonvolatile memory.

OFF-HOOK

The condition of a telephone line that corresponds to picking up the

telephone receiver.  A modem creates an off-hook condition when it

tries to communicate on a telephone line.

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                       Page 7

�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

ON-HOOK

The condition of a telephone line that corresponds to hanging up the

telephone receiver.  A modem creates an on-hook condition to break its

connection to a telephone line.

ON-LINE MODE

One of the two operating modes of the modem, also called data mode.

(The other is command mode.) In on-line mode, the modem interprets all

information sent to it as data, not commands.  The only exception is

the escape sequence (normally +++), which returns the modem to command

mode without breaking the connection.  The modem is placed in on-line

mode when it makes a connection with a remote modem, or when you enter

the O command to return to a previously established connection.

ORIGINATE MODE

The mode used by the modem when calling another computer. See also

"Answer mode."

PARITY

A data encoding scheme that computers (and terminals) use to check the

validity of transmitted characters.  This scheme adds an extra bit to

each character, which the transmitting computer sets or clears based on

the type of parity the computers agree to use (odd or even).  For

example, if the computers use even parity, the transmitting computer

sets or clears the parity bit so that there are an even number of bits

set in each character it transmits.  The receiving computer checks each

character and flags a parity error if any character has an odd number

of bits set.

PROTOCOL

A set of rules that governs how data is transmitted.  To communicate

successfully, two computers must use the same protocol.

PULSE DIALING

A method of dialing the telephone where the modem sends pulses (which

you hear through the handset as clicks) to represent the telephone

numbers (one pulse for a one, two pulses for a two, etc.). Pulse

dialing is normally associated with rotary-dial phones. See also "tone

dialing."

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                       Page 8

�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

RELIABLE CONNECTION

A connection between two modems where they communicate using an error

control protocol (such as LAPM or MNP).

REQUEST TO SEND (RTS)

An RS-232C signal that requests the modem to send data. It initiates

any data transmission between the computer (or terminal) and the modem.

It is answered by a Clear to send (CTS) signal.  See also "Clear to

send."

RESULT CODES

When you enter most modem commands, the modem displays a message on

your screen, called a result code, that tells you the results of the

command. In the factory setting, result codes are returned as English

words that describe the conditions.  You can also set the modem to

display corresponding numbers instead of words.

RS-232C

A standardized connection system for connecting a device to the serial

port of a computer or terminal.  This is the recommended standard of

the Electronic Industries Association (EIA) for exchanging information

between Data Terminal Equipment (such as computers) and Data

Communications Equipment (such as modems).

S-REGISTERS

RAM in your modem that is used to store the current configuration

profile (operating characteristics) of your modem.

SERIAL PORT

The circuits and connector that let your computer communicate with

serial devices such as printers, modems, plotters, mice, and custom

laboratory equipment.  (Also called a COM or communications port.)

SOFTWARE FLOW CONTROL

See XON/XOFF.

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                       Page 9

�������������������������������������������������������������������������ͻ

�                                                                         �

�                    Glossary of Common Modem Terms                       �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

START/STOP BITS

The bits at the beginning and end of a data block when using

asynchronous data transmission.  See also "Asynchronous Data

Transmission."

THROUGHPUT

The total useful information processed or communicated in a specified

amount of time.  Data compression increases the throughput of your

modem by allowing you to send more information in the same number of

bits.

TONE DIALING

One of two methods of dialing the telephone.  (The other is pulse

dialing.)  With tone dialing, the modem sends tones of different

frequencies to represent the telephone numbers.  Tone dialing is

normally associated with push-button (touch-tone) phones and is also

called Dual Tone Multi-Frequency (DTMF) dialing.

V.21, V.22, V.22BIS, V.32, V.32BIS, V.42, V.42BIS, V.54

CCITT specifications for modems.  For details, see "CCITT."

XON/XOFF

A way of controlling the flow of data between a modem and its host

computer and between two modems, also called software flow control.

XON stands for "Transmitter On," and XOFF stands for "Transmitter Off."

If the modem receiving data needs time to process the data or do some

other task, it sends an XOFF signal to the host computer (or sending

modem).  The host computer (or sending modem) then waits until it

receives an XON signal before sending more data.

���������������������������������������������������������������������������

May 8, 1992               Intel FaxBACK # 4308                      Page 10