�������������������������������������������������������������������������ͻ

�                                                                         �

�               INTEL EXTERNAL MODEM COMPUTER COMPATIBILITY               �

�                                                                         �

�������������������������������������������������������������������������ͼ

The following information tells whether or not certain computers are

compatible with your Intel product.  In some cases there are also special

installation instructions.  "(Field Report)" means the information came

from our customers and may not have been verified by Intel's compatibility

lab.

GENERAL INFORMATION

Intel external modems are compatible with any computer or terminal with

a serial port that supports asynchronous communications.  This includes

IBM PC, XT, AT and compatibles, IBM PS/2 Models 30, 50, 50z, 70, and

80, and Apple computers.

AMIGA 500

Intel tested this computer and is is compatible with Intel external

modems.

WARNING:    As stated in the Amiga owners manual, pins 9 and 10 of the

Amiga serial port are connected to +/-12 volts.  Users must

either buy a cable that doesn't use these pins or disconnect

these two pins from the cable.  FAILURE TO DO SO COULD DAMAGE

THE MODEM!

AMIGA 2500

Intel tested this computer and is is compatible with Intel external

modems.

WARNING:    As stated in the Amiga owners manual, pins 9 and 10 of the

Amiga serial port are connected to +/-12 volts.  Users must

either buy a cable that doesn't use these pins or disconnect

these two pins from the cable.  FAILURE TO DO SO COULD DAMAGE

THE MODEM!

DATA TERMINALS

The Intel 2400EX modem is compatible with data terminals that conform

to the RS-232C serial port standard.  If the modem doesn't work, you

may need a "null modem" cable, a cable that has pins 2 and 3 cross

connected.  Pin 2 (send) on one end is connected to pin 3 (receive) on

the other end.  Some terminals, such as the Televideo 925, can swap

these pins internally.

The pin assignments for the modem cable are as follows:

��������������������������������������������������������������������������

1/91                       Intel FaxBack # 4324                     Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�               INTEL EXTERNAL MODEM COMPUTER COMPATIBILITY               �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

Pin      Description

�����   �����������������

2       Send

3       Receive

4       Ready To Send

5       Clear To Send

6       Data Set Ready

7       Signal ground

8       Carrier Detect

20       Data Terminal Ready (DTR)

22       Ring Indicator

The minimum required pins are pins 2, 3, 7, and 20.

��������������������������������������������������������������������������

1/91                       Intel FaxBack # 4324                     Page 2