�������������������������������������������������������������������������ͻ

�                                                                         �

�                9600EX Compatibility with Other Modems                   �

�                                                                         �

�������������������������������������������������������������������������ͼ

MODEM COMPATIBILITY IN GENERAL

Intel modems and faxmodems are compatible with virtually all 300, 1200,

and 2400bps modems.  Be aware that there are proprietary high speed

standards that Intel does not support ("PEP" Protocol used with

Telebits, HST Used by US Robotics Modems).

HAYES ULTRA

This modem has been tested with all Intel modems and is compatible.

The Hayes Ultra is a V.32 compatible modem that incorporates V.42bis

and MNP levels 1-5. All of our modems will talk to the Ultra at 300,

1200, and 2400bps. Intel High Speed modems can talk to the Ultra at

4800 and 9600bps when V.32 mode on the Ultra is enabled. Keep in mind

the Ultra also has the Hayes Proprietary "ping-pong" 9600bps mode that

is NOT V.32. When the Ultra has V.32 mode disabled, only other Hayes

modems can talk to it at 4800 and 9600bps.

SatisFAXtion/100:

Intel's lab tested this modem with the SatisFAXtion/100 board, and it

is compatible.

HAYES SMARTMODEM V SERIES 9600

This modem has been tested with all Intel modems and is compatible.

All Intel modems can talk to this modem at 300, 1200, and 2400bps. Some

versions of the Smartmodem 9600 have MNP and V.42 capabilities. At

4800bps and higher, Hayes uses a proprietary "ping-pong" 4800/9600bps

protocol that only other Hayes modems can connect to. Intel High Speed

Modems  cannot talk to the Hayes Smartmodem V 9600 at any speed higher

than 2400bps.

HAYES SMARTMODEM 2400 V SERIES

This modem has been tested with all Intel modems and is compatible.

All Intel modems can talk to the Smartmodem 2400 at 300, 1200, and

2400bps. Some versions of the 2400 V series support V.42bis. All

versions support MNP.

SatisFAXtion/100:

Intel's lab tested this modem with the SatisFAXtion/100 board, and it

is compatible.

MULTITECH MULTIMODEM V.32

The Multitech MultiModem has been tested at 300, 1200, and 2400bps with

all other non-V.32 modems and was found to be compatible. Intel High

Speed Modems can communicate with this modem at 4800bps and 9600bps.

Some versions of this modem support V.42bis and/or MNP Level 4&5.

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4328                       Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�                9600EX Compatibility with Other Modems                   �

�                                                                         �

������������������������������ (continued) ������������������������������ͼ

MULTITECH MULTIMODEM 224

This modem was tested with all Intel modems and found to be fully

compatible at 300, 1200 and 2400bps. No error correction or data

compression is offered in this unit.

MULTITECH MULTIMODEM 224E

This modem was tested with all Intel modems and found to be fully

compatible at 300, 1200 and 2400bps. The 224E is similar to the 224,

but offers MNP Levels 4&5.

RACAL-VADIC 9632VP

14.4EX, SatisFAXtion/400 & 400E:

The Intel SatisFAXtion/400 & 400E and the 14.4EX will not connect with

older versions of this modem when the Intel modem originates the call.

If the 9632VP originates the call, the modems connect.  As of this

writing, the current version of the 9632VP does not have this problem.

9600EX:

The 9600EX works with all versions of this modem.

US ROBOTICS HST

The US Robotics HST modem has been tested with all Intel modems and is

compatible.  The US Robotics HST modem can connect to any Intel modem

at 300, 1200, and 2400bps. At 4800bps and higher, it uses the

proprietary HST (High Speed Technology) protocol that is supported only

by other US Robotics modems. It will not connect to any modems at this

high speed except for other US Robotics modems.

SatisFAXtion/100:

Intel's lab tested this modem with the SatisFAXtion/100 board, and it

is compatible.

US ROBOTICS HST DUAL STANDARD

This modem has been tested with all Intel modems and found to be

compatible.  The US Robotics dual standard can talk to all Intel modems

at 300, 1200, and 2400bps. Be aware that there are two modes of 9600bps

operation for this modem: HST and V.32. When the US Robotics is in HST

mode, Intel High Speed Modems can only talk to it at 300, 1200, and

2400 because the HST mode is proprietary to just US Robotics modems.

When in V.32 mode, Intel High Speed Modems can talk to it at 4800 and

9600bps.

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4328                       Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�                9600EX Compatibility with Other Modems                   �

�                                                                         �

������������������������������ (continued) ������������������������������ͼ

US ROBOTICS COURIER V.32

The US Robotics Courier V.32 has been tested at Intel and is compatible

with all Intel modems. This modem is the functional equivalent to the

9600EX and will connect at 300, 1200, 2400, 4800 and 9600bps. Some

versions of this modem support MNP Levels 4&5 and V.42bis.

US ROBOTICS SPORTSTER

Intel has tested this modem and has found it to be compatible with all

of our modems. It will support speeds of 300, 1200, 2400, 4800 and

9600bps but does not support error correction or data compression.

TELEBIT T-1000

The Telebit T-1000 has been tested by Intel with all modems and is

compatible at 300, 1200 and 2400bps. At 4800bps and higher, Telebit

uses a proprietary "PEP" protocol which is ONLY COMPATIBLE WITH OTHER

TELEBIT MODEMS - THE T-1000 IS NOT V.32 COMPATIBLE. Some versions of

this modem have MNP Levels 4&5 and/or V.42bis.

TELEBIT T-1500/T-1600

The Telebit T-1500 and T-1600 have been tested at Intel and are

compatible with Intel modems at 300, 1200, 2400, 4800 and 9600bps. They

are the functional equivalent to the 9600EX. Some versions of these

modems support V.42bis and/or MNP Levels 4&5.

TELEBIT T-2500

The Telebit T-2500 has been tested with all Intel modems and was found

to be compatible with all modems. When in V.32 mode, the T-2500 can

connect to Intel High Speed Modemss at 4800 and 9600bps. Keep in mind

that this modem has the ability to use Telebit's proprietary "PEP"

protocol in addition to or instead of V.32. When in "PEP" mode, all

Intel modems can talk to the T-2500 at 300, 1200 and 2400bps.

Intel High Speed Modems can also connect with this modem using V.42bis

data compression/error correction with Telebit ROM Version 7.02 or

higher.

TELEBIT TRAILBLAZER PLUS

The Telebit Trailblazer Plus is compatible with all Intel modems at

300, 1200 and 2400bps. At 4800bps and higher it uses Telebit's

proprietary "PEP" mode that only other Telebits can use. Some versions

have V.42bis and/or MNP Levels 4&5.

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4328                       Page 3