�������������������������������������������������������������������������ͻ

�                                                                         �

�                  GENERAL MODEM TROUBLESHOOTING: PART 1                  �

�                                                                         �

�������������������������������������������������������������������������ͼ

The following information can help you solve certain problems with your

Intel product.  "(Field report)" means that the information comes from a

customer and may not have been verified by Intel's compatibility lab.

ARE YOU ON A PBX PHONE SYSTEM?

If your modem loses data or has degraded performance, it could be

because it's connected to a PBX telephone system.  Check with the phone

company to determine if you're connected to a PBX or other phone system

that requires a special digit to dial out.

CARRIER ALREADY ESTABLISHED MESSAGE

If you receive this message before the modem dials the phone number,

use a communication program or TESTCOM to change the values of Data

Carrier Detect (DCD) and Data Terminal Ready (DTR).  Intel sets these

signals to 0 on non-MNP modems (2400B Calssic, 2400EX, and CCP Modem

Option) to match the Hayes default, but many communication programs

require a value of 1 to work.  The 2400B Classic MNP, 2400EX MNP, and

9600EX

If you use TESTCOM, select 6 from the main menu.  This allows you to

use the Hayes command set to address the modem hardware.  You can also

enter the following line at a place in your modem software that

responds to the Hayes AT Command Set:

AT&C1&D2&W

The C1 changes the DCD to 1, the D2 changes the DTR to 2, and the W

writes the information into modem memory.

DETECTING PORTS WITH DEBUG

If you're having a problem determining which parallel and serial ports

your computer is using, you can check for the I/O ports using the DEBUG

program.  Follow these steps to use DEBUG:

1.  At the DOS prompt, type DEBUG and press Enter.

2.  At the DEBUG prompt (a dash), type D 40:0 and press Enter.  This

displays the hexadecimal codes of the active I/O ports, first

serial then parallel:

0040:0000  F8 03 00 00 00 00 00 00-BC 03 00 00 00 00 00 00

This display indicates one serial port installed at 3F8 and one

parallel port installed at 3BC.  (Note that the high and low bytes

are reversed.)

��������������������������������������������������������������������������

1/91                       Intel FaxBack # 4460                     Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�                  GENERAL MODEM TROUBLESHOOTING: PART 1                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

3.  To quit DEBUG, type q and press Enter.

INTERMITTENT SWITCH TO COMMAND MODE

If your modem intermittently switches to Command mode after connecting

with a remote system, change the DTR option to &D2 with this command:

AT&D2

LEASED LINE SETUP

If modems fail to communicate over leased lines, but work fine on

standard lines, change the &L setting to &L1.  The default setting is

&L0.  See your modem command guide for more information.

In addition, if the leased line is a 4-wire line rather than a 2-wire

line, a 4-wire to 2-wire converter will be required.  This converter is

often a simple switch on the leased line telephone equipment in your

office.

MODEM DIALS BUT DOES NOT CONNECT

Communications software has each of the COM ports configured for a

specific I/O address and IRQ combination.  If the modem and software

configurations do not match exactly, the modem may dial but be unable

to signal the computer that it has achieved a connection.  Compare your

modem I/O and IRQ settings to those in your software to ensure an exact

match.

This problem may also appear when there is another device using the

same IRQ as the modem.  Adjust the modem and conflicting device to

different IRQ settings so they no longer conflict.

NO DIALTONE MESSAGE

Some computers (typically mainframes) require that modems call once,

hang up, wait for a return call, and then call again (using ATD).  With

2400B modems, when the modem picks up the phone to dial the second

time, it detects no dialtone and hangs up.  To avoid the problem, type

this command:

ATX3&W

then press Enter.  This suppresses the NO DIALTONE result code allowing

the modem to connect.

��������������������������������������������������������������������������

1/91                       Intel FaxBack # 4460                     Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�                  GENERAL MODEM TROUBLESHOOTING: PART 1                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

PHONE LINES ATTACHED CORRECTLY?

Your modem won't work if the phone lines are connected incorrectly to

the back of the modem.  The connector marked "line" for the phone line

coming from the wall, and the one marked "phone" is for the phone line

going to the telephone.  Neither the computer nor the modem need to be

turned on to use the phone for normal voice calls.

��������������������������������������������������������������������������

1/91                       Intel FaxBack # 4460                     Page 3