�������������������������������������������������������������������������ͻ

�                                                                         �

�                  GENERAL MODEM TROUBLESHOOTING: PART 2                  �

�                                                                         �

�������������������������������������������������������������������������ͼ

The following information can help you solve certain problems with your

Intel product.  "(Field report)" means that the information comes from

a customer and may not have been verified by Intel's compatibility lab.

TESTCOM.EXE

TESTCOM.EXE is a menu-driven diagnostic program that tests your modem.

With TESTCOM, you can do the following:

�   Run Modem diagnostic test

�   Check phone line connection

�   Call the Intel Bulletin Board Service (BBS)

You need both the TESTCOM.EXE and PHONE.BBS files on the disk from

which you run TESTCOM.  (PHONE.BBS is a data file with the BBS phone

number.)

If TESTCOM successfully connects with the Intel BBS, the modem hardware

is working properly.  Any problems encountered with other communication

programs are probably from improper configuration or execution.

NOTE:   TESTCOM is not a substitute for communications software.  You

need a program such as Crosstalk, Procomm, or Smartcom for

normal modem communications.

TESTCOM & OS/2

TESTCOM won't run in either the OS/2 environment or the OS/2

compatibility box.  Currently, no communications software runs in the

OS/2 compatibility box.

TESTCOM FAILS DIGITAL TEST

(Field report)  A Silencer power supply can cause TESTCOM to fail the

digital test.  The power supply provides the incorrect voltage.

TESTCOM PASSES -- OTHER PROGRAMS FAIL

If your modem passes all the tests in TESTCOM but your communications

program fails, look for IRQ conflicts.  TESTCOM doesn't use IRQs, but

some communications programs, like Procomm, do.  Check for other

devices which use an IRQ such as serial ports, mouse boards, scanner

boards, LAN boards, etc.  Read the modem installation guide to

��������������������������������������������������������������������������

1/91                       Intel FaxBack # 4462                     Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�                  GENERAL MODEM TROUBLESHOOTING: PART 2                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

determine which IRQ your modem is using.  Set the modem and other

devices so they don't conflict.

NOTE:   COM 1 and COM 3 use IRQ4 and COM 2 and COM 4 use IRQ3 in

classic bus systems. In microchannel systems, COM 1 uses IRQ4

and COM 2-7 use IRQ3.

STUCK AFTER "EXECUTING AUTOMATIC LOGON..."

CAUSE:  This message may appear when using the "Call Intel BBS"

feature.  You can get stuck with the message on screen if any

of the followin happens:

�   a bad connection to the BBS

�   the BBS was unexpectedly locked up and the modems were still

answering

�   the automatic logon has been temporarily changed.

CURE:   Try the call again later.

TESTCOM'S HIDDEN MENU SELECTIONS

TESTCOM's main menu contains two selections that aren't displayed and

aren't mentioned in the TESTCOM documentation.  These two selections

can help you verify your modem's operation.  The two hidden selections

are as follows:

(5) Lets you directly access the AT command set from the keyboard and

control the modem at 1200 BPS.

(6) Lets you directly access the AT command set from the keyboard and

control the modem at 2400 BPS.

Pressing the ESC key exits from these options.

��������������������������������������������������������������������������

1/91                       Intel FaxBack # 4462                     Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�                  GENERAL MODEM TROUBLESHOOTING: PART 2                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

SERIAL PORTS & INTERRUPTS

��������������������������������������������Ŀ

�                 Interrupts                 �

����������������������������������������������������Ĵ

�       �  Above  �  2400B  �  Connection  �         �

�       �  Board  � Classic � CoProcessor  �  2400B  �

� Port  � Serial  �  Modem  � Modem Option � Modem 2 �

����������������������������������������������������͵

� COM 1 �  IRQ 4  �  IRQ 4  �    IRQ 4     �  IRQ 4  �

� (3F8) �         �         �              �         �

����������������������������������������������������Ĵ

� COM 2 �  IRQ 3  �  IRQ 3  �    IRQ 3     �  IRQ 3  �

� (2F8) �         �         �              �         �

����������������������������������������������������Ĵ

� COM 3 �  IRQ 2  �  IRQ 4  �    IRQ 4     �  IRQ 3  �

� (3E8) �         �         � or IRQ 5     �  IRQ 3  �

����������������������������������������������������Ĵ

� COM 4 �  IRQ 5  �  IRQ 3  �    IRQ 3     �  IRQ 3  �

� (2E8) �         �         � or IRQ 2     �  IRQ 3  �

����������������������������������������������������Ĵ

� COM 5 �   NA    �   NA    �     NA       �  IRQ 3  �

����������������������������������������������������Ĵ

� COM 6 �   NA    �   NA    �     NA       �  IRQ 3  �

����������������������������������������������������Ĵ

� COM 7 �   NA    �   NA    �     NA       �  IRQ 3  �

������������������������������������������������������

In PS/2 computers,  COM ports 3 through 7 always use IRQ 3.  Classic

bus computers have no industry standard defining the IRQ for COM ports

3 and 4.  The Connection CoProcessor 2400B Modem Option has switches

that let you to choose between two IRQs for COM 3 and 4.  Above Board

serial port addresses and IRQs are independently selectable.  The

values shown for Above Board are Intel's recommendations.

SWITCHED DIGITAL PHONE LINES

Modems don't work on switched digital phone lines.  You should check

with the phone company to find out if you're connected to a switched

digital lines.  Modems will work on switched analog lines.

��������������������������������������������������������������������������

1/91                       Intel FaxBack # 4462                     Page 3