������������������������������������������������������������������������������

RINGER 1.10  John A. Qualtrough

(c) 1992 Q-Ware Las Cruces NM

������������������������������������������������������������������������������

RINGER is distributed as Shareware NOT Freeware.  If you like RINGER and

find it to be useful, you are asked to pay about $5.00 for a registered copy.

With this registration, you will receive any pending updates and a disk full

of other utilities to make your PC a more useful tool.

Send Check, Money Order, or Cold Hard Cash to:

John A. Qualtrough

315 Linda Vista Rd.

Las Cruces, NM 88005

Questions or comments may be left on:

Waterfront West Wildcat! BBS

300 - 19200,N,8,1  (24Hrs)

(505) - 523 - 4528

Leave [M]essage for John Qualtrough

������������������������������������������������������������������������������

Purpose for RINGER:

RINGER is a small TSR designed mainly for SYSOPs to protect a system from

a lock-up due to system problems.  What RINGER does, is monitor your modem

ring indicator to determine when the phone rings. If the phone rings more than

three times without your BBS answering, RINGER performs a system cold re-boot.

In this way, an unattended system can be protected from "hanging" due to DOS,

live door programs, or other errors which normally require operator attention.

When your BBS does answer, RINGER will be inactive as long as a carrier is

detected.  When the call is complete, RINGER will begin monitoring once again

for the next call.

������������������������������������������������������������������������������

Requirements for RINGER:

RINGER requires DOS 3.x (or later) and 640K is recommended.  RINGER uses

around 600 bytes of DOS RAM while resident.  RINGER also requires that the

cable used to connect your modem to the computer contains at least two of the

signals which RINGER uses.  These are Ring Indicator (pin-22) and Carrier

Detect (pin-8).  Virtually all factory made cables will contain these lines,

and internal modems produce these signals automatically.

������������������������������������������������������������������������������

Installing RINGER:

Install RINGER by typing:     RINGER port

Where "port" is the COM port number to which your BBS modem is connected.

Valid values are 1, 2, 3, or 4.

Example:

C:\>RINGER 3            (Monitor COM3: modem)

The installation should be placed in the BAT file used to start your BBS, or

in your AUTOEXEC.BAT file so that monitoring will automatically take place

each time the system is booted.

RINGER will now monitor for more than 3 rings of the phone line attached to

this modem.  The ring status is checked 18.2 times per second in the

background and should not interfere with your main BBS program.  Before

performing a cold re-boot, RINGER will drop the COM port RTS/DTR signals so

the modem attached will hangup.  This will allow your software to re-configure

the modem after booting without the modem remaining on-line.

������������������������������������������������������������������������������

Software Compatability:

Although RINGER should run under most applications, there may be some

programs which are not compatible.  Any application which directly accesses

hardware or does not use DOS/BIOS for I/O is possibly incompatible.  Sorry,

but trial and error are the best debuggers!  I have tried RINGER on a

number of different hardware platforms running either DOS 3.3 or DOS 5.0 and

have had no problems.

������������������������������������������������������������������������������

End Of Document