VGA TEXT ATTRIBUTES AND SPECIAL EFFECTS

Written by Chris Dunford

Edited by Jim Fowler

This document is an attempt to explains how VGA text attributes work,

and how the effects in FXCOLOR are possible.  It also provides tips for

creating your own special effects.

What's different with the VGA?

------------------------------

The CGA implemented a direct relationship between a 4-bit attribute and

a color.  Each attribute mapped to one color, and the color could not be

changed.

That changed with the EGA (which had palette registers to play with),

and changed again, radically, with the VGA.  The attribute number

specifies the displayed color very indirectly; it is combined with bits

and pieces of 5 different internal VGA registers before it appears on

the screen as one of the 262,144 possible colors.  There are, therefore,

several ways to change a character's color other than by changing its

attribute.  And, of course, if you change the way an attribute is

displayed, all characters with that attribute are affected.  Thus, it is

possible to "design" attributes with interesting effects, and to alter

the way entire screen fields are displayed very efficiently.

Mapping attributes to colors

----------------------------

Before we start, let me note that we are working here with 4-bit

attributes.  The 8-bit attribute that's associated with a character is,

of course, two 4-bit attributes: one for the foreground, and one for the

background.  This document discusses how these 4-bit attributes become

colors; everything here applies equally to foreground and background

attributes (thus it is possible to use blinking backgrounds, etc.)

Screen colors in text modes are controlled by the VGA's attribute

controller.  In one of its two modes (the one that the PS/2 BIOS sets

by default), here is how a color is selected by an attribute:

AND

attribute |-------| color plane enable register

|

| selects

|

palette register 0-15     color select register

|                         |

|                         |

| bits 0-5       bits 6-7 |

+------------+------------+

|

|      AND

+-------+--------video DAC mask register

|

| selects

|

video DAC color register 0-255

This appears complex, but it can be simplified.  First, notice the two

logical AND operations.  Since both the color plane enable register and

the video DAC mask register normally contain all ones, they tend to drop

out of the picture, resulting in:

attribute 0-15

|

| selects

|

palette register 0-15     color select register

|                         |

|                         |

| bits 0-5       bits 6-7 |

+------------+------------+

|

| selects

|

video DAC color register 0-255

The 4-bit attribute (0-15) selects one of the 16 palette registers.  The

palette register contains a number.  This number is combined with the

contents of the color select register to form another number.  For

example, suppose:

attribute = 13

palreg[13] = 1Bh

CSR = 3

Attribute 13 selects palette register 13, which contains 1Bh.  The color

select register contains 3.  We put the contents of the color select

register in bits 6-7 of the final number, and the contents of palreg[13]

in bits 0-5:

3   1Bh

--  ------

11  011011

The resulting number is 11011011 binary, 219 decimal.  This number

selects one of the 256 video DAC color registers.

The color that will be displayed is the color defined by the selected

video DAC color register (DAC stands for Digital-to-Analog Converter).

Each video DAC register is an 18-bit register that contains three 6-bit

values: one for each of the red, green, and blue primary colors.  Each

value specifies the intensity of that primary color on the screen.  A

zero for a primary means that it's not included in the color; 63 is

maximum intensity.

Suppose, in our example, that video DAC register 219 contains the

numbers 0, 3Fh, and 3Fh (it's convenient to think of the register as

three 6-bit values rather than one 18-bit value).  This means that red

will be off, and green and blue will be at maximum intensity.  The color

displayed by attribute 13 will be bright cyan.

Notice that you could change the color of a character with FG attribute

N in four different ways:

1. Change the attribute

2. Change the contents of palreg[N]

3. Change the contents of the video DAC register specified by

palreg[N] and the color select register

4. Change the contents of the color select register

These all have different effects on the total screen display.  Changing

an attribute affects a single character; changing a palette register

affects all characters with that attribute; changing a video DAC

register affects any character whose attribute, when combined with color

select, selects that register -- this could be several attributes;

changing the color select register could very well affect everything on

the screen.

Another way to visualize color selection

----------------------------------------

There is a simpler way to look at the process.  The two bits used from

the color select register always end up in the high two bits of the

video DAC color register number.  Thus, the selected DAC color register

will always be one of:

CSR=0: 00xx xxxx  (register range 0-63)

CSR=1: 01xx xxxx  (64-127)

CSR=2: 10xx xxxx  (128-191)

CSR=3: 11xx xxxx  (192-255)

The remaining 6 bits come directly from the selected palette register.

If we break the 256 video DAC color registers into four palettes of 64

colors each, as follows:

DAC registers   Palette

0-63          0

64-127          1

128-191          2

192-255          3

then the color select register selects one of the four 64-color

palettes, and the palette register (which is selected by the attribute)

selects one of the 64 colors of the CSR-selected palette.

Thus, it's simply stated: the attribute selects one of the 64 colors

available from the palette selected by the color select register.

Mode 1

------

I mentioned that the above represents one of the two modes supported by

the attribute controller; call it "mode 0".  The other mode (mode 1) is

very similar; the only difference is in which bits are combined from the

palette register and the color select register.  The simplified diagram

is as follows:

attribute 0-15

|

| selects

|

palette register 0-15     color select register

|                         |

|                         |

| bits 0-3       bits 4-7 |

+------------+------------+

|

| selects

|

video DAC color register 0-255

The difference is that there are four bits each from the palette

register and the color select register (instead of 6 and 2 under the

mode 0).

The effect of mode 1 is that the 256 video DAC color registers are

broken down into sixteen 16-color palettes instead of four 64-color

palettes:

DAC registers   Palette

0-15          0

16-31          1

32-47          2

...

240-255         15

The palette register (still selected by the attribute) selects one of

the 16 colors available in the palette selected by the current color

select register.

The more complex effects (fading, pulsing, etc.) of FXCOLOR are

generated under mode 1.  This is because all 256 colors defined by the

video DAC color registers can be accessed by simply changing the

contents of the color select register, which is very efficient (it

takes less than 1/100th of a second).

Under mode 0, only 75% of the available colors can be accessed by just

changing the color select register (this is because there are 64 colors

per palette, but only 16 attributes).  To get to the remaining colors,

you have to change the palette registers.  But there are 16 palette

registers; if you need to alter more than one displayed color, this is

less efficient than using the color select register.

A mode 1 technique

------------------

As mentioned, there are a number of ways to alter displayed colors.  I

will concentrate on one technique: creating a series of palettes and

then switching palettes by systematically changing the contents of the

color select register.  This document concentrates on mode 1; some of

the same effects (such as periodic intensification and simulated

blinking) can be obtained in mode 0.

Note that the use of mode 1 largely dictates that you must reinitialize

the palette registers.  The VGA BIOS initializes some of these registers

with values greater than 15; these will be masked in mode 1 to the low

4 bits, yielding values in the range 0..15.  These values will probably

not result in the colors you intend (two attributes will yield red

foreground, for example).  The simplest solution is to just fill the

palette registers sequentially with numbers from 0..15; this largely

causes the palregs to drop out of the picture:  attribute N will map

directly to color N within the palette selected by the color select

register.

Assuming that the palette registers are set as described, create a

"base" palette in palette 0.  That is, fill palette 0 (video DAC color

registers 0-15) with 16 base color definitions.  It makes sense to set

the standard color combinations for the 16 CGA-like colors that people

will expect to see (color intensity values in hex):

DAC regs  Attr  R   G   B   Color

0-2      0    0   0   0   Black

3-5      1    0   0  2A   Blue

6-8      2    0  2A   0   Green

9-11     3    0  2A  2A   Cyan

12-14    4   2A   0   0   Red

15-17    5   2A   0  2A   Magenta

18-20    6   15  2A   0   Brown

21-23    7   2A  2A  2A   White

24-26    8   15  15  15   Grey

27-29    9    0   0  3F   Bright Blue

30-32   10    0  3F   0   Bright Green

33-35   11    0  3F  3F   Bright Cyan

36-38   12    3F  0   0   Bright Red

39-41   13    3F  0  3F   Bright Magenta

42-44   14    3F 3F   0   Yellow

45-47   15    3F 3F  3F   Bright White

(In FXCOLOR, 35h is used in place of 3Fh.  This leaves some room to

pulse even the bright colors by intensification.)

The second step is to duplicate palette 0 into palettes 1-15, and then

vary the color definitions for attributes of interest in some systematic

way.  For example, let's specify that attribute 1 is not going to be

"blue"; it's going to be a green that continuously cycles from normal

intensity to high intensity and back.  The effect is similar to blinking

text, but more subtle.  To do this, set the colors for attribute 1 in

each of the palettes as follows (RGB values in hex):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 2C 2E 31 33 35 36 37 38 39 3A 3B 3C 3D 3E 3F

B      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

Note that the intensity of the green primary slowly increases from

normal (2Ah) in palette 0 to maximum intensity (3Fh) in palette 15.

Also note that attribute 1 no longer has anything to do with blue --

it's green.

If these values are loaded into the video DAC color registers, and you

then sequence through the palettes in a cyclical pattern (0->15->0),

characters with attribute 1 will slowly "pulse" from green to bright

green.  The effect is more subtle - and readable - than simply switching

the characters off and on.

The palette change is easily and efficiently accomplished by setting the

color select register -- say, in a timer tick intercept routine.  Using a

timer intercept is an ideal way to accomplish this sort of special

effect.

It is both efficient and flexible:  the speed of the effect can be

adjusted by altering the values of either the incremental rate of the

palette changes and/or the interval period of timer ticks before a change

is made.  Increasing the increment value for palette changes causes the

pulse rate to speed up by skipping palettes; increasing the interval

period between timer ticks before making a change slows the pulsing by

skipping clock ticks.

Effects

-------

Many "special effects" are possible using this technique.

PERIODIC INTENSIFICATION

Pulsing is a gradual change from a low intensity color to a high

intensity color.  The effect can be hardened (resulting in blinking) by

using the normal color for half of the palettes and an intensified

version for the other half.  For example, the following palettes blink

cyan to bright cyan (RGB values in hex):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 2A 2A 2A 2A 2A 2A 2A 3F 3F 3F 3F 3F 3F 3F 3F

B     2A 2A 2A 2A 2A 2A 2A 2A 3F 3F 3F 3F 3F 3F 3F 3F

[ fx_Blink( "bg", "bg+" ) ]

The effect can be varied by altering the ratio of low intensity palettes

to high intensity palettes ("beacons", below, are simply extreme cases

of periodic intensification).

COLOR CHANGES

Another effect is to blink text from one color to another.  This set of

palettes blinks bright green to bright red:

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     3F 3F 3F 3F 3F 3F 3F 3F  0  0  0  0  0  0  0  0

B      0  0  0  0  0  0  0  0 3F 3F 3F 3F 3F 3F 3F 3F

[ fx_Blink( "g+", "r+" ) ]

GRADED COLOR CHANGES

An interesting effect is to "grade" (fade) one color into another.

Rather than simply blinking green to red, change it gradually by fading

out the green primary and fading in the red (RGB values in hex):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  2  4  6  9 0C 0F 12 15 18 1B 1E 21 24 27 2A

G     2A 27 24 21 1E 1B 18 15 12 0F 0C  9  6  4  2  0

B      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

[ fx_Fade( "g", "r" ) ]

This attribute will still change from green to red, but the change is

not instantaneous; it slowly fades from one color to the other, passing

through various other unnamed colors (all combinations of red and green)

on the way.

SIMULATED BLINKING

Regular hardware blinking can be simulated via a variant of the above

scheme; simply use the background color for half of the palettes (or the

foreground color, if you want to flash the background) (RGB values in

hex):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R     3F 3F 3F 3F 3F 3F 3F 3F  0  0  0  0  0  0  0  0

G     3F 3F 3F 3F 3F 3F 3F 3F  0  0  0  0  0  0  0  0

B      0  0  0  0  0  0  0  0 2A 2A 2A 2A 2A 2A 2A 2A

[ fx_Blink( "gr+", "b" ) ]

The example flashes yellow text if displayed on a blue background.

SOFTENED BLINKING

By placing the background attribute in half of the palettes (as for

flashing) and fading out the foreground in the other half, a softer

version of blinking is possible (RGB values in hex):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 24 1E 19 14 0F 0A  5  0  0  0  0  0  0  0  0

B     2A 24 1E 19 14 0F 0A  5  0  0  0  0  0  0  0  0

[ fx_Blink( "bg", "n", .T. ) ]

The example shows softened blinking of cyan text on a black background.

If any of the background primaries are also components of the

foreground color, do not fade those (RGB values in hex):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 24 1E 19 14 0F 0A  5  0  0  0  0  0  0  0  0

B     2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A

[ fx_Blink( "bg", "b", .T. ) ]

The example shows softened blinking of cyan on blue.

FADING

This is an even softer version of blinking text.  It fades the

foreground over the full 16 palettes by grading the foregound color

into the background color (RGB values in hex):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 27 24 21 1E 1B 18 15 12 0F 0C  9  6  4  2  0

B      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

[ fx_Fade( "g", "n" ) ]

The example fades a green foreground into a black background.  If the

background is a color other than black, a graded color change from the

FG color to the BG color is used.

STROBES AND BEACONS

These are two other effects (not in the demo).  A strobe is a color

that's invisible most of the time (BG=FG) but flashes briefly to maximum

intensity.  The following palettes strobe yellow when used with a blue

background (RGB values in hex):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0 3F

G      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0 3F

B     2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A  0

A beacon is a low intensity color that briefly flashes to maximum

intensity.  The following palettes create a blue beacon (RGB values in

hex):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

B     20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 3F

IMPORTANT:  Note that only palettes that are a multiple of the palette

incremant rate are ever used.  E.g., if the palette increment rate is 2,

palette 15 will not be used; put the strobe or beacon color in palette

14 instead.

Advantages/disadvantages

------------------------

Creating special effects in this manner has several advantages over

using hardware effects (such as blinking), and some disdvantages.  Among

the advantages are:

- MANY more effects are possible; I have mentioned only a few of the

possibilities.

- Transition rates are under your control; the hardware blink rate

is fixed.

- Effects can be mixed on one screen (one attribute could be a

strobe, another a graded color change).

- Any effect can be applied to background as well as foreground.

Among the disadvantages are:

- A background timer routine has some effect on system efficiency

(but it's small).

- Effects are associated with attribute numbers, not with bit

positions.

The last one bears some explanation.  Hardware blinking is associated

with a single bit (bit 7) of the FG/BG attribute byte.  Thus, you have

16 FG colors, all of which can be blinked.  Software-controlled special

effects, however, are associated with attributes.  If you set, say,

attribute 6 to be pulsed green, you no longer have a brown attribute

available.  Thus, the number of effective "base" colors is reduced for

each special effect attribute you create.

Mode 0

------

I have covered attribute control mode 1 in some detail, but some of the

same effects can be accomplished in mode 0.  The primary difference is

that you have only four palettes to work with.  The simpler effects such

as color changes, periodic intensification, and simulated blink can

easily be accomplished in mode 0 by putting the second color in palette

1, and then alternating between palettes 0 and 1.

More complex effects would require periodic alterations to the palette

registers and/or color plane enable register.

Programming

-----------

The programming required to accomplish these special effects is

relatively straightforward.  The VGA BIOS provides the necessary

services; but register-level programming is used in FXCOLOR to eliminate

distracting "blips" caused when RGB values or palettes are changed.

The linkable object module

--------------------------

FXCOLOR.OBJ(ASM) provides a linkable object module containing callable

functions for Clipper 5.01 that can be used to generate all of the

special effects described above, and others.

Be sure to call fx_Enable() when you start and fx_Disable() when you

exit.

The demo program (FXDEMO.PRG) demonstrates use of the commands.

Author and reference

--------------------

The original document is by:

Chris Dunford

CompuServe ID# 76703,2002

The original document is copyright (C) 1989 by Chris Dunford.  Permission

is granted to distribute freely by electronic or other means, but it may

not be republished without permission.  This document is an edited version

of the original and is presented here with permission.

Edited by:

Jim Fowler

CompuServe ID# 73340,3325

Clipper is a registered trademark of Nantucket Corp.