WHAT YOU ALWAYS WANTED	TO KNOW	ABOUT MATH COPROCESSORS

This document has been	created	to provide the net.community

with some detailed information	about mathematical coprocessors

for the Intel 80x86 CPU family. It may	also help to answer

some of the FAQs (frequently asked questions) about this topic.

The focus of this document is on 387 compatible chips,	but

there is also some information	on the other chips in the 80x87

family	and the	Weitek coprocessors. Care was taken to make the

information included as accurate as possible. If you think you

have discovered erroneous information in this text, or	think

that a	certain	detail needs to	be clarified, or want to suggest

additions to this text, feel free to contact me at:

S_JUFFA@IRAVCL.IRA.UKA.DE

or at my snail	mail address:

Norbert Juffa

Wielandtstr. 14

7500 Karlsruhe	1

Germany

This is the second version of this document and I'd like to thank

those who have	helped improving it by commenting on the previous

version:

Fred Dunlap (cyrix!volt!fred@texsun.Central.Sun.COM), Peter

Forsberg (peter@vnet.ibm.com),	Richard	Krehbiel (richk@grevyn.com),

Arto Viitanen (av@cs.uta.fi), Jerry Whelan (guru@stasi.bradley.edu),

Eric Johnson (johnson%camax01@uunet.UU.NET), Bengt Ask

(f89ba@efd.lth.se), Thomas Hoberg (tmh@prosun.first.gmd.de),

Nhuan Doduc (ndoduc@framentec.fr), John Levine

(johnl@iecc.cambridge.ma.us)

CONTENTS of this document

1) What are math coprocessors?

2) What applications benefit from using a math	coprocessor

3) Installing a math coprocessor

4) Description	of available math coprocessors,	special	features,

available speeds, packaging, power consumption

5) Price information

6) How	do math	coprocessors work

7) Performance	comparison of math coprocessors

8) Test for IEEE-754 conformance and accuracy of transcendental

functions for different math coprocessors

9) References (literature)

10)Addresses of manufacturers of math coprocessors

11)Appendix A:	Test programs for partial compatibility	checks

12)Appendix B:	Benchmark programs TRNSFORM and	PEAKFLOP

What are math coprocessors?

A coprocessor in the traditional sense	is a processor that extends

the capabilities of a CPU in a	transparent manner. This means that

from the programmer's view the CPU and coprocessor together look

like one machine. The 80x87 math coprocessors are typical examples

of such coprocessors. The 80x86 CPUs (with the	exception of the 80486,

which has a built-in 'coprocessor') can only handle 8,	16, or 32 bit

integers as their primary data	types. However,	many applications

require the use of floating-point numbers. Simply put,	use of floating

point numbers enables one to express not only integers, but also

fractional values over	a wide range. The most common application

of floating point numbers is in scientific applications, where	very

small (e.g. Planck's constant) and very large numbers (e.g. speed

of light) have	to be expressed. But floating-point numbers are	also

useful	for business applications such as computing interest. Since

the 80x86 CPUs	do not support floating-point numbers or operations

on them directly, they	have to	be programmed using the	CPU's integer

capabilities. This results in slow computations when floating-point

numbers are used. This	is where the 80x87 coprocessors	come in.

Adding	a 80x87	to a 80x86 based system	augments the CPU architecture

with eight floating point registers, five additional data types and

over 70 additional mnemonics. This greatly enhances the system's

capability to do floating-point computations, as the coprocessor is

specifically designed to handle floating-point	numbers	efficiently.

Like most things in life, floating-point arithmetic has been

standardized. The relevant standard, to which I will refer quite

often in this document, is IEEE-754 Standard for Binary Floating-Point

Arithmetic [10,11]. The standard specifies numeric formats, value

sets and how the basic	arithmetic (+,-,*,/,sqrt, remainder) has to

work. All the coprocessors covered in this document claim full	or

at least partial compliance with this standard. When browsing the

literature for	information on math coprocessors, you will also

encounter quite a few acronyms	that refer to them: MCP	(Math

CoProcessor), NDP (Numerical Data Processor), NPX (Numerical

Processor eXtension), FPU (Floating Point Unit). The latter usually

refers	to the 'built-in coprocessor' of the i486.

The only data type the	80x87 coprocessors (and	the 80486 floating

point unit, or	FPU) can hold in their registers is an 80-bit long

floating-point	number.	This data type (called temporary real or

double	extended precision) can	represent numbers which	range in

size between 3.36*10^-4932 and	1.19*10^4932 (3.65*10^-4951 to

1.19*10^4932 including	denormal numbers) where	the '^'	denotes	the

power operator. For those familiar with floating point	formats, this

format	has 64 mantissa	bits, 15 exponent bits and 1 sign bit for

the total of 80 bits. This format provides a precision	of about

19 decimal places. The	80x87 can handle additional data types

that are converted to/from the	internal format	upon being loaded/

stored	to/from	the coprocessor. These include 16 bit, 32 bit, and

64 bit	integers as well as a 18 digit BCD (binary coded decimal)

occupying 10 bytes and	two additional floating	point types. The

short real data type, also called single precision, has 32 bits

that split into 23 mantissa bits, 8 exponent bit and a	sign

bit. This format provides a precision of about	6-7 decimal places

and can represent numbers between 1.17*10^-38 and 3.40*10^38

(1.40*10^-45 to 3.40*10^38 including denormal numbers). The long

real, or double precision, data type has 64 bits, consisting of

52 mantissa bits, 11 exponent bits and	the sign bit. It provides

15-16 decimal digits of precision and can handle numbers from

2.22*10^-308 to 1.79*10^308 (4.94*10^-324 to 1.79*10^308 including

denormal numbers).

In addition to	load/store the above mentioned operand types, the

80x87 coprocessors can	perform	all the	basic arithmetic operation

on floating point numbers. Besides 'knowing' how to add, subtract,

multiply and divide they can also compare floating-point numbers,

change	the sign, take the square root or absolute value, compute

the remainder and compute some	of the transcendental functions,

like the logarithm. The eight registers in the	80x87 are organized

in a stack-like manner	which takes some time getting used to if

one programs the coprocessor directly in assembler. However,

nowadays the compilers	or interpreters	for most high level

languages (HLL) can give the programmer access	to the coprocessor's

data types and	use their instructions,	so there is not	much need

to worry about	the rather unusual architecture	of the 80x87.

Strictly speaking, the	Weitek Abacus 3167 and 4167 are	not

coprocessors in that they do not transparently	extended the

CPU architecture. Rather they could be	described as special

memory	mapped IO-devices. Since the term coprocessor has been

traditionally used for	these chips, they are also called by

that term in this document. The architecture of the Weitek

chips differs significantly from the 80x87. The Weitek's register

file consists of 31 32-bit register, each one capable of holding

an IEEE single	precision number. Pairs	of consecutive single

precision registers can also be used as 64-bit	IEEE double

precision register. Thus there	are 15 double precision	registers.

The Weitek register file has the standard organization	known from

other registers files like those in the 80386,	not the	special

stack-like organization of the	80x87 coprocessors. The	Weitek

coprocessors have been	tuned for maximum performance. Therefore,

only a	small instruction set has been implemented, but	each

instruction executes at a very	high speed, usually only a few

clock cycles each. Instructions available are load/store, add,

subtract, subtract reverse, multiply, multiply	and negate,

multiply and accumulate, multiply and take absolute value,

divide	reverse, negate, absolute value, compare/test, convert

fix/float, and	square root. Note that the Weitek Abacus does not

support a double extended format, has no built-in transcendental

functions, and	does not support denormals. The	ressources required

to implement such features have instead been devoted to implement

the basic arithmetic operations as fast as possible. While the

80x87 coprocessors perform all	internal calculations in double

extended precision and	therefore have about the same performance

for single and	double precision calculations, the Weitek features

explicitly single and double precision	operations. For	applications

that require only single precision the	Weitek provides	additional

performance that way, as single precision operations are about

twice as fast as their	double precision counterparts. Since the

Weitek	Abacus has more	registers than the 80x87 coprocessors,

values	can be kept in registers more often and	have to	be loaded

from memory less frequently. This also	leads to a performance gain.

To the	CPU, the Weitek	Abacus looks like a 64 kB block	of memory

starting at physical address 0C0000000h. Every	address	in this

range corresponds to a	coprocessor instruction. Accessing a

specified memory location within this block with the 386/486's

MOV instruction causes	the corresponding instruction to be executed.

The instructions have been assigned to	memory locations in such a

way that loads	to consecutive coprocessor registers can make use

of the	386/486	MOVS string instruction. The memory mapped interface

of the	Weitek coprocessors is much faster than	the IO-oriented

protocol that is used to couple the CPU to the	80287 and 80387

coprocessors. The Weitek's starting address of 0C0000000h is only

a physical address. The Weitek's memory block can be assigned to

any logical address using the MMU (memory managment unit) in the

386/486's protected and virtual modes. This also means that the

Weitek	Abacus 3167 and	4167 can *not* be used in the real mode

of those processors, since the	physical start address of the

Weitek	coprocessors is	not within the 1 MByte address range and

the MMU is inoperable in real mode. However, DOS programs can

make use of the Weitek	Abacus by using	a DOS extender or a

memory	manager	like EMM386 that run in	protected/virtual mode

themself and can therefore map	the Weitek's memory block to

any desired location in the 1 MByte address range. Typically

the FS	segment	register is then set up	to point to the	Weitek's

memory	block. On the 80486, this technique has	severe draw backs,

as using the FS: prefix takes an additional clock cycle, thereby

nearly	halving	the performance	of the 4167. Most DOS based compilers

exibit	this problem, so the only way aroun it is to code in assembly

language [75].	The Weitek Abacus 3167 and 4167	are also supported

by the	UNIX operating system [33].

What applications will	profit by using	a math coprocessor?

According to the Intel	387DX User's Guide, there are more

than 2100 commercial programs that can	make use of a 387

compatible coprocessor. Every program that uses floating

point arithmetic somewhere and	supports a 80x87 coprocessor

can gain speed	by installing a	coprocessor. However, the

speedup will vary from	program	to program and even within

the same program depending on how computation intensive the

program or operation within the program is. Typical applications

that benefit from the use of a	80x87 coprocessor are:

- Business graphics programs, such as Arts&Letters, Freedom

of Press, and Freelance

- Spreadsheet programs	like Lotus 1-2-3, Excel, Quattro, and

Wingz

- CAD programs	such as	AutoCAD, VersaCAD, and GenericCAD

- Database programs such as dBase IV, FoxBase,	and Paradox

- Math	and Science programs such as Mathematica, TKSolver,

SPSS/PC, and	Statgraphics

Note that for spreadsheets and	databases, a coprocessor

only helps if some kind of floating point computations

is performed. This is true more often for spreadsheets

than for data bases. Also note	that the speed of many

programs depends quite	heavily	on the speed of	the graphics

adapter (CAD) or the disk performance (databases), so the

computational performance is only a (small) part of the

total performance of the application. There are some programs

that won't run without a coprocessor, among them AutoCAD R10

and later and Mathematica. Most GUIs (graphical user interfaces)

such as Microsoft Windows or OS/2's Presentation Manager do

*not* gain additional speed from using	a *mathematical* coprocessor,

since their graphics operations only use integer arithmetic [71].

They benefit from a graphics board with a graphics 'coprocessor'

though	that speed up certain common operations	such as	BitBlt

or line drawing. A few	GUIs used on PCs, e.g. X-Windows, use

a certain amount of floating point operations for operations

such as arc drawing. However, the use of floating-point operations

in X-Windows seems to have decreased significantly in versions

after X11R3, so the overall performance impact	of a coprocessor

is small [72].	Applications running under any GUI may take

advantage of a	math coprocessor though, e.g. Excel under MS-Windows.

While support for 80x87 coprocessors is very common in	application

programs, the Weitek Abacus coprocessors do not enjoy such wide

spread	support. Due to	their high price, only a few high-end PCs

have been equipped with Weitek	coprocessors. Therefore	most of

the programs that support these coprocessors are also high-end

products like AutoCAD and Versacad-386.

Installing a math coprocessor

Usually, installing a coprocessor doesn't pose much of a problem,

as every coprocessor comes with installation instructions and a

diagnostic disk that lets you check for correct operation once

the coprocessor has been installed. In	addition, the user manuals

of most computers have	a section on coprocessor installation.

1) Make sure to get the right coprocessor for your system. An

8087 works together	with 8086, 8088, V20, and V30 CPUs. An

80287, 287XL or compatible works together with a 80286 CPU.

There are also some	old 386	motherboards that accept a 80287

coprocessor, but they usually also provide a socket	for the

387	and I recommend	to get a 387 then for use with these

systems. A 80387, 387DX or compatible coprocessor is for 386

based systems, as is the Intel RapidCAD. 387 coprocessors

also work together with Cyrix' 486DLC CPU which despite its

name does not include an FPU. Similarly, the 387SX or compatible

coprocessor	go into	systems	whose CPU is a 386SX or	Cyrix 486SLC.

The	Weitek Abacus 3167 works with a	386 CPU	but requires a

121-pin EMC	socket in your system. Some computers, such as

IBM's PS/2s, don't have this socket. The Weitek Abacus 4167

works together with	the 486	and requires the appropriate

142-pin socket to be present.

Always install a coprocessor that is rated at the same speed

as the CPU.	That is, for a 40 MHz 386 system using AMD Am386-40,

install a coprocessor rated	for 40 MHz such	as a Cyrix 83D87-40,

C&T	38700DX-40, IIT	3C87-40, or ULSI 83C87-40. Running a

coprocessor	above its specified frequency rating may cause it to

produce false results, which you might fail	to recognize as	such.

I have personally experienced this problem with a Cyrix 83D87-33

that I tried to push to 40 MHz. It passed all the diagnostic

benchmarks on the Cyrix diagnostic disk and	the tests of some

commercial system test programs. However, I	found it to fail the

Whetstone and Linpack benchmarks, which include accuracy

checks. So although	there is usually no problem with overheating

when pushing a coprocessor over the	specified maximum frequency

rating, be warned that operation of	a coprocessor above the

maximum ratings stated by the manufacturer makes operation

unreliable.	Some 386 boards	allow the coprocessor to be clocked

differently	than the CPU. This is called asynchronous operation

and	allows you to run the coprocessor at 33	MHz while the CPU

runs at 40 MHz, for	example. Please	note that only the Intel

80387 and 387DX support asynchronous operation. The	387 'clones'

from Cyrix,	C&T, IIT and ULSI always run at	the full speed of the

CPU, even if you have set up your motherboard for asynchronous

operation.

2) Once you've got the correct coprocessor for your system you

can	start the actual installation process:

- turn off the computer's power switch and unplug the power

cord from	the wall outlet

- remove the cover of your computer

- locate the math coprocessor socket. This socket is located

right next to the	CPU, which can be identified by	the

printing on top of the chip. The CPU usually is one of the

biggest chips on the board. The 8078 and 80287 DIL sockets

are rectangular sockets with 20 pin holes	on each	of the

longer sides. The	387SX PLCC socket is a square socket that

has 17 vertical connector	strips on the 'wall' of	each side.

The 387 PGA socket is square and has two rows of pin holes

on each side. The	EMC socket is similar but has three rows

of holes on each side. The PGA socket for	the Weitek 4167	is

also square with three rows of holes on each side. If the	CPU

and coprocessor socket is	on a separate card rather than on

the motherboard (typical for modular systems), you have to

remove the card and place	it on a	flat and hard surface free

of static	electricity. If	you can't find the math coprocessor

socket, consult your owner's manual or your computer dealer.

If you want to install the Intel RapidCAD	in a 386 system,

you will have to remove the 386 CPU before starting to

install the two RapidCAD chips. Intel provides an	easy to

use chip extractor and a storage box for the 386 chip for

this purpose. Just follow	the instructions in the	RapidCAD

installation manual.

- On many systems, the motherboard is only supported at a small

number of	points.	Since considerable force is required to

insert a pin grid	chip like the 80387, RapidCAD, or Weitek

Abacus 3167 into it's socket, the board may bend quite a lot

due to the insertion pressure. This may lead to cracks in

the board	that may render	it inoperable. Damage done to the

board in this way	is usually not covered by the warranty on

the part.	Therefore, it may be a good idea to check how much

the board	bends by pressing on the math coprocessor socket

with a finger. If	you find it to bend easily, try	to put

something	under the board	directly beneath the coprocessor

socket. If this is impossible, like in most desktop cases,

consider removing	the whole mother board from the	case, and

placing it on a hard and flat surface free of static electricity.

- Be sure you are properly grounded	before you remove the

coprocessor from its antistatic box. Static electricity

can damage the coprocessor. Make sure you	do not touch

the pins.

- Check if all pins	are straight and not bend. If you find

bent pins, carefully straigthen them with	needle-nose pliers

or tweezers.

- Match the	coprocessor's orientation with the orientation

of the socket. 8087 and 287 coprocessors have a notch on one

the shorter sides	of their rectangular DIL package that should

be matched with the notch	of the coprocessor socket. Usually

the 286 CPU and the 287 coprocessor are placed alongside each

other and	both have the same orientation,	that is	their

respective notches point in the same direction. 387SX

coprocessors feature a white dot or similar mark that matches

with some	sort of	marking	on the socket. 387 coprocessors

have a beveled corner that is also marked	with a white dot

or similar marking. This should be matched with the beveled

or otherwise marked corner of the	socket.	If you install

a	387 coprocessor	in an EMC socket, leave	one row	of holes

free on each side. Correct orientation of	the coprocessor

is absolutely essential, because if you insert it	the

wrong way	it may be damaged. If you have found the correct

orientation, make	sure all pins are correctly aligned with

their respective holes. Press firmly and evenly on the chip.

You may have to press hard to seat the coprocessor all the

way. Make	sure your motherboard does not bend more than

slighty under the	insertion pressure. Otherwise it may

develop cracks that could	damage the signal lines	on the

board. For 8087, 287, and	387 coprocessors it is normal that

the coprocessor does not go all the way in but about one

millimeter (1/25 inch) of	space is left between the socket

and the bottom of	the coprocessor	chip. This enables the

insertion	of a extraction	device should it become	necessary

to remove	the coprocessor. Note that the construction of the

387SX's PLCC socket makes it next to impossible to remove

the coprocessor once fully inserted, as the top of the chip

is level with the	socket's 'walls' then.

3) Check your computer's manual for the jumpers and/or switches

you	may have to set	for coprocessor	operation.

Put	the cover back on the system unit and reconnect	the power.

Turn on your computer. Depending on	your BIOS, you may have

to run the setup or	configuration program to register the

coprocessor.

Use	the diagnostic disk included with your coprocessor to

check for correct operation	of your	coprocessor.

Coprocessor emulations

In the	absence	of a coprocessor, floating-point calculations

are most often	performed by a software	package	that simulates

the operations	of the coprocessor. Such a program is called

a coprocessor emulator. Simulating the	coprocessor has	the

advantage that	identical code can be generated	for use	with

either	the coprocessor	and the	emulator so that it is possible

to write programs that	run on both, systems with and systems

without a coprocessor,	without	any changes. Wether the	program

is to use the coprocessor or the emulator can then be determined

at run-time by	checking if a math coprocessor is present in

the system.

Two approaches	to interface an	80x87 emulator to programs are

common. While the first method	works with all 80x86 processors,

the second only works from the	80286 on. The first method makes

use of	the fact that all coprocessor instruction start	with the

same five bit pattern 11011. Thus the first byte of a coprocessor

instruction will be in	the range D8-DF	hexadecimal. In	addition,

coprocessor instructions usually are preceeded	by a WAIT instruction

(opcode 9Bh) which is one byte	long (the reason for doing this

is described in a later chapter on the	operation of the 80x87).

One common approach is	to replace the WAIT instruction	and the

first byte of the coprocessor instruction with	one of eight

interrupts; the remaining bytes of the	coprocessor instruction

are left unchanged. Interrupts	34 to 3B hexadecimal are used for

this emulation	technique. Note	that the sequences 9B D8 .. 9B DF

can be	easily converted to the	interrupt instructions CD 34 ..	CD 3B

by simple addition and	subtraction of constants. The compiler or

assembler produces code that contains the appropriate interrupt

calls instead of the coprocessor instructions.	If a coprocessor

is detected at	run-time, the emulator interrupts point	to a short

routine that converts the interrupts calls back to coprocessor

instructions (self modifying code). If	no coprocessor is found

the interrupts	point to an emulation package which examines the

byte(s) following the interrupt intruction to determine what

operation to perform. The method described is used by the compilers

from Microsoft	and Borland for	example. It works with every

80x86 CPU from	the 8086/8088 on.

The second method to interface	an emulator is only available on

286 and 386 machines. If the emulation	bit in the machine status

word of these processors is set, the processors will generate an

interrupt 7 whenever a	coprocessor instruction	is encountered.

The interrupt vector then points to an	emulation package that

decodes the instruction and performs the desired operation. This

approach has the advantage that the emulator doesn't have to be

included in the program code, but can be loaded as a TSR or

device	driver once and	then used by every program that	requires

a coprocessor.	Emulation via interrupt	7 is transparent, which

means that programs containing	coprocessor instructions execute

just like a coprocessor was present, only slower. This	approach

is taken by the public	domain EM87 emulator and the commercial

Franke387 emulator, for example. Even programs	that require a

coprocessor to	run like AutoCAD are 'fooled' to believe that

a coprocessor is present with emulators using INT 7.

The size of the emulator used by TP 6.0 is about 9.5 kB, EM87

occupies about	15.8 kB	as a TSR, and Franke387	uses about 13.4	kByte

as a device driver. Note that Franke387 and especially	EM87 model

a real	coprocessor much more closely than Turbo Pascal's emulator

does. In particular, EM87 supports denormal numbers, precision

control, and rounding control.	The emulator in	TP 6.0 does not

implement these features. The version of Franke387 tested (V2.4)

supports denormals in single and double precision, but	not

double	extended precision. It supports	precision control, but

not rounding control. Intel's E80287 is supposed to be an 100%

exact emulation of the	80287 coprocessor [44].	Generally, the

more closely a	real coprocessor is modelled by	the emulator,

the slower does the emulator run and the larger the code for the

emulator gets.

Relative execution times of coprocessor vs. software emulators

for particular	coprocessor instructions

Intel 387DX    TP 6.0 Emulator	 EM87 Emulator

FADD ST, ST(0)	      1		     26		       104

FDIV [DWord]	      1		     22		       136

FXAM		      1		     10			73

FYL2X		      1		     33		       102

FPATAN		      1		     36		       110

F2XM1		      1		     38		       110

The following table is	an excerpt from	[44]:

Intel 80287  Intel E80287 Emulator

FADD ST, ST(0)	      1		     42

FDIV [DWord]	      1		    266

FXAM		      1		    139

FYL2X		      1		     99

FPATAN		      1		    153

F2XM1		      1		     41

The following has been	adapted	from [43] and merged with my own

data:

Intel 8087  TP 6.0 Emul. (8086)	 Intel Emul. (8086)

FADD ST, ST(0)	      1		     20			94

FDIV [DWord]	      1		     22			82

FPTAN		      1		     18		       144

F2XM1		      1		      6		       171

FSQRT		      1		     44		       544

One of	the reasons emulators are so slow is that they are

often designed	to run with every CPU from the 8086/8088 on.

This is the case with the emulators built into	the compiler

libraries of the Turbo	Pascal 6.0 (also used by Turbo C/C++)

and Microsoft C 6.0 compiler (probably	also used in other

Microsoft products) and is also true for the EM87 emulator

in the	public domain. By using	code that can run on a 8086/8088,

these emulators forego	the speed advantage offered by the

additional instructions and architectureal enhancements (such

as 32-bit registers) of the more advanced Intel 80x86 processors.

A notable exception is	the Franke387 emulator,	a commercial

emulator that is also sold as shareware. It uses 386 specific

32-bit	code and only runs on 386/386SX	computers.

Besides being slow, coprocessor emulators have	other drawbacks

compared with real coprocessors. Most of the emulators	do not

support the additional	instructions that the 387 compatible

coprocessors offer over the 80287. Often, some	of the low-level

stack-manipulating instructions like FDECSTP are not emulated.

For example, [76] lists the coprocessor instructions not emulated

by Microsoft's emulator (included in the MS-C and MS-Fortran

libraries) as follows:

FCOS	      FRSTOR	  FSINCOS      FXTRACT

FDECSTP      FSAVE	  FUCOM

FINCSTP      FSETPM	  FUCOMP

FPREM1	      FSIN	  FUCOMPP

Often,	some parts of the coprocessor architecture, like the status

register, are not or only partially emulated. Some emulators do not

conform to the	IEEE-754 standard in their implementation of the basic

arithmetic functions, while the coprocessors do. Also,	they sometimes

lack the support for denormals	(a special class of floating point

numbers) although it is required by the standard. Not all the 80x87

emulators support rounding control and	precision control (features

required by IEEE-754).	Most of	the omissions are aimed	at making

the emulator faster and smaller. Because of the shortcomings of

coprocessor emulators,	a real coprocessor is a	must for anybody

planning to do	some serious computations. At todays prices, this

shouldn't pose much of a problem to anybody.

Nhuan Doduc (ndoduc@framentec.fr) has tested a	number of standalone

coprocessor emulators for PCs,	among them the two emulators, EM87

and Franke387 V2.4, already mentioned.	He found Franke387 to be the

best in terms of reliablity, speed, and accuracy.

Available coprocessors, CPU+FPU as of 08-10-92:

Intel 8087	[43] was the first coprocessor that Intel brought

out for	the 80x86 family. It was introduced in 1980

and therefore does not have full compatibility with

the IEEE-754 standard for floating point arithmetic,

which was finally released in 1985. It complements

the 8088 and 8086 CPUs and can also be interfaced

to the 80188 and 80186 processors. It comes in a

40 pin CERDIP (ceramic dual inline package). It

is available in	5 MHz, 8 Mhz (8087-2), and 10 MHz

(8087-1) versions. The 8087 is implemented using

NMOS. Power consumption	is rated at max. 2400 mW [42].

A neat trick to	enhance	the processing power of	the

8087 for computations that use only the	basic

arithmetic operations (+,-,*,/)	and do not require

high precision is to set the precision control to

single precision. This gives one a performance

increase of up to 20%. For details about programming

the precision control, see program PCtrl in appendix A.

Intel 80187	is a rather new	coprocessor designed to	support	the

80C186 embedded	controller. It was introduced in 1989

and implements the complete 80387 instruction set.

It is available	in a 40	pin CERDIP (ceramic dual

inline package)	and a 44 pin PLCC (plastic leaded

chip carrier) for 12.5 and 16 MHz operation. Power

consumption is rated at	max. 675 mW for	the

12.5 MHz version and max. 780 mW for the 16 MHz

version	[37].

Intel 80287	[44] is	the original Intel coprocessor for the 80286

and was	introduced in 1983. It uses the	same execution

unit as	the 8087 and therefore has the same speed

(sometimes slower due to additional overhead in	CPU

coprocessor communication). As the 8087, it does not

provide	full compatibility with	the IEEE-754 floating

point standard released	in 1985. It was	manufactured

in NMOS	technology. There are 6	MHz, 8 MHz, and	10

MHz versions. The chip comes in	a 40 pin CERDIP

(ceramic dual inline package). Power consumption can

be estimated to	be the same as that for	the 8087,

which is max. 2400 mW. The 80287 has been replaced

in the Intel 80x87 family with its successor, the

Intel 287XL, which has been introduced in 1990.	The

287XL is done in CMOS. It is based on the 387 core

and therefore much faster than the 80287. There	may

still be a few of the old 80287	chips on the market

though.

Intel 80287XL	is the second generation 287 introduced	by Intel

in 1990. Since it is based on the 387 core, it

features full IEEE 754 compatibility and faster

execution of coprocessor instructions. Intel claims

about 50% faster operation than	the 80287 for typical

benchmark test such as Whetstone [45]. Comparison

with benchmark results for the AMD 80C287, which is

identical to the Intel 80287, support this claim [1].

The Intel 287XL	performed 66% faster than the AMD

80C287 on the fractal benchmark	and 66%	faster on

the Whetstone benchmark	in these tests.	Whetstone

results	from [46] show the Intel 287XL at 12.5 MHz

to perform 552 kWhets/sec as opposed to	the AMD's

80C287 289 kWhets/sec, a 91% performance increase.

A benchmark using the MathPak program showed the

Intel 287XL to be 59% faster than the Intel 80287

(6.9 sec. vs. 11.0 sec.) [26]. Since the 287XL

has all	the additional instructions and	enhancements

of a 387, most software	automatically identifies

it as an 80387 compatible coprocessors and makes

use of the extra features available like the FSIN

and FCOS instructions. The 287XL is done in CMOS

and therefore uses less	power than the older 80287,

which was done in NMOS.	The 287XL is rated for

speeds of up to	12.5 MHz. At 12.5 MHz, the power

consumption is rated at	max. 675 mW, about 1/4 of

the 80287 power	consumption. The 287XL comes in

either a 40 pin	CERDIP (ceramic	dual inline package)

or a 44	pin PLCC (plastic leaded chip carrier).	The

latter version is called the 287XLT and	intended

mainly for laptop use.

AMD 80C287	is an exact clone of the old Intel 80287 that was

brought	to market by AMD in 1989. It contains the

original microcode of the 80287	and is therefore

100% compatible	with this chip.	However, as the	name

indicates, the 80C287 is manufactured in CMOS and

therefore uses less power than an equivalent Intel

80287. At 12.5 Mhz, its	power consumption is rated

at max.	625 mW or slightly less	than that of the

Intel 80287XL [27]. There is also another version

called AMD 80EC287 that	uses an	'intelligent' power

save feature to	reduce the power consumption below

80C287 levels. Tests at	10.7 MHz show typical power

consumption for	the 80EC287 to be at 30mW, compared

to 150 mW for the AMD 80C287, 300 mW for the Intel

287XL and 1500 mW for the Intel	80287 [57]. The

80EC287	is therefore ideally suited for	low power

laptop systems.	The AMD	80C287 is available in speeds

of 10, 12, and 16 MHz. I have only seen	it being

offered	in 10 MHz and 12 MHz versions though. At

about US$ 50, it is the	cheapest coprocessor available.

Note that it provides less performance than the

newer Intel 287XL (see above for details). The AMD

80C287 is available in 40 pin ceramic and plastic

DIPs (dual inline package) and as 44 pin PLCC

(plastic leaded	chip carrier). Due to recent legal

battles	with Intel over	the right to use the 287

microcode, which AMD lost, AMD may have	to discontinue

this product (disclaimer: I am not a legal expert).

Cyrix 82S87	was developed from the Cyrix 83D87, Cyrix' 387 'clone'

and has	been available since 1991. It implements the

full 387 instruction set. It totally complies with

the IEEE-754 standard for floating point arithmetic

and features nearly total compatibility	with Intel's

coprocessors. It implements the	transcendental

functions with the same	degree of accuracy and the

superior speed of the Cyrix 83D87. This	makes the

Cyrix 82S87 the	fastest	[1] and	most accurate 287

compatible coprocessor available. Documentation	by

Cyrix [46] rates the 82S87 at 730 kWhets/sec for a

12.5 MHz system, while the Intel 287XL performs	only

552 kWhets/sec.	82S87 chips sold after about August

1992 use the internals of the Cyrix 387+, which

succeeds the original 83D87 [73]. The 82S87 is a

fully static CMOS design with very low power

requirements that can run at speeds of 6 to 20 MHz.

Cyrix documentation shows the 82S87 to consume about

the same amount	of power as the	AMD 80C287 (see	above).

The 82S87 comes	in a 40	pin DIP	or a 44	pin PLCC

(plastic leaded	chip carrier) compatible with the

pinout of the Intel 287XLT and ideally suited for

laptop use.

IIT 2C87	was the	first 287 clone	available. It was introduced

to the market in 1989. It has about the	same speed

as the Intel 287XL [1].	The 2C87 implements the

full 387 instruction set [38]. Tests I ran on the

3C87 seem to indicate that it is not fully compatible

with the IEEE-754 standard for floating-point

arithmetic (see	below for details), so it can be

assumed	that the 2C87 also fails these test as it

presumably uses	the same core as the 3C87. The IIT

2C87 provides extra functions not available on any

other 287 chip [38]. It	has 24 user accessible

floating-point registers organized into	three register

banks. Additional instructions (FSBP0, FSBP1, FSBP2)

allow switching	from one bank to another. Transfers

between	registers in different banks are not

supported however, so this feature by itself

is of limited usefulness. Also there seems to

be only	one status register (containing	the

stack top pointer), so it has to be manually

loaded and stored when switching between banks

with a different number	of registers in	use [40].

The register bank's main purpose is to aid the

fourth additional instruction the 2C87 has

(F4X4),	which does a full multiply of a	4x4 matrix

by a 4x1 vector, an operation common in	3D graphics

applications [39]. The built-in	matrix multiply

speeds this operations up by a factor of 6 to 8

compared with a	programmed solution according to

the manufacturer [38]. Tests show the speed-up

to be indeed in	this range [40]. For the 3C87, I

measured the execution time of F4X4 to be about

280 clock cycles, the execution	time on	the 2C87

should be somewhat bigger. I estimate it to be

around 310 clock cycles	due to the higher CPU-NDP

communication overhead in instruction execution	in

286/287	systems	(~45-50	clock cycles) compared with

386/387	systems	(~16-20	clock cycles). As useful as

the F4X4 instruction may seem, there are only very

few applications that make use of this feature if

a IIT coprocessor is detected at run time, among

them Schroff Development's Silver Screen and

Evolution Computing's Fast-CAD 3-D [25]. The 2C87

is available for speeds	of up to 20 MHz. It is

implemented in an advanced CMOS	process	and has

therefore a low	power consumption of typically

about 500 mW [38].

Intel 387	was the	first generation of coprocessors for the

Intel 386. It was introduced in	1986, about one

year after introduction	of the 80386. Early 386

system were therefore equipped with a 80287 and	a

80387 socket. The 80386	works together with the

80287 but the numerical	performance is hardly

adequate for such a system. The	80387 has since

been superseeded by the	Intel 387DX introduced

by a quiet change in 1990. You might find it

when aquiring an old 386 machine, though. The

80387 is about 20% slower than the newer 387DX

(see the paragraph below for detailed information).

Like the other 387 coprocessors, the 80387 is packaged

in a 68-pin ceramic PGA. The Intel 80387 is

manufactured using Intel's older 1.5 micron CHMOS

III technology that has	moderate power requirements.

Power consumption at 16	MHz is max. 1250 mW (750 mW

typical), at 20	MHz it is max. 1550 mW (950 mW

typical), and at 25 MHz	it is max. 1950	mW (1250 mW

typical) [60].

Intel 387DX	is the second generation Intel 387 that	was quietly

introduced in 1989. This version is done in a more

advanced CMOS process than the 80387 that enables

the coprocessor	to run at a maximum frequency of 33

MHz, while the 80387 had a maximum frequency of	25 MHz.

The 387DX is about 20% faster than the 80387 on	the

average	for the	same clock frequency. For a 386/387

system operating at 29 MHz the Whetstone benchmark

compiled with the highly optimizing Metaware High-C

V1.6 runs at 2377 kWhetstones/sec for the 80387	and

at 2693	kWhetstones/sec	for the	387DX, a 13% increase.

In a fractal calculation programmed in assembly

language, the 387DX performance	was 28%	higher than

the performance	of the 80387. The transcendental

functions have also sped up from the 80387 to the

387DX. In the Savage benchmark compiled	with the

Metaware High-C	V1.6 optimizing	compiler and running

on a 29	MHz system, the	80387 evaluated	77600 function

calls/second, while the	387DX evaluated	97800 function

calls/second, a	26% increase [7]. Some instructions

have been sped up a lot	more more than the average

20%. For example the FBSTP instruction has been	sped

up by a	factor of 3.64.	The Intel 387DX	(and its

predecessor 80387) are the only	387 coprocessors

that support asynchronous operation of CPU and NDP.

The 387	consists of a bus interface unit and a

numerical execution unit. The bus interface unit

always runs at the speed of the	CPU clock (CPUCLK2).

If the CKM (ClocK Mode)	pin of the 387 is strapped

to Vcc,	the numerical execution	unit runs at the

same speed as the bus interface	unit. If CKM is	tied

to ground, the numerical execution unit	runs at	the

speed provided by the NUMCLK2 input. The ratio of

NUMCLK2	(coprocessor clock) to CPUCLK2 (CPU clock)

must lie within	the range 10:16	to 14:10. For example,

for a 20 MHz 386, the Intel 387DX could	be clocked

from 12.5 MHz to 28 MHz	via the	NUMCLK2	input. On

the Cyrix 83D87, Cyrix 387+, ULSI 83C87, and the IIT

387, the CKM pin is not	connected. These coprocessors

always run at the speed	of the CPU. The	Intel 387DX

is manufactured	using Intel's advanced low power

CHMOS IV technology. Power consumption at 20 MHz is

max. 900 mW (525 mW typical), at 25 MHz	it is max.

1050 mW	(625 mW	typical), and at 33 MHz	it is 1250

mW (750mW typical) [59].

Intel 387SX	is the coprocessor for the Intel 386SX.	The 386SX is

an Intel 386 with a 16-bit data	path. This reduces

somewhat the costs to build a complete system as

compared to a full 32-bit design required by the

80386DX. The 386SX main	purpose	was to replace the

80286 CPU, which Intel subsequently stopped producing.

Due to the 16-bit data path, the 386SX is slower than

the 386DX and offers about the same speed as a 80286

at the same clock frequency for	16-bit applications.

As the 386SX is	a complete 80386, it offers also the

possibility to run 32-bit applications and supports

the virtual 8086 mode used for example by Windows'

enhanced mode. The 387SX has all the features the

Intel 387DX offers, including the ability for

asynchronous operation of CPU and coprocessor

(see the above paragraph on the	Intel 387DX for

details). Due to the 16	bit data path between the

CPU and	the coprocessor, the 387SX is a	bit slower

than a 387DX operating at the same frequency. The

387SX comes in a 68-pin	PLCC (pastic leaded chip

carrier) package and is	available in 16	Mhz and	20

MHz versions. Coprocessors for faster 386SX systems

based on the Am386SX CPU are available from IIT,

Cyrix, and ULSI. Power consumption for the 387SX

at 16 MHz is max. 1250 mW (740 mW typical), for

the 20 MHz version it is max. 1500 mW (1000 mW

typical) [62].

IIT 3C87	came out in 1989 at about the same time	as the

Cyrix 83D87. Both coprocessors are faster than

Intel's 387DX coprocessor. Tests I ran with the

IEEETEST program show that the 3C87 is not fully

compatible with	the IEEE-754 standard for

floating-point arithmetic although the manufacturer

claims differently. It is well possible	that the

reported errors	are due	to personal interpretations

of the standard	by the program's author that have

been incorporated into IEEETEST	and that the

standard also supports the different interpretation

chosen by IIT. On the other hand, the IEEE test

vectors	incorporated into IEEETEST have	become

somewhat of an industry	standard [66] and Intel's

387, 486, and RapidCAD chips pass the test without

a single failure, so the fact that the IIT 3C87

fails some of the tests	indicates that it is not

fully compatible with the Intel	387 coprocessor.

My tests also show that	the IIT	3C87 does not

support	denormals for the double extended format.

It is not entirely clear wether	the IEEE standard

mandates support for extended precision	denormals,

as the IEEE-754	document explicitly only mentions

single and double precision denormals. Missing

support	for denormals is not a critical	issue with

most applications but there are	some programs for

which support of denormals is quite helpful, if	not

important [41].	Anyhow,	failure	of the 3C87 to

support	extended precision denormal numbers is an

incompatibility	with the Intel 387 and 486. The	3C87

provides extra functions not available on any other

387 chip [38]. It has 24 user accessible floating-point

registers organized into three register	banks.

Additional instructions	(FSBP0,	FSBP1, FSBP2)

allow switching	from one bank to another. Transfers

between	registers in different banks are not

supported however, so this feature by itself

is of limited usefulness. Also there seems to

be only	one status register (containing	the

stack top pointer), so it has to be manually

loaded and stored when switching between banks

with a different number	of registers in	use [40].

The register banks main	purpose	is to aid the

fourth additional instruction the 3C87 has

(F4X4),	which does a full multiply of a	4x4 matrix

by a 4x1 vector, an operation common in	3D graphics

applications [39]. I measured this instruction to

execute	in about 280 clock cycles, during which

time it	executes 16 multiplications and	12 additions.

The built-in matrix multiply speeds the	matrix by

vector multiply	up by a	factor of 3 compared

with a programmed solution according to	IIT [39].

The results for	my own TRNSFORM	benchmark support

this claim (see	results	below),	showing	a performance

increase by a factor of	about 2.5. This	makes

matrix multiplies on the IIT 3C87 nearly as fast as

on an Intel 486	at the same clock frequency. However,

there are only very few	applications that make use

of this	feature	if a IIT 3C87 is detected at run time,

among them Schroff Development's Silver Screen and

Evolution Computing's Fast-CAD 3-D [25]. Like the

387 'clones' from Cyrix	and ULSI, the 3C87 does	not

support	asynchronous operation of the CPU and the

coprocessor. The 3C87 always runs at the full speed

of the CPU. The	3C87 is	implemented in an advanced

CMOS process and has low power requirements of

typically about	600 mW.	It is available	in 16, 20,

25, 33,	and 40 MHz versions.

IIT 3C87SX	is the version of the IIT 3C87 that is intended	for

use with Intel's 386SX or AMD's	Am386SX	CPU. It	is

functionally equivalent	to the IIT3C87.	Due to the

16-bit data path between the CPU and the coprocessor

in a 386SX based system, coprocessor instructions

will execute somewhat slower than on the 3C87. The

IIT 3C87SX is the only 387SX coprocessor that is

offered	at speeds of 16, 20, 25, and 33	MHz right

now. I have read that Cyrix has	also annouced a

83S87-33, but haven't seen it being offered yet.

The 3C87SX is packaged in a 68-pin PLCC.

Cyrix 83D87	was introduced in 1989,	only shortly after the

coprocessors from IIT. It has been the fastest

387 compatible coprocessor in several benchmark

comparisons [1,7,68,69]. It also came out as the

fastest	coprocessor in my own tests (see benchmark

results	below).	Although the Cyrix 83D87 provides

up to 50% more performance than	the Intel 387DX

in benchmarks comparisons, the speed advantage

over other 387 compatible coprocessors in real

applications is	usually	much smaller. For example,

in a test using	the program 3D-Studio, the Cyrix

83D87 was 6% faster than the Intel 387DX [1].

Besides	being the fastest 387 coprocessor, the

83D87 also offers the most accurate transcendental

functions results of all coprocessors tested (see

test results below). The new version of	the 83D87,

which is sold as 387+ in Europe, even surpasses

the level of accuracy of the original 83D87 design.

Unlike Intel's coprocessors, which use the CORDIC

[18,19]	algorithm to compute the transcendental

functions, Cyrix uses polynomial and rational

approximations to the functions. In the	past the

CORDIC method has been popular since it	requires

only shifts and	adds which makes it easy to implement.

It is also reasonably fast. Recently, the cost for

the implementation for fast floating-point multipliers

has dropped significantly due to the availablity of

VLSI, making the use of	polynomial and rational

approximations superior	to CORDIC for the generation

of transcendental functions [61]. The Cyrix 83D87

uses a fast array multiplier, making its transcendental

functions faster than those of any other 387 compatible

coprocessor. It	also uses 75 bit for the mantissa

in intermediate	calculations (as opposed to 68 bits

on other coprocessors),	making its transcendental

functions more accurate	than those of any other

coprocessor or FPU (see	results	below).	The 83D87

and its	successor, the 387+ are	the 387	'clones'

with the highest degree	of compatibility. There

are only very few SW and HW incompatibilties with

the Intel 387DX. These have been documented by

Cyrix [12]. The	software differences are caused

by some	bugs present in	the 387DX that Cyrix fixed

for the	83D87. Unlike the Intel	387DX, the 83D87

(and all other 387 'clones' as well) does not support

asynchronous operation of CPU and coprocessor. There

have also been problems	in the past with the CPU -

coprocessor communication, causing the 83D87 to

hang on	some machines. The reason was that Cyrix

shaved off a wait state	in the communication protocol,

which caused a communications breakdown	between	the

CPU and	the 83D87 for some systems running at 25 MHz

or faster. One notable example of this behavior	was

the Intel 302 board. The problem is only rarely

encountered with the current generation	of 386

motherboards. It is possible that the problem has

been entirely eliminated in the	387+, the sucessor

to the 83D87. To reduce	power consumption the 83D87

features advanced power	saving features. Those

portions of the	coprocessor that are not needed

are automatically shut down. If	no coprocessor

instructions are being executed, all parts except

the bus	interface unit are shut	down [12]. Maximal

power consumption of the Cyrix 83D87 at	33 MHz is

1900 mW, typical power consumption at this clock

frequency is 500 mW [15].

Cyrix EMC87	is basically a special version of the Cyrix 83D87.

In addition to the normal 387 operating	mode, in

which coprocessor-CPU communication is handled thru

reserved IO-ports, it also offers a memory-mapped

mode of	operation similar to the operation principle

of the Weitek Abacus. Please note that the EMC87 is

*not* compatible with Weitek's Abacus coprocessor.

They both use the same interface technique (memory

mapping) but while the EMC87 uses the standard 387

instruction set, the Weitek coprocessors use a

different instruction set of their own.	Like the

Weitek Abacus, the EMC87 occupies a 64 kByte memory

block starting at physical address C0000000h. It can

therefore only be accessed in the protected or virtual

modes of the 386 CPU. DOS programs can access the

EMC87 with the help of DOS-extenders or	memory

managers like EMM386 which run in protected/virtual

mode themself. Since the EMC87 provides	also the

standard CPU interface via IO-ports, it	can be used

just like any other 387	compatible coprocessor and

delivers the same performance as the Cyrix 83D87 in

this mode. However, using the memory mapped mode of

the EMC87 provides a significant speed advantage.

The traditional	387 CPU-coprocessor interface via

IO-ports has an	overhead of about 14-20	clock cycles.

Since the Cyrix	83D87 executes some operations like

addition and multiplication in much less time, its

performance is limited by the CPU-coprocessor

interface. The memory-mapped mode has much less

overhead and allows all	coprocessor instructions to

be executed at full speed and with no penalty. For

this reason, Cyrix introduced the EMC87	in 1990.

In a test, the EMC87 at	33 MHz ran the single

precision Whetstone benchmark at 7608 kWhetstones/sec,

while the Cyrix	83D87 at 33 MHz	had a speed of

only 5049 kWhetstones/sec, an increase of 50.6%	[63].

In another test, the EMC87 ran a fractal computation

at two times the speed of the Cyrix 83D87 and 2.6

times as fast as an Intel 387DX	[64]. A	third test

found the EMC87's overall performance to be 20%

higher than the	performance of the Cyrix 83D87

[65]. The Cyrix	FasMath	EMC87 has also been sold

as Cyrix AutoMATH by Cyrix. The	two chips are 100%

identical. Unlike the Cyrix 83D87, which fits into

the 68-pin 387 coprocessor socket, the EMC87 comes

in a 121-pin PGA and requires the 121-pin EMC

(Extended Math Coprocessor) socket. Note that not

all boards have	such a socket, a notable exception

being IBM's PS/2s, for example. Originally, Cyrix

claimed	support	for the	fast memory mapped mode	of

the EMC87 from a lot of	software vendors (including

Borland	and Microsoft).	However, there are only

very few applications that make	use of it, among

them Evolution Computing's FastCAD 3D, MicroWay

Inc.'s NDP FORTRAN-386 compiler, Metaware's High-C

compiler version 1.6 and newer,	and Intusofts's

Spice [63,73]. Cyrix has implemented some additional

instructions in	the EMC87: FRICHOP, FRINT2, and

FRINEAR. These instructions enable rounding to

integer	without	setting	the rounding mode by

manipulating the coprocessor control word and are

intended to make life easier for compiler writers.

The EMC87 is available 25 and 33 MHz versions. Cyrix

is currently phasing out the EMC87.

Cyrix 387+	is the successor to the	Cyrix 83D87. The name 387+

is only	used for European distribution.	In other

parts of the world, the	second generation 387 'clone'

from Cyrix still goes by the name 83D87. In my tests,

I found	the Cyrix 387+ to be about five	to 10 percent

*slower* than the Cyrix	83D87. However,	some

instructions like the square root (FSQRT) now

run at only half the speed at which they ran in

the 83D87 and the transcendental functions show

a 40% drop in performance comapred with	the 83D87

on the average (see performance	results	below).	I

also found the transcendental functions	on the 387+

to be a	bit *more* accurate than those implemented

in the 83D87. According	to a source with Cyrix [73],

the 387+ was designed to make a	smaller	and thus

cheaper	coprocessor chip, that also can	go at

higher frequencies than	the 83D87. The new design

uses a slower hardware multiplier that needs 6

clock cycles to	multiply the floating point mantissa

of an internal precision number, while the multiplier

in the 83D87 takes only	4 clocks to accomplish the

same task. Since the transcendental functions are

generated by polynomial	and rational approximations

in Cyrix math coprocessors, this slows them down

quite a	bit. The divide/square root logic has also

been changed from the 83D87 design. The	original design

used an	algorithm that could generate both, the

quotient and square root, so the excution times	for

these instructions were	nearly identical. The algorithm

choosen	for the	division in the	387+ doesn't allow

the square root	to be taken so easily, so it takes

nearly twice as	long. The 387+ is available in

versions of up to 40 MHz. In the 387+, the available

argument range for the FYL2XP1 instruction has been

extended from the usual	range -1+sqrt(2)/2..sqrt(2)/2,

that is	found on all 80x87 coprocessors, to all

floating-point numbers.	Also, four additional

instructions have been implemented: FRICHOP (opcode

DD FC),	FRINT2 (opcode DB FC), FRINEAR (opcode DF FC),

and FTSTP (opcode D9 E6)..

Cyrix 83S87	is the SX version of the Cyrix 83D87. Just like	the

Cyrix 83D87 is the fastest 387 compatible coprocessor,

the Cyrix 83S87	is the fastest of the 387SX compatible

coprocessor [1]. Besides being the fastest 387SX

'clone', the Cyrix 83S87 also features the most

accurate transcendental	functions. 83S87 chips sold

after about August 1992	use the	internals of the

Cyrix 387+, the	successor to the original 83D87	[73].

The 83S87 is packaged in a 68-pin PLCC and is available

in 16, 20 and 25 MHz versions. Due to the advanced

power saving features of the Cyrix coprocessor,	the

typical	power consumption of the 20 MHz	version	is

about 350 mW [67].

ULSI 83C87	is a 387 'clone' that came out in early	1991, well

after the IIT 3C87 and Cyrix 83D87. Like all clones,

it is somewhat faster than the Intel 387DX. Especially

the basic arithmetic functions are fast, while the

transcendental functions show only a slight speed

improvement over the Intel 387DX (see benchmark

results	below).	In my tests, the ULSI had the most

inaccurate transcendental functions. However, the

maximum	relative error is still	within the limits

set by Intel, so this is probably not an important

issue in all but very few applications.	The ULSI

83C87 shows some minor flaws in	the tests for IEEE

754 compatiblity, but this, too, is unimportant

under typical operating	conditions. It is interesting

to note	that an	ULSI 83S87 manufactured	in 92/17

showed less errors in the IEEETEST test	run [74] than

the ULSI 83C87,	manufactured in	91/48, I used in

my original test. This indicates that ULSI might

have applied some quick	fixes to newer revisions

of their math coprocessors. ULSI claims	that the

program	IEEETEST, which	was used to test for IEEE

compatibility, contains	many personal interpretations

of the IEEE standard by	the program's author and

states that there is no	ANSI-certified IEEE-754

complicency test. While	this is	may be true, it

is also	a fact that the	IEEE test vectors used in

IEEETEST are sort of an	industry standard and that

Intel's 387, 486, and RapidCAD chips pass it

without	a single failure, as do	the coprocessors from

Cyrix. Since the ULSI Math*Co 83C87 fails some of

the tests, it is certainly less	than 100% compatible

with Intel's chips, although this will hardly make

any difference in typical operating conditions.

The ULSI 83C87 fails to	be compatible with the

IEEE-754 in that is does not implement the precision

control	feature. While all the internal	operations of

80x87 coprocessors are usually done with the maximum

precision available (double extended presision with

64 mantissa bits), the 80x87 also offer	the possiblity

to force lower precision to be used for	the basic

arithmetic functions add, subtract, multiply, divide,

and square root. This feature is required by IEEE-754

for all	coprocessor that can not store results

*directly* to a	single or double precision location.

Since the 80x87	coprocessors lack this capability,

the have to implement this capability to provide

correctly rounded single and double precision results

according to the floating-point	standard. All 80x87

coprocessors except the	ones from ULSI support this

feature. For programs that make	use of precision

control, e.g. Interactive UNIX,	correct	implementation

of the feature may be essential	for correct arithmetic

results. Like the other	387 'clones', the 83C87	does

not support asynchronous operation of the CPU and the

coprocessor. This means	that the 83C87 always runs at

the full speed of the CPU. The ULSI 83C87 is available

in 20, 25, 33, and 40 MHz versions. The	ULSI is

produced in high perfromance, low power	CMOS. Power

consumption at 20 MHz is max. 800 mW (400 mW typical),

at 25 MHz it is	max. 1000 mW (500 mW typical), at 33

MHz it is max. 1250 mW (625 mW), and at	40 MHz the

ULSI Math*Co 83C87 consumes max. 1500 mW (750 mW

typical) [58]. The 83C87 is packaged in	a 68-pin

ceramic	PGA. ULSI coprocessors come with a lifetime

warranty. ULSI Systems,	Inc. will replace the

coprocessor up to three	times free of charge should

it ever	fail.

ULSI 83S87	is the SX version of the ULSI 83C87 for	operation

with an	Intel 387SX or an AMD Am387SX. It is

functionally equivalent	to the 83C87. To aid low

power laptop designs, the ULSI 83S87 features an

advanced power saving design with a sleep mode and

a standby mode with only minimal power requirements.

Power consumption under	normal operating conditions

(dynamic mode) is max. 400 mW at 16 MHz	(300 mW

typical), max. 450 mW at 20 MHz	(350 mW	typical),

and max. 500 mW	at 25 MHz (400 mW typical) [58].

The ULSI 83S87 is packaged in a	68-pin PLCC.

C&T 38700DX	is the latest entry into the 387 'clone' market.

Originally announced in	October, 1991, it has

apparently not been available to end users before

third quarter of 1992, at least	here in	Germany.

The C&T	38700DX	is compatible with the Intel 387DX.

My tests show that compatibility is indeed very	good,

even for the more arcane features of the 387DX and

comparable to the coprocessors from Cyrix. Like

the coprocessors from Cyrix and	Intel, it passes

the IEEETEST program without a single failure. It

passes,	of course, all tests in	Chips &	Technologies

own compatibility test program SMDIAG. However,	some

of the tests (transcendental functions)	in this	program

are selected in	such a way that	the C&T	38700 passes

while the Cyrix	83D87 or Intel RapidCAD	fail, so they

are not	very useful. There is also a 'bug' in the test

for FSCALE that	hides a	true bug in the	C&T 38700. In

my own speed tests [see	below] and those reported in

[1], the C&T 38700DX showed performance	at about 90-

100% the level of the Cyrix 83D87, which is the	387

'clone'	with the highest performance. For floating

point intensive	benchmarks the C&T 38700DX provides up

to 50% more computational power	than the Intel 387DX.

However, as with all other 387 compatible coprocessors,

the speed advantage over the Intel 387DX is hardly

measurable in real application.	The accuracy of	the

transcendental functions on the	C&T 38700DX varies.

Overall	accuracy of the	transcendental function	is

slightly better	than on	the Intel 387DX. The SuperMATH

38700DX	is implemented in 1.2 micron CMOS with on-chip

power managment, which makes for low power consumption.

The 38700DX is packaged	in a 68-pin ceramic PGA	(pin

grid array and available in speeds of 16, 20, 25, 33,

and 40 MHz.

C&T 38700SX	is the SX version of the 38700DX and compatible	to

the Intel 387SX. It provides performance similar to

the Cyrix 83S87	[1], the 387SX 'clone' with the

highest	performance. Compatibility with	the Intel

387SX is very good and comparable with high degree

of the compatibility found in the Cyrix	83S87. It

has low	power consumption. The SuperMATH 38700SX is

packaged in a 68-pin PLCC (plastic leaded chip carrier)

and available in speeds	of 16, 20, and 25 MHz.

Intel RapidCAD	is not a coprocessor, strictly seen, although it

is marketed as one. Rather, it is a CPU	replacement.

It is basically	an Intel 486DX without the cache and

with a 386 pinout. RapidCAD is delivered as a set of

two chips. RapidCAD-1 goes into	the 386	socket and

contains the CPU and FPU, RapidCAD-2 goes into the

coprocessor socket and contains	a PAL that generates

the Ferr signal	that is	normally generated by a

coprocessor and	used by	the motherboard	circuitry to

provide	287 compatible coprocessor exception handling

in 386/387 systems. The	RapidCAD instruction set is

compatible with	the 386, so it doesn't know the 486

specific instructions like BSWAP. Since	the RapidCAD

CPU core is very similar to 486	CPU core, most of the

register to register instructions execute in the same

number of clock	cycles as on the 486. The use of the

386 bus	interface causes instructions that access memory

to execute at about the	same speed as on the 386. The

integer	performance on the RapidCAD is definitely

limited	by the low memory bandwidth provided by	the

386 bus	interface (2 clock cylces per bus cycle)

and the	lack of	an internal cache. CPU instructions

often execute faster than they can be fetched from

memory,	even with a big	and fast external cache.

Therefore, the integer performance of the RapidCAD

exceeds	that of	a 386 by *at most* 35%.	This value

was derived by running some programs that use

mostly register-to-register operations and few

memory accesses. This finding is supported by the

SPEC ratings that Intel	reports	for the	386-33

and the	RapidCAD-33. While the 386-33 has a

SPECint	of 6.4,	the RapidCAD has a SPECint of 7.3

[28], a	14% increase. Note that	these tests used

the old	(1989) SPEC benchmarks suite. While CPU

instructions often execute in one clock	cycle on

the RapidCAD, FPU instructions always take more

than seven clock cycles. They are therefore rarely

slowed down by the low memory bandwidth	provided

by the 386 bus interface. My tests show	a 70%-100%

performance increase for floating-point	intensive

benchmarks (see	below) over a 386 based	system

using the Intel	387DX math coprocessor.	This is

consistent with	the SPECfp rating reported by Intel.

The 386/387 at 33 MHz is rated at 3.3 SPECfp, while

the RapidCAD is	rated at 6.1 SPECfp at the same

frequency, a 85% increase. This	means that a system

that uses the RapidCAD is faster than any 386/387

combination, regardless	of the type of 387 used

(Intel 387DX or	faster clone). The diagnostic disk

for the	RapidCAD also gives some application

performance data for the RapidCAD compared to the

Intel 387DX:

Application    Time w/ 387DX  Time w/ RapidCAD	Speedup

AUTOCAD	11	      52 sec	     32	sec	  63%

AutoShade/Renderman  180 sec	    108	sec	  67%

Mathematica(Windows) 139 sec	    103	sec	  35%

SPSS/PC+ 4.01	      17 sec	     14	sec	  21%

RapidCAD is available in 25 MHz	and 33 MHz versions.

It is distributed through other	channels than the

other Intel math coprocessors. Therefore, I have been

unable to obtain a data	sheet for it. The RapidCad-1

chip gets quite	hot when operating and it can be

assumed	that its power consumption is similar to

the 486-33. Therefore, I recommend extra cooling

for this chip (see the paragraph below on the 486 for

details). The RapidCAD-1 is packaged in	a 132-pin

PGA, just like the 80386, and the RapidCAD-2 is

packaged in a 68-pin PGA like a	80387 coprocessor.

Intel 486DX	is not a coprocessor. This chip, brought out in

1989 functionally combines the CPU (a heavily pipelined

implementation of the 386 architecture)	with an

enhanced 387 (the floating-point unit, FPU) and

8 kB of	unified	code/data cache	on one chip. Of

course,	this description is simplified,	for a

detailed hardware description, see [52]. The

486DX offers about two to three	times the integer

performance of a 386 at	the same frequency.

Floating point performance is about three to four

times as high as on the	Intel 387DX at the same

clock rate [29]. Since the FPU is on the same

chip as	the CPU, the considerable communication

overhead between CPU and coprocessor in	a 386/387

system is omitted, letting FPU instructions run

at the full speed permitted by the implementation.

The FPU	also takes advantage of	the on-chip cache

and the	highly pipelined execution unit. Besides

the higher speed, the 486 FPU features more accurate

transcendental functions than the Intel	387DX

coprocessor according to tests run by me (see below).

To achieve better interrupt latency, FPU instructions

with a long execution time have	been made abortable

in the case an interrupt occurs	during their

execution. The concurrent execution of CPU and

coprocessor instructions typical for 80x86/80x87

systems	is still in existence on the 486, but

some FPU instructions like FSIN	have nearly no

concurrency with CPU instructions, indicating

that they make heavy use of both, CPU and FPU

resources [53, 1]. The 486DX comes in a	168 pin

ceramic	PGA (pin grid array). It is available in

25 MHz and 33 Mhz versions. Since the end of 1991,

there is also a	50 MHz version available done in

a CHMOS	V process (the 25 MHz and 33 MHz are

produced using the CHMOS IV process). Maximum

power consumption is 3500 mW for the 25	MHz 486

(2600 mW typical), 4500	mW for the 33 MHz version

(3500 mW typical), and 5000 mW (4000 mW	typical)

for the	50 MHz chip. Due to the	considerable amount

of heat	produced by these chips, and taking into

consideration the slow air flow	provided by the

fan in garden variety PC tower cases, I	recommend

an extra fan directly above the	CPU for	safer

operation. If you measure the surface temperature

of an i486 in a	normal tower case without extra

cooling	after some time	of operation, you may well

come up	with something like 80 - 90 degrees Celsius

(that is 176 - 194 degrees Fahrenheit for those	not

familiar with metric units) [54,55]. You don't need

the well known and expensive IceCap(tm)	to effectively

cool your CPU. A simple	fan mounted directly above

the CPU	can bring the temperature down to about	50

to 60 degrees Celsius (122 - 140 degrees Fahrenheit)

depending on the room temperature and the temperature

within the PC case (which depends on the total power

dissipation of all the components and the cooling

provided by the	fan in the power unit).	According

to a simple rule known as Arrehnius' Law, lowering

the temperature	by 10 degrees Celsius slows down

chemical reactions by a	factor of two, thus lowering

the temperature	of your	CPU by 30 degrees should

prolong	the live of the	device by a factor of eight

due to the slower aging	process. If you	are reluctant

to add a fan to	your system because of the additional

noise, settle for a low-noise fan like those

available from the German manufacturer Pabst (this

is not meant to	be an advertisement. I am just the

happy owner of such a fan. Besides that, I have	no

connections to the firm).

Intel 486DX2	is the name for	Intel latest generation	of 486 CPUs.

Using the DX2 suffix instead of	simply DX is meant

to be an indicator that	these are clock-doubled

versions. A normal 486DX operates at the frequency

provided by the	incoming clock signal. A 486DX2

generates a new	clock signal from the incoming clock

by means of a PLL (phase locked	loop). In the DX2,

this clock signal has twice the	frequency of the

incoming clock,	hence the name clock-doubler. All

internal parts of the 486DX2 (cache, CPU core, FPU)

run at this higher frequency. Only the bus interface

runs at	the normal speed. That way, a 486DX-50 can

run on a motherboard designed for 25 MHz operation.

Since motherboards for 50 MHz operations are much

harder to design than those for	25 Mhz,	this makes

a 486DX2-50 system easier to built and cheaper than

a 486DX-50 system. For all operations that don't

access off-chip	resources (e.g.	register operations)

a 486DX2-50 provides exactly the same performance as

a 486DX-50 and twice the performance of	a 486DX-25.

However, since the main	memory in a 486DX2-50 systems

still operates at 25 MHz, all instructions involving

memory accesses	are potentially	slower than in a

486DX-50 system, whose memory also runs	at 50 Mhz.

The internal cache of the 486 helps this problem a

bit, but overall performance of	a 486DX2-50 is still

lower than that	of a 486DX-50, although	Intel's

documentation [32] shows this drop to be quite small.

It depends a lot on the	code one runs, though. The

nice thing about the 486DX2 is that it allows easy

upgrading of 25	and 33 Mhz 486 systems,	since the

486DX2 is completely pin-compatible with the 486DX.

Just take out the 486DX	and plug in the	new 486DX2.

Note that power	consumption of the 486DX2-50 equals

that of	the 486DX-50 (4000 mW typical),	and that the

486DX2-66 exceeds this by about	30%. These chips get

really hot in a	standard PC case with no extra cooling.

See the	above paragraph	for more detailed information

on this	problem.

Intel 487SX	is the coprocessor intended for	use in 486SX systems.

The 486SX is basically a 486DX without the floating-

point unit (FPU) [48, 50]. Originally Intel sold

486DXs with a defective	FPU as 486SXs but it has

now completly removed the FPU part from	the 486SX

mask for mass production. The introduction of the

486SX in 1991 has been viewed mainly as	a marketing

'trick'	by Intel to take market	share from the 386

based systems once AMD became successful with their

Am386 (AMD has taken as	much as	40% of the 386 market

due to some superior features such as higher clock

frequency, lower power consumption, and	a fully

static design).	A 486SX	at 20 MHz delivers a bit

less integer performance than a	40 MHz Am386. To add

floating-point capabilities to a 486SX based system,

it would be easiest to swap the	486SX with a 486DX

which includes the FPU.	However, Intel has prevented

this easy solution by giving the 486SX a slightly

different pin out [48, 51]. Since only three pins

are assigned differently, clever board manufacturers

have come out with boards that accept anything from

a 486SX-20 to a	486DX2-50 in their CPU socket and

provide	a clean	upgrade	path this way. A set of

three jumpers ensures correct signal assignment	to

the pins for either configuration. To upgrade systems

without	this feature, one has to buy the 487SX and

put it into the	"Performance Upgrade Socket" present

in most	486SX systems. Once the	487SX was available,

it was quickly found out that it is just a normal

486DX with a slightly different	pin out	[49]. Inserting

the 487SX effectively shuts down the 486SX in the

486SX/487SX system, so the 486SX could be removed

once the 487SX is installed. Since the shut down is

logical, not electrical, the 486SX still uses power

if used	with the 487SX,	although it is unoperational.

Technically speaking, the solution Intel chose was

the only practical way to provide a 486SX system with

the high level of floating-point performance the

486DX offers. The CPU and FPU have to be on the	same

chip, otherwise	the FPU	can not	make use of the	cache

on the CPU chip	and there would	be considerable

overhead in CPU-FPU communication (similar to a

386/387	system), nullifying most of the	arithmetic

speedups over the 387. That the	486SX, 487SX, and

486DX are not pin-compatible seems to be purely	for

marketing reasons. To upgrade a	486SX based system,

Intel also offers the OverDrive	chip, which is just

the same as a 487SX with internal clock	doubling. It

goes also goes into the	"Performance Upgrade Socket"

found in 486SX systems.	The OverDrive roughly doubles

the performance	of a 486SX/487SX based system. For a

explanation of clock doubling, see the description

of the 486DX2 above. As	the 486SX, the 487SX is

available in 20	MHz and	25 MHz versions. At 20 MHz,

the 487SX has a	power consumption of max. 4000 mW.

It is available	in a 169 pin ceramic PGA (pin grid

array).

Weitek	1167	was the	predecessor to the Weitek Abacus 3167 math

coprocessor. It	was actually a small printed circuit

board with three chips mounted on it. As opposed to

the Weitek 3167, the 1167 did not have a square	root

instruction, instead the square	root function was

computed by means of a subroutine in the Weitek

transcendental function	library. However, the 1167

did have a mode	in which it supported denormals.

The Weitek 3167	and 4167 only implement	the 'fast'

mode, in which denormals are not supported. Overall

performance of the 1167	is slightly less than that

of the Weitek 3167.

Weitek	3167	was introduced in 1989 to provide the fastest

floating point performance possible on a 386 based

system at that time. The Weitek	Abacus 3167 is not

a real coprocessor, strictly speaking, but rather

a memory mapped	peripheral device. The Weitek 3167

was optimized for speed	wherever possible. Besides

using the faster memory	mapped interface to the	CPU

(the 80x87 uses	IO-ports), it does not support many

of the features	of the 80x87 coprocessors, allowing

all of the chip's ressources to be concentrated on

the fast execution of the basic	arithmetic operations.

For a more detailed description	of the Weitek 3167 see

the first chapter of this document. In benchmark

comparisons, the Weitek	3167 provided up to 2.5	times

the performance	of an Intel 387DX coprocessor. For

example, on a 33 MHz 3167 the Whetstone	benchmark

performed at 7574 kWhetstones/sec compared with	the

the 3743 kWhetstones/s for the Intel 387DX. Note

however	that these are single precision	results	and

that the Weitek	3167's performance would drop to

about half the stated rate for double precision,

while the value	for the	Intel 387DX would not change

much. Anyhow, before the advent	of the Intel RapidCAD,

the Weitek 3167	usually	beat all 387 compatible

coprocessors even for double precision operations

[63,65,69]. For	typical	applications the advantage

of the Weitek 3167 over	the 387	clones is much smaller.

In a benchmark test using AutoDesk's 3D-Studio the

Weitek 3167 performed at 123% of the Intel 487DX's

perfromance comapred with 106% for the Cyrix FasMath

83D87 and 118% for the Intel RapidCAD. The Weitek

Abacus 3167 is packaged	in a 121-pin PGA that fits

into an	EMC socket provided by most 386	based systems.

It does	*not* fit into the normal coprocessor socket

designed to hold a 387 compatible coprocessor in a

68-pin PGA. To get the best of both worlds, one	might

want to	use a Weitek 3167 and a	387 compatible

coprocessor in the same	system.	These coprocessors

can coexist in the same	system just fine. Only problem

is that	most 386 based systems contain only one

coprocessor socket, usually of the EMC (extended math

coprocessor) type. Thus, you can install either	a

387 coprocessor	or a Weitek 3167, but not both.	There

are little daughter boards available though that fit

into the EMC socket and	provide	two sockets, an	EMC

and a standard coprocessor socket. At 25 MHz, the

Weitek 3167 has	a power	consumption of max. 1750 mW.

At 33 MHz, the max. power consumption is 2250 mW.

Weitek	4167	is a memory mapped coprocessor that has	the same

architecture as	the 3167 and is	designed to provide

486 based systems with the highest floating point

performance available. It executes coprocessor

instructions at	three to four times the	speed of

the Weitek 3167. Although it is	up to 80% faster

than the Intel 468 in some benchmarks [1,69], the

performance advantage for real application is more

like 10%. The introduction of the 486DX2 processors

has more or less obliterated the need for a Weitek

4167, since the	DX2 CPUs provide the same performance

and all	the additional features	the 80x87 has over

the Weitek Abacus. The Weitek 4167 is packaged in

a 142-pin PGA package that is only slightly smaller

than the 486's package. At 25 MHz, it has a max.

power consumption of 2500 mW [32].

Pricing

Due to	a recent price slashing	by Cyrix and subsequently by Intel

for 387 coprocessors, prices have dropped significantly for all

287 and 387 compatible	coprocessors with hardly any price difference

between manufacturers.	387DX compatible coprocessors typically	sell

for ~US$ 100 for all speeds except for	40 MHz versions	which are

typically ~US$	130. 387SX compatible coprocessors sell	for ~US$ 90

regardless of speed with the exception	of the 33 MHz versions,	which

are ~US$ 100. The Intel 287XL sells for ~US$ 100, while the IIT 2C87

and Cyrix 82S87 sell for about	US$ 70.	8087s may be more expensive,

the price of an 8087-10 being US$ 150.	I bought the Intel RapidCAD

for US$ 320 and haven't seen it offered for a better price. I see

the Weitek Abacus 3167-33 being offered for US$ 780 and the 4167-33

being offered for US$ 1100. This price	information reflects the

price situation as of 08-14-92. Prices	can be expected	to drop

slightly in the near future.

If you	have a demand for high floating-point performance, you

should	consider to buy	a 486 based system rather than buying

a 386 based system with an additional coprocessor. A 386 mother

board for 33 MHz operation sells for ~	US$ 300, together with the

coprocessor, cost totals ~ US$	400. A 486-33 ISA-board	sells for

US$ 650. While	the 486-33 system is 60% more expensive	than the

386/387 system, it also provides 100% more integer and	floating-

point performance (twice the performance). If you want	to push

your 386 based	system to maximum floating-point performance and

can't switch to a 486 based system for some reason, I recommend

the Intel RapidCAD. It	is both	faster [1] and cheaper than installing

a Weitek Abacus 3167 with your	386, which used	to be the highest

performing combination	before the RapidCAD came out. Similarily,

the introduction of the 486DX2	clock-doubler chips have obliterated

the need for a	Weitek 4167 to get maximum floating-point performance

out of	a 486 based system. A 486DX2-66	performs at or above the

performance level of a	33 Mhz Weitek 4167, even if the	latter

uses single precision rather than double precision. The 486DX-66

is rated by Intel at 24700 double precision kWhetstones/sec and

3.1 double precision Linpack MFLOPS. Of course, these benchmarks

used the highest performance compilers	available. But even with

a Turbo Pascal	6.0 program, I managed to squeeze 1.6 double precision

MFLOPS	out of the 486DX2-66 for the LLL benchmark (for	a description

of the	benchmarks mentioned, see the paragraph	on benchmarks below).

Although I haven't yet seen 486DX2-66 processors being offered

to end	users for upgrade purposes, I recommend	the 486DX2-66

to those that need highest floating-point performance and are

planning to buy a new PC. The price difference	between	a 33 MHz

486DX motherboard and a 486DX2-66 motherboard is around US$ 600,

well below the	price for the Weitek Abacus 4167.

Operation

In a 80x86/80x87 system CPU instructions and coprocessor

instructions are executed concurrently. This means that

the CPU can execute CPU instructions while the	coprocessor

executes a coprocessor	instruction at the same	time. The

concurrency is	restricted somewhat by the fact	that the

CPU has to aid	the coprocessor	in certain operations. As

the CPU and the coprocessor are fed from the same instruction

stream	and both instruction streams may operate on the	same

data, there has to be a synchronizing mechanism between the

CPU and the coprocessor.

8086/8087 or 8088/8087	system,	both of	the chips look at the

opcodes coming	in from	the bus. To do this, both chips	have

the same BIU (bus interface unit) and the 8086	BIU sends the

status	signals	of its prefetch	queue to the 8087 BIU. This

assures that both processors always decode the	same instructions

in parallel. Since all	coprocessor instruction	start with the

bit pattern 11011, it is easy for the 8087 to ignore all other

instructions. Likewise	the CPU	ignores	all coprocessor	instructions

except	if they	access memory. In this case, the CPU computes

the address of	the LSB	(least significant byte) of the	memory

operand and does a dummy read.	The 8087 then takes the	data											and does a dummy read.	from the data bus. If more than	one meory access is needed to load an memory op

erand, the 8087	requests the bus from the CPU,

generates the consecutive addresses of	the operand's bytes

and fetches them from the data	bus. After completing the operation,

the 8087 hands	bus control back to the	CPU. Since 8087	and CPU

are hooked up to the same synchronous bus, they have to run at

the same speed. This means that with the 8087,	only synchronous

operation of CPU and coprocessor is possible. Another 8087

coprocessor instruction can only be started if	the previous one

has been completed in the NEU (numerical execution unit) of the

8087. To prevent the 8086 from	decoding a new coprocessor

instruction while the 8087 is still excuting the previous

coprocessor instruction, the following	mechanism is used: The

compilers and assemblers automatically	generate a WAIT	instruction

before	each coprocessor instruction. The WAIT instruction tests

the /TEST pin until its input becomes "LOW". In 8086/8087 systems,

the 8086 /TEST	pin is connected to the	8087 BUSY pin. As long

as the	NEU executes a coprocessor instruction,	it forces its

BUSY pin "HIGH". Thus the WAIT	instruction in front of	every

coprocessor instruction stops the CPU until a still executing

previous coprocessor instruction has finished.	The same

synchronization is used before	the CPU	accesses data that

was written by	the coprocessor. A WAIT	instruction after the

coprocessor instruction that writes to	memory causes the CPU to

stop until the	coprocessor has	transferred the	data to	memory,

after which the CPU can safely	access the data.

With the help of an additional	chip, the 8087 can also	be inter-

faced to the 80186 [36]. The 80186 was	the CPU	in some	PCs (e.g.

from Philips, Siemens)	in the 1982/1983 time frame, but with

the introduction of the IBM AT	which used the 80286, it lost all

significance for the PC market. The 80C186 (CMOS version of the

80186)	nowadays sells as an embedded controller and can be combined

with a	80C187 coprocessor which is based on the internals of the

Intel 387 [37].

The 80287 CPU-interface is totally different from the solution

used in the 8087. Since the 80286 implements memory protection

via an	MMU based on segmentation, it would have been much to

expensive to duplicate	the whole protection logic on the coprocessor

for an	interface solution similar to the 8087.	In a 80286/80287

system, the CPU fetches and stores all	opcodes	and operands for

the coprocessor. Information is passed	through	ports F8h - FFh.

As these ports	are accessible under program control, care must

be taken to not accidentally perform write operation to them, as

this could corrupt the	information in the math	coprocessor.

The execution unit of the 80287 is practically	identical to that

of the	8087, that is, nearly all coprocessor instructions execute

in the	same number of clock cycles on both coprocessors. Due to

the additional	overhead of the	CPU/coprocessor	interface (at

least ~40 clock cycles), a 8 MHz 80286/80287 combination can be

slower	than a 8086/8087 system	running	at the same speed for

floating point	intensive programs. Additionally, most of the

older 286 boards were configured to run the coprocessor at 2/3

the speed of the CPU, making use of the ability of the	80287

to run	asynchronous with the CPU. The 80287 has a CKM pin that

causes	the incoming system clock to be	divided	by three for

the coprocessor if it is tied to ground. The 80286 always

divides the system clock by two internally. Thus the ratio 2/3.

However, when the CKM (ClocK Mode) pin	is tied	high on	the 80287,

it does not divide the	CLK input. This	feature	has been exploited

by the	maker of coprocessor speed sockets. These sockets tie

CKM high and supply their own CLK signal with a built-in oscillator,

thereby allowing the 80287 or compatible to run at a much higher

speed than the	CPU. With an IIT or Cyrix 287 one can have a

20 MHz	coprocessor running with a 8 MHz 80286.	Note however that

the floating-point performance	in such	a configuration	does not

scale linearly	with the coprocessor clock, since all the data

has to	be passed through the much slower CPU. If the coprocessor

executes mostly simple	intructions such as addition and multiplication

doubling the coprocessor clock	in a 10	MHz system to 20 MHz does

not show any performance increase at all [24].	The 80C287 by AMD

is a 100% clone of the	original Intel 80287, but is produced in

CMOS not in NMOS as the original Intel	chip. This makes for lower

power consumption.

The 80287XL, the Cyrix	82S87, and the IIT 2C87	contain	the internals

of a 387 coprocessor, but are pin-compatible to the original 287.

However, these	chips divide the system	clock by two internally,

as opposed to three in	the original Intel 80287. Since	the 80286

also divides the system clock by two, they usually run	synchronously

with the CPU. They can	also run asynchronously, though.

The 8087/8087 combination can be characterized	as a cooperation of

partners with equal rights, while the 80286/287 is more a master-

slave relationship. This makes	synchronization	much more easy,	since

the complete instruction and data flow	of the coprocessor goes	thru

the CPU. Before executing most	coprocessor instructions, the 80286

tests its /BUSY pin which is hooked up	to the 287 coprocessor and

signals if the	80287 is still executing a previous coprocessor

instruction or	has encountered	an exception. The 80286	then waits

until the 80287 is not	busy before loading the	coprocessor instruction

into the coprocessor. Therefore, a WAIT instruction before every

coprocessor instruction is not	required. These	WAITs are permissible,

but not necessary in 80287 programs. The second form of WAIT

synchronisation after the coprocessor has written a memory operand is

still necessary on 286/287 systems.

The coprocessor interface in 80386/80387 systems is very similar to

the one found in 286/287 systems. However, to prevent corruption

of the	coprocessor's contents by programming errors, the IO-ports

800000F8 - 800000FF are used which are	not user accessible. The

interface has been optimized and uses 32-bit transfers. The overhead

of the	interface has been reduced to about 16-20 clock	cycles.	For

some operations on the	387 'clones', that take	less than 16 clock

cycles	to complete this effectively limits the	execution rate of

coprocessor instructions. The only sensible solution to provide

even higher floating point performance	was to integrate the CPU

and coprocessor functionality onto the	same chip. This	is what

Intel did with	the 80486. The FPU in the 486 also benefits from

the instruction pipelining and	from the integrated cache.

Performance

Several computer magazines have published performance comparisons

at the	application level for the 387 coprocessors and Weitek's

ABACUS	3167 and 4167 chips [1,25,68,70]. Applications tested included

AutoCAD R11, RenderStar, Quattro Pro, Lotus 1-2-3, and	AutoDesk's

3D-Studio. For	most tests, performance	improvements for the 387

clones	over Intel's 387DX were small to marginal, the clones running

the applications no more than 5% to 15% faster	than the Intel 387DX.

In the	test of	3D-Studio, one of the few programs that	supports

the Weitek Abacus, the	Weitek 3167 improved performance by 23%

over an Intel 387DX and the 4167 improved performance by 10% over

the 486 [1].

The Intel Math	Coprocessor Utilities Disk that	accompanies the

Intel 387DX coprocessor has a demonstration program that shows

the speedup of	certain	application programs when run with the

Intel coprocessor vs. a system	with no	coprocessor.

Application	  Time w/o 387	 Time w/ 387  Speedup

Art&Letters	      87.0 sec	    34.8 sec	 150%

Quattro Pro	       8.0 sec	     4.0 sec	 100%

Wingz		      17.9 sec	     9.1 sec	  97%

Mathematica	     420.2 sec	   337.0 sec	  25%

The following table is	an excerpt from	[70]:

Application	  Time w/o 387	 Time w/ 387  Speedup

Corel Draw	     471.0 sec	   416.0 sec	  13%

Freedom Of Press    163.0 sec	    77.0 sec	 112%

Lotus 1-2-3	     257.0 sec	    43.0 sec	 597%

The following table is	an excerpt from	[25]:

Application	  Time w/o 387	 Time w/ 387  Speedup

Design	CAD, Test1    98.1 sec	    50.0 sec	  96%

Design	CAD, Test2    75.3 sec	    35.0 sec	 115%

Excel,	Test 1	       9.2 sec	     6.8 sec	  35%

Excel,	Test 1	      12.6 sec	     9.3 sec	  35%

The performance statistics below were put together with the

help of four widely known numeric benchmarks and two benchmarks

developed by me. Three	Pascal programs, one FORTRAN program,

and two assembly language program were	used. The assembly language

programs were linked with Turbo-Pascal	6.0 for	library	support,

especially to include the coprocessor emulator	of the TP 6.0

run-time library. The Pascal programs were compiled with Turbo

Pascal	6.0 from Borland International,	a non-optimizing compiler

that produces 16-bit code. The	FORTRAN	program	was compiled using

MS FORTRAN 5.0, an optimizing compiler	that generates 16-bit

code. All programs except PEAKFLOP and	SAVAGE,	which use double

extended precision, use double	precision variables. Note that

using a highly	optimizing compiler producing 32-bit code you

will see much higher performance for some benchmarks. For example,

Intel rates the 33 MHz	386/387DX at 3290 KWhetstones/sec and 0.4

double	precision LINPACK MFLOPS [28,29]. The 33 MHz Intel 486 is

rated by Intel	at 12300 KWhetstones/sec and 1.6 double	precision

LINPACK MFLOPS	[30]. The compilers used in these benchmarks run by

the chip vendor are the ones that give	the highest performance

available. These compilers are	in the US$ 1000+ price range.

Some of them may be experimental or prereleased versions not

available to the general public. The relative performance of

one coprocessor to another could vary depending on the	code

generated by compilers. Non-optimizing	compilers tend to generate

a high	percentage of operations which access variables	in memory,

while optimizing compiler produce code	that contains many

operations involving registers. Thus it is well possible that

coprocessor A beats coprocessor B running benchmark Z if compiled

with compiler C, but B	beats A	when the same benchmark	is compiled

using compiler	D. All benchmark in this overview were run from

floppy	under a	'bare-bones' MS-DOS 5.0	without	the CONFIG.SYS

and AUTOEXEC.BAT files. This way, it was made sure no TSR or

other program unnecessarily stole computing resources from the

benchmarks.

Coprocessor performance also depends on the motherboard, or more

specifically the chip set used	on the motherboard. In [34] and	[35]

identically configured	motherboards using different 386 chip sets

were tested. Among other tests	a coprocessor benchmark	was run

which is based	on a fractal computation and its execution time

recorded. The following tables	showing	coprocessor performance

to vary with the chip set have	been copied from these articles

in abridged form.

Cyrix				       Cyrix

chip set      387+		    chip set	       83D87

Opti, 40 MHz  24.57 sec   97.0%    PC-Chips, 33 MHz  26.97 sec	  93.0%

Elite,40 MHz  24.46 sec   97.4%    UMC,      33 MHz  27.69 sec	  90.5%

ACT,  40 MHz  23.84 sec  100.0%    Headland, 33 MHz  25.08 sec	 100.0%

Forex,40 MHz  23.84 sec  100.0%    Eteq,     33 MHZ  27.38 sec	  91.6%

This shows that performance of	the same coprocessor can vary by

up to ~10% depending on the chip set used on your board, at least

for 386 motherboards (similar numbers for 286,	386sx, and 486 are

unfortunately not available). The benchmarks for this article were

run on	a board	with the Forex chip set, which is one of the fastest

386 chip sets there is, not only with respect to floating-point

performance [35].

Description of	benchmarks

PEAKFLOP is the kernel	of a fractal computation. It consists

mainly	of a tight loop	written	in assembly code and fine tuned

to give maximum performance. All variables are	held in	the

CPU's and coprocessor's registers, so the only	memory access

is for	opcode fetches.	The main loop contains three multiplications

and five additions/subtractions. This ratio is	fairly typical

for other floating point intensive programs as	well. The whole

program fits nicely into even a very small CPU	cache. Due to

the nature of this program, its MFLOPS	rate is	hardly to be

exceeded by any program that calculates anything useful. Thus

the name PEAKFLOP. You	will find the source code for PEAKFLOP

in appendix B.

TRNSFORM multiplies an	array of 8191 vectors with a 3D-transformation

matrix	(a 4x4 matrix).	Each vector consists of	four double precision

values. Multiplying vectors with a matrix is a	typical	operation in

the manipulation (e.g.	rotation) of 3D	objects	which are made up from

many vectors decribing	the object. This benchmark stresses addition

and multiplication as well as memory access. For each vector, 16

multiplications and 12	additions are used. About 256 kByte of data

is accessed during the	benchmark. TRNSFORM is implemented as an

optimized assembler program linked with the Turbo Pascal 6.0 library.

For the IIT 3C87, a special version was written that makes use	of

the special F4X4 instruction available	on that	coprocessor. F4X4

does a	full multiplication of a 4x4 matrix by a 4x1 vector in a

single	instruction. The full source code for the TRNSFORM program is

in appendix B.

LLL is	short for Lawrence Livermore Loops [21], a set of kernels

taken from real floating point	extensive programs. Some of these

loops are vectorizable, but since we don't deal with vector

processors here, this doesn't matter. For this test, LLL was

adapted from the FORTRAN original [20]	to Turbo Pascal	6.0. By

variable overlaying (similar to FORTRAN's EQUIVALENCE statement)

memory	allocation for data was	reduced	to 64 kB, so all data fits

into a	single 64 kB segment. The older	version	of LLL is used here

which contains	14 loops. There	also exists a newer, more elaborate

version consisting of 24 kernels. The kernels in LLL exercise only

multiplication	and addition. The MFLOPS rate reported is the

average of the	MFLOPS rate of all 14 kernels as reported by the

LLL program. LLL and Whetstone	results	(see below) are	reported

as returned by	my COMPTEST test program in which they have been

included as a measure of coprocessor/FPU performance. COMPTEST

has been compiled under Turbo Pascal 6.0 with all 'optimizations'

on and	using my own run-time library, which gives higher perfor-

mance than the	one included with TP 6.0. My library is	available

as TPL60N16.ZIP from garbo.uwasa.fi and ftp-sites that	mirror

this site.

Linpack [5] is	a well known floating-point benchmark that also

heavily exercises the memory system. Linpack operates on large

matrices and takes up about 570 kB in the version used	for this

test. This is about the largest program size a	pure DOS system

can accomodate. Linpack was originally	designed to estimate

performance of	BLAS, a	library	of FORTRAN subroutines that

handles various vector	and matrix operations. It uses two routines

from BLAS which are thought to	be typical of the matrix operations

used by BLAS. Both routines only use addition/subtraction and

multiplication. The FORTRAN source code for Linpack can be

obtained from the automated mail server netlib@ornl.gov. Linpack

was compiled using MS Fortran 5.0 in the HUGE memory model (which

can handle data structures larger than	64 kB) and with	compiler

switches set for maximum optimization.	Linpack	repeatedly does

the same test.	The number reported is the maximum MFLOPS rate

returned by Linpack. Linpack MFLOPS ratings for a great number

of machines are contained in [6]. This	PostScript document is

also available	from netlib@ornl.gov.

Whetstone [2,3,4] is a	synthetic benchmark based upon statistics

collected about the use of certain control and	data structures

in programs written in	high level languages. Based on these

statistics, Whetstone tries to	mirror a 'typical' HLL program.

Whetstone performance is expressed by how many	theoretical

'whetstone' instructions are executed per second. It was

originally implemented	in ALGOL. Unlike PEAKFLOP, LLL,	and

Linpack, Whetstone not	only uses addition and multiplication

but exercises all basic arithmetic operations as well as some

transcendental	functions. Whetstone performance depends on the

speed of the coprocessor as well as on	the speed of the CPU,

while PEAKFLOP, LLL, and Linpack place	a heavier burden on the

coprocessor/FPU. There	exists an old and a new	version	of

Whetstone. Note that results from the two versions can	differ

by as much as 20% for the same	test configuration. For	this

test, the new version in Pascal from [3] was used. It was

compiled with Turbo Pascal 6.0	and my own library (see	above)

with all 'optimizations' on.

SAVAGE	tests the performance of transcendental	function

evaluation. It	is basically a small loop in which the sin,

cos, arctan, ln, exp, and sqrt	functions are combined in a

single	expression. While sin, cos, arctan, and	sqrt can be

evaluated directly with a single 387 coprocessor instruction

each, ln and exp need additional preprocessing	for argument

reduction and result conversion. According to [14], the Savage

benchmark was devised by Bill Savage, and is distributed by:

The Wohl Engine Company, Ltd.,	8200 Shore Front Parkway,

Rockaway Beach, NY 11693, USA.	Usually, Savage	is programmed

to make 250,000 passes	though the loop. Here only 10,000 loops

are executed for a total of 60,000 transcendental function

evaluations. The result is expressed in function evaluations

per second. SAVAGE source code	was taken from [7] and compiled

with Turbo Pascal 6.0 and my own run-time library (see	above).

Benchmark results for 387 coprocessors, coprocessor emulators and

the Intel RapidCAD and	Intel 486 CPUs.

40 MHz		PEAKFLOP TRNSFORM LLL	  Linpack Whetstone Savage

MFLOPS	 MFLOPS	  MFLOPS  MFLOPS  kWhet/sec Func/sec

386, EM87	0.0084	 0.0080	  0.0060  0.0060	 31	 502 ##

386, Franke387	0.0369	 0.0295	  0.0233  0.0215	164	4002 $$

386, TP 6 Emu	0.0316	 0.0273	  0.0200  0.0190	160	3794 %%

Intel 387DX	0.9204	 0.7212	  0.3932  0.3211       2428    52677

ULSI 83C87	1.2093	 0.7936	  0.3890  0.3120       2528    56926

IIT 3C87	1.0196	 0.7145	  0.3834  0.3179       2663    58766

IIT 3C87,4x4	1.0196	 1.7244	  0.3834  0.3179       2663    58766 ??

C&T 38700	1.0722	 0.7908	  0.4007  0.3222       2837    74906

Cyrix 387+	1.1305	 0.8162	  0.3945  0.3208       2946    80322

Intel RapidCAD	2.2128	 1.8931	  0.7377  0.5432       4810    86957

Intel 486	2.4762	 2.1335	  1.1110  0.8204       6195    98522

33.3 MHz	PEAKFLOP TRNSFORM LLL	  Linpack Whetstone Savage

MFLOPS	 MFLOPS	  MFLOPS  MFLOPS  kWhet/sec Func/sec

386, EM87	0.0070	 0.0040	  0.0050  0.0050	 26	 418 ##

Franke387	0.0307	 0.0246	  0.0194  0.0179	137	3335 $$

386, TP 6 Emu	0.0263	 0.0227	  0.0167  0.0158	133	3160 %%

Intel 387DX	0.7647	 0.6004	  0.3283  0.2676       2046    43860

ULSI 83C87	1.0097	 0.6609	  0.3239  0.2598       2089    47431

IIT 3C87	0.8455	 0.5957	  0.3198  0.2646       2203    49020

IIT 3C87,4X4	0.8455	 1.4334	  0.3198  0.2646       2203    49020 ??

C&T 38700	0.9455	 0.6907	  0.3338  0.2700       2376    62565

Cyrix 387+	0.9286	 0.6806	  0.3293  0.2669       2435    66890

Cyrix 83D87	1.013	    N/A	  0.333	  0.273	       2550	 N/A

Intel RapidCAD	1.8572	 1.5798	  0.6072  0.4533       3953    72464

Intel 486	2.0800	 1.7779	  0.9387  0.6682       5143    82192

For comparison:

PEAKFLOP TRNSFORM LLL	  Linpack Whetstone Savage

MFLOPS	 MFLOPS	  MFLOPS  MFLOPS  kWhet/sec Func/sec

i486DX2-66	4.1601	 3.4227	  1.6531  1.3010      10655   163934

i486DX2-50	3.0589	 2.6665	  1.2537  0.9744       7962   123203

i387, 20 MHz	0.2253	 0.3271	  0.1434  0.1171	952    21739 ++

i387DX, 20 MHz	0.3567	 0.4444	  0.1484  0.1161       1034    24155 &&

i80287, 5 MHz	0.0281	 0.0310	  0.0242  0.0222	150	3261 !!

i8087,9.54 MHz	0.0636	 0.0705	  0.0321  0.0219	234	5782 **

HW configuration for test of 387 coprocessors and Intel RapidCAD:

System	A: Motherboard with Forex chip set, 128	kB CPU Cache, 8	MB RAM

HW configuration for test of 486 FPU (extra fan for 40	MHz operation):

System	B: Motherboard with SIS	chip set, 256 kB CPU Cache, 8 MB RAM

## EM87 V1.2 by Ron Kimball is	a public domain	coprocessor emulator

that loads as a TSR. It uses INT 7 traps emitted by	80286, 80386

systems with no coprocessor	upon encountering coprocessor

instructions to catch coprocessor instructions and emulate them.

Whetstone and Savage benchmarks for	this test were compiled

with the original TP 6.0 library, as EM87 chokes on	the 387

specific FSIN and FCOS instructions	used in	my own library if

a 387 is detected. Obviously EM87 identifies itself	as a 387,

but	has no support for 387 specific	instructions.

$$ Franke387 is a commercial 387 emulator that	is also	available in

a shareware	version. For this test,	shareware version V2.4 was

used. Franke387 unlike many	other emulators	supports all 387

instructions. It is	loaded as a device driver and uses INT 7

to trap coprocessor	instructions.

%% These benchmarks were run using the	built-in coprocessor emulators

of the TP 6.0 and the MS FORTRAN 5.0 run-time libraries.

?? The	3C87 specific F4X4 instruction was used	in the vector trans-

formation benchmark.

++ Older motherboard with no chip set (discrete logic), no CPU	cache,

16 MB RAM

&& System A, CPU cache	disabled via extended set-up, turbo-switch

set	to half	speed (that is,	20 MHz)

!! 80386 @ 20 MHz / Intel 80287 @ 5 MHz, no CPU cache,	4 MB RAM

due	to the fast CPU	used here, performance figures are somewhat

higher than	can be expected	for a 80286/287	combination, except

for	the PEAKFLOP benchmark,	which is basically coprocessor limited

** 8086/8087 system with 640 kB RAM

Since neither a Weitek	coprocessor nor	a compiler that	generates

code for the Weitek chips were	available, performance data for

the Weitek Abacus is given here according to [31,32] and scaled to

show performance of a 33 MHz system. The benchmarks were compiled

using highly optimizing 32-bit	compilers.

Single Prec.     Double Prec.    Double Prec.

3167    4167     3167    4167	 387	486

Linpack MFLOPS	       1.8     5.0	0.8	3.2	 0.4	1.6

Whetstone kWhet/sec  7470   22700     4900   14000	3290  12300

Note that for the Intel coprocessors, running programs	in single

vs. double precision doesn't provide much of an performance advantage

since all internal calculations are always done in extended precision.

Using Weitek coprocessors however, performance	nearly doubles when

switching from	double to single precision. For	double precision

calculations using only basic arithmetic, the Weitek Abacus can

provide performance at	twice the level	of the respective Intel

coprocessor (387/486) clocked at the same speed at most.

Speed of various coprocessor instructions measured in clock cycles

as measured with my program 87TIMES. Error is +/- one clock cycle,

except	for the	Intel 80287. Times for the 80287 were determined on

a system with a 20 MHz	80386 and a 5 MHz Intel	80287. Therefore,

times may differ from a genuine 80286/287 system, especially for

those instructions that access	an operand in memory. Since the

times are stated as the number	of coprocessor clock cycles used,

the faster 386	which can execute four clock cycles where the 80287

executes one clock cycle may decrease memory access times as seen

by the	coprocessor.

Intel	Intel  Cyrix Cyrix C&T	 ULSI  IIT  Intel Intel

i486 RapidCAD 83D87 387+  38700 83C87 3C87 387DX 80387

FLD1    4	   3	 14    14    14	   18	 24    23    26

FLDZ    4	   3	 14    14    14	   18	 24    23    31

FLDPI    7	   8	 14    15    14	   18	 24    38    45

FLDLG2    7	   8	 14    14    14	   18	 24    33    45

FLDL2T    7	   8	 14    14    14	   19	 24    38    45

FLDL2E    7	   8	 14    14    14	   19	 24    38    45

FLDLN2    7	   8	 14    14    14	   19	 24    38    45

FLD ST(0)    4	   4	 14    14    14	   14	 24    20    21

FST ST(1)    3	   4	 14    14    14	   14	 19    18    22

FSTP ST(0)    4	   4	 14    14    14	   15	 19    19    22

FSTP ST(1)    4	   4	 15    15    14	   15	 19    20    22

FLD ST(1)    4	   4	 14    14    14	   14	 24    18    21

FXCH ST(1)    4	   4	 14    20    14	   19	 24    24    27

FILD [Word]   12	  16	 33    37    32	   42	 38    47    62

FILD [DWord]    8	  11	 26    26    21	   32	 28    35    45

FILD [QWord]    9	  15	 30    30    25	   36	 32    34    54

FLD [DWord]    3	   5	 26    26    21	   23	 28    20    25

FLD [QWord]    3	   7	 30    30    25	   27	 32    24    35

FLD [TByte]    5	  11	 46    46    46	   46	 47    46    57

FBLD [TByte]   83	  90	 66    86   106	  146	197    71   278

FIST [Word]   31	  31	 37    40    37	   42	 51    69    90

FIST [DWord]   29	  30	 35    40    35	   40	 49    66    84

FST [DWord]    7	   7	 35    37    32	   40	 33    37    40

FST [QWord]    8	   9	 43    43    39	   47	 40    45    51

FISTP [Word]   32	  32	 42    40    37	   43	 46    70    90

FISTP [DWord]   31	  31	 40    40    35	   41	 50    67    87

FISTP [QWord]   29	  29	 44    44    42	   48	 56    73    92

FSTP [DWord]    8	   8	 38    36    32	   41	 35    38    43

FSTP [QWord]    9	   9	 46    43    39	   48	 42    46    49

FSTP [TByte]    8	   8	 50    45    49	   50	 48    53    58

FBSTP [TByte]  170	 172	 98    98   114	  129	218   144   533

FINIT   17	  31	 15    16    15	   15	 16    16    25

FCLEX    7	  20	 15    16    16	   16	 16    16    25

FCHS    7	   8	 14    15    14	   14	 19    30    33

FABS    5	   5	 14    15    14	   14	 19    30    33

FXAM   12	  13	 14    15    14	   14	 19    39    43

FTST    5	   5	 19    25    14	   24	 24    34    38

FSTENV   67	  82	125   125   124	  132	124   159   165

FLDENV   44	  59	106   106   112	  120	106   119   129

FSAVE  181	 169	355   355   374	  361	376   469   511

FRSTOR  130	 203	358   358   385	  372	371   420   456

FSTSW [mem]    4	   5	 14    14    14	   14	 14    14    17

FSTSW AX    3	   4	 12    12    11	   11	 11    11    14

FSTCW [mem]    4	   5	 14    14    13	   13	 13    14    18

FLDCW [mem]    4	  11	 26    26    31	   32	 27    32    36

FADD	ST,ST(0)    8	   9	 19    20    19	   19	 24    24    32

FADD	ST,ST(1)    9	   9	 19    20    19	   18	 24    20    32

FADD	ST(1),ST   10	  10	 19    20    19	   18	 24    24    37

FADDP	ST(1),ST   11	  11	 19    19    19	   16	 24    25    37

FADD [DWord]    9	  10	 25    28    22	   23	 23    21    34

FADD [QWord]    9	  10	 32    32    26	   27	 27    25    38

FIADD [Word]   20	  21	 34    34    33	   40	 40    52    80

FIADD [DWord]   20	  21	 27    28    27	   30	 30    37    61

FSUB	ST(1),ST   10	  10	 19    20    19	   19	 24    24    38

FSUBR	ST(1),ST    9	  10	 19    22    19	   19	 24    27    38

FSUBRP	ST(1),ST   10	  10	 19    19    22	   20	 24    25    38

FSUB [DWord]   11	  12	 27    28    27	   23	 29    27    32

FSUB [QWord]   11	  12	 32    32    31	   27	 33    26    44

FISUB [Word]   21	  21	 34    34    34	   40	 40    52    80

FISUB [DWord]   21	  22	 27    28    27	   29	 30    40    60

FMUL	ST,ST(1)   16	  17	 19    25    24	   24	 29    38    57

FMUL	ST(1),ST   16	  17	 19    24    24	   24	 29    40    62

FMULP	ST(1),ST   17	  17	 19    24    24	   25	 29    40    58

FIMUL [Word]   22	  23	 40    40    37	   46	 46    52    80

FIMUL [DWord]   22	  23	 27    28    27	   36	 35    45    68

FMUL [DWord]   11	  12	 27    28    27	   28	 29    25    45

FMUL [QWord]   14	  15	 32    32    31	   32	 33    37    61

FDIV	ST,ST(0)   73	  74	 26    40    59	   54	 54    89   100

FDIV	ST,ST(1)   73	  74	 36    45    59	   54	 54    77   100

FDIV	ST(1),ST   73	  74	 36    45    59	   55	 54    78   102

FDIVR	ST(1),ST   73	  74	 36    45    59	   54	 54    77   102

FDIVRP	ST(1),ST   73	  74	 36    44    59	   55	 54    76   106

FIDIV [Word]   84	  85	 52    58    75	   76	 76   105   141

FIDIV [DWord]   84	  85	 45    46    65	   65	 65   101   123

FDIV [DWord]   73	  74	 45    46    63	   56	 59    77   101

FDIV [QWord]   73	  74	 50    50    67	   60	 63    78   103

FSQRT (0.0)   25	  25	 19    19    14	   19	 24    29    37

FSQRT (1.0)   83	  84	 36    74    54	   89	 59   109   132

FSQRT (L2T)   86	  87	 36    74    54	   89	 59   104   137

FXTRACT (L2T)   17	  17	 19    19    19	   28	 79    53    72

FSCALE (PI,5)   30	  30	 36    24    24	   49	 79    59    82

FRNDINT (PI)   31	  31	 19    29    24	   34	 29    49    82

FPREM (99,PI)   58	  59	 54    99    44	   54	 49    79    96

FPREM1(99,PI)   90	  91	 54    99    44	   59	 54   104   121

FCOM    5	   6	 15    20    19	   25	 19    29    32

FCOMP    6	   6	 15    19    19	   25	 19    30    33

FCOMPP    7	   7	 15    19    19	   25	 19    31    40

FICOM [Word]   16	  17	 34    34    33	   46	 34    58    76

FICOM [DWord]   16	  16	 21    28    21	   35	 23    45    57

FCOM [DWord]    5	   6	 21    28    22	   23	 23    27    34

FCOM [QWord]    5	   8	 27    32    25	   27	 27    31    39

FSIN (0.0)   24	  24	 14    99    14	   19	 24    39    43

FSIN (1.0)  310	 313	114   164   144	  494	219   509   596

FSIN (PI)   88	  89	118   189    64	   64	214   134   152

FSIN (LG2)  292	 295	 72    89   139	  454	184   449   531

FSIN (L2T)  299	 302	123   179   164	  469	214   454   536

FCOS (0.0)   24	  24	 19   159    14	   19	 24    34    42

FCOS (1.0)  302	 305	 84   104   139	  489	214   459   547

FCOS (PI)   88	  89	154   254    64	   64	224   199   232

FCOS (LG2)  300	 303	108   149   139	  454	194   504   583

FCOS (L2T)  307	 310	159   239   164	  469	224   509   601

FSINCOS (0.0)   25	  25	 14    19    19	   18	 34    38    55

FSINCOS (1.0)  353	 356	124   174   254	  493	419   538   636

FSINCOS (PI)  105	 106	162   263    79	   68	424   228   277

FSINCOS (LG2)  340	 343	119   159   249	  458	359   533   627

FSINCOS (L2T)  347	 350	168   248   274	  473	424   538   646

FPTAN (0.0)   25	  25	 14    19    19	   18	 29    38    46

FPTAN (1.0)  266	 269	119   149   184	  538	309   323   396

FPTAN (PI)  145	 146	134   228   104	  108	304   168   211

FPTAN (LG2)  244	 246	 94   129   179	  498	274   298   363

FPTAN (L2T)  247	 249	139   219   204	  513	304   298   365

FPATAN (0.0)   38	  39	 19    24    19	   20	 29    95    93

FPATAN (1.0)  294	 298	124   159    29	  375	604   360   433

FPATAN (PI)  304	 308	139   188   279	  360	424   375   472

FPATAN (LG2)  290	 293	128   154   269	  365	379   375   448

FPATAN (L2T)  304	 308	144   189   274	  359	424   375   468

F2XM1 (0.0)   25	  25	 14    14    14	   19	 24    34    37

F2XM1 (LN2)  209	 211	 89   119   169	  394	284   299   348

F2XM1 (LG2)  204	 206	 78   104   159	  379	284   294   337

FYL2X (1.0)   60	  61	 36    39    24	   75	 94   115   127

FYL2X (PI)  294	 297	108   163   249	  450	359   395   504

FYL2X (LG2)  311	 314	108   159   249	  460	339   410   518

FYL2X (L2T)  293	 296	108   164   249	  439	359   390   501

FYL2XP1 (LG2)  334	 337	 99   169   234	  460	284   435   538

80386 +   80386 +  80386 +

Intel Intel	 Franke387 TP 6.0   EM87

8087  80287	 Emulator  Emulator Emulator

FSTP ST(0) |   26	    54	   507	     358      2115

FLD1 |   26	    55	   481	     422      1626

FLDZ |   21	    53	   480	     416      1646

FLDPI |   26	    55	   486	     443      1626

FLDLG2 |   26	    56	   486	     423      1626

FLDL2T |   26	    55	   486	     440      1626

FLDL2E |   26	    53	   486	     423      1626

FLDLN2 |   26	    55	   486	     441      1626

FLD ST(0) |   31	    55	   493	     362      1851

FST ST(1) |   26	    54	   489	     355      1931

FSTP ST(1) |   21	    55	   507	     356      2116

FLD ST(1) |   26	    55	   493	     362      1852

FXCH ST(1) |   21	    57	   497	     486      2187

FILD [Word] |   58	    90	   667	     712      2259

FILD [DWord] |   64	    74	   608	     812      2164

FILD [QWord] |   74	    93	   652	     707      2971

FLD [DWord] |   49	    44	   633	     473      2077

FLD [QWord] |   54	    57	   641	     524      2336

FLD [TByte] |   59	    45	   607	     492      2063

FBLD [TByte] |  309	   310	  2019	    1512     17827

FIST [Word] |   79	    72	   854	     766      2418

FIST [DWord] |   84	    80	   865	     518      2325

FST [DWord] |   89	    85	   686	     441      2200

FST [QWord] |   99	    92	   703	     516      2481

FISTP [Word] |   79	    80	   864	     794      2620

FISTP [DWord] |   79	    81	   879	     541      2523

FISTP [QWord] |   88	    75	   904	     916      3226

FSTP [DWord] |   89	    75	   713	     467      2400

FSTP [QWord] |   93	    72	   732	     538      2678

FSTP [TByte] |   49	    21	   685	     467      2124

FBSTP [TByte] |  528	   472	  3305	    1555     27013

FINIT |   11	    10	   742	     641      1369

FCLEX |   11	    10	   440	     323       912

FCHS |   21	    54	   460	     354      1744

FABS |   21	    54	   456	     349      1738

FXAM |   21	    54	   481	     380      1551

FTST |   51	    75	   585	     386      2721

FSTENV |   54	    57	   928	     519      2104

FLDENV |   48	    50	  1125	     450      1631

FSAVE |  214	   244	  1949	     976      2749

FRSTOR |  209	   227	  2182	     657      2225

FSTSW [mem] |   28	    10	   516	     401      1189

FSTSW AX |  N/A	    55	   451	     N/A       N/A

FSTCW [mem] |   28	    10	   506	     359      1167

FLDCW [mem] |   19	    47	   524	     437      1584

FADD	ST,ST(0) |   86	   128	   643	     706      2805

FADD	ST,ST(1) |   85	   116	   707	     808      3093

FADD	ST(1),ST |   92	   131	   664	     812      3146

FADDP	ST(1),ST |   92	   129	   704	     799      3143

FADD [DWord] |  105	   122	   874	     969      3139

FADD [QWord] |  115	   122	   888	    1021      3396

FIADD [Word] |  115	   122	   940	    1211      3330

FIADD [DWord] |  125	   122	   882	    1297      3215

FSUB	ST(1),ST |   88	   130	   738	     817      3156

FSUBR	ST(1),ST |   96	   132	   740	     868      3004

FSUBRP	ST(1),ST |   99	   132	   733	     805      3301

FSUB [DWord] |  119	   122	   918	    1018      3127

FSUB [QWord] |  129	   123	   932	    1070      3632

FISUB [Word] |  115	   123	   977	    1081      3802

FISUB [DWord] |  125	   125	   940	     980      4161

FMUL	ST,ST(1) |  145	   151	   810	    1368      3924

FMUL	ST(1),ST |  145	   151	   817	    1377      3962

FMULP	ST(1),ST |  148	   168	   840	    1365      4164

FIMUL [Word] |  132	   151	  1039	    1517      4039

FIMUL [DWord] |  141	   151	   980	    1643      3976

FMUL [DWord] |  125	   123	   948	    1480      3445

FMUL [QWord] |  175	   192	   991	    1602      4416

FDIV	ST,ST(0) |  201	   207	   726	    1536      9789

FDIV	ST,ST(1) |  203	   218	   808	    1658     10332

FDIV	ST(1),ST |  207	   214	   825	    1655     10342

FDIVR	ST(1),ST |  201	   206	   819	    1806     10213

FDIVRP	ST(1),ST |  201	   205	   845	    1803     10409

FIDIV [Word] |  237	   227	   980	    1779     11225

FIDIV [DWord] |  246	   227	   944	    1680     11572

FDIV [DWord] |  229	   226	   893	    1722     10577

FDIV [QWord] |  236	   227	   993	    1777     10829

FSQRT (0.0) |   21	    57	   512	     382      1755

FSQRT (1.0) |  186	   206	  1106	    2504     37836

FSQRT (L2T) |  186	   207	  1398	    2467     37925

FXTRACT (L2T) |   51	    56	   726	     571      3326

FSCALE (PI,5) |   41	    56	   817	     443      3194

FRNDINT (PI) |   51	    58	   808	     800      7092

FPREM (99,PI) |   81	   131	  1696	     941      4098

FPREM1(99,PI) |  N/A	   N/A	  1625	     N/A       N/A

FCOM |   56	    75	   582	     483      2799

FCOMP |   61	    92	   616	     485      2983

FCOMPP |   61	    90	   661	     476      3198

FICOM [Word] |   79	    77	   808	     861      3654

FICOM [DWord] |   89	    77	   750	     964      3684

FCOM [DWord] |   74	    75	   741	     625      3643

FCOM [QWord] |   74	    76	   754	     667      3771

FSIN (0.0) |  N/A	   N/A	   639	     N/A       N/A

FSIN (1.0) |  N/A	   N/A	  4640	     N/A       N/A

FSIN (PI) |  N/A	   N/A	  2488	     N/A       N/A

FSIN (LG2) |  N/A	   N/A	  3911	     N/A       N/A

FSIN (L2T) |  N/A	   N/A	  3767	     N/A       N/A

FCOS (0.0) |  N/A	   N/A	   740	     N/A       N/A

FCOS (1.0) |  N/A	   N/A	  4777	     N/A       N/A

FCOS (PI) |  N/A	   N/A	  2557	     N/A       N/A

FCOS (LG2) |  N/A	   N/A	  4176	     N/A       N/A

FCOS (L2T) |  N/A	   N/A	  3905	     N/A       N/A

FSINCOS (0.0) |  N/A	   N/A	   714	     N/A       N/A

FSINCOS (1.0) |  N/A	   N/A	  6049	     N/A       N/A

FSINCOS (PI) |  N/A	   N/A	  4091	     N/A       N/A

FSINCOS (LG2) |  N/A	   N/A	  5640	     N/A       N/A

FSINCOS (L2T) |  N/A	   N/A	  5405	     N/A       N/A

FPTAN (0.0) |   41	    58	   752	    8381      2324

FPTAN (1.0) |  581	   582	  6366	   10817     29824

FPTAN (PI) |  606	   587	  4388	   12410      2300

FPTAN (LG2) |  516	   513	  5939	   12502     26770

FPTAN (L2T) |  576	   586	  5723	   12483      2301

FPATAN (0.0) |   41	    55	   616	    1208     10578

FPATAN (1.0) |  736	   736	  1426	   13446     34208

FPATAN (PI) |  206	   207	 12835	   13305     46903

FPATAN (LG2) |  756	   736	 12490	   13319     41312

FPATAN (L2T) |  206	   204	 12922	   13364     50149

F2XM1 (0.0) |   16	    56	   563	     723      1722

F2XM1 (LN2) |  631	   624	  4178	   11070     33823

F2XM1 (LG2) |  611	   585	  4798	   11116     32163

FYL2X (1.0) |   56	    57	   961	    1214      4327

FYL2X (PI) |  946	   961	  8987	   12858     40148

FYL2X (LG2) | 1081	  1038	  8933	   12748     46821

FYL2X (L2T) |  926	   886	  8982	   12712     38986

FYL2XP1 (LG2) | 1026	  1037	 10485	   11867     44708

The Weitek 3167 and 4167 coprocessors only implement the basic

arithmetic functions (add, subtract, multiply,	divide,	square

root) in hardware. Transcendental functions are implemented

by means of a software	library	supplied by Weitek that	uses

the Weitek hardware to	approximate the	transcendental functions

with polynomial and rational approximations. The clock	cycle

timings for the transcendental	functions are average values,

since execution time differs with the value of	argument. The

speed of transcendental functions for the 4167	is estimated

based on the numbers in [31,33], from which this timing

information has been extracted.

Execution time	for floating-point operations in clock cycles on

Weitek	coprocessors

Single Precision	   Double Precision

3167	     4167	   3167	       4167

ABS	     3		2	      3		  2

NEG	     6		2	      6		  2

ADD	     6		2	      6		  2

SUB	     6		2	      6		  2

SUBR	     6		2	      6		  2

MUL	     6		2	     10		  3

DIVR	    38	       17	     66		 31

SQRT	    60	       17	    118		 31

SIN	   146	      ~50	    292	       ~100

COS	   140	      ~50	    285	       ~100

TAN	   188	      ~60	    340	       ~110

EXP	   179	      ~60	    401	       ~130

LOG	   171	      ~60	    365	       ~120

F->ASCII 1000	      N/A	   1700		N/A  //

ASCII->F 1100	      N/A	   1800		N/A  //

// rough average of the timings given for different numeric

formats by Weitek. Note that these conversions routines

do much more work than the FBLD and	FBSTP instructions

provided by	the 80x87 coprocessors.	FBLD and FBSTP are

useful for conversion routines but quite a bit of additional

code is need for this purpose.

Accuracy

The IEEE-754 Standard for Binary Floating-Point Arithmetic [10,11]

is fully implemented by Intel's 387 coprocessor [17]. Among other

things, this means that the add, subtract, multiply, divide,

remainder, and	square root operations always deliver the 'exact'

result. By exact it is	meant that the coprocessor always delivers

the machine number closest to the real	result,	which may not

be representable exactly in the available numeric format. The

80387 implements the single, double, and double extended formats

as specified in the standard as well as all functions required

by it [17]. Note that earlier Intel coprocessors (the 8087 and

the 80287) comply with	a draft	version	of the standard	that differs

from the final	version. These chips came out before the IEEE-754

standard was finally accepted in 1985.	As in the 80387, the basic

arithmetic in the 8087	and the	80287 is exact in the sense that

the computed result is	always the machine number closest to the

real result. However, there are some differences regarding certain

operands like infinities and some operations like the remainder are

defined differently. Some instructions	have been added	in the 80387,

most notably the FSIN and FCOS	operations. The	argument range for

some transcendental function has been extended	[17]. Note that	the

IEEE-754 standard says	nothing	about the quality of the implementation

of transcendental functions like sin, cos, tan, arctan, log. Intel

uses a	modified CORDIC	[18,19]	technique to compute the transcendental

functions. Intel claims that maximum error in the 8087, 80287,	and

80387 for all transcendental functions	does not exceeed two bits

in the	mantissa of the	double extended	format,	which features 64

mantissa bits for an accuracy of approximately	19 decimal places

[22,23]. This claim has been independently verified by	a competing

vendor	[13]. This means that at least 62 of the 64 mantissa bits

in a transcendental function result are correct.

The Weitek Abacus 3167	and 4167 are 'mostly compatible' with

IEEE-754 [31,32,33]. They support the single precision	and double

precision numeric formats formats described in	the standard as

well as the four rounding modes required by it. However, due to

the need for extremely	high speed operation, some of the finer

points	of IEEE-754 have not been implemented. One of the most

notable omissions is the missing support for denormal numbers.

Denormals are always flushed to zero.

The 387 clone makers claim 100% compatibility with Intel's 80387.

So one	would expect the same accuracy from their chips. For example,

on the	packaging of the IIT 3C87 it says that ".. the requirements

of ANSI/IEEE standards	are fulfilled and exceeded". Cyrix states

that their 83D87 complies fully with the IEEE-754 standard [12].

Cyrix delivers	with their copocessors some diagnostic software.

This includes the program IEEETEST which is based on the IEEE test

vectors from the Ph.D.	thesis of Jerome T. Coonen [9].	A test using

the IEEE test vectors has also	been included into the RUNDIAG

program on the	Intel RapidCAD diagnostic disk.	Rather than performing

random	tests, the test	vectors	check specific cases that may

be hard to get	right. Each test vector	specifies the operation

to be performed, the operands,	precision and rounding mode to be

used, and the result (including flags set) to be expected according

to IEEE-754. I	ran IEEETEST on	all the	available coprocessors/FPUs.

The Intel 486,	Intel RapidCAD,	Intel 387, Intel 387DX,	Cyrix 83D87,

and the Cyrix 387+ passed with	no errors. The ULSI 83C87 showed

some minor flaws in the FCOM, FDIV, FMUL, and FSCALE operations,

getting flag errors in	about 1% of the	tested cases, but no

computational errors. However,	for the	IIT 3C87, the IEEETEST

program showed	flag *and* some	computational errors (that is, wrong

results) for all tested operations except FXTRACT and FCHS. The Intel

80287 shows numerous errors, but this it not surprising, since	the

80287 does not	comply with IEEE-754 but with an earlier draft of that

standard, so it does some thing differently than required by the final

version of the	standard.

Although IEEETEST is written in Turbo Pascal, the coprocessor

emulator in the TP 6.0	library	could not be tested since IEEETEST

was compiled with the $E- switch excluding the	emulator from

program code. The public domain emulator EM87 could be	tested,	but

hung in the last test which checks the	implementation of the

remainder operation. This is probably caused by some bug in the

emulation of the FPREM	instruction tested in this test. It is

interesting to	note how the error profile of EM87 matches exactly

that of the Intel 80287, so it	can be assumed that EM87 is a very

good emulation	of the 80287. The Franke387 V2.4 emulator hung in

the division test quite early in IEEETEST. The	tests performed

up to the division test reported several errors.

Explanatory text printed at the start of the IEEETEST program:

JT Coonen's 1984 UC Berkeley Ph.D. thesis centers around his

activities as a member	of the floating-point working group that

defined the IEEE 754-1985 Standard for	Binary Floating-Point

Arithmetic. Appendix C	of his thesis presents FPTEST, a Pascal

program written by J Thomas and JT Coonen. IEEETEST is	a port of

FPTEST	and runs on PCs	whose math coprocessor accepts 80387

compatible floating-point instructions.

IEEETEST reads	test vectors from the file TESTVECS and	compares

the answer returned by	the math coprocessor with the answer listed

in the	test vector. If	these answers differ an	'F' is displayed,

otherwise a '.'is displayed. Answers can differ due to	two types

of failures: numeric failures or flag failures. Numeric failures

occur when the	computed answer	has the	wrong value. Flag failures

occur when the	status (invalid	operation, divide by zero, underflow,

overflow, inexact) is incorrectly identified.

TESTVECS is the concatenation of unmodified versions of all the

test vectors distributed by UC	Berkeley. The test data	base is

copyrighted by	UC Berkeley (1985) and is being	distributed with

their permission. FPTEST and the test data base can be	obtained

by asking for 'IEEE-754 Test Vector' from UC Berkeley,	Electrical

Engineering and Computer Science, Industrial Liaison Program,

479 Corey Hall, Berkeley, CA, 94720 (415)643-6687.

The initial version of	this test data base for	the proposed IEEE

754 binary floating-point standard (draft 8.0)	was developed for

Zilog,	Inc. and was donated to	the floating-point working group

for dissemination. Errors in or additions to the distributed data

base should be	reported to the	agency of distribution,	with copies

to Zilog, Inc., 1315 Dell Avenue, Campbell, CA, 95008.

IEEETEST output for Intel 80387, Intel	387DX, Intel 486, C&T 38700,

Cyrix 83D87, Cyrix	387+, Intel RapidCAD

IEEE-754 Test Vector  Precisions: S=Single D=Double E=Double Extended

|     TESTS     | numeric TYPE OF FAILURE flag

Operation Code | Passed	Failed |    S	 D    E	|   S	 D    E

----------------------------------------------------------------------

Absolute Value    A |    216	     0 |    0	 0    0	|   0	 0    0

Addition    + |   3528	     0 |    0	 0    0	|   0	 0    0

Comparison    C |   4320	     0 |    0	 0    0	|   0	 0    0

Copy Sign    @ |   1488	     0 |    0	 0    0	|   0	 0    0

Division    / |   4311	     0 |    0	 0    0	|   0	 0    0

Fraction Part    F |    624	     0 |    0	 0    0	|   0	 0    0

Logb    L |    960	     0 |    0	 0    0	|   0	 0    0

Multiplication    * |   3978	     0 |    0	 0    0	|   0	 0    0

Negation    - |    216	     0 |    0	 0    0	|   0	 0    0

Next After    N |   2832	     0 |    0	 0    0	|   0	 0    0

Round to Integer    I |    558	     0 |    0	 0    0	|   0	 0    0

Scalb    S |    948	     0 |    0	 0    0	|   0	 0    0

Square Root    V |    744	     0 |    0	 0    0	|   0	 0    0

Subtraction    - |   3528	     0 |    0	 0    0	|   0	 0    0

Remainder    % |   2984	     0 |    0	 0    0	|   0	 0    0

Totals |  31235	     0 |

IEEETEST output for ULSI 83C87	(manufactured 91/48)

IEEE-754 Test Vector  Precisions: S=Single D=Double E=Double Extended

|     TESTS     | numeric TYPE OF FAILURE flag

Operation Code | Passed	Failed |    S	 D    E	|   S	 D    E

----------------------------------------------------------------------

Absolute Value    A |    216	     0 |    0	 0    0	|   0	 0    0

Addition    + |   3528	     0 |    0	 0    0	|   0	 0    0

Comparison    C |   4312	     8 |    0	 0    0	|   0	 0    8

Copy Sign    @ |   1488	     0 |    0	 0    0	|   0	 0    0

Division    / |   4250	    61 |    0	 0    0	|  28	28    5

Fraction Part    F |    624	     0 |    0	 0    0	|   0	 0    0

Logb    L |    960	     0 |    0	 0    0	|   0	 0    0

Multiplication    * |   3936	    42 |    0	 0    0	|  19	19    4

Negation    - |    216	     0 |    0	 0    0	|   0	 0    0

Next After    N |   2828	     4 |    0	 0    0	|   0	 0    4

Round to Integer    I |    558	     0 |    0	 0    0	|   0	 0    0

Scalb    S |    930	    18 |    0	 0    0	|   6	 6    6

Square Root    V |    744	     0 |    0	 0    0	|   0	 0    0

Subtraction    - |   3528	     0 |    0	 0    0	|   0	 0    0

Remainder    % |   2984	     0 |    0	 0    0	|   0	 0    0

Totals |  31102	   133 |

IEEETEST output for ULSI 83S87	(manufactured 92/17)

(data kindly supplied by Bengt	Ask, f89ba@efd.lth.se)

IEEE-754 Test Vector  Precisions: S=Single D=Double E=Double Extended

|     TESTS     | numeric TYPE OF FAILURE flag

Operation Code | Passed	Failed |    S	 D    E	|   S	 D    E

----------------------------------------------------------------------

Absolute Value    A |    216	     0 |    0	 0    0	|   0	 0    0

Addition    + |   3528	     0 |    0	 0    0	|   0	 0    0

Comparison    C |   4320	     0 |    0	 0    0	|   0	 0    0

Copy Sign    @ |   1488	     0 |    0	 0    0	|   0	 0    0

Division    / |   4296	    15 |    0	 0    0	|   5	 5    5

Fraction Part    F |    624	     0 |    0	 0    0	|   0	 0    0

Logb    L |    960	     0 |    0	 0    0	|   0	 0    0

Multiplication    * |   3966	    12 |    0	 0    0	|   4	 4    4

Negation    - |    216	     0 |    0	 0    0	|   0	 0    0

Next After    N |   2828	     4 |    0	 0    0	|   0	 0    4

Round to Integer    I |    558	     0 |    0	 0    0	|   0	 0    0

Scalb    S |    930	    18 |    0	 0    0	|   6	 6    6

Square Root    V |    744	     0 |    0	 0    0	|   0	 0    0

Subtraction    - |   3528	     0 |    0	 0    0	|   0	 0    0

Remainder    % |   2984	     0 |    0	 0    0	|   0	 0    0

Totals |  31102	    45 |

IEEETEST output for IIT 3C87 (manufactured 92/20)

IEEE-754 Test Vector  Precisions: S=Single D=Double E=Double Extended

|     TESTS     | numeric TYPE OF FAILURE flag

Operation Code | Passed	Failed |    S	 D    E	|   S	 D    E

----------------------------------------------------------------------

Absolute Value    A |    200	    16 |    0	 0   16	|   0	 0    0

Addition    + |   3336	   192 |    0	 0  128	|   0	 0   96

Comparison    C |   4224	    96 |    0	 0   96	|   0	 0    0

Copy Sign    @ |   1488	     0 |    0	 0    0	|   0	 0    0

Division    / |   4159	   152 |    0	 0  124	|   0	 0  116

Fraction Part    F |    600	    24 |    0	 0   24	|   0	 0   24

Logb    L |    960	     0 |    0	 0    0	|   0	 0    0

Multiplication    * |   3702	   276 |    0	 0  248	|   0	 0  100

Negation    - |    200	    16 |    0	 0   16	|   0	 0    0

Next After    N |   2248	   584 |    0	 0  584	|   0	 0  168

Round to Integer    I |    542	    16 |    0	 0    4	|   0	 0   16

Scalb    S |    874	    74 |    5	 5   44	|   8	 8   20

Square Root    V |    688	    56 |    0	 0   56	|   0	 0   56

Subtraction    - |   3336	   192 |    0	 0  128	|   0	 0   96

Remainder    % |   2844	   140 |    0	 0  140	|   0	 0  116

Totals |  29401	  1834 |

IEEETEST output for Intel 80287 run together with a 80386 CPU

IEEE-754 Test Vector  Precisions: S=Single D=Double E=Double Extended

|     TESTS     | numeric TYPE OF FAILURE flag

Operation Code | Passed	Failed |    S	 D    E	|   S	 D    E

----------------------------------------------------------------------

Absolute Value    A |    216	     0 |    0	 0    0	|   0	 0    0

Addition    + |   2886	   642 |   16	16  112	| 174  174  174

Comparison    C |      0	  4320 | 1324 1324 1324	|1332 1332 1332

Copy Sign    @ |   1488	     0 |    0	 0    0	|   0	 0    0

Division    / |   3777	   534 |   18	18   37	| 169  169  165

Fraction Part    F |    552	    72 |   24	24   24	|  24	24   24

Logb    L |    900	    60 |   12	12   12	|  20	20   20

Multiplication    * |   2944	  1034 |  105  105  197	| 303  303  231

Negation    - |    216	     0 |    0	 0    0	|   0	 0    0

Next After    N |    348	  2484 |  768  768  768	| 504  504  526

Round to Integer    I |    546	    12 |    0	 0    0	|   4	 4    4

Scalb    S |    663	   285 |   45	43   26	| 102	98   46

Square Root    V |    720	    24 |    4	 4    4	|   8	 8    8

Subtraction    - |   2886	   642 |   16	16  112	| 174  174  174

Remainder    % |    708	  2276 |  768  768  560	| 216  216  216

Totals |  18850	 12385 |

IEEETEST output for EM87 coprocessor emulator run on an Intel 386 CPU

IEEE-754 Test Vector  Precisions: S=Single D=Double E=Double Extended

|     TESTS     | numeric TYPE OF FAILURE flag

Operation Code | Passed	Failed |    S	 D    E	|   S	 D    E

----------------------------------------------------------------------

Absolute Value    A |    216	     0 |    0	 0    0	|   0	 0    0

Addition    + |   2886	   642 |   16	16  112	| 174  174  174

Comparison    C |      0	  4320 | 1324 1324 1324	|1332 1332 1332

Copy Sign    @ |   1488	     0 |    0	 0    0	|   0	 0    0

Division    / |   3777	   534 |   18	18   37	| 169  169  165

Fraction Part    F |    552	    72 |   24	24   24	|  24	24   24

Logb    L |    900	    60 |   12	12   12	|  20	20   20

Multiplication    * |   2944	  1034 |  105  105  197	| 303  303  231

Negation    - |    216	     0 |    0	 0    0	|   0	 0    0

Next After    N |    348	  2484 |  768  768  768	| 504  504  526

Round to Integer    I |    546	    12 |    0	 0    0	|   4	 4    4

Scalb    S |    663	   285 |   45	43   26	| 102	98   46

Square Root    V |    720	    24 |    4	 4    4	|   8	 8    8

Subtraction    - |   2886	   642 |   16	16  112	| 174  174  174

To complement the checks done by IEEETEST I wrote some	short

programs DENORMTS, RCTRL, PCTRL in Turbo Pascal 6.0 that test

the following features:

1. support for	denormals in all precisions (single, double, extended)

2. support for	the four IEEE rounding modes (up, down,	nearest, chop)

3. support for	precision control

Note that passing all tests is	required for IEEE conformance and

required for compatibility with Intel's coprocessors. Precision

control forces	the results of the FADD, FSUB, FMUL, FDIV, and FSQRT

instruction to	be rounded to the specified precision (single, double,

double	extended). This	feature	is provided to obtain compatibility

with certain programming languages [17]. By specifying	lower

precision, one	effectively nullifies the advantages of	extended

precision intermediate	results. The IEEE-754 standard for floating

point arithmetic demands that processors/fp-packages that can not

store the result of operations	*directly* to single and double

precision location must provide precision control. The	programs

that test precision control and rounding control are designed to

return	a different result for each of the modes for the same sequence

of operation. The source code of the programs can be found in appendix

A. The	Intel 8087 and 80287 were not tested with DENORMTS since Turbo

Pascal	does not support extended precision denormals on 8087/80287

processors, so	the denormal test fails	anyway.	The 8087 and 287

pass the RCTRL	and PCTRL tests, though.

These are the results for the Intel 387, Intel	387DX, Intel 486,

Intel RapidCAD, Cyrix 83D87, Cyrix 387+, C&T 38700, and the EM87

emulator (on a	80386 machine)

Precision Control	     SINGLE   1.13311278820037842E+0000

DOUBLE   1.23456789006442125E+0000

EXTENDED 1.23456789012337585E+0000

Rounding Control	     NEAREST -1.23427629010100635E+0100

DOWN    -1.23427623555772409E+0100

UP	     -1.23457760966801097E+0100

CHOP    -1.23397493540770643E+0100

Denormal support

SINGLE	denormals supported

SINGLE	denormal prints	as:    4.60943116855005E-0041

Denormal should be printed as 4.60943...E-0041

DOUBLE	denormals supported

DOUBLE	denormal prints	as:    8.75000000000016E-0311

Denormal should be printed as 8.75...E-0311

EXTENDED denormals supported

EXTENDED denormal prints as:  1.31640625000000E-4934

Denormal should be printed as 1.3164...E-4934

These are the results for the ULSI 83C87

Precision Control	     SINGLE   1.23456789012337585E+0000

DOUBLE   1.23456789012337585E+0000

EXTENDED 1.23456789012337585E+0000

Rounding Control	     NEAREST -1.23427629010100635E+0100

DOWN    -1.23427623555772409E+0100

UP	     -1.23457760966801097E+0100

CHOP    -1.23397493540770643E+0100

Denormal support

SINGLE	denormals supported

SINGLE	denormal prints	as:    4.60943116855005E-0041

Denormal should be printed as 4.60943...E-0041

DOUBLE	denormals supported

DOUBLE	denormal prints	as:    8.75000000000016E-0311

Denormal should be printed as 8.75...E-0311

EXTENDED denormals supported

EXTENDED denormal prints as:  1.31640625000000E-4934

Denormal should be printed as 1.3164...E-4934

These are the results for the IIT 3C87

Precision Control	     SINGLE   1.13311278820037842E+0000

DOUBLE   1.23456789006442125E+0000

EXTENDED 1.23456789012337585E+0000

Rounding Control	     NEAREST -1.23427629010100635E+0100

DOWN    -1.23427623555772409E+0100

UP	     -1.23457760966801097E+0100

CHOP    -1.23397493540770643E+0100

Denormal support

SINGLE	denormals supported

SINGLE	denormal prints	as:    4.60943116855005E-0041

Denormal should be printed as 4.60943...E-0041

DOUBLE	denormals supported

DOUBLE	denormal prints	as:    8.75000000000016E-0311

Denormal should be printed as 8.75...E-0311

EXTENDED denormals not	supported

These are the results for the TP 6.0 coprocessor emulator:

Precision Control	     SINGLE   1.23456789012351396E+0000

DOUBLE   1.23456789012351396E+0000

EXTENDED 1.23456789012351396E+0000

Rounding Control	     NEAREST -1.23457766383395931E+0100

DOWN    -1.23457766383395931E+0100

UP	     -1.23457766383395931E+0100

CHOP    -1.23457766383395931E+0100

Denormal support

SINGLE	denormals not supported

DOUBLE	denormals not supported

EXTENDED denormals not	supported

The test results show that the	IIT 3C87 does not conform to the

IEEE-754 floating-point standard in that it does not support

denormals in double extended precision. The ULSI 83C87	does not

conform to that standard in that it does not support precision

control, but uses double extended precision for all operations.

The TP	6.0 emulator supports neither precision	control, rounding

control nor support for any denormals.	In addition, its basic

arithmetic operations do not seem to conform to the IEEE standard

as the	results	of the test programs differ from that of any result

computed by a coprocessor for any mode.

With regard to	the accuracy of	transcendental functions, Cyrix

claims	that the relative error	of the transcendental functions

on the	83D87 never exceeds 0.5	units in the last place	(0.5 ULP)

of the	double extended	format [13]. This means	that the maximum

relative error	is below 2**-64, while Intel's published error

limit is 2**-62. While	Intel uses a modified CORDIC algorithm

[18,19] to compute the	transcendental functions, Cyrix	uses

rational approximations that utilize a	very fast array	multiplier.

For an	explanation why	this approach is superior to CORDIC with

todays	technology, see	[61]. Also, Cyrix uses an internal 75 bit

data path for the mantissa [15], so intermediate computations in

the generation	of transcendental function values will enjoy some

additional accuracy over the 64 bits provided by the double

extended format. Using	75 mantissa bits also provides an advantage

over other coprocessors like the Intel	387DX and ULSI 83C87 which

use only a 68 bit data	path for the mantissa [58,59]. Note that a

maximum relative error	of 0.5 ULP for the Cyrix coprocessor does

not mean that it returns the 'exact' result (machine number closest

to infinitely precise result) all the time. Just consider the case

where the infinitely precise result of	a transcendental function

falls nearly half way between two machine numbers. A relative error

of 0.5	ULP can	cause the result to be either of the numbers after

rounding, depending on	the direction of the error. But	the 83D87

should	deliver	results	that never differ from the 'exact' result

by more than one ULP. Please note that	the claim of relative error

being below 0.5 ULPs is slightly exaggerated. 0.6 ULPs	would be a

more realistic	error limit. Imagine that the infinitely precise

result	for some argument to a transcendental was xxx..xxx1001...

(where	the xxx...xxx represent	the first 64 bits of the result),

but that the coprocessor computes the result as xxx..xxx0111 and

then round this down to xxx..xxx0000. Then the	relative error is

(1001b-0b)/1000b = 0.5625 ULPs. I tested some of the transcendental

functions of the Cyrix	387+ and found the relative error to be

always	below 0.6 ULPs.	Cyrix also claims that its transcendental

functions satisfy the monotonicity criterion [13], a claim not

made by any of	the competitors, which does not	mean that the

transcendental	functions on the other 387 compatibles may not be

monotonic, too. Monotonicity means that for all x1 > x2, it always

follows that f(x1) >= f(x2) for an increasing function	like sin

on [0..pi/4]. Likewise, for a decreasing function like	cos on

[0..pi/4], for	all x1 > x2, it	follows	that f(x1) <= f(x2).

The Weitek Abacus 3167	and 4167 implement only	the basic arithmetic

operations (add, subtract, negate, multiply, divide, square root)

in hardware. Transcendental functions are provided via	a software

library provided by Weitek. For these library functions Weitek

claims	a maximum relative error of 5 ULPs [31,33] (ULP	= Unit in

the Last Place, numeric weight	of the least significant mantissa

bit). This means that the last	three bits in the mantissa of a

double	precision result can be	wrong. Note that the Intel 387 and

compatible math coprocessors generate the transcendental functions

with a	small relative error with regard to the	*extended double

precision* format. Thus, when rounded to double precision, their

function values are nearly always 'exact'. 387	type coprocessors

have superior accuracy	when compared with Weitek's coprocesssors.

The test diskette distributed with early versions of the

Cyrix 83D87 contained a program TRANCK	that checks the

accuracy of the transcendental	functions in the coprocessor

against a more	precise	software arithmetic [16]. I used this

program to compare the	accuracy of the	transcendental functions

on those 287/387/486 coprocessors/FPUs	available to me. As TRANCK

will not accept negative numbers as intervall limits, I tested

each function on an intervall along the positive x-axis. The

functions tested are F2XM1 (2**x-1), FSIN (sine), FCOS	(cosine),

FPTAN (tangent), FPATAN (arctangent), FYL2X (y	* log2 (x)),

and FYL2XP1 (y	* log2 (x+1)). These are all the transcendental

functions implemented on the 80387. Note that the square root

(FSQRT) is *not* a transcendental function. For every function,

100,000 arguments were	evaluated. The arguments were uniformally

distributed within the	intervall tested. The EM87 emulator could

not be	checked	with TRANCK, since the multiple	precision package

in TRANCK would always	return with an error message immediately.

However, the Franke387	emulator could be tested.

Test results for accuracy of transcendental functions for double

extended precision as returned	by the program TRANCK. 100,000

trials	per function.

%wrong	     is	the percentage of results that differ from the 'exact'

result (infinitely	precise	result rounded to 64 bits)

ULP_hi	     is	the number of results where the	returned result	was

greater than the 'exact' (correctly rounded) result by

one ULP (the numeric weight of the	last mantissa bit,

2**-63 to 2**-64 depending	of the size of the number).

ULPs_hi     is	the number of results where the	returned result	was

greater than the 'exact' result by	two or more ULPs.

ULP_lo	     is	the number of results where the	returned result	was

smaller than the 'exact' (correctly rounded) result by

one ULP (the numeric weight of the	last mantissa bit,

2**-63 to 2**-64 depending	of the size of the number).

ULPs_lo     is	the number of results where the	returned result	was

smaller than the 'exact' result by	two or more ULPs.

max ULP err is	the maximum deviation of a returned result from	the

'exact' answer expressed in ULPs.

Franke387 V2.4	emulator

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	    39.042  25301     708  13029       4       2

COS	0,pi/4	    75.714  49827   25887      0       0       3

TAN	0,pi/4	    76.976  14230   10029  24323   28394       9

ATAN	0,1	    55.826  26028    1529  24044    4225       4

2XM1	0,0.5	    96.717	0	0  47910   48807       5

YL2XP1	0,sqrt(2)-1 93.007    578	9  27416   65004       8

YL2X	0.1,10	    62.252  16817    4712  37082    3641    2953

Microsoft's coproc. emulator (part of MS-C and MS-Fortran libraries)

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	       N/A    N/A     N/A    N/A     N/A     N/A

COS	0,pi/4	       N/A    N/A     N/A    N/A     N/A     N/A

TAN	0,pi/4	    40.828  27764    1520  11445      99       2

ATAN	0,1	    32.307  18893     485  12530     299       2

2XM1	0,0.5	    52.163   8585     189  37745    5644       3

YL2XP1	0,sqrt(2)-1 88.801   4714     916  14239   68932      11

YL2X	0.1,10	    36.598  13813    3272  13866    5647      11

INTEL 8087, 80287

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	       N/A    N/A     N/A    N/A     N/A     N/A

COS	0,pi/4	       N/A    N/A     N/A    N/A     N/A     N/A

TAN	0,pi/4	    37.001  18756     524  17405     316       2

ATAN	0,1	     9.666   6065	0   3601       0       1

2XM1	0,0.5	    19.920	0	0  19920       0       1

YL2XP1	0,sqrt(2)-1  7.780    868	0   6912       0       1

YL2X	0.1,10	     1.287    723	0    564       0       1

INTEL 387

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	    28.872   2467	0  26392      13       2

COS	0,pi/4	    27.213  27169      35      9       0       2

TAN	0,pi/4	    10.532    441	0  10091       0       1

ATAN	0,1	     7.088   2386	0   4691       1       2

2XM1	0,0.5	    32.024	0	0  32024       0       1

YL2XP1	0,sqrt(2)-1 22.611   3461	0  19150       0       1

YL2X	0.1,10	    13.020   6508	0   6512       0       1

INTEL 387DX

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	    28.873   2467	0  26393      13       2

COS	0,pi/4	    27.121  27090      22      9       0       2

TAN	0,pi/4	    10.711    457	0  10254       0       1

ATAN	0,1	     7.088   2386	0   4691       1       2

2XM1	0,0.5	    32.024	0	0  32024       0       1

YL2XP1	0,sqrt(2)-1 22.611   3461	0  19150       0       1

YL2X	0.1,10	    13.020   6508	0   6512       0       1

ULSI 83C87

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	    35.530   4989	6  30238     297       2

COS	0,pi/4	    43.989  11193     675  31393     728       2

TAN	0,pi/4	    48.539  18880    1015  26349    2295       3

ATAN	0,1	    20.858     62	0  20796       0       1

2XM1	0,0.5	    21.257	4	0  21253       0       1

YL2XP1	0,sqrt(2)-1 27.893   9446	0  18213     234       2

YL2X	0.1,10	    13.603   9816	0   3787       0       1

IIT 3C87

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	    18.650  11171      0    7479       0       1

COS	0,pi/4	     7.700   3024      0    4676       0       1

TAN	0,pi/4	    20.973   9681      0   11291       1       2

ATAN	0,1	    19.280  13186      0    6094       0       1

2XM1	0,0.5	    25.660  17570      0    8090       0       1

YL2XP1	0,sqrt(2)-1 45.830  23503   1896   19654     777       3

YL2X	0.1,10	    10.888   5638    357    4845      48       3

C&T 38700DX

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	     1.821   1272      0     549       0       1

COS	0,pi/4	    23.358  12458      0   10901       0       1

TAN	0,pi/4	    17.178  10725      0    6453       0       1

ATAN	0,1	     9.359   7082      0    2277       0       1

2XM1	0,0.5	    15.188   3039      0   12149       0       1

YL2XP1	0,sqrt(2)-1 19.497  12109      0    7388       0       1

YL2X	0.1,10	    46.868    261      0   46607       0       1

CYRIX 83D87

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	     1.554   1015      0     539       0       1

COS	0,pi/4	     0.925    143      0     782       0       1

TAN	0,pi/4	     4.147    881      0    3266       0       1

ATAN	0,1	     0.656    229      0     427       0       1

2XM1	0,0.5	     2.628   1433      0    1194       0       1

YL2XP1	0,sqrt(2)-1  3.242    825      0    2417       0       1

YL2X	0.1,10	     0.931    256      0     675       0       1

CYRIX 387+

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	     1.486    864	0    622       0       1

COS	0,pi/4	     2.072     12	0   2060       0       1

TAN	0,pi/4	     0.602     63	0    539       0       1

ATAN	0,1	     0.384     12	0    372       0       1

2XM1	0,0.5	     1.985     27	0   1958       0       1

YL2XP1	0,sqrt(2)-1  3.662   1705	0   1957       0       1

YL2X	0.1,10	     0.764    367	0    397       0       1

INTEL RapidCAD, Intel 486

max

funct.	intervall   %wrong ULP_hi ULPs_hi ULP_lo ULPs_lo ULP err

SIN	0,pi/4	    16.991   1517	0  15474       0       1

COS	0,pi/4	     9.003   7603	0   1400       0       1

TAN	0,pi/4	    10.532    441	0  10091       0       1

ATAN	0,1	     7.078   2386	0   4691       1       2

2XM1	0,0.5	    32.025	0	0  32025       0       1

YL2XP1	0,sqrt(2)-1 21.800    533	0  21267       0       1

YL2X	0.1,10	     3.894   1879	0   2015       0       1

The test results above	indicate that all 80x87	compatibles do not

exceed	Intel's stated error bound of 3 ULPs for the transcendental

functions. However, some coprocessors are more	accurate than others.

Rating	the coprocessors according to the accuracy of their trans-

cendental functions gives the following list (highest accuracy

first): Cyrix 387+, Cyrix 83D87, Intel	486, Intel RapidCAD, Intel

80287(!), C&T 38700DX,	Intel 387DX, Intel 80387, IIT 3C87, ULSI 83C87.

The tests also	show that the problems with excessive inaccuracy of

the transcendental functions in early versions	of the IIT coprocessors

with errors of	up to 8	ULPs [8] have been eliminated. According to

[56], certain problems	with the FPATAN	instruction on the IIT 3C87

occuring under	the UNIX version of AutoCAD have been corrected	in

June, 1990. The Franke387 has acceptable accuracy for the FSIN, FCOS,

and FPATAN instructions, taking into consideration that according to

its documentation, Franke387 uses only	64 bits	of precision for the

intermediate results, while coprocessors typically use	68 bits

and more. However, the	larger error in	the FPTAN, F2XM1, FYL2XP1,

and especially	the FYL2X operations show that the emulator doesn't

use state of the art algorithms, which	ensure an error	of only	a

very few ULPs even if no extra	precise	intermediate results are

available. Microsoft's coprocessor emulator provides transcendental

functions with	rather good accuracy, except for the logarithmic

operations, which seem	to contain some	minor flaws.

Chips and Technologies	has included a program SMDIAG on the diagnostic

disk V1.0 for the SuperMATH 38700DX to	test the compatibility of the

computational results and flag	settings returned by the coprocessor

with the Intel	387DX. However,	the tests for the transcendental

functions seem	to have	been tweaked to	let the	C&T 38700DX pass, while

coprocessors like the Intel RapidCAD and the Cyrix 83D87 fail.	Also,

SMDIAG	shows failure in the FSCALE test for the Intel RapidCAD,

Cyrix 83D87, Cyrix 387+, and ULSI 83C87, although they	return the

correct result	according to Intel's documentation for the Intel

387DX (Intel's second generation 387), which is indeed returned by

the 387DX. SMDIAG expects the result returned by the original Intel

80387.

Results of running the	SMDIAG program on 387 compatible coprocessors

(p = passed, f	= failed)

Intel  Intel  Intel  Cyrix  Cyrix	   IIT	 ULSI	 C&T

Test	   RapidCAD  387DX  80387   387+  83D87	  3C87	83C87  38700

1  (fstore)	  f	 p	p      p      f	     f	    f	   p ##

2  (fiall)	  p	 p	p      p      p	     p	    f	   p

3  (faddsub)	  p	 p	p      p      p	     p	    p	   p

4  (faddsub_nr)  p	 p	p      p      f	     f	    f	   p

5  (faddsub_cp)  p	 p	p      p      f	     f	    f	   p

6  (faddsub_dn)  p	 p	p      p      f	     f	    f	   p

7  (faddsub_up)  p	 p	p      p      f	     f	    f	   p

8  (fmul)	  p	 p	p      p      p	     f	    f	   p

9  (fdivn)	  p	 p	p      p      p	     p	    p	   p

10 (fdiv)	  p	 p	p      p      p	     p	    f	   p

11 (fxch)	  p	 p	p      p      p	     p	    p	   p

12 (fyl2x)	  p	 p	p      f      f	     f	    f	   p ++

13 (fyl2xp1)	  f	 p	p      f      f	     f	    f	   p ++

14 (fsqrt)	  p	 p	p      p      p	     p	    p	   p

15 (fsincos)	  f	 p	p      f      f	     f	    f	   p ++

16 (fptan)	  p	 p	p      f      p	     f	    f	   p ++

17 (fpatan)	  p	 p	p      f      f	     f	    f	   p ++

18 (f2xm1)	  p	 p	p      f      f	     f	    f	   p ++

19 (fscale)	  f	 f	p      f      f	     f	    f	   p **

20 (fcom1)	  p	 p	p      p      p	     f	    f	   p

21 (fprem)	  p	 p	p      p      p	     p	    p	   p

22 (misc1)	  p	 p	p      p      p	     f	    f	   p

23 (misc3)	  p	 p	p      p      p	     p	    p	   p

24 (misc4)	  p	 p	p      p      f	     f	    p	   p

failed	modules:  4	 1	0      7     12	    16	   17	   0

## the	failure	of the Intel RapidCAD is caused	by the fact that

it stores the value	of BCD INDEFINITE differently from the

Intel 387DX. It uses FFFFC000000000000000, while the 387DX uses

FFFF8000000000000000. However, both	encodings are valid according

to Intel's documentation, which defines the BCD INDEFINITE as

FFFFUUUUUUUUUUUUUUUU, where	U is undefined.	So failure of the

RapidCAD to	deliver	the same answer	as the 387DX is	*not* an

error, just	a very slight incompatibility.

** the	FSCALE errors reported for the Intel 387DX, Intel RapidCAD,

Cyrix 83D87, Cyrix 387+, and ULSI 83C87 are	due to a single

'wrong' result each	returned by one	of the FSCALE computations.

SMDIAG expects the result returned by the first generation

Intel 80387	(and the C&T 38700DX). However,	this result is

wrong according to Intel's documentation and the behavior was

corrected in the second generation Intel 387DX. Therefor, the

Intel RapidCAD, Cyrix 83D87, Cyrix 387+, and ULSI 83C87 return

the	result compatible with the Intel 387DX.

++ The	failures for the test of transcendental	functions are caused

by the tested coprocessor returning	results	that differ from the

ones returned by the Intel 387DX. On the Cyrix 83D87, Cyrix	387+,

and	Intel RapidCAD,	this is	simply due to the improved accuracy

these coprocessors provide over the	Intel 387DX. The failures of

the	IIT 3C87 and ULSI 83C87	are mainly due to the lesser accuracy

in the transcendental functions of these coprocesosors, but	for

the	IIT 3C87 an additional source of failures is its inability to

handle extended precision denormals.

References

[1]  Schnurer,	G.: Zahlenknacker im Vormarsch.

c't 1992, Heft 4, Seiten 170-186

[2]  Curnow, H.J.; Wichmann, B.A.: A synthetic	benchmark.

Computer Journal,	Vol. 19, No. 1,	1976, pp. 43-49

[3]  Wichmann,	B.A.: Validation code for the Whetstone	benchmark.

NPL Report DITC 107/88, National Physics Laboratory, UK,

March 1988

[4]  Curnow, H.J.: Wither Whetstone? The Synthetic Benchmark after

15 Years.

In: Aad van der Steen (ed.): Evaluating Supercomputers.

London: Chapman and Hall 1990

[5]  Dongarra,	J.J.: The Linpack Benchmark: An	Explanation.

In: Aad van der Steen (ed.): Evaluating Supercomputers.

London: Chapman and Hall 1990

[6]  Dongarra,	J.J.: Performance of Various Computers Using Standard

Linear Equations Software.

Report CS-89-85, Computer	Science	Department, University of

Tennessee, March 11, 1992

[7]  Huth, N.:	Dichtung und Wahrheit oder Datenblatt und Test.

Design & Elektronik 1990,	Heft 13, Seiten	105-110

[8]  Ungerer, B.: Sockelfolger.

c't 1990, Heft 4, Seiten 162-163

[9]  Coonen, J.T.: Contributions to a Proposed	Standard for Binary

Floating-Point Arithmetic

Ph.D. thesis, University of California, Berkeley,	1984

[10] IEEE: IEEE Standard for Binary Floating-Point Arithmetic.

SIGPLAN Notices, Vol. 22,	No. 2, 1985, pp. 9-25

[11] IEEE Standard for	Binary Floating-Point Arithmetic.

ANSI/IEEE	Std 754-1985.

New York,	NY: Institute of Electrical and	Electronics

Engineers	1985

[12] FasMath 83D87 Compatibility Report. Cyrix	Corporation, Nov. 1989

Order No.	B2004

[13] FasMath 83D87 Accuracy Report. Cyrix Corporation,	July 1990

Order No.	B2002

[14] FasMath 83D87 Benchmark Report. Cyrix Corporation, June 1990

Order No.	B2004

[15] FasMath 83D87 User's Manual. Cyrix Corporation, June 1990

Order No.	L2001-003

[16] Brent, R.P.: A FORTRAN multiple-precision	arithmetic package.

ACM Transactions on Mathematical Software, Vol. 4, No. 1,

March 1978, pp. 57-70

[17] 387DX User's Manual, Programmer's	Reference. Intel Corporation,

1989

Order No.	231917-002

[18] Volder, J.E.: The	CORDIC Trigonometric Computing Technique.

IRE Transactions on Electronic Computers,	Vol. EC-8, No. 5,

September	1959, pp. 330-334

[19] Walther, J.S.: A unified algorithm for elementary	functions.

AFIPS Conference Proceedings, Vol. 38, SJCC 1971,	pp. 379-385

[20] Esser, R.; Kremer, F.; Schmidt, W.G.: Testrechnungen auf der

IBM 3090E	mit Vektoreinrichtung.

Arbeitsbericht RRZK-8803,	Regionales Rechenzentrum an der

Universit"at zu Kln, Februar 1988

[21] McMahon, H.H.: The Livermore Fortran Kernels: A test of the

numerical	performance range.

Technical	Report UCRL-53745, Lawrence Livermore National

Laboratory, USA, December	1986

[22] Nave, R.:	Implementation of Transcendental Functions on a	Numerics

Processor.

Microprocessing and Microprogramming, Vol. 11, No. 3-4,

March-April 1983,	pp. 221-225

[23] Yuen, A.K.: Intel's Floating-Point Processors.

Electro/88 Conference Record, Boston, MA,	USA, 10-12 May 1988,

pp. 48/5-1 - 48/5-7

[24] Stiller, A.; Ungerer, B.:	Ausgerechnet.

c't 1990, Heft 1, Seiten 90-92

[25] Rosch, W.L.: Handfeste Hilfe oder	Seifenblase?

PC Professionell,	Juni 1991, Seiten 214-237

[26] Intel 80286 Hardware Reference Manual. Intel Corporation,	1987

Order No.210760-002

[27] AMD 80C287 80-bit	CMOS Numeric Processor.	Advanced Micro Devices,

June 1989

Order No.	11671B/0

[28] Intel RapidCAD(tm) Engineering CoProcessor Performance Brief.

Intel Corporation, 1992

[29] i486(tm) Microprocessor Performance Report. Intel	Corporation,

April 1990

Order No.	240734-001

[30] Intel486(tm) DX2 Microprocessor Performance Brief. Intel

Corporation, March 1992

Order No.	241254-001

[31] Abacus 3167 Floating-Point Coprocessor Data Book.	Weitek

Corporation, July	1990

DOC No. 9030

[32] WTL 4167 Floating-Point Coprocessor Data Book. Weitek

Corporation, July	1989

DOC No. 8943

[33] Abacus Software Designer's Guide. Weitek Corporation,

September	1989

DOC No. 8967

[34] Stiller, A.: Cache & Carry.

c't 1992, Heft 6, Seiten 118-130

[35] Stiller, A.: Cache & Carry, Teil 2.

c't 1992, Heft 7, Seiten 28-34

[36] Palmer, J.F.; Morse, S.P.: Die mathematischen Grundlagen der

Numerik-Prozessoren 8087/80287.

Mnchen: tewi 1985

[37] 80C187 80-bit Math Coprocessor Data Sheet. Intel Corporation,

September	1989

Order No.	270640-003

[38] IIT-2C87 80-bit Numeric Co-Processor Data	Sheet. IIT, May	1990

[39] Engineering note 4x4 matrix multiply transformation. IIT,	1989

[40] Tscheuschner, E.:	4 mal 4	auf einen Streich.

c't 1990, Heft 3, Seiten 266-276

[41] Goldberg,	D.: Computer Arithmetic.

In: Hennessy, J.L.; Patterson, D.A.: Computer Architecture A

Quantitative Approach. San Mateo,	CA: Morgan Kaufmann 1990

[42] 8087 Math	Coprocessor Data Sheet.	Intel Corporation, October 1989,

Order No.	205835-007

[43] 8086/8088	User's Manual, Programmer's and	Hardware Reference.

Intel Corporation, 1989

Order No.	240487-001

[44] 80286 and	80287 Programmer's Reference Manual. Intel Corporation,

1987

Order No.	210498-005

[45] 80287XL/XLT CHMOS	III Math Coprocessor Data Sheet. Intel

Corporation, May 1990

Order No.	290376-001

[46] Cyrix FasMath(tm)	82S87 Coprocessor Data Sheet. Cyrix Coporation,

1991

Document 94018-00	Rev. 1.0

[47] IIT-3C87 80-bit Numeric Co-Processor Data	Sheet. IIT, May	1990

[48] 486(tm)SX(tm) Microprocessor/ 487(tm)SX(tm) Math CoProcessor

Data Sheet. Intel	Corporation, April 1991.

Order No.	240950-001

[49] Schnurer,	G.: Die	gro"se Verlade.

c't 1991, Heft 7, Seiten 55-57

[50] Schnurer,	G.: Eine 4 f"ur alle.

c't 1991, Heft 6, Seite 25

[51] Intel486(tm)DX Microprocessor Data Book. Intel Corporation,

June 1991

Order No.	240440-004

[52] i486(tm) Microprocessor Hardware Reference Manual. Intel

Corporation, 1990

Order No.	240552-001

[53] i486(tm) Microprocessor Programmer's Reference Manual. Intel

Corporation, 1990

Order No.	240486-001

[54] Ungerer, B.: Kalte H"ute.

c't 1992, Heft 8, Seiten 140-144

[55] Ungerer, B.: Hei"se Sache.

c't 1991, Heft 4, Seiten 104-108

[56] Rosch, W.L.: Handfeste Hilfe oder	Seifenblase?

PC Profesionell, Juni 1991, Seiten 214-237

[57] Niederkr"uger, W.: Lebendige Vergangenheit.

c't 1990, Heft 12, Seiten 114-116

[58] ULSI Math*Co Advanced Math Coprocessor Technical Specification.

ULSI System, 5/92, Rev. E

[59] 387(tm)DX	Math CoProcessor Data Sheet. Intel Corporation,

September	1990.

Order No.	240448-003

[60] 387(tm) Numerics Coprocessor Extension Data Sheet. Intel

Corporation, February 1989.

Order No.	231920-005

[61] Koren, I.; Zinaty, O.: Evaluating	Elementary Functions in	a

Numerical	Coprocessor Based on Rational Approximations.

IEEE Transactions	on Computers, Vol. C-39, No. 8,	August 1990,

pp. 1030-1037

[62] 387(tm) SX Math CoProcessor Data Sheet. Intel Corporation,

November 1989

Order No.	240225-005

[63] Frenkel, G.: Coprocessors	Speed Numeric Operations.

PC-Week, August 27, 1990

[64] Schnurer,	G.; Stiller, A.: Auto-Matt.

c't 1991, Heft 10, Seiten 94-96

[65] Grehan, R.: FPU Face-Off.

Byte, November 1990, pp. 194-200

[66] Tang, P.T.P.: Testing Computer Arithmetic	by Elementary Number

Theory. Preprint MCS-P84-0889, Mathematics and Computer Science

Division,	Argonne	National Laboratory, August 1989

[67] Ferguson,	W.E.: Selecting	math coprocessors.

IEEE Spectrum, July 1991,	pp. 38-41

[68] Schnabel,	J.: Viermal 387.

Computer Pers"onlich 1991, Heft 22, Seiten 153-156

[69] Hofmann, J.: Starke Rechenknechte.

mc 1990, Heft 7, Seiten 64-67

[70] Woerrlein, H.; Hinnenberg, R.: Die Lust an der Power.

Computer Live 1991, Heft 10, Seiten 138-149

[71] email from Peter Forsberg	(peterf@vnet.ibm.com),

email from Alan Brown (abrown@Reston.ICL.COM)

[72] email from Eric Johnson (johnsone%camax01@uunet.UU.NET),

email from Jerry Whelan (guru@stasi.bradley.edu),

email from Arto Viitanen (av@cs.uta.fi),

email from Richard Krehbiel (richk@grebyn.com)

[73] email from Fred Dunlap (cyrix!volt!fred@texsun.Central.Sun.COM)

[74] correspondence with Bengt	Ask (f89ba@efd.lth.se)

[75] email from Thomas	Hoberg (tmh@prosun.first.gmd.de)

[76] Microsoft	Macro Assembler	Programmer's Guide Version 6.0,

Microsoft	Corporation, 1991. Document No.	LN06556-0291

Manufacturer's addresses

Intel Corporation

3065 Bowers Avenue

Santa Clara, CA 95051

USA

IIT Integrated	Information Technology,	Inc.

2540 Mission College Blvd.

Santa Clara, CA 95054

USA

ULSI Systems, Inc.

58 Daggett Drive

San Jose, CA 95134

USA

Chips & Technologies, Inc.

3050 Zanker Road

San Jose, CA 95134

USA

Weitek	Corporation

1060 East Arques Avenue

Sunnyvale, CA 94086

USA

AMD Advanced Microdevices, Inc.

901 Thompson Place

P.O.B.	3453

Sunnyvale, CA 94088-3453

USA

Cyrix Corporation

P.O.B.	850118

Richardson, TX	75085

USA

Appendix A

{$N+,E+}

PROGRAM PCtrl;

VAR B,c: EXTENDED;

Precision,	L: WORD;

PROCEDURE SetPrecisionControl (Precision: WORD);

(* This procedure sets	the internal precision of the NDP. Available *)

(* precision values:  0  -  24	bits (SINGLE)			     *)

(*		       1  -  n.a. (mapped to single)		     *)

(*		       2  -  53	bits (DOUBLE)			     *)

(*		       3  -  64	bits (EXTENDED)			     *)

VAR CtrlWord: WORD;

BEGIN {SetPrecisionCtrl}

IF Precision = 1 THEN

Precision := 0;

Precision := Precision SHL 8; { make mask for PC field in ctrl word}

ASM

FSTCW	[CtrlWord]	  { store NDP control word }

MOV	AX, [CtrlWord]	  { load control word into CPU }

AND	AX, 0FCFFh	  { mask out precision control field }

OR	AX, [Precision]	  { set	desired	precision in PC	field }

MOV	[CtrlWord], AX	  { store new control word }

FLDCW	[CtrlWord]	  { set	new precision control in NDP }

END;

END; {SetPrecisionCtrl}

BEGIN {main}

FOR	Precision := 1 TO 3 DO BEGIN

B := 1.2345678901234567890;

SetPrecisionControl (Precision);

FOR L :=	1 TO 20	DO BEGIN

B := Sqrt (B);

END;

FOR L :=	1 TO 20	DO BEGIN

B := B*B;

END;

SetPrecisionControl (3);	  { full precision for printout	}

WriteLn (Precision, B:28);

END;

END.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

{$N+,E+}

PROGRAM RCtrl;

VAR B,c: EXTENDED;

RoundingMode, L: WORD;

PROCEDURE SetRoundingMode (RCMode: WORD);

(* This procedure selects one of four available rounding modes	*)

(* 0  -  Round	to nearest (default)				*)

(* 1  -  Round	down (towards negative infinity)		*)

(* 2  -  Round	up (towards positive infinity)			*)

(* 3  -  Chop (truncate, round	towards	zero)			*)

VAR CtrlWord: WORD;

BEGIN

RCMode := RCMode SHL 10;	  { make mask for RC field in control word}

ASM

FSTCW	[CtrlWord]	  { store NDP control word }

MOV	AX, [CtrlWord]	  { load control word into CPU }

AND	AX, 0F3FFh	  { mask out rounding control field }

OR	AX, [RCMode]	  { set	desired	precision in RC	field }

MOV	[CtrlWord], AX	  { store new control word }

FLDCW	[CtrlWord]	  { set	new rounding control in	NDP }

END;

END;

BEGIN

FOR	RoundingMode :=	0 TO 3 DO BEGIN

B := 1.2345678901234567890e100;

SetRoundingMode (RoundingMode);

FOR L :=	1 TO 51	DO BEGIN

B := Sqrt (B);

END;

FOR L	:= 1 TO	51 DO BEGIN

B := -B*B;

END;

SetRoundingMode (0);	   { round to nearest for printout }

WriteLn (RoundingMode, B:28);

END;

END.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

{$N+,E+}

PROGRAM DenormTs;

VAR E:	EXTENDED;

D:	DOUBLE;

S:	SINGLE;

BEGIN

WriteLn ('Testing support and printing of denormals');

WriteLn;

Write ('Coprocessor is: ');

CASE Test8087 OF

0: WriteLn ('Emulator');

1: WriteLn ('8087 or compatible');

2: WriteLn ('80287 or compatible');

3: WriteLn ('80387 or compatible');

END;

WriteLn;

S := 1.18e-38;

S := S * 3.90625e-3;

IF S = 0 THEN

WriteLn ('SINGLE denormals not supported')

ELSE BEGIN

WriteLn ('SINGLE denormals supported');

WriteLn ('SINGLE denormal prints as:   ', S);

WriteLn ('Denormal should be printed as 4.60943...E-0041');

END;

WriteLn;

D := 2.24e-308;

D := D * 3.90625e-3;

IF D = 0 THEN

WriteLn ('DOUBLE denormals not supported')

ELSE BEGIN

WriteLn ('DOUBLE denormals supported');

WriteLn ('DOUBLE denormal prints as:   ', D);

WriteLn ('Denormal should be printed as 8.75...E-0311');

END;

WriteLn;

E := 3.37e-4932;

E := E * 3.90625e-3;

IF E = 0 THEN

WriteLn ('EXTENDED denormals not supported')

ELSE BEGIN

WriteLn ('EXTENDED denormals supported');

WriteLn ('EXTENDED denormal prints as: ', E);

WriteLn ('Denormal should be printed as 1.3164...E-4934');

END;

END.

Appendix B

; FILE: APFELM4.ASM

; assemble with MASM /e APFELM4 or TASM /e APFELM4

CODE	     SEGMENT BYTE PUBLIC 'CODE'

ASSUME  CS: CODE

PAGE    ,120

PUBLIC  APPLE87;

APPLE87     PROC    NEAR

PUSH    BP			 ; save	caller's base pointer

MOV     BP, SP		 ; make	new frame pointer

PUSH    DS			 ; save	caller's data segment

PUSH    SI			 ; save	register

PUSH    DI			 ;  variables

LDS     BX, [BP+04]	 ; pointer to parameter	record

FINIT			 ; init	80x87	       FSP->R0

FILD   WORD  PTR [BX+02]	 ; maxrad	       FSP->R7

FLD    QWORD PTR [BX+08]	 ; qmax		       FSP->R6

FSUB   QWORD PTR [BX+16]	 ; qmax-qmin	       FSP->R6

DEC    WORD  PTR [BX+04]	 ; ymax-1

FIDIV  WORD  PTR [BX+04]	 ; (qmax-qmin)/(ymax-1)FSP->R6

FSTP   QWORD PTR [BX+16]	 ; save	delta_q	       FSP->R7

FLD    QWORD PTR [BX+24]	 ; pmax		       FSP->R6

FSUB   QWORD PTR [BX+32]	 ; pmax-pmin	       FSP->R6

DEC    WORD  PTR [BX+06]	 ; xmax-1

FIDIV  WORD  PTR [BX+06]	 ; delta_p	       FSP->R6

MOV    AX,	[BX]		 ; save	maxiter,[BX] needed for

MOV    [BX+2], AX		 ;  80x87 status now

XOR    BP,	BP		 ; y=0

FLD    QWORD PTR [BX+08]	 ; qmax		       FSP->R5

CMP    WORD  PTR [BX+40], 0 ; fast	mode on	8087 desired ?

JE	    yloop		 ; no, normal mode

FSTCW  [BX]		 ; save	NDP control word

AND    WORD PTR [BX], 0FCFFh; set PCTRL = single precision

FLDCW  [BX]		 ; get back NDP	control	word

yloop:	     XOR    DI,	DI		 ; x=0

FLD    QWORD PTR [BX+32]	 ; pmin		       FSP->R4

xloop:	     FLDZ			 ; j**2= 0	       FSP->R3

FLDZ			 ; 2ij = 0	       FSP->R2

FLDZ			 ; i**2= 0	       FSP->R1

MOV    CX,	[BX+2]		 ; maxiter

MOV    DL,	41h		 ; mask	for C0 and C3 cond.bits

iteration:  FSUB   ST,	ST(2)		 ; i**2-j**2	       FSP->R1

FADD   ST,	ST(3)		 ; i**2-j**2+p = i     FSP->R1

FLD    ST(0)		 ; duplicate i	       FSP->R0

FMUL   ST(1), ST		 ; i**2		       FSP->R0

FADD   ST,	ST(0)		 ; 2i		       FSP->R0

FXCH   ST(2)		 ; 2*i*j	       FSP->R0

FADD   ST,	ST(5)		 ; 2*i*j+q = j	       FSP->R0

FMUL   ST(2), ST		 ; 2*i*j	       FSP->R0

FMUL   ST,	ST(0)		 ; j**2		       FSP->R0

FST    ST(3)		 ; save	j**2	       FSP->R0

FADD   ST,	ST(1)		 ; i**2+j**2	       FSP->R0

FCOMP  ST(7)		 ; i**2+j**2 > maxrad? FSP->R1

FSTSW  [BX]		 ; save	80x87 cond.codeFSP->R1

TEST   BYTE PTR [BX+1], DL	 ; test	carry and zero flags

LOOPNZ iteration		 ; until maxiter if not	diverg.

MOV    DX,	CX		 ; number of loops executed

NEG    CX			 ; carry set if	CX <> 0

ADC    DX,	0		 ; adjust DX if	no. of loops<>0

; plot point here (DI = X,	BP = y,	DX has the color)

FSTP   ST(0)		 ; pop i**2	       FSP->R2

FSTP   ST(0)		 ; pop 2ij	       FSP->R3

FSTP   ST(0)		 ; pop j**2	       FSP->R4

FADD   ST,ST(2)		 ; p=p+delta_p	       FSP->R4

INC    DI			 ; x:=x+1

CMP    DI,	[BX+6]		 ; x > xmax ?

JBE    xloop		 ; no, continue	on same	line

FSTP   ST(0)		 ; pop p	       FSP->R5

FSUB   QWORD PTR [BX+16]	 ; q=q-delta_q	       FSP->R5

INC    BP			 ; y:=y+1

CMP    BP,	[BX+4]		 ; y > ymax ?

JBE    yloop		 ; no, picture not done	yet

groesser:   POP    DI			 ; restore

POP    SI			 ;  register variables

POP    DS			 ; restore caller's data segm.

POP    BP			 ; save	caller's base pointer

RET    4			 ; pop parameters and return

APPLE87     ENDP

CODE	     ENDS

END

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

UNIT Time;

INTERFACE

FUNCTION Clock: LONGINT;	   { same as VMS; time in milliseconds }

IMPLEMENTATION

FUNCTION Clock: LONGINT; ASSEMBLER;

ASM

PUSH    DS		  { save caller's data segment }

XOR	    DX,	DX	  { initialize data segment to }

MOV	    DS,	DX	  {  access ticker counter }

MOV	    BX,	46Ch	  { offset of ticker counter in	segm.}

MOV	    DX,	43h	  { timer chip control port }

MOV	    AL,	4	  { freeze timer 0 }

PUSHF		  { save caller's int flag setting }

STI			  { allow update of ticker counter }

LES	    DI,	DS:[BX]	  { read BIOS ticker counter }

OUT	    DX,	AL	  { latch timer	0 }

LDS	    SI,	DS:[BX]	  { read BIOS ticker counter }

IN	    AL,	40h	  { read latched timer 0 lo-byte }

MOV	    AH,	AL	  { save lo-byte }

IN	    AL,	40h	  { read latched timer 0 hi-byte }

POPF		  { restore caller's int flag }

XCHG    AL,	AH	  { correct order of hi	and lo }

MOV	    CX,	ES	  { ticker counter 1 in	CX:DI:AX }

CMP	    DI,	SI	  { ticker counter updated ? }

JE	    @no_update	  { no }

OR	    AX,	AX	  { update before timer	freeze ? }

JNS	    @no_update	  { no }

MOV	    DI,	SI	  { use	second }

MOV	    CX,	DS	  {  ticker counter }

@no_update:NOT	    AX		  { counter counts down	}

MOV	    BX,	36EDh	  { load multiplier }

MUL	    BX		  { W1 * M }

MOV	    SI,	DX	  { save W1 * M	(hi) }

MOV	    AX,	BX	  { get	M }

MUL	    DI		  { W2 * M }

XCHG    BX,	AX	  { AX = M, BX = W2 * M	(lo) }

MOV	    DI,	DX	  { DI = W2 * M	(hi) }

ADD	    BX,	SI	  { accumulate }

ADC	    DI,	0	  {  result }

XOR	    SI,	SI	  { load zero }

MUL	    CX		  { W3 * M }

ADD	    AX,	DI	  { accumulate }

ADC	    DX,	SI	  {  result in DX:AX:BX	}

MOV	    DH,	DL	  { move result	}

MOV	    DL,	AH	  {  from DL:AX:BX }

MOV	    AH,	AL	  {   to }

MOV	    AL,	BH	  {    DX:AX:BH	}

MOV	    DI,	DX	  { save result	}

MOV	    CX,	AX	  {  in	DI:CX }

MOV	    AX,	25110	  { calculate correction }

MUL	    DX		  {  factor }

SUB	    CX,	DX	  { subtract correction	}

SBB	    DI,	SI	  {  factor }

XCHG    AX,	CX	  { result back	}

MOV	    DX,	DI	  {  to	DX:AX }

POP	    DS		  { restore caller's data segment }

END;

BEGIN

Port [$43] := $34;		 { need	rate generator,	not square wave}

Port [$40] := 0;		 { generator as	prog. by some BIOSes }

Port [$40] := 0;		 { for timer 0 }

END. {	Time }

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

{$A+,B-,R-,I-,V-,N+,E+}

PROGRAM PeakFlop;

USES Time;

TYPE ParamRec = RECORD

MaxIter, MaxRad, YMax, XMax: WORD;

Qmax, Qmin,	Pmax, Pmin: DOUBLE;

FastMod: WORD;

PlotFkt: POINTER;

FLOPS:LONGINT;

END;

VAR Param: ParamRec;

Start: LONGINT;

{$L APFELM4.OBJ}

PROCEDURE Apple87 (VAR	Param: ParamRec);     EXTERNAL;

BEGIN

WITH Param DO BEGIN

MaxIter:= 50;

MaxRad := 30;

YMax   := 30;

XMax   := 30;

Pmin   :=-2.1;

Pmax   := 1.1;

Qmin   :=-1.2;

Qmax   := 1.2;

FastMod:= Word (FALSE);

PlotFkt:= NIL;

Flops  := 0;

END;

Start := Clock;

Apple87 (Param);	     { executes	104002 FLOP }

Start := Clock - Start;  { elapsed time in milliseconds }

WriteLn ('Peak-MFLOPS: ', 104.002 /	Start);

END.

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

; FILE: M4X4.ASM

;

; assemble with TASM /e M4X4 or MASM /e M4X4

CODE	   SEGMENT BYTE	PUBLIC 'CODE'

ASSUME  CS:CODE

PUBLIC  MUL_4x4

PUBLIC  IIT_MUL_4x4

FSBP0	   EQU	   DB  0DBh, 0E8h	 ; declare special IIT

FSBP1	   EQU	   DB  0DBh, 0EBh	 ;  instructions

FSBP2	   EQU	   DB  0DBh, 0EAh

F4X4	   EQU	   DB  0DBh, 0F1h

;---------------------------------------------------------------------

;

; MUL_4x4 multiplicates a four-by-four	matrix by an array of four

; dimensional vectors.	This operation is needed for 3D	transformations

; in graphics data processing.	There are arrays for each component of

; a vector. Thus there	is an ;	array containing all the x components,

; another containing all the y	components and so on. Each component is

; an 8	byte IEEE floating point number. Two indices into the array of

; vectors are given. The first	is the index of	the vector that	will be

; processed first, the	second is the index of the vector processed

; last.

;

;---------------------------------------------------------------------

MUL_4x4   PROC	   NEAR

AddrX   EQU DWORD PTR [BP+24] ; address of X	component array

AddrY   EQU DWORD PTR [BP+20] ; address of Y	component array

AddrZ   EQU DWORD PTR [BP+16] ; address of Z	component array

AddrW   EQU DWORD PTR [BP+12] ; address of W	component array

AddrT   EQU DWORD PTR [BP+8]	 ; addr. of 4x4	transform. mat.

F	   EQU WORD  PTR [BP+6]	 ; first vector	to process

K	   EQU WORD  PTR [BP+4]	 ; last	vector to process

RetAddr EQU WORD  PTR [BP+2]	 ; return address saved	by call

SavdBP  EQU WORD  PTR [BP+0]	 ; saved frame pointer

SavdDS  EQU WORD  PTR [BP-2]	 ; caller's data segment

PUSH	   BP			 ; save	TURBO-Pascal frame pointer

MOV	   BP, SP		 ; new frame pointer

PUSH	   DS			 ; save	TURBO-Pascal data segment

MOV	   CX, K		 ; final index

SUB	   CX, F		 ; final index - start index

JNC	   $ok			 ; must	not

JMP	   $nothing		 ;  be negative

$ok:	   INC	   CX			 ; number of elements

MOV	   SI, F		 ; init	offset into arrays

SHL	   SI, 1		 ; each

SHL	   SI, 1		 ;  element

SHL	   SI, 1		 ;   has 8 bytes

LDS	   DI, AddrT		 ; addr. of transformation mat.

FLD	   QWORD PTR [DI]	 ; load	a[0,0]	 = R7

FLD	   QWORD PTR [DI+8]	 ; load	a[0,1]	 = R6

$mat_mul: LES	   BX, AddrX		 ; addr. of x component	array

FLD	   QWORD PTR ES:[BX+SI]	 ; load	x[a]	 = R5

LES	   BX, AddrY		 ; addr. of y component	array

FLD	   QWORD PTR ES:[BX+SI]	 ; load	y[a]	 = R4

LES	   BX, AddrZ		 ; addr. of z component	array

FLD	   QWORD PTR ES:[BX+SI]	 ; load	z[a]	 = R3

LES	   BX, AddrW		 ; addr. of w component	array

FLD	   QWORD PTR ES:[BX+SI]	 ; load	w[a]	 = R2

FLD	   ST(5)		 ; load	a[0,0]	 = R1

FMUL	   ST, ST(4)		 ; a[0,0] * x[a] = R1

FLD	   ST(5)		 ; load	a[0,1]	 = R0

FMUL	   ST, ST(4)		 ; a[0,1] * y[a] = R0

FADDP   ST(1), ST		 ; a[0,0]*x[a]+a[0,1]*y[a]=R1

FLD	   QWORD PTR [DI+16]	 ; load	a[0,2]	 = R0

FMUL	   ST, ST(3)		 ; a[0,2] * z[a] = R0

FADDP   ST(1), ST		 ; a[0,0]*x[a]...a[0,2]*z[a]=R1

FLD	   QWORD PTR [DI+24]	 ; load	a[0,3]	 = R0

FMUL	   ST, ST(2)		 ; a[0,3] * w[a] = R0

FADDP   ST(1), ST		 ; a[0,0]*x[a]...a[0,3]*w[a]=R1

LES	   BX, AddrX		 ; get address of x vector

FSTP	   QWORD PTR ES:[BX+SI]	 ; write new x[a]

FLD	   QWORD PTR [DI+32]	 ; load	a[1,0]	 = R1

FMUL	   ST, ST(4)		 ; a[1,0] * x[a] = R1

FLD	   QWORD PTR [DI+40]	 ; load	a[1,1]	 = R0

FMUL	   ST, ST(4)		 ; a[1,1] * y[a] = R0

FADDP   ST(1), ST		 ; a[1,0]*x[a]+a[1,1]*y[a]=R1

FLD	   QWORD PTR [DI+48]	 ; load	a[1,2]	 = R0

FMUL	   ST, ST(3)		 ; a[1,2] * z[a] = R0

FADDP   ST(1), ST		 ; a[1,0]*x[a]...a[1,2]*z[a]=R1

FLD	   QWORD PTR [DI+56]	 ; load	a[1,3]	 = R0

FMUL	   ST, ST(2)		 ; a[1,3] * w[a] = R0

FADDP   ST(1), ST		 ; a[1,0]*x[a]...a[1,3]*w[a]=R1

LES	   BX, AddrY		 ; get address of y vector

FSTP	   QWORD PTR ES:[BX+SI]	 ; write new y[a]

FLD	   QWORD PTR [DI+64]	 ; load	a[2,0]	 = R1

FMUL	   ST, ST(4)		 ; a[2,0] * x[a] = R1

FLD	   QWORD PTR [DI+72]	 ; load	a[2,1]	 = R0

FMUL	   ST, ST(4)		 ; a[2,1] * y[a] = R0

FADDP   ST(1), ST		 ; a[2,0]*x[a]+a[2,1]*y[a]=R1

FLD	   QWORD PTR [DI+80]	 ; load	a[2,2]	 = R0

FMUL	   ST, ST(3)		 ; a[2,2] * z[a] = R0

FADDP   ST(1), ST		 ; a[2,0]*x[a]...a[2,2]*z[a]=R1

FLD	   QWORD PTR [DI+88]	 ; load	a[2,3]	 = R0

FMUL	   ST, ST(2)		 ; a[2,3] * w[a] = R0

FADDP   ST(1), ST		 ; a[2,0]*x[a]...a[2,3]*w[a]=R1

LES	   BX, AddrZ		 ; get address of z vector

FSTP	   QWORD PTR ES:[BX+SI]	 ; write new z[a]

FLD	   QWORD PTR [DI+96]	 ; load	a[3,0]	 = R1

FMULP   ST(4), ST		 ; a[3,0] * x[a] = R5

FLD	   QWORD PTR [DI+104]	 ; load	a[3,1]	 = R1

FMULP   ST(3), ST		 ; a[3,1] * y[a] = R4

FLD	   QWORD PTR [DI+112]	 ; load	a[3,2]	 = R1

FMULP   ST(2), ST		 ; a[3,2] * z[a] = R3

FLD	   QWORD PTR [DI+120]	 ; load	a[3,3]	 = R1

FMULP   ST(1), ST		 ; a[3,3] * w[a] = R2

FADDP   ST(1), ST		 ; a[3,3]*w[a]+a[3,2]*z[a]=R3

FADDP   ST(1), ST		 ; a[3,3]*w[a]...a[3,1]*y[a]=R4

FADDP   ST(1), ST		 ; a[3,3]*w[a]...a[3,0]*x[a]=R5

LES	   BX, AddrW		 ; get address of w vector

FSTP	   QWORD PTR ES:[BX+SI]	 ; write new w[a]

ADD	   SI, 8		 ; new offset into arrays

DEC	   CX			 ; decrement element counter

JZ	   $done		 ; no elements left, done

JMP	   $mat_mul		 ; transform next vector

$done:	   FSTP	    ST(0)		 ; clear

FSTP	    ST(0)		 ;  FPU	stack

$nothing: POP	    DS			 ; restore TP data segment

POP	    BP			 ; restore TP frame pointer

RET	    24			 ; pop parameters and return

MUL_4X4   ENDP

;---------------------------------------------------------------------

;

; IIT_MUL_4x4 multiplicates a four-by-four matrix by an array of four

; dimensional vectors.	This operation is needed for 3D	transformations

; in graphics data processing.	There are arrays for each component of

; a vector.  Thus there is an array containing	all the	x components,

; another containing all the y	components and so on. Each component is

; an 8	byte IEEE floating point number. Two indices into the array of

; vectors are given. The first	is the index of	the vector that	will be

; processed first, the	second is the index of the vector processed

; last. This subroutine uses the special instructions only available

; on IIT coprocessors to provide fast matrix multiply capabilities.

; So make sure	to use it only on IIT coprocessors.

;

;---------------------------------------------------------------------

IIT_MUL_4x4   PROC    NEAR

AddrX   EQU DWORD PTR [BP+24] ; address of X	component array

AddrY   EQU DWORD PTR [BP+20] ; address of Y	component array

AddrZ   EQU DWORD PTR [BP+16] ; address of Z	component array

AddrW   EQU DWORD PTR [BP+12] ; address of W	component array

AddrT   EQU DWORD PTR [BP+8]	 ; addr. of 4x4	transf.	matrix

F	   EQU WORD  PTR [BP+6]	 ; first vector	to process

K	   EQU WORD  PTR [BP+4]	 ; last	vector to process

RetAddr EQU WORD  PTR [BP+2]	 ; return address saved	by call

SavdBP  EQU WORD  PTR [BP+0]	 ; saved frame pointer

SavdDS  EQU WORD  PTR [BP-2]	 ; caller's data segment

Ctrl87  EQU WORD  PTR [BP-4]	 ; caller's 80x87 control word

PUSH	   BP			 ; save	TURBO-Pascal frame ptr

MOV	   BP, SP		 ; new frame pointer

PUSH	   DS			 ; save	TURBO-Pascal data seg.

SUB	   SP, 2		 ; make	local variabe

FSTCW   [Ctrl87]		 ; save	80x87 ctrl word

LES	   SI, AddrT		 ; ptr to transformation matrix

FINIT			 ; initialize coprocessor

FSBP2			 ; set register	bank 2

FLD	   QWORD PTR ES:[SI]	 ; load	a[0,0]

FLD	   QWORD PTR ES:[SI+32]	 ; load	a[1,0]

FLD	   QWORD PTR ES:[SI+64]	 ; load	a[2,0]

FLD	   QWORD PTR ES:[SI+96]	 ; load	a[3,0]

FLD	   QWORD PTR ES:[SI+8]	 ; load	a[0,1]

FLD	   QWORD PTR ES:[SI+40]	 ; load	a[1,1]

FLD	   QWORD PTR ES:[SI+72]	 ; load	a[2,1]

FLD	   QWORD PTR ES:[SI+104] ; load	a[3,1]

FINIT			 ; initialize coprocessor

FSBP1			 ; set register	bank 1

FLD	   QWORD PTR ES:[SI+16]	 ; load	a[0,2]

FLD	   QWORD PTR ES:[SI+48]	 ; load	a[1,2]

FLD	   QWORD PTR ES:[SI+80]	 ; load	a[2,2]

FLD	   QWORD PTR ES:[SI+112] ; load	a[3,2]

FLD	   QWORD PTR ES:[SI+24]	 ; load	a[0,3]

FLD	   QWORD PTR ES:[SI+56]	 ; load	a[1,3]

FLD	   QWORD PTR ES:[SI+88]	 ; load	a[2,3]

FLD	   QWORD PTR ES:[SI+120] ; load	a[3,3]

; transformation matrix loaded

MOV	   AX, F		 ; index of first vector

MOV	   DX, K		 ; index of last vector

MOV	   BX, AX		 ; index 1st vector to process

MOV	   CL, 3		 ; component has 8 (2**3) bytes

SHL	   BX, CL		 ; compute offset into arrays

FINIT			 ; initialize coprocessor

FSBP0			 ; set register	bank 0

$mat_loop:LES	   SI, AddrW		 ; addr. of W component	array

FLD	   QWORD PTR ES:[SI+BX]	 ; W component current vector

LES	   SI, AddrZ		 ; addr. of Z component	array

FLD	   QWORD PTR ES:[SI+BX]	 ; Z component current vector

LES	   SI, AddrY		 ; addr. of Y component	array

FLD	   QWORD PTR ES:[SI+BX]	 ; Y component current vector

LES	   SI, AddrX		 ; addr. of X component	array

FLD	   QWORD PTR ES:[SI+BX]	 ; X component current vector

F4X4				 ; mul 4x4 matrix by 4x1 vector

INC	   AX			 ; next	vector

MOV	   DI, AX		 ; next	vector

SHL	   DI, CL		 ; offset of vector into arrays

FSTP	   QWORD PTR ES:[SI+BX]	 ; store X comp. of curr. vect.

LES	   SI, AddrY		 ; address of Y	component array

FSTP	   QWORD PTR ES:[SI+BX]	 ; store Y comp. of curr. vect.

LES	   SI, AddrZ		 ; address of Z	component array

FSTP	   QWORD PTR ES:[SI+BX]	 ; store Z comp. of curr. vect.

LES	   SI, AddrW		 ; address of W	component array

FSTP	   QWORD PTR ES:[SI+BX]	 ; store W comp. of curr. vect.

MOV	   BX, DI		 ; ofs nxt vect. in comp. arrays

CMP	   AX, DX		 ; nxt vector past upper bound?

JLE	   $mat_loop		 ; no, transform next vector

FLDCW   [Ctrl87]		 ; restore orig	80x87 ctrl word

ADD	    SP,	2		 ; get rid of local variable

POP	    DS			 ; restore TP data segment

POP	    BP			 ; restore TP frame pointer

RET	    24			 ; pop parameters and return

IIT_MUL_4x4   ENDP

CODE	   ENDS

END

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

{$N+,E+}

PROGRAM Trnsform;

USES Time;

CONST VectorLen = 8190;

TYPE  Vector	 = ARRAY [0..VectorLen]	OF DOUBLE;

VectorPtr = ^Vector;

Mat4	 = ARRAY [1..4,	1..4] OF DOUBLE;

VAR   X: VectorPtr;

Y: VectorPtr;

Z: VectorPtr;

W: VectorPtr;

T: Mat4;

K: INTEGER;

L: INTEGER;

First: INTEGER;

Last:  INTEGER;

Start: LONGINT;

Elapsed:LONGINT;

PROCEDURE MUL_4X4     (X, Y, Z, W: VectorPtr;

VAR T: Mat4; First, Last: INTEGER); EXTERNAL;

PROCEDURE IIT_MUL_4X4 (X, Y, Z, W: VectorPtr;

VAR T: Mat4; First, Last: INTEGER); EXTERNAL;

{$L M4X4.OBJ}

BEGIN

WriteLn ('Test8087 = ', Test8087);

New	(X);

New	(Y);

New	(Z);

New	(W);

FOR	L := 1 TO VectorLen DO BEGIN

X^ [L] := Random;

Y^ [L] := Random;

Z^ [L] := Random;

W^ [L] := Random;

END;

X^ [0] := 1;

Y^ [0] := 1;

Z^ [0] := 1;

W^ [0] := 1;

FOR	K := 1 TO 4 DO BEGIN

FOR L :=	1 TO 4 DO BEGIN

T [K,	L] := (K-1)*4 +	L;

END;

END;

First := 0;

Last  := 8190;

Start := Clock;

MUL_4X4 (X,	Y, Z, W, T, First, Last);

{ IIT_MUL_4X4 (X, Y, Z, W, T, First, Last);	}

Elapsed := Clock - Start;

WriteLn ('Number of vectors: ', Last-First+1);

WriteLn ('Time: ', Elapsed,	' ms');

WriteLn ('Equivalent to ', (28.0*(Last-First+1)/1e6)/

(Elapsed*1e-3):0:4, ' MFLOPS');

WriteLn;

WriteLn ('Last vector:');

WriteLn;

WriteLn (X^[Last]);

WriteLn (Y^[Last]);

WriteLn (Z^[Last]);

WriteLn (W^[Last]);

END.