INTRODUCTION

This disk contains a limited version of GASFLO, along with

instructions and sample problems. This version is limited

only in the number of pipe sizes (4 inch, 12 inch, 20 inch)

available for use; all other features of GASFLO are included.

GASFLO is a program to assist the Engineer in the sizing

and operational analysis of buried natural gas pipelines. In

addition to calculating flow, pressure drop, pipe size, and

efficiency GASFLO estimates the temperature profile of the

pipeline. The temperature is used interactively when calculating

flow, pressure, pipe size, and efficiency.

GASFLO is menu driven, has screen display of input and

results, and will print a full report.

Please drop me a line if you have problems running

GASFLO or have comments on the program.

If you find GASFLO useful, the full featured version (pipe

size 4 inch through 48 inch) is available for $100, and

includes a complete user guide and free telephone support.

ORDER FORM

NAME___________________________________DATE____________________

COMPANY________________________________PHONE___________________

STREET_________________________________________________________

CITY_____________________________STATE_____________ZIP_________

Specify 5.25" or 3.5" disks

QUANTITY________________@$100.00=______________________________

Texas Residents Add    Sales Tax ______________________________

TOTAL_________________

Make checks and money orders payable to and mail to:

G. E. STEWART

25303 Pepper Ridge

Spring, TX 77373

(713) 288-4982

#

INSTRUCTIONS

GASFLO is a basic language program written with GW-BASIC,

ver 3.20, (c) Copyright Microsoft 1983,1984,1985,1986.

GASFLO uses a rational, isothermal flow equation and

calculates the friction factor with the Colebrook-White algorithm.

The program is divided into a pipeline DESIGN section and

a pipeline ANALYSIS section. The DESIGN section solves for (1)

flow rate, or (2) outlet pressure, or (3) required pipe size.

The ANALYSIS section solves for (1) pipeline efficiency and (2)

soil thermal conductivity from data taken from an operating

pipeline.

To run the design section, choose the DESIGN option from

the MAIN MENU. Input data as prompted, pressing ENTER to pass

over requests for data to be calculated. ELEVATION CHANGE is

entered as + or -. Outlet above inlet is a (+) change. Default

values for SOIL CONDUCTIVITY, PIPELINE EFFICIENCY, TEMPERATURE

BASE, and PRESSURE BASE are shown and can be accepted or revised.

At the end of input you will be given an opportunity to edit

the data.

To edit, press ENTER to accept value shown or input

revised data. There is one exception; when editing ELEVATION

CHANGE a value must be entered each time. The indicated value

cannot be accepted by pressing ENTER.

At the completion of input a calculation menu appears.

Choose calculation desired. At completion of calculation the

input and calculated data are displayed, along with a menu of

options. The velocity indicated is at the outlet of the

pipeline.

To run the analysis section choose the ANALYSIS option

from the MAIN MENU. Input data as described above in the DESIGN

section and all prompts (except ELEVATION CHANGE) require input.

The PIPELINE EFFICIENCY is calculated using the pressure, flow

rate, gas temperatures from an operating pipeline and the SOIL

CONDUCTIVITY is calculated using the same flow, pressure, gas

temperature data along with ground temperature.

EXAMPLE NO.1-DESIGN

Calculate the flow expected through a 4.062 inch inside

diameter pipeline two miles long. Inlet pressure is 80 psi,

outlet pressure 45 psi. Inlet temperature is 100 deg F, with

out any elevation difference between inlet and outlet.The

pressure base is 14.7 psia, temperature base is 520 deg R,

pipeline efficiency is .94, soil thermal conductivity is

.8, pipe cover is 36 inches, and ground temperature is 58 deg

F. Gas specific gravity is .75, specific heat is .6, viscosity

is .013 centipoise and the Joules-Thompson coefficient is 69

deg F per 1000 psi.

GASFLO calculates a flow rate of 1.7 MMSCFD, maximum

velocity of 52.2 feet per second, outlet gas temperature of

57.7 deg F and an average compressibility of .9774.

For a copy of this problem and solution select PRINTOUT

in the display menu.

#

EXAMPLE NO.2-DESIGN

Calculate the outlet pressure of a 19.25 inch inside

diameter pipeline 75 miles long.Inlet pressure is 1400 psi

and the flow is 235 MMSCFD. Inlet temperature is 120 deg F,

with the line outlet 5000 feet above the inlet. The pressure

base is 14.7, temperature base is 520 deg R, pipeline

efficiency is .94,soil conductivity is .8,pipe cover is 36

inches, and ground temperature is 58 deg F. Gas specific

gravity is .6, viscosity is .013 centipoise and the Joules-

Thompson coefficient is 65 deg F per 1000 psi.

GASFLO calculates an outlet pressure of 963.2 psi,maximum

velocity of 16.6 feet per second, outlet gas temperature of

49 deg F and an average compressibility of .844.

For a copy of this problem and solution select PRINTOUT

in the display menu.

EXAMPLE NO.3-DESIGN

Calculate the minimum pipe inside diameter required for

a 75 mile long pipeline flowing 260 MMSCFD. Inlet pressure is

1400 psi, outlet pressure is 800 psi. All other data is same

as EXAMPLE NO.2.

GASFLO calculates a minimum diameter of 18.14 inches.

Select an actual pipe inside diameter larger than the

calculated minimum.

Calculate flow using the pipe diameter selected and the

specified inlet and outlet pressures.

GASFLO calculates a flow of 306.5 MMSCFD, maximum velocity

of 26.7 feet per second, outlet gas temperature of 48.8 deg F

and an average compressibility of .8514.

For a copy of this problem and solution select PRINTOUT

in the display menu.

EXAMPLE N0.4-ANALYSIS

Calculate pipeline efficiency and soil conductivity for

a 12 inch inside diameter 25 mile long pipeline flowing

166.6 MMSCFD. Inlet pressure is 1400 psi,outlet pressure is

550 psi. Inlet temperature is 120 deg F, outlet temperature

is 49.9 deg F, with the line outlet 500 feet lower than the

inlet. The pressure base is 14.7, temperature base 520 deg

R, pipe cover is 36 inches, and ground temperature is 58 deg

F. Gas specific gravity is .65, specific heat is .6,

viscosity is .013 centipoise and the Joules-Thompson

coefficient is 60 deg F per 1000 psi.

GASFLO calculates a pipeline efficiency of .93 and a

soil conductivity of .6, maximum velocity of 55.2 feet per

second, and an average compressibility of .8358.

For a copy of this problem and solution select PRINTOUT

in the display menu.