FAX483 480 LINE FACSIMILE PROGRAM

Dr. Ralph E. Taggart WB8DQT

602 S. Jefferson

Mason, MI  48854

COPYRIGHT NOTICE

The  FAX483 program is copyrighted by Dr. Ralph E.  Taggart.

You are free to make copies of this software for your own use and

to distribute copies to others, provided that no charge is levied

for such distribution. ALL OTHER RIGHTS ARE RESERVED. You may not

incorporate  any  elements of this program  into  other  programs

offered  for  commercial sale without written permission  of  the

author.

INTRODUCTION

Since  the  introduction of the 480 line  amateur  facsimile

standard  in  the  February 1993 issue of  QST,  there  were  two

additional  upgrades  (distributed as FAX481 and FAX482)  to  the

original  program  code.  These  upgrades  improved  the  overall

operation  of the program and added display of 120 lpm HF  WEFAX.

The  FAX480  mode has proved to be very popular among  those  who

have tried it and I was approached to add similar capabilities to

the Pasokon SSTV interface. As part of that effort, I elected  to

create a single program structure to support both the Pasokon and

ViewPort  interfaces. The FAX483 program is the ViewPort  version

while FAX480P will be the Pasokon version.

Creating  the common software skeleton gave me an excuse  to

upgrade several aspects of the program. If you have never used  a

previous  version of FAX480, the changes are not relevant to  you

and  you  can  move  on in the  documentation.  What  follows  is

intended  as an outline of the major changes for the  benefit  of

previous users of the program.

ON THE AIR

On-the-air    use    by   an  ever-increasing    number   of

individuals   has  proved  VERY gratifying.  In addition  to  the

enthusiasm  for the more  detailed images   (everyone   considers

them  MOST impressive), the  FAX  480 mode has proved to be  VERY

resistant  to  the effects of noise  and QRM.   Stations  running

barefoot  with simple antennas are  getting  excellent   results.

Situations ranging from mediocre to poor  for conventional   SSTV

are  yielding  excellent fax images.  You  don't have  to   be  a

"big gun" to get good results.  In  addition,  the images hold up

very  well  when relayed from station to station.  In  short,  it

looks like the mode will be a winner.

DISTRIBUTION FILES

The   following  files  are  included  in  the   FAX483.ZIP

distribution file:

FAX483.EXE - the operating program

FAX483.DOC - this documentation file

HOUSE.480 - a sample fax image file

CONVERT.EXE  - a file conversion program that  converts  old

image  files  (ie. image.a and image.b) to  the  new  single-file

format (image.480).

K4OVV.STV - a sample SSTV image file

VGACAP.COM  - the Gozum SHAREWARE program for  image  screen

capture.

HARDWARE REQUIREMENTS

The  following hardware requirements must be  satisfied  for

proper operation of the FAX483 program:

(1)  An  IBM PC/AT/PS1/PS2 or compatible with a  10  MHz  or

faster clock speed and a minimum of 640K of RAM.

(2)  A VGA display adapter that is bios-compatible  with

the  IBM VGA standard. Virtually all VGA adapters  meet

this requirement.

(3) A monochrome or color analog VGA monitor.

(4)  A minimum of two 360K floppy disk drives, or  a  single

1.2  meg 5.25  or 1.4 meg 3.5 inch floppy drive,  or  a

hard  drive. The choice of drive options has no  effect

on  the  operation of the program but does  impact  the

number of fax or SSTV images that can be stored as well

as  the speed with which images can be saved or  loaded

from  disk. All things being equal, you will enjoy  the

program more if you have a hard-drive (a statement true

for  most software), but, if you lack one, there  won't

be any limitations on what you can do.

(5) An A&A Engineering ViewPort VGA SSTV interface, modified

by  the addition of a 4 MHz clock circuit as  described

in the February, 1993 issue of QST.

(6) An available parallel printer port.

SOFTWARE LIMITATIONS

On  slower computers or those with lethargic VGA cards,  the

system  has  to  run flat out to keep up  with  the  image  data.

ANYTHING  that  slows  down  such a  computer  may  result  in  a

situation where the system falls behind on receive, resulting  in

a  "tilted"  image,  even  if the interface  clock  is  right  on

frequency.  There  are  lots of things  than  can  bog-down  your

system. I know of at least one case where one station with a  486

was  unable  to keep up with the data while other  stations  were

doing  fine  on  a  10  MHz  286!  A  poorly-implemented  WINDOWS

installation can slow your system. Properly implemented PIF files

can  allow  WINDOWS and FAX480 to coexist  without  impeding  the

operation  of the latter. Some TSR programs may also steal  clock

cycles  and  result  in  unpredictable  problems.  If  you   have

problems,  try operating the program under DOS with no  TSRs  and

see  how  things  work.  On a very  complex  system,  it  may  be

difficult to avoid some of these "enhancements", since things can

be  going  on  in  your system even though  you  think  you  have

programs like WINDOWS completely disabled.

If you have unexplained difficulties, try preparing a floppy

boot  disk for your A drive that will bring you system  up  under

DOS with no TSRs or other operating enhancements and see how  the

program works. The present release does not make any great memory

demands on the system since the screen memory actually serves  as

the  image  buffer.  There  is  no  need  for  elaborate   memory

management  and  there is minimal chance that  the  program  will

conflict  with your system in terms of resources. The reverse  is

not necessarily so, however.

INSTALLATION

Hardware

Installation  is a simple matter of connecting the  parallel

port cable between the interface and your computer and making the

needed  audio  connections.  A shielded  audio  cable  should  be

connected between the receiver audio output and the SSTV IN  jack

on  the rear of the interface. A microphone is connected  to  the

MICROPHONE IN connector on the front panel while a cable connects

between  the MICROPHONE OUT connector on the rear apron and  your

transceiver's  microphone  input connector.  The  two  microphone

connectors on the interface have the the following pin-outs:

Pin       Function

1        Microphone audio

2        PTT "hot"

3        PTT return/ground

4        Microphone ground

A  connector  compatible with your transceiver  input  should  be

installed  at  the  transceiver  end of  the  cable  between  the

interface  and  the  rig.  Consult  the  documentation  for  your

transceiver  for  the  equivalent  pins  on  your   transceiver's

microphone  connector. You can test the connections by  operating

your rig with the interface either on or off. In either case, the

rig  should  key  with normal microphone  audio.  The  microphone

patches  through the interface at all times unless the  interface

is actually transmitting an image.

The cable used to connect the parallel port of the  computer

with the DB-25 connector on the rear apron of the interface is  a

double-male  and  is  usually sold as a  "parallel  cable".  Such

cables  will  have all connector  pins  wired.  Similar-appearing

cable, usually marked "serial cable", will not be fully-wired and

will NOT function with the interface.

Software

If you have a hard drive, the best results will be  obtained

by  operating  the software from the hard disk.  From  your  root

directory, create a FAX subdirectory by typing:

md\fax  <ENTER>

Switch to the new directory:

cd\fax  <ENTER>

Now   copy   ALL  the distribution files into   this   directory.

Assuming the files were on a disk in the A drive, you would type:

copy a:*.* <ENTER>

In operation, you would enter the FAX subdirectory to  execute

the  program  and  all  image  files  would  be  stored  in   the

same  subdirectory.   Image  file capacity is  limited  only   by

available hard-disk storage.

The  FAX483  program  can be operated from  a  floppy  drive

without any penalty other than the number of file images that can

be stored on a single floppy disk. Copy the programs noted  above

on to a formatted disk for the drive in question. Operating  from

a  floppy drive would normally involve booting the software  from

your  program disk, after which you can remove the  program  disk

and insert formatted disks for image file storage. Each fax image

file  requires  128K of available disk storage while  SSTV  image

files use 64K, so you will have the following image capacity  for

various floppy drive configurations:

Drive     Fax Images     SSTV Images

360K           2             5

720K           5            11

1.2 meg         9            18

1.44 meg       11            22

These figures assume that no other files are stored on the  disks

in question.

MAJOR CHANGES FROM PREVIOUS RELEASES

The  following  is a summary of the changes in  the  program

from  previous versions (FAX480-FAX482). If you have  never  used

the  FAX480  program before, simply move ahead to  the  OPERATION

section.  The next two sections are intended for previous  FAX480

users.

(1)  Simplified image file structure. In previous  releases,

each fax image was broken down into two 64K files. In the current

release,  images are stored as single 128K files. This  makes  it

much easier to keep track of your images and maintain your  image

files.  This does mean however that previous users will  have  to

convert their "two-piece" image files for use in the new version.

A program, CONVERT.EXE, is included in the distribution .ZIP file

to  accomplish  this  and it is very  quick.  See  the  following

section  on IMAGE FILE CONVERSION before you attempt to  use  the

software for the first time.

(2)  Faster  screen  writes. When loading  or  recalling  an

image, it could take up to 30-40 seconds to "paint" the screen if

you had a slow CPU or VGA card. Those operations now take only 2-

3 seconds with the new software.

(3)  Elimination of the printing option. It seems  that  few

people were using this option so I dropped it from this  release.

If you really want it, I will incorporate it into a utility  that

will print directly from your image files.

(4)  Major  improvement  of  the  307K  screen-capture  file

conversion. This option is now called directly from the Main Menu

and  posts  the image to the screen as  conversion  proceeds.  In

addition to providing visual feedback, the routine now takes just

30  seconds on my 386 system instead of the 5+ minutes that  used

to be required! It also incorporates complete control over  which

portion of the original 640x480 image you include in the  512x480

fax image file.

(5)  Simplify  the  Calibration Tone  option  on  fax.  This

function  now results in the automatic transmission of 5  seconds

of black tone (1500 Hz), making it more convenient to use.  Since

it  only takes about 2 seconds to recall an image, I expect  that

more  stations  will be making use of the  Tuning  Indicator  and

Calibration  Tone options. Operation of this option on  SSTV  has

not changed, since other stations vary in terms of the  alignment

tones they may need for tuning.

(6)  Fast (a few seconds) on-screen contrast enhancement  of

displayed  fax  images. If you like the result, you can  save  it

using the original file name (thus replacing the original  file),

or you can save it to a new name. Of course you can quit  without

making any changes or saves if you desire.

(6)  Fast  FAX to SSTV conversion. Now that I  have  greatly

speeded up the conversion of screen capture files to FAX, the FAX

image files are now treated as the primary image files. The  SSTV

file conversion option now consists of a routine that quickly (10

seconds or so) creates an SSTV file from a FAX image file.  Given

the speed of conversion, interesting FAX images can be  converted

on  the  fly  for  re-transmission on SSTV  for  the  benefit  of

stations that do not have fax capability. This should  contribute

to peaceful coexistence.

New  Users can bypass the next section but those of you  who

have used previous versions should read the next section prior to

using the program for the first time.

CONVERTING IMAGE FILES

A  program to convert the two-piece image files of  previous

releases  into the single image file used by the new  program  is

included  in  the  distribution  .ZIP  file  (CONVERT.EXE).  This

program  converts the original image files and then deletes  them

once  conversion  is  complete.  Since  you  probably  are   very

protective of your previous image files, I would suggest that you

back them up to a floppy prior to conversion. In this way, in the

unlikely  event  that  you don't like the new  program,  you  can

recover the files in their original format. Alternatively, if you

have  set  up  a  FAX  directory,  as  indicated  under  software

installation,  you can copy image files you want to convert  from

your old FAX480 directory. You can also copy over any SSTV images

you  wish. In this way, all your old setup is retained until  you

decide you want to stay with the new program.

Assuming  that  CONVERT.EXE is in  your  working  directory,

simply boot the program:

convert <ENTER>

You  will  see a menu of the available image files  and  you

will  be  requested  to input the file you wish  to  convert  (no

extension).  At that point the program will display the  familiar

FAX480  display screen and the file will be loaded and the  image

will scroll down the screen in the normal fashion. When the image

has been displayed, it will quickly (a few seconds) be  converted

to the new format, the screen will clear, and you have the chance

to  convert  another file. You will note that the file  you  have

just converted is now missing from the directory listing. To quit

the  conversion program, simply input <Q> in place of a  filename

and you will be returned to DOS.

When you have converted the last file in the directory,  you

will be unceremoniously dropped back to DOS via an error message.

I  didn't  think  that an elaborate error  handling  routine  was

required for a one-time event!

If you  now type:

dir *.480 <ENTER>

you  will see all your old files listed, but they are now  single

128K (131,000+ bytes) files with a *.480 extension.

OPERATION

Booting the Program

Normal  operation  would  involve switching  to  the  FAX480

directory, at which point you would boot the software:

fax483 <ENTER>

If you want to use the call-insertion function provided with SSTV

transmission,  you boot by inserting your call after the  program

name:

fax483 urcall <ENTER>

It doesn't matter if you enter your call in upper or lower case -

it will be converted to upper-case for storage.

To  make  booting the software even move convenient  in  the

future, you might consider using your word processor (in the non-

document  or  ASCI  mode)  to create a batch file  (FAX.BAT)   in

your  root directory. This file would have the following format:

cd\fax480

fax483 urcall

cd\

with the batch file installed, simply typing:

fax <ENTER>

from  the root directory and you will move automatically  to  the

FAX480  subdirectory and boot the software with your  call.  When

you exit the program, you will be back in the root directory.

As the program boots, a "CHECKING PARALLEL PORTS..." message

will be posted as the computer checks the three possible parallel

port  addresses  looking for the interface.  The  interface  unit

should be connected and turned on PRIOR to booting the  software.

If the program doesn't find the interface unit, an error  message

will be posted, outlining the possible problems. You can continue

at  this point (type <C>) or exit the program (type <Q>). If  you

continue,  all image file functions will be available to you  but

the  program will not let you use the receive modes  or  transmit

modes, since,  lacking a connection to the interface, the program

would "hang up" and require a keyboard or hard reset to get  back

on  line. If the problem was a lack of power to the interface  or

failure  to  connect or switch the parallel  port  cable,  simply

hit  the <T> key to repeat the port test. If you don't intend  to

use the interface for a particular operating session, hit <C>  to

continue.  Almost all program functions will be available to  you

in  such a case, except that the system will not let you  display

or transmit images. Typing <Q> will cause you to exit the program

if your problem requires more in the way of thought or action.

MAIN MENU

This  menu  is  the crossroads of  the  FAX483  program  and

gives you access to all operating functions.

Fax   functions   are   organized  into  three   columns   -

CRYSTAL-LOCKED,  LINE  TRIGGERED,  and  MISC.  OPTIONS  and  this

organization  will  be followed in  the  functional  descriptions

which follow. Also note that a 16-step grayscale is posted at the

bottom  of  the  display  screen.  The  brightness  and  contrast

controls  of your monitor should be adjusted to  clearly  resolve

each of the steps.

The Main Menu incorporates a screen-saver routine that  will

blank  the  display after 5 minutes with  no  keyboard  activity.

Hitting  ANY key on the keyboard will instantly restore the  menu

display.

CRYSTAL-LOCKED Options

These  options represent the preferred display  modes  since

they  use  the crystal-clock reference that gives  the  mode  its

extreme noise immunity. Your options here are as follows:

<1> FAX480 Auto-start

This is the preferred receiving mode since image loading  is

initiated  by the start tone and controlled by the crystal  clock

circuit,  providing the maximum in noise immunity. This  mode  is

used to display transmissions from other stations where the image

is  transmitted directly from the other computer  ("live").

Although  you can initiate display at any time prior to  the

start of image transmission, the best results will be obtained if

you  key <1> as soon as you hear the distinctive 5  second  start

tone.  When you  key  <1>  the image display screen  will  appear

and   the   ** ACTIVE ** message will be posted to let  you  know

that  the   system  is attempting to load  an  image.  The  start

routine  is  EXTREMELY  reliable,  even  under  VERY  poor   band

conditions.  BE  PATIENT as nothing will happen  immediately.  In

order  to reserve the screen for useful image display, the  auto-

start  routine bypasses the 5 second start tone and most of th  5

second phasing interval that follows before it begins to display.

Do NOT expect to see anything for about 8 seconds after you  have

keyed <1>!! If you have problems with auto-start, it is  probably

a result of mis-tuning on the station you wish to receive.  Check

out  the  <T>une Display option under MISC. OPTIONS  for  how  to

assure that you are accurately tuned on receive.

If  you entered the mode by accident, you missed  the  start

tone,  or  there  was a problem with  the  transmitting  station,

simply hit the <A> key to exit. The ** ACTIVE ** message will  be

replaced  by  ** COMMAND **, at which point you can  key  <Q>  to

return  to  the Main Menu. See <3> below for how  to  display  an

image when you have missed the image start tone.

Assuming image transmission proceeds normally, the  received

picture will be displayed, line by line, in the 512 x 480 display

window.  Reception can be aborted at any time by hitting the  <A>

key, at which point the ** ACTIVE ** message will be replaced  by

** COMMAND ** and you can use <Q> to exit back to the Main Menu.

Assuming  the transmission proceeds to completion, when  the

display  window  is  filled with image data,  the  **  ACTIVE  **

message  will  be replaced by ** COMMAND **. At this  point,  the

received  image  will be retained and you have three options,  as

indicated by the menu on the left side of the screen:

<S>ave Image

<T>ransmit

<C>ontrast

<Q>uit

See the FAX POST-DISPLAY options for the use of these functions.

<3> FAX480 Manual Start

If you missed the start tone for any reason, or the  display

did  not auto-start, you can display the image by keying <3>.  In

all  other  respects, the option is identical  to  the  preferred

auto-start routine.

<5> 120 LPM HF WEFAX

This  is  the  120 line/minute  HF  WEFAX  display  routine.

Display  will  require that  you be accurately tuned to  the  fax

station,   a  requirement which  is  complicated  by   the   fact

that   there  is  no  voice transmission to use as a  guide.  See

the discussion on the use  of the  tuning indicator for advice on

how to tune a fax signal.  To start  display,  key <5> AFTER  the

distinctive image start tone. Once display is complete, you  have

the  same  options  available   as for   the   480  line  receive

options. If you  select  the  TRANSMIT option,  the image will be

transmitted in the 480 line  format  - NOT 120 lpm.

LINE-TRIGGERED Options

The ONLY reason for use of this option is to display  images

that  you have recorded on audio tape. These options do NOT  have

the noise immunity of the crystal-locked options since each  line

is triggered by a received sync pulse, which can be disrupted  by

noise or interference.

<2> FAX480 Auto-start

The operation of this option is identical to that  described

for FAX480 Auto-start under the CRYSTAL-LOCKED options.

<4> FAX480 Manual Start

This option functions identically to the FAX480 Manual Start

option under the CRYSTAL-LOCKED options.

FAX POST-DISPLAY OPTIONS

Any  of  the  fax display options  (1-5)  will  provide  the

following options when image display is complete:

<S>ave Image

<T>ransmit

<C>ontrast

<Q>uit

<S>ave Image

Keying  <S> will switch you will be prompted to provide  the

name  to be used for the image save. This name can be  any  valid

DOS  filename (8 characters max.) and should not incorporate  any

extensions. If you are using a floppy drive, be sure a disk is in

the  drive with enough capacity to hold the 128K image.  Normally

the file will be created in the default drive/directory. You  can

save to another drive/directory by including the appropriate path

data  in the file name you provide. NOTE: Files save  to  another

drive/directory  will NOT appear on the listing when you  attempt

to load images from the fax menu. If you want access to the image

you have saved, the file must be transferred at a later time. See

the  IMAGE  FILES section before attempting to do so.  The  image

remains posted during and after the save operation (it takes just

a  few seconds). If you want to return to the Main Menu  after  a

save, use the <Q> option described below.

<T>ransmit

Hitting the <T> key will key the interface  and begin  image

transmission. When you use this option for the  first time,   you

will   need to adjust the MIKE  GAIN  control  on  the  interface

for  at RF output reading on your transceiver  that   is  between

1/2  and 2/3 of the PEAK reading normally  observed   with  voice

transmission.   Keeping  the  audio  level   down   will   reduce

distortion  and the possibility of over-driving the  transmitter.

It is also important since fax transmission (like SSTV)  involves

a  100%  duty-cycle and you do not want to  overheat  your  final

amplifier  stage(s).  Using  a  dummy  load  to  make  the   gain

adjustment  is highly recommended. The setting of the  MIKE  GAIN

control  has  no  effect on normal  microphone  audio,  which  is

normally  set by a level or gain control on your transceiver.  If

you  change this control, readjustment of the MIKE  GAIN  control

will probably be required.

Transmission  can be aborted at any time by hitting the  <A>

key,  at which point the interface will drop out of the  transmit

mode and the ** ACTIVE ** prompt will be replaced by **COMMAND**.

When   transmission  is  completed  (normally  about   2  minutes

and  18   seconds,  the interface will drop out of  the  transmit

mode and ** ACTIVE ** will be replaced by **COMMAND**.

<C>ontrast

Sometimes  you  will encounter situations  where  your  file

image  is  less  than optimum in terms of  contrast.  Some  hand-

scanner  files  have  this  problem,  some  color  to   grayscale

conversions  are a bit soft, or you may receive a  lower-contrast

image  due to mis-tuning or less than optimum image quality  from

the transmitting station. Keying <C> will result in a simple  and

fast on-screen contrast enhancement. If you like the result,  you

can save the image (<S>) to the old file name (thus replacing the

original) or to a new name.

If you wish to use this option on a received image, save the

image first, and then do the enhancement.

<Q>uit

Keying  <Q>  will result in a return to the Main  Menu.  The

current displayed image is saved to a temporary disk file and can

be  restored  in about 2 seconds using the  <R>ecall  Last  Image

option from the Main Menu.

MISC. OPTIONS

<L>oad from Disk

When  you key <L> to invoke this function, the names of  all

FAX483   image  files  on  the  default   directory/drive   will

be displayed.  Each  of these will have an ".480" extension  (see

the  section  on FAX IMAGE FILES). Enter the name of the file  to

load  (less   the  .480 extension) and the  image  file  will  be

loaded,   the system  will  switch to the image  display  screen,

and  the  image will be transferred to the screen, a process that

takes  about 3 seconds. Any of the secondary  functions  (<S>ave,

<T>ransmit, or <Q>uit can be used if  desired. Note  that you can

save   the  image  to  the  default   directory/drive  under    a

different    file   name   or   you   can   save   to     another

directory/drive  with  the save or a different  file  name.  This

provides a convenient way to copy interesting images to a  floppy

for  backup  purposes or to another disk to  provide  to  another

fax/SSTV operator.

<R>ecall Last Image

If you have previously received an image or loaded one  from

disk,   that  image will be posted to the active window  of   the

display  screen.  At  that  point,  you  have  the  same  options

available to you  as described under the receive modes.

If  you  have  not loaded or received  an  image  since  the

program  was booted, a black screen will be displayed.  IMPORTANT

NOTE:  Although  unlikely, if you do something  that  causes  the

program   to  abort,  the  temporary disk  buffer  file  will  be

retained and you can recall the image when you re-boot.

<C>alibration Tone

Keying  <C> from the Main Menu will cause the  interface  to

send a 5 second 1500 Hz (black) calibration tone, at which  point

you  will  be returned to the Main Menu. You would  normally  use

this  option if the other station requests a calibration tone  to

assist them in accurately tuning your signal.

<T>une Display

Proper     grayscale     rendition     requires     accurate

tuning/carrier  insertion   at  your  receiver. Usually  you  can

achieve sufficient accuracy by tuning for natural voice  quality,

but   this  can  be  ambiguous  at   times.   The   tune  display

provides    a   graphic   aid   for   precision   tuning.     Two

vertical, parallel lines define the range from 1200 Hz (Sync)  at

the  top of the display to 2300 Hz (White) at the bottom. If  you

are uncertain about the accuracy of your tuning, request that the

transmitting station transmit a 1500 Hz (Black) calibration  tone

prior  to sending an image. Key <T> for the tone display and  you

will  note  a  horizontal line between the  two  vertical  lines.

Carefully adjust the receiver tuning (use the RIT control in  the

case of a transceiver) until the horizontal line is lined up with

the two arrows. These arrows are positioned at the 1500 Hz  point

on the frequency display. When the line is properly aligned  with

the arrows, your are precisely tuned for image reception.

WEFAX   tuning  presents a special problem as there  is   no

voice  signal to use, nor are sync pulses transmitted. Your  best

option  is  to wait for a start tone - this  should  produce  two

horizontal  bars   on  the  tune display.  Carefully  tune   (you

have   about  5 seconds) to place one bar at black and the  other

at white.

No  matter what your tuning mode, hitting the <Q>  key  will

return you to the fax menu.

<F>ile Conversion

The file conversion option provides a means to convert  307K

screen-capture  files  to  the disk/screen  format  used  by  the

FAX483  program.  While the conversion routine itself  is  quite

simple,  it  is  part of the larger  question  about  sources  of

images,  image  file formats and the like. For this  reason,  the

operation  of the file conversion option is deferred to  a  wider

discussion   of  image  file  conversion  at  the  end   of   the

documentation.

<S>STV

Keying <S> will route you to the SSTV menu.

<Q>uit FAX480

Keying  <Q> will result in an exit to your operating  system

(typically  DOS).  All  temporary files are  erased  on  exit  to

conserve disk space. If you have previously displayed a fax image

that  you  want to keep, be sure to save it prior to exit  or  it

will be lost.

FAX IMAGE FILES

All FAX480 images are stores as 128K disk files with a *.480

extension.  These  files can be erased, copied, or  renamed  just

like any other DOS file. If you rename a file, be sure to include

the  *.480  extension or the file will NOT be  displayed  on  the

image  file directory when you go to load images. Images save  to

another directory can be copied to the FAX480 directory, at which

point they are displayed on the image directory. You may wish  to

copy  over-the-air images to another directory (or  to  floppies)

for long-term storage, just to reduce the "clutter" on the  image

directory display when you go to load images.

FILE CONVERSION MENU

Keying   <F>   from  the  Main Menu  will  invoke  the  file

conversion  option.  Some  discussion of  screen  image  capture,

sources  of  images, and image formats is  appropriate  prior  to

looking at how you perform the image conversion.

SCREEN IMAGE CAPTURE

The  key  to the conversion option is  the  ability  to

capture  computer  images from various sources into  a  form  the

conversion  routine can manipulate. This is accomplished with  a

family of programs called "screen capture" software. As noted  in

the  original  article on the fax standard, I prefer  the  VGACAP

shareware  program,  which I have included  on  the  distribution

disk.  This  is  a TSR program that you boot  from  its  resident

directory:

vgacap <ENTER>

Aside  from a short (and transient notice), you then  proceed  to

run your applications software in the normal manner. VGACAP  sits

in the background until you have an  image  displayed,  at  which

point  you  invoke  the "capture" by keying  <Alt-F4).  At   this

point,  VGACAP takes over and converts the screen image  (without

altering it) to a binary data file. If the image was in the 640 x

480  x  16 format (suitable for fax), the result will be  a  307K

binary  file containing all the data for the screen  image.  This

file will reside in the current directory and will be named:

screenxx.raw

where  xx  is  a number from 00 to 99, depending  upon  how  many

screen   captures   you have executed since booting   VGACAP  and

whether there are other screenxx.raw files left on the disk  from

previous conversions. There will also be a matching palette  file

(screenxx.plt),  which  we  do need.   Other    capture  programs

will   operate   in   a   similar manner,  but  I  would  suggest

the  use  of   VGACAP   for   the   most  predictable    results.

VGACAP.COM  is now bundled  in  the  .ZIP distribution file. This

program  is  SHAREWARE and you should  read the  text  associated

with  it.  The  file  conversion  routine  converts  these   307K

screenxx.raw  files to 480 line FAX483 files. In order  that  we

have  a  screen file that CAN be converted, we must  execute  the

capture  under very specific conditions, within the context of  a

viewing program.

IMAGE VIEWERS

Many  image  options (see below)  will  support  full-screen

display of images they create, but if this is not the case,  they

will  support conversion of the images to one of several  popular

formats  (.GIF,  .PCX, .TIF, .IMG, or .TGA). In such  cases,  you

need  a  viewing  program that will import the  image  files  and

provide  the  full-screen  display needed for  capture  as  noted

above. There are many programs that will do this, but one I  have

had  good success with is VUIMAGE, a shareware package  available

on  the DRIG BBS. This is an excellent program  supporting  .GIF,

.PCX, and .TIF display, a wide range of monitor modes, and a very

wide range of scaling options. Most of my images, from a  variety

of  sources,  are displayed using VUIMAGE to execute  the  actual

screen capture. I set the mode to 640x480x16 for high  resolution

images   I   intend  to capture as fax files.  By  adjusting  the

scale,  I  can get just the screen display I  wish   to  capture,

gaining  considerable control over the  dimensions  and  coverage

of the ultimate image file.

SOURCES OF IMAGES

Hand  scanners  and  flatbed scanners can also  be  used  to

generate images and most of these devices come with programs that

support the 640 by 480 x 16 VGA mode. Programs such as VGACAP can

be used to capture the screen images as described above.

TV  frame  grabbers  can  be  used  if  they  support   high

resolution  image  capture.  Most inexpensive  boards  will  only

capture  240-256  video  lines. In such cases,  it  makes  little

sense to transmit these as fax, since 240 line SSTV formats  will

match the  resolution  of  the  captured images. A high   quality

black   and  white   camera   will   typically   produce  sharper

images    than   a   comparable   color   camera.   These    high

resolution  frame grabbers usually support 640 x 480 VGA  display

so  programs  like VGACAP can capture the  images.

Other sources, such as paint programs, image libraries,  BBS

sources,  etc. can all provide images suitable for conversion  to

fax.

COLOR VS. MONOCHROME

Image conversion is quite predictable with monochrome images

and most unpredictable with color. B&W hand-scanners and TV image

capture boards generate such monochrome images automatically  and

present no problems when used with viewers such as VUIMAGE.

Color images must be converted to monochrome prior to screen

capture  is acceptable results are to be obtained.  Many  viewing

programs  allow  color images to be displayed with  a  monochrome

palette (VUIMAGE, despite its other useful qualities, does NOT!),

in  which  case you can screen grab with  good  results. Programs

such  as  Graphics Workshop can be used to  clip,  re-scale,  and

perform color to monochrome conversion. It the image is in a  640

x  480 format with 16 colors, programs such as PC Paintbrush  can

be used to convert color images to grayscale.

All of this sounds very complex to start with, but you can't

hurt  anything  by  experimenting.  Once  you  have  played  with

grabbers,  viewers,  and  images for a bit, it  will  all  become

second nature.

307K IMAGE CONVERSION

This  routine will convert the 307K screen binary  files  to

FAX483  fax image files. You will be prompted to  provide  three

items of information:

(1) The name of the file to convert

This is going to be a numbered screen file that should be entered

with NO extension:

screen06 <ENTER>

If  the screen file is not in the current directory, specify  the

path (using normal DOS conventions) as part of the filename:

\vuimage\screen06 <ENTER>

It  is  VERY important that the designated screen  file  actually

exist, for if it does not, the program will created a 0K file  of

that  name  and proceed to convert an all-black  image.  No  harm

done, but it wastes time to no good purpose. If you want to abort

the file conversion, enter a <Q> in place of the screen filename.

(2) Name of the fax file

This  can be any valid DOS filename (8 char. max.), entered  with

NO extension:

alison <ENTER>

The  routine will automatically append the .480 extension to  the

final  image file (ie. alison.480). Try to assure that your  name

is NOT identical to that of an existing file or the old file will

be  over-written with the new one. When in doubt, use  a  "dummy"

name  (like  <screen06>, for example, and rename the  file  later

under DOS.

(3) Starting pixel (0-128)

The converted file will represent a 512 x 480 subset of  the

original  640  x 480 screen image. The starting  pixel  selection

controls the position of the fax image, relative to the  original

screen display. Selecting <0> will result in the converted  image

shifted to the extreme left of the displayed image, omitting  the

last  128  pixels of every line. Set to <128>, the  fax  "window"

will  be  shifted to the extreme right, omitting  the  first  128

pixels  of  every  line. If set to <64>, the fax  image  will  be

centered on the display image.

Once  you have entered the required data, the  display  will

indicate  that the image palette file is being re-ordered,  after

which  (a  few  seconds) the monitor will  shift  to  the  normal

display  screen  and  the  image will  be  posted  as  conversion

proceeds. When display is complete (about 30 seconds or so on  my

386  system), the new image file will be written to disk and  you

can use the <Q> key to escape to the Main Menu, you can  transmit

the  image  (<T>), or you can save it to still  another  filename

(<S>).

At 307K per file, screenxx files eat up disk storage, so  it

is usually a good idea to erase the .raw and .plt files that  you

have successfully converted. This can be done individually:

delete screen06.*

which will simultaneous erase both files, or you can do a  global

deletion:

delete screen*.*

which  will  eliminate  ALL screen files (.raw and  plt)  in  the

current directory.

SSTV OPTIONS

I originally added the SSTV options simply as a convenience,

since  I felt that most ViewPort operators would be using  John's

software.  While  they do, particularly those with  32K  Hi-color

cards, I am amazed at the number of folks who are using the  SSTV

option  on a regular basis. In part, this is because the  program

is  so  easy to use, but it also is the result of the  fact  that

good  quality black and white pictures can be quite  superior  to

mediocre color, especially if you have a standard VGA display.

A  factor I had not considered is the fact that the  ability

to  send and receive SSTV easily while operating the fax  program

is  a real plus in that the SSTV guys on frequency don't  get  as

restless if there is the opportunity to work slow scan as well as

fax. The ability, with this version, to make fast conversions, in

real-time, between fax files and SSTV files, helps as well, as  a

received  fax  picture  (or one of your own  files)  can  be  re-

transmitted  on  SSTV  for  the  benefit  of  those  without  fax

capability.

RECEIVE FUNCTIONS

This column lists all the available slow-scan receive modes,

including Robot B&W and color modes, Scottie modes, and Amiga AVT

94. Each will briefly be described in the sections which  follow.

Prior  to this however, a few general notes about SSTV  reception

are  in  order. When you key ANY of the modes listed  below,  the

program will post the SSTV display screen and proceed to  display

the  incoming  picture. Since the SSTV display screen  has  lower

resolution  than the fax screen, the additional information  that

can be included is more limited.

For  example,  the fax receive modes post an  **  ACTIVE  **

prompt  when  displaying an image and a ** COMMAND **  prompt  to

indicate  that you can input commands. On SSTV, this function  is

achieved  with a white square that appears in the lower  left  of

the  screen.  When actively acquiring an image,  this  square  is

displayed  (equivalent to the ** ACTIVE ** prompt on  fax).  When

the  square  disappears (equivalent to ** COMMAND  **),  you  can

initiate one of the post-display options to be discussed shortly.

While the square is posted and image data is being  scrolled

onto the screen, the keyboard ignores ALL keyboard inputs  EXCEPT

the <A> key for the abort function. If you need to abort  display

for  any  reason, simply key <A> and the square  will  disappear,

allowing you to selection one of the post-display options.  These

will  be  discussed under SSTV POST-DISPLAY OPTIONS  in  a  later

section.

Proper  tuning is essential for reliable SSTV  display.  You

will also note that, except for the synchronous 36 second option,

all the SSTV options will be far less immune to noise than a  fax

station under the same conditions.

In all the modes that follow, you initiate display by simply

hitting  the indicated key. In most cases, best results  will  be

achieved if you use the key just after you hear the image  start.

Remember,  although  you can display images  transmitted  in  the

various  color  formats listed below, they will be  displayed  as

high-quality (64 grayscale values) grayscale images. If you  want

color,  use the ViewPort VGA. Note that, if you have  a  standard

VGA  adapter  that will only handle 256 display colors,  you  may

prefer the quality of the monochrome images.

Robot B&W Modes

<1> 8/8.5 Second

Most  stations will not be using the original  120/128  line

SSTV format (all the other modes produce 200/240 line images) but

there  are operators out there using older equipment that  cannot

transmit or receive 240 line pictures. Display will take about  8

seconds.  Note  that the image appears quite "coarse"  as  it  is

scrolled  on the screen, because of the small number  of  pixels.

Some  software displays 8 second/120 line pictures in  a  smaller

format for this reason. FAX480 displays the image at normal  size

and  solves  the  resolution dilemma AFTER  the  image  has  been

scrolled  onto  the  screen. You will note that  just  after  the

screen is filled, the image suddenly looks softer and a good-deal

less "digital" in appearance. This is because the program does  a

pixel averaging routine to produce a much better looking image on

the display.

Unlike many programs, you CAN re-transmit an 8 second image,

in  either  8 seconds or 36 seconds. It will look just  like  the

original  in  the  former case but will look even  better  in  36

seconds because of the processing used for the display.

While not indicated on the menu, this option will also prove

for display of Robot 12 second color images.

<2> 36 Second <S>

This is a nice mode because it provides full resolution  but

without the time required for color display. It is the  preferred

mode  for  SSTV operation between FAX480 stations. Note  the  <S>

that follows on the menu display. This provides for display of of

36  second transmissions using the crystal clock instead  of  the

normal line triggering. If you are exchanging 36 second  pictures

with  another  FAX480 station, key <S> instead of <2>  AFTER  the

image  starts. The result will be reception with essentially  the

same  noise  immunity as fax. When conditions are poor,  the  <S>

option  may be worth a try with other stations since  some  Robot

and Amiga units will produce a pretty straight picture.

Robot Color

<3> 36 Second

The  36  second Robot color mode (NOT the same thing  as  36

second B&W) lets stations transmit 240 line color images in  less

time.  For  many reasons, it is a compromise  that  will  produce

results inferior to the use of the 72 second color mode.

<4> 72 Second

This  is the "standard" mode for SSTV color transmission  in

that  virtually  any  system can receive  and  transmit  in  this

format. The mode is used a lot so you will become quite  familiar

with  the key location! This is the highest quality  Robot  color

mode and will produce excellent monochrome pictures.

Scottie Color Modes

Although  the Scottie modes add a line sync pulse, they  are

designed  as  synchronous modes, keyed to a crystal  clock,  much

like  the  FAX480 system (but with 240 lines of  resolution).  As

presently implemented, I use the sync pulse, so you will not  see

any inherent noise immunity. NOTE: If someone can provide me with

PRECISE timing data for these modes, I would be happy to  convert

the routines to synchronous operation.

<5> Mode 1

This mode provides optimum resolution and it gets used a lot

because it works better than Robot 72 under noisy conditions.

<6> Mode 2

Like  Robot 36 color, Scottie 2 is a compromise  in  sending

240  line images in less time. Like the Robot 36 color mode,  the

pictures lack the quality of the Scottie 1 mode, but they do take

less time and are MUCH better than 120 line images.

Amiga AVT Color

Although  there are several AVT color modes, I  support  the

AVT 94 because it is the most widely used. Precision tuning is  a

MUST for AVT display and if the display will not trigger, you are

probably  slightly  off frequency and should resort to  the  Tune

Display option.

<7> 94 Second

This is a synchronous mode and I have retained that in  this

software (there are no sync pulses). When you key <7>, the  white

"active" square will be posted as the software attempts to decode

the  header. If it does so, display will begin just as  you  hear

the  distinctive header replaced by the actual video data. If  it

cannot decipher the header, the square will go away. There is  no

way  to  restart  display if this occurs.  As  noted  above,  the

problem is likely to be mis-tuning of your receiver. If you  want

to try again, ask the other station to end an alignment tone  and

use the Tune Display option.

SSTV POST-DISPLAY OPTIONS

When display of any SSTV mode is complete (the white  square

at   the  lower-left  disappears),  you  have  several   options,

indicated by the letters posted at the left side of the screen:

<S>

<T>

<C>

<R>

<Q>

These same options are visible if you have loaded an image  file.

Each will be described below.

Save Image

Keying  <S>  will save the image currently on  display.  The

screen will clear and you will be asked to provide a name for the

image  file.  This can be ANY valid DOS  filename  (8  characters

maximum, no extension). If you use the name of an existing  image

file,  the old file will be over-written. Type the  desired  file

name and hit <ENTER> and the image will reappear for a moment, it

will  be  saved  under  the  name  you  provided  (with  an  .STV

extension),  and  you  will be returned to  the  SSTV  Menu.  The

process  takes just a second or two. If you want the image  back,

key  <R>  from  the SSTV Menu to  recall  it  (discussed  below).

Otherwise,  you can get it back later using the Load  Image  from

Disk option.

Transmit

Keying  <T> will cause the screen to clear and you will  see

the following options:

<1> 8 Second

<2> 36 Second

<Q>uit

Keying  <1>  will transmit the displayed picture  in  the  8

second/120  line  format,  while  <2> will  send  it  in  the  36

second/240 line format. The latter is obviously preferred  unless

you  are working someone with older gear, such as the Robot  400,

which cannot handle 240 line pictures. In either case, the  image

will be re-posted, the interface will key, and the image will  be

transmitted.  When transmission is complete, the  interface  will

drop  out  of  transmit  and the image  will  remain  posted.  To

transmit again (in the same or another mode), simply key <T>.

If  you need to abort transmission for any reason, use  the

<A> key. If you are using your interface for the first time,  see

the  notes under the fax transmission section for how to set  the

transmit audio level.

Contrast Expansion

Images  may be received with less-than optimum contrast  due

to slight mis-tuning or poor image setup at the transmitting end.

Always save such an image first, but you can then either load  it

from  disk (or recall it after the save) and perform  a  contrast

enhancement by keying <C>.

If  you  key  <C>, the computer will BEEP  to  indicate  the

command was received, but nothing else will seem to happen for  a

minute  or  so while the computer does an analysis of  the  pixel

distribution of the image. The process takes about 55 seconds  on

my 20 MHz 386. The computer will BEEP again when the analysis  is

complete  and  the posted image will be cleared and  a  graph  of

pixel brightness values will be displayed. The graph consists  of

a horizontal bar where the extreme left is black and the  extreme

right is white. The length of the vertical lines below the bar is

proportional  to  the  number of  pixels  at  various  brightness

levels.

You must now specify what thresholds you would like for  the

black  and  white ends of this range. For example, if  the  image

seemed  too light, there will probably be a gap between the  left

end  of the bar and where you first see a significant  clustering

of  pixels. If the picture was too dark there will be  a  similar

gap at the right (white) end of the bar. A really "soft"  picture

may have gaps at both ends! If this is all more than you want  to

deal  with,  you can use <Q> to quit with no changes,  but  let's

forge ahead.

The  first  task is to specify the black threshold.  If  you

look  carefully, you will se a sort vertical bar extending  ABOVE

the bar at the extreme left. If you key <+> (use <SHIFT> and  <+>

if  your keyboard doesn't have a dedicated <+> key) to move  this

bar to the right until you reach the first significant cluster of

pixels. If you over-shoot, the <-> key will move the bar back  to

the  left.  When  it is position where you want it,  key  <B>  to

specify the black threshold. If you already have plenty of pixels

down at the black end of the range, simply key <B> without moving

the reference bar.

Once you key <B>, the bar will disappear and reappear at the

extreme  right (white). Now use the <-> key, if needed,  to  move

the bar to the left until you reach the first significant cluster

of  pixels (or <+> to move back if you over-shoot). When the  bar

is  positioned where you want it, key <W> to establish the  white

threshold.  You  can  key  <W> right away  if  you  already  have

significant clustering at the white end of the contrast range.

Once  you  key  <W>, the image will  be  re-posted  and  the

computer  will scroll down through it, readjusting pixel  values.

Unless  you  made only very minor threshold changes,  the  effect

should  be  fairly  obvious. On my system,  it  takes  about  1.5

minutes to scroll down and the computer will BEEP when the job is

finished.

If you like the altered picture more than the original,  use

<S>  to  save it to the original filename. You can also  save  it

with another name if you wish. If you don't like the effect,  you

can  use  <Q>  to  quit. If you want  to  experiment  with  other

threshold  settings,  load the original image from disk  and  try

again.

Repair

Noise, interference, or slight mis-tuning can result in line

trigger  errors  where lines are offset slightly  to  the  right,

spoiling  an otherwise nice image. If you have just received  the

image, save it first (<S>) but then call it back. The defects can

be  almost completely repaired by keying <R>. When you do  so,  a

small horizontal bar will appear just beyond the upper right edge

of  the display. Use <U> or <D> to move the bar up or down  until

it  is opposite an offset line (look carefully). When it is,  key

<A>,  and  the line in question will slide LEFT  by  exactly  one

pixel. Key <A> as many times as required to properly position the

line.  You  can then reposition the bar (<U> or  <D>)  to  repair

other  lines. When you are done, use <D> to move the bar down  to

the  bottom image margin and the repaired image will be  replaced

by  a  prompt, asking if you want to replace the image  with  the

repaired version. Key <Y> to replace or <N> to leave the original

intact.

It  takes a few minutes to "repair" and image but it may  be

worth it to fix a valuable image marred by a few line offsets.

Quit

Keying <Q> will return you to the SSTV Menu. The last  image

posted  can be recalled at any time using the Recall  Last  Image

function to be discussed shortly.

OTHER FUNCTIONS

<R>ecall Last Image

If you have previously received an image or loaded one  from

disk,   that  image will be posted to the active window  of   the

display  screen.  At  that  point,  you  have  the  same  options

available to you  as described under the receive modes.

IMPORTANT  NOTE:  Although  unlikely, if you do  something   that

causes   the  program  to abort, the temporary disk  buffer  file

will be retained and you can recall the image when you re-boot.

<L>oad Image from Disk

When  you key <L> to invoke this function, the names of  all

FAX483 SSTV  image  files  on  the  default   directory/drive   will

be  displayed.   Each  of these will have  an  ".STV"  extension.

Enter  the name of the file to  load (less  the  .STV  extension)

and the image file will be  loaded,  the system  will  switch  to

the image display screen, and  the  image will be transferred  to

the  screen,  a  process that takes about a second.  Any  of  the

secondary  functions (<S>,  <T>, <C>, <R>, or <Q>uit can be  used

if   desired.  Note  that you can save the image to  the  default

directory/drive  under   a  different  file  name  or   you   can

save  to   another directory/drive  with  the save or a different

file   name.  This provides a convenient way to copy  interesting

images  to a  floppy for  backup  purposes or to another disk  to

provide  to  another fax/SSTV operator.

<C>alibration Tone

Keying  <C>  from the SSTV Menu will post a menu  where  you

have  the choice of transmitting a sync, black, or white tone  or

using <Q> to quit. Make you choice on the basis of what the other

operator  is  expecting. In the case of FAX480  users,  the  best

choice is always black (1500 Hz). The interface will then key and

transit  the  desired tone until you hit the <Q>  key,  at  which

point you will return to the SSTV Menu.

<T>une Display

Proper     grayscale     rendition     requires     accurate

tuning/carrier  insertion   at  your  receiver. Usually  you  can

achieve sufficient accuracy by tuning for natural voice  quality,

but   this  can  be  ambiguous  at   times.   The   tune  display

provides    a   graphic   aid   for   precision   tuning.     Two

vertical, parallel lines define the range from 1200 Hz (Sync)  at

the  top of the display to 2300 Hz (White) at the bottom. If  you

are uncertain about the accuracy of your tuning, request that the

transmitting station transmit a 1500 Hz (Black) calibration  tone

prior  to sending an image. Key <T> for the tone display and  you

will  note  a  horizontal line between the  two  vertical  lines.

Carefully adjust the receiver tuning (use the RIT control in  the

case of a transceiver) until the horizontal line is lined up with

the two arrows. These arrows are positioned at the 1500 Hz  point

on the frequency display. When the line is properly aligned  with

the  arrows, your are precisely tuned for image reception.  Other

SSTV  system have different alignment options (sync,  keyed  sync

and black, etc., in which case you should tune appropriately.

No  matter what your tuning mode, hitting the <Q>  key  will

return you to the SSTV menu.

<F>ile Conversion

Most  FAX480 users will be using ViewPort VGA  software  for

their  primary  SSTV operations and this software  supports  file

inports  in  a number of formats. In deciding what type  of  file

conversion  to support in the FAX480 SSTV module, I decided  that

the best option was to provide a means to convert FAX480 files to

SSTV.  Now  that the new screen-capture file conversion  is  very

fast and much easier to use, operators will be accumulating a lot

of  interesting  fax  files.  If these  files  could  be  RAPIDLY

converted  to  SSTV,  it  would help  to  keep  the  no-fax  SSTV

operators  at bay when they start to get restless.  This  version

operates  very fast (about 10 seconds on my 386), so it is  quite

practical  to  create an SSTV file of a fax image that  has  just

been  received  and  transmit  it for the  benefit  of  those  on

frequency without fax capability.

When  you  key  <F>, you will see a display of  all  of  the

FAX480 image files (just like the fax Load from Disk option), and

you  simply  type the name of the file you want  to  convert  (no

extension, as usual) and hit <ENTER>. The system will display the

480  line image (2-3 seconds) and will then proceed to  make  the

conversion, a process that will take another 6-8 seconds on  most

systems.  When  conversion is complete, the SSTV  image  will  be

posted  VERY briefly as it is saved, and you will be back to  the

file menu. You can proceed to convert another file or exit at any

time  by using <Q>. The filename applied to the SSTV  picture  is

the  same  as that of the fax file, except that it has  the  .STV

slow scan extension instead of the .480 fax version.

In addition to being fast, the routine does an excellent job

of  conversion because in employs a pixel averaging  scheme  that

increases  the  number of grayscale values  even  while  reducing

resolution. The output is a very nice looking SSTV version of the

picture.  The  top  and  bottom  20  lines  are  clipped  in  the

conversion, but, other than that, the images are equivalent.

FAX480 OPTIONS ON OTHER SYSTEMS

When  you operate FAX480 on the air, you will get  questions

about  the  mode and how to implement it.  Obviously,  getting  a

ViewPort  is  the simplest solution and  quite  economical.  Most

FAX480  users will agree that this mode alone justifies the  cost

of  the interface. As long as an operator has the  computer,  the

ViewPort  is a reasonable add-on, even if they don't plan to  use

it for SSTV.

At  least  one station has developed software  for  the  MFJ

interface  running  on a Mac (talk about a $100 saddle on  a  $10

horse),  so a PC version may be practical. John Hoot of  Software

Systems Consulting has modified the SSC PC-Fax program to  handle

FAX480  so that is another option. I am working on a version  for

the  Pasokon, so it may not be long before we have most  computer

bases  covered.  The Amiga is a problem because I don't  know  of

anyone  programming for that system. If you know of anyone,  have

them  contact me, as the Amiga should be a resonavle platform  in

its 640 x 400 mode.

That leaves the Robot folks and there is no way to implement

480  lines  on  that unit. You could sample the  image  but  that

provides  no advantage, other than looking in! I think  the  best

option for Robots connected to computers is to use the Robot as a

"dumb  terminal" with the computer providing the actual  display.

If I can get a Robot manual, I may work up such a program!

###################################################################