File T3000.DOC                                                    December 1992

T3000.SCR: TELEBIT MODEM DIALING FOR MS-DOS KERMIT

C. Gianone, F. da Cruz

Columbia University

New York, New York, USA

T3000.SCR (stored in Kermit Distribution as MSMT3000.SCR) is a dialing script

for the Telebit T3000 modem.  It should also work with the Telebit T1600,

QBlazer, and WorldBlazer models.  It requires MS-DOS Kermit 3.11 or later.

To use this script rather than the default HAYES.SCR (MSMHAYES.SCR) for

dialing, store it in the same directory as your MSKERMIT.INI file, rename it

(if necessary) to T3000.SCR, and, prior to starting MS-DOS Kermit, set the DOS

environment variable, MODEM, to T3000:

SET MODEM=T3000

so the DIAL macro will use it (alternatively, you can tell MS-DOS Kermit to

DEFINE _MODEM T3000).  If you always use a Telebit T3000, T1600, QBlazer, or

WorldBlazer, you can add the above command to your AUTOEXEC.BAT file, or you

can add the DEFINE _MODEM T3000 command to your MSCUSTOM.INI file.

When this command file is executed, the given telephone number (passed in the

\%1 variable) is dialed on the current port at the port's current speed.  If

the \%1 value is = (equal sign) or empty, the modem is initialized but no

dialing takes place.

The modulation technique and modem interface speed are determined by Kermit's

interface speed.  The modem tries to use the highest modulation technique

appropriate to the interface speed, and then falls back until it finds one in

common with the answering modem.  The local Telebit modem's interface speed is

locked to match Kermit's and does not change even if a slower connection speed

is arrived at.

MS-DOS KERMIT SPEED    MODULATION TECHNIQUE AND FALLBACK

14400 bps or greater   V.32bis (14400) -> V.32 (9600) -> V.22bis (2400) -> ...

9600 bps              V.32 (9600) -> V.22bis (2400) -> V.22 (1200) ...

2400-4800 bps         V.22bis (2400) -> V.22 / Bell 212A (1200) -> Bell 103

1200 bps              V.22 / Bell 212A (1200) -> V.21 / Bell 103 (110-300)

110-300 bps          V.21 / Bell 103 (110-300)

(Note: 14400 bps is not supported in MS-DOS Kermit 3.12 or earlier; use

19200 or higher for V.32bis connections.)

V.32bis is supported by the T3000 and WordlBlazer, but not by the T1600 or

QBlazer.  The 1200-bps and lower connection methods (V.22 vs Bell 212A; V.21

vs Bell 103) are determined by your modem's configuration, and can be changed

with the modem's B command:

ATB0  Use CCITT V.22 and V.21 standards

ATB1  Use Bell 212A and 103 standards

View your modem's configuration with the AT&V command.

To avoid data overruns that could occur in case of speed mismatch, hardware

flow control is used by both MS-DOS Kermit and the local Telebit modem.

However, flow control will be effective only if the answering modem and

computer also have an effective flow control method between them.

Dialing uses ATD (the modem's default dialing method) to dial the number,

rather than ATDT, because not all locales offer Touch-Tone dialing.  To force

Touch-Tone dialing, include T as first character of phone number; to force

pulse dialing, include P as first character, for example:

DIAL T7654321

This command file configures your Telebit modem as follows:

X12          Extended result codes, wait for dialtone, recognize busy signal

&D2          Disconnect call and return to command mode if DTR drops

S50=0        Automatic modulation speed determination

S94=1        Allow connection at any speed and allow modulation fallback

S58=2S68=255 Use bidirectional RTS/CTS hardware flow control

S60=0        Data format: 8 data bits, no parity

S61=0S63=0   Pass BREAK through transparently

Error control and compression selection:

S180=2S181=1 Error control fallback: V.42->MNP->buffered,lock interface speed

S190=1       Compression allowed in both directions

The S180, S181, and S190 settings will not necessarily work in all situations.

For example, in some cases it is necessary to disable MNP negotiation if the

answering modem does support MNP.  It is also necessary to disable compression

if the answering modem does not support compression OR if it does not support

hardware flow control.  Substitute the following settings for the ones given

above in such cases:

S180=1S181=1 Error control fallback: V.42 -> buffered, lock interface speed

S190=1       Compression disabled in both directions

(This can be done by uncommenting one line in the T3000.SCR file.)

Please read your Telebit modem manual for further details, or give the modem

command AT~H (AT tilde H) to view its built-in help text.

If Kermit's HANGUP command does not work for you, it probably means your cable

or connector is not wired correctly.  In that case:

(1) Change all occurrences of HANGUP to HANGITUP in the T3000.SCR file.

(2) Add the following line to your MSCUSTOM.INI file:

define hangitup pause 1, output +++, pause 1, output ath0\13

Please feel free to make changes, suggest improvements, or adapt this script

program to dial other types of modems.

(End of T3000.DOC)