STS-57 PRESS KIT

NATIONAL AERONAUTICS AND SPACE ADMINISTRATION

SPACE SHUTTLE MISSION STS-57  PRESS KIT

JUNE 1993

SPACEHAB - 01  /  	EURECA RETRIEVAL

CONTENTS

GENERAL BACKGROUND

General Release	4

Media Services Information	7

Quick-Look Facts	8

Payload and Vehicle Weights	9

Summary Timeline	10

Orbital Events Summary	12

Crew Responsibilities	13

CARGO BAY PAYLOADS & ACTIVITIES

SPACEHAB	15

Commercial Materials Science Experiments	22

Commercial Life Science Experiments	28

Johnson Space Center (JSC) Investigations	37

Space Station Experiments	43

Supporting Hardware	44

Science Experiments Summary Charts	46

European Retrievable Carrier (EURECA)	54

Get Away Special (GAS)	59

Consortium for Materials Development in Space/Complex

Autonomous Payload (CONCAP)	    63

Super Fluid Helium On Orbit Transfer (SHOOT) Demonstration 64

STS-57 Extravehicular Activity (EVA)	67

MIDDECK PAYLOADS

Fluid Acquisition and Resupply Experiment (FARE)	68

Air Force Maui Optical Station (AMOS)	69

SPECIAL EVENTS & EDUCATIONAL ACTIVITIES

GAS #324 - CAN DO	70

Shuttle Amateur Radio Experiment-II (SAREX-II)	71

CREW BIOGRAPHIES & MISSION MANAGEMENT

STS-57 Crew Biographies	73

Mission Management for STS-57	75

STS-57 General Release

FIRST SPACEHAB FLIGHT HIGHLIGHTS STS-57 SHUTTLE MISSION

RELEASE:  93-78

The beginning of a new era in the commercial development of space and the

retrieval of a European satellite highlight NASA's Shuttle Mission STS-57. The

mission, scheduled for early June 1993, also will see Space Shuttle Endeavour

and her six-person crew use experiments designed by and for students, operate a

payload which may improve crystal growth techniques and demonstrate possbile

on-orbit refueling techniques.

A rendezvous with the European Space Agency's European Carrier (EURECA)

satellite is scheduled to take place on the fourth day of the mission.  The

Shuttle's robot arm will be used to grapple the satellite.  It then will be

lowered into Endeavour's cargo bay and stowed so it can be returned to Earth.

The EURECA satellite has been on-orbit collecting data since its deployment

during Shuttle Mission STS-46 in July 1992.

On STS-57, NASA will be leasing a privately-developed mid-deck

augmentation module known as SPACEHAB. The primary objective is to support the

agency's commercial development of space program by providing additional access

to crew-tended, mid-deck locker or experiment rack space.  This access is

necessary to test, demonstrate or evaluate techniques or processes in

microgravity.

NASA's secondary objective is to foster the development of space

infrastructure which can be marketed by private firms to support commercial

microgravity research payloads.  In this instance, SPACEHAB, Inc., has the

capability of leasing SPACEHAB facility space to other commercial customers on

upcoming flights of the module.

The experiments flying inside this first SPACEHAB include investigations

ranging from drug improvement, feeding plants, cell splitting, the first

soldering experiment in space by American astronauts and high-temperature

melting of metals.

Included are 13 commercial development of space experiments in material

processing and biotechnology, one NASA biotechnology experiment and five other

NASA investigations related to human factors and the Endeavor's environment and

a space station environmental control system test.

Three other payloads, the Get Away Special (GAS), the Consortium for

Materials Development in Space Complex Autonomous Payload-IV (CONCAP-IV) and

the Superfluid On-Orbit Transfer (SHOOT) payload will be carried in Endeavour's

cargo bay.

The GAS system, which has flown many times on the Space Shuttle, allows

indiviudals and organizations around the world access to space for scientific

research.  During the STS-57 mission, 10 GAS payloads from the United States,

Canada, Japan and Europe will perform a variety of microgravity experiments.

The CONCAP-IV payload is the fourth area of investigation in a series of

payloads.  It will investigate the growth of nonlinear organic crystals by a

novel method of physical vapor transport in the weightlessness of the space

environment.  Nonlinear optical materials are the key to many optical

applications now and in the future with optical computing being a prime

example.

The SHOOT payload is designed to develop and demonstrate the technology

required to re-supply liquid helium containers in space.  Because so little

experience exists with cryogen management in microgravity, SHOOT is designed to

gather data about how the liquid feeds to pumps, the behavior of the

liquid/vapor discriminators and the slosh and cool down of the liquid.  Middeck

Experiments

Two experiments which previously have flown aboard the Shuttle will be

carried in Endeavour's middeck area.  The Fluid Acquisition and Resupply

Experiment (FARE), which last flew on Shuttle Mission STS-53 in November 1992,

will continue to investigate the fill, refill and expulsion characteristics of

simulated propellant tanks.  It also will study the behavior of liquid motion

in microgravity.

The Air Force Maui Optical System (AMOS) is an electro-optical facility

located on the Hawaiian Island of Maui. The primary objectives of AMOS are to

use the orbiter during flights over Maui to obtain imagery and/or signature

data from the ground-based sensors.

Spacewalk on STS-57

STS-57 crew members David Low and Jeff Wisoff will perform a 4-hour

extravehicular activity (EVA) on the fifth day of the flight as a continuation

of a series of spacewalks NASA plans to conduct to prepare for construction of

the space station.

The spacewalk tests, the first of which was performed on STS-54 in January

1993, are designed to refine training methods for spacewalks, expand the EVA

experience levels of astronauts, flight controllers and instructors, and aid in

better understanding the differences between true weightlessness and the ground

simulations used in training.

In addition, since the Shuttle's remote manipulator system mechanical arm

will be aboard Endeavour to retrieve EURECA, the STS-57 spacewalk will assist

in refining several procedures being developed to service the Hubble Space

Telescope on mission STS-61 in December.

Education

NASA's on-going educational efforts will be represented by two payloads.

The Get-Away Special (GAS) #324 - CAN DO experiment is designed to take 1,000

photos of the Earth allowing students to make observations and document global

change by comparing the CAN DO photos with matched Skylab photos.

The primary payload of CAN DO, known as GEOCAM, contains four Nikon 35mm

cameras equipped with 250 exposure film backs.  The GEOCAM system will match

closely the larger Skylab film format in both coverage and quality allowing

direct examination and comparison of the changes that have occurred to the

planet in the last 20 years.  The canister also contains 350 small, passive,

student experiments.

STS-57 crew members will take on the role of teacher as they educate

students from around the world about their mission objectives and what it is

like to live and work in space by using the Shuttle Amateur Radio Experiment

(SAREX) experiment.  Brian Duffy and Janet Voss will operate SAREX. Operating

times for school contacts are planned into the crew's activities.

Mission Summary

Leading the six-person STS-57 crew will be Mission Commander Ronald Grabe

who will be making his fourth space flight.  Pilot for the mission is Brian

Duffy, making his second flight.  Leading the science team will be Payload

Commander David Low who also is designated as Mission Specialist 1 (MS1) and is

making his third flight.  The three other mission specialists for this flight

are Nancy Sherlock (MS2), Jeff Wisoff (MS3) and Janice Voss (MS4), all of whom

will be making their first flight.

The mission duration for STS-57 is planned for 6 days, 23 hours, 19

minutes.  However, the mission may be extended by 1 day immediately after

launch if projections calculated at that time for energy and fuel use during

the EURECA rendezvous permit.  If for some reason STS-57 remains a 7-day

flight, the extravehicular activity scheduled for flight day five would be

cancelled.  The STS- 57 mission will conclude with a landing at Kennedy Space

Center's Shuttle

Landing Facility.

This will be the fourth flight of Space Shuttle Endeavour and the 56th

flight of the the Space Shuttle system.

- end -

STS-57 MEDIA SERVICES INFORMATION

NASA Select Television Transmission

NASA Select television is available on Satcom F-2R, Transponder 13,

located at 72 degrees west longitude; frequency 3960.0 MHz, audio 6.8 MHz.

The schedule for television transmissions from the orbiter and for mission

briefings will be available during the mission at Kennedy Space Center, Fla;

Marshall Space Flight Center, Huntsville, Ala.; Ames-Dryden Flight Research

Facility, Edwards, Calif.; Johnson Space Center, Houston and NASA Headquarters,

Washington, D.C. The television schedule will be updated to reflect changes

dictated by mission operations.

Television schedules also may be obtained by calling COMSTOR 713/483-

5817.  COMSTOR is a computer data base service requiring the use of a telephone

modem.  A voice update of the television schedule is updated daily at noon

Eastern time.

Status Reports

Status reports on countdown and mission progress, on-orbit activities and

landing operations will be produced by the appropriate NASA newscenter.

Briefings

A mission press briefing schedule will be issued prior to launch.  During

the mission, status briefings by a Flight Director or Mission Operations

representative and when appropriate, representatives from the science team,

will occur at least once per day.  The updated NASA Select television schedule

will indicate when mission briefings are planned.

STS-57 Quick Look

Launch Date/Site:	June 3, 1993/Kennedy Space Center - Pad 39A

Launch Window:		6:13 p.m. - 7:24 p.m. EDT

Orbiter: 			Endeavour (OV-105) - 4th Flight

Orbit/Inclination: 	250 nautical miles/28.45 degrees

Mission Duration:	6 days, 23 hours, 19 minutes

Landing Date: 		June 10

Primary Landing Site: 	Kennedy Space Center, Fla.

Abort Landing Sites:	Return to Launch Site - KSC, Fla.

TransAtlantic Abort landing 	 - Banjul, The Gambia

- Ben Guerir, Morroco

- Moron, Spain

Abort Once Around 	- Edwards AFB, Calif.

Crew: 	Ronald Grabe, Commander (CDR)

Brian Duffy, Pilot (PLT)

David Low, Payload Commander/Mission Specialist 1 (MS1)

Nancy Sherlock, Mission Specialist 2 (MS2)

Jeff Wisoff, Mission Specialist 3 (MS3)

Janice Voss, Mission Specialist 4 (MS4)

Cargo Bay Payloads:	EURECA-1R (European Retrievable Carrier - Retrieval)

SPACEHAB (Space Habitation Module)

SHOOT (Super-fluid Helium On-Orbit Transfer)

CONCAP-IV (Consortium for Materials Development in

Space Complex Autonomous Payload-IV)

GAS Bridge (Get-Away Special Bridge)

In-Cabin Payloads: 	AMOS (Air Force Maui Optical Site)

FARE (Fluid Acquisition and Resupply Experiment)

SAREX-II (Shuttle Amateur Radio Experiment-II)

DTOs/DSOs:

DTO 412: 	   On-orbit Fuel Cell Shutdown

DTO 623: 	   Cabin Air Monitoring

DTO 700-2: 	   Laser Range, Range-Rate Device

DSO 603B: 	   Orthostatic Function During Entry, Landing and Egress

DSO 604 OI-1:   Visual Vestibular Integration as a Function of Adaptation

DSO 618: 	   Effects of Intense Exercise During Space Flight on

Aerobic Capacity and Orthostatic Function

DSO 624: 	   Pre-Flight and Post-Flight Measurement of

Cardiorespiratory Response

DSO 901: 	   Documentary Television

DSO 902: 	   Documentary Motion Picture Photography

DSO 903: 	   Documentary Still Photography

STS-57 VEHICLE AND PAYLOAD WEIGHTS

Vehicle/Payload	Pounds

Orbiter (Endeavour) empty and 3 Shuttle Main Engines	173,023

Spacehab-1/support hardware	9,628

EURECA (berthed)	9,800

GAS bridge, cans	5,652

SHOOT/support hardware	3,570

FARE	126

SAREX-II	46

Total Vehicle at solid rocket booster Ignition	4,516,091

Orbiter Landing Weight	224,111

STS-57 SUMMARY TIMELINE

NOTE: The STS-57 mission is planned to be 6 days, 23 hours, 19 minutes long.

However, it may be extended by 1 day immediately after launch if projections

calculated at that time for energy and fuel use during the EURECA rendezvous

permit.  If STS-57 remains a 6-day (MET) flight, the extravehicular activity

scheduled for flight day five would be cancelled.  Activities planned for the

first four flight days would be unchanged.  Flight control system checkout,

reaction control system hot-fire and Spacehab deactivation would take place on

flight day seven.  Entry and landing would be on flight day eight.

The following is a schedule for the extended, 7-day, 23-hour (MET) mission:

Flight Day One	                     Flight Day Six

Ascent	                             Spacehab operations

OMS-2 (251 n.m. x 169 n.m.)	     FARE operations

Spacehab activation

Spacehab operations

NC-1 burn (251 n.m. x 174 n.m.)

Flight Day Two	                     Flight Day Seven

Remote manipulator system checkout   Spacehab operations

SHOOT operations	             FARE operations

Spacehab operations

NC-2 burn (251 n.m. x 178 n.m.)

Flight Day Three	             Flight Day Eight

SHOOT operations	             Spacehab operations

Spacehab operations	             Flight control systems checkout

NC-3 burn (251 n.m. x 184 n.m.)	     Reaction control system hot-fire

Spacehab deactivation

Flight Day Four	                     Cabin stow

EURECA retrieval

NSR burn (251 n.m. x 248 n.m.)	     Flight Day Nine

NH-4 burn (257 n.m. x 250 n.m.)	     Spacehab deactivation completed

TI-burn (259 n.m. x 256 n.m.)	     Deorbit preparations

EURECA grapple	                     Deorbit burn

EURECA berth	                     Entry

Spacehab operations	             Landing

Flight Day Five

Extravehicular activity preparations

Extravehicular activity (4 hours)

SPACE SHUTTLE ABORT MODES

Space Shuttle launch abort philosophy aims toward safe and intact recovery

of the flight crew, orbiter and its payload.  Abort modes include:

o Abort-To-Orbit (ATO) -- Partial loss of main engine thrust late

enough to permit reaching a minimal 105-nautical mile orbit with orbital

maneuvering system engines.

o Abort-Once-Around (AOA) -- Earlier main engine shutdown with the

capability to allow one orbit around before landing at Edwards Air Force Base,

Calif.

o TransAtlantic Abort Landing (TAL) -- Loss of one or more main engines

midway through powered flight would force a landing at either Banjul, The

Gambia; Ben Guerir, Morocco; or Moron, Spain.

o Return-To-Launch-Site (RTLS) -- Early shutdown of one or more

engines, and without enough energy to reach Banjul, would result in a pitch

around and thrust back toward KSC until within gliding distance of the Shuttle

Landing Facility.

STS-57 contingency landing sites are the Kennedy Space Center, Edwards Air

Force Base, Banjul, Ben Guerir and Moron.

STS-57 Orbital Events Summary (for 1-day extended mission)

EVENT		START TIME			VELOCITY CHANGE	      ORBIT

(dd/hh:mm:ss)        	       (feet per second)      (n.m.)

OMS-2		00/00:44:00		           241 fps	   251 x 169

NC-1	        00/05:21:00		           8 fps	   251 x 174

(adjusts the rate at which Endeavour is closing on EURECA)

SH-1	        00/22:18:00	            	   3.4 fps	   251 x 176

(performed as part of the Super Fluid Helium On-Orbit Transfer experiment)

NPC		01/03:04:00	             	   6.2 fps	   251 x 175

(aligns Endeavour's orbit directly below EURECA's orbit)

NC-2		01/04:28:00	              	   4 fps	   251 x 178

(adjusts the rate at which Endeavour is closing on EURECA)

SH-2		01/19:53:00	               	   3.6 fps	   251 x 180

(performed for the SHOOT experiment)

SH-3		01/21:26:00		           3.6 fps	   251 x 182

(performed as part of the SHOOT experiment)

NC-3		02/03:36:00	                     4 fps	   251 x 184

(adjusts the rate at which Endeavour is closing on EURECA)

NSR		02/19:03:00		           109 fps	   251 x 248

(circularizes Endeavour's orbit)

NH		02/21:27:00		            15 fps	   257 x 250

(adjusts the altitude of Endeavour's orbit)

NC-4 		02/21:27:00		            8.6 fps	   258 x 255

(adjusts the rate at which Endeavour is closing on EURECA)

TI		03/00:35:00	                    3.1 fps  	   258 x 256

(begins Endeavour's proximity operations with EURECA)

GRAPPLE		03/02:50:00					   259 x 256

DEORBIT		07/21:36:00	                    414 fps

LANDING		07/23/19:00

NOTE: Engine firings are likely to change slightly after launch as they are

recalculated by flight controllers.  In addition, some of the smaller firings

may be deleted altogether if navigation information during the rendezvous

allows.  However, the time frame and other information regarding the larger

burns is unlikely to change dramatically.

STS-57 CREW RESPONSIBILITIES

TASK/PAYLOAD	        PRIMARY         BACKUP

EURECA-RMS	        Low  	       	Sherlock

EURECA systems	        Sherlock	Duffy

EURECA rendezvous	Grabe	        Duffy, Wisoff

EVA	                Low, Wisoff	N/A

EVA-RMS	                Sherlock	Voss

Spacehab systems	Low         	Voss

SHOOT	                Voss        	Wisoff

FARE	                Wisoff         	Duffy

GBA	                Sherlock	Grabe

SAREX	                Duffy        	Voss

SPACEHAB experiments:

ASPECS	                Wisoff        	Sherlock

BPL                    	Sherlock	Wisoff

CR/IM-VDA              	Low        	Voss

HFA: EPROC	        Voss         	Sherlock

HFA: Light, sound	Grabe         	Duffy

HFA: Trans	        Sherlock	Grabe

NBP	                Duffy          	Grabe

PSE	                Grabe         	Voss

SCG	                Voss         	Low

TES-COS	                Voss         	Grabe

APCF	                Voss          	Low

ASC-2	                Sherlock	Duffy

CGBA	                Wisoff         	Voss, Low

CPDS	                Voss        	Low

3DMA	                Voss          	Low

ECLIPSE-HAB	        Voss        	Low

EFE	                Low        	Sherlock

GPPM	                Voss         	Low

IPMP	                Grabe

LEMZ-1	                Voss        	Wisoff

ORSEP	                Voss        	Low

SAMS                 	Voss        	Low

ZCG	                Voss        	Low

SPACEHAB-01

Why The Need For SPACEHAB?

During the last decade, the commercial development of space became one of

NASA's primary objectives, as directed by legislation and national policy.

Through the many facets of its commercial development of space program, NASA

has developed and maintains a high level of commitment to this objective.  To

that end, NASA has actively invested in the continued technological leadership

of the United States and her future economic growth through the direct

promotion and support of private sector space-related activities.

As a result of NASA's objective, in the late 1980's, its commercial

development of space program identified a significant number of payloads to be

flown to further program objectives.  To viably sustain this program, the

Office of Commercial Programs -- now the Office of Advanced Concepts and

Technology (OACT) -- had to provide a level of flight activity necessary to

support the various payload requirements.

In September 1989, the office conducted an analysis which revealed that

planned Space Shuttle flight activity would not meet its needs for

middeck-class accommodations.  Mission experience has clearly demonstrated that

the orbiter middeck is a very cost-effective area to conduct "crew-tended"

scientific and commercial microgravity research.  However, the size and number

of experiments that can be accommodated in the middeck are severely limited and

have conflicting requirements from Shuttle operations and other NASA programs.

To provide the necessary support for commercial development of space

payloads, the Commercial Middeck Augmentation Module (CMAM) procurement was

initiated in February 1990, through NASA's Johnson Space Center (JSC).

Consequently, in November 1990, NASA awarded a 5-year contract to SPACEHAB,

Inc., of Arlington, Va., for the lease of their pressurized modules, the

SPACEHAB Space Research Laboratories. These laboratories provide additional

space for "crew-tended" payloads as an extension of the Shuttle orbiter middeck

into the Shuttle cargo bay.

This 5-year lease arrangement will cover several Shuttle flights and

requires SPACEHAB, Inc., to provide for the physical and operational

integration of the SPACEHAB Space Research Laboratories in the Space Shuttle

orbiters, including experiments and integration services, such as safety

documentation and crew training.

NASA's primary objective for leasing the SPACEHAB Space Research

Laboratory is to support the agency's commercial development of space program

by providing the access to space.  This access is necessary to test,

demonstrate or evaluate techniques or processes in the environment of space and

thereby reduce risks to a more feasible level.

NASA's secondary objective is to foster the development of space

infrastructure which can be marketed by private firms to support commercial

microgravity research payloads.  NASA is only partially using the SPACEHAB

Space Research Laboratory multi-flight capacity, therefore, SPACEHAB, Inc., is

marketing the additional portion to other commercial users.  It is expected

that significant commercial demand will result from the successful

demonstration of SPACEHAB capabilities on this first flight.

SPACEHAB Accommodations

The SPACEHAB Space Research Laboratory is located in the forward end of

the Shuttle orbiter cargo bay and is accessed from the orbiter middeck through

a tunnel adapter connected to the airlock.  SPACEHAB weighs 9,628 pounds, is

9.2 feet long, 11.2 feet high and 13.5 feet in diameter.  It increases

pressurized experiment space in the Shuttle orbiter by 1100 cubic feet,

quadrupling the working and storage volume available.  Environmental control of

the laboratory's interior maintains ambient temperatures between 65 and 80

degrees Fahrenheit.

The laboratory has a total payload capacity of 3000 pounds and in addition

to facilitating crew access, provides experiments with services such as power,

temperature control and command/data functions.  Other services, such as late

access/early retrieval, also are available.

The SPACEHAB Space Research Laboratory can provide various physical

accommodations to users based on size, weight and other requirements.

Experiments are commonly integrated into the laboratory in Shuttle middeck-

type lockers or SPACEHAB racks.  The laboratory can accommodate up to 61

lockers, with each locker providing a maximum capacity of 60 pounds and 2.0

cubic feet of volume.

The laboratory also can accommodate up to two SPACEHAB racks, either of

which can be a "double-rack" or "single-rack" configuration, but each rack used

reduces the number of usable locker locations by 10 lockers.  A "double- rack"

provides a maximum capacity of 1250 pounds and 45 cubic feet of volume, whereas

a "single-rack" provides half of that capacity.  The "double-rack" is similar

in size and design to the racks planned for use in the space station.

The use of lockers or racks is not essential for integration into the

SPACEHAB Space Research Laboratory. Payloads also can be accommodated by

directly mounting them on the laboratory.

SPACEHAB Operations Philosophy

By its very nature, the Office of Advanced Concepts and Technology (OACT)

flight programs assume a certain level of risk in order to approach the

payloads from the commercial standpoint, including payload development costs

incurred by industry partners.  Each of the investigators is aware of and

accepts a self- established level of risk for mission success.  However, crew

and orbiter safety requirements are always fully met.

The preparations for the flight of SPACEHAB-1 have included the

development of a number of backup and contingency operations for each payload

appropriate to that payload's relative design simplicity.  These backup

procedures include scenarios which might possibly affect crew or orbiter safety

and each payload has procedures associated with it and which the crew has been

trained in which will deactivate and/or safe the payload.

The SPACEHAB-01 Payload Complement

From improving drugs to feeding plants, from cell splitting to

intergalactic particles, from the first soldering experiment in space by

American astronauts to high-temperature melting of metals, the SPACEHAB-01

payloads represent a wide range of space experimentation.

Included are 13 commercial development of space experiments in material

processing and biotechnology, 12 of which are sponsored by NASA Centers for the

Commercial Development of Space (CCDS) and one by the NASA Langley Research

Center, Hampton, Va. There is one NASA biotechnology experiment and five other

NASA investigations related to human factors and the Endeavour's environment.

Finally, there is a space station environmental control system test and as

supporting hardware, two accelerometers -- one from a CCDS and one from the

NASA Lewis Research Center, Cleveland.

Each of the 13 commercial development of space payloads has been screened

by OACT to review the viability of the commercial aspects of the proposed

activity as well as the technical soundness.  Some of the SPACEHAB-01 CCDS

payloads have flown on the Shuttle before, with the SPACEHAB-01 flight

representing the continuation of industry-driven research toward a new or

improved commercial product or process.  Many of the CCDS payloads, including

the CCDS-sponsored accelerometer, have participated in the NASA OACT Consort

series of suborbital sounding rocket flights to test hardware operation and

gain flight worthiness.

The five investigations sponsored by the NASA Johnson Space Center,

involving biotechnology and human factors, were included to assure full

utilization of the first flight of the SPACEHAB Space Research Facility and

have been reviewed for their support to commercial objectives.  These

experiments include equipment testing for future uses on the space station such

as the first- ever American soldering experiment performed in space.

Also on-board the SPACEHAB Space Research Laboratory is an investigation

sponsored by the NASA space station office in Reston, Va., on closed systems to

improve water recycling in the future space station environment.

The experiments, housed in the SPACEHAB Space Research Laboratory on this

its maiden voyage to space, represent a tremendous effort by government and

industry to stretch the possibilities of space as the final frontier -- an

effort focussed on fostering economic growth.

NASA Centers for the Commercial Development of Space

The CCDS program is the cornerstone of NASA's commercial development of

space activities, generating 13 of the 21 total flight hardware packages on

this SPACEHAB Space Research Laboratory. NASA's nationwide CCDS network

represents a unique example of how government, industry and academic

institutions can create partnerships which combine resources and talents to

strengthen America's industrial competitiveness.

The CCDSs are designed to increase private sector investment and interest

in commercial space-related activities, while encouraging U.S. economic

leadership and stimulating advances in promising areas of research and

development.  The CCDSs are based at universities and research institutions

across the country and benefit from links with each other and with NASA field

centers.

Since 1985, OACT has issued four proposal solicitations in various areas

of promising space-related commercial research and development.  From the

solicitations, 17 centers have been established in eight industry-driven,

space- based, high-technology research areas such as materials processing,

biotechnology, remote sensing, communications, automation and robotics, space

propulsion, space structures and space power.

NASA OACT provides annual funding of up to $1 million to each center, with

additional funding to those centers to cover specific programs or flight

activities, as appropriate.  NASA offers the CCDSs its scientific and technical

expertise through NASA field centers, opportunities for cooperative activities

and other forms of continuing assistance.  A key facet of the CCDSs is the

additional financial and in-kind contributions from industry affiliates, state

and other government agencies, which, on the average, exceed the NASA funding

level.

Through creative and enterprising partnerships with industry, the CCDS

program helps move emerging technologies from the laboratory to the marketplace

with speed and efficiency.  The accomplishments of CCDS participants include

significant advances in a number of scientific fields and hundreds of Earth-

and space-based applications.

As an incubator for future commercial space industries, the CCDS program,

since its inception, has facilitated a number of new commercial space ventures

and supported a wide range of ongoing efforts.  The CCDS program continues to

be the key facilitator for U.S. industry involvement in commercial development

of space activities, encouraging and supporting new and ongoing space- related

ventures, as well as spawning research and development advancements that

promise enormous social and economic benefits for all.

1993 - The Year of Commercial Space

Since late 1988, 37 commercial development of space payloads have

successfully flown on the Space Shuttle including the outstanding performance

of four payloads as part of the first United States Microgravity Laboratory

(USML- 1) mission in June 1992.  Additionally, 27 commercial space research

payloads have flown on several suborbital sounding rockets.

During 1993, 56 research payloads are planned, including those on the

first two flights of the SPACEHAB Space Research Laboratory and the first

flight of the COMmercial Experiment Transporter (COMET).  The same period also

will mark the first flight of a commercial free-flyer research facility, the

Wake Shield Facility, as well as several Space Shuttle secondary payloads and

the launch of the Advanced Communications Technology Satellite (ACTS).  Another

suborbital sounding rocket flight in the Consort series already has been

successfully accomplished with nine payloads on-board in February 1993.

Two attributes of these innovative programs are the relatively small

amount of federal funds expended and the low number of NASA personnel involved.

The associated development of additional spaceflight services has spurred

commercial space infrastructure capabilities while reducing a considerable

backlog and reliance upon the Space Shuttle.

In citing 1993 as The Year of Commercial Space, Greg Reck, Acting

Associate Administrator for the OACT said, "The success of the Centers for the

Commercial Development of Space and their many industry and academic affiliates

should be recognized."

"They are entrepreneurial visionaries, formulating and implementing an

industry-driven program to identify and capitalize on the real possibilities in

space-related commerce," Reck said. "The bright outlook for 1993 will stand as

a landmark for the realization of the commercial potentials of space and as a

benchmark for the development of the space frontier.  The ultimate benefits for

all of us will be more than we can now imagine."

SPACEHAB-01 Commercial Material Science Experiments

Equipment for Controlled Liquid Phase Sintering Experiments

The CMDS, based at the University of Alabama in Huntsville (UAH), has

developed the Equipment for Controlled Liquid Phase Sintering Experiments

(ECLiPSE), making its first long-duration space flight on STS-57 in the

SPACEHAB. The UAH CMDS is a NASA Center for the Commercial Development of Space

(CCDS).

The ECLiPSE experiment investigates the liquid phase sintering (LPS) of

metallic systems.  Sintering is a process by which metallic powders are

consolidated into a metal at temperatures only 50-75 percent of that required

to melt all of the constituent phases.  In LPS, a liquid coexists with the

solid, which can produce sedimentation, thus producing a material that lacks

homogeneity and dimensional stability.  To control sedimentation effects,

manufacturers limit the volume of the liquid.  The ECLiPSE experiment examines

metallic composites at or above the liquid volume limit to more fully

understand the processes taking place and to produce materials that are

dimensionally stable and homogeneous in the absence of gravity.

The ECLiPSE project is focused on composites of hard metals in a tough

metal matrix.  This composite will have the excellent wearing properties of the

hard material and the strength of the tough material.  Applications of such a

composite include stronger, lighter, more durable metals for bearings, cutting

tools, electrical brushes, contact points and irregularly shaped mechanical

parts for high stress environments.

Kennametal, Inc., is an industry partner of the UAH CMDS participating in

the ECLiPSE experiment and has immediate applications for materials

improvements in the ceramic composites tested.  Kennametal is developing

stronger, more durable tool bits.  Wyle Laboratories also is an industrial

partner with the UAH CMDS on the ECLiPSE experiment.

This Shuttle flight of the ECLiPSE payload is building on the experience

of other ECLiPSE flights on suborbital sounding rockets.  Suborbital flights

have provided only 1-3 minutes of sample processing time, and now the longer

flight durations possible on the Shuttle are required.  Because the hardware

was originally designed to fly in suborbital rockets, it is very automated,

requiring little crew interaction.  The UAH CMDS is planning more suborbital

rocket flight testing and future SPACEHAB missions of the ECLiPSE experiment as

part of its sintered and alloyed materials project.

Principal Investigator for ECLiPSE is Dr. James E. Smith, Jr., Associate

Professor and Head, Department of Chemical and Materials Engineering,

University of Alabama in Huntsville.

Gas Permeable Polymeric Materials

The Gas Permeable Polymeric Materials (GPPM) payload is sponsored by the

Instrument Research Division, NASA Langley Research Center (LaRC), through a

joint NASA/industry program initiated in 1987 with OACT. STS-57 and at least

one future space flight of this polymer study program will determine if certain

types of polymers made in microgravity are very different from the same

polymers made simultaneously on the ground.

Plastic materials, which are made of very large molecules called

"polymers," are used in everyday life in many ways.  Some polymers prevent

gases, such as oxygen, from passing through.  These polymers are used in

keeping foods fresh for long periods of time in a refrigerator or freezer.

Other polymers allow one or more gases to pass through.  These polymers, called

gas permeable polymeric materials, also have many uses.

The Gas Permeable Polymeric Materials (GPPM) flight experiment will

determine if certain types of polymers made in low gravity while the Space

Shuttle is in orbit are very different from the same polymers made at the same

time on the ground.

Gas permeable polymeric materials are being developed for many uses.

These include special contact lenses for long-term wear and for use by pilots

and astronauts; medical applications such as dialysis and blood gas monitoring;

control of fermentation and other industrial processes and commercial

production of pure gases.

Another promising use is the development of sensors that will measure any

gas in the air in very small amounts.  In this device, a very thin layer of the

polymer is coated on a sensor.  The polymer allows only the gas which is to be

measured to pass through it.  The sensor then measures the amount of gas that

is present.  These devices will be used in monitoring indoor air quality and in

detecting dangerous gases, such as carbon monoxide.

Gravity may affect many properties of the polymer while it is being made.

As early as 1984, it was suggested that these effects may be eliminated or at

least reduced if the polymer was made in the low gravity of space flight.  A

better understanding of how these polymers are formed also can be learned under

these conditions.  These experiments must be carried out on the Space Shuttle

with the assistance of the astronaut crew because the rates at which the

polymers are formed are very slow.  If these polymers are very different as

expected, many new and improved products will result from them.

The gas permeable polymeric materials being studied by NASA are useful to

the contact lens and industrial gas industries.  In addition, the polymers

being developed by these industries are of special interest to NASA.

A joint NASA and industry program to study polymers made in low gravity

was approved in January 1987 by the NASA Office of Commercial Programs, now the

OACT. The Instrument Research Division at the NASA LaRC is the NASA

organization performing the study.  A leading manufacturer of polymers for

contact lenses, the Paragon Optical Co., of Phoenix, Ariz., is the Industrial

Guest Investigator.

The GPPM flight experiment will be carried out in a sealed aluminum

container called the Polymerization Module, developed by the Systems

Engineering Division at LaRC. The flight Polymerization Module will be

installed in a Commercial Refrigerator/Incubator Module (CRIM) developed by

Space Industries Inc., Webster, Texas. The CRIM, which is a small refrigerator

and oven in a single unit, can maintain temperatures over a range of 4 degrees

C to 40 degrees C for indefinite periods.

Twenty-eight polymer materials will be placed into the flight

Polymerization Module and CRIM in an experiment preparation room at the

SPACEHAB Payload Processing Facility near the NASA Kennedy Space Center.

Identical materials will be placed in another Polymerization Module and

laboratory CRIM. The materials will be kept at 4 degrees C until the start of

the experiment.

The Polymerization Module and CRIM will be used on future missions in the

SPACEHAB Space Research Laboratory or in a middeck locker on the Shuttle. At

least one more mission is being planned by NASA and Paragon researchers.  This

mission also may provide an opportunity for additional industrial guest

investigators to perform an experiment.

Investigations into Polymer Membrane Processing

The Investigations into Polymer Membrane Processing (IPMP) payload will

make its eighth Space Shuttle flight for the Ohio-based Battelle Advanced

Materials Center, a NASA CCDS.

The objective of IPMP is to investigate the physical and chemical

processes that occur during the formation of polymer membranes in microgravity,

such that the improved knowledge base can be applied to commercial membrane

processing techniques.  The STS-57 mission will provide additional data on the

polymer precipitation process to the knowledge base being developed by Battelle

and its industrial partners.

Polymer membranes are porous films which have numerous industrial

applications in separation and filtration devices for pollution control, food,

chemical and drug purification, and kidney dialysis.  The largest potential

market may be the environmental sector.  Space-based polymer membrane

experiments and resulting product improvements could play an important role in

pollution control and may serve to significantly reduce the growing problem of

dangerous gas emissions in the environment.  Amoco Chemical Co., Du Pont and

Bend Industries, Inc., have contributed to this project due to the impact it

may have on gas separation technology.

A two-step process is used frequently to make polymer membranes.  A sample

mixture of polymer and solvents is applied to a casting surface.  The first

step is the evaporation of solvents from the mixture.  In the second step, the

remaining sample is immersed in a fluid bath (typically water) to precipitate

the membrane, form the solution and complete the process.

Results from IPMP's previous seven Shuttle and two sounding rocket flights

indicate that polymers grown in space do show consistently different material

properties than those produced on Earth. The latest flights have produced

polymers that demonstrate the gravitational influence on both the size and

distribution of the pores, which is a determining factor in the ability of the

commercial sector to use polymers for filtration and separation processes.

The results and knowledge gained from all of the IPMP commercially-

applied research flights are being analyzed for potential process-enhancing

applications in existing industrial processing plants.  Through the

dissemination of this information, it is expected there will be increased

interest on the part of U.S. materials, chemical and environmental companies to

grow polymers and other materials in space on a commercial basis.

IPMP Principal Investigator is Dr. Vince McGinniss, Battelle Advanced

Materials Center, Columbus, Ohio. IPMP Program Manager is Lisa McCauley, also

of Battelle.

Liquid Encapsulated Melt Zone

The Liquid Encapsulated Melt Zone (LEMZ) experiment is sponsored by the

Consortium for Commercial Crystal Growth based at Clarkson University, Potsdam,

N.Y., a NASA CCDS. The LEMZ payload is developed by the University of Florida,

Gainesville, an academic affiliate of the consortium.

LEMZ is the first experiment in a series of activities to determine the

feasibility of commercial, space-based production of materials for applications

in the computer, optics and sensor/detector industries.  These materials are

needed for the next generation of high speed optoelectronic digital circuits,

optoelectronic devices and transportation systems.  Researchers at the

University of Florida have produced small gallium arsenide single crystals

encapsulated in molten boron oxide using LEMZ in ground-based experiments.

One of the major thrust areas in materials science is the growth of single

crystals with improved homogeneity (uniform parts), purity and structural

perfection.  However, single crystals grown on Earth have many flaws and

impurities because they are in contact with a container.  The naturally

occurring low gravity conditions of space flight allow large crystals to be

grown without touching a container -- a process called floating zone crystal

growth.

Floating zone crystal growth is expected to result in large single crystals

with purity, compositional homogeneity and structural perfection unattainable

on the ground.

The hardware used in the LEMZ experiment is the Fluid Experiment Apparatus

(FEA) constructed by an industrial partner of the Consortium for Commercial

Crystal Growth, Rockwell International. In orbit, several indium bismuth rods

will be melted in the FEA. Indium bismuth is a low-melting- temperature

compound being used on STS-57 to test the value of liquid encapsulation.  Other

materials of greater commercial interest will be used on future flights of

LEMZ.

The Consortium for Commercial Crystal Growth is teaming with Rockwell

International, the University of Florida, McDonnell Douglas and the State of

Florida Technology Research and Development Authority (TRDA) on the LEMZ

payload.  The LEMZ program is part of the consortium's goal to produce high

quality single crystals of semiconductors, complex oxides, non-linear optical

materials and sensor/detector crystals.

Principal Investigator for LEMZ is Professor Reza Abbaschian, Chairman and

Professor, Materials Science and Engineering Department, University of Florida

at Gainesville.

Support of Crystal Growth Experiment

The Battelle Advanced Materials Center, a NASA CCDS based in Columbus,

Ohio, is sponsoring the Support of Crystal Growth (SCG) Experiment on STS-57.

This experiment is a successor to one conducted in the Spacelab glovebox

flown on the first United States Microgravity Laboratory (USML-1) mission in

July 1992.  SCG supports the Zeolite Crystal Growth (ZCG) experiment also

flying in the SPACEHAB Space Research Laboratory in that it provides the

invaluable information required to establish the ZCG autoclave mixing protocol

so that the resulting crystal growth is optimized.  To do this, SCG will assist

the crew member and principal investigator in determining how the solutions

should be mixed for each of several solution combinations and mixer

configurations.

Ground-based and flight research has shown that mixing of the zeolite

precursor solutions is critical to producing high quality crystals.  Nuclear

magnetic resonance imaging studies, KC-135 flights and analysis of the USML-1

results demonstrate the need to optimize the mixing process (uniform mixing

while minimizing shear).  Determining the proper amount of mixing remains an

empirical science and therefore, must utilize crew observation and judgement

which requires extensive training and experience.

SCG consists of 12 transparent "autoclaves," comparable to the solution

containment portion of the ZCG autoclaves, and a battery-powered screwdriver to

activate the mixing process.  The "autoclaves" are transparent to facilitate

on- board observation by a crew member.  Throughout the activation process, a

crew member will observe the progression and condition of the mixing of the two

solutions.  The crew member will downlink video of each activation/mixing and

consult with the principal investigator regarding application to the ZCG

autoclave activation.  This experiment is critical to the success of the ZCG

experiment and thus, to the success of the Battelle CCDS zeolite program as a

whole.

The Principal Investigator is Dr. Al Sacco, Jr., Worcester Polytechnic

Institute, Worcester, Mass. Lisa A. McCauley, Battelle Advanced Materials

Center, is the flight program manager.

Zeolite Crystal Growth

STS-57 will be the second Shuttle flight of the Zeolite Crystal Growth

(ZCG) payload, developed by the Battelle Advanced Materials Center, Columbus,

Ohio, a NASA CCDS. The ZCG experiment flew on the first United States

Microgravity Laboratory (USML-1) Shuttle mission (July 1992) and the results

appear very positive, and all mission objectives were accomplished.

Zeolite crystals are complex arrangements of silica and alumina which

occur both naturally and synthetically.  An open, three-dimensional,

crystalline structure enables the crystals to selectively absorb elements or

compounds.  As a result, the crystals are highly useful as catalysts, molecular

sieves, absorbents and ion exchange materials.

Zeolites are used for purification and catalytic purposes.  As a purifier,

zeolites work as molecular-scale sieves to remove contaminants from solutions.

If improved zeolites were used in kidney dialysis as a purifier, the time

needed to complete dialysis could be significantly reduced.  Zeolites also

could help in removing impurities in blood molecules, which would be helpful in

blood transfusions.  As catalysts, zeolites aid in making industrial processes

more efficient.  The catalytic procedure used to process crude oil into

gasoline could benefit from improved zeolites, potentially increasing the yield

of gasoline, thus reducing U.S. dependence on foreign oil sources.  Amoco

Chemical Co. and Du Pont are Battelle's industrial affiliates on this flight of

ZCG.

Ultimately, space-produced zeolite crystals are expected to be larger and

of higher quality than their ground-produced counterparts, providing tremendous

industrial potential for such crystals.  The zeolites produced in microgravity

are considered high value-added products and will be scaled up to production

quantities using the space station and recoverable orbital systems launched by

expendable launch vehicles.

The nucleus of the experiment will consist of 38 autoclaves, each

containing two solutions in separate chambers and a screw-activated mixing

assembly.  To activate the experiment, a crew member will operate the screw

assembly with a battery-powered screwdriver, which mixes the two zeolite

precursor solutions.  By repeating this process several times, proper mixing of

the two solutions can be obtained (several different mixing devices are to be

used on this mission).  Results from the Support to Crystal Growth experiment,

also flying in the

SPACEHAB Space Research Laboratory, will be used to determine the appropriate

mixing protocol for each autoclave.

Principal Investigator for ZCG is Dr. Albert Sacco, Jr., Worcester

Polytechnic Institute, Worcester, Mass. ZCG Program Manager is Lisa McCauley,

Battelle Advanced Materials Center.

SPACEHAB-01 Commercial Life Science Experiments

ASTROCULTUREt

The ASTROCULTUREt payload is sponsored by the Wisconsin Center for Space

Automation and Robotics (WCSAR), a NASA CCDS located at the University of

Wisconsin, Madison.

Currently, no satisfactory plant growth unit is available to support

long-term plant growth in space.  Increases in the duration of space missions,

including stays on the space station, have made it necessary to develop plant

growth technology that could minimize the cost of life support while in space.

Plants can reduce costs of providing food, oxygen and pure water and also lower

costs of removing carbon dioxide in human space habitats.

Before plants can be grown in the ASTROCULTUREt unit, however, a series of

experiments are being conducted on the Space Shuttle to evaluate the critical

subsystems essential for the space-based applications which also will have

tremendous uses on Earth, such as improved dehumidification/humidification

units, water-efficient irrigation systems and energy-efficient lighting systems

for plant growth.

Results from the flight of the first ASTROCULTUREt experiment on STS- 50,

the flight of the first United States Microgravity Laboratory (USML-1) in July

1992, indicate that the experiment successfully achieved all of its goals, and

experiment results are expected to provide new information dealing with the

performance of water and nutrient delivery in space.  ASTROCULTUREt has been

approved for four more Shuttle flights.

The ASTROCULTUREt unit consists of a covered cavity with two growth

chambers containing inert material that serves as the root matrix; a water

supply system consisting of a porous stainless steel tube embedded into the

matrix, a water reservoir, a pump and appropriate valves for controlling the

pressure flow of water through the stainless steel tube; a water recovery

system consisting of the same components as the water supply system; and a

microprocessor system for control and data acquisition functions.  The flight

hardware for this mission is self-contained in a SPACEHAB locker and weighs

approximately 50 pounds.

This flight of ASTROCULTUREt will evaluate the performance of other

important aspects of the water and nutrient delivery system not studied during

the first space experiment.  In addition, the STS-57 experiment will provide

information on the performance of a light emitting diode (LED) lighting system

during an extended period of microgravity.  A preliminary evaluation of the LED

system was made on the Consort-5 sounding rocket flight in November 1992.

In orbit, the water supply and recovery systems will be activated to

initiate circulation of a nutrient solution through the porous tubes.

Subsequently, the solution will move through the wall of each porous tube into

the matrix by capillary forces.  In the matrix, the small pores will be filled

with the solution and the large pores with air, thereby providing a

non-saturated state.  The recovery system will operate at several pressure

levels to determine the rate at which the solution will move through the matrix

and the capacity of the supply system to provide the solution to the matrix.

The amount of solution transferred from the supply reservoir to the

recovery reservoir will be monitored, and data collected by the computer will

indicate the supply system's overall capacity for replacing water and nutrients

removed by plants growing in microgravity.

The current industry affiliates on ASTROCULTUREt include Automated

Agriculture Assoc., Inc., Dodgeville, Wisc.; Biotronics Technologies, Inc.,

Waukesha, Wisc.; Quantum Devices, Inc., Barveveld, Wisc.; and Orbital

Technologies Corpo., Madison, Wisc. Principal Investigator is Dr. Raymond J.

Bula, WCSAR.

BioServe Pilot Laboratory

The BioServe Pilot Laboratory (BPL) is sponsored by BioServe Space

Technologies, a NASA CCDS based at the University of Colorado in Boulder.

The BPL will play an important role in providing the commercial and

scientific communities affordable access to space for material and life

sciences research.  The main focus of the project is to provide a "first step"

opportunity to companies interested in exploring materials processing and life

science experiments in space.  The notion behind the project is to allow

industry a mechanism for entry level "proof of concept" flights.  Thus, the BPL

is a crucial screening device for more complex, targeted space research and

development activities.

The BPL payload will support investigations in a wide variety of life

sciences areas with primary emphasis on cellular studies.  For STS-57, two

series of investigations will be carried out on bacterial products and

processes.

One investigation series examines Rhizobium trifolii behavior in

microgravity.  Rhizobia are special bacteria that form an intimate and

advantageous, or symbiotic, relationship with plants.  The bacteria infect the

plants early in seedling development to form nodules on the plant roots.  The

bacteria in these nodules derive nutritional support from the plant while in

turn providing the plant with nitrogen fixed from the air.  Plants that form

such relationships with rhizobia are called legumes and include alfalfa, clover

and soybean.  Such plants do not require synthetic fertilizers to grow.  In

contrast, many important crop plants such as wheat and corn are dependent on

synthetic fertilizers since they do not form symbiotic relationships with

rhizobia.

The experimental system employing Rhizobium trifolii is a model that can

be used to better understand the multi-step process associated with rhizobia

infection of legumes.  Once understood, it may become possible to manipulate

the process to cause infection of other crop plants.  The potential savings in

fertilizer production would be tremendous.

One of the commercial goals of the BioServe center is to determine whether

microgravity might be exploited as a tool for rhizobial infection of

significant crop plants.  This BPL investigation, along with complimentary

investigations in BioServe's Commercial Generic Bioprocessing Apparatus (CGBA)

also flying in the SPACEHAB Space Research Laboratory, should provide the data

needed to address this goal.

Another series of investigations being flown in the BPL concerns the

bacteria E. Coli. These bacteria are normally found in the gastrointestinal

tracts of

mammals, including humans.  E. Coli have been well studied as a model system

for bacterial infection and population dynamics and in genetics research.  With

regard to commercial application, the genetic material in E. Coli has been

manipulated to produce bacteria capable of secreting important pharmaceutical

products.  These bacteria also serve as a model for bacteria used in waste

treatment and water reclamation.

For STS-57, these bacteria are being studied to determine changes in

growth and behavior that occur as a consequence of exposure to microgravity.

The commercial objectives for this investigation include understanding and

controlling bacterial infection in closed environments, exploiting bacteria and

other micro-organisms in the development of ecological life support systems and

waste management, and determining the opportunity for enhanced genetic

engineering and enhanced pharmaceutical production using bacterial systems.

Yet another BPL investigation examines a biomedical test model based on

cells derived from a frog kidney.  This investigation is intended to provide

insight into effects of microgravity on cell behavior - especially cell

division.  Gravitational effects on such cell systems may be used as models of

diseases or disorders that occur on Earth. For STS-57, the kidney cell system

is being examined to determine feasibility for use as such a test model.

On STS-57, the BPL will consist of 40 Bioprocessing Modules (BPMs) stowed

in a standard locker in the SPACEHAB Space Research Laboratory. The BPMs will

contain the biological sample materials.  The stowage locker also will contain

an Ambient Temperature Recorder (ATR) which will provide a temperature history

of the payload throughout the mission.

Each BPM consists of three syringes held together on an aluminum tray.

Generally, the center syringe in each BPM will be loaded with the cell culture

system.  Adjacent syringes will contain process initiation and termination

fluids, respectively.  A three-way valve is mounted on the trays which permits

fluid transfer from one syringe to the next.  The syringes, valve tubing and

fittings provide for containment of the sample materials.  The hardware is

further enclosed in heat-sealed plastic bags to provide additional levels of

containment.

For most of the investigations, simultaneous ground controls will be run.

Using similar hardware and identical sample fluids, ground personnel will

activate and terminate BPMs in parallel with the flight crew.  Synchronization

will be accomplished based on voice downlink from the crew.  Ground controls

will be conducted at the SPACEHAB Payload Processing Facility at Cape

Canaveral, Fla.

Dr.  Marvin Luttges, Director of the BioServe CCDS, is Program Manager.

Drs. Louis Stodieck and Michael Robinson, also of BioServe, are responsible for

mission management.

Commercial Generic Bioprocessing Apparatus

The Commercial Generic Bioprocessing Apparatus (CGBA) payload is sponsored

by BioServe Space Technologies, a NASA CCDS located at the University of

Colorado, Boulder. The purpose of the CGBA is to allow a wide variety of

sophisticated biomaterials, life sciences and biotechnology

investigations to be performed in one device in the low gravity environment of

space.

During the STS-57 mission, the CGBA will support 27 separate commercial

investigations, which can be loosely classified in three application areas:

biomedical testing and drug development, controlled ecological life support

system (CELSS) development and agricultural development and manufacture of

biological-based materials.

Biomedical Testing and Drug Development -- To collect information on how

microgravity affects biological organisms, the CGBA will include eight

biomedical test models.  Of the eight test models, four are related to immune

disorders: one will investigate the process in which certain cells engulf and

destroy foreign materials (phagocytosis); another will study bone marrow cell

cultures; two others will study the ability of the immune system to respond to

infectious-type materials (lymphocyte and T-cell induction); and one will

investigate the ability of immune cells to kill infectious cells (TNF-Mediated

Cytotoxicity).

The other four test models -- which are related to bone and developmental

disorders, wound healing, cancer and cellular disorders -- will investigate

bone tissue, brine shrimp development, inhibition of cell division processes,

stimulation of cell division processes and the ability of protein channels to

pass materials through cell membranes.

Test model results will provide information to better understand diseases

and disorders that affect human health, including cancer, osteoporosis and

AIDS. In the future, these models may be used for the development and testing

of new drugs to treat these diseases.

Controlled Ecological Life Support System (CELSS) Development -- To gain

knowledge on how microgravity affects micro-organisms, small animal systems,

algae and higher plant life, the CGBA will include 13 ecological test systems.

Two of the test systems will examine miniature wasp development.  Seven

separate studies will concern seed germination and seedling processes related

to CELSS development.  Another three test systems will investigate bacterial

products and processes and bacterial colonies for waste management

applications.  Finally, one other system will study new materials to control

build- up of unwanted bacteria and other micro-organisms.

Test system results will provide research information with many commercial

applications.  For example, evaluating higher plant growth in microgravity

could lead to new commercial opportunities in controlled agriculture

applications.  Test systems that alter micro-organisms or animal cells to

produce important pharmaceuticals could later be returned to Earth for

large-scale production.  Similarly, it may be possible to manipulate

agricultural materials to produce valuable seed stocks.

Biomaterials Products and Processes -- The CGBA also will be used to

investigate six different biomaterials products and processes.  Two

investigations will attempt to grow large protein and RNA crystals to yield

information for use in commercial drug development.  A third investigation will

evaluate the assembly of virus shells for use in a commercially-developed drug

delivery system.

Another experiment will use bacteria to form magnetosomes (tiny magnets) for

potential use in advanced electronics.  Two other investigations will use

fibrin clot materials as a model of potentially implantable materials that

could be developed commercially as replacements for skin, tendons, blood

vessels and even cornea.

Results from the 27 investigations will be carefully considered in

determining subsequent steps toward commercialization.  STS-57 marks the third

of six CGBA flights.  Future flights will continue to focus on selecting and

developing investigations that show the greatest commercial potential.

For most of the investigations, simultaneous ground controls will be run.

Using identical hardware, samples fluids and materials, ground personnel will

activate and terminate FPAs in parallel with the flight crew.  Synchronization

will be accomplished based on indications from the crew as to when specific

GAPs are operated.  A temperature controlled environment at the SPACEHAB

Payload Processing Facility (SPPF), Cape Canaveral, Fla., will be used to

duplicate flight conditions.

Dr.  Marvin Luttges, Director of the BioServe CCDS, is program manager for

CGBA. Drs. Louis Stodieck and Michael Robinson, also of BioServe, are

responsible for mission management.

Organic Separation

The Consortium for Materials Development in Space (CMDS) based at the

University of Alabama in Huntsville (UAH), has developed the Organic Separation

(ORSEP) payload for flight on STS-57. The UAH CMDS is a NASA CCDS.

ORSEP offers the commercial and scientific communities the opportunity to

separate cells and particles by a mechanistic technique unavailable on Earth.

The potential commercial value of separations includes the opportunity to

culture cell subpopulations on return to Earth, the revelation that

subpopulations exist and as is the case for protein crystal growth in space, in

scientific study of the purified samples.

The ORSEP hardware was built by Space Hardware Optimization Technology

(SHOT), Inc., Floyd Knobs, Ind. It is of considerably lower cost than existing

phase partitioning devices, and SHOT may be able to capture a good portion of

the commercial market on Earth. The hardware is a modular design which can be

configured for use with the Shuttle middeck, Spacelab, Get Away Special

canisters, the SPACEHAB Space Research Laboratory, sounding rockets and

parabolic flight aircraft.

It is a multi-sample, multi-step, fully automated device that separates

non- biological particles, as well as biological cells, particles,

macromolecular assemblies and organelles in low gravity via partitioning in

liquid polymer two- phase systems.  The hardware has been designed to perform

partitioning in microgravity for a long duration because 2-3 hours are required

for each separation step.  Commercial interests were factored into the hardware

design in its multi-sample capability that offers temperature control and

sterility.

On STS-57, the SPACEHAB Space Research Laboratory makes available

continuous power, which allows for constant heating/cooling for the experiment

while the vacuum of space provides thermal insulation.  As a result of these

design features, four samples can be processed through 12 purification steps

while being held at 4 degrees C in a sterile environment.

Four particle samples will be processed on STS-57 in the ORSEP apparatus.

Delicate biological materials have been avoided in order to verify that the

separations are due to the operation of the ORSEP rather than an unexpected

response of a sensitive sample, such as to a launch delay or a delay in the

recovery of the payload.

The CCR CCDS is using ORSEP to study the separation of organic materials

from unwanted impurities.  When making any type of drug or any material to be

used for medical purposes, purity is an extremely important characteristic to

the ultimate usefulness of the product.  Enhanced purity will enable smaller

quantities of drugs to be used, with reduced chances of unwanted side effects.

When certain fluids containing pharmaceuticals are mixed in space, the two

fluids will separate, much like oil and water.  During this process, impurities

will often separate out and be located in the boundary between the two fluids.

They then may be removed, leaving the ultra-pure desired products.

The Principal Investigator for ORSEP in Dr. James M. Van Alstine,

University of Alabama in Huntsville.

Protein Crystal Growth

The Center for Macromolecular Crystallography (CMC), based at the

University of Alabama in Birmingham (UAB), is sponsoring Protein Crystal Growth

(PCG) experiments on STS-57. The CMC is a NASA CCDS, which forms a bridge

between NASA and private industry to stimulate biotechnology research for

growing protein crystals in space and offers other protein crystallography

services to a wide range of pharmaceutical, chemical and biotechnology

companies.

The objective of space-based protein crystal growth experiments is to

produce large, well-ordered crystals of various proteins.  These crystals will

be used in ground-based studies to determine the three-dimensional structures

of the proteins.  These experiments also continue to investigate how to control

and optimize protein crystal growth in order to reduce uncertainties or risks

associated with using this space-based process as a vital and enabling

technology for many critical areas.

Since proteins play an important role in everyday life -- from providing

nourishment to fighting diseases - research in this area is quickly becoming a

viable commercial industry.  Scientists need large, well-ordered crystals to

study the structure of a protein and to learn how its structure determines a

protein's functions.

The technique most-widely used to determine a protein's three- dimensional

structure is x-ray crystallography, which requires large, well-ordered crystals

for analysis.  Crystals produced on Earth often are large enough to study, but

they usually have numerous gravity-induced flaws.  However, space-produced

crystals tend to have more highly-ordered structures that significantly

facilitate x- ray diffraction studies.

Studies of such crystals not only can provide information on basic

biological processes, but they may lead to the development of food with higher

protein content, highly resistant crops and - of great importance - more

effective drugs.  By studying the growth rates of crystals under different

conditions, scientists can find ways to improve crystal growth in microgravity,

thus providing higher- quality crystals for study and the ability to produce

satisfactory protein crystals that are hard or impossible to grow on Earth. For

these reasons, the CMC has conducted protein crystal growth experiments on 17

Shuttle missions including STS-57.

Vapor Diffusion Apparatus and Crystallization Facility Experiments

There are three PCG experiments on STS-57, two of which are contained in

thermal control enclosures called Commercial Refrigerator/Incubator Modules

(CRIM).  One of the CRIM will hold three Vapor Diffusion Apparatus (VDA) trays

at a temperature of 22 degrees C. One side of each VDA tray holds 20 double-

barreled syringes, while the other side holds plugs that cap the tips of the

syringes.  Protein solution will be stored in one barrel of each syringe, and

the other barrel will house precipitant solution.  A reservoir of concentrated

precipitant solution surrounds each syringe inside the crystal growth chamber.

A second CRIM contains the Protein Crystallization Facility (PFC).  This

equipment will utilize changing temperature as a means of producing protein

crystals in microgravity.  The PFC apparatus consists of four containers which

can individually hold as much as 500 ml of protein solution.  The buffered

protein solution is initially maintained at a temperature which will not induce

crystallization.  Once in orbit, the CRIM is programmed by the crew to begin

slowly changing temperature on a temperature profile which will optimize the

crystallization process.

Due to each protein's short lifetime and the crystals' resulting

instability, the protein crystal growth experiments will be retrieved within 3

hours of landing and returned to the CMC for post-flight analyses.

Direct-Control Protein Crystal Growth

A third crystallization system on STS-57 will test new protein crystal

growth space hardware.  The crystallization system will consist of six syringes

in a VDA tray and will be contained in a Thermal Enclosure System (TES) which

occupies two SPACEHAB lockers and provides a hermetically-sealed and thermally-

controlled environment.  Within the TES, the Crystal Observation System (COS)

will allow real-time crew monitoring during the crystal growth period.

The COS video system will provide individual experiment observation via

video cameras mounted to allow viewing of each growth chamber.  The system will

allow crew members to focus from the front of the droplet to the back, thereby

providing the ability to detect individual crystals, study their growth rate

and morphology, and continually observe the crystals on board or send video

downlink images of the crystals to scientists in the Payload Operations and

Control Center (POCC).  This new hardware will provide critical information

regarding differences in crystal growth rates and vapor equilibration times in

the microgravity environment.

The COS in its hermetically sealed thermally controlled environment

represents a significant step towards the dynamic control of the several

variables that affect protein crystal growth.  By developing the ability to

create tailor made, monitored and programmed environments for each sample, such

systems are expected to be able to significantly reduce the risks involved in

growing valuable crystals of the most troublesome proteins.

Industrial samples will be flown in each of the protein crystal growth

hardware - the VDA, PFC and COS - including malic enzyme from Upjohn

Pharmaceuticals, recombinant human insulin from Eli Lilly and Company and

alpha-thrombin from Du Pont Merck Pharmaceuticals.

The CMC has flown over 50 different types of proteins in space, seeking

protein structure data and techniques for predictable enhancement by growth in

microgravity.  Crystallographic analysis has revealed that on average 20

percent of proteins grown in space are superior to their Earth-grown

counterparts.  As a result of advances made by the CMC in its microgravity

crystallographic technologies, 40 percent of the proteins flown on the first

United States

Microgravity Laboratory (USML-1) mission in July 1992, yielded diffraction size

crystals, several of which were superior to any previously grown on Earth.

With continued research, the commercial applications developed using

protein crystal growth have phenomenal potential, and the number of proteins

that need study exceeds tens of thousands.  Current research with the aid of

pharmaceutical companies may lead to a whole new generation of drugs, which

could be able to help treat diseases such as cancer, rheumatoid arthritis,

periodontal disease, influenza, septic shock, emphysema, aging and AIDS. These

possibilities plus drugs and other products for agriculture, proteins for

bioprocessing in manufacturing processes and waste management and other

biotechnical applications, represent critical capabilities for dealing with the

future of the world.

A number of companies are participating in the CMC's protein crystal

growth project including BioCryst Pharmaceuticals, Inc., Eli Lilly & Co.,

Schering- Plough Research, Du Pont Merck Pharmaceuticals, Sterling Winthrop

Inc., Eastman Kodak Co., The Upjohn Co., Smith Kline Beecham Pharmaceuticals

and Vertex Pharmaceuticals, Inc. Principal Investigator for the protein crystal

growth experiments is Dr. Charles E. Bugg, Director of the UAB CMC.

Physiological Systems Experiment

The Center for Cell Research (CCR), a NASA CCDS based at Pennsylvania

State University, is sponsoring the third Physiological Systems Experiment

(PSE) payload on STS-57.

The PSE-03 payload is the result of a collaboration by the CCR and the

Space Dermatology Foundation (SDF), a group of dermatologists and scientists

concerned with the future implications and effects of space travel and

habitation on the human skin.  It will investigate the role of two growth

factors involved in accelerating or enhancing tissue repair.  Microgravity

appears to slow down the normal tissue repair process.  The slow down mimics

changes associated with conditions on Earth.

The objective of PSE-03 on STS-57 is to increase the dermatologic database

and to demonstrate the value of microgravity in dermatologic studies.  The

results of the experiment will be shared with the medical community and the

pharmaceutical and biotechnical industries through the SDF. The SDF plans to

develop and maintain a database of space-related dermatology and dermatologic

conditions, which will be the only one of its kind.

PSE-03 is a first step in exploring how microgravity can improve the

understanding of the ways growth factors regulate tissue repair and

regeneration.  The knowledge gained in these studies may be useful in the

development of new medicines for burn victims, diabetics, elderly surgical

patients, bed sore sufferers or other skin injury patients for whom healing is

slow and difficult.

The results also may provide additional information about how the basic

gene processes underlying blood vessel and soft tissue formation are turned on

and off.  In addition, the experiment may have direct application in space by

helping dermatologists devise therapies to treat astronauts who receive skin

and/or soft tissue injuries during prolonged space flight.

Prior to space flight, the growth factors will be implanted in six

different areas in each of the 12 male adult rats.  The rats will be housed in

groups of six in two completely self-contained units equipped with food and

water.  Fans will circulate cabin air through the units.  The units, known as

Animal Enclosure Modules (AEM), were developed by NASA's Ames Research Center,

Mountain View, Calif. The AEM hardware provides the rats with appropriate life

support throughout the mission and returns them in good health at the end of

the mission.  No interaction with the crew is required in orbit, however, clear

plastic covers on the AEM hardware will permit the crew to visually inspect the

condition of the rats.

When returned, the tissues surrounding the implantation sites will be

examined to determine the effect of the growth factors.  Those tissues and

others will be studied by researchers affiliated with the CCR, SDF and with

pharmaceutical and biotechnical companies.  The experiment designers expect the

7 day mission to provide sufficient exposure to microgravity to study the

initial phases of tissue repair and the manner in which the two growth factors

affect the process.

PSE-01, conducted in 1990 with Genentech Inc., San Francisco, increased

basic scientific knowledge regarding human bone and muscle disease and immune

cell deficiency.  PSE-02, conducted in 1992 with Merck & Co., Inc., West Point,

Penn., tested a developmental drug designed to counteract the effects of

osteoporosis.

Dr.  W. C. Hymer is Director of the Center for Cell Research at

Pennsylvania State University and co-investigator for PSE-03. Dr. William W.

Wilfinger is the CCR Director of Physiological Testing. Dr. Steven R. Kohn,

President, Space Dermatology Foundation, is the SDF representative.

SPACEHAB-01 Johnson Space Center Investigations

Application Specific Pre-programmed Experiment Culture System

The Application Specific Pre-programmed Experiment Culture (ASPEC) System

is sponsored by the Medical Sciences Division, Space and Life Sciences

Directorate, NASA Johnson Space Center (JSC), Houston. The ASPEC system is a

part of the bioreactor project which is aimed at developing a series of

hardware concepts for facilitating the development of human cells and tissue

cultures in the weightless or microgravity environment of space flight where

cells can grow in all directions for extended periods of time.

Medical science is unable to grow large high-fidelity human tissue models

in Earth's gravity.  Microgravity or its emulation will allow cells to be

suspended for long-term growth and development.  Tissues grown in this way are

useful in testing chemotherapeutic protocols, understanding growth requirements

and treating specific medical maladies.  Potential medical science spin-offs

include investigations of viral growth, cancer models and therapeutics, and

transplantation tissue.

"A near-term goal is to test the equipment and its impact on a growing

colon cancer," said Glenn Spaulding, Manager of the biotechnology program at

the Johnson Space Center. "From this study, we will be able to refine culture

techniques here and in space."

The ASPEC system is a set of self-contained cell growing and cell

maintenance units for use in space flight experiments.  Cell cultures may be

initiated in the device or mature cell cultures may be transferred into the

ASPEC, which can maintain a cell culture experiment for as long as 14 days.

The ASPEC system will carry several culture vessels on STS-57, its first

space flight.  Each culture vessel has the potential of carrying one complete

experiment.  On STS-57 the experiment is being flown with colon cancer cells to

be grown in the chamber and brought back for study.  On Endeavour's last

mission in January, the culture chamber was flown as a testbed to demonstrate

movement of fluid through the unit to provide constant nutrients to growing

cells.

The hardware of the ASPEC system includes three ASPEC units, an ASPEC

power cable, a locker with a modified door and packing foam.  Each ASPEC unit

has an independent plumbing and sensor system to regulate temperature and pH

and to provide a fresh growth medium and serum to the cells as needed.

The STS-57 crew will routinely check power indicators and airflow through

the ASPEC units and clean the vent screens as necessary.  The crew also will

take still photographs of the system configuration.  The shutdown procedure

will be initiated by the crew.  This will begin an automated process for

removing experiment materials from the reactor chamber, chilling the removed

samples to 10 degrees C to prevent protein breakdown and other degradation and

injecting formalin into the vessels to "fix" the remaining cells.

A near term goal of the experiment is to provide toxicology testing that

will identify the potential long-duration hazards on shorter Shuttle missions.

This forms a bridge between identifying specific toxicants and their biological

impacts.

On the Shuttle, ASPEC will serve as the "foundation experiment" for the

space station.  Growing cells to full maturity may take several months, which

can only be done on long-duration flights aboard the station.

Principal Investigator for ASPEC is Dr. Glenn Spaulding, Medical Sciences

Division, Space and Life Sciences Directorate, JSC.

Charged Particle Directional Spectrometer

The Charged Particle Directional Spectrometer (CPDS) experiment on STS- 57

is sponsored by the Solar System Exploration Division, Space and Life Sciences

Directorate, Johnson Space Center (JSC).  The CPDS performs the functions of

both a research instrument and an operational monitor.  It detects and records

the many different types of nuclear radiation that bombard an orbiting space

vehicle.  In so doing, information is gathered about the characteristics of

these particles at the orbital altitude, and a record is made of the amount and

type of exposure the crew members receive.

The particles come from two groups.  First are particles trapped in orbit

around the Earth by the Earth's magnetic field.  These particles mainly consist

of protons, although other varieties of the nuclear population also are

present.  The second are intergalactic particles, or cosmic rays, that happen

to be passing by the Earth. All of these particles can be considered orbital

debris on a nuclear scale.

Knowledge of the particle's type, energy and direction is of interest to

basic research in physics.  Medical researchers can use much of the same

information, but in addition, they are concerned with the linear energy

transfer of the particle, particularly in living tissue such as human beings.

The measurement indicates how much potential damage the particles do as they

transverse through living beings.  Such information is necessary to help

determine guidelines that will ensure the long-term health and safety of

astronauts.  Several CPDS instruments are intended to be included as standard

equipment on the space station.

The CPDS experiment consists of three different instruments: a pair of

Area Passive Dosimeters (APDs), the Tissue Equivalent Proportional Counter

(TEPC) and the actual CPDS apparatus.  The APDs are routinely flown on Space

Shuttle missions.  They are similar to film strips.  Particles which strike the

strips leave a distinctive signature.  The strips are analyzed after the flight

and give a good indication of total dosage received during the flight.

The TEPC utilizes a detection element that absorbs particle energy in a

manner similar to living tissue.  The data received from this instrument are

particularly useful in assessing possible hazards to the crew.  And since the

TEPC is an active electronic instrument, a time record of when each particle

strikes is maintained.  TEPCs have flown on several Shuttle missions and have

been instrumental in, among other things, determining the configuration of the

South Atlantic Anomaly.

The CPDS apparatus is the most sophisticated instrument of the experiment

hardware.  It consists of several layers of different types of detectors.  The

various detectors have different characteristics to enable the instrument to

gather as much data as possible from each particle strike.  One important new

feature of the CPDS is its ability to determine the direction of individual

particles.  Particle flux is believed to be more intense in some directions

than in others.  If this is confirmed, future spacecraft designs may position

crews to receive maximum shielding from the spacecraft structure.

The CPDS experiment is completely housed in a SPACEHAB locker mounted high

on the aft bulkhead.  It requires only electrical power to be operational.  The

instruments are activated by the crew as soon after reaching orbit as practical

and are turned off just before the descent back to Earth. Data are retained in

internal memories and are read out and analyzed post-flight.

Principal Investigator for CPDS is Dr. Gautam D. Badwar, Solar System

Exploration Division, Space and Life Sciences Directorate, JSC.

Human Factors Assessment

The Human Factors Assessment (HFA) experiment is being conducted on STS-57

by the Crew Interface Analysis Section of the Flight Crew Support Division,

Space and Life Sciences Directorate, Johnson Space Center (JSC).  The primary

concerns of human factors engineers at JSC are the investigation and evaluation

of human-machine and human-environment interfaces unique to spaceflight which

affect crew productivity and ultimately mission success.

During the mission, data will be collected on three different aspects of

crew activity in space: the acoustic and lighting environments of the orbiter,

ease of movement -- or translation -- through the middeck-to-SPACEHAB transfer

tunnel and the use of electronic procedures to perform tasks.  The hardware to

facilitate data collection includes a MacIntosh Powerbook computer with a voice

recognition system using Supercard displays and for environmental measurements,

a B&K Type 2231 Modular Precision Sound Level Meter and a Minolta Photographic

Spotmeter.

Evaluation of the acoustic and lighting environments (HFA-SOUND and HFA-

LIGHT, respectively) seeks to gain objective and subjective measures of the

noise and lighting environments during the STS-57 mission and also will assess

any effects on crew performance attributable to these environments.  HFA- SOUND

additionally seeks to determine if noise is more bothersome to the crew as the

mission progresses and to compare noise levels and crew-perceived annoyance

across missions.

The HFA-SOUND and -LIGHT investigations will determine whether current

spacecraft acoustic and lighting design criteria are being met, and what levels

are indeed acceptable to the crew during the mission to minimize negative

effects of these environments on crew performance.  Ten 1/3 octave sound level

and several lighting measures will be taken in the SPACEHAB Space Research

Laboratory, the middeck and the flight deck.  This investigation will help

identify noise-producing hardware and problematic lighting configurations that

are particularly detrimental to crew member performance.

The investigation assessing translation through the transfer tunnel (HFA-

TRANS) seeks to assess the SPACEHAB tunnel adapter and hatch designs for ease

of crew translation and equipment transfer between the middeck and the SPACEHAB

Space Research Laboratory.

HFA-TRANS data will provide basic information on translation speeds in the

weightless environment of space and techniques which will contribute to

training and timelining of tasks for subsequent SPACEHAB and Spacelab missions

and on the space station.  The data also will be compared to data collected on

crew translation through the Spacelab tunnel during STS-40 (June 1991) and

STS-47 (September 1992).

Comments on the various features of the SPACEHAB adapter and tunnel

designs will contribute to recommendations for the design of more efficient

translation areas in the future.  Translation video will be collected early and

late in the mission.

The electronic procedures portion of this experiment (HFA-EPROC) seeks to

facilitate future use of electronic flight procedures.  Crew performance with

electronic procedures must be at least equal to that achieved with paper

procedures.

Current EPROC research will help define baseline paper procedures

performance and identify specific strong and weak points of both paper and

computer procedures.  The current research also will help define specific ways

to achieve improved performance with computer procedures.

EPROC will be of particular significance for future, longer-duration

missions which will increasingly rely on electronic procedures since they are

more easily launched, updated in flight and offer automatic or on-request

capabilities not available with paper.  The development of human factors design

guidelines for such electronic procedures will be increasingly important for

future space missions.

The HFA-EPROC experiment consists of two tasks: a computer task which will

simulate a space station propulsion system task and a non-computer task

performed in conjunction with the Tools and Diagnostic Systems Soldering

Experiment. Each task will be performed with paper and computer-based

procedures.

The Principal Investigator for HFA is Sue Adam, Flight Crew Support

Division, Space and Life Sciences Directorate, JSC.

Neutral Body Posture

The Space and Life Sciences Directorate, JSC, is sponsoring the Neutral

Body Posture (NBP) experiment on STS-57. NBP will investigate the changes in

posture of the human body over the course of a space mission.  Previous space

missions have shown that in addition to lengthening of the spine, posture takes

on a configuration unique to spaceflight.  The data from NBP will be useful in

the design of future space facilities, workstations and hardware, especially

since the last in-depth study of this nature was conducted during the Skylab

program in the early 1970s.

A minimum of two STS-57 crew members will be evaluated.  As time allows,

data may be collected on additional crew members.  The crew members to be

evaluated will wear a special sleeveless T-shirt and be photographed with

orbiter camcorders and 35mm cameras mounted roughly along orthoganal axes with

respect to the vehicle.  The crew members under evaluation will assume a

relaxed position while photos are collected.  This process will be performed

both early and late in the mission.

Principal Investigator for NBP is Frances E. Mount, Flight Crew Support

Division, Space and Life Sciences Directorate, JSC.

Tools and Diagnostic Systems

The Tools and Diagnostic Systems (TDS) experiment is sponsored by the

Space and Life Sciences Directorate, JSC. The objective of TDS is to

demonstrate the maintenance of experiment hardware on-orbit and evaluate the

adequacy of its design and the crew interface.  The TDS experiment on STS-57

will mark the first demonstration of soldering on an American space mission.

The TDS experiment is a group of equipment selected from the tools and

diagnostic equipment to be supplied to the space station program.  These tools

and diagnostic equipment will provide the space station program with on- orbit

diagnostic and repair capability.  The hardware consists of off-the-shelf

equipment modified to perform acceptably in the space environment.

There are two parts to TDS: the Soldering Experiment (SE) to demonstrate

practical soldering in the microgravity environment and to evaluate the use of

a new restraint configuration for crew members performing precise tasks and the

Diagnostic Equipment (DE) experiment to demonstrate microgravity maintenance

capabilities using state-of-the-art diagnostic equipment.

In the SE, a crew member will solder a printed circuit board containing 45

connection points, then de-solder 35 points on a similar printed circuit board.

The soldering work station consists of a glovebox to contain debris, mounted on

a SPACEHAB-supplied work bench, where the circuit boards will be held in a

clamp, which is in turn mounted on an experiment rack.  Of interest to

investigators are the techniques used by the crew member and the quality of the

work of the crew member, which is dependent on the ability of the crew member

to properly place the solder on the heated connection point.

The crew member also will be asked to evaluate two types of foot

restraints used while performing the SE. One restraint consists of adjustable

foot loops similar to the current Space Shuttle design.  The other is an

arrangement of foot restraint bars designed for use on the space station.  Two

crew members will perform the experiment twice, but it may be repeated if time

allows.

The DE experiment will operate the development unit for the space station

diagnostic equipment caddy.  This diagnostic caddy contains a function sweep

generator, a logic analyzer/oscilloscope and a multimeter.  This combination of

equipment is able to produce an analog or digital test signal, which is input

to the test equipment, and captures and displays the resultant output.

The work station consists of the SPACEHAB-supplied work bench mounted on a

rack, which provides a recess into which the diagnostic equipment caddy will be

mounted.  A frequency counter also is supplied for analysis.

As part of the DE experiment, a failure in flight will be simulated, after

which the Payload General Support Computer (PGSC) will uplink a troubleshooting

procedure, a test equipment configuration file and a test setup diagram.  The

file to configure the test equipment will allow the complex diagnostic

equipment setup to be performed by the support crew on the ground.  Then the

flight crew will perform the procedures and record and downlink the results.

The ground crew will analyze the data obtained and uplink files for a

"fix" to the problem for the crew.  Upon completion of the repair, the test

will culminate with successful performance of the frequency counter's function.

The Principal Investigator for TDS is Jackie Bohannon, Flight Crew Support

Division, Space and Life Sciences Directorate, JSC.

SPACEHAB-01 Payloads Space Station Experiment

Environmental Control and Life Support System Flight Experiment

NASA's space station office in Reston, Va., is sponsoring the

Environmental Control and Life Support System (ECLSS) Flight Experiment (EFE)

to test components of the water recycling system being developed for the space

station.

With a projected rate of four crew members at a time aboard the space

station, they will use about 50 pounds of water a day.  Without an efficient

system for reusing this water over and over again, about 10 tons of water would

have to be sent to the space station every 90 days, requiring special Space

Shuttle flights just for the replenishment of the water supply.

Engineers at the Marshall Space Flight Center (MSFC), Huntsville, Ala.,

have succeeded in developing a prototype system that can recycle shower and

wash water, urine and even respiration and perspiration captured from the air

back into potable drinking water.  Taste tests and other end-use tests run at

MSFC during 1992 demonstrated that the systems work well and that the recycled

water is clean and acceptable for crew use.  However, now the systems must be

tested onboard the Space Shuttle in low Earth orbit to make sure they perform

just as well in the microgravity environment of space flight.

The EFE consists of three pieces of recycling hardware -- a bellows tank,

a gas/water phase separator and two unibeds (filters).  These components will

be housed in two containers occupying the equivalent of four lockers in the

forward bulkhead of the SPACEHAB Space Research Laboratory. The bellows tank

features a Pyrex see-through window that will allow crew members to observe how

gas and water behave inside the tank in microgravity -- examining, for

instance, whether the air bubbles colonize or cling to the tank walls.  The

phase separator will separate the gas from the mixture.

The experiment also will carry about a half gallon of pure water, into

which will be mixed potassium iodide (simulating a wastewater contaminant).

The iodide mixture will be run through the unibed filters to purify the water.

The purification experiment will test both the efficiency of the unibeds in

purifying the water and the rate at which the unibeds are depleted.

Two types of unibeds will be flown on STS-57 as part of the ECLSS Flight

Experiment, one which is spring-loaded and the other which is not.  The two

types will be tested to determine if the spring is required for the unibeds to

work properly in the microgravity environment.  If it is not required, the

spring can be eliminated to reduce the weight of the hardware.

The current industry affiliates on the ECLSS Flight Experiment are Boeing

Aerospace, Life Systems, Inc., and Hamilton Standard. The Principal

Investigators for the ECLSS Flight Experiment are NASA Marshall Space Flight

Center, Huntsville, Ala., and Boeing Aerospace.

SPACEHAB-01 Payloads Supporting Hardware

Three-Dimensional Microgravity Accelerometer

The Consortium for Materials Development in Space (CMDS), is sponsoring

the Three-Dimensional Microgravity Accelerometer (3-DMA) on the STS- 57

mission.  The CMDS is a NASA CCDS based at the University of Alabama,

Huntsville (UAH).

The acceleration measurement system will help chart the effects of

deviations of microgravity on the experiments being conducted in space.  The

microgravity environment inside the SPACEHAB Space Research Laboratory will be

measured in three dimensions by the 3-DMA, allowing researchers to review

experiment results against deviations from microgravity.  This information will

be used to determine the degree of microgravity achieved inside the SPACEHAB

Space Research Laboratory. Disturbances caused by operating various experiments

in SPACEHAB and the residual microgravity resulting from orbiter rotational

motions and by drag will be measured.

The 3-DMA hardware consists of four accelerometer assemblies to be located

in different parts of the SPACEHAB Space Research Laboratory. The

accelerometers provide the acceleration data to a central control box located

in a single locker.  The data are recorded in flight on two gigabyte magnetic

hard drive devices.

The accelerometer package comprises three remotely located standard

three-dimensional systems and new invertible accelerometers in the central

unit.  The new, unique invertible feature permits measurements of absolute

microgravity and low-level, quasi-steady, residual accelerations (i.e.,

atmospheric drag) that have proven difficult to measure in the past.

A potential application of 3-DMA would be to characterize the microgravity

environment of space platforms in support of experiments, research and

commercialization activities.

Principal Investigator for 3-DMA is Jan Bijvoet of the UAH CMDS. Space

Acceleration Measurement System

NASA's Lewis Research Center (LeRC), Cleveland, is sponsoring the Space

Acceleration Measurement System (SAMS) on the STS-57 mission.  SAMS is designed

to measure and record low-level acceleration during experiment operations.  The

signals from these sensors are amplified, filtered and converted to digital

data before being stored on optical disks and sent via downlink to the ground

control center.

SAMS has flown on six previous Shuttle flights and acquired nearly 15

gigabytes of data which represents 50 days of operation.  Approximately two

gigabytes of data will be acquired on the SPACEHAB mission.

The high density floppy disks have approximately one megabyte of capacity.

The capacity of a double-sided optical disk used on Shuttle missions is 400

megabytes.  This compares to approximately 400 high density floppy disks or 40

standard boxes of ten disks.  All the data will fit on one optical disk

measuring about 5 inches square and one-half inch thick.

Three sensors will be flown.  One will measure the disturbances near an

Environmental Control Support System, another sensor will be located on the

support structure of the SPACEHAB Space Research Laboratory and the third will

be attached to a locker door to determine the level of disturbances experienced

by experiments in the locker and nearby.  The second and third sensors are

primarily to measure the acceleration characteristics of the SPACEHAB Space

Research Laboratory for future experiments.

Scientists will use the SAMS data in different ways, depending on the

nature of the science experiment and the principal investigators' experience

and ground-based testing results.  The principal investigators will typically

look for acceleration events or conditions that exceed a threshold where the

experiment results could be affected.  This may be, for example, a frequency

versus amplitude condition, an energy content condition or simply an

acceleration magnitude threshold.

Data from previous missions have shown the levels of disturbance evident

in the Spacelab module by the use of the crew exercise treadmill located in the

middeck.  This data, along with other missions' data, are important in order to

reduce and isolate disturbances on future missions, including on the space

station.

SAMS flight hardware was designed and developed in-house at LeRC. The

Principal Investigator for SAMS is Charles Baugher of NASA's Marshall Space

Flight Center, Huntsville, Ala., and the Project Manager is Richard DeLombard

of NASA Lewis.

EUROPEAN RETRIEVABLE CARRIER (EURECA)

F. Schwan - Industrial Project Manager

Deutsche Aerospace, ERNO Raumfahrttechnik

Bremen, Germany

W. Nellessen - ESA Project Manager

ESTEC Noordwijk, The Netherlands

The European Space Agency's (ESA) EURECA spacecraft was launched on July

31, 1992, by the Space Shuttle Atlantis (STS-46) and deployed at an altitude of

230 nautical miles (425 km).  It ascended using its own propulsion to the

operational orbit of 270 nautical miles (500 km).  Several weeks prior to the

STS 57 launch, ground controllers will lower EURECA's altitude where it will be

retrieved by Endeavour and brought back to Earth.

The EURECA-1 mission primarily has been devoted to research in the fields

of material and life sciences and radiobiology, all of which require a

controlled microgravity environment.  The selected microgravity experiments

have been carried out in seven facilities.  The remaining payload comprises

space science and technology.

During the mission, EURECA's residual carrier accelerations have not

exceeded 10-5g.  The platform's altitude and orbit control system made use of

magnetic torquers augmented by cold gas thrusters to keep disturbance levels

below 0.3 Nm during the operational phase.

Physical characteristics

o Launch mass	9,900 lbs (4491 kg)

o Electrical power solar array	5000 W

o Continuous power to EURECA experiments	1000 W

o Launch configuration	dia: 14.76 ft (4.5 m.)

length:  8.33 ft (2.54 m)

o Volume	132 cubic ft (40.3 m)

o Solar array extended 	66 ft x 11.5 ft (20 m x 3.5 m)

User friendliness

Considerable efforts have been made during the design and development

phases to ensure that EURECA is a "user friendly" system.  As is the case for

Spacelab, EURECA has standardized structural attachments, power and data

interfaces.  Unlike Spacelab, however, EURECA has a decentralized payload

control concept.  Most of the onboard facilities have their own data handling

device so that investigators can control the internal operations of their

equipment directly.  This approach provides more flexibility as well as

economical advantages.

Operations

All EURECA operations are controlled by ESA's Space Operations Centre

(ESOC) in Darmstadt, Germany. During the deployment and retrieval operations,

ESOC functions as a Remote Payload Operations Control Centre to NASA's Mission

Control Center, Houston, and the orbiter is used as a relay station for all the

commands.

Throughout the operational phase, ESOC has controlled EURECA through two

ground stations at Maspalomas, Canary Islands (Spain), and Kourou, French

Guiana. EURECA has been in contact with its ground stations for a relatively

short period each day.  When it was out of contact, its systems operated with a

high degree of autonomy, performing failure detection, isolation and recovery

activities to safeguard ongoing experimental processes.

An experimental advanced data relay system, the Inter-orbit Communication

Package, was included in the payload.  This package communicated with the

European Olympus Communication Satellite to demonstrate the possible

improvements for future communications with data relay satellites.  Such a

system will significantly enhance real time data coverage.

EURECA Retrieval Operations

The EURECA free-flying experiment platform will be retrieved on the fourth

day of STS-57. EURECA was deployed from Atlantis on STS-46 on Aug. 1, 1992.

During its approximately10-month stay in orbit, EURECA has supported

investigations in processing metallurgical samples, growing crystals and

conducting biological and biochemical studies.  Several weeks before the STS-57

launch, EURECA controllers will lower the spacecraft's orbit from 270 nautical

miles (500 km) high to 257 nautical miles (300 km) in preparation for the

retrieval.

David Low will grasp the 5-ton EURECA with the Shuttle's robot arm and

lower the platform into latches in the aft cargo bay.

Beginning on flight day one, a series of engine firings will adjust

Endeavour's catch-up rate so that on the morning of flight day four, a final

altitude adjustment burn will move Endeavour up to the 257-nautical-mile EURECA

orbit.  During the catch-up maneuvers, Endeavour's onboard navigational star

trackers will sight on EURECA during the best lighting period, from noon to

sunset of each orbit, to provide the most accurate course correction

information for each maneuver.

For the final mid-course corrections, the crew will use Endeavour's

rendezvous radar to refine their information about the position of EURECA in

relation to Endeavour. For about the final one and a half miles of Endeavour's

approach to EURECA, Commander Ron Grabe will fly the Shuttle's maneuvers

manually.

EURECA RETRIEVABLE CARRIER

Structure

The EURECA structure is made of high strength carbon-fibre struts and

titanium nodal points joined together to form a framework of cubic elements.

This provides relatively low thermal distortions, allows high alignment

accuracy and simple analytical verification, and is easy to assemble and

maintain.

Larger assemblies are attached to the nodal points.  Instruments weighing less

than 220 lbs (100 kg) are assembled on standard equipment support panels

similar to those on a Spacelab pallet.

Thermal Control

Thermal control for EURECA combines active and passive heat transfer and

radiation systems.  Active transfer, required for payload facilities which

generates more heat, is achieved by means of a freon cooling loop which

dissipates the thermal load through two radiators into space.  The passive

system makes use of multilayer insulation blankets combined with electrical

heaters.  During nominal operations, the thermal control subsystem rejects a

maximum heat load of about 2300 W.

Electrical Power

The electrical power subsystem generates, stores, conditions and

distributes power to all the spacecraft subsystems and to the payload.  The

deployable and retractable solar arrays, with a combined raw power output of

some 5000 W together with four 40 Ah nickel-cadmium batteries, provide the

payload with a continuous power of 1000 W, nominally at 28 V, with peak power

capabilities of up to 1500 W for several minutes.

Attitude and Orbit Control

A modular attitude and orbit control subsystem (AOCS) was used for

attitude determination and spacecraft orientation and stabilization during all

flight operations and orbit control maneuvers.

An orbit transfer assembly, consisting of two redundant sets of four

thrusters was used to boost EURECA to its operation attitude at 500 km and to

return it to its retrieval orbit at about 300 km.

EURECA has been developed under ESA contract by DASA (Deutsche

Aerospace/ERNO Raumfahrttechnik) (Germany), and their subcontractors Sener

(Spain), AIT (Italy), SABCA (Belgium), AEG (Germany), Fokker (The Netherlands),

Matra (France), Snia BPD (Italy), BTM (Belgium) and Laben (Italy).

EURECA SCIENCE

Solution Growth Facility - a multi-user facility dedicated to the growth of

monocrystals from solution, consisting of a set of four reactors and their

associated control system.

Protein Crystallization Facility - a multi-user solution growth facility for

protein crystallization in space.  The object of the experiments is the growth

of single, defect-free protein crystals of high purity and of a size sufficient

to determine their molecular structure by x-ray diffraction.

Exobiology and Radiation Assembly - a multi-user life science facility for

experiments on the biological effects of space radiation.

Multi-Furnace Assembly - a multi-user facility dedicated to material science

experiments.  It is a modular facility with a set of common system interfaces

which incorporates 12 furnaces of three different types, giving temperatures of

up to 1400 degrees C.

Automatic Mirror Furnace - an optical radiation furnace designed for the growth

of single, uniform crystals from the liquid or vapor phases, using the

traveling heater or Bridgman methods.

Surface Forces Adhesion Instrument - studies the dependence of surface forces

and interface energies on physical and chemical-physical parameters such as

surface topography, surface cleanliness, temperature and the deformation

properties of the contacting bodies.

High Precision Thermostat Instrument - an instrument designed for long term

experiments requiring microgravity conditions and high precision temperature

measurement and control.

Solar Constant And Variability Instrument - designed to investigate the solar

constant, its variability and its spectral distribution, and measure:

o	 fluctuations of the total and spectral solar irradiance

o	 short term variations of the total and spectral solar irradiance

within time scales ranging from hours to few months, and

o	 long term variations of the solar luminosity in the time scale

of years (solar cycles) by measuring the absolute solar irradiance.

Solar Spectrum Instrument - designed to study solar physics and the solar-

terrestrial relationship in aeronomy and climatology.  It measures the absolute

solar irradiance and its variations in the spectral range from 170 to 3200 nm,

with an expected accuracy of 1 percent in the visible and infrared ranges and 5

percent in the ultraviolet range.

Occultation Radiometer Instrument - designed to measure aerosols and trace gas

densities in the Earth's mesosphere and stratosphere.

Wide Angle Telescope - designed to detect celestial gamma and X-ray sources

with photon energies in the range 5 to 200 keV and determine the position of

the source.

Timeband Capture Cell Experiment - an instrument to study the microparticle

population in near-Earth space -- typically Earth debris, meteoroids and

cometary dust.

Radio Frequency Ionization Thruster Assembly - designed to evaluate the use of

electric propulsion in space and to gain operational experience before

endorsing its use for advanced spacecraft technologies.

Advanced Solar Gallium Arsenide Array - to provide valuable information on the

performance of gallium arsenide (GaAs) solar arrays and on the effects of the

low Earth orbit environment on their components.

GET AWAY SPECIAL EXPERIMENTS

NASA's Get Away Special (GAS) program, managed by the Goddard Space Flight

Center (GSFC), Greenbelt, Md., is a vehicle that allows the world to get

involved in the U.S. space program.  Individuals and organizations of all

countries have gained access to space by sending scientific research and

development experiments onboard the Space Shuttle via the GAS program.  Since

its inception, 87 payloads have flown on 18 Space Shuttle missions.

NASA's GAS bridge assembly, a structure which fits across the payload bay

of the Space Shuttle orbiter, is capable of holding up to 12 GAS cannisters (or

cans), and offers an efficient and convenient way to fly multiple payloads

simultaneously.

On STS-57, the GAS bridge is flying with a total of 10 GAS payloads from

the U.S., Canada, Japan and Europe. Also on the bridge is one secondary

payload, a commercialization experiment sponsored by the Consortium for

Materials Development in Space at the University of Alabama, Huntsville and one

GAS ballast can.  Clarke Prouty is GAS Mission Manager and Lawrence R. Thomas

is Customer Support Manager for the Shuttle Small Payloads Project at Goddard.

The 10 GAS payloads are:

G-022  Liquid Gauging Technology Experiment

Customer:  European Space Agency, European Space Research and Technology

Centre, Noordwijk, The Netherlands

Customer Manager:  Manfred Trischberger

NASA Technical Manager:  Richard Hoffman, GSFC

This experiment demonstrates two in-orbit methods of gauging liquids in

tanks - the Periodic Volume Stimulus (PVSM) and the Foreign Mass Injection

Method (FMIM).

Both approaches work well in the presence of gravity, but the peculiar

properties of liquid under microgravity conditions could lead to a lower

measurement accuracy.  This experiment will study, in particular, errors caused

by the following effects:

o 	liquid distribution in the tank

o 	unconnected liquid quantities

o 	uneven heating

o 	unintentional intrusion of fluid in pipes, sensor apertures, etc.

G-324  CAN DO

Customer:  Charleston County School District, Charleston, S.C.

Customer Manager:  Carol A. Tempel

NASA Technical Manager:  Neal Barthelme, GSFC

A detailed description of G-324 CAN DO is available in the educational

activities part of the STS-57 press kit.

G-399 Insulin Tagging and Artemia Growth Experiments

Customer:  Dr. Ronald S. Nelson, Inc., Fresno, Calif.

Customer Manager:  Dr. Ronald S. Nelson

NASA Technical Manager:  Gary Sneiderman, GSFC

This payload is composed of two experiments.  Its primary objective is to

successfully operate an insulin tag experiment and a brine shrimp Artemia

physiology experiment.  The experiments also will focus on educating students

about all aspects of carrying out scientific experiments.

G-450  Multiple Experiments

Customer:  Vandenberg Section, American Institute of Aeronautics and

Astronautics, Vandenberg AFB, Calif.

Customer Manager:  Martin Waldman

NASA Technical Manager:  Mark Anderson, GSFC

This is a multidisciplinary package composed of six self-contained

modules, each containing multiple experiments.  The experiments are designed

and developed by California Central Coast elementary, middle and high schools.

o Module one contains solidification/crystallization of saccharin and cryogen

transfer.

o Module two addresses the effects of radiation on bacteria and effects of

microgravity on sprouting seeds.

o Module three is bacteria survival in radiation and zero point energy.

o Module four consists of electrode occlusion and bubble formation and

microgravity bonding.

o Module five contains osmosis, reverse osmosis and effects of radiation on

seeds.

o Module six is crystal growth, fluids in microgravity and silver crystal

growth.

G-452  Crystal Growth of Gallium-Arsenide

Customer:  Society of Japanese Aerospace Companies, Tokyo, Japan

Customer Manager:  Dr. Naotake Tateyama

NASA Technical Manager:  Herb Foster, GSFC

This GAS can consists of 12 small electric furnaces.  The four kinds of

experiments to be carried out in low gravity are:

o Growth of a single crystal gallium-arsenide from liquid phase

o Growth of a crystal gallium-arsenide based mixed crystal

o Addition of a heavy element to gallium-arsenide

o Addition of a heavy element to indium-antimony crystal

G-453  Semi-Conductors/Superconductor Experiment

Customer:  Society of Japanese Aerospace Companies, Tokyo, Japan

Customer Manager:  Dr. Naotake Tateyama

NASA Technical Manager:  Herb Foster, GSFC

This GAS can contains four different kinds of experiments.  Three of the

four are materials experiments on semi-conductors and a superconductor, and the

other is on boiling an organic solvent under weightlessness.

G-454  Crystal Growth

Customer:  Society of Japanese Aerospace Companies, Tokyo, Japan

Customer Manager:  Dr. Naotake Tateyama

NASA Technical Manager:  Herb Foster, GSFC

This experiment studies the crystal growth of indium gallium arsenic from

vapor phase under weightlessness, the crystal growth of three selenic- niobium

from vapor phase, the crystal growth of an optoelectric crystal by the

diffusion method and formation of superferromagnetic alloy.

G-535  The Pool Boiling Experiment

Customer:  NASA Headquarters, Office of Space Science and

Applications, Microgravity Sciences Division, Washington, D.C.

Customer Manager:  Warren Hodges

NASA Technical Manager:  Tom Dixon, GSFC

The objective of this experiment is to improve the understanding of the

boiling process.  This involves putting a pool of liquid in contact with a

surface that can supply heat to the liquid.  The experiment will observe

heating and vapor bubble dynamics associated with bubble growth/collapse and

subsequent bubble motion.  The lack of gravity driven motion makes the boiling

process easier to study in microgravity.

G-601  High Frequency Variations of the Sun

Customer:  San Diego Section, American Institute of Aeronautics and

Astronautics, San Diego, Calif.

Customer Manager:  Brian Dubow

NASA Technical Manager:  Neal Barthelme, GSFC

This experiment will measure and analyze high-frequency variations of the

Sun analyzing light that the Sun releases to the Earth. It also will determine

better the physics of the Sun and other stars.  The primary purpose of the

experiment involves measuring rapid variations in the solar flux.

G-647  Configurable Hardware for Multi-Disciplinary Projects in Space

(CHAMPS)

Customer:  Canadian Space Agency, Ottawa, Ontario, Canada

Customer Manager:  Duncan Burnside

NASA Technical Manager:  Russ Griffin, GSFC

This is a versatile payload that will provide an inexpensive means for

Canadian scientists to conduct their materials science experiments in space.

CHAMPS, built by MPB Technologies in Montreal, is designed to be adaptable,

combining the advantages of generic and dedicated research facilities for

materials processing in space.  This experiment will examine a

recently-developed technique for crystal growth called Liquid Phase

Electro-Epitaxy (LPEE) in a microgravity environment.  LPEE regulates crystal

growth by passing an electrical current through a subject material.

GAS Ballast Payload

Customer:  Goddard Space Flight Center, Greenbelt, Md.

GAS ballast payloads are flown for stability when a GAS payload drops out

and no GAS payload is available to replace it.  This ballast payload contains a

small accelerometer package furnished by Goddard to record accelerations during

the mission.

Sample Return Experiment

Jet Propulsion Laboratory, Pasadena, Calif.

Principal Investigator:  Dr. Peter Tsou

The Sample Return Experiment (SRE) sits on top the ballast GAS can.  The

primary science objective of the GAS SRE is the quantification of

extraterrestrial particles and other orbital debris present in the orbiter bay.

A secondary objective of this experiment will be a realistic test for comet

sample collection concepts.  The sample particles that are to be encountered

and collected have speeds of 10-14 km/second (16-22 m.p.h.) and diameters of

10-200 micrometers.

Consortium for Materials Development in Space Complex Autonomous Payload

The Consortium for Materials Development in Space Complex Autonomous

Payload-IV (CONCAP-IV) is a small, Shuttle cargo bay payload sponsored by the

University of Alabama in Huntsville (UAH) Consortium for Materials Development

in Space (CMDS).  The CMDS is one of the NASA Centers for the Commercial

Development of Space (CCDS) managed by NASA's new Office of Advanced Concepts

and Technology (OACT).

CONCAP-IV is the fourth area of investigation in a series of payloads

deriving their name from the consortium in CMDS and the Complex Autonomous

Payload (CAP) program managed by the NASA Goddard Space Flight Center. On

STS-57, CONCAP-IV will investigate the growth of nonlinear organic (NLO)

crystals by a novel method of physical vapor transport in the weightlessness of

the space environment.

Nonlinear optical materials are the key to many optical applications now

and in the future -- optical computing is a prime example.  Many studies have

suggested that the photonics industry ultimately will grow to the scale of the

current electronics industry.  Just as materials improvements in silicon were

essential to electronics, so too are improved optical materials required for

advanced in photonics.  The investigations in the CONCAP series seek to

determine whether crystals grown in space can speed the evolution of photonics.

During the experiment, it is anticipated that the microgravity in space

will facilitate two goals of improved NLO crystal growth -- it will avoid

convection, leading to crystals grown with more uniform composition, and it

will avoid the deformation of the crystals under their own weight at the

relatively high growth temperatures where they are extremely soft.

The experiment operation involves heating up a chamber containing the

material needed to produce the crystal, but keeping one spot on the chamber

walls cooler than the rest of the chamber walls.  This method causes the vapor

of the material to condense onto the cold spot where the crystal grows, much

like water vapor condenses into dew on grass.

Within CONCAP-IV there are six NLO "ovens," each containing two NLO growth

cells.  Each of the twelve cells is comprised of a glass chamber, about 1 inch

in diameter and 2 inches long and contains the sample material to be processed.

Each cell is wrapped in a heater.  Within each cell is a small copper plug that

is kept slightly cooler than the rest of the cell, providing a place for sample

material vaporized by the heater to recondense and grow the desired crystal or

thin film material.

The "ovens" are constructed from two aluminum cylinders, one inside the

other, with the area between them vented to space to form an insulating vacuum

-- like a thermos bottle.  The resulting reduction in heat loss is a very

important consideration since CONCAP-IV is battery-operated, providing a

limited power supply.  The high and low temperatures in each chamber are

controlled by a miniaturized computer designed and built specially for this

purpose.

The crystals being grown have two important properties.  First, when a

laser beam passes through the crystal it comes out with twice the frequency

(half the wavelength) of the original beam.  This is important because it

doubles the range of frequencies available for laser applications.  Currently,

lasers only operate efficiently at a limited number of frequencies, with some

very important frequencies missing for scientific and commercial applications.

Second, when an electric field is applied to some NLO materials, the

refractive index of the material changes.  When the refractive index changes,

so does the path of light traveling through the crystal.  This is like having a

prism which will bend a light beam different degrees when voltages are applied

to the crystal.  The changing of the path of a light beam results in the

crystals or thin film acting as a high speed, nearly instantaneous switch.

Such properties are extremely important to the optoelectronics and

photonics industry, especially for optical computing.  Without NLO materials,

optical computers would be impossible.  Nonlinear optical materials could play

the same role in photonics and optoelectronics that semiconductors do in the

electronics industry.

Displaytech, Inc., Boulder, Colo., is participating with the UAH CMDS in

CONCAP-IV. Displaytech is a commercializer of high-performance electro- optical

devices.  The Principal Investigator is Dr. Thomas Leslie, Associate Professor,

Chemistry Department, UAH. The Payload Manager is William Carswell, Research

Associate, UAH.

Superfluid Helium On-Orbit Transfer (SHOOT) Flight Demonstration

The Superfluid Helium On-Orbit Transfer (SHOOT) Flight Demonstration,

managed by NASA's Goddard Space Flight Center, is an experiment designed to

develop and demonstrate the technology required to resupply liquid helium

containers in space.  In addition, components developed for SHOOT may find use

in future space cryogenic (low temperature) systems.

Many detectors for astrophysics and observation of Earth require cooling

to extremely low temperatures to achieve high sensitivity.  To achieve these

low temperatures, liquid helium is used for cooling.  By allowing the liquid to

slowly vaporize, an instrument may be cooled to temperatures below 2 Kelvin (K)

(-519o F, -271o C).

Examples of facilities that already have flown using liquid helium are the

Infrared Astronomy Satellite (IRAS), launched in 1983, which discovered more

than a quarter million new infrared objects and the Cosmic Background Explorer

(COBE), launched in 1989, which has studied the Big Bang, the primeval

explosion that started the expansion of the universe.

The liquid helium gradually vents to space as it cools the instruments,

therefore the instrument has a finite lifetime.  It takes only a small amount

of heat to evaporate liquid helium.  For example, a 100 watt light bulb left on

in a 53 gallon (200 liter) dewar would evaporate all the liquid in less than

1.5 hours.  Both IRAS and COBE ran out of liquid in 10 to 11 months of

operation.

To achieve lower temperatures in the liquid, the vapor pressure is

lowered.  On the ground, this is accomplished by powerful vacuum pumps.  In

orbit, this is achieved by venting to the vacuum of space.  By doing this, IRAS

achieved a temperature of 1.64 K and COBE a temperature of 1.37 K. By contrast,

cold, deep space is a relatively warm 2.74 K, twice as warm as COBE's liquid

helium.  Thus, COBE was the coldest known object in the universe outside of the

Earth's atmosphere.  SHOOT is expected to set a new low temperature record at

1.1 K.

The SHOOT experiment consists of two vacuum insulated ThermosR- like

containers, called dewars, each holding 55 gallons (207 liters) of liquid

helium.  The two dewars are connected by a vacuum insulated transfer line.

Liquid helium is pumped from one dewar to another at rates from 1.3 to 4.4

gallons per minute (300 to 1000 liters per hour).  Each of dewar's plumbing,

including pumps, valves and instrumentation, is nearly identical so that each

dewar in turn may act as the supply or receiver dewar.

The SHOOT dewars are attached to a Hitchhiker bridge which spans the width

of the orbiter bay.  The SHOOT electronics interface to the Shuttle through the

Hitchhiker avionics.

Having no viscosity, superfluid helium will leak through the smallest

hole.  Because the space around the cryogen tank must be a very good vacuum to

insulate the liquid, no leak of any size can be tolerated.  A leak at room

temperature would be approximately 10,000 times greater with superfluid helium.

Even an air leak which is so small that it would take 20 million years to fill

the dewar is unacceptable.  SHOOT has approximately 160 welds and 60 removable

metal seals between the superfluid and the vacuum space.  All of these have

been checked and shown to be absolutely leak tight.

SHOOT will consist of experiments in liquid management in low gravity,

filling a large gap in the knowledge of the behavior of cryogens in space.  The

problem of controlling the position of cryogenic liquids in orbit is a

difficult one.  The evaporating gas must be allowed to leave the dewar, but the

liquid must be contained.  On the ground this is easy since the liquid is

denser than the gas and gravity holds it in the bottom of the tank.  In a low

gravity environment, the liquid location is not well defined.  Surface tension,

heat inputs and the small residual accelerations of a spacecraft all play a

role in positioning the liquid.

SHOOT will accomplish the first active management of liquid cryogens in

space.  A device called a phase separator allows helium vapor to leave the tank

while the liquid is retained.  The liquid will be gathered from the walls of

the tank and fed to a superfluid pump.  This pump converts heat directly to

pressure by an effect unique to superfluid helium called the fountain effect.

SHOOT is part experiment and part demonstration.  Because so little

experience with cryogen management in low gravity exists, the first part of

SHOOT's on-orbit operations will be to gather as much data as possible about

how the liquid is delivered to the pumps by liquid acquisition devices, the

behavior of the liquid/vapor discriminators and the slosh and cooldown of the

liquid.  Control of the experiment will be from the ground through the Payload

Operations Control Center at Goddard with the astronauts monitoring from the

Shuttle's aft flight deck at key times.

At one point the astronauts will accelerate the orbiter to settle the

liquid in one end of the dewar to calibrate sensors.  During two transfers, the

Shuttle will be accelerated to move the liquid away from the pump to see if

such disturbances interupt the flow.  Once the transfer of liquid stops, the

acceleration will be stopped and the return of the liquid to the pump will be

monitored.

Near the end of the operations, the astronauts will control a transfer

completely from the aft flight deck.  They will use a program which has some

expert system capabilities to control the transfer and diagnose any problems

which may occur.  This will be the first use of an expert system for a payload

on the orbiter.

SHOOT was developed and managed by Goddard for NASA's Office of Space

Systems Development, Advanced Program Division, Washington, DC.

SHOOT will:

o	achieve the lowest temperature ever in orbit -- 1.1 K (-457o F, 1.1

degrees above absolute zero).

o	demonstrate the first active management of liquid cryogen in space.

o	demonstrate the first use of an expert system in space.

o	demonstrate two types of liquid acquisition systems for delivering

liquid to the pump.

o	make the first observations of thermal layering and mixing of a cryogen

in orbit.

o	demonstrate superfluid mass gauging to 1 percent accuracy.

o	demonstrate controlled cooldown of a warm dewar.

SHOOT spinoff technologies include:

o Cryogenic motor driven valves are leak tight after hundreds of

cycles.

o A liquid/gas phase separator for use with normal liquid helium as

well as superfluid, enabling easier ground servicing of future small dewars.

o Liquid/vapor discriminators which can be used for other cryogens

as well as liquid helium.

o A relatively simple thermometry system to obtain resolution of 0.00001 K

or better.

STS-57 EXTRAVEHICULAR ACTIVTY:

DETAILED TEST OBJECTIVE 1210

STS-57 crew members David Low and Jeff Wisoff will perform a 4-hour

extravehicular activity (EVA) on the fifth day of the flight as a continuation

of a series of spacewalks NASA plans to conduct to prepare for the construction

of the space station.

STS-57 will be launched as a 6-day, 22-hour, 40-minute flight.  After

launch, if calculations of the amount of fuel and energy required to retrieve

EURECA and operate Spacehab match preflight projections, the flight will be

extended by 24 hours.  The EVA is the lowest priority of any objective or

experiment on the flight, and the spacewalk will be performed only if the

flight is extended by one day to become about a 7-day, 23-hour flight.

The space station demonstration EVAs, the first of which was performed on

STS-54 in January 1993, are designed to refine training methods for spacewalks,

expand the EVA experience levels of astronauts, flight controllers and

instructors, and aid in better understanding the differences between true

microgravity and the ground simulations used in training.

In addition, since the Shuttle's remote manipulator system (RMS)

mechanical arm will be aboard Endeavour to retrieve EURECA, the STS-57

spacewalk will assist in refining several procedures being developed to service

the Hubble Space Telescope on mission STS-61 in December 1993.

Low will be designated extravehicular crew member 1 (EV1) and Wisoff will

be designated extravehicular crew member 2 (EV2).  Pilot Brian Duffy will serve

as intravehicular crew member 1 (IV1), assisting the spacewalkers from inside

the crew cabin of Endeavour.

During the spacewalk, Low and Wisoff first will take turns in a foot

restraint mounted on the end of the robot arm, holding their fellow crew member

in various ways to imitate moving a large, inanimate piece of equipment.  Next,

they will investigate different methods of managing their safety tethers while

mounted in the robot arm restraint.

Another objective is planned to have each crew member, mounted in the

robot arm restraint, practice aligning their fellow crew member into a foot

restraint mounted on the side of the cargo bay, simulating the task of aligning

a large object into a tightly fitting restraint.  The crew members also will

practice working with various tools while in the robot arm restraint and gauge

the ability of the restraint to hold them steady as they tighten or loosen a

bolt.

The information gathered by these tests is expected to apply to both the

HST servicing spacewalks and space station construction planning, since moving,

aligning and installing objects with large masses from the end of the robot arm

will be integral to both jobs.

Among the items hoped to be better determined are the speed at which the

arm can be moved while an astronaut holds an object on the end, how large an

object it is feasible to handle while in the arm foot restraint, the amount of

time required for such tasks using an EVA crew member and the arm and how much

stability is supplied by the arm during hands-on work such as tightening bolts

and other attachment equipment.

STS-57 MIDDECK PAYLOADS

FLUID ACQUISITION AND RESUPPLY EXPERIMENT II

Principal Investigator: Susan L. Driscoll Marshall Space Flight Center,

Huntsville, Ala.

The Fluid Acquisition and Resupply Experiment (FARE II) will investigate

the dynamics of fluid transfer in microgravity.  The experiment previously flew

as FARE I on STS-53 in 1992 and also as the Storable Fluid Management

Demonstration (SFMD) on STS 51-C in 1985.

In space, liquid in a container does not readily settle on the bottom or

leave a pocket of gas on top as it does on Earth. The position of liquids in

weightlessness is highly unpredictable because the liquid and gas may locate or

mix in any area within the container.  To replenish on-board fluids and prolong

the life of space vehicles such as the space station, satellites and extended

duration orbiters, methods for transferring gas-free propellants and other

liquids must be developed.

FARE I was conducted primarily to assess the ability of a screen channel

capillary system to drain liquids while working in a microgravity environment.

Additionally, some experimentation was conducted regarding the control of

liquid motion during tank refill sequences.

Housed in four middeck lockers of the orbiter Endeavour, FARE II is

designed to demonstrate the effectiveness of a device to alleviate the problems

associated with vapor-free liquid transfer.  The device exploits the surface

tension of the liquid to control its position within the tank.

The basic flight hardware consists of a 12.5 inches (30.48 cm) spherical

supply tank and a 12.5 inches (30.48 cm) spherical receiver tank made of

transparent acrylic.  Additional items include liquid transfer lines, two

pressurized air bottles, a calibrated cylinder and associated valves, lines,

fittings, pressure gauges and a flowmeter display unit.

The experiment is essentially self-contained, with the exception of a

water- fill port, air-fill port and an overboard vent connected to the orbiter

waste management system.

Mission specialists will conduct this experiment eight times during the

flight, using a sequence of manual valve operations.  Air from the pressurized

bottles will be used to force fluid from the supply tank to the receiver tank

and back to the supply tank.  This process should take about 1 hour each time

it is performed.  An overboard vent will remove the vapor from the receiver

tank as the fluid level rises.

The FARE II control panel, containing four pressure gauges and one

temperature control gauge, will be used by the crew to monitor and control the

experiment.  Camcorder video tapes and 35-mm photographs will be made during

the transfer process.  The crew also will have the option of using air-to-

ground communication to consult with the principal investigator, if necessary.

The test fluid used for this experiment is water with iodine, used as a

disinfectant; blue food coloring, which will allow better visibility of the

liquid movement; a wetting solution, known as Triton X-100, to give the fluid

the consistency of a propellant; and an anti-foaming emulsion agent to prevent

bubbles from forming in the receiver tank.

Post-mission analysis of FARE II will include evaluation of the experiment

equipment, as well as review of camcorder video tapes and 35-mm photographs.

Because there will be no real-time data downlink during this experiment,

detailed study and analysis of test data will not be conducted until after the

mission.

Historically, problems dealing with fluid orbital transfer have been dealt

with by using bellows to move liquid without any pressurant gas or vapor

surface.  These systems are heavier, more complex, more expensive and more

prone to leakage during the transfer process than conventional methods of

liquid containment, such as the FARE II equipment.

The mission managed by NASA's Marshall Space Flight Center, Huntsville,

Ala., will utilize equipment developed by Martin Marietta. At Marshall, Susan

L. Driscoll is the Principal Investigator for FARE II.

Air Force Maui Optical System

The Air Force Maui Optical System (AMOS) is an electrical-optical facility

on the Hawaiian island of Maui. No hardware is required aboard Endeavour to

support the experimental observations.  The AMOS facility tracks the orbiter as

it flies over the area and records signatures from thruster firings, water

dumps or the phenomena of "Shuttle glow," a well-documented fluorescent effect

created as the Shuttle interacts with atomic oxygen in Earth orbit.  The

information obtained by AMOS is used to calibrate the infrared and optical

sensors at the facility.

STS-57 SPECIAL EVENTS & EDUCATIIONAL ACTIVITIES

Get-Away-Special #324 -- CAN DO

The Charleston County School District's CAN DO experiment (GAS #324) is

designed to take 1,000 photos of the Earth allowing students to make

observations and document global change by comparing the CAN DO photos with

matched Skylab photos.  The canister also contains 350 small passive student

experiments.  CAN DO is sponsored by NASA's Langley Research Center, Hampton,

Va., and supported by the South Carolina Space Grant Consortium.

The CAN DO payload uses GAS hardware and is housed in a 5 cubic foot

canister.  The canister is sealed with a 0.92 inch fused silica window, which

is optically flat and ground to a quarter wave tolerance, permitting

photography in visible light, infrared and ultraviolet wavelengths.

The primary payload of CAN DO, known as GEOCAM, contains four Nikon 35-mm

cameras equipped with 250 exposure film backs.  The GEOCAM system will match

closely the larger Skylab film format in both coverage and quality allowing

direct examination and comparison of the changes that have occurred to the

planet in the last 20 years.

One thousand photographs will be taken by the four cameras over the course

of the Shuttle mission.  Photographic targets will be chosen by teachers and

students based on weather, Sun angle, orbiter orientation and crew activities.

Targets selected will be sent to the Shuttle once a day as crew notes.

These efforts will be managed through a student-run mission control room

at the Medical University of South Carolina. Student-teacher teams of 12 to 20

will operate four desks monitoring crew activities and mission timeline,

monitoring weather data, targeting geological or environmental interests and

communicating the target objectives with NASA's Johnson Space Center's Earth

Observations Lab and the Shuttle Small Payload Customer Support Room.

Activities and reports from the control room will be televised to students

throughout the state by the South Carolina Educational Television Network. The

Medical University of South Carolina will provide technical assistance.

The CAN-DO teachers have designed classroom activities for grades K- 12

using the 1,000 photos to make observations.  The photos comprise the first

educational payload to photograph the Earth from space.  Students will search

for natural and human-induced environmental changes that may have taken place

during the past 20 years.  Comparison between the photos, past and present,

enables students to discover for themselves the major effects caused by

deforestation, urbanization, river sediment loads, desertification, coastal

erosion, lake levels wetlands and pollution.

With assistance from atmospheric scientists, the photos should provide

clues to the degradation of the air quality often mentioned by astronauts.

Faculty members of the College of Charleston will aid in the interpretation of

the results.

The second experiment carried on CAN DO is 350 student-designed

experiments.  No other GAS payload in the history of the space program has ever

undertaken this many different experiments.  These experiments have been

submitted from more than 60 Charleston County classrooms and from invited

school districts from Maryland, Virginia, Texas, Arizona and Massachusetts.

These experiments allow students to participate directly in research by

testing the effect of space on various materials.  Students from K-12 have

chosen materials ranging from brine shrimp eggs to bubble solution to lipstick

to cotton seeds to fly in space.  A major goal of the student experiments is to

teach the skills of proper experimental design as well as execution of valid

scientific experiments.

Each student team has five samples of their materials in small 5 ml

cryovials: one to fly in space, one to serve as a passive control and one each

to be exposed to high doses of radiation, extreme cold and centrifuge.  The

control procedures will be carried out at the Medical University of South

Carolina as part of its Business/Education Partnership Program with the

Charleston County School District Office of Math, Sciences and Technology.

In addition to the students' vial experiments, the WESTVACO Forestry

Division has donated Sycamore and Loblolly Pine seeds to be placed into the

canister.  Classes participating in CAN DO will receive seedlings grown from

space-exposed seeds and encouraged to raise "space trees."

Students and teachers from the Poquoson School District in Poquoson, Va.,

are participating in the payload's student-designed experiments.  Also, a team

of Poquoson secondary students will travel to Charleston and operate the

weather desk at the student mission control.  NASA Langley atmospheric research

scientists will provide appropriate training to the Poquoson students for their

weather desk assignment.

Shuttle Amateur Radio Experiment-II

The Shuttle Amateur Radio Experiment-II (SAREX-II) provides for public

participation in the space program, supports educational initiatives and

demonstrates the effectiveness of making contact between the Space Shuttle and

low-cost amateur "ham" radio stations on the ground.

On STS-57, Pilot Brian Duffy, call sign N5WQW, and Janice Voss, call sign

to be determined, will operate SAREX. Duffy has operated SAREX in flight before

during Shuttle mission STS-45. Operating times for school contacts are planned

into the crew's activities.  The school contacts generate interest in science

as students talk directly with Voss or Duffy. There will be voice contacts with

the general ham operator community as time permits. and short wave listeners

(SWL's) worldwide also may listen.  When Voss or Duffy are not available,

SAREX- II will be in an automated digital response mode.

On STS-57, SAREX-II will include VHF FM voice and VHF packet.  The primary

voice frequency that SAREX-II uses is 145.55 MHz downlink.  There are a variety

of uplink frequencies.  Contacts with Endeavour will be possible between 42

degrees north latitude to 42 degrees south latitude, covering the lower half of

the continental United States and Hawaii, all of Africa, most of South America,

Australia, the East and the Far East.

SAREX has flown previously on STS-9, STS-51F, STS-35, STS-37, STS-45,

STS-50, STS-47, STS-55 and STS-56. SAREX is a joint effort of NASA, the

American Radio Relay League (ARRL), the Amateur Radio Satellite Corp. (AMSAT),

and the Johnson Space Center's Amateur Radio Club. Information about orbital

elements, contact times, frequencies and crew operating schedules will be made

available during the mission by these agencies and by amateur radio clubs at

some other NASA centers.

Hams from the JSC club, W5RRR, will be operating on amateur short wave

frequencies, and the ARRL station, W1AW, will include SAREX information in its

regular voice and teletype bulletins.  The amateur radio station at the Goddard

Space Flight Center, WA3NAN, in Greenbelt, Md., will operate around-the- clock

during the mission, providing information and re-transmitting live Shuttle

air-to- ground audio.  The JSC Public Affairs Office will operate a SAREX

information desk during the mission, and mission information also will be

available on the dial-up computer bulletin board (BBS) at JSC.SAREX

Frequencies.

Shuttle Transmitting	Shuttle Receiving

Frequency	        Frequency

U.S. 	        145.55 MHz          	144.99 MHz

South America 	145.55                 	144.97

& Asia	        145.55                 	144.95

145.55	                144.93

145.55	                144.91

Europe	        145.55 MHz         	144.70 MHz

145.55                 	144.75

145.55                	144.80

South Africa	145.55 MHz            	144.95 MHz

Packet         	145.55                	144.49

GSFC Amateur Radio Club (WA3NAN) planned HF operating frequencies

3.860 MHz		7.185 MHz

14.295 Mhz             21.395 MHz

28.395 Mhz

To connect to the JSC Compute Bulletin Board, BBS, (8 N 1 1200 baud)

dial 713/483-2500 then type 62511.

STS-57 CREW BIOGRAPHIES

Ronald J. Grabe, 47, Col., USAF, will be Commander (CDR) of STS-57.

Selected as an astronaut in August 1981, Grabe considers New York, N.Y., his

hometown and will be making his fourth space flight.

Grabe graduated from Stuyvesant High School, New York, in 1962.  He

received a bachelors degree in engineering from the United States Air Force

Academy in 1966 and studied aeronautics as a Fulbright Scholar at the

Technische Hochschule, Darmstadt, West Germany in 1967.

Grabe first flew as Pilot for Shuttle mission STS-51J in October 1985.  On

his second flight, he was Pilot for Shuttle mission STS-30 in May 1989.  On his

most recent flight he was Commander of Shuttle mission STS-42 in January 1992.

Grabe has logged more than 387 hours in space.

Brian Duffy, 39, Col., USAF, will serve as Pilot (PLT).  Selected as an

astronaut in June 1985, Duffy was born in Boston, Mass., and will be making his

second space flight.

Duffy graduated from Rockland High School, Rockland, Mass., in 1971.  He

received a bachelors degree in mathematics from the Air Force Academy in 1975

and a masters degree in systems management from the University of Southern

California in 1981.

Duffy first flew as Pilot of STS-45 in March 1992 and has logged more than

214 hours in space.

G.  David Low, 37, will serve as Payload Commander and Mission Specialist

1 (MS1).  Selected as an astronaut in May 1984, Low was born in Cleveland and

will be making his third spaceflight.

Low graduated from Langley High School, McLean, Va., in 1974.  He received

a bachelors degree in physics-engineering from Washington and Lee University in

1978, a bachelors degree in mechanical engineering from Cornell University in

1980 and a masters degree in aeronautics and astronautics from Stanford

University in 1983.

Low first flew as a mission specialist aboard STS-32 in January 1990.  His

next flight was as a mission specialist on STS-43 in August 1991.  He has

logged more than 474 hours in space.

Nancy Jane Sherlock, 34, Capt., USA, will serve as Mission Specialist 2

(MS2).  Selected as an astronaut in January 1990, Sherlock considers Troy,

Ohio, her hometown and will be making her first space flight.

Sherlock graduated from Troy High School in 1977.  She received a

bachelors degree in biological science from Ohio State University in 1980 and a

masters degree in safety engineering from the University of Southern California

in 1985.

After serving as a Neuropathology Research Assistant for 3 years at the

Ohio State University College of Medicine, Sherlock was commissioned in the U.

S. Army in 1981.  She attended the Army Aviation School and later served as a

UH-1H instructor pilot and a standardization instructor pilot for all phases of

rotary wing flight.  She has logged more than 2,900 hours flying time in rotary

wing and fixed wing aircraft.

Sherlock was assigned to NASA as a flight simulation engineer on the

Shuttle Training Aircraft at the Johnson Space Center in 1987, developing and

directing engineering flight tests, a position she held at the time of her

selection.

Peter J. K. (Jeff) Wisoff, 34, will serve as Mission Specialist 3 (MS3).

Selected as an astronaut in January 1990, Wisoff was born in Norfolk, Va., and

will be making his first space flight.

Wisoff graduated from Norfolk Academy in 1976.  He received a bachelors

degree in physics from the University of Virginia in 1980, a masters degree in

applied physics from Stanford University in 1982 and a doctorate in applied

physics from Stanford in 1986.

After completing his doctorate, Wisoff joined the Rice University faculty

in the Electrical and Computer Engineering Department, researching the

development of new vacuum ultraviolet and high intensity laser sources and the

medical application of lasers to the reconstruction of damaged nerves.  He is

currently collaborating with researchers at Rice University on developing new

techniques for growing and evaluating semiconductor materials using lasers.

Janice Voss, 36, will serve as Mission Specialist 4 (MS4).  Selected as an

astronaut in January 1990, Voss considers Rockford, Ill., her hometown and will

be making her first space flight.

Voss graduated from Minnechaug Regional High School in Wilbraham, Mass.,

in 1972.  She received a bachelors degree in engineering science from Purdue

University in 1975, a masters degree in electrical engineering from the

Massachusetts Institute of Technology (MIT) in 1977 and a doctorate in

aeronautics and astronautics from MIT in 1987.

Voss was a cooperative education employee at the Johnson Space Center from

1973 to 1975, working with computer simulations in the Engineering and

Development Directorate. In 1977, she returned to JSC to work as a crew

trainer, teaching entry guidance and navigation.  After completing her

doctorate, she joined Orbital Sciences Corp., working on mission integration

and flight operations support for the Transfer Orbit Stage, a position she held

at the time of her selection.

STS-57 MISSION MANAGEMENT

NASA HEADQUARTERS, WASHINGTON, D.C.

Office of Space Flight

Jeremiah W. Pearson III - Associate Administrator

Bryan O'Connor - Deputy Associate Administrator

Tom Utsman - Space Shuttle Program Director

Leonard Nicholson - Space Shuttle Program Manager (JSC)

Brewster Shaw - Deputy Space Shuttle Program Manager (KSC)

Office of Space Systems Development

Arnold D. Aldrich - Associate Administrator

Michael T. Lyons - Deputy Associate Administrator (Flight Systems)

Lewis Peach, Jr. - Director, Advanced Programs

George Levin - Chief, Advanced Space Systems

Michael Card - Program Manager, SHOOT

Office of Advanced Concepts and Technology

Gregory M. Reck - Associate Administrator (Acting)

Jack Levine - Director (Acting), Flight Projects Division

Andrew B. Dougherty - Spacehab Utilization Program Manager

Richard H. Ott - Director (Acting), Space Processing Division

Ana M. Villamil - Deputy Director (Acting), Space Processing Division

Dan Bland - Commercial Middeck Augmentation Module Project Manager

(JSC)

Office of Safety and Mission Assurance

Col. Frederick Gregory - Associate Administrator

Charles Mertz - (Acting) Deputy Associate Administrator

Richard Perry - Director, Programs Assurance

Office of Life and Microgravity Sciences and Applications

Gary Martin - SAMS Program Manager

KENNEDY SPACE CENTER, FLA.

Robert L. Crippen - Director

James A. "Gene" Thomas - Deputy Director

Jay F. Honeycutt - Director, Shuttle Management and Operations

Robert B. Sieck - Launch Director

John "Tip" Talone - Endeavour Flow Director

J. Robert Lang - Director, Vehicle Engineering

Al J. Parrish - Director of Safety, Reliability and Quality Assurance

John T. Conway - Director, Payload Management and Operations

P. Thomas Breakfield - Director, Shuttle Payload Operations

MARSHALL SPACE FLIGHT CENTER, HUNTSVILLE, ALA.

Thomas J. Lee - Director

Dr. J. Wayne Littles - Deputy Director

Harry G. Craft, Jr. - Manager, Payload Projects Office

Alexander A. McCool - Manager, Shuttle Projects Office

Dr. George McDonough - Director, Science and Engineering

James H. Ehl - Director, Safety and Mission Assurance

Otto Goetz - Manager, Space Shuttle Main Engine Project

Victor Keith Henson - Manager, Redesigned Solid Rocket Motor Project

Cary H. Rutland - Manager, Solid Rocket Booster Project

Parker Counts - Manager, External Tank Project

JOHNSON SPACE CENTER, HOUSTON

Aaron Cohen - Director

Paul J. Weitz - Deputy Director

Daniel Germany - Manager, Orbiter and GFE Projects

David Leestma - Director, Flight Crew Operations

Eugene F. Kranz - Director, Mission Operations

Henry O. Pohl - Director, Engineering

Charles S. Harlan - Director, Safety, Reliability and Quality Assurance

STENNIS SPACE CENTER, BAY ST. LOUIS, MISS.

Roy S. Estess - Director

Gerald Smith - Deputy Director

J. Harry Guin - Director, Propulsion Test Operations

AMES-DRYDEN FLIGHT RESEARCH FACILITY, EDWARDS, CALIF.

Kenneth J. Szalai - Director

Robert R. Meyers, Jr. - Assistant Director

James R. Phelps - Chief, Shuttle Support Office.

AMES RESEARCH CENTER, MOUNTAIN VIEW, CALIF.

Dr. Dale L. Compton - Director

Victor L. Peterson - Deputy Director

Dr. Joseph C. Sharp - Director, Space Research

GODDARD SPACE FLIGHT CENTER, GREENBELT, MD.

Dr. John Klineberg - Director

Thomas E. Huber - Director, Engineering Directorate

Robert Weaver - Chief, Special Payloads Division

David Shrewsberry - Associate Chief, Special Payloads Division