AI-LANBIOS Low-level Driver For The InterLan AT Adapter (ILANAT.DOC)

-----------------------------------------------------

(C) Copyright 1992 Artisoft, Inc.

Portions (C) Copyright 1992 Racal-Datacom, Inc.

(Revised 11/10/92)

Introduction

============

The ILANAT low-level AI-LANBIOS driver was developed by Racal-Datacom in

conformance with the specification outlined in the Artisoft AI-LANBIOS Driver

Development Program.  The low-level driver interfaces directly to the network

hardware and provides basic unreliable data transfer capabilities.  The

AI-LANBIOS high-level driver provides the reliable data transfer capabilities

including all NETBIOS functions.

The ILANAT.COM low-level driver allows you to use your InterLan AT

adapter in a LANtastic network.  The driver interfaces with all versions

of the InterLan AT adapter card:

InterLan AT

InterLan AT-3M

InterLan AT-TP

Compatibility Statement

=======================

Version 3.00 of ILANAT.COM is compatible with Artisoft's LANtastic/AI

Network Operating System v4.1, except for the following functional

considerations:

1. It has been observed that under very heavy prolonged stress, the network

may eventually fail when using a Packard Bell 386SX/16 machine.  This

tendency was NOT observed when testing with other machine types.  It

is therefore recommended that the Racal InterLAN adapter not be used

in a Packard Bell 386SX/16 machine if used as a heavy-use server.

2. The driver copyright message does not include reference to Artisoft as

required by the Artisoft AI-LANBIOS Driver Development Program.

3. The switch /CARD= will hang the system if a card number is used that does

not contain an InterLAN adapter.

4. The /PACKET_SIZE switch is documented as accepting values in the range of

[46-1500], yet any packet size less than 100 results in an error.

5. Unlike the other LANtastic/AI drivers, ILANAT cannot parse abbreviated

command line parameters.  For example, it will not accept VE or VER

as an abbreviation of VERBOSE.

6. Switches must be delimited by a space even if forward slashes are used as

separators.  For example, the ILANAT driver cannot parse the

following correctly: "/MPX=C7/VERBOSE".

Vendor Information

==================

Racal-Datacom, Inc., LAN Internetworking Division, 155 Swanson Rd., Boxborough

MA 01719.  Phone: 508-263-9929.

Running ILANAT

==============

The ILANAT low-level driver must be run before the AI-LANBIOS.  The

ILANAT driver interfaces to the AI-LANBIOS using a DOS multiplex

interrupt number.  The default multiplex number is C7.  If you are only

running one low-level driver and one copy of AI-LANBIOS then you will

not have to worry about which multiplex number to assign.  Multiplex

numbers 00 through BF are reserved for DOS use so you must use a multiplex

number between C0 through FF.

Each invocation of the ILANAT driver supports one InterLan AT adapter

board.  You must run multiple copies of ILANAT if you have multiple cards

installed in your computer.  For each invocation of ILANAT you must run a

separate invocation of the AI-LANBIOS program.  When running multiple

invocations of the ILANAT driver each invocation must have a unique

multiplex number and adapter number (specified by the /CARD switch).

Note:   The ILANAT driver scans the I/O space looking for InterLan AT

adapter cards and assigns each AT adapter a unique adapter number.

Adapter numbers are assigned by increasing order of I/O base

address.  Thus if there are two InterLan AT cards in the system

with I/O addresses of 320h and 360h, the adapter at I/O address

320h will be assigned an adapter number of 1 and the adapter at

I/O address 360h will be assigned an adapter number of 2.

The first invocation of ILANAT.COM will normally bind to the first

adapter found in the system (adapter number 1).  This may be overridden

by specifying another adapter using the /CARD switch.  Subsequent

invocations of ILANAT.COM MUST specify a different adapter number using

the /CARD switch.

The ILANAT low-level driver program name is ILANAT.COM.  The syntax for

ILANAT is

ILANAT [switches . . . ] [ ; comment]

where [switches . . . ] denotes zero or more optional command line switches.

Switches may be delimited by blanks or slashes (/).  If a switch takes a

value, the switch must be followed by an equal sign (=) or a colon (:).

Some values may be hexadecimal numbers while other values may be decimal.

All switch values are range checked and illegal values produce errors.

You may place a comment on the command line by preceding the comment with

a semicolon (;).  You must precede the comment character with a blank.

The valid switch formats are:

SWITCH

SWITCH:value

/SWITCH

/SWITCH=value

@file

The valid ILANAT switches are enumerated below.  The letters "ddd"

denote a decimal number.  The letters "hh" denote a hexadecimal number.

Values enclosed in [] indicate default values if the switch is omitted.

Values after [] denote the valid range of the switch.

----------

@switch-file

This option specifies that further switches are to be taken from a

switch file.  Any switches after the @switch-file name will be used,

and the switch settings take precedence over any settings in the switch

file.  You may invoke switch files from within switch files as often as

you wish.  The file should contain valid switches and may contain comment

characters (;) at the beginning of each line or after switches.

For example,

ILANAT @setup

The file SETUP contains

; ILANAT AI-LANBIOS driver setup file

[ILANAT]

CARD=1                ; Use adapter number 1

verbose               ; Display verbose information

----------

HELP or ?

This switch displays information about the valid command line switches,

but does not install the ILANAT driver.  For example,

ILANAT /help

ILANAT ?

See the messages section for a detailed description of the HELP display

output.

----------

CARD=d [1] 1-5

This switch specifies the adapter number to which the driver should bind.

Adapter numbers are determined by I/O address where the adapter with

the lowest I/O address is assigned adapter #1 and so on.  See the section

on "Running ILANAT" for more information.

----------

MPX=hh [C7] C0-FF

This switch specifies the multiplex (MPX) interrupt number to use for

communication to the high-level AI-LANBIOS.  You will need to change this

value only if the MPX number is in use by another application or low-level

driver.

----------

PACKET_SIZE=dddd [1500] 46-1500

This switch specifies the maximum size of transmitted IEEE 802.3 packets.

You may need to change this value when using certain bridges that do not

support the full 1500 byte packet size.  Even when you reduce the packet

size, the low-level driver can still receive large packets.

----------

REMOVE

This switch removes ILANAT.COM from memory.  You must remove TSR programs

in the opposite order they were loaded.  For example, if you loaded

ILANAT.COM and then AILANBIO, you must remove AILANBIO before ILANAT.COM

In addition, if you have more than one invocation of ILANAT.COM and

AILANBIO you must use the MPX= switch on the command line to specify which

copy of ILANAT.COM (but not AILANBIO) that you want to unload.  For

example, if you load two copies of ILANAT.COM and AILANBIO:

ILANAT /CARD=1 /MPX=C7

AILANBIO /ADAPTER=0 /MPX=C7

ILANAT /CARD=2 /MPX=D7

AILANBIO /ADAPTER=1 /MPX=D7

To unload these copies of ILANAT.COM and AILANBIO:

AILANBIO /REMOVE

ILANAT /MPX=D7 /REMOVE

AILANBIO /REMOVE

ILANAT /MPX=C7 /REMOVE

The REMOVE option will remove the most recently run copy of AILANBIO.

----------

VERBOSE

This switch causes the ILANAT driver to be installed and detailed

information about the configuration of the ILANAT driver to be displayed.

See the messages section for a detailed description of the verbose output.

----------

XEROX

This switch causes all packets to be transmitted in the XEROX format

instead of the IEEE 802.3 standard although ILANAT will continue to

receive both XEROX and IEEE 802.3 packets.

ILANAT messages

===============

The ILANAT driver produces two types of messages: Informative and Error.

All error messages cause the ILANAT driver not to be installed.

--------

MESSAGE:

InterLan AT AI-LANBIOS Driver Vn.nn  (C) Copyright 1992 Racal-Datacom, Inc.

This message appears each time the low-level driver is invoked.

--------

MESSAGE:

---- InterLan AT AI-LANBIOS Driver installed ----

This message appears after the low-level driver is successfully

installed.

--------

MESSAGE:

---- InterLan AT AI-LANBIOS driver *NOT* installed ----

This message appears if the low-level driver is not installed.  The driver

will not be installed if help information is displayed or an error

occurred.

--------

MESSAGE:

Command line       /verbose

Ethernet address = 020701010203

Packet type      = IEEE 802.3

Packet data size = 1500

Multiplex ID     = C7

Interrupt (IRQ)  = 5

I/O Base Address = 0300

Mem Base Address = D000

Resident size    = 4512

This message appears if the VERBOSE switch is specified.  The values

displayed specify how the low-level driver is configured.

--------

MESSAGE:

Valid command line switches:

?

HELP

XEROX

VERBOSE

REMOVE

CARD=n [1-5]

MPX=xx [C0-FF]

PACKET_SIZE=dddd [46-1500]

@<filename>

This message appears when the HELP or ? switch is used.  Each valid switch

is listed with the acceptable range of values (if any) and the base (hex

or decimal) that the numbers must be entered in.  The low-level driver is

not installed when this switch is used.

--------

MESSAGE:

---- InterLan AT AI-LANBIOS Driver removed ----

The ILANAT.COM driver has been removed from memory.

--------

Can't REMOVE - InterLan AT AI-LANBIOS driver is not loaded

Meaning: You have issued the ILANAT /REMOVE command and do not have

ILANAT.COM loaded into memory.

Remedy:  Only issue this command after the program is loaded.

--------

Can't REMOVE -- INT 2F hooked

Meaning: You have attempted to remove ILANAT.COM and the Multiplex

interrupt (INT 2F) is in use by another TSR.

Remedy:  Remove any TSRs loaded after ILANAT.COM.  For example, if you

load ILANAT.COM then AILANBIO.EXE, you must first remove

AILANBIO.EXE, then you can remove ILANAT.COM.

--------

Can't REMOVE -- Different version of driver installed

Meaning: You have loaded a version of the ILANAT.COM program and have

attempted to remove it from memory with the /REMOVE switch using

a different version of ILANAT.COM.

Remedy:  Remove ILANAT.COM with the same version of the program that was

loaded.

--------

*** ERROR: MPX ID already in use

Meaning: The Multiplex (MPX) ID specified on the command line (or C7h by

default) is being used by another application or low-level driver.

Remedy:  Specify another MPX ID on the command line with the /MPX switch.

--------

*** Error: ILANAT Adapter #n not found

Meaning: InterLan AT adapter number n (where n is the adapter number

in the /CARD switch or 1 by default) could not be found.

Remedy:  Check to see that there are at least n properly configured

InterLan AT adapters in the system.

--------

*** Error: Adapter configured for IRQ 12 or 15 in 8-bit slot

Meaning: The InterLan AT adapter is jumper configured for either IRQ 12

or 15 and the adapter is in an 8-bit slot.  The adapter must be

in a 16-bit slot to use these IRQ values.

Remedy:  Move the adapter to an 8-bit slot or re-configure the adapter

using the jumpers (see the user manual for the adapter) to use

an IRQ other than 12 or 15..

--------

*** Error in switch ??????: - Unknown switch

Meaning: The switch ?????? is not a recognized switch.

Remedy:  You may only specify legal switches on the command line.

--------

*** Error in switch: ?????? - Illegal switch value

Meaning: The switch ?????? is followed by a character other than "=" or

":" or the switch does not take a value.

Remedy:  Only use the characters "=" or ":" to specify switch values and

do not place characters after switches that do not take values.

--------

*** Error in switch: ?????? - Switch value out of range

Meaning: The value nnnn is not a valid value for the switch.

Remedy:  The switch value must be in the acceptable range of valid

values.

--------

*** Error in file: ?????? - File not found

Meaning: The file "file" cannot be opened as an indirect file.

Remedy:  Verify that the file is in the current directory or that the

specified pathname is valid.

--------

*** Error in file: ?????? - No driver section found

Meaning: No driver section header was found in the file.

Remedy:  The file lines containing the driver switches must be preceded

by a line containing the name of the driver in brackets, e.g.:

[ILANAT]

verbose

card=1

--------

*** Error in file: ?????? - Invalid switch

Meaning: The indirect file contained a switch that was either unknown

or had an invalid value.

Remedy:  Verify that all switches in the indirect file under the driver

section are valid.

Testing ILANAT return code (error level)

========================================

You may test the ILANAT return code or error level in a batch file by

using the IF ERRORLEVEL command.  The following error levels are returned:

0     No error has encountered.  ILANAT driver installed

1     Switch error

2     Multiplex ID already in use

3     Adapter not found or configuration error

4     Adapter jumpered for IRQ 12 or 15 in an 8-bit slot

-------------------