CONVERGE - A CRT Testing Program

DESCRIPTION:

CONVERGE is a highly shareware program which is a useful

evaluation tool for testing VGA (and higher) resolution monitors.

CONVERGE will show such anomalies as geometric distortion,

pincushion, misconvergence, and color smearing. Such testing is

extremely valuable in evaluating monitor performance (you will be

amazed at how many high-priced high-rez monitors are mis-

converged!) when purchasing, or to spot potential trouble in an

existing monitor.

DISCUSSION:

Color CRT monitors use 3 separate colors (red, blue and green)

to optically generate the various shades of the color spectrum on

the monitor face. Proper operation requires that each of the

electron guns inside the picture tube be precisely aligned to

it's assigned color phosphor dots on the picture tube face. (If

you look at your monitor face with a magnifying glass, you can

actually see the 3 colors of phosphor dots.) If any of these guns

are mis-aligned, the resulting display can exhibit blurred color,

bad color, or a distorted image.

Besides mis-convergence (the color alignment mentioned above),

monitors are often plagued with raster (scanning) distortion. To

paint the image on the picture tube, the beams from the 3

electron guns are swept across the face of the picture tube in a

particular pattern. The pattern is an extremely fast left-to-

right sweep that is repeated in a downwards fashion. Older CGA

monitors used a very slow sweep rate, and as a result, tended to

flicker. New high-resolution monitors operate at a much faster

rate, so the eye does not perceive any noticeable flicker from

the screen.

If the circuitry inside the monitor is operating improperly, or

if the circuitry is of a cheap design, then certain raster

distortion effects become apparent when using the convergence

grid.

PINCUSHION DISTORTION is a 'bending' of the left and right

edges of the picture on the screen:

_______________________

\                     /

\                   /

|                  |

|                  |

/                    \

/______________________\

Page 1

CONVERGE - A CRT Testing Program

This type of distortion is caused by a mis-adjustment in the

horizontal (left-right) deflection circuitry. You can test for

this type of distortion by selecting the convergence grid and

holding a ruler or other straight-edged object up to the monitor

screen. By examining the white vertical bars at both the left and

right edges of the screen, any pincushion distortion should be

clearly seen. Some monitors such as the NEC 3F,4F,5F and 6F

series have an adjustment control to correct this.

KEYSTONE DISTORTION is a trapezoidal shaped pattern. Typically it

appears as an image which is wider at the top of the screen, then

tends to become narrower at the bottom:

______________________

|                    |

|                    |

|                  |

|                  |

|                |

|________________|

This type of distortion is invariably caused by an inadequate or

malfunctioning power supply in the monitor. Occasionally a weak

flyback transformer will create this symptom as well. The

convergence grid pattern again may be used, only in this case the

top and bottom white lines are measured for length. If there is a

noticeable difference (approximately 1/8" for a 14" monitor),

there is something wrong with the monitor or it's design.

GEOMETRIC DISTORTION refers to a non-symmetrical scanning

pattern. Usually this is evidenced by a distorted corner,

typically in the upper right corner of the screen:

________________________

|                      /

|                     /

|                    |

|                    |

|                    |

|____________________|

Geometric distortion comes from an improperly corrected picture

tube. During manufacture, small magnets are placed in certain

locations on the rear portion of the picture tube. Most monitor

manufacturers do a pretty good job of alignment, but occasionally

one slips through. This is particularly true of the Taiwanese

monitors. Again, the convergence grid may be employed to check

for this, as the outer white rectangle should have 90 degree

square corners. Occasionally some types of monitors will exhibit

a 'blooming' effect, most notably in the center of the screen.

This appears as a bulging outwards of the grid squares, much like

that of a balloon surface that is expanding.

Page 2

CONVERGE - A CRT Testing Program

COLOR ALIGNMENT (CONVERGENCE) is crucial for a clear, sharp

image, particularly at high resolutions. As mentioned earlier in

this discussion, the three electron guns (red, blue and green)

must align with their respective phosphor dots to produce a

proper image. If this alignment is off, the edges of any graphic

elements will tend to have fuzzy edges, or a slight color shadow.

This is where the convergence grid really shines. The vertical

lines are yellow in color, which requires exact alignment of the

red and green guns. The horizontal lines are magenta, which

requires exact alignment of the red and blue guns. Finally, the

outer rectangle is white, which requires the exact alignment of

all three guns.

Any color mis-convergence will show up as a separation of

color, or the appearance of color shadowing. For example, if the

red and blue guns are off, the horizontal magenta line will tend

to be blue on one side, and red on the other. Occasionally

geometric scan distortion will appear, causing the line to be

perfect at one end, and separate at the other. The author recalls

one instance where a monitor was so bad that the magenta line

actually turned into separate blue and red lines! A similar

situation occurs with the vertical yellow lines, only this time

the colors will be red and green.

Lastly, the overall color balance of the monitor may be

observed with each of the 3 color flood screens. Look for any

blotchy areas or shadows. Blotches are usually caused by a defect

in the phosphor coating. Shadowy patches are usually a result of

improper demagnetization of the shadow mask, and can usually be

cleared up with a thorough degaussing. (Turn the monitor on and

off several times, waiting about 2 to three minutes per cycle.)

The white screen shows the overall color balance and 'color

temperature' of the phosphors. Some monitors tend to have a

'warm' white, leaning a bit towards the red, while others tend to

appear greenish, similar to florescent lamps. This is more of a

personal preference.

For those people who are trained in electronic repair of

televisions and monitors, this program is also a handy tool to

use when fixing monitors. WARNING! - If you are NOT TRAINED to

perform these types of repairs, DO NOT disassemble or take your

monitor apart! If your monitor is exhibiting any of the symptoms

mentioned, please take your monitor to a repair facility! Lethal

electrical current is present inside the monitor, which can cause

serious injury or death!

PROGRAM OPERATION:

The program initially tests to see if a VGA-compatible graphics

adapter is present. If so, then the program will display the

convergence grid. By pressing the space bar, the program will

cycle through each screen indefinitely. To exit the program,

Page 3

CONVERGE - A CRT Testing Program

press the ESC key. NOTE: the space bar has no effect if a

particular screen is specified in the command line.

PROGRAM PARAMETERS:

Sytax: CONVERGE [/c] [/b] [/g] [/r] [/w] [?]

? - Echos a brief help text on the screen. No action is

taken.

/c - Displays the convergence grid. Each square is exactly

on a 1:1 pixel aspect ratio. (Each side is exactly the

same number of pixels in length.)

/b - Displays the blue flood screen.

/g - Displays the green flood screen.

/r - Displays the red flood screen.

/w - Displays the white flood screen.

SHAREWARE CONCEPT:

Shareware is a non-conventional approach to software

distribution. Unlike regular commercial programs, shareware

allows a 'try-before-you-buy' approach. In most cases, the

programs are just as bit as good (if not better in some

instances!) as the programs you buy at a computer store.

Shareware ISN'T freeware! In most cases, the software author is

a professional who makes at least part of his or her living on

the registration of the software program. Typically the user is

enticed into registration by added features, free updates, etc.

Shareware depends upon the user to make it work. It's up to

you!

Registration:

This program is 'donate-ware', which is similar in concept to

shareware. The author makes no claims as being a professional

software engineer, rather, the author is a disabled hardware

engineer who enjoys tinkering with programs, and making the

hardware work better.

If you find that this program is useful, and saves you some

time and headaches, the author would appreciate a $5.00 (US)

suggested donation. This amount would include a 'registered' copy

of the program (without the shareware notice), and the eternal

thanks from the author. Considering that the cost of monitors,

this program may make the difference between a wise purchase and

a costly mistake. With that in mind, 5 bucks is a very small

investment! Of course, any amount is appreciated. Oh, please

Page 4

CONVERGE - A CRT Testing Program

specify whether you want a 360K, 5-1/4" or 720K, 3-1/2" floppy

when you register.

.....and the address is:

Brent Turner

P.O. Box 3612

Fullerton, CA, 92634-3612

NOTE: Foreign (non-US) users please include a few extra bucks

for the postage! Thanks.

THE LEGAL STUFF:

This software is distributed on an as-is basis. The author has

made reasonable attempts to ascertain it's performance, but

cannot guarantee that it will work in every instance. The author

is not responsible for any damages, or loss of data resulting

from the use of this program.

The shareware version of this program may be freely distributed

as long as all files are included in the package. The registered

version may not be copied or duplicated.

Shareware vendors may distribute this program as long as the

cost per disk does not exceed $5.00. This program may be included

(bundled) with other programs of like nature, so long as the

above cost-per-disk stipulation is met.

The original source code is not available.

Page 5