�������������������������������������������������������������������������������

CIDBINK v1.1 Copyright (c) 1996

From the RBBS/CDOR Files of   By: Daniel T. Drinnon * All Rights Reserved

The Cellar Door RBBS/CDOR     �����������������������������������������������

Sysop: Dan Drinnon            Purpose:  To extract CID info

Home of the CDOR Mods!        from your BinkleyTerm log file

for use with BBS's that handle CID

�������������������������������������������������������������������������������

Usage:    CIDBINK [ConfigFile]

ConfigFile is optional.  The default ConfigFile name is

CIDBINK.CFG in the current directory.

�������������������������������������������������������������������������������

Included Files:

FILE_ID  DIZ     description file

CIDBINK  EXE     the program

CIDBINK  DOC     this documentation

CIDBINK  CFG     sample configuration file

�������������������������������������������������������������������������������

Usage:

������

CIDBINK will read your BINKLEY.LOG file for the CID strings you set in its

configuration file.  The LAST CID info it finds in the log file will then be

written to an output file you specify (normally a batch-type file) that can

execute to set environment variables for use with other programs.  The

format of the output file is controlled by what you set in the CIDBINK.CFG

file.  The included CIDBINK.CFG file here will create a file called

SETCID.BAT that contains DOS SET commands that can be used by your BBS

program's command line.  Since I only know of one BBS package (RBBS/CDOR)

that handles CID, this program leans towards that end for explaination and

usage, however I do believe it could be easily adapted to any other BBS

program out there that takes CID info on the command line.

Because the CIDBINK execution is a child process of your operating system,

it cannot alter the environment to the effect that it will change or create

new DOS envirornment variables when it is finished.  The technical reason is

explained simply that DOS gives a copy of the environment to a program which

the program is free to do whatever it wants with, leaving the parent

process's environment unaltered.  When the child process terminates, it's

copy of the environment is discarded and the parent environment is again

active.  The solution is to have CIDBINK create an output file which you

could run after CIDBINK is finished that will do whatever you want at the

parent process level.  If this doesn't make sense, go to the library and

find Ray Duncan's book "Advanced MSDOS Programming".

To avoid confusion, here is a general flow of operation in DOS-Speak:

1. BINKLEY.BAT is runs

2. Binkley gets a BBS caller and exits with an errorlevel

3. Binkley gives control to BBSBATCH.BAT

4. BBSBATCH gives control to SPAWNBBS.BAT

5. SPAWNBBS.BAT runs CIDBINK

6. SPAWNBBS.BAT runs SETCID.BAT which was created by CIDBINK

7. SPAWNBBS.BAT runs the BBS

8. goto top, BINKLEY.BAT runs again

Here is how it would look in reality AFTER Binkley exits to run BBSBATCH.BAT,

with BBSBATCH.BAT running SPAWNBBS.BAT and then SPAWNBBS.BAT running RBBS/CDOR:

[BINKLEY.BAT->BBSBATCH.BAT->SPAWNBBS.BAT]:

@ECHO OFF

C:\BINKLEY\CIDBINK C:\BINKLEY\NODE1\BINKLEY.LOG

CALL C:\BINKLEY\NODE1\SETCID.BAT

CD\RBBS

CALL RBBS.BAT %4 %1 %2 %5

SET CIDNMBR=

SET CIDNAME=

CD\BINKLEY

BINKLEY

[RBBS.BAT]: (simplified)

@ECHO OFF

RBBS-PC.EXE %node% RBBS%node%PC.DEF /T:%1 /B:%2 /C:%3 /R:%4 /I:NAME=%CIDNAME% NMBR=%CIDNMBR%

Notes:

������

1. To free up DOS environment space, CIDNMBR and CIDNAME are nulled out in

the SPAWNBBS.BAT file shown above after they are no longer needed.

2. You should configure your modem to return formated as opposed to raw text

output for the CID info for your binkley log.

3. Although everyone has their idea of the best USR V.Everything init string

for Binkley, I show mine here only for those who need more information.  I

know there are other ways to init the modem, but these work for me and like

they say, if it ain't broke....:

[BINKLEY.CFG portions removed]:

NoFilter /

ExtBaudRates

ModemConnect  CONNECT

ModemFailure  BUSY

ModemFailure  ERROR

ModemFailure  NO ANSWER

ModemFailure  NO CARRIER

ModemIncoming NO DIAL TONE

ModemFailure  OK

ModemFailure  VOICE

ModemIgnore   +FDM

ModemIgnore   RINGING

ModemIgnore   RING RESPONSE

ModemIgnore   DIALING

ModemIgnore   RRING

ModemIgnore   DIAL TONE

ModemRinging  RING

Port      1

Baud      57600

Carrier   80

TermInit   AT&FM1X4Q0V1&A3|~AT&B1&C1&D2&H1|~AT&K3&R2&S1&T5|AT&ZC=+FCLASS=0H1|~ATS0=0S2=255|~ATS7=45S10=20S11=45|~ATS15=72S19=5S38=1|~ATS69=0S70=0#CID=0|~AT&W|

Init      v^ATZ|ATB0M1&M4X4H0|AT+FCLASS=0|AT#CID=1|

Prefix    AT+FCLASS=0&K3DT

Answer    ~~AT+FCLASS=2\.0|~AT+FAA=1+FLI="703 913 5621"+FNR=1,1,1;B0A|

Busy      AT+FCLASS=0H1|

ModemFax  +FCO

ExtrnMail 99 +FCO

DTRHigh

LockBaud /Arq

AutoBaud

I use Binkley's Terminal to actually perform a full initialization instead

of using it as an actual terminal program.

Binkley's Recent Activity screen will say "No Answer" right after displaying

the CID info, however this doesn't seem to have an adverse affect on its

operation.

This configuration also uses the adaptive answering mode to handle incoming

FAX calls.

�������������������������������������������������������������������������������

Revisions: 03-01-96 Initial Release

03-08-96 Added handling of Unavailable CID Info

Fixed up the docs

Added short program info display

�������������������������������������������������������������������������������

DANIEL T. DRINNON IS NOT RESPONSIBLE TO YOU FOR DAMAGES, INCLUDING BUT NOT

LIMITED TO, ANY LOST PROFITS, LOST MONIES, OR OTHER SPECIAL, GENERAL, INCIDENTAL

OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE (INCLUDING

BUT NOT LIMITED TO LOSS OF DATA OR DATA BEING RENDERED INACCURATE OR LOSSES

SUSTAINED BY THIRD PARTIES OR A FAILURE OF THE PROGRAM TO OPERATE WITH ANY OTHER

PROGRAMS) OR ANY OTHER LOSS EVEN IF YOU HAVE BEEN ADVISED OF THE POSSIBILITY OF

SUCH DAMAGES, OR FOR ANY CLAIM BY ANY OTHER PARTY.

��������������������������������������������������������������������������������

BinkleyTerm is copyright by Bit Bucket Software Co.

��������������������������������������������������������������������������������

Liscense:  You are authorized to USE this program for an indefinite period

of time without fee.  This program may be included on CD-ROM's as

part of a collection of files as long as it is included in its

entirity.

��������������������������������������������������������������������������������

Dan Drinnon

The Cellar Door RBBS/CDOR

Download on First Call!

703-913-5621 (V.Everything)

703-913-5624 (14.4)

FidoNet: 1:109/331

��������������������������������������������������������������������������������