Technical Note : FAST SERIAL PORTS

Author : Michael Spalter, S/E/G Communications

Email : segcom@cix.complulink.co.uk

Origin : United Kingdom, EC

Last Modified : 04/12/92

Status : (c) 1993 Michael Spalter

NOTE: This document may be distributed  freely  providing  that  all

credits  remain  intact  and that it is not modified in any way. Any

suggestions for improving this document are welcome. Please  contact

the  author  as  shown  at  the foot of the document. Bulletin board

sysops are welcome to make this file available as a 'Bulletin'.

��������������������������������Ŀ

� DO I NEED A FAST SERIAL PORT ? �

����������������������������������

���������������������������������������������������Ŀ

�                 C O N T E N T S                   �

�                                                   �

�  0) INTRODUCTION                                  �

�  1) What is a UART ?                              �

�  2) Why the sudden craze for high speed cards ?   �

�  3) Why aren't normal PC ports good enough ?      �

�  4) How would I know if I needs a faster port ?   �

�  5) Okay, I want one ! What should I buy ?        �

�  6) Is there just one type of 16550 UART ?        �

�  7) Supplier information                          �

�����������������������������������������������������

INTRODUCTION

------------

Fast  serial  cards  have  become  very  popular  lately  due to the

proliferation of high speed (V32/V32bis) modems. Whilst the computer

press have been keen to cover modems in general, little coverage has

been given to  serial  cards  (they're  just  not  sexy  enough  for

mainstream  articles).  This  means that, whilst many people want or

need fast serial cards, there is much  confusion  about  them.  This

document attempts to answer the most common questions asked.

1) What is a UART ?

----------------

Throughout this document, we will refer to something called a 'UART'

(pronounced 'you-art').  As this is a very important  component,  it

is important to understand what it is.

A  UART  is  an IC (chip!) which takes parallel data from the PC and

outputs it to the serial port as a timed serial stream of data.   It

is  the  main component of the serial card; all other components are

there to support the UART.  Here is a simple functional diagram:

PC's BUS Slot                         Serial port

1 ------------> ��������ͻ

1 ------------> �  UART  �

0 ------------> �        �

1 ------------> �        � ---------> 11010001

0 ------------> �        �

0 ------------> �        �

0 ------------> �        �

1 ------------> ��������ͼ

2) Why the sudden craze for fast serial ports ?

--------------------------------------------

Now that high speed modems (V32, V32bis) have  become  very  common,

many PC serial ports are having problems running fast enough to keep

up  with the modems which can be driven at speeds of up to 57600 bps

(bits per second).  A standard '8250 UART'  based  serial  port  (or

one  using a multi i/o chip) cannot possibly run at that speed.  The

'16450' UART is only slightly better.

3) Why aren't normal PC ports good enough ?

----------------------------------------

Standard serial ports were designed  to  drive  mice,  plotters  and

modems.  At the time, modems ran at speeds of up to 4800 bps, and we

thought that was fast - anyone who has been using  modems  for  more

than  5  years  will  remember how lucky we thought we were !

Problems with serial ports arise for two reasons:

a) The 8250/16450 or equivalent UARTS are simply not designed to run

at the high speeds required by fast modems. Whilst the  BIOS/UART

will  allow  you  to  select  these speeds, they may not reliably

work at these speeds.

b) PC's are busy things, with lots of interrupts to service. If  the

port is running  at  high  speed,  the  PC  may  miss  interrupts

generated by incoming data - this then means that  the  UART  has

nowhere  to  put  the incoming byte and so it gets overwritten by

the next byte received.  This  usually  causes  CRC  errors.  The

faster you drive the port, the more  CRC  errors  you  will  get.

This is almost *always* a problem if you  are  multitasking  with

MS-Windows or Desqview.

The '16550' UART gets around both of these problems:

a) It is designed to run at  high  speed;  a  16550  chip  will  run

happily  at 57600 bps or faster which is fast enough for even the

fastest modem.

b) The 16550 UART incorporates a 16 byte buffer;  if  the  PC  isn't

ready to received the current byte then the UART can retain it in

the buffer until the PC is ready for it.  16 Bytes is  plenty  as

serial    interrupts   are   serviced   very   regularly.    Many

communications  packages  will  empty  the  FIFO  buffer at every

interrupt   possible,  rather  than  only  taking  one  byte  per

interrupt.  The  buffer  on  the  UART is a 'FIFO'  buffer.  This

simply means that the first byte  in will be the first byte  out.

The alternative to this would be a LIFO buffer, but that would be

useless for a UART as all the data would become scrambled !

4) How would I know if I needed a faster serial card ?

---------------------------------------------------

If you are multitasking using MS-Windows or Desqview, then you  will

almost  certainly need a fast buffered serial card. Typical symptoms

will be CRC errors or block retries reported when downloading files.

As you will probably be using error correction,  then  these  errors

are unlikely to be caused by anything else.

Even  if you are not using Desqview or MS-Windows, you may still get

the symptoms details above, especially if you were driving the modem

at 38400 bps or faster, in which case you would be wise to invest in

a  faster serial card. Having said that, there are some people using

high speed modems whose existing serial port  may  be  adequate,  in

which case there is no point investing in a faster one !

5) Okay, I want one, what should I buy ?

-------------------------------------

The  important part about a fast serial card is its UART; this means

that you may be able to simply swap the UART of your existing serial

card, rather than having to fit a whole new card.  You can  only  do

this  if your current serial card uses a 40 pin UART (8250, 16450 or

equivalent). This needs to be  socketed  for  easy  removal,  unless

you're  particularly  experienced  with  a  soldering iron and fancy

desoldering the chip yourself, but this isn't recommended. If you do

decide to desolder your existing UART,  replace  it  with  a  socket

rather than soldering your 16550 directly in.

Unfortunately, most PC's nowadays seem to come with multi i/o  cards

with one multi-purpose chip which performs serial, parallel and disk

operations; if you have a multi i/o card, it will normally be a PLCC

package  in  the centre of the board ( a flat 'surface mounted' chip

with pins on all four sides). This obviously cannot be replaced with

a  16550.   Some  PC's  have  the  i/o  circuitry  built  into   the

motherboard,  in which case it is also unlikely that you can replace

the UART.

Thankfully, and more easily, you can simply buy  a  new  card  based

around a 16550 UART. This can either replace or add to your existing

serial  ports and your supplier will advise you on the best type.

6) Is there just one type of 16550 UART ?

--------------------------------------

No! As with all  semiconductors, many manufacturers have  jumped  on

the 16550  bandwaggon. This is good inasmuch that there is a choice,

but bad in that some manufacturers have got it  wrong  -  some  have

FIFO  buffers which don't work and some aren't  recognised correctly

by applications (this can sometimes be worked around but ideally get

one which you know will work).

The 16550 chip 'by which others are  gauged'  is  made  by  National

Semiconductor, and the part no. is NS16550AFN.  There is an older NS

version,  but  make  sure you order the 'AFN' type. If your supplier

stocks a different 16550 version, ensure that it is 100%  compatible

with the NS16550AFN, and ideally get a guarantee that you can return

it if you find that it isn't ! The NS type is the most expensive, so

you will probably save money by buying a clone, but take note of the

above guidelines.

The  FIFO  buffer  on  a  16550 chip can be enabled and disabled. By

default it will be DISABLED. Many comms programs will  automatically

detect  and  enable the FIFO buffer, but if yours doesn't, there are

several programs to enable them,  which  you  can  include  in  your

AUTOEXEC.BAT file or another batch file. Ask your supplier.

There are several programs around to test 16550 chips, once they are

installed  so you'll soon know if your new chip/card is okay. If you

can't get hold of one, ask around on BBS's or ask your supplier.

Both  US  Robotics and Hayes make high speed serial cards  which are

very flexible; ask your supplier for details. If your local supplier

cannot get hold of a fast serial card, see below.

7) Supplier information

--------------------

S/E/G  Communications  stock  a range of 16550 UARTs and 16550 based

serial cards at very  competitive  prices.   For  full  details  and

pricing information contact:

����������������������������������������Ŀ

�                                        �

�    S/E/G Communications                �

�    137 Hale Lane                       �

�    Edgware                             �

�    Middlesex HA8 9QP                   �

�    Tel: (081) 959 3377                 �

�    Fax: (081) 959 2137                 �

�    Email: segcom@cix.compulink.co.uk   �

�                                        �

������������������������������������������

Credit Card orders are welcome. Trade enquires welcome.

------------------ End of Document ----------------

(c) 1993 Michael Spalter, S/E/G Communications

This  document may only be distributed un-modified. Preproduction as

part  of  any  commercial  transaction  or  distribution   via   any

non-electronic  medium is prohibited in part of full without written

permission from the author.

----- end of file -----