X-10 Professional Developers Library

For

C

=========================================

|                                       |

|              TUTORIAL.DOC             |

|                                       |

=========================================

Version 3.1

Manual and Software Copyright 1993, 1994

Smart Home Technology

24123 94th Ave. S.

Kent,  WA  98031

(206) 813-8410

ALL RIGHTS RESERVED

INSTALLATION OF THE X10 PDL

What is required to use the X10 PDL?

Any IBM PC compatible computer with a serial port.

MS-DOS 3.0 or later.

A TW523 interface module.

A serial port interface adapter.

A Turbo C / Borland C / Microsoft QuickC or C compiler.

How do I install the X10 PDL?

1. Unzip the library into a temporary directory of your choosing.

ie: A:X10C31 C:\TEMP

2. Copy the header (*.h) files to your working directory.

3. Copy the library (*.lib) files to your working directory.

4. Copy the examples (*.c & *.prj) to your working directory.

5. I suggest that you print and study the following files:

QREF.DOC        Quick reference to X10 PDL commands.

X10_C.H         Include file for using the library.

X10_SHEL.C      A shell of a program for you to use.

X10_EXMP.C      An example of a real working program.

6. You should locate the following switches for your compiler and

set them accordingly:

Memory Model:            Medium

Calling convention:      C

Instruction set:         8088/8086

Default char type:       Signed

Alignment:               Byte

Generate underbars:      On

Merge duplicate strings: Off

Standard stack frame:    Off

Test stack overflow:     Off

Line numbers:            Off

OBJ debug information:   Off

Optimize for:            Speed

Use register variables:  Off

Register optimization:   Off

Jump optimization:       Off

All errors and messages should be turned On.

AN X10 PDL TUTORIAL

One of the first things we would like to get across is that most of the

internal workings of the library have default settings built in.  We

include access to all internal parts of the library for those of you

that are advanced programmers, so don't be intimidated by all of the

functions available in the library or having to know what each one

does.  For most of us you will only need to be concerned with the

functions dealing with modules and timers.

With that out of the way lets go thru the steps of creating an

automation schedule.

STEP 1    Make a listing of all the addresses of your X-10 controllers

and modules.  When you make this listing identify your

modules with the following types:

SWITCH - Wall switch that includes dimming.

LAMP   - Plug-in lamp module.

APPL   - Appliance module or switch, ON/OFF only.

CTRLER - Controller.

Example of a listing for your modules might be:

Location                      Addr        Type

Living room table lamp         A1         LAMP

Living room ovhd lights        A2         SWITCH

Living room fluorescents       A3         APPL

Kitchen coffee pot             D1         APPL

Kitchen dishwasher             D2         APPL

Example of a listing for your controllers might be:

Location                      Addr        Type

Living room lamp table        B1 - 4    Sundowner 4B

Living room wireless          C1 - 8    Hand held 8B

STEP 2    Locate the longitude and latitude and time zone for your

city.  You can get this from a local map or by calling your

local weather service or local library.

STEP 3    Start your C compiler Integrated Development Environment,

IDE, as you normally would.  Open a new file from within your

IDE and select the file X10_SHEL.C. So that you don't change

the original file X10_SHEL.C, you should immediately change

the name of the file to a name of your choosing.  Maybe

X10_TUT.C.

STEP 4    We need to let the system know the custom particulars for

your program.  Go down to about line #32 where we have

provided easy access to setting standard yet unique

constants.  Enter your own custom information into the

appropriate define statements.  The define statements and

what goes into them should be self explanatory.

STEP 5    Now you need to cursor down to the startup() function.  It is

in this function that we need to initialize your modules.

All modules need to be identified to the library with a call

to the function initialize_module().  Add as many

initialize_module() lines as necessary.  You don't need to

initialize controllers but you can if you want to keep track

of their status.

This startup() function will also be where you will

initialize your own global variables and maybe your group

structures.  But for now lets just continue on.

STEP 6    We are now ready to work on your schedule.  Cursor down some

more until you come to the function chk_time_event().  This

function has been provided to make your schedule easier to

manage.  Here is where we will program events that will be

dealing with time of day.

Go to this line:    if (time_is("00:01")) {

We will be inserting some new lines after this one so cursor

down to the first blank line following this one and press

enter to create a new line.

Insert the following block of text, replacing the time and

module address with ones of your own choosing:

if (time_is("07:00"))

turn_module("A1", ON);

This block is pretty self explanatory, if its 7 a.m. then

turn module A1 On.

STEP 7    Now lets go deal with received X10 commands.  Cursor down

some more lines until you come to the function

chk_ctrler_event().  In this function we will respond to

incoming X10 commands.

Go to this line:    if (x10cmd_is(&X10Cmd, "C1", ON)) {

Replace "C1" with one of your controller addresses then add

some functions you would like to happen when that controller

button is pressed on.

For example you might want to have something like this

happen:

if (x10cmd_is(&X10Cmd, "C1", ON)) {

turn_all_modules("O", ON);    /* outside lights on   */

dim_module("A1", 8);          /* table lamp to lvl 8 */

start_timer(1, "00:10:00");   /* start 10 min timer  */

}                                  /* to bring A1 to full */

/* brightness          */

Again substitute your addresses for ours.  As you can see

this block of text is self explanatory, if someone presses C1

ON then turn all modules with house code O ON, dim the module

at address A1 to level 8, and start a 10 minute timer.  Now

we need to identify what we want to do when the timer

expires.

STEP 8    Cursor down a few more lines to the function

chk_timer_event().  In this function we will program the

responses to timers that expire.

Go to this line:    if (timer_expired(SAVE_STATUS_TIMER)) {

Now go to the first blank line after this and press enter for

a new line.  We are going to be adding a new timer block for

timer #1 that we set previously.  Enter the following text:

if (timer_expired(1)) {

flash_module("A1", 5);        /* just for fun flash   */

}

Again this should be self explanatory, if timer #1 has

expired then flash the module at address A1 five times.

Notice that after the module flashes the library will reset

the module to the status it was before the flash, in this

case to level 8.

STEP 9    We have just created a schedule.  Granted most schedules will

have a lot more to it, but there really is nothing more

complicated than what we have just done.  So now save your

program.  Before we go further you need to set the memory

model your compiler is using.  I suggest as a good starting

point use the medium model.

Depending on your compiler you need to create a project file

that would contain the following text, which is for Turbo C:

X10_TUT.C

X10_TCM.LIB

You can now compile to an executable, exit your C compiler

and run your simple schedule from DOS.

Hopefully this gives you a little idea of how easy it really

is to create a schedule.  All you need to do is expand on

what we just went thru.  Take a look at X10_EXMP.C for an

example of a real schedule.  Any problems.... Call us!

NOTES     Now for you QuickC folks, there are some small modifications

that need to be done before you can successfully compile.

1. QuickC 2.5 does not have a clrscr() function.  So we did a

cheap work around, system("cls"), at about line 69.

2. QuickC 2.5 also does not have the functions gotoxy() and

wherey().  So we have commented out this line, about 145.