cFos CAPI 2.0                     'cFos' ISDN CAPI Driver Family

----------------------------------------------------------------

'cFos/DOS' unterstuetzt ab heute CAPI 2.0. Das Executable

heisst CFOS20.EXE bzw. CFOS20N.EXE.

Multiple-Subscriber-Numbers:

Um 'cFos CAPI 2.0' einsetzen zu koennen, muss man zuerst

mittels AT&A0...AT&A9 die MSNs im Modem Profile einstellen,

die dem (Euro-)ISDN Anschluss zugeteilt sind, z.B.

AT&A0=2233445    fuer 2233445 als MSN 0

AT&A1=1234567    fuer 1234567 als MSN 1

AT&A2=9876543    fuer 9876543 als MSN 2

Dies ist unbedingt fuer einen erfolgreichen Verbindungsaufbau

erforderlich!

Das AT&E Kommando selektiert dann bei abgehenden Rufen, mit

welcher MSN man sich melden moechte, z.B. selektiert

AT&E0 in obigem Falle die Nummer 2233445.

Mit dem AT&L Kommando stellt man an, auf welche MSNs man hoeren

moechte. AT&L012 wuerde z.B. alle drei oben angegebenen MSNs

selektieren. Moechte man nur auf die 1234567 hoeren, ist AT&L1

das richtige Kommando.

Hinweis: Bei einem 1.TR.6 Anschluss muss man als MSNs die EAZs

eintragen (die MSNs sind hier also einstellig).

Modem-Register:

In MODEM.DOC sind die neuen CAPI 2.0 spezifischen Modem-Register

ab S100 aufgefuehrt. Man kann aber auch die "alten" Register

benutzen. 'cFos' mapped diese Werte dann in die neuen Register.

Die neuen CAPI 2.0 Register und MSN Settings kann man mit

AT&V1 und AT&V2 anschauen.

Folgende Features sind derzeit von 'cFos CAPI 2.0' noch nicht

unterstuetzt:

- Subaddressing

- Analog-Modem Support

Elink-Mode bei eingehenden Rufen kann mit CAPI 2.0 aus

prinzipiellen Gruenden nicht unterstuetzt werden.

CAPI Causes:

Im Euro-ISDN (und somit auch in CAPI 2.0) werden andere Causes

verwendet, hier die Liste:

CAPI 2.0        Erklaerung                      CAPI 1.1 Vergleich

3481         unassigned number                       34b5

3482         no route to specified transit           34b5

network

3483         no route to destination                 34b5

3486         channel unacceptable                    348a

3487         call awarded and being deli-            3480

vered in an established channel

3490         normal call clearing                    3480

3491         user busy                               34bb

3492         no user responding                      34ba

3493         no answer from user                     34ba

3495         call rejected                           34be

3496         number changed                          34b8

349a         non-selected user clearing              34be

349b         destination out of order                34b9

349c         invalid number format                   34b5

349d         facility rejected                       3490

349e         response to STATUS ENQ.                 3480

349f         normal, unspec.                         3480

34a2         no circuit/channel avail.          34d9/34bb

34a6         network out of order                    34d9

34a9         temporary failure                  34d9/34b9

34aa         switch equipment congestion             34d9

34ab         access information discarded            3480

34ac         req. circuit/channel not avail.         348a

34af         resources unavailable                   34f1

34b1         quality of service unavail.             3480

34b2         req. facility not subscribed            3491

34b9         bearer capab. not authorized            3483

34ba         bearer capab. not present               34d9

34bf         service/option not avail.               34b5

34c1         bearer capab. not implemented           3483

34c2         channel type not implemented            3480

34c5         req. facility not implemented           3490

34c6         only restr.digital info avail.          3483

34cf         service/option not avail.               3483

34d1         invalid call ref. value                 3481

34d2         ident. channel non-existent             3480

34d3         a suspended call exists, but            3487

this call identity does not

34d4         call identity in use                    3488

34d5         no call suspended                       34f0

34d6         call having the requested               3487

call identity has been cleared

34d8         incompatible destination                34ba

34db         invalid transit network                 34b5

34df         invalid message                         34f0

34e0         mandantory info element                 34f0

missing

34e1         message type non-existent or            34f0

not implemented

34e2         message not compatible with             34f0

call state/message type

non-existent or not impl.

34e3         info element non-existent               34f0

or not implemented

34e4         invalid info element contents           34f0

34e5         message incompatible with               3480

call state

34e6         recovery on timer expiry                34f0

34ef         protocol error                          34f0

34ff         interworking, unspecified               34b5