Ethernet Packet Driver MLID, v1.01

Introduction

------------

PDEther.Exe is an ODI to Packet Driver adaptor.  It provides an ODI

interface for an arbitrary Ethernet Packet Driver.  This allows ODI

access to any Ethernet board which has a Packet Driver and also

allows Packet Driver applications and ODI applications using

different network layer protocols to coexist in the same DOS system.

Using PDEther

-------------

In Packet Driver terms, PDEther is an application.  In ODI terms, it

is a LAN driver, also know as a "link driver" or "MLID".  The loading

sequence will be something like this:

1. Load the packet driver supporting your card.

2. Load LSL.

3. Load PDEther.

4. Load IPXODI, NetX, and/or other ODI applications.

Like all good ODI modules, PDEther reads Net.Cfg for configuration

information.  The next section of this document describes PDEther's

requirements for Net.Cfg, including a description of PDEther's Link

Driver section.

Net.Cfg

-------

You must configure at least one 1514 byte ODI receive buffers in the

Link Support section of the Net.Cfg file using the Buffers keyword:

Link Support

Buffers 4 1514

A few extra buffers may allow PDEther to handle congested conditions

better.  If no buffers are allocated or they are not 1514 bytes long,

PDEther prints a warning message.

Some packet drivers will work with smaller buffer sizes, but it is

difficult to tell which will and which won't.  For that reason, i

recommend always using 1514 byte ODI buffers but, on the other hand,

PDEther will try to muddle through even if 1514 byte buffers are not

available.

Your Net.Cfg file should also contain a Link Driver section for

PDEther.  Such a section begins with the unindented text "Link

Driver PDEther".  As is normal in the Net.Cfg file, the section

continues until the next unindented line.  (In other words, all lines

in the Link Driver section should be indented by one or more spaces

or tabs.)

PDEther ignores case, extra white space, and comments when reading

Net.Cfg.  Comments begin with "#" and continue to the end of the

current line.

The Link Driver section of the Net.Cfg file supports three keywords:

INT <hex interrupt>

The INT line specifies the software interrupt of the Packet

Driver the MLID should use.  The value is a hex number

between 60 hex and 80 hex.  The number may optionally be

preceeded by "0x" to indicate a hex number, but the number is

read in hex whether or not the "0x" is present.  If PDEther

sees no INT line, it uses the Packet Driver at software

interrupt 60 hex.

FRAME Ethernet_II

This is just the standard frame type parameter that all MLIDs

support.  Since this driver only supports Ethernet, only

"Ethernet_II" is accepted.  For the same reason, this line is

entirely optional.

PROTOCOL <protocol name> <hex E-type> Ethernet_II

This is the standard protocol identification line.  The

protocol name (case insensitive) identifies the protocol

to LSL.  E-type is a hex number indicating the Ethernet type

value for this protocol.  (Again, it may be preceeded by "0x",

but it is always interpretted in hex.)  The media name is

Ethernet_II.  Again, Ethernet_II is the only media supported,

so contrary to standard Net.Cfg practice, specifying

Ethernet_II is optional.

===>>>	*ALL* protocols this driver is to support must be specified

in the Link Driver section, since the MLID must be able to

tell the Packet Driver specifically about any protocols the

MLID wants to receive.  There is one typical Novell exception:

IPX is set up automatically if it is not specified explicitly.

On the other hand, do *not* specify protocols which the

packet driver is supporting directly.  This will make PDEther

take control of those protocols, so your packet driver

application won't be able to use them.

Here are some example lines.  By the way, if you are using an ODI

TCP/IP protocol stack, you will need these protocol lines in your

Net.Cfg file to tell PDEther about the IP protocols.

Int 60

Frame Ethernet_II

Protocol IP   800  Ethernet_II

Protocol ARP  806  Ethernet_II

Protocol RARP 8035 Ethernet_II

Loading a Packet Driver

-----------------------

PDEther supports the 1.09 Packet Driver specification.  I assume

later Packet Driver interface versions will also work.  In addition, I

have no particular reason to think it wouldn't work on older Packet

Driver interfaces, since it uses no features flagged as new in 1.09,

but i've only been using 1.09.

Releases of the Clarkson Packet Drivers since release 7 support

version 1.09 of the spec.  Release 7 was also the first Packet

Drivers which support the "raw" 802.3 IPX framing convention via "-n"

switch described below.  At this time of this writing, the latest

Clarkson release is release 9.

The details of loading a Packet Driver should be provided with your

Packet Driver and are beyond the scope of this document.  Generally,

you tell the packet driver which software interrupt to use and how to

talk to the hardware through parameters on the command line.

Standard Packet Drivers take *all* input in decimal by default,

although you may specify hex by using the "0x" prefix.  This is

counter-intuitive, since almost all parameters to any packet driver

are numbers which make little or no sense in decimal.

Standard Packet Drivers take the software interrupt as the first

argument.  The software interrupt *must* agree with the INT parameter

in the Link Driver PDEther section of the Net.Cfg file.  (But again,

remember to prefix the packet driver parameter with "0x".  The INT

parameter in Net.Cfg is always interpreted as a hex number.)

If your network uses the popular "Ethernet_802.3" frame type (the

illegal Novell "raw" 802.3 framing type) for IPX traffic, give the

Packet Driver the "-n" switch to get that framing for IPX traffic.

This tells the Standard Packet Drivers to send IPX traffic in the

"raw" 802.3 format.  The "-n" switch does not affect other

protocols.  The default IPX protocol ID (8137 hex) *must* be used so

the Packet Driver can identify IPX packets.

Note that when using the "-n" switch, you continue to specify

"Ethernet_II" in the Net.Cfg file; you do *not* indicate the frame

type Ethernet_802.3 as you would in a normal ODI Link Driver.  The

"raw" 802.3 framing is handled by the Packet Driver without PDEther's

knowledge.

Here's an example of a typical packet driver command line which loads

a Packet Driver on software interrupt 60 hex to support an NE2000

ethernet card on hardware interrupt 5, port 340 hex, using normal

Ethernet framing.

ne2000 0x60 5 0x340

To load the same Packet Driver but have it send IPX packets using

Novell's default Ethernet_802.3 framing instead of the more proper

Ethernet_II, use a command line like this:

ne2000 -n 0x60 5 0x340

Limitations

-----------

Although the Clarkson Packet Drivers now support 802.3/802.2 and

802.3/802.2/SNAP, this MLID does not support those two framing

types.

Some of the less important features of the Packet Driver are not

supported.  For example, it cannot deal with a Packet Driver

supporting multiple boards.

If you are using PDEther under applications other than IPX, each

protocol (except IPX) must be explicitly listed in the PDEther's Link

Driver section.  Although "normal" from the point of view of the

Net.Cfg design, it is quite abnormal in practice.  Novell's TCPIP for

ODI, for example, will automatically add the TCP/IP protocol IDs if

they are not present, but THIS DOES *NOT* HELP PDEther which must

find them in its section of Net.Cfg.  This is unfortunate.  Failure

to list the protocols in the Net.Cfg file is not a detectable error

so all the software will be satisfied, but no arriving packets will

be delivered to IP.

Bibliography

------------

FTP Software, "PC/TCP Packet Driver Specification", version 1.09,

September, 1989.

Novell, "Open Data-Link Interface Developer's Guide for

NetWare DOS Workstation Drivers", Revision A, 30 January 1991.

Conclusion

----------

Let me know if you have any problems.

don provan

donp@novell.com

11/1/91