FREEWARE

Courtesy of KD9JQ

PLL3 Version 2.00

USER'S  GUIDE

INTRODUCTION

PLL3 Version 2.00 provides the designer with an interactive tool in

developing 3rd Order Phase Locked Loops.  Design parameters are now

accessible to the user in making design decisions which previously

involved long, tedious calculations, subject to errors.

PLL3 is an Engineering Tool used to calculate Loop Filter component

values through use of previously developed equations (Ref. 3 - 6).

A perfect integrator is assumed during this process.  PLL3 should

not be considered to be a "Spice" like simulation program.

Through the use of a BODE Plot and subsequent calculations of pertinent

design data the engineer is assured a high probability of certainty that

his/her design will provide the desired performance.

A Spice or other simulation program can then be run on the PLL circuit

using these calculated loop components and user selected component

models to verify that use of these parts will meet design criteria.

Verification of actual circuit performance is left up to the user.

MENU SELECTION

(1) New Design    "For 1st time designs"

(2) Old Design    "Input Component & System Constants of a

previous design or load a previous design"

(3) Circuits      "Schematic representation of the Loop Filter

components and alternatives, Speedup Circuit."

NEW DESIGN

[Use Exponential Values for Numeric Entries "100E-12, 100E3, ETC."]

If you picked (1) NEW DESIGN then the INPUT DESIGN PARAMETERS Window

will pop up requesting input data.  Pop-up Help screens can be pulled

up by entering H.  Press "F1" for default test component values.

1ST ENTRY - Enter desired nominal Phase Margin between of 20 to 70

Degrees. Range of .03 to 89.97 allowed.

2ND ENTRY - Enter the desired Loop 3dB Bandwidth BW (Hz).

3RD ENTRY - Enter the VCO Tuning Sensitivity KV in HZ/VOLT.

The program automatically multiplies this by 2*PI to

provide the correct RAD/SEC/VOLT units.

Page 2

4TH ENTRY - Enter the Phase/Frequency Detector Constant KD in VOLTS/RAD

After entering your value, a window will popup requesting

selection of a generic detector you will be using in your

design.  Enter the number of the detector.  This will now

provide a more accurate value for the Lockin Range, Time,

Pullin Range, Time, and Tracking Range.  Default = 1.

5TH ENTRY - Enter any optional DC Amplification KA (Voltage Ratio)

located between the Detector and Integrator, or Integrator

and VCO. Take care not to saturate any following stage.

Any losses can be entered by values less than one.

Normally use a value of 1 in new designs.

6TH ENTRY - Enter required Frequency Division "N".

7TH ENTRY - Enter VCO Modulation BW in Hz.  If not known then a value

around 1/10TH of the VCO Tuning Sensitivity will suffice.

8TH ENTRY - Enter Open Loop Gain KO (Voltage Ratio) of OP-Amp used for

the Integrator.  This is the DC gain of the OP-Amp below

its' 1st Pole.

9TH ENTRY - Enter OP-Amp 1st Pole Frequency in Hz. (Where DC Gain drops

3dB)

10TH ENTRY - Enter Reference Frequency FR (Hz) being used.

A new window will now appear displaying the Filter Time Constants and

desired Phase Margin.  A Warning message will appear if the Loop Gain

is inadequate at this point.  The user can press F10 to adjust KA

to bring the design into range (more than one attempt may be required)

or <CR> to bypass.  If you use the "bypass" option then the parameters

may be incorrect or non-exsistent.

Once the correct Gain has been found enter a standard value for

R2 (Ohms).  1000 ohms is normally a good starting point.

Another Window will then pop up displaying calculated values for the

Loop Filter.  If these values a close to standard values press (2).

If not, press (1) and enter a new R2.  Higher values of R2 will give

lower Cap. values, while lower R2 values will increase Cap. values.

Once you have pressed (2) you will be asked to enter standard values

for R1, C1, R2, C2.  Units are Ohms and Farads.

The designer can simply hit the Carriage Return when this window pops

up if the calculated values are to be used.

The BODE PLOT will then be displayed.

Page 3

OLD DESIGN

If you picked (2) OLD DESIGN then the INPUT DESIGN PARAMETERS Window

will pop up requesting if you wish Manual Data Entry of an previous

design or Select a previous design file.

If the "Manual Entry" is selected a window will pop up requesting

you to input data for R1, C1, R2, C2 and the some of the system

parameters requested in the "NEW DESIGN" window.

If the "Select Entry" is selected then a previous design file can be

picked by entering the name of the file.  The .PL3 extension is assumed.

Loop Gain range warnings are disabled for old designs.  Messages are

re-activated once the Interative Mode [F3] is entered.

The BODE PLOT will then be displayed.

BODE PLOT

The BODE Plot provides the designer with useful visual input for

determining design criteria.  Loop Gain range detection to +/- 1%

is in effect and will prompt you for KA Adjustments or <CR>.

In most cases if the T1 > T2 > T3 rule was followed, then the BODE Plot

will show the operating BW (Marker fo) situated between f2 and f3.

Marker f1 should be << f2 and marker fr >> fo.  The operating BW could

be anywhere between f2 and f3 but optimum is the geometric center of

their frequencies.  This will provide room for component tolerances and

temperature coefficients.

Any positions outside of these markers will flag the designer with a

Warning Message and a general solution to take.

The Loop Filter Response plot (White) should normally fall on or below

unity gain at f2, fo, and f3 for adequate gain margin.

The Closed Loop Gain (Blue) should be greater than the Loop Filter

Response (White).

Observe the Closed Loop Gain relative to unity (0dB) at Marker fr.

This will indicate the relative amount of Noise Contribution or

Suppression due to the N divisor taken at the Reference Frequency.

See REF 7 for more details on Sideband Suppresion.

Observe the value of VCO Error Response (Green) below Marker fo

relative to unity (0dB).  This indicates the Low Frequency Tracking of

the VCO taken at .1*Fo.

The Phase Margin plot will show the Phase Response determined by T2, T3.

Page 4

TRANSIENT ANALYSIS RESPONSE

Pressing the TAB key will display the Normalized Frequency and Phase

Step Transient Responses for the actual Damping Factor versus a range

of 0 to 10 Time Constants.  The Frequency Step will response is shown

as yellow, while the Phase Step is shown as green.  Generally takes

5 Time Constants for the response to settle to the 5% point.  The user

is reminded that this is only an approximation of the 3rd Order

Transients utilizing 2nd order equations.  This can still be useful to

the designer in determining settling times though.

DESIGN PARAMETER DOCUMENTATION WINDOW

The DOCUMENTATION WINDOW can be accessed by Pressing the Return Key.

A compilation of current design data will then be displayed.

The designer then has the option of printing out the documentation

(SHIFT + F1), starting a new design (F2), modifing current values (F3),

show the BODE Plot again (F4), or saving the design to a file (F5).

Pressing "Q" will end the program without any file save.

Pressing "H" will popup a window defining the display parameters.

The NEW PARAMETERS selection (F3) will pop up a Window of all current

design parameters.  To change values simply enter the selection and then

enter the new value.  Pressing the Return Key enters the new value.

To Recalculate the loop filter component values press the "R" key. This

is useful when changing parameters other than the Loop Filter Components

R1, C1, R2, and C2. To display the new BODE Plot to see the affects of

any changes done to the Loop Filter Components press the "P" Key.

SYMBOL DEFINITIONS

R1   Input Gain Set Resistor to Inverting Input of OP-Amp

(Integrator Stage).

C1   Set F1 Pole Frequency in conjunction with R1.  Connected from

Inverting Input of OP-Amp (Integrator Stage) to junction of R2/C2.

R2   Sets F2 Zero in conjunction with C1.  Sets F3 in conjunction

with C2.  Parallel connection with C2 in series with C1 and OP-Amp

(Integrator Stage) output.

C2   Sets F2 Zero in conjunction with C1 and R2.  Sets F3 Pole in

conjunction with R2.  Parallel connection with R2 in series with

C1 and OP-Amp (Integrator Stage) output.

T1   Time Constant of R1 * C1

T2   Time Constant of R2 * C1

Normally T2 = R2 * (C1 + C2)    See REF 3-6

T3   Time Constant of R2 * C2

F1   F1 = 1 / (KO * 2 * PI * T1)

F2   F2 = 1 / (2 * PI * T2)

Page 5

F3   F3 = 1 / (2 * PI * T3)

LOOP BW @ (T2T3) is the Loop BandWidth defined by T2 and T3 excluding

other system parameters.

LOOP BW @ 0 Gain is the Loop Bandwidth at Unity Gain.

LOCK UP RANGE FL Approximate range (Hz) where lock occurs without cycle

slippage.  ~= KL*K*T2/(2*PI*T1)  where K=KA*KD*KV/N

See Table 1 under PHASE DETECTOR KD for KL

LOCK UP TIME TL  Approximate time required to lock with initial

frequency delta less than the Loop BW.  ~= 1/WN  (Sec)

PULLIN RANGE FP  Approximate Pullin Frequency (Hz)

FP ~= KL*SQR(4*DF*WN*K-2*WN^2)/(2*PI)

Multiply by N to ref to the VCO.   See Page 76-77 of REF 1 & REF 6.

PULL IN TIME TP  Time to lock from an initial frequency error equal to

FP.  TP ~= SQR((PI*T2^2)*(WP/K)) (Sec)  where WP=2*PI*FP.

TRACKING RANGE FT  Linear Tracking range.  Dynamic range (Hz) for Stable

operation. Multiply by N to ref to the VCO.

FT ~= KL*1.8*WN*(1+DF)/(2*PI) where WN ~= SQR(K/T1)

and  DF ~= .5*WN*SQR(T2^2-T3^2)

REF FREQUENCY FR is the system reference frequency in Hz to the Phase

Detector.  Equal to VCO frequency divided by N.

NOISE (N,BW,FR) Design Aid for showing the relative amount in dB of

added noise due to the N-Divisor modified by the Loop BW to Reference

Frequency relationship.  Values approaching 0 dB desired for least

amount of added noise.

Noise(N,BW,FR) = 20*LOG(SQR(1+(N*BW/FR)^2))

NOISE BW  Noise BW (Hz) of the Loop.

Noise BW ~ Wn*(DF+(.25/DF))/PI

Note:  Minimum Noise BW occurs when GM LO = GM HI.

NOISE VCO (dBV) of Noise at .1 of the Loop BW by the VCO filtering.

This value is calculated whereas a realistic maximum value of

suppression is normally 40dB - 60dB. This is the VCO Error Response

or the Low Frequency Tracking capabilites of the VCO.

PHASE MARGIN is the Phase Margin in Degrees at Unity Gain Loop BW.

= ATN(WO*T2) - ATN(WO*T3) - ATN(WO/WM) - (WO/WR)*((N-1)/N)   (Deg)

See REF 5      WM = BV * 2 * PI    WR = FR * 2 * PI

GAIN MARGIN LO & HI =>   Margins of > 20 are desirable.

GM LO = 20*LOG(KO*T1) dB

GM HI is calculated at 0 Degree crossover point of Phase Margin.

VCO MODULATION BW (BV) is the maximum modulation BW of the VCO. Use a

value ~ 1/10 of Kv if not known.

Page 6

OP AMP 1st POLE is the Vendor specifed 1st Pole Frequecny (Hz)

usually in the 10HZ to 2KHZ range.

OP AMP OPEN LOOP GAIN KO is the Vendor specified DC Gain of the OP-Amp

below the 1st Pole.

FREQ DIVISION N is the Frequency Division of VCO to Reference Frequency.

VCO TUNING SENSITIVITY KV is the VCO tuning rate in HZ/Volt.

This value is multiplied by 2 * PI in the program to provide the proper

units of RAD/Sec/Volt.

PHASE DETECTOR KD  Phase Detector Sensitivity in VOLTS/RAD.  REF 1,7

Digital Phase/Freq Detectors  KD ~= .10 to .80 V/RAD

See 6.3 of REF 1 and REF 7 for more information on Phase Detectors.

Table 1       REF1 Pg 55 , REF8 Pg 11

Detector #         KL

1           1

2          PI/2

3          PI

4          PI/2

5          PI

6          2*PI

BODE PLOT RESPONSES

OPEN LOOP GAIN (dBV) is displayed in Red.  Desirable to maintain

Loop Filter Response (White) below this point all the way up to F3.

O.L.G. defined as G(jw)H(jw)

CLOSED LOOP GAIN (dBV) is displayed in Blue on the BODE Plot.

C.L.G. defined as G(jw) / (1 + G(jw)H(jw))

E/EN (VCO Error Response) is displayed in Green.  This is the Low

frequency tracking of the loop (dBV) due to the VCO integration.

E/EN defined as 1 / (1 + G(jw)H(jw))

LOOP FILTER RESPONSE is displayed in White.  This displays the Loop

Filter Response as defined by the Loop Filter Components and System

Constraints.  See REF4 & REF5

PHASE MARGIN is displayed in Yellow.

1) Phase Margins of 20 to 70 Degrees are desirable.

Range of .03 to 89.97 Degrees allowed.

2) Optimum Operating BW on the BODE Plot should fall at the geometric

center of F2 and F3 as defined by T2, T3.

3) Low Phase Margins give less Loop stability while Higher Margins give

improved Loop stability.  REF 2

OPERATING LOOP BW  (Marker fo)

Let BW be 1/50th of the Reference Frequency as a starting point if not

known.  REF 2

Page 7

A larger Operating Loop BW gives increased Noise reduction but decreased

Reference Frequency Rejection, while smaller Loop BW's give decreased

Noise reduction but higher Reference Frequency Rejection.  REF 2

DYNAMIC BODE PLOTTING

This feature allows the designer to "move" around the plot.

Pressing the up arrow key will Zoom in and the down arrow key will

Zoom out by changing the magnitude scaling.  A range of +/- 15db to

+/- 150dB is provided.  Initial plotting defaults to +/- 60dB.

Pressing the left arrow key will shift the plot lower in Bandwidth while

pressing the right arrow key will shift the plot up in Bandwidth.  The

Plot always defaults to the Loop Bandwidth the first time.

Paging up will shift the plot up by 1/2 screen, while paging down will

shift the plot down 1/2 screen.

Scaling is automatic.  The Phase Margin Plot does not change in this

feature except for BW tracking.

The "SHIFT + F1" key will cause a B&W Screen Dump to printer.

See DISPLAY.CFG configuration file for setting up your printer.

The Foreground and Scaling colors can be set in the PLL3.CFG file.

The Plotting colors still remain the same, therefore care should

be taken in selection of the colors used.  The color range for

each is 0 - 15.

PLL3.CFG

line 1 = background color

line 2 = grid color

line 3 = Closed Loop "N" Flag  0 for no N, 1 for N

MARKERS:

fO = Operating BW point         (Violet)

f1 = Lower Pole of Loop Filter  (White)

f2 = Zero of Loop Filter        (Green)

f3 = Upper Pole of Loop Filter  (Yellow)

fr = Reference Frequency        (Cyan)

GENERAL DESIGN RULES:

T1 > T2 > T3  likewise  f1 < f2 < f3

f2 < fo < f3  where fo is the operating BW

fr >> fo

A Warning sign will Flash after the BODE Plot is finished indicating

any problems and a general solution.

Page 8

REFERENCES

REF 1   PHASELOCK TECHNIQUES

Second Edition (C) 1979 by John Wiley & Sons, Inc.

Floyd M. Gardner,  Author

REF2    ELECTRONIC DESIGN 19, February13, 1978  Pgs 134-137

"Optimize Phase Lock Loops to meet your needs - or why you can't"

Andrzej B. Przedpelski, Author

REF3    RF DESIGN,  March/April 1983  Pgs 18-28

"PLL Primer"  Part 1  by Andrzej B. Przedpelski, Author

REF4    RF DESIGN,  May/June 1983  Pgs 12-28

"PLL Primer"  Part 2  by Andrzej B. Przedpelski, Author

REF5    RF DESIGN,  July/August 1983  Pgs 48-

"PLL Primer"  Part 3  by Andrzej B. Przedpelski, Author

REF6    RF DESIGN,  November 1987  Pgs 88-96

"PLL Primer"  Part 4  by Andrzej B. Przedpelski, Author

REF7    MOTOROLA MECL DEVICE DATA  Q1/89 DL122 REV 4 Pgs 6-22 to 6-36

REF8    DIGITAL PLL FREQUENCY SYNTHESIZERS  Theory and Design

(C) 1983 by Prentice-Hall, Inc.

Ulrich L. Rohde, Author

SoftWare Innovations For Technology Enterprises (R) 1989