Introduction to Falken Serial I/O capabilities.

Falken's serial I/O capabilities are provided by an internal interrupt

handler.  It is not necessary (or smart) to load an external program to

control the serial ports.  Falken uses an 'interrupt-driven poll' method

to control the serial ports.  In short, this is what happens :

Falken examines the hardware configuration, and determines the

highest baud rate used by the system.  It then calculates how many

times per second it must test the serial hardware to see if any

of the modems need servicing.  The timer chip in the PC is then

reprogrammed to interrupt Falken at that interval, and Falken

queries the appropriate serial ports, looking for ports that

require servicing (character received, transmit ready, DCD change,

etc.).

This method overcomes a very serious shortcoming in PC hardware

design - there are only 2 IRQ vectors assigned to serial ports.

If more than 2 serial devices are required, they must share these

IRQ vectors.  If the devices that are sharing a vector are on

different cards (2 or more internal modems, for instance), then

interrupts can be lost.  So Falken does not use the normal serial

I/O interrupts.  In fact, the IRQ setting of the serial device is

irrelevant, and not used by Falken at all.

Falken's serial driver is Fossil compatible.  This means that the driver

conforms to most of the Fossil specification found in the FOSSIL.DOC file

included with this archive.  It supports a 'text' mode also.

In 'text' mode, Falkens internal Fossil drivers will :

Echo incoming characters (keystrokes) back out to the port.

Filter input according to an internal translation table.

Characters to be echoed back to the port are held until the

current output has completed.  An hex FF (0xff) character is

used as an output separator.  If you start typing in the middle

of a menu, for example, your keystrokes will not be echoed until

the current transmission is completed.

The 'flush output buffer' is more intelligent - allows the current

output line to complete before flushing the output buffer.  In

effect, it removes everything AFTER the next carriage return.

On output, Line Feed characters are ignored, but a line feed is

automatically transmitted after every carriage return.

Backspace is echoed as '<BS><SPACE><BS>', so that a BS is destructive,

but a BS will only be echoed if there are characters in the input

buffer.

Characters in the output buffer are not transmitted while there are

characters in the input buffer.  This prevents the user's

input from being intermixed with output text.

The net effect is a more fluid user interface.  Once the user's input

starts echoing out, it will not be interrupted by output from Falken.

Aborted output will not terminate in the middle of a line of text.

Output from Falken will not be interrupted by echoed keystrokes, until

the current 'record' is completed.

A door program that receives input from Falken via message queues does

not have to worry about echoing characters or cr/lf conversions, etc.

The line will still be in 'text' mode, since Falken is still in

control of the port.  This is not true if the door uses the port directly.

If a door program requests direct control of the serial port, the port

is placed into 'raw' mode.  All 'text' mode controls are disabled, and

the program is given a raw serial port which responds to Fossil calls.

To take direct control of the serial port, a door *MUST* inform Falken

that it wishes to do so.  Otherwise, both Falken and the door program

will be trying to obtain input and send output to the same port, and

very likely nothing will work right.  To gain control of the serial port,

simply send a message to Falken on your tasks queue, with message type '8'

like this :

int m1;

m1 = 8;

send_md_msg(outq, &m1, 2);

As soon as Falken receives this message, it places the port into binary

mode (i.e. takes it out of text mode), and relinquishes all control of

the port.  When the door stops, Falken takes the port back.

The only real checking that Falken does while the door has control of

the port, is to check for Carrier Detect.  If DCD drops, Falken will

clean up the port and terminate the door.

See the file transfer protocols for examples of taking control of the

serial port.

Falken's built-in FOSSIL support provides the following standard FOSSIL

services to doors that have control of the serial port :

Function 0  : Set Baud Rate (will not work if port is FIXED BAUD)

Function 1  : Transmit Character with wait

Function 2  : Receive Character with wait

Function 3  : Status Request

Function 4  : Initialize Port

Function 5  : De-initialize Port

Function 6  : Raise/Lower DTR

Function 7  : Return System Timer Parameters

Function 8  : Flush Output Buffer

Function 9  : Purge Output Buffer

Function A  : Purge Input Buffer

Function B  : Transmit Character, no wait

Function C  : Non-destructive 'peek' at input buffer

Function F  : Flow Control

Function 18 : Read Block

Function 19 : Write Block

Function 1A : Start/Stop Break Signal

Function 1B : Get Driver Information

Functions 1, 2, and 8 specify that the Fossil driver will not return to

the caller until the condition is satisfied.  In this case, the Fossil

driver will use the Cswitch RELINQ() call to allow other tasks to run

until the condition is met.  Performance is not affected by using these

calls.

In addition, Falken provides additional FOSSIL services not defined in

FOSSIL.DOC.  Some of these services are reserved for Falken.  Services

available for doors tasks are :

Function 41h : get address of I/O control structures

Inputs :

none

Outputs :

ax : segment address of i/o control structure array

bx : offset address of i/o control structure array

Note : This function is useful if you need to manipulate the

flags and variables that the internal Fossil driver uses

to control serial I/O.  In most cases, this direct

manipulation is not needed, and can be dangerous to the

system.  It is recommended that you use the other Fossil

calls to control the serial port.

Function 43h : Set buffer sizes

Inputs :

DX = Line Number

AH = 43h

BX = new input buffer size

CX = new output buffer size

Returns :

AX :  0 if successful, -1 if error

Note : This function is used to alter the Fossil's internal buffer

sizes.  Falken allocates 4096 bytes of buffer space to each

line.  Normally, the input and output buffers use only 512

bytes each.  These buffers can be re-sized dynamically,

provided the total buffer space does not exceed 4096 bytes.

Sizes must be multiples of 16, i.e. 64, 128, 512, 2048, etc.

This is mostly useful for file transfer protocols.  Upload

protocols will allocate larger input buffers, while download

protocols will allocate larger output buffers.

Function 44h : Return total buffer space reserved for each line.

Inputs :

AH = 44h

Returns :

AX :  Total buffer space limit (Falken sets this to 4096)

Function 49h : Put a character into the Fossil input buffer

Inputs :

AH = 49h

DX = line number

AL = character to be placed on the line's input buffer

Outputs :

none

Note : This function places the specified character into the line's

input buffer.  If the line is in text mode, the character will

be echoed to the user, just as if it were typed by the user.

If the sysop is monitoring this line, the character will

be displayed in the monitor window, just as if it were typed

by the user.

Function 4ah : Return number of bytes available in output buffer

Inputs :

AH = 4ah

DX = line number

Outputs :

AX = number of bytes available in output buffer.  This is the

number of bytes that can be sent to the Fossil driver

without data loss.  This value is equal to :

[output buffer size] - [bytes in output buffer]

Function 4bh : Return number of bytes available in input buffer

Inputs :

AH = 4bh

DX = line number

Outputs :

AX = number of bytes waiting in the Fossil input buffer