File:    MANUAL_E.DOC

Purpose: Describes how to use the NAVLOGIK System.

Note:    This is an evaluation version of the NAVLOGIK system. The programs

NAVLOGP.exe and NAVLOGI.exe are provided for you to exercise

the system. The programs NAVLOGIC.exe and NAVLOGPC.exe are not

included in this evaluation version.

NAVLOGIK SYSTEM

NAVLOG USER'S MANUAL

Version 2.0

Copyright (c) Denney Systems, 1995

Copyright (c) Denney Systems, 1995

869 Amador Circle

Carson City, NV 89705

NOTICE :

The data logger software and this manual are copyrighted and

contain proprietary information. Reproduction and distribution of the

material contained herein must be in accordance with the licensing

agreement. All rights are reserved. Denney Systems reserves the right

to make improvements in the product described in this manual at any

time, and without notice. This product is licensed without any

warranty of any kind. Use of this product constitutes agreement that

Denney Systems, its owners, employees, affiliates and any other

parties related to this product shall not be held liable for any

damages which may result from the use of this product.

i

TABLE OF CONTENTS                                NAVLOG USER'S MANUAL

---------------------------------------------------------------------

CHAPTER                                                     PAGE

1    INTRODUCTION                                      1-1

1.1  General Description                          1-1

1.2  Terminology Primer                           1-1

1.3  NAVLOG Features                              1-1

1.4  NAVLOG Versions                              1-1

1.5  Navigation Receiver Requirements             1-2

1.6  PC Requirements                              1-2

2    SIMPLIFIED INSTRUCTIONS                           2-1

2.1  Serial Connection                            2-1

2.2  Nav Receiver Output Format                   2-1

2.3  Running NAVLOG                               2-1

2.4  On Line Help                                 2-1

3    NAVLOG VERSION SELECTION                          3-1

3.1  Overview                                     3-1

3.2  NAVLOGI.exe                                  3-1

3.3  NAVLOGP.exe                                  3-1

3.4  NAVLOGIC.exe                                 3-1

3.5  NAVLOGPC.exe                                 3-1

3.6  Summary                                      3-2

4    PROGRAM EXECUTION                                 4-1

4.1  Installing NAVLOG                            4-1

4.2  Executing NAVLOG                             4-1

4.3  Command Line Parameters - Expanded           4-2

4.4  Logging File - Path and Directory            4-2

5    DYNAMIC COMMANDS                                  5-1

5.1  Overview                                     5-1

5.2  Bell                                         5-1

5.3  Echo                                         5-1

5.4  Pause                                        5-1

5.5  Resume                                       5-1

6    FILE DECOMPRESSION - UNPAKNAV.EXE                 6-1

6.1  Description                                  6-1

6.2  Usage                                        6-1

ii

TABLE OF CONTENTS                                NAVLOG USER'S MANUAL

---------------------------------------------------------------------

APPENDIX

A    PC TO NAV RECEIVER CONNECTIONS                    A-1

A.1  Basic Cable Connections                      A-1

A.2  Dual Serial Port Connections                 A-1

A.3  Communication Protocols                      A-1

B    DEFAULT NAVLOG PARAMETERS                         B-1

C    SYSTEM MESSAGES                                   C-1

C.1  NAVLOG Messages                              C-1

C.2  UNPAKNAV Messages                            C-1

D    NMEA 0183 INFORMATION                             D-1

D.1  Serial Communication Parameters              D-1

D.2  Sentence Elements                            D-1

D.3  Sentence Structure                           D-2

D.4  Sentence Formatters                          D-2

D.5  Sample Sentences                             D-4

E    ACRONYMS AND ABBREVIATIONS                        E-1

INDEX

INDEX                                                  index 1

iii

Chapter 1 - INTRODUCTION                         NAVLOG USER'S MANUAL

---------------------------------------------------------------------

1.1  General Description

------------------------

The NAVLOG system runs on a PC, in the background, logging data

from a navigation receiver (e.g., GPS, Loran, etc.). When a NAVLOG

program is executed, it installs itself, terminates and stays

resident in memory (TSR). While in memory, NAVLOG monitors the PC's

serial port, checks for targeted navigation sentences, and logs the

sentences to a disk file.

1.2  Terminology Primer

-----------------------

Com                - serial communication.

Data               - unit of navigation information.

Foreground program - the application you run with NAVLOG.

Nav                - navigation.

NAVLOG             - General NAVLOGIK system.

PC                 - computer which runs the NAVLOG program.

Receiver           - the nav receiver providing serial data.

Note 1: Hereafter, "NAVLOGxx" will be used as an abstract reference

to the version of NAVLOG you choose to implement.

Note 2 : See Appendix E -- Acronyms and Abbreviations, for more

terminology.

1.3  NAVLOG Features

--------------------

NAVLOG permits you to run another program while simultaneously

capturing nav data. You may choose logging intervals from two seconds

to eighteen hours. Optional run time data compression is available to

minimize file size which is particularly useful if your PC uses a RAM

drive.

NAVLOG searches through the large volume of nav data and targets

only the data you wish to record. NAVLOG has several versions to

enable optimized logging with your particular PC and receiver

hardware combination.

1-1

Chapter 1 - INTRODUCTION                         NAVLOG USER'S MANUAL

---------------------------------------------------------------------

1.4  NAVLOG Versions

--------------------

The NAVLOG system has four program versions which use different

methods to capture and store the serial data. The capture methods are

the com interrupt service routine (ISR) and com polling. The storage

methods are ASCII data and compressed binary data.  The program

versions are:

Version        Description

------------   --------------------------------

NAVLOGI .EXE   Com ISR, no compression

NAVLOGP .EXE   Com Polling, no compression

NAVLOGIC.EXE   Com ISR, data compression

NAVLOGPC.EXE   Com Polling, data compression

The compressed data can be expanded at any time with the

UNPAKNAV.EXE program (see Chapter 6). Compressed data files are

approximately one-fourth the size of the non-compressed files.

1.5  Navigation Receiver Requirements

-------------------------------------

The receiver unit must output data in compliance with the

National Marine Electronics Association's (NMEA 0183) standard for

interfacing marine electronic devices.

1.6  PC Requirements

--------------------

The logging computer must be PC DOS capable and have a minimum

of one serial com port. The com port must be capable of 4800 bps

serial data communication.

1-2

Chapter 2 - SIMPLIFIED INSTRUCTIONS              NAVLOG USER'S MANUAL

---------------------------------------------------------------------

2.1  Serial Connection

----------------------

Connect line "A" of the receiver to the PC's received data (RXD)

line. Connect the receiver "B" line to the PC's signal ground (SGND)

line. (See Appendix A for the connection diagrams.)

2.2  Nav Receiver Output Format

-------------------------------

Refer to your receivers user's manual and select "NMEA 0183" as

the output data format.

2.3  Running NAVLOG

-------------------

To install and execute NAVLOGxx with no command line parameters,

at the DOS prompt, type:

NAVLOGxx

and press return.

Note 1: Substitute I, IC, P, or PC for the xx in NAVLOGxx.

Note 2: Your data will be saved to disk in accordance with the

default parameters outlined in Appendix B.

2.4  On Line Help

-----------------

To receive help information from NAVLOG, at the DOS prompt type:

NAVLOGxx ?

and press return.

2-1

Chapter 3 - NAVLOG VERSION SELECTION             NAVLOG USER'S MANUAL

---------------------------------------------------------------------

3.1 Overview

------------

Review the following descriptions to select the most suitable

NAVLOG version for your PC/receiver combination.

3.2 NAVLOGI.exe

----------------

This version installs a com interrupt service routine (ISR) for

the designated com port (1, 2, 3, or 4). It requires exclusive access

to the designated com port. Use this version if:

(1)  You are not running a foreground program; or

(2)  Your foreground program does not use a com port; or

(3)  You can provide data to your foreground program via one com

port and data to NAVLOGI via a separate com port.

3.3  NAVLOGP.exe

----------------

This "P" version uses a polling method to collect data. Use when

NAVLOGP must share the com port with the foreground program.

NAVLOGP.exe will take over the PC for approximately 2 seconds during

each data log.

3.4  NAVLOGIC.exe

-----------------

Identical to NAVLOGI.exe except the data is compressed prior to

being logged.

3.5  NAVLOGPC.exe

-----------------

Identical to NAVLOGP.exe except the data is compressed prior to

being logged.

3-1

Chapter 3 - NAVLOG VERSION SELECTION             NAVLOG USER'S MANUAL

---------------------------------------------------------------------

3.6  Selection Summary

---------------------

The ISR versions are more user transparent but require a

dedicated com port. The polling versions can "time share" their com

port with a foreground program; however, they tie up the computer for

two seconds every log cycle. If your foreground program requires nav

data and you have a second com port, you can use a special cable to

run a separate com line to the second port without disturbing the

data on the first. (See Appendix A.2 -- Dual Serial Port Connections)

Finally, to save disk space -- use the compressed data versions.

3-2

Chapter 4 - PROGRAM EXECUTION                    NAVLOG USER'S MANUAL

---------------------------------------------------------------------

4.1  Installing NAVLOG

----------------------

Execute NAVLOGxx prior to running your foreground program.

NAVLOGxx will install in memory as a TSR and remain until the

computer is reset. The data logging can be halted at any time with

dynamic commands (see Chapter 5).

4.2  Executing NAVLOG

---------------------

If you execute NAVLOGxx without command line parameters then the

default parameters will be used (see Appendix B). NAVLOG usage with

parameters :

NAVLOGxx [filename] [COMPORT] [interval] [target_1 ... [target_n]

Variable  Meaning                            Values / Range

--------  -------                            --------------

filename  File name to log data on disk      n/a

COMPORT   Serial port to read NMEA data      1, 2, 3, or 4

interval  Logging interval in seconds      * 2 to 65535

target_1  first formatter target             n/a

target_n  last formatter target              (up to fifteen targets)

* 4 to 65535 for the "P" versions

Example 1:

NAVLOGIC navdat.cmp 2 20 RMB XTE

This command will install the com interrupt/compressed data

version of NAVLOG, log data to a file named "navdat.cmp", expect

com data from serial port #2, log every 20 seconds, and

target the $xxRMB and $xxXTE NMEA sentences.

Example 2:

NAVLOGP

This command will install the polling version of NAVLOG, log

data to a file with the default file name, expect data from the

default com port, use the default logging interval, and target

the default targets.

4-1

Chapter 4 - PROGRAM EXECUTION                    NAVLOG USER'S MANUAL

---------------------------------------------------------------------

Example 3:

NAVLOGI navdata.asc 1

This command will install the straight ASCII file version. The

data will be logged to a file named "navdata.asc", the com port

will be #1. Default interval and target strings will be used.

Note: Command line parameters are order dependent; e.g., if you

specify a logging interval, you must also specify the filename and

comport preceding the interval parameter.

4.3  Command Line Parameters - Expanded

---------------------------------------

The file name parameter requires you to specify a file extension

if one is desired.

The minimum interval for NAVLOGI and NAVLOGIC is 2 seconds. This

corresponds to the NMEA sentence block cycle. The minimum interval

for NAVLOGP and NAVLOGPC is 4 seconds. Unlike the "I" versions, the

"P" versions will disable foreground program activity for 2 seconds

while a sentence block is being read. Keep this in mind when

selecting a logging interval with the "P" versions. With respect to

both methods, NMEA data is usually output in 2 second cycles so you

should use an even logging interval (i.e., do not try to log every 3

seconds, use either 2 or 4 as a value).

NAVLOG targets the sentence formatter of the address field of a

NMEA sentence. Once it finds a match, NAVLOG extracts the entire

sentence. The formatter consists of the last three characters of the

address field. (See Appendix D for more information on NMEA 0183

sentence structure). For example, to capture the GPS minimum specific

data sentence "$GPRMC", enter RMC as your target parameter; to

capture the Loran-A cross track error sentence "$LAXTE", your target

parameter would simply be XTE.

4.4  Logging File - Path and Directory

--------------------------------------

NAVLOG remembers the drive and directory that were current when

the NAVLOG program was initiated. NAVLOG will write data to the same

drive and directory even if you change them with your foreground

program. The foreground drive and directory are not affected.

4-2

Chapter 5 - DYNAMIC COMMANDS                     NAVLOG USER'S MANUAL

---------------------------------------------------------------------

5.1  Overview

-------------

Once installed (executed) NAVLOGxx can be sent several commands

to change its behavior. These commands must be sent one at a time and

cannot be combined on the command line. To use the dynamic commands

type :

NAVLOGxx command

where "command" may be one of the valid command tokens: BELL, ECHO,

PAUSE, and RESUME. Note: if NAVLOGxx has not been installed, and a

dynamic command is issued, an error message will be issued. These

command tokens are described below.

5.2  Bell

---------

The Bell command is a toggle which enables and disables the

sounding of the PC's bell (beep). The bell sounds when NAVLOG goes to

log data. The interval between soundings will correspond to the

logging interval. This feature is useful to verify proper logger

operation.

5.3  Echo

---------

The ECHO command is a toggle which enables and disables the

display of captured target sentences. This feature is useful when

checking proper logger operation, prior to, or during foreground

program execution. (Note: The echo display could interfere with your

foreground program's output.)

5.4  Pause

----------

The PAUSE command suspends logging and closes the NAVLOG com

until a RESUME command is issued.

5.5  Resume

-----------

RESUME opens the NAVLOG com and resumes the logging of data.

5-1

Chapter 6 - FILE DECOMPRESSION, UNPAKNAV.EXE     NAVLOG USER'S MANUAL

---------------------------------------------------------------------

6.1  Description

----------------

UNPAKNAV.EXE (UN PAcK NAV data) is a utility for expanding the

compressed data files created by NAVLOGIC or NAVLOGPC. The use of

UNPAKNAV produces an ASCII file of the captured NMEA nav data.

6.2  Usage

----------

At the DOS command line type :

UNPAKNAV packedfile [asciifile] [R]

Where "packedfile" is your compressed file name and "asciifile"

is the name of the file being created. The packed file name is

required and the ASCII file name is optional. The optional switch

"R", is used if you want to write over an existing file with the same

name. If the output file name is omitted, then the packed file base

name will be used with ".asc" used as its extension.

Sample usage 1:

UNPAKNAV navdata.cmp navdata.asc r

UNPAKNAV looks for an existing compressed file named

"navdata.cmp" and creates an ASCII file named "navdata.asc". Any

existing file named "navdata.asc" is written over.

Sample usage 2:

UNPAKNAV voyage.dat

The compressed file named "voyage.dat" is expanded producing an

ASCII file named "voyage.asc". If a file named "voyage.asc"

already exists, UNPAKNAV will abort unless the "R" switch is

used.

6-1

Appendix A - PC TO NAV RECEIVER CONNECTIONS      NAVLOG USER'S MANUAL

---------------------------------------------------------------------

A.1  Basic Cable Connections

----------------------------

NAVLOG requires a minimum two wire hookup. This arrangement

assumes the PC is configured as DTE with either a DB-9 or DB-25

serial cable connector.

NMEA NAV DEVICE                            PC (DTE)

---------------                       -----------------

LINE                         DB-9 PIN#  DB-25 PIN#  USED AS

---------  ----------  --------

A    --------------------->     2          3         RXD

(not used)             3          2         TXD

B    <-------------------->     5          7         SGND

A.2  Dual Serial Port Connections

---------------------------------

A cable can be configured so that data is available for two or

more serial ports. This arrangement is used when the foreground

program needs nav data and either the NAVLOGI or NAVLOGIC program is

employed.

NMEA NAV DEVICE                            PC (DTE)

---------------                       -----------------

COM #1      COM #1

LINE                         DB-9 PIN#  DB-25 PIN#  USED AS

---------  ----------  --------

A1 --------------->     2          3         RXD  1

B1 <-------------->     5          7         SGND 1

-----> A1

A

-----> A2

-----> B1

B

-----> B2                        COM #2     COM #2

DB-9 PIN#  DB-25 PIN#  USED AS

---------  ----------  --------

A2 --------------->     2          3         RXD  2

B2 <-------------->     5          7         SGND 2

A.3  Communication Protocols

----------------------------

Most DOS PCs support the RS-232-C serial protocol. Serial

protocol for the NMEA 0183 standard is EIA-422-A. Interfacing between

RS-232-C and EIA-422-A is possible in most cases.

A-1

Appendix B - DEFAULT NAVLOG PARAMETERS           NAVLOG USER'S MANUAL

---------------------------------------------------------------------

Parameter           NAVLOGI             NAVLOGIC

---------------     ------------        -----------------

Data file name      navdata.asc         navdata.cmp

Serial Port         COM 1               COM 1

Logging interval    4 seconds           4 seconds

Targets             RMB,RMC,RMA,RMZ     RMB,RMC,RMA,RMZ

Echo                off                 off

Bell                off                 off

Parameter           NAVLOGP             NAVLOGPC

---------------     ------------        -----------------

Data file name      navdata.asc         navdata.cmp

Serial Port         COM 1               COM 1

Logging interval    10 seconds          10 seconds

Targets             RMB,RMC,RMA,RMZ     RMB,RMC,RMA,RMZ

Echo                off                 off

Bell                off                 off

B-1

Appendix C - SYSTEM MESSAGES                     NAVLOG USER'S MANUAL

---------------------------------------------------------------------

C.1  NAVLOG Messages

--------------------

Message : ERROR - NAVLOG already installed.

Meaning : You cannot install a new NAVLOG over an old NAVLOG.

Message : ERROR - NAVLOG not installed.

Meaning : You tried to issue a dynamic command before NAVLOG was

installed.

Message : [no com or no targeted data present].

Meaning : NAVLOG either does not detect any serial data or no

targeted data is detected.

Message : SUCCESSFULLY INSTALLED !

Meaning : NAVLOG has been installed and is resident in memory.

C.2  UNPAKNAV Messages

----------------------

Message : Error : unable to open input file.

Meaning : Unable to locate the named file to decompress.

Message : Error : unable to open output file.

Meaning : Problem opening the named output file.

Message : Error : output file already exists.

Meaning : You cannot write over an existing file unless you

use the 'R' switch.

C-1

Appendix D - NMEA 0183 INFORMATION               NAVLOG USER'S MANUAL

---------------------------------------------------------------------

D.1  Serial Communication Parameters

------------------------------------

Baud rate      4800

Data bits      8

Parity         None

Stop bits      One

Note: Designed for NMEA 0183 standard, versions 1.5 and 2.0.

D.2  Sentence Elements

----------------------

A data sentence contains, in the order shown, the following elements:

"$"                    ASCII value 36 - Start of sentence

<address field >       TALKER identifier and sentence formatter

[","<data field>]      Zero or more data fields

.

.

.

[","<data field>]

["*"<checksum field>]  Optional checksum field

<CR><LF>               ASCII 13, 10 - End of sentence

D-1

Appendix D - NMEA 0183 INFORMATION               NAVLOG USER'S MANUAL

---------------------------------------------------------------------

D.3  Sentence Structure

-----------------------

Summary explanation of the sentence structure :

$aaccc,c---c*hh<CR><LF>

ASCII          DESCRIPTION

-----          -----------

"$"            START OF SENTENCE.

aaccc          ADDRESS FIELD - Alphanumeric characters identifying

type of TALKER (aa), and the sentence formatter (ccc).

The NAVLOG system allows the user to target the

sentence formatter (ccc).

","            FIELD DELIMITER - Starts each field except address and

checksum fields. If it is followed by a null field, it

is all that remains to indicate no data in a specific

field.

c---c          DATA SENTENCE BLOCK - Follows the address field and is

a series of data fields containing all of the data to

be transmitted.

"*"            OPTIONAL CHECKSUM DELIMITER - Follows the last data

field of the sentence

hh             OPTIONAL CHECKSUM FIELD

<CR><LF>       SENTENCE TERMINATING CHARACTERS

D.4  Sentence Formatters

------------------------

Note: these are sentence address field characters 3, 4, and 5 (ccc).

AAM - Waypoint Arrival Alarm

ALM - GPS Almanac data

APB - Autopilot Sentence "B"

ASD - Autopilot System data

BEC - Bearing & Distance to Waypoint, Dead Reckoning

BOD - Bearing, Origin to Destination

BWC - Bearing & Distance to Waypoint

BWR - Bearing & Distance to Waypoint, Rhumb Line

D-2

Appendix D - NMEA 0183 INFORMATION               NAVLOG USER'S MANUAL

---------------------------------------------------------------------

BWW - Bearing, Waypoint to Waypoint

DBT - Depth below transducer

DCN - DECCA position

DPT - Depth

FSI - Frequency Set Information

GGA - Global Positioning System Fix data

GLC - Geographic position, Loran-C

GLL - Geographic position, Latitude/Longitude

GSA - GPS DOP and Active Satellites

GSV - GPS Satellites in view

GXA - TRANSIT position

HDG - Heading, Deviation & Variation

HDT - Heading, True

HSC - Heading Steering Command

LCD - Loran-C Signal Data

MTW - Water Temperature

MWV - Wind speed and angle

OLN - Omega Lane Numbers

OSD - Own Ship Data

RMA - Recommended Minimum Specific Loran-C data

RMB - Recommended Minimum Navigation Information

RMC - Recommended Minimum Specific GPS/TRANSIT data

ROT - Rate of Turn

RPM - Revolutions

RSA - Rudder Sensor Angle

RSD - RADAR System Data

RTE - Routes

SFI - Scanning Frequency Information

STN - Multiple data ID

TRF - TRANSIT Fix Data

TTM - Tracked Target Message

VBW - Dual Ground/Water Speed

VDR - Set and Drift

VHW - Water Speed and Heading

VLW - Distance traveled through the water

VPW - Speed, Measured Parallel to Wind

VTG - Track Made Good and Ground Speed

WCV - Waypoint Closure Velocity

WNC - Distance, Waypoint to Waypoint

WPL - Waypoint Location

XDR - Transducer Measurements

XTE - Cross-Track Error, measured

XTR - Cross-Track Error, Dead Reckoning

ZDA - Time & Date

ZFO - UTC & Time from Origin Waypoint

ZTG - UTC & Time to Destination Waypoint

D-3

Appendix D - NMEA 0183 INFORMATION               NAVLOG USER'S MANUAL

---------------------------------------------------------------------

D.5  Sample Sentences

---------------------

Sample 1:

$GPRMB,A,0.04,L,SAN,LAX,3356.55,N,11824.48,W,78.2,320.0,,V*3A

1      2 3    4 5   6   7         8          9    10  11 12 13

1.  GPS Nav unit, Recommended minimum navigation information.

2.  Data status - good.

3.  Cross track error - 0.04 nautical miles.

4.  Direction to steer - steer left.

5.  Origin waypoint - SAN.

6.  Destination waypoint - LAX.

7.  Destination latitude - 33 degrees, 56.55 minutes north.

8.  Destination longitude - 118 degrees, 24.48 minutes west.

9.  Range to destination - 78.2 nautical miles.

10.  Bearing to destination - 320.0 degrees true.

11.  Closing velocity - (null or zero, data missing).

12.  Arrival status - have not passed arrival circle.

13.  Checksum - 3A hexadecimal.

Sample 2 :

$GPRMC,220345,V,3659.01,N,10258.01,W,111.0,270.1,200894,010.3,E*A5

1      2      3 4         5          6     7     8      9      10

1.  GPS Nav unit, Minimum specific GPS/TRANSIT data.

2.  time - 22:03:45 hours UTC.

3.  Nav receiver status - bad.

4.  Latitude - 36 degrees, 59.01 minutes north.

5.  Longitude - 102 degrees, 58.01 minutes west.

6.  Ground speed - 111.0 knots.

7.  Ground track - 270.1 degrees true north.

8.  Date - August 20, 1994.

9.  Magnetic variation - 10.3 degrees east.

10.  Checksum - A5 hexadecimal

D-4

Appendix D - NMEA 0183 INFORMATION               NAVLOG USER'S MANUAL

---------------------------------------------------------------------

Sample 3 :

$LCRMA,A,4226.265,N,07125.890,W,14172.33,26026.71,8.53,275.,14.0,W*53

1      2 3          4           5        6        7    8    9    10

1.  Loran-C unit, Recommended Minimum Specific Loran-C Data.

2.  Nav receiver status - good.

3.  Latitude - 42 degrees, 26.265 minutes north.

4.  Longitude - 71 degrees, 25.890 minutes west.

5.  Time difference A - 14172.33 micro seconds.

6.  Time difference B - 26026.71 micro seconds.

7.  Ground speed - 8.53 knots.

8.  Ground track - 275 degrees true.

9.  Magnetic variation - 14.0 degrees west.

10.  Checksum - 53 hexadecimal.

D-5

Appendix E - ACRONYMS AND ABBREVIATIONS          NAVLOG USER'S MANUAL

---------------------------------------------------------------------

A              NMEA device communication line

ASCII          American Standard Code for Information Interchange

B              NMEA device communication line

bps            bits per second

C              Compression

COM            Serial communication port

com            Serial Communication

DB-9           Nine pin serial cable connector

DB-25          Twenty five pin serial cable connector

disk           diskette

DOS            Disk Operating System

DTE            Data Terminal Equipment

EIA            Electronic Industries Association

EIA-422-A      EIA standard for serial communication

GPS            Global Positioning System

I              Interrupt

ISR            Interrupt Service Routine

nav            Navigation

NAVLOG         NAVigation LOGger system

NAVLOGI        Navigation Logger - Interrupt

NAVLOGIC       Navigation Logger - Interrupt - Compression

NAVLOGP        Navigation Logger - Polling

NAVLOGPC       Navigation Logger - Polling - Compression

NMEA           National Marine Electronics Association

P              Polling

PC             Personal Computer

RAM            Random Access Memory

RMA            (See Appendix D)

RMB            (See Appendix D)

RMC            (See Appendix D)

RMZ            (See Appendix D)

RS-232-C       An EIA standard for serial communications

RXD            Serial Received Data line

SGND           Serial Signal Ground line

TSR            Terminate and Stay Resident

TXD            Serial Transmitted Data line

UNPAKNAV       UNPAcK NAVigation data

XTE            (See Appendix D)

E-1

Index                                            NAVLOG USER'S MANUAL

---------------------------------------------------------------------

Commands,

Command Line, 2-1, 4-2

Dynamic, 4-1, 5-1, C-1

Bell, 5-1, B-1

Echo, 5-1, B-1

Pause, 5-1

Resume, 5-1

Communications,

Cables, 3-2, A-1, E-1

Connectors, A-1, E-1

Methods

Interrupt, 1-2, 3-1, 4-1, E-1

Polling, 1-2, 3-1, 4-1, 4-2, E-1

Protocols

EIA-422-A, A-1, E-1

RS-232-C, A-1, E-1

Data,

Compression, 1-2, 3-1, 3-2, 4-1, 6-1, E-1

Decompression, 6-1, C-1

Navigation - defined, 1-1

Directories, 4-2

Fields - address, 4-2, D-1, D-2

Foreground program, 1-1, 3-1, 3-2, 4-1, 4-2, 5-1, A-1

Help - on line, 2-1

Interval - logging, 1-1, 3-1, 4-1, 4-2, 5-1, B-1

ISR, 1-2, 3-1, 3-2, E-1

Messages - system, 5-1, C-1

Names - file, 4-1, 4-2, 6-1, B-1, C-1

file extensions, 4-2, 6-1

NMEA, 1-2, 2-1, 4-1, 4-2, 6-1, A-1, D-1, E-1

Path, 4-2

Receivers - navigation

GPS, 1-1, 4-2, D-2, D-3, D-4, E-1

Loran, 1-1, 4-2, D-3, D-5

Serial Data, 1-1, 1-2, C-1

Serial Port, 1-1, 4-1, A-1, B-1

Switches, 6-1, C-1

Targets, 1-1, 4-1, 4-2, 5-1, B-1, C-1, D-2

TSR, 1-1, 4-1, E-1

UNPAKNAV.EXE, 6-1

index 1