+--------------------+

| INT 14 H Extension |

+--------------------+

CDRV.COM is a "Terminate and Stay Resident" program. It provides

several new functions for INT 14H.

These functions use the same parameters as the usual BIOS ones :

the function number is to be placed in AH, the COM port number

(0 to 7) is to be placed in DX, whenever required.

To install CDRV.COM, just type CDRV [Enter] at the DOS prompt,

or insert this command into the AUTOEXEC.BAT file.

The functions then available are :

=======

AH =  0	Init port using AL (same as BIOS)

=======

AL bit meaning is :

+---+---+---+

| 7 | 6 | 5 |

+---+---+---+

|   |   |

0   0   0  for  110 bps,

0   0   1  for  150 bps,

0   1   0  for  300 bps,

0   1   1  for  600 bps,

1   0   0  for 1200 bps,

1   0   1  for 2400 bps,

1   1   0  for 4800 bps,

1   1   1  for 9600 bps.

+---+---+

| 4 | 3 |

+---+---+

|   |

0   0  for No Parity,

0   1  for Odd Parity,

1   1  for Even Parity.

+---+

| 2 |

+---+

|

0  for 1 stop bit,

1  for 2 stop bits.

+---+---+

| 1 | 0 |

+---+---+

|   |

1   0  for 7 data bits,

1   1  for 8 data bits.

Upon exit AH = LSR and AL = MSR (see function 3).

=======

AH =  1	Send character in AL (same as BIOS)

=======

Upon exit AH = LSR with MSB is set in case of T.O.

=======

AH =  2	Recv character in AL (same as BIOS)

=======

Upon exit AL = char and AH = LSR.

In case of T.O., AL = MSR and AH = LSR with MSB set.

=======

AH =  3	Get port status (same as BIOS)

=======

Upon exit AH = LSR and AL = MSR. The meaning of their bits is :

+---+---+---+---+---+---+---+---+

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | (for AH)

+---+---+---+---+---+---+---+---+

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---> data ready (OK to read)

|   |   |   |   |   |   +-------> overrun

|   |   |   |   |   +-----------> parity error

|   |   |   |   +---------------> framing error

|   |   |   +-------------------> break detected

|   |   +-----------------------> THRE (OK to send)

|   +---------------------------> TSRE

+-------------------------------> always zero

+---+---+---+---+---+---+---+---+

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | (for AL)

+---+---+---+---+---+---+---+---+

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---> delta clear to send

|   |   |   |   |   |   +-------> delta data set ready

|   |   |   |   |   +-----------> delta ring indicator

|   |   |   |   +---------------> delta carrier detect

|   |   |   +-------------------> CTS

|   |   +-----------------------> DSR

|   +---------------------------> RI

+-------------------------------> CD

=======

AH =  4	Write AL to MCR (bit 7 = 0) or LCR (bit 7 = 1)

=======

+---+---+---+---+---+---+---+---+

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | (for MCR)

+---+---+---+---+---+---+---+---+

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---> DTR

|   |   |   |   |   |   +-------> RTS

|   |   |   |   |   +-----------> OUT1

|   |   |   |   +---------------> OUT2 (enable IRQ)

|   |   |   +-------------------> enable loopback (*)

|   |   +-----------------------> always zero

|   +---------------------------> always zero

+-------------------------------> 0 to select MCR

In loopback mode : TXD ---> RXD

DTR ---> DSR

RTS ---> CTS

OUT1 ---> RI

OUT2 ---> CD

+---+---+---+---+---+---+---+---+

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | (for LCR)

+---+---+---+---+---+---+---+---+

|   |   |   |   |   |   |   |

|   |   |   |   |   |   0   0	....... 5 data bits

|   |   |   |   |   |   0   1 ....... 6 data bits

|   |   |   |   |   |   1   0 ....... 7 data bits

|   |   |   |   |   |   1   1 ....... 8 data bits

|   |   |   |   |   |

|   |   |   |   |   0 ............... 1 stop bit

|   |   |   |   |   1 ............... 2 stop bits

|   |   |   |   |

|   |   |   |   0 ................... no parity

|   |   |   |   1 ................... parity

|   |   |   |

|   |   |   0 ....................... odd parity

|   |   |   1 ....................... even parity

|   |   |

|   |   0 ........................... no stick parity

|   |   1 ........................... stick parity

|   |

|   0 ............................... no break

|   1 ............................... send break

|

1 to select LCR

=======

AH =  5	Set baud rate according to CX (units are bits per second).

=======

=======

AH =  6	Set SEND TIME OUT, where CX = # of clock tics.

=======

=======

AH =  7	Set RECV TIME OUT, where CX = # of clock tics.

=======

=======

AH =  8	Set BREAK DURATION in clock tics (CX = duration).

=======

=======

AH =  9	Send BREAK on port DX.

=======

=======

AH = 10	Get version number in AH (major) and AL (minor).

=======

=======

AH = 11	Set I/O parms for asynch port (COM1: to COM8:)

=======

DX = COM port number (0 to 7)

CX = I/O base address (eg 03F8H)

AL = IRQ mask (eg 00010000B for IRQ 4)

=======

AH = 12	Get I/O parms for asynch port (COM1: to COM8:)

=======

DX = COM port number (0 to 7)

CX and AL set upon exit (see function 11)

=======

AH = 13	Set IRQ mode for port number DX.

=======

Upon exit AX = 0 if OK, AX # 0 otherwise (*).

(*) An error is returned if any com port is already IRQ driven,

or if no IRQ line is specified for this port.

=======

AH = 14	Reset IRQ mode for current port.

=======

=======

AH = 15	Check for char in buffer.

=======

CX returns # of chars in buffer, DX returns # of lost chars (if any)

AH = number of port IRQ driven (0 for none, 1 for COM1:, etc...)

AL holds the next char to be read from buffer (the char is not

removed from buffer).

=======

AH = 16	Get char from ring buffer into AL.

=======

CX returns # of chars still waiting in buffer,

DX returns # of lost chars (if any),

AH returns the IRQ driven port #, MSB is set in case of T.O.