VESA DPMS screen saver

����������������������

OVERVIEW

DpmSave is a screen saver for MSDOS PC's with a power-saving monitor

that conform to the VESA DPMS standard.

This screen saver does not simply blank the screen, it

switches your monitor into power-saving (dpms suspend) mode.

REQUIREMENTS

Required:

- PC compatible computer running MS-DOS

- VGA compatible graphics card

- color computer monitor that conforms to the VESA DPMS standard

Optional:

- XMS memory and a driver program for upper memory blocks.

(If this is available, DpmSave can load itself into an

upper memory block, using none of the main 640kB memory.)

WARNING

DPMS (Display Power Management Signaling) signals the monitor to

activate suspend or sleep mode by switching off one or both video

sync signals.

OLDER COMPUTER MONITORS WITH NO DPMS SUPPORT CAN BE PERMANENELY

DAMAGED IF THEY ARE OPERATED WITHOUT SYNC SIGNALS!

The program DpmSave can not detect whether or not your monitor does

support DPMS. It's your responsibility to find out and not use this

program if your monitor does not support DPMS. Please consult the

manual of your monitor or ask the dealer or manufacturer.

If DPMS is not mentioned in the manual, then the monitor most

probably does not support it.

If you hear a high pitch sound from your monitor while DpmSave

has activated or the picture gets squeezed or scrambled for more

than about two seconds, then immediately switch the monitor off!

Please note that absence of these warning signals does not tell

that your monitor does support DPMS. You can not try it, you must

look it up in the manual.

LICENSE

This program including its documentation is copyrighted:

(c) Copyright 1995 by Klaus Hartnegg, Kleist-street 7,

D-79331 Teningen, Germany.

Use and distribution is allowed only according to this license.

Any other use would violate international copyright laws.

You are not required to accept this license, but then you

are not allowed to use or copy this program in any way.

By using or copying this program you accept this license!

The program is provided 'as is' without warranty of any kind,

either expressed or implied. You are using this program entirely

on your own risk. The copyright owner may not be liable for any

damage caused by the use of or the inability to use the program.

You may copy and distribute this program if you copy the

whole package (all files including this documentation)

and no file is modified in any way.

You may use this version of DpmSave free of charge.

QUICK START

This section is for experienced users only. Please skip this

if you want to know more about the program before trying it.

1. Make sure that your computer meets all requirements mentioned above,

especially the monitor must conform to the DPMS standard!

2. Execute the command 'DPMSAVE t30s', then wait 20 seconds without

pressing any keys. You should now hear several short beeps.

After another 10 seconds the screen should become blank and

your monitor should switch to suspend mode.

IF IT DOESN'T WORK, PRESS RESET IMMEDIATELY TO REBOOT YOUR PC!

3. If it did work: reboot to get rid of the trial version, then

execute the command 'DPMSAVE t15 umb klr' to install it with more

useful options (activate after 15 minutes, load into upper memory,

define hotkey left+right shift key).

4. To have it installed each time you start your computer,

insert the above command in the file c:\autoexec.bat

DESCRIPTION

DpmSave monitors all keyboard input. Whenever no key was pressed for a

certain time, it switches off the vertical sync signal. This causes

a VESA DPMS monitor to go to suspend mode.

DpmSave will issue beeps starting 10 seconds before activation.

If you hear this and don't want it do activate right now, press

any key to restart the counter.

Once DpmSave is activated, it continues to monitor the keyboard.

Any further keypress will cause the program to switch the monitor

back on.

DpmSave will not activate while Microsoft Windows is running.

Whenever it wants to activate, it tests for presence of Windows.

If it is found, the time counter is reset. See the section

about LIMITATIONS below for more informations.

USAGE

There are two ways to use this program:

command mode or resident mode.

In resident mode it activates automatically after a certain time

of inactivity (no keys pressed).

This mode is described in the next chapter (installation).

The other method is a the command line use. This is explained here:

Syntax:    DpmSave now [blank | both] [nowait]

Options:   blank  : leave sync signals on

both   : switch *both* sync signals off

nowait : terminate after blanking screen

Example:   DpmSave now both

The word 'now' is required to select command mode usage.

With no further arguments the program will:

- switch the computer monitor off

- wait for a keypress

- switch the monitor back on

There are a few options to adjust the behaviour:

The option 'blank' tells it to switch of the video signal only,

not a sync signal. Some monitors want it this way. The manual

of your monitor should tell you.

The option 'both' tells it to switch off both sync signals instead

of only one.

The option 'nowait' causes the program to terminate after switching

the monitor off, i.e. the monitor will stay off until you switch it

back on somehow (for example with the command 'mode co80').

Please note that the options 'both' and 'blank' do currently work

only in the command line mode, not in resident mode.

INSTALLATION

To install DpmSave resident in memory, you must specify the time

after that power saving shall be activated. If you specify no time,

the program will not stay resident, but display its online help.

Syntax:    DpmSave [tX[s]] [umb] [kXY] [nobeep]

Options:   tX     : install and activate after X minutes

tXs    : install and activate after X seconds

umb    : load into upper memory if available

kXY    : define hotkey

nobeep : do not beep shortly before activating

Example:   DpmSave t15 umb klr

To have DpmSave started automatically each time the computer is started,

include the command from the example above in the file C:\AUTOEXEC.BAT

IMPORTANT NOTE:

You should choose an activation time that does not cause the monitor

to be switched on and off to often because this can shorten the lifetime

of the monitor. Power-Saving should be activated only a few times per

day and it should usually not be deactivated shortly after activation.

The manual of you monitor should define safe limits for this.

HOTKEY:

The power saving can optionally be activated by pressing a hotkey.

When this key is pressed, power saving will be immediately be

activated without waiting for the activation time to pass.

To enable this feature, a hotkey must be specified when installing

DpmSave.

The hotkey is a combination of two, three or four of the special keys

Alt, Ctrl, Left-Shift, Right-Shift pressed simultaneously.

To define a hotkey, use the option kXY, where X and Y are keys from

the following table:

A: Left Alt

C: Left Ctrl

L: Left Shift

R: Right Shift

Example: to use Alt+Ctrl as hotkey, specify the option KAC.

You could use the command line: DpmSave t10 umb kac

Then whenever you press the Alt and Control-Key together,

the power saving will be immediately activated.

MEMORY USAGE

This is a TSR (terminate and stay resident) program. This means that

part of it will stay resident in memory.

I have made every effort to make the memory requirement as small as

possible. The result is one of the smallest TSR programs that I have

ever seen: the memory-resident part occupies only 512 bytes!

DpmSave can even load itself into an upper memory block,

using absolutely none of the lower 640kB of memory.

> DO YOU WANT TO BE ABLE TO WRITE SUCH TSR PROGRAMS YOURSELF

> USING TURBO PASCAL? PLEASE CONTACT ME FOR INFORMATIONS ABOUT

> MY TSR DEVELOPER TOOLKIT!

Loading DpmSave into upper memory should be done with the 'umb'

option of the program, not with the loadhigh-command of a memory

manager. With a memory manager about 20000 bytes of upper memory

are required during installation. If this amount is not available,

loading will fail. When loading itself high, DpmSave can do this

even with only 512 bytes available. What a difference!

If you load DpmSave low (i.e. not into upper memory), DOS does

automatically allocate additional memory for a copy of all environment

variables. DpmSave will release this memory. This creates a memory

hole. This memory will be recycled if another program is loaded that

needs the same amount for its environment. Otherwise the hole stays empty.

Hint: You can save memory by installing TSR programs before setting

environment variables.

LIMITATIONS

The current version of DpmSave has some limitations.

Some of them will likely be removed in future releases:

Monochrom monitors are not supported

Mouse movements are not recognized, only keyboard usage

DpmSave will not activate while Microsoft Windows is running

because inside Windows it cannot detect keyboard input. Windows

stops all memory resident programs that intercept the keyboard

interrupt. If somebody knows how to change this, please tell me!

Please forgive my poor english, it's not my native language!

KNOWN BUGS

If the display mode changes while the monitor is in power-saving mode

(for example the computer switches into graphics mode), then the mode

might be disturbed when DpmSave de-activates. Use the DOS command

MODE CO80 to reset the display.

The internal timer is not reset to zero when Windows is quit.

This means that the program will activate earlier than it should

if you do not press any keys after leaving Windows.

Some monitors seem to like better if the other sync signal

is switched off. I will try to offer both in future versions.

If you find another bug or problem, or have suggestions, please

write me an e-mail to my Internet address below.

FUTURE PLANS

de-install and unload from memory

change parameters without reinstallation

optionally require password for deactivation (see my program KeyLock)

optionally load into high memory area instead of upper memory block

switch off hard disks (many modern IDE and almost all SCSI disks)

monitor mouse movements and mouse keys (anybody interested?)

This program will *never* activate inside Microsoft Windows when

installed resident. The command mode should work in Windows though.

Further suggestions, comments etc. are welcome.

REVISION HISTORY

v1.0  23-Apr-95: first published version

v1.1  02-May-95: fix: beep forgot to save cx-register

change: command line syntax

new: load itself into XMS upper memory block

new: hotkey

v1.2  13-May-95: new: needs less memory in upper memory block

v1.3  27-May-95: new: load itself into DOS 5+ upper memory block

v1.3a 06-Jun-95: change: docs updated

v1.3b 21-Jun-95: fix: set overflow register

new option: nobeep

v1.3c 10-Aug-95: use smallest umb block when allocating via xms

v1.3d 27-Aug-95: fix: int 1b was disturbed when using the umb option

new options: now, blank, both, nowait

v1.3e 03-Oct-95: new options: on, off, unload

LEGAL STUFF

All trademarks mentioned in this document are trademarks or registered

trademarks of their respective companies and are hereby acknowledged.

Turbo Pascal is a great Pascal Compiler from Borland. MS-DOS and

MS-Windows are from Microsoft Corp. VGA, VESA and DPMS are likely

registered too. VGA was developped by IBM. VESA is the Video

Electronic Standard Association. DPMS is the Display Power Management

Signalling standard defined by the VESA. Some trademarks may be

missing in this list; this does not mean that they are not protected,

I just forgot to mention them. If I made other mistakes, please

forgive me - I'm not a lawyer, just a programmer.

CONTACTING THE AUTHOR

This software is under development.

Comments and suggestions are welcome!

Internet e-mail:

hartnegg@einstein.freinet.de   (home machine)

hartnegg@ruf.uni-freiburg.de   (university)

Address:

Klaus Hartnegg

Kleist-Strasse 7

D-79331 Teningen

Germany

-end-