Doc for BENCH12.SCR 12/21/92

How to run.---------------------------------------------------

First, completely configure AutoCAD for the driver environment

to be tested.  Use either the drivers configuration program

and/or AutoCAD video configuration to set this up.  Ensure that

32 bit display space operation is enabled in the driver, if it

has this option.

NOTE:  This benchmark will not work with any driver that only

supports 16 bit display space, as that these drivers are not

true ADI 4.2 drivers.

After configuration, return back to DOS.

All of the Autodesk supplied drivers will execute this

benchmark.  The ACAD.DWG prototype drawing should be the default

one shipped by Autodesk.

Starting from the directory where ACAD.EXE is located, enter

ACAD.  You might want to use the STATUS command to see how much

memory is available, and to see how much memory is used by the

driver without any drawing loaded.

Use the SCRIPT command to invoke BENCH12.SCR.  Depending on your

system speed, amount of memory, and driver, this benchmark will

take anywhere from under 2 minutes up to A LONG TIME.  On a

486/66 DX/2 with 16 meg, Autodesk VGA driver takes a little

under 5 minutes, and about 11 meg of memory.

The files used.--------------------------------------------------

This script file benchmarks driver performance using 32 bit

display list with Release 12.  It must be run from the ACAD

directory (or the main directory where ACAD.EXE is located).  It

assumes the standard AutoCAD installation to find the following

files:

SAMPLE\PSSHADOW.DWG

SAMPLE\HOUSEPLN.DWG

SUPPORT\CHROMA.DWG

You must have read-write access to the directory where the

script was started from, because its creates a temporary file

$$$TEMP.EPS for the postscript output/input functions.  The

TEMPPREFIX SETVAR variable cannot be used because the PSIN

function cannot be used from LISP due to an AutoCAD bug.

The only place any files are used is at the top of the script

file, so you may easily change the path names to enable this to

work on a network.

You should have at least 11 meg of free memory for AutoCAD to

use run this benchmark.  The Autodesk STANDARD VGA 640x480 16

color driver uses about 11 meg to run the script.

The amount of extra memory required by the display driver will

make a difference in the performance of this benchmark.

What this benchmark measures.----------------------------------

This benchmark was developed due to requests by many users,

customers, graphics board manufacturers, users groups, and

reviewers to design a benchmark that shows actual AutoCAD

performance as affected by a display list driver.  Traditional

benchmarks only show the benefits of faster zoom and pan

operations, but they ignored the side-effects that a driver may

bring to the system, caused by normal editing operations.

The main objective was to never generate a regen.  Autodesk is

promoting that 32 bit display space built into AutoCAD R12

dramatically reduces the need for a regen, so this script will

not generate a regen on any driver that is fully ADI 4.2 and

supports 32 bit display space, after the initial Zoom Extents.

The second objective was to show the performance gains or losses

because a display list driver is installed.  Many edits are

performed which typically cause display list drivers to

dramatically increase their memory usage.  We received many

requests that the benchmark show when a driver does not

effectively manage display list editing and memory management.

Typical display list operations are also performed (zooming and

panning) to show traditional display list gains.

The third objective was to test the driver with 32 bit display

list objects, to ascertain if the driver performs correct

scaling and clipping.

About the Script.----------------------------------------------

This script file first inserts PSSHADOW.DWG, PSOUT it to an .EPS

file, and then imports it with PSIN, to get the shaded image.

This portion is not timed since it is virtually 100% CPU and

Hard disk related.

The drawing is generated by moving the shaded image, importing

the HOUSEPLN.DWG drawing, importing the HOUSEPLN.DWG drawing at

a reduced scale, importing the CHROMA drawing, and importing the

CHROMA drawing at a reduced scale.  The total time for this

section is reported at the end.

A ZOOM E is performed (which will cause a regen) and a donut is

created in the corner, and this time is reported at the end.

A series of ZOOM and PAN are used to test the classic display

list benefits, and its time is reported at the end.  Accelerated

boards should benefit from this section and should have smaller

times than non-accelerated boards.  About 28 bits of display

list space (out of 31) are used to prevent regens at very high

resolutions and other aspect ratio considerations.  The donut

created in the Zoom E section is used to check polygon clipping.

A number of edits are performed such as rotations, UNDO,

changing of PLINE widths, DONUT, and circles.  This section

contains redraws after most edits, simulating a typical user

cleaning up the pick blips, which shows the effect of edits on

display list performance.  This time is reported at the end.

The final section tests MOVE and ERASE of large items, and is

reported at the end.

At this point, the original drawing is completely erased, and a

small drawing is created tallying the results.

A STATUS command is execute to show the amount of virtual memory

used by AutoCAD and the driver.

What to look for.-----------------------------------------------

If a regen occurs, most likely the driver does not implement the

full 32 bit display space, and is an ADI 4.1 driver (Release 11)

pretending to be an ADI 4.2 driver.

During the edits, check to see if redraws are showing the

previous edits.  These may appear as "temporary or flashing

ghosts".  If so, then the driver is not performing real-time

display list editing.  If the Virtual Memory Allocated to

Program grows  more then a couple of meg, then the driver is not

re-using erased display list memory.  Some drivers may cause the

system to page to the hard disk excessively due to the amount of

memory they are using.

If you "hang", reboot, get "Divide by zero", "Abnormal Program

Termination", "Fatal Error", or other messages (or lack of),

then it is the driver that caused the problem.

In general, run the benchmark with the Autodesk supplied VGA

driver to get a feel for what the display should look like

during the execution, then run with the driver to test, to see

if the driver is operating correctly.

The future.---------------------------------------------------

Some of the areas that this script does not address are:

Multiple Viewports, Paper Space, AVE Render, Grid, Command Area

text speed, dialog-box speed, and others.

We are more than open to any comments, and we plan on enhancing

this over time.  If you have any feelings, either positive or

negative, please let us know so that this script will continue

develop to test your needs.

Enjoy,

Vibrant Graphics, USA.

Tel: 512-250-1711, Fax: 512-250-5811

Vibrant Graphics produces high-performance display drivers for

AutoCAD 10/386, AutoCAD 11/386, AutoCAD 12, AutoSHADE,

3D-Studio, CADKEY, and MicroStation.