--------------------------------------------------------------------

X10DC X-10 Direct Control

--------------------------------------------------------------------

Version 1.60

October 1994

Copyright 1993,1994 - D.Huras - All Rights Reserved

-----------------------------------------------X10DC.DOC v1.60------

CONTENTS

--------------------------------------------------------------------

1. DESCRIPTION                12. MENU BAR

2. WARRANTY                   13. UNDOCUMENTED CP290 COMMANDS

3. FEATURE OVERVIEW           14. CP290 SELF TEST

4. REQUIREMENTS               15. SIGNAL TEST GENERATOR

5. INSTALLATION INSTRUCTIONS  16. CP290 TRANSMISSION LOGGING

6. COMMAND LINE ARGUMENTS     17. CLOCK SYNCHRONIZATION

7. COMMUNICATION PORTS        18. BASE HOUSECODE

8. OPERATING MODES            19. PROBLEM RESOLUTION

9. DEBUG MODE                 20. REGISTRATION INFORMATION

10. PROGRAM OVERVIEW           21. UPGRADE INFORMATION

11. KEYBOARD USAGE             22. RELEASE HISTORY

1. DESCRIPTION

--------------------------------------------------------------------

X10DC is a program which sends immediate commands to the X10 CP290

(*1) Computer Interface.  The CP290 transmits these commands over the

power line to control X10 devices.  X10DC does not currently modify

the EVENT DATA stored in the CP290 Interface (*2).  X10DC is a DOS

program that uses a simple point & click user interface.  (A Mouse

though not required is highly recommended).  X10DC is designed to be

robust software that is processor speed independent.  This program is

shareware and you are invited to evaluate it for 30 days. If you

continue to use the program after the 30 day evaluation period you are

asked to register it.  The author retains all rights to the program.

(*1) The CP290 Computer Interface is a device manufactured by X-10

that can be connected to a micro-computer serial port.  The CP290

also plugs into an electrical outlet and it is through this

connection that the CP290 imposes X-10 commands onto the powerline.

X-10 commands use the power line as a carrier; other X-10 devices

connected to the power line will respond to these commands (ie.

switch ON/OFF lights, appliances).

2. WARRANTY

--------------------------------------------------------------------

X10DC is supplied as is.  The author disclaims all warranties,

expressed or implied, including, without limitation, the warranties

of merchantability and of fitness for any purpose.  The author

assumes no liability for damages, direct or consequential, which may

result from the use of X10DC.

3. FEATURE OVERVIEW

--------------------------------------------------------------------

- User-Friendly point & click DOS character-based Interface

- All program functions can be controlled using a mouse; no keyboard

intervention is required.

- Supports Standard CommPorts 1 thru 4 and a User Defined CommPort

which can support any IRQ between 1 & 15 (inclusive) and any

Base I/O Address.

- SUPPORTS * 5 * UNDOCUMENTED CP290 Commands

- The Program supports 3 operating modes; each mode uses a different

combination of on-screen controls.  Mode 1 is suggested for users

not using a mouse.

- When running in Mode 3, the program reads in an X10.DAT file

[this file format is used by the X10 Program supplied with

the CP290 Interface] and presents a list box of device

descriptions/locations which can be targetted for commands.

(The Unregistered version will only send commands to the 1st

8 Devices found; SEE REGISTRATION BENEFITS)

- Commands can be sent to all 256 X10 device addresses.

- Commands can be targetted for any number of units within an X10

House Code

- Supports 16 discrete dimming levels.

- Comprehensive CP290 Diagnostic Features

- A software-based Signal Test Generator is available to assist in

identifying and isolating X10 transmission/reception problems.

- A CP290 Self Test with event preservation is supported.

(*2) NOTE, the self test overwrites the EVENT/ICON DATA.  X10DC

will preserve the EVENT/ICON DATA by downloading it from the

CP290, saving it in memory, running the Self Test, and

finally uploading the saved EVENT/ICON DATA back to the CP290.

- CP290 Transmission Logging; captures the CP290 'command echoes'

that accompany each X-10 command the CP290 transmits.  Includes

Transmission Log Print Facility.

- Acknowledgements sent from the CP290 (after receiving commands

from the program) are monitored and checksums are verifed.

- A Debug Mode is available which reports all data sent from the

CP290 as well as timing information.

- A scrollable Command History is available for browsing while the

program is running.

- Clock Synchronization function for the CP290 and the PC

- Change Base HouseCode stored in the CP290 function with

or without Event & Icon Data Preservation.

- Detects and exploits EGA/VGA 43/50 line mode

- A comprehensive on-line Help sub-system that can be disabled

at the user's option to decrease program load time.

- The program WILL load without a CP290 Interface attached, however

all attempts at serial communications will result in errors; the

program will attempt to trap all such errors.

4. REQUIREMENTS

--------------------------------------------------------------------

- An IBM-Compatible PC with at least 1 serial port.

- An X10 CP290 Computer Interface attached to a serial port.

- A Microsoft-Compatible Mouse is highly recommended

- A Colour adapter/display is recommended although the program

should run with a monochrome display/adapter.  If you

experience problems with the default colors, then try the /BW

command line argument which will set everything to Black &

White.

The program requires approximately 450K available conventional memory

to run.  The Command History will consume up to an additional 32K

of memory however this translates to approximately 1000 commands; its

unlikely that this many commands will be sent during one invokation of

the program.  However, if Debug Mode is on, the Command History will

quickly fill up since all CP290 responses are reported in the

Command History list box.

6. INSTALLATION INSTRUCTIONS

--------------------------------------------------------------------

There are no special installation instructions; simply copy the

program to a sub-directory of your choice and run it.  The program

will look for 3 files in the directory it is run from.  However,

the program will run with or without the following 3 files:

1) X10.DAT

If you want to use Mode 3 then you may want to copy an

X10.DAT file to the sub-directory where you have installed

X10DC.EXE.  Alternatively, you can load an X10.DAT file from

any sub-directory by using the 'Load X10 File' menu option.

An X10.DAT file is a file format used by the X10 program

supplied by X10 with the CP290 interface.  The unit

addresses, locations and descriptions are read from this

file and presented in a list box by X10DC.  X10DC DOES NOT

MODIFY THE X10.DAT FILE.  A sample X10.DAT is provided

with X10DC.

2) X10DC.HLP

X10DC retrieves the on-line help text from this file. If the

file is not found then X10DC will display a message during

startup and then continue.  The help feature will be

unavailable in this case.  If you decide that you no longer

need the help sub-system, then you should run the program

with the /NH command line argument; this will suppress the

'Help not found' message and decrease program load time.

3) X10DC.INI

The program will look for a X10DC.INI file in the current

directory.  The X10DC.INI file is a text file containing all

program parameters; there is no need to create this file

yourself since the program will generate one whenever you

select the 'Save Options' menu item.  A sample X10DC.INI file

(generated by X10DC.EXE) is shown below:

;

;X10DC.INI File Created: 10-04-1994 06:30:22

;All Lines prefixed with ; are ignored

; ---------------------------------------------------------

;

; CommPort=1,2,3 OR 4  DEFAULT=1

;

[Communications]

CommPort=2

;

; Startup Options

; ---------------

; Mode=1,2 OR 3                  DEFAULT=1

; Debug=ON/OFF                   DEFAULT=OFF

; X10File=[d:]\FILENAME.EXT      DEFAULT=X10.DAT

; LoadHelp=YES/NO                DEFAULT=YES

; Sounds=ON/OFF                  DEFAULT=ON

; PreserveICONData=YES/NO        DEFAULT=NO

; RefreshClocks=0 TO 9           DEFAULT=9

;

[Startup]

Mode=3

Debug=OFF

X10File=F:\X10\XA\X10.DAT

LoadHelp=YES

Sounds=ON

PreserveICONData=YES

RefreshClocks=0

;

[Display]

ThreeD=YES

Shadow=YES

Graphics=YES

;

; Colour Values

; -------------

; 0=Black  4=Red       8=Gray          12=Bright Red

; 1=Blue   5=Magenta   9=Bright Blue   13=Pink

; 2=Green  6=Brown    10=Bright Green  14=Yellow

; 3=Cyan   7=White    11=Bright Cyan   15=Bright White

;

BackColor=7

ForeColor=0

HiLiteKey=12

DisabledItem=8

MenuBackColor=1

MenuForeColor=7

MenuSelectedBackColor=0

MenuSelectedForeColor=7

LabelBackColor=15

LabelForeColor=1

Hilite1ForeColor=4

Hilite2ForeColor=1

Hilite3ForeColor=14

HelpTitleBackColor=5

HelpTitleForeColor=15

StatusTitleBackColor=4

StatusTitleForeColor=15

WarningTitleBackColor=12

WarningTitleForeColor=15

;

; End of X10DC.INI (06:30:22)

This X10DC.FILE was created using the 'Save Options' menu item.

NOTE:the DEBUGFORECOLOR directive was replaced with HILITE1FORECOLOR

effective with Version 1.50.  However, X10DC will still

recognize DEBUGFORECOLOR and map it to HILITE1FORECOLOR.

When the INI file is rewritten, the old directive will be

replaced.

6. COMMAND LINE ARGUMENTS

--------------------------------------------------------------------

DEFAULTS:  If the program is run without any command line arguments

it will default to COMMPORT 1,OPERATING MODE 1,DEBUG OFF,

F:X10.DAT.

The following command line arguments can be entered in any order:

/Annnn(n) - Base I/O Address applicable to User Defined CommPort

(Must be specified as a decimal number)

If /I agrument is present, but /A is not; then Base

I/O Address will default to 1016 (3F8 Hex).

/BW  - Use Monochrome colours (Black & White)

/Cn  - COMMPORT where n = 1,2,3 & 4

ALWAYS overrides User Defined CommPort during Program Startup

/D   - TURNS ON DEBUG MODE

When DEBUG MODE is ON, the CP290 responses & checksums are

displayed in the Command History List Box.

/F:[d]\filename - X10.DAT FILE SPECIFICATION for MODE 3

where [d] = drivespec eg. /F:C:\X10\X10.DAT

/In  - IRQ applicable to User Defined CommPort where n = 1 - 15

If /A argument is present, but /I is not; then IRQ

will default to 4.

/Mn  - MODE where n = 1,2 or 3

MODE 1 Provides a List Box of Unit Codes; only 1 Unit Code

can be selected.  However, this mode will be easier to

use if you don't have a mouse since you'd have to TAB

through all the unit code check boxes used in MODE 2.

MODE 2 Provides a Check Box for each Unit Code plus ALL and

NONE buttons to quickly Check/Uncheck all check boxes.

In this mode, multiple Unit Codes can be targetted for

the X10 Command.

MODE 3 Provides a List Box of Devices which are read from

an X10.DAT file; X10DC will look for "X10.DAT" in the

current directory UNLESS the /F: command line argument

is supplied.  If X10.DAT cannot be found, the program

will switch to Mode 1.  Select 'Load X10 File' under

the File menu to load an X10.DAT file from another

sub-directory. (Once the file is loaded, its location

can be saved [REGISTERED VERSION ONLY] by selecting

'Save Options' under the Options menu.

/NH  - Do Not Load HELP Sub-System

/NH- - Load HELP Sub-System (Override INI directive)

/NS  - No Sounds during Signal Test

/NS- - Sounds during Signal Test (Override INI directive)

/P   - Toggles ON 'Preserve ICON Data'

Normally, X10DC will only preserve the Event Memory stored

in the CP290.  This is because the DOS-based CP290 software

only utilizes the CP290 Event Memory and does not utilize the

CP290 Icon memory area.  However, X10EC does store information

in the Icon memory, and X10DC has been upgraded to preserve

the Icon memory area for compatibility reasons.

NOTE: ONLY THE 1st 8 DEVICES CAN BE TARGETTED FOR

COMMANDS WITH THE UNREGISTERED VERSION.

1) Most Command Line directives (with the exception

exception of /A,/I & /NH) can be modified once the

program loads by selecting the appropriate menu items

under the Options Menu.  Registered Users can save all

these settings to the X10DC.INI file by using the 'Save

Options' menu item.

2) COMMAND LINE OPTIONS OVERRIDE ALL X10DC.INI SETTINGS

7. COMMUNICATION PORTS

--------------------------------------------------------------------

X10DC supports "pseudo standard" CommPorts 1 thru 4 using the

following settings:

+----------+------+------------------+

|          |      | Base I/O Address |

| CommPort | IRQ  +---------+--------+

|          |      | Decimal |  HEX.  |

+----------+------+---------+--------+

|    1     |   4  |  1016   |  3F8   |

+----------+------+---------+--------+

|    2     |   3  |   760   |  2F8   |

+----------+------+---------+--------+

|    3     |   4  |  1000   |  3E8   |

+----------+------+---------+--------+

|    4     |   3  |   744   |  2E8   |

+----------+------+---------+--------+

If one of these CommPort settings matches your hardware

configuration simply select the appropriate CommPort from the

Communications Options screen.  (You could also start X10DC with the

/C command line argument or use the COMMPORT= X10DC.INI directive).

You can define your own CommPort by selecting User Defined CommPort

from the Communications Options screen and setting the IRQ interrupt

and Base I/O Address.  (You could also use the /I and /A Command

Line Args or IRQ= & Base_IO_Address= X10DC.INI file directives).

IRQ can be set to any value between 1 and 15 (inclusive).  Base I/O

Address can be set to any value greater than zero.  CAUTION: your

hardware may "lock up" if incorrect settings are used.  Once you

have enabled communications with the CP290, then use the 'Save

Options' menu item to save the information to the X10DC.INI file.

THE BASE I/O ADDRESS MUST BE SPECIFIED IN DECIMAL NOTATION.

IBM PS/2 CommPort Parameters are shown below:

+----------+------+------------------+

|   PS/2   |      | Base I/O Address |

| CommPort | IRQ  +---------+--------+

|          |      | Decimal |  HEX.  |

+----------+------+---------+--------+

|    1     |   4  |  1016   |  3F8   |

|    2     |   3  |   760   |  2F8   |

|    3     |   3  | 12832   | 3220   |

|    4     |   3  | 12840   | 3228   |

|    5     |   3  | 16928   | 4220   |

|    6     |   3  | 16936   | 4228   |

|    7     |   3  | 21024   | 5220   |

|    8     |   3  | 21032   | 5228   |

+----------+------+---------+--------+

A User Defined CommPort must be used for

PS/2 CommPorts 3-8.

8. OPERATING MODES

--------------------------------------------------------------------

Operating Mode can be set using the 'Options' menu, the MODE command

button (cycles through the modes), or the /M command line argument.

--------------------------------------------------------------------

MODE 1                    Unit

Code            NOTE: Mode can be toggled

In this mode only        +-----+                at runtime by clicking

one Unit Code can be     |   1 #                the MODE button.

selected from the        |   2 �                (Keyboard users press

Unit Code List Box.      |   3 �                 ALT-m).

|   4 �

|   5 �                Can also be set from

|   6 #                the Options Menu.

+-----+

RECOMMENDED IF YOU DON'T HAVE A MOUSE.

--------------------------------------------------------------------

MODE 2                             Unit Code(s)

+----------------------------+

In this mode multiple    |[X] 1  [ ] 5  [ ] 9   [ ] 13|

Unit Codes can be        |[ ] 2  [ ] 6  [ ] 10  [ ] 14|

checked; in addition     |[ ] 3  [ ] 7  [ ] 11  [ ] 15|

the ALL button can be    |[ ] 4  [ ] 8  [ ] 12  [ ] 16|

clicked to check all     |+-----------+  +-----------+|

the Unit Codes. The      ||    ALL    |  |    NONE   ||

NONE button unchecks     |+-----------+  +-----------+|

all the unit codes.      +----------------------------+

--------------------------------------------------------------------

MODE 3

Unit Location               Description

An X10.DAT      +----------------------------------------------------+

is read and     | A  1|DINING ROOM           |TABLE LAMP             #

a List Box      | A  2|FAMILY ROOM           |TV                     �

replaces the    | A  3|GARAGE                |CEILING LIGHT          �

House/Unit      | A  4|GUEST ROOM            |DE-HUMIDIFIER          �

Controls used   | G  7|HALL                  |CEILING LIGHT          #

in Modes 1 & 2. +----------------------------------------------------+

Also note that   {11 Device(s) Found} Brightness <۰������> 1�00.0%

the Brightness

List Box has been replaced by a Horizontal Slider Control. (above)

--------------------------------------------------------------------

9. DEBUG MODE

+--------------------------------------------------------------------+

| Using COMMPORT: 2   X10 - X-10 Direct Control   CP290 Day: Tuesday |

|Base House Code: P         Version 1.50         CP290 Time: 11:51 AM|

|Mode: 1   Debug ON      (c)1993 - D.Huras          PC Time: 11:52 AM|

+--------------|-----------------------------------------------------+

+----|

--+-----

When enabled, the DEBUG ON message appears in the screen Header and all

responses from the CP290 are displayed in the Command History List Box,

as well as Checksums and Timing Information.

Debug Mode can be toggled ON/OFF from the Options Menu.

NOTE: Debug Mode increases program overhead; memory requirements will

be greater and program performance will be somewhat degraded.  It is

recommended that you only toggle Debug Mode ON when necessary.

10. PROGRAM OVERVIEW

--------------------------------------------------------------------

When X10DC initially loads, it requests the base house code and time

stored in the CP290 via serial I/O using the selected COMMPORT.

This information will be reported in the Screen Header if found.

Note:  the CP290 Time and PC Time reported in the Screen are static

and are only updated when the Base House Code is changed, a

Self-Test is run or the Time is set.  You can use the 'Refresh' menu

item under the 'Clock' menu to force an update to the on-screen Time

displays.  The House Code List Box defaults to the Base House Code

if the CP290 is found.  Once the program is loaded the SEND command

button is disabled until you have selected all required

information.  In some cases you need only to select a House Code and

an Action (eg. All Units Off).  If the selected action is DIM, then

a Brightness Level must be be selected before the SEND button

becomes enabled.

The program supports 3 operating modes; use whatever mode suits

your requirements.  Mode 3, which reads in Device Descriptions

from an X10.DAT file, is likely the most user-friendly since you

do not have to remember specific House/Unit code addresses.  X10EC

a companion program to X10DC can read, write, and create X10.DAT

files.

The Command History list box is scrollable and will store up to 1000

commands after which it becomes a circular FIFO buffer.  If you

suspect problems with your CP290, then you may want to toggle

Debug Mode ON; in this case the Command History list box will

contain additional information including responses from the CP290.

You may also want to perform a CP290 Self-Test to ensure that

the CP290 Memory is not being corrupted.

Finally, if you are experiencing X-10 transmission problems in your

application; you may want to take advantage of the Signal Test

Generator or Transmission Logging features provided by X10DC.  These

subjects are covered in detail in a later section of this document.

11. KEYBOARD USAGE

--------------------------------------------------------------------

Use the TAB/Shift-TAB keys to move among the various controls.(*3)

The Up/Down arrows can be used to move up and down List Boxes, and

the Spacebar and Enter key can be used to select the current List

Box item, check/uncheck a check box, or invoke a command button.

The ESCAPE key will exit the current function or the program if no

other windows are visible.  All of the Command Buttons and controls

have quick access keys:

House Code - Alt-o [Mode 1 & 2]

Unit Code(s) - Alt-u [Mode 1 & 2]

Device List - Alt-u [Mode 3]

ALL - Alt-l [Mode 2]

NONE - Alt-n [Mode 2]

Action - Alt-a

Brightness - Alt-b [when enabled]

MODE - Alt-m (*3)

SEND - Alt-s

EXIT - Alt-x

HELP - Alt-h

Command History - Alt-c

(*3) The MODE Command Button is NOT in the TAB STOP sequence;

Keyboard users must use ALT+character to access this

button. (This eliminates extraneous TAB keystrokes)

12. MENU BAR

--------------------------------------------------------------------

Press & release the ALT key to activate the Menu Bar.  Menu items

can be selected by pressing the highlighted key (when applicable),

or by positioning the highlight bar using the up/down arrow keys and

pressing ENTER.  Some screen button functions are replicated within

the menus (eg. Exit, Help).  However, there are some functions which

are only available from within menus.  CommPort, Operating Mode,

Debug, Sounds & Preserve Icon Data can all be set from the Options

Menu; the current state of each of these parameters is indicated on

the menu itself with a '-' character to the left of the menu item.

The following short-cut keys can also be used at any time to

select menu items:

*FILE* Menu

Load X10 File - Ctrl-L

Exit      - Ctrl-X (Same as Exit Button, Alt-X)

*CLOCK* Menu

Set PC Clock - Ctrl-T (Enter Time)

Set CP290 Clock - Ctrl-K (Synchronize w/PC) - Registered Version

Refresh Clocks - Ctrl-R

Set Base HouseCode - Ctrl-B (Preserve Events) - Registered Version

*TEST* Menu

CP290 Self Test - Ctrl-E

Signal Test Generator - Ctrl-N

Log CP290 Transmissions - Ctrl-S

*OPTIONS* Menu

Mode 1 - Shift-F1

Mode 2 - Shift-F2

Mode 3 - Shift-F3

Sounds Toggle - Ctrl-U

Debug Mode Toggle - Ctrl-D

Preserve ICON Data - Ctrl-I

Communications - Ctrl-C

Screen Colours - Ctrl-G

Save Options - Ctrl-O

*HELP* Menu

Help - F1 (Same as Help Button, Alt-H)

About - Ctrl-A

13. UNDOCUMENTED CP290 COMMANDS

--------------------------------------------------------------------

X10DC supports the following undocumented CP290 commands:

Fade OFF   - DIMS Light(s) to 0 then turns Unit(s) OFF

(Can target multiple units for this command)

X-Light ON - Exclusive Light ON - Issues an ALL LIGHTS OFF

then Brightens selected Unit(s)

(Can target multiple units for this command)

All Lights ON  - All Lights on selected House Code set ON

All Lights OFF - All Lights on selected House Code set OFF

All Units OFF  - All Units on selected House Code set OFF

NOTE:  X-10 Light Switches usually respond to the ALL LIGHTS ON

command; however some Lamp Modules appear to ignore it;

increasing the DIM level when using this command occasionally

resolves this apparent hardware problem.

14. CP290 SELF TEST

--------------------------------------------------------------------

The CP290 can perform a Self-Test to determine if there are any

detectable problems within the device.  The Self-Test will return

either a pass or fail status when it is run.  X10DC will report the

results of the test in the CP290 Status window.  Whenever a

Self-Test is run the memory contents of the CP290 are overwritten.

X10DC can preserve the CP290 Event and Icon Data by downloading it

from the CP290 prior to the Self-Test and uploading it back to the

CP290 after the Test.  (X10DC also allows a Self-Test to be invoked

without event preservation, but this leaves the device in a state

which may cause problems with other programs that communicate with

the CP290. It is recommended that the Self-Test always be invoked

with Event Preservation regardless of whether or not there is any

stored Event Data.)

15. SIGNAL TEST GENERATOR

--------------------------------------------------------------------

The Signal Test Generator is a powerful feature that enables the

user to track down and isolate X-10 transmission/reception problems

and faulty modules.  This feature will send X-10 ON/OFF commands

repeatedly until the test is ended by the user.  (Similar to the

Leviton 6385 device).  The address to be targetted for commands can

be a fixed House Code and Unit Code; alternatively, either the House

Code or Unit Code (or both) can be varied. This allows you to send

commands repeatedly to:

- A 'Fixed' House Code & 'Fixed' Unit Code (eg. A1)

- A 'Fixed' House Code & 'Variable' Unit Code (eg. A1 - A16)

- A 'Variable' House Code & 'Variable' Unit Code (ie. A1 - P16;

All 256 addresses would be tested in this case)

- A 'Variable' House Code & 'Fixed' Unit Code (eg. A1 - P1)

When the Signal Test Generator is selected, a window is

displayed that allows you to set the House Code & Unit Code to

either 'Fixed' or 'Variable' mode; if 'Fixed' is selected then

a List Box is enabled so that the fixed code can be selected.

The Delay between commands can be set from 0 to 60 seconds;

this delay applies to the pause between each pair of ON/OFF

commands.  Once all required settings have been made the OK

button becomes enabled.  When the OK button is clicked the

test will begin; the test can be terminated by pressing [Esc].

Note, if the Delay is set to 0, then the [Esc] key may have to

be hit several times to stop the Signal Test.  X10DC will emit

a brief beep before each ON command is sent, and another lower

pitched beep before each OFF command is sent.  These beeps can

be disabled by toggling the Sounds item on the 'Options'

Menu.

'Fixed' Mode Testing

--------------------

Generally speaking, you would want to use the 'Fixed' option

to test the range of X-10 signal reception by starting the

Signal Test using address P1 for example (the "only" address

that the Leviton 6385 will test) and then walking around to a

variety of outlet locations within your home and plugging in a

module set to P1 to see if the ON & OFF commands are being

received at all locations.  The program attempts to let you

know when the commands are sent by beeping although you may

not be able to hear the beep from all outlet locations.

'Variable' Mode Testing

-----------------------

The 'Variable'(*4) option can be used when you suspect that an

X-10 module has trouble receiving commands targetted for

specific House and/or Unit Codes.  In this case, instead of

walking around to locations, you would adjust the House and/or

Unit Code dials on the module as the Signal Test cycles

through each combination.  You may want to toggle the Sounds

off in this case.

----

(*4) Not available in UNREGISTERED Version

16. CP290 TRANSMISSION LOGGING

--------------------------------------------------------------------

This diagnostic feature captures and displays the CP290 'command

echoes' in a scrollable list box that can be printed at any time.

Whenever the CP290 imposes an X-10 command onto the powerline it

also transmits a 'command echo' via the serial interface.  The

'command echo' is transmitted for both scheduled events and for

X-10 commands initiated using the rocker keys on top of the CP290.

This feature allows you to confirm that the CP290 is actually

transmitting X-10 commands as per the event schedule stored in

its memory.  The command echoes are captured by X10DC and a checksum

is verified before an entry is added to the Transmission Log. Any

errors detected during the serial transmission of the command echo

are also added to the Transmission Log.  The Transmission Log can

be printed at any time by using the 'Print Log' command button.

During the printing process transmission logging is temporarily

suspended.  There are some limitations associated with the command

echoes transmitted by the CP290, specifically:

- the DIM level associated with DIM commands is not transmitted.

- there is no distinction made between OFF (documented) and

FADE-OFF (undocumented) command transmissions.

The capacity of the CP290 Transmission Log is 1000 entries, once

this maximum has been reached the oldest log entry will be deleted

each time a new log entry is added.

17. CLOCK SYNCHRONIZATION

--------------------------------------------------------------------

Clocks tend to run either fast or slow and therefore should be

synchronized periodically.  X10DC allows you to set the time in both

the PC and CP290 manually by specifying hh:mm and also allows you to

synchronize the PC and CP290 by using one of the clocks to set the

other.  When the PC clock is used to synchronize the CP290 clock,

X10DC will wait until the seconds are at zero since the resolution

of the CP290 clock is limited to minutes.  (This wait can be

cancelled by pressing the [Esc] key).  When the CP290 is used to

synchronize the PC, the hours and minutes from the CP290 are used

immediately to set the PC Clock; since the CP290 cannot provide

'seconds', it is recommended that a CP290->PC Clock Sync be followed

by a PC->CP290 Clock Sync.  This will ensure that both Clocks are

synchronized.

18. BASE HOUSECODE

--------------------------------------------------------------------

The Base HouseCode stored in the CP290 is only applicable to the

rocker keys on the top of the CP290.  These keys are labelled 1 to 8

and correspond to whatever House Code is stored as the Base

HouseCode in the CP290.  The CP290 re-initializes itself whenever

the Base HouseCode is changed; as a result, all of the CP290 Event

Data is erased.  X10DC gives you the option of either Preserving or

Erasing the Event and Icon Data whenever the Base HouseCode is

changed.  If the user elects to preserve the Event Data, it is

downloaded from the CP290 before the Base HouseCode is changed and

then uploaded back to the CP290 after the Base HouseCode has been

changed.

19. PROBLEM RESOLUTION

--------------------------------------------------------------------

Generally, if your CP290 works with the X-10 supplied software then

you should have no problems running X10DC.  X10DC has been

successfully tested under Microsoft Windows v3.1 and OS/2 v2.1.  The

program will load if you run it without any arguments (thereby

defaulting to COMMPORT 1) and your CP290 is actually attached to

COMMPORT 2.  If this is the case X10DC will report 'Timed Out

waiting for a response' when it attempts to retrieve the CP290 Time,

Day of Week and Base HouseCode.  You can toggle to the correct

COMMPORT once the program is running by using the Options,

Communications menu.  The Save Options menu option will save all

settings to the X10DC.INI which obviates the need for command line

arguments.

If you run X10DC with a modem attached to the COMMPORT, you may get

unpredictable results since the modem will echo everything back to

X10DC.

If you run X10DC with a modem attached to the COMMPORT, you may get

unpredictable results since the modem will echo everything back to

X10DC.  If you're using a COMMPORT that shares an interrupt with

another COMMPORT (eg. 1 & 3, 2 & 4), then you must ensure that no

memory resident software (eg. a FAX TSR) that uses the other COMMPORT

is loaded as it will not release the interrupt.

If you use a Serial Mouse, this may cause a conflict if X10DC

attempts to communicate with a CP290 on the same CommPort.  You may

want to start X10DC with the /C command line argument to ensure it

does not default to CommPort 1 if your mouse is also on CommPort 1.

X10DC will not run properly if it attempts to communicate with a

COMMPORT that has been initialized with a Mouse Driver (ie. you

cannot use a switchbox between the CP290 and a Mouse unless you

reboot without loading the Mouse Driver).

The X10DC executable file is encrypted and there is a remote

possibility that the decryption process may fail with certain

processors; a blank screen immediately after attempting to run the

program typifies this problem.  Please contact the author if you

encounter this problem.

In some cases X10DC may report communication errors (ie.  time-outs,

unmatched checksums, etc).  This may occur when scheduled Events

collide with CP290 Uploads/Downloads that are in progress.  (The

CP290 transmits onto the serial port line whenever a scheduled event

occurs and when you press the rocker buttons on the CP290).  X10DC

attempts to minimize collisions by discarding unexpected serial

input, however the possibility still exists.  You should just

re-invoke the interrupted action to recover from this type of

problem.

If you are sure that the X10DC is using the correct COMMPORT and

problems persist, try using Debug mode and then scroll through the

Command History List Box; see if the calculated checksums match;

look at the Elapsed and Wait Time Values. If the Elapsed Time

exceeds the Wait Time value then the program is timing out while

waiting for X-10 responses. PLEASE REPORT ANY TIMING PROBLEMS TO ME;

adjustments can be made to the program to improve its compatibility

if this turns out to be a problem.  As a last resort, run the CP290

Self Test, but only do so if you have backed up your EVENT DATA

using a program you are sure works within your operating

environment.  (X10DC will preserve the CP290 Event Data but if you

are experiencing problems with X10DC then this function may fail).

The open architecture of the PC compatible operating environment and

the near infinite combinations of hardware and software complicate

the compatibility of applications software.  While every effort has

been made to ensure that X10DC is compatible with all

hardware/software combinations, there may be operating environments

in which X10DC will not run properly.

Problems can be reported to:

David Huras

37 Ruddell Crescent

Georgetown, Ont. L7G 5N4

CANADA

FAX: 905-873-6429

CIS: 72767,472

Internet: david.huras@canrem.com

20. REGISTRATION INFORMATION

--------------------------------------------------------------------

The registration fee for X10DC is $20. (US Dollars for US shipping

addresses, CDN Dollars for Canadian shipping addresses).  X10DC is

currently supplied as functioning software without built-in delays

or obtrusive "nag" screens prompting you to register.  The Signal

Test Generator & Mode 3 are the only features not fully supported in

the shareware version of X10DC.  A SIGNIFICANT amount of time and

effort went into this program; please register the program if you

intend to use it beyond the 30-day evaluation period.  Registered

Users will be shipped a printed manual and a 3.5" diskette (if you

would prefer a 5.25" diskette please advise) containing the

registered version of the program.  Registration Benefits:

- Full support for the Signal Test Generator which allows you to

cycle through all House & Unit Codes and send an ON/OFF command

to each address.

- Full support for operating mode 3 which reads in an X10.DAT file

and builds a "list box" of device names which can be targetted

for X10 commands.  (Unregistered Version will only send commands

to the first 8 Devices).

- BONUS PROGRAMS: Registered Users will also receive 2 bonus

programs:

1)  X10DAT reads an X10.DAT file and prepares a report

listing all installed devices and scheduled events including

"frozen" events and multiple unit events. See the X10DAT.RPT

file for a report sample.

2)  XDC issues immediate X-10 command directives to the CP290

directly from the DOS prompt.  Alternatively, you can pass

a filename to XDC containing 'batched' X-10 command directives.

XDC supports the 5 undocumented CP290 commands that are

supported by X10EC & X10DC. See the XDC.DOC file for additonal

information.

- DISCOUNTED REGISTRATION FEE for X10EC; X-10 Event Control.

Registered Users can register X10EC for $20; the regular

registration fee is $25.

To expedite your registration, you may wish to register using

CompuServe Shareware Registration; you will receive a Registration

Key via CompuServe E-Mail within 1 business day that will enable the

registered program features. (See CompuServe Shareware Registration

below).  Alternatively, you can charge the Registration Fee to your

VISA, Master Card, or American Express.  (See PsL Shareware

Registration Program below).  If you elect to use either of methods

your software will ship within 1 business day.

If you do not wish to register via CompuServe Shareware Registration

or the PsL Shareware Registration Program then complete the order

form in the REGISTER.FRM file and send with your payment to:

David Huras

37 Ruddell Crescent

Georgetown, Ont. L7G 5N4

CANADA

CompuServe Shareware Registration

=================================

You can register X10DC, X10EC or BOTH using CompuServe Shareware

Registration.  The Registration Fee will be charged to your

CompuServe billing account and the latest version of the software

along with a printed User Guide will be shipped within one business

day.  In addition, you will receive a Registration Key via

CompuServe E-Mail (within one business day) that will immediately

allow you to run X10DC with the registered version features enabled

while you wait for your diskette to arrive.  GO SWREG while on

CompuServe to use Shareware Registration.

Program Title         Program Name    Program ID#   Charge

===================   ============    ===========   =======

X-10 Direct Control      X10DC             620       $20.00

X-10 Event Control       X10EC            1774       $25.00

X-10 Control Bundle      X10CB            1775       $40.00

(X10DC + X10EC)

PsL Shareware Registration Program

==================================

Charge the Registration Fee to your Visa, Master Card, or American

Express by ordering X10DC or X10EC (specify Program # below) from

Public (software) Library, call 800-242-4PsL (713-524-6394 overseas)

or FAX your order to 713-524-6398.  PsL will forward your order to me

via Compuserve E-Mail and your order will be shipped within one

business day.  Note, these phone numbers are for ordering only.  ALL

OTHER INQUIRIES (eg. technical support, volume discounts site

licensing) should be directed to the author: see program

documentation.

Program Title         Name  Program #  Charge   Handling  Total

===================  ====== =========  ======   ======== ======

X-10 Direct Control  X10DC   10924     $20.00    $0.00   $20.00

X-10 Event Control   X10EC   11595     $25.00    $1.00   $26.00

21. UPGRADE INFORMATION

--------------------------------------------------------------------

Quick Upgrade

=============

REGISTERED USERS of prior versions of X10EC can immediately run the

shareware version of X10EC v1.10 and all future shareware versions

of X10EC with the registered features enabled.  X10EC will look in

the current sub-directory for the X10EC.KEY file; if a valid file is

found then X10EC will enable all registered program features.  This

means you can upgrade by downloading the latest shareware version of

X10EC and ensuring that your X10EC.KEY file resides in the same

sub-directory.

Existing Registered Users

=========================

If you registered a version of X10DC prior to 1.10; then you are

entitled to an X10DC.KEY file that will enable you to immediately

use the shareware version of X10DC v1.60 with the registered

features enabled.  Your X10DC.KEY file can be forwarded via

CompuServe E-Mail, Internet E-Mail, or you can download it directly

from the author.  Please contact the author via E-Mail or FAX to

arrange for the delivery of your X10DC.KEY file.

Diskette Upgrade

================

REGISTERED USERS of prior versions of X10DC can request a diskette

containing the current version of the software at ANY TIME by simply

sending a diskette containing their registered version of the

software and $2 for postage to the author.

22. RELEASE HISTORY - X10DC Shareware/Registered Version

--------------------------------------------------------------------

Date      Version     Changes

------   ----------   ----------------------------------------------

OCT'94    1.60        -Added CP290 Transmission Logging Feature

(with Print capability)

-QUICK Registration/Upgrade Feature Added

-Save Options now enabled in Shareware Version

-INI File processing now enabled in Shareware

Version

-INI parsing enhanced; invalid directives are

now ignored with a message regardless of

DEBUG status

-Command Argument parsing enhanced; invalid

arguments are now ignored with a message

-X10DC will now prompt you to Save Options if

program exit is requested and options have

been modified but not saved

-Communications Setup enhanced; a dedicated

screen is now used that allows dynamic setup

of User Defined CommPort

-All SPIN Controls changed to Vertical Scroll

Bars for performance (and compatibility with

X10EC)

-Common Dialog Forms loaded at Program Startup

-/F: Command Line Argument forces Mode 3 on Startup

DEC'93    1.50        -Added support for 5 UNDOCUMENTED CP290 Cmds

-Added support for ICON Data Preservation

-Optimized screen layout for all 3 Modes

-Added support for EGA/VGA 43/50 Line Mode

-Added graphic 'percentage complete' gauges

-Countdown Timer added to Status Window while

waiting for CP290 responses.

NOV'93    1.42        -Corrected problem reporting Base House Code

when CP290 returns bad data.

AUG'93    1.41        -Public (software) Library registration method

added to documentation.

-X10DAT Bonus Program added as registration

benefit.

-X10DC.INI File Comment re: X10File argument

corrected in documentation.

AUG'93    1.40        -Corrected 'Load X10 File' problem when X10.DAT

located in Root Directory.

-Corrected problem whereby X10DC.INI was not

processed when X10DC was started from a Root

Directory (impacted Registered Version only).

AUG'93    1.40 �      -COMMPORT 3 & 4 Support Added.

-User Defined CommPort support Added.

-Minor changes to default colours (LABELS).

JUL'93    1.32        -Corrected problem with DEBUG MODE colour setting

not 'sticking'; setting from INI is workaround.

(Not released).

JUL'93    1.31        -Fixed mouse cursor reset problem if Signal Test

is run without a CP290 device. (Not released).

JUL'93    1.30        -ENABLED SEND Command under MODE 3 in Unregistered

version for 1st 8 Devices in X10.DAT File

-ENABLED ALL Clock Sychronization options in

Unregistered version

-ENABLED Change Base HouseCode with Event

Preservation in Unregistered version

-ENABLED Screen Colours dialogue in Unregistered

version

-Test menu item added to Menu Bar;

- CP290 Self Test disassociated with Debug Mode

and Self Test button removed from Screen

Header; Self-Test feature now a sub-menu

item under Test.

- Signal Test Generator feature added; send

ON/OFF commands repeatedly to a fixed/variable

House Code/Unit Code address.  Unregistered

version supports the fixed option only.

-Help Sub-system added; contains additional

X10 background information as well as extracts

from the documentation file.

-/NH command line argument added to optionally

not load the help system for faster program

load times.

-/NS command line argument added to disable

sounds generated during the Signal Test.

-Sounds Toggle added as a sub-menu item under

Options.

-INI Overwrite warning now uses the WARNING

screen title colour(*5)

-'Save Options' displays a message box whenever

INI options are saved(*5)

-The access key for the 'Exit' option under the

'File' menu changed from 'E' to 'x'.

-[Esc] key will now back out any screen

colour changes while in the Screen Colours

dialogue

-[Esc] key will now skip the wait while

SYNC'ing the CP290 clock based on the PC

Clock

MAR'93    1.20        -Merged Shareware & Registered Versions;

Registered Version features are disabled in

the Unregistered version, specifically:

- Complete Mode 3 Support (X10.DAT File)

- Complete Screen Color Customization

Dialogue

- INI File Support to save/modify

Program settings/colors

- Preserve/Restore Event Data when

Base House Code changed

- PC � CP290 Clock Synchronization

- CP290 � PC Clock Synchronization

-ENABLED Self Test EVENT Preservation

in Unregistered version

-ENABLED MODE 3 Switch & X10.DAT File Load

in Unregistered version (SEND-Disabled)

-ENABLED some Clock Functions, & Base House

Code Setting in Unregistered Version

-CP290 Self Test changed to reset interface

upon completion (The self test overwrites

the Event Data and this data may cause

problems with some programs)

-Added /BW command line argument to startup

with monochrome color settings

-Added 'Refresh' menu item to refresh on-screen

Clock Displays

-Error handling improved

-Retry logic added for all CP290 communications

with automatic wait time adjustment (if program

times out waiting for a response, it will

increase the wait time if the command is retryed)

-Command History caption now indicates which

entry is highlighted (ie. entry x of n)

MAR'93    1.10        -MENU BAR Added - CommPort,Mode, & Debug can

now be changed using the Options Menu

-Mode Indicator added to Screen Header

-Minor Screen Colour modifications

FEB'93    1.00        -Mode Switch Command Button Added (Alt-M)

-Debug Command Button TAB Stop disabled-keyboard

users must use ALT-T

-CIS Software Registration # added to ABOUT

FEB'93    1.00 �.02   -Operating Mode 2 Added (FIRST PUBLIC RELEASE)

-Operating Mode 3 Added to Registered Version

-Clock/Base House Code manipulation added to

Registered Version

JAN'93    1.00 �.01   -Debug Mode Added

JAN'93    1.00 �.00   -Initial Beta (not publicly released)

------

(*5) Applicable to REGISTERED VERSION Only.