' (IBM-char code 254) : start of a new section

'--'  :  start  of  a  new  function  description  or  some  other

sub-section

Please read the informations about your rights  to  use  and  copy

this program in the text file LICENSE.DOC.

A  guide  for  the  quick  installation  may  be found in the file

INSTALL.DOC.

Answers to some common  and  special  questions  are listed in the

file QUESTION.DOC.

The file REFER.DOC contains an overview  about  all  configuration

verbs, options, parameters and functions in detail.

______________________________________________________________________

ZuTSR   has   been   developed   for   use  in  MS-DOS  compatible

environments.  With other operating  systems,  ZuTSR may work in a

"DOS-Box".

ZuTSR will be loaded as a resident program into main  memory,  and

occupies  about  10  kByte  there.   Except  for  that, it will do

nothing if no other program requests it to act.

The latter can happen in two different ways:

--

> Using ZuTSR as a function library for your own programs            <

ZuTSR  contains  some  elementary   functions  for  recording  and

replaying voice data to/from a modem  with  the  so-called  "voice

mode".   Some  more functions provide the ability to send commands

to the modem, await responses,  detect DTMF codes and more.  These

functions can be used from  any  other  programming  language,  by

issuing  simple interrupt calls to the ZuTSR program, which has to

be loaded into memory before.

Beside ZuTSR, you will only need  a Fossil driver (see below), and

a little knowledge about using interrupt calls.

More detailed information about this way to use ZuTSR is contained

in the file REFER.DOC.

--

> Using ZuTSR as an add-on for your mailer or BBS software           <

You may configure ZuTSR that way, that your mailer will call it to

answer incoming calls, therefore  replacing  the  usual  data-only

answering  with  a  sequence of voice messages, message recording,

automatic voice/data call differentation, and much more.  The only

requirement for this  is  a  mailer  or  mailbox  software using a

Fossil driver (see below) for communicating with  the  modem.   To

use  the voice functions, you will also need a modem equipped with

a "voice mode", which  has  to  be  quite  compliant to the IS 101

standard (ZyXEL modems are, and modems equipped  with  a  Rockwell

chipset  and  the  "voice option" are also quite compliant with IS

101)

As a resident program,  ZuTSR  will  watch every command sent from

the mailer to the modem.  As soon as the mailer sends  the  answer

command  (e.g.  "AT A") to the modem, ZuTSR takes over control and

does its actions.

Using a  very  simple  script  construction,  you  can  define the

sequence of actions to take place.

Depending on  the  configuration,  ZuTSR  may  add  the  following

functions to your software :

- differentiation between fax/data- and human calls.

- replaying of various voice messages, automatically or on request

(e.g. product informations)

- transferring  calls  via  a  PBX  system  to  other devices; for

example, to transfer  fax  calls  to  a  "real"  fax machine, to

transfer human  calls  to  another  phone  branch,  or  even  to

transfer calls to different persons to different branches (touch

tone controlled)

- execute  external programs to handle special calls - e.g.  voice

mail boxes, complex answering machines, etc.

- recording  of  DTMF-access  codes  for  voice  mailbox  software

(serving as a front end)

- complete answering machine functionality (message recording)

- detection of different ring  signals,  as provided by some phone

companies (currently possible only with ZyXEL modems)

ZuTSR features a real-time signal analyzer (again,  currently  for

use  with  a  ZyXEL  modem only), optimized to distinguish between

voice and data calls; very reliable and fast.

______________________________________________________________________

--

> A Fossil driver is a program to provide basic serial communication <

functions like "send a byte to  the modem" or "receive a couple of

bytes from the modem".  It is used by many  popular  programs  for

networking and BBS systems.

The  main  advantage  of  Fossil drivers is, that the main program

does not have to care about the handling of the serial hardware in

detail.  There are Fossil drivers available for almost any kind of

serial cards and similar  devices,  so programmers of applications

do not have to waste time on developing  many  hardware  dependent

routines.

Well  known  Fossil drivers for standard hardware are X00, BNU, or

DRCOMM,  and  many  others.   They  should  be  available  on many

bulletin board systems for free download, and on some ftp servers.

The interface between the application and the Fossil driver  is  a

very simple one.  The application sets some values, and then calls

the Fossil driver to do something, dependent on the values.

As  the  Fossil  driver  functions are executed using an interrupt

call, it is very easy  to  link in another resident program, which

will then be called instead of the Fossil driver  first,  look  at

the values, and modify them for various purposes.

--

>  This is the most important advantage of a Fossil driver, from the <

view of the future ZuTSR user.  ZuTSR is an utility that has to be

linked  into  the   chain   mailer   -   fossil  driver  -  serial

hardware(modem), so it will look like this :

mailer/BBS software - ZuTSR - Fossil driver - hardware (modem).

In this position, ZuTSR can watch the  data  flowing  through  the

Fossil driver from the mailer to the modem and vice versa.

For  example,  ZuTSR  can  fake  data  coming  from  the modem, by

catching data  requests  from  the  mailer  before  they reach the

Fossil driver.  Whenever the mailer  tries  to  "ask"  the  Fossil

driver  for  new  data  from  the  modem, this query reaches ZuTSR

first.  ZuTSR can return some  values, instead of actually passing

the query to the Fossil driver.  The mailer will see  data  coming

from "the modem".

--

> This  way,  ZuTSR can simulate complete responses, which appear to <

be real modem responses.   Naturally,  the  mailer will be able to

act on these responses, as if they  were  modem  responses  -  for

example  by  executing an external program on some special CONNECT

responses.

You may think, "why not execute the mailer or BBS only when a data

connection already has been  established,  instead of using tricky

memory resident programs" ?  None of the mailers (the author knows

of) is designed to  act  on  an  already  established  connection.

Mailers  like  FrontDoor,  D'Bridge, BinkleyTerm and other need to

answer the call themselves, mainly  to avoid timing problems while

doing automatic session handshaking.  They might be configured  to

do  it like described above, but this is always a difficult thing,

and normally  decreases  the  reliability  of  the complete system

compared to a solution using (a program like) ZuTSR.

______________________________________________________________________

Enhancing normal data modems  with  the  ability  to  receive  fax

transmissions  was  the  first  step  to built a device to use all

possibilities of modern phone installations.

Nowaday, many homes own an automatic answering machine, devices to

handle human calls automatically.  The fact, that modems basically

are equipped with the  all  necessary  hardware to handle not only

very specific sounds, but to process all kinds of  audio  signals,

lead  to the development of modems which have also the software to

do so.

Modems which are equipped  with  the  necessary software are often

called "modem with voice option" or "modem with voice mode".

--

> A modem used in voice  mode  is  able  to  convert  digital  data, <

received  from  a  computer system, into analog audio signals, and

vice versa.  Through the analog  interface to the phone line, this

audio signal can then be sent through the phone line and be  heard

on the other end of the line.  The usual functions like taking the

phone  off-hook,  dialing, and so on, are still available in voice

mode.

--

> While the modem converts  data,  ie.  replays greetings or records <

messages, it is able to analyze the audio signal in the same time.

It  checks  for  silence,  or  special  frequencies,  and  reports

anything special to the computer.  This way, the computer then  is

able to react on the different events, for example to hang up if a

caller  became  silent,  or  to react on touch tone codes received

from the caller (remote controlling).

A computer controlling the modem is needed beside it to record the

digital data on its hard disk,  or  to send to digital data to the

modem for digital to analog conversion.  Most  modems  cannot  act

stand-alone.   It  always needs the computer, and the computer has

to  be  running  special  software  to  make  use  of  the special

functions provided by the modem.

Therefore,  if  you  depend   on   special   software   for   data

communications,  you  cannot  use  the  new  features,  as the old

software cannot handle the voice mode.

What remains, is the method to run the old programs as subprograms

of new, specialized software,  or  to  use  tricks  to add the new

functions to the old software.

--

> For one group of those programs that cannot use  the  voice  mode, <

ZuTSR is currently the only program to add voice functionality.

______________________________________________________________________

A machine, that answers calls automatically, and shall make use of

both,  data  and  voice  mode, must be able to distinguish between

incoming voice and data calls.

--

> What makes the difference  between  those two types of calls, from <

the view of the called site ?  Fax machines send  a  special  beep

tone,  using  a  standardized frequency, the "calling tone".  This

tone can be detected and identifies the fax machine as such.  Some

modems also send  a  beep  tone,  slightly  different from the fax

calling tone, and can be identified as a modem  because  of  this.

Remaining  are a couple of modems which do not make any noise when

calling another  modem.   Therefore,  they  do  not  differ from a

calling human being in their behaviour.   A  human  normally  also

does not make any noise before the called one identified himself.

--

> You see: the answering person has to say something, and then await <

the reaction of the caller.  Depending on the reaction, it will be

able to distinguish between a calling modem and a calling human.

There  are  two  possibilities.   First,  the  answering person or

machine might start to  establish  a  data connection blindly.  If

this fails, one has to assume that the  caller  is  not  a  modem.

However, a human being, hearing the really annoying beeping sounds

from  the  phone  hand  set, normally will immediately hang up and

tell "there has been  a  fax  answering  on that number" the other

day.

Modems  are  not  that  sensitive.   So,  just  start  trying   to

"establish  a  voice  connection"  by  saying "Hello?".  If it's a

human calling, he will respond with "Hello!" or something similar.

If it's a modem  or  some  other  kind  of machine, it will remain

silent, as it cannot take any information from a spoken word.

--

> For simple handling, one might  now  just  listen  if  the  caller <

remains silent, or if the caller makes some noise of any kind.

But: Even if the caller does not make any noise, there might occur

some weird noise coming from telecommunication devices on the long

way  from  the  caller's  modem to yours, ie.  clicks, noise, etc.

Using a simple  silent/not  silent  detection,  the recognition of

modem callers therefore would not be possible in case of bad  line

conditions.

--

> Again another method might help in this cases.  You might request  <

the use of a DTMF touch tone sender from your callers.  It is very

easy to detect the presence  of  a DTMF signal really reliable, if

there is one.  The  answering  system  can  then  really  reliable

distinguish  between  data  and  voice  callers.   But  this would

require anyone who dials your number to use a touch tone sender.

Using ZuTSR,  you  will  be  able  to  realize  all  of  the methods

mentioned here, and if there is something else you can  think  of,

possibly it can be realized using ZuTSR also.

______________________________________________________________________

The actions, which will take place while ZuTSR answers a call, are

mainly  determined  by the various "events" reported by the modem.

Those might be the detection  of  a  DTMF tone, silence, voice, or

something of similar kind.

Using a "script", you define a sequence of actions to  happen  for

each single possible event which might occur while a call is being

answered.   (A detailed description of possible actions and events

is listed in the file REFER.DOC).

A single script, to be executed on event <Event>, looks like this:

Script <Event> <Action>,<Action>,...

<Event> means a single-char identifier for a special  event  (e.g.

"c" stands for for "calling tone detected", or "s" for "silence on

the line").

Some event codes are predefined in the firmware of your modem, and

you cannot use them for anything else than for what they have been

implemented in the modem.  Other event codes do not have a meaning

before you define them yourself.

A  script  with  an  event  code,  which is not a code used by the

modem, will be executed whenever

- it is started from another script using the "goto" ("G") command

- you type the event code character on your local keyboard

(upper case and lower case letters ARE different)

Following the event  code,  define  a  sequence  of actions in the

wanted order (from left to right), up to 16  per  event,  in  some

cases less.  This should be enough for most purposes.  If you need

a longer action sequence, just split it into two parts (one with a

second  event  code  that is not used anywhere else), and call the

second one  from  the  first  one  using  the  "goto" ("G") script

command.

If any other event should take place while a script  is  executed,

and there is a script defined for that event, the execution of the

current  script  is  stopped  immediately.   ZuTSR then starts the

appropriate script for that event from the beginning.

The first script, which is used  to make the actions to answer the

call in general, is always the script with the event code "-".

The simpliest script configuration for ZuTSR is something like

Script - MA,H

This causes ZuTSR to answer a call, replay a message, and then  to

hang up immediately.